**********************************************************

SSAP9 (Weight and C.G. calculation)

Copyright (c) 1989 Algor Interactive Systems, Inc.

**********************************************************

This new SSAP9 Version 8.xxx is a brand-new processor.  It

contains many new functions:

-------------------------- ------ ------ -------------------------

Description            V7.623 V8.xxx         Remarks

-------------------------- ------ ------ -------------------------

1. Include element types  Yes    Yes    New version keeps all the

1,2,3,4,5,6,7,10,12                  original available element

types

2. Include element types  No     Yes    Types 13,14 and 35 can be

8 (3-D brick),                      included in the input file,

13 (Rigid element),                  but their weights are

14 (Gap element),                    ignored

35 (Thermal bound.),

39 (3-D thermal),

40 (2-D thermal).

4. Automatically run      No     Yes    Old version need to type

and check centrifugal                "CENTRIF <CR>","RUN <CR>"

force                                to run the processor.  New

version avoids these steps

5. Exact mass moment of   No     Yes    Old version uses "lumped

inertia calculation                  mass",  whereas the new

one uses "consistent mass"

6. Area, area moment of   No     Yes    Area, Ixx, Iyy and Ixy are

of inertia for 2-D                   calculated in the new

problem                              version

7. Volume calculation     No     Yes    Total volume of the

structure is given

in the new version

8. Dynamic memory         No     Yes    "SSAP9 file-name <CR>"

allocation and                       can be used to run the

run with "command                    new processor

line"

9. Calculation speed      Slow   Swift  New version is about 40

times faster than the old

one; for some cases this

number is even bigger.

10. Processor capacity     Small  Huge    New one can handle about

35,000 nodes (NOT d.o.f)

for about 580 KB free RAM

memory, no limitation on

(1) No. of element types,

(2) No. of elements,

(3) No. of concentrated

loads,

(4) No. of load cases.

It is only limited by

(1) No. of nodes

(2) No. of material types

in one type of element

11. Create ASCII output     Yes    Yes    Old one creates ".L" file

New one creates ".WCG" file

12. Handle input files      No     Yes    Old one only handled input

used for SSAP1,4,6,                   file used for SSAP0

14,10

13. Control concentrated    No     Yes    The concentrated loads are

loads used in Weight,                 always included in the

C.G. calculation                      weight, C.G. calculation

regardless of the load

cases and input units in

the old version.  New one

has the options to let

the users make their own

choices.  The rules are

given in the next section.

14. Temporary files         Yes     No    No temporary files are

needed in the new one

15. Generate the missing    Yes     No    New version requires that

element number                        all the element numbers be

input sequentially

16. Echo complete input     Yes     No    New version only partially

information                           echoes vital input data

used in the calculation

**********************************************************

Rules on the control of "Input nodal loads/masses used in

the Weight and C.G calculation" in SSAP9

**********************************************************

The following rules apply to the input nodal loads/masses

used in the Weight, C.G. and mass moment of inertia

calculation:

----------------------- Legend -------------------------------

NDYN     = Analysis code

Static   = ALGOR static  analysis input file (NDYN = 0,6,14)

Dynamic  = ALGOR dynamic analysis input file (NDYN = 1,4)

Thermal  = ALGOR thermal analysis input file (NDYN = 10,11)

LC       = Load case number (in the input file)

KEY      = Units flag (in the input file)

(Decoder set KEY to zero by default)

FX,FY,FZ = Input concentrated load/mass

MX,MY,MZ = Input concentrated moment/rotational inertia

GR       = Gravitational constant (length/sec**2)

B == A   = Assign A value to B value (as in FORTRAN language)

|A|      = Take the absolute value of A value

> A     = Bigger than A value

< A     = Less than A value

-------- ----  ------  ---------------------  ------------------

Analysis  LC     KEY     FX,FY,FZ,MX,MY,MZ        Remarks

-------- ----  ------  ---------------------  ------------------

Static    0   -1,0,1  Ignored                 Old rule

Static    >1  -1,0,1  Ignored                 Old rule

Static    1    0      Ignored                 Old rule

Static    1    1,-1   Included, only |FX|     New rule

and |MX| are used:

FX == |FX|/GR

FY == |FX|/GR

FZ == |FX|/GR

MX == |MX|/GR

MY == |MX|/GR

MY == |MX|/GR

Dynamic 1,>1  -1,0,1  Ignored                 New rule

Dynamic   0     0     Included, only |FX|     New rule

and |MX| are used:

if (FX > 0.0) then

FX == |FX|

FY == |FX|

FZ == |FX|

if (FX < 0.0) then

FX == |FX|/GR

FY == |FX|/GR

FZ == |FX|/GR

if (MX > 0.0) then

MX == |MX|

MY == |MX|

MY == |MX|

if (MX < 0.0) then

MX == |MX|/GR

MY == |MX|/GR

MY == |MX|/GR

Dynamic   0    -1,+1  Included, only |FX|     New rule

and |MX| are used:

FX == |FX|/GR

FY == |FX|/GR

FZ == |FX|/GR

MX == |MX|/GR

MY == |MX|/GR

MY == |MX|/GR

Thermal  10   -1,0,1  Ignored                 New rule

Note: if |KEY| > 1, all the loads/masses are ignored