Algor is pleased to announce the release of SSAP1, Ver. 9.40-S

and SSAP1H Ver. 9.40-3H and 9.40-3W dynamic analysis processors.

Following are improvements in the newly released version:

1) The hyper -3H version has been made faster. Two changes have both

contribuited to this improvement. One is a more equal distribuition

of equations among blocks, and the other is the use of highly optimized

coding in the more calculation intensive parts of the program.

2) Frequencies are now includes in the ASCII files .LOG and .FRQ for

easier access. The next release of Superview should be able to display

frequency information while showing the corresponding mode shapes.

3) The saved files from a previous modal analysis of the same model

can now be used to give SSAP1 starting vectors for subspace iteration.

This can save time when a second analysis with a different number of

frequencies is desired. To use this feature, use a text editor to change

the dynamic analysis control line at the bottom of the ASCII input file.

The NFO variable in columns 36-40 should be set to the number of frequencies

in the previous analysis. The processor will then prompt for the previous

file name when it reaches this line in the file. Since it may take some time

to reach this stage, it is more convenient when running the program from

the Dos prompt to include the restart file name on the command line.

For example, the following line

SSAP1H NEWFILE /SUPTRN/RUN /#LASTFILE

will start the SSAP1H processor analyzing the file "NEWFILE" using the

SUPTRN option to suppress mode shape printing to the NEWFILE.L report

file and uses the "/#" key in the fourth command line item to indicate

to the SSAP1 program that the saved files from the analysis of "LASTFILE"

are to be used to supply starting iteration vectors.

4) Mode shape printout can now be suppressed as a run time option by using

the SUPTRN key word. The TRANS and SUPTRN options override the DEFPCH

setting on the master control line of the ASCII input file.

5) Analyses that quit because of certain solution difficulties may be able

to be solved with use of the KEEPON runtime option. Using this option

causes the processor to attempt to recover from situations that would

otherwise halt the processor. Use of this option could possibly allow

less than meaningful solutions in some cases.

Following were improvements in the previous 9.20 version:

1) The processing time and disk space required for many problems

have been reduced. In some cases the space reduction or speed-up can be

quite dramatic. The estimates of disk space necessary are now

harder to predict.

2) A change in the beam slave node logic has been made. The default

is now to simply use the master degree of freedom slot for all slaved

degree of freedom stiffnesses, i.e., use simple constraint equation

logic. The old SAP IV method is still available by setting  NPAR(7) = 1

in column 35 on the beam element control line for any beam group

where the previous SAP IV method is desired.

3) The logic allowing control of the rotational stiffness perpendicular to

the plane of plate elements has been improved. Use a value of -2 for the

coefficient for twisting moment on the type 6 plate element material

property line in columns 11-20 to disable the automatic stiffness

contribution.