************************************************

Supplemental document to ALGOR linear processors

************************************************

1-st edition : 6-26-90

I. General notes:

-----------------

Some additional ASCII output files are created by the

ALGOR linear processors:

1. (file-name).LOG (after release 9.02)

It contains processor's statistic information, such

as temporary file space, processing time and free hard

disk space.

2. (file-name).TOP (after release 9.02)

Currently it contains the options that are internally

used by the processors for the post-processing.

3. (file-name).S (after release 9.00)

It contains the stress output at the nodal points for

each element.

II. "WSO", "/RUN" options:

-------------------------

1. By default, the filename.L files will NOT contain the element

stresses.  This does not affect SSAP1 directly but will affect

the output from the restart modal superposition/combination

processors (SSAP2, SSAP3, SSAP5 and SSAP7) that use the results

from SSAP1.   In order to produce old element stress output

in the (file-name).L file from the restart processors (SSAP2,

SSAP3, SSAP5 and SSAP7),  the run-time option "WSO" must be

used in SSAP1/SSAP1H.   Note that the "WSO" option is NOT

needed for the other processors to make the "stress-to-node"

calculation.  The (filename).S11 file will be saved for the

restart processors.

2. Several runtime options are available in the FEA processor

with command line separated by "/".  For example, "SSAP1

VE05002 /TRANS/WSO/RUN" should start the SSAP1 processor

and runs the file VE05002.  The "/RUN" option should be

the last one specified.  Up to 32 characters can be used

for these options.

III. Response spectrum analysis (SSAP3, SSAP3H)

-----------------------------------------------

1. SSAP3 produces stress results for each mode shape of the

dynamic modal analysis where these results reflect the

participation factors for each mode.  These modal stress

values can be viewed with SUPERVIEW in a similar manner

as results from a static stress analysis.  In addition,

there are resultant stresses included in an extra last

"load case" viewable with SUPERVIEW.  The .DO displacement

output file also has an extra "mode shape" correspondingly.

SUPERVIEW makes it possible to display the resultant

stresses (and displacements) computed using the specified

combination method chosen for the analysis.

However, since these combination methods all involve taking

square roots of sums of squares, all resulting stress

components (or displacement components) are non-negative.

This means that results obtained by taking combinations of

these combined stresses are not very meaningful.  In

particular, superview beam dithers will not be meaningful

for the "extra mode" unless version 1.001 (or later) of

MKNSO3/MKNSO3H is used.  Von mises, "vector" stress tensor,

and Tresca combinations are also generally not meaningful

for the "extra mode".

Component stress dithers from the SUPERVIEW "S tensor"

sub-menu should be used to examine the resultant stresses.

In addition, for element types 3, 4 and 5, principal stresses

are calculated from components for each mode and are then

combined by the selected combination method, therefore

principal stress dithers for these element types are RMS

based values.  However, for other element types the

principal stresses are computed in superview and should not

be used as the resultant RMS stresses.

The "extra mode" displaced shape can be useful but it must be

realized that the combination method forces all displacement

components to be non-negative, and that "cusps" or "notches"

may appear.  A little thought should make it clear which

features of the combined displaced shape are easily

explainable artifacts of the combination method.

IV. Direct time integration for dynamic analysis (SSAP4, SSAP4H)

----------------------------------------------------------------

1. The single displacement component having the largest absolute

value  for all time steps is printed to the screen and is

written to the filename.L file.  CAUTION: the highest stresses

may not correspond to this time step having the maximum

displacement component. Note that except for 1-dimensional

problems the highest nodal (vector) displacement may not even

occur in the same time step as the maximum displacement

component, i.e., since DS=SQRT(DX**2 + DY**2 + DZ**2), maximum

DS may not occur at the same time as maximum DZ because DX and

DY also contribute.  Note also that the printed displacement

component may be a rotation in units of radians, if (due to

model size or choice of length units) the numerical magnitude

of some rotation is greater than that of any translation

component.

2. The accelerations at each output time step are now written to

a filename.AO file.  This file has identical formatting to the

filename.DO file, and can be best examined at present by one

of two methods. One is to use the TRANDO module to convert

the file to ASCII format. The other is to rename the file

to have a .DO extension, for example by typing

"REN filename.AO newname.DO".

Then the acceleration components can be examined just like

displacements.  The main way to do this with graphics

is to copy the filename.SST file to newname.SST, then use

SUPERVIEW while remembering that what you are seeing are

accelerations.  Another way is by using the SDNODE program

to make a graph plotting the desired acceleration component

value as a function of time step for viewing with SUPERDRAW

-II. The non-graphical POSTD program for producing tabular

displacement data can also be used (if there are less than

100 output time steps since the current POSTD version is

limited to 100 load cases).

3. Even if the interval for output time steps does not divide

evenly into the number of time steps to be used, the final

time step results will be written to the filename.TSO,

filename.DO, and filename.AO files.

IV. Eigenvalue buckling analysis (SSAP6/6H, SSAP6S/6SH)

-------------------------------------------------------

1. Since the buckling shape is normalized to the largest

component of the vector, buckling direction information may

be lost during this procedure.

2. A negative buckling load multiplier refers to the buckling

loadings that are reverse to the applied loading direction.

3. The SSAP6/6S processor produces an .SO file corresponding

to the static stress analysis. This file can be converted

into a analysis stress distribution.

V. Random vibration analysis (SSAP7/7H)

---------------------------------------

SSAP7 is used to analyze structures under spectral density

loading.  Power spectral density loading consists of g**2/hz

versus frequency.  This method is widely used in aerospace

applications.

The random vibration processor is a helpful tool in the design

and analysis of vibration systems.  It can be used in the

evaluation of a preliminary design to find the probability of

exceeding a specified response.

In random vibration analysis, excitations are assumed to follow

a Gaussian (normal) distribution with zero mean value.  This is

a reasonable assumption for practical applications based on the

central limit theorem.  Since the vibrations are a random

phenomena, and this method is based on probability theory, it

cannot be guaranteed that the displacement will not exceed the

specified value.  However, with the random vibration processor,

we can predict the probability of exceeding this response.  This

technique is found in practical applications to provide confidence

that a design will work.

To use the random vibration processor, the user must supply the

curve of Power Spectral Densities (PSD) amplitudes versus

frequencies.  The PSD's are associated with the vibrations or

excitations, which in general can be obtained by means of an

analog spectral density analyzer.  A random vibration data

preparation program, PSSAP7, is supplied to help the user

interactively define the data needed by the processor.  The

excitation PSD can be specified in any combination of 3-D

directions.  The PSD can also be coupled with cross-spectral

densities.

The random vibration processor takes the spectral densities input

and computes the RMS (square root of mean square) responses, i.e.,

displacements, forces, moments or stresses.  The RMS responses

then characterize the probabilities of the system responses.

There are three methods that can be used to compute RMS responses.

The approximation method uses closed-form integration and neglects

cross-spectral effect.  The second method uses the Gaussian

numerical integration technique to compute the modal spectral

matrix.  This technique will allow the user to examine the

cross-spectral excitation effect.  The user can also turn off this

effect if desired.  The last method uses the same technique as the

second one but neglects the cross-spectral effects.

This processor can handle modal analysis file from SSAP1

or SSAP1C (composite element modal analysis).  This will

allow the user to use composite element for Random Vibration

Analysis.

The SSAP7 will call MKNSOD - dynamic stress analysis module

after it finishes the analysis. MKNSOD will produce graphic

files .NSO to be used in the SuperView.

A verification problem, VE02006A (SSAP7 input file), can be found

in the VERIFY sub-directory from \ALGOR.