ALGOR  Dynamic Stress Processor - MKNSOD.EXE Release Note

MKNSOD Version 1.02  - Release note 7/7/90

1) This version allows the suppression of the filename.S file

by using the "NOS" option when starting SSAP3 or SSAP7. Use of this

option can substantially reduce the amount of hard disk space

required.

2. A filename.LOG file shows both disk storage and elapsed time,

as well as showing the release date and version number of the

program modules involved in stress calculations.

3. The 386 HYPER versions are now linked with a different linker

for wider compatibility with memory resident programs.

MKNSOD Version 1.002 - Release Note 8/30/89

1) For plate element, the nodal stress calculations scheme has been

modified such that a smoother results will be calculated.

2) The composite element output from previous versions contain detailed

results at each integration point as well as those at node points.

However, the output are voluminous. The new version, by default, will

output only node point results. If results at Gaussian points are

desirable, the user can set NPAR(6) (see Reference Manual) to 1, and

more detailed results will be printed.

3) Enhancements have been implemented to avoid some real*4 to real*8

conversion problems.  Some diffculties had occurred when very small

coordinate numbers are used with extremely large element stiffnesses.

This numerical problem has been cured in the current version.

MKNSOD Version 1.001 - Release Note 7/20/89

Algor is pleased to announce the release of MKNSOD dynamic stress

processor.  The MKNSOD processor will take the displacement output

file and compute node point stresses. The stresses will be first

computed within the element, and then extrapolated to the nodes

of the elements.  This will give the user more accurate and much

easier to use stress results.

MKNSOD should only be used for SSAP3 and SSAP7.  The processors

will automatically call MKNSOD to compute dynamic stresses. It

will be transparent to the user. For SSAP3, the MKNSOD will

further chain to the MKNSO3 automatically.

MKNSOD will recognize all elements which currently available in

the SUPERSAP FEA processors (types 1,2,3,4,5,6,7,13,14,15,16).

Internally, MKNSOD will need a new .SST file to compute the

element stresses.  MKSV.exe can be used to produce the .SST file

for MKNSOD.exe. However, this should be transparent to the user

since the processor will automatically call MKSV and MKNSOD.