Microsoft LAN Manager 2.0 Installation

Follow the installation instructions in the LAN Manager

Installation Guide. The "Importing Additional Network Device

Drivers" section describes the process of loading the drivers,

and the "Network Drivers Dialog Box" section describes how to

specify which drivers to use.

NOTE: Under OS/2, CVS16LM.OS2 must have access to the file

CVS16LM.MSG. This can be accomplished by setting the DPATH

variable in the CONFIG.SYS file to the directory where

CVS16LM.MSG is located.

Microsoft LAN Manager 1.0 Installation

Install LAN Manager as described in the LAN Manager

installation guide, if you have not already done so. Then

perform the following steps to install the Corvus 16/4 Token-

Ring adapter drivers for LAN Manager.

1. Copy the appropriate driver, CVS16LM.OS2 or CVS16LM.DOS,

from the installation diskette to the desired fixed disk

drive and directory.

2. Add the following line to the CONFIG.SYS.

DEVICE=[drive:directory]CVS16LM.OS2

or

DEVICE=[drive:directory]CVS16LM.DOS

This line should be entered immediately following the line

which installs the PROTMAN.OS2 or PROTMAN.DOS device

driver.

3. Add the following section to PROTOCOL.INI

[CVS16LM_NIF]

DriverName = CVSLM$

4. Add CVS16LM_NIF to the bindings list of the protocol in the

PROTOCOL.INI. An example protocol section follows.

[NETBEUI_XIF]

DriverName = NETBEUI$

Bindings = CVS16LM_NIF

CVS16LM Parameters

The PROTOCOL.INI file defines the configuration of protocol

and NDIS drivers. The following table summarizes CVS16LM

parameters. For additional information about the PROTOCOL.INI

file and network device drivers, refer to the Microsoft LAN

Manager Network Device Driver Guide.

�������������������������������������������������������������������������Ŀ

�  Parameter         �         Range                � Default  � Default  �

�                    �                              �   DOS    �   OS/2   �

�������������������������������������������������������������������������Ĵ

�  DRIVERNAME        � CVSLM$ or CVSLMn$            � Required � Required �

�  IOADDRESS         � 0xA00, 0xA20, 0xA40, 0xA60   �     -    �     -    �

�  MAXTRANSMITS      � 3 - 50                       �     3    �     6    �

�  NUMRECEIVEBUFFERS � 3 - 100                      �     3    �     6    �

�  RECEIVEBUFFERSIZE � 256 - 17960                  �    2048  �    2048  �

�  PSEUDODMA         �                              �     -    �     -    �

�  NOEARLYRELEASE    �                              �     -    �     -    �

�  NETADDRESS        � "400000000000"-"7FFFFFFFFFFF"�    BIA   �    BIA   �

�  WINDOWS386        �                              �     -    �    N/A   �

���������������������������������������������������������������������������

DRIVERNAME

Identifies the network device driver. The DRIVERNAME is CVSLM$

for the first driver and CVSLM2$ for the second, and so on.

This parameter is required.

IOADDRESS

If no IOADDRESS is specified, the driver will search for an

adapter at all I/O addresses until one is found. Otherwise,

only the specified I/O address is checked. This parameter is

required when multi adapter support is desired.

MAXTRANSMITS

Sets the number of transmits that can be queued simultaneously

by the driver.

NUMRECEIVEBUFFERS

Sets the number of receive buffers to allocate.

RECEIVEBUFFERSIZE

Sets the size of buffers used to queue received frames in the

driver. Several buffers may be required to queue a large

frame. Performance is improved if a received frame fits in one

buffer. Use the following formula to determine maximum frame

size. MediaMaxSize equals 17960 @ 16 Mbps, or 4490 @ 4 Mbps.

MaxFrameSize =

min(min(8, NumReceiveBuffers) x ReceiveBufferSize, MediaMaxSize)

PSEUDODMA

This entry informs the driver to use pseudo DMA on 16 bit ISA

adapters. Normally, 16 bit adapters use DMA bus master mode.

Note that the pseudo DMA option is not supported on Micro-Channel

adapters. Also note that the 8 bit ISA adapter does not support bus

master DMA, and always defaults to the pseudo DMA mode. This entry

is not followed by an equal sign.

NOEARLYRELEASE

This entry informs the driver to disable early token release.

Early token release reduces the average time another network

adapter must wait to transmit a frame. This entry is not

followed by an equal sign.

NETADDRESS

This entry allow the driver to override the burned in address

(BIA). The entry must be a 12 character string of hexadecimal

digits, and must be enclosed in quotes.

WINDOWS386

This entry informs the driver that it is going to be run under

Microsoft Windows in 386 enhanced mode. This entry may also be required

when using other 386 control programs such as VM 386 and Quarterdeck QEMM.

This parameter is not necessary when utilizing an 8 bit ISA adapter or if

pseudo DMA is enabled. This entry is not followed by an equal sign and is

valid only for the DOS driver.

For example, the following is a sample PROTOCOL.INI file:

[CVS16LM$_NIF]

DRIVERNAME = CVSLM$

IOADDRESS = 0xA20

MAXTRANSMITS = 10

MAXRECEIVES = 10

NUMRECEIVEBUFFERS = 12

RECEIVEBUFFERSIZE = 1024

NETADDRESS = "400000000312"

PSEUDODMA