Local Area Network Software Drivers

for

TokenCard EliteSeries LAN Adapters

KNOWN OR SUSPECTED PROBLEMS

17 October 1992

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

Following is a list of known or suspected problems with drivers on

this version of the SuperDisk driver diskette.  This list is here

to help you install and use the TokenCard EliteSeries successfully.

All of these problems are being investigated and fixed, and the

latest drivers are always available from SMC's Technical Support

Department and BBS (refer to README.DOC file).

----------------------------------------------------------------

Problem Summary for: TRDIAG -- Token-Ring Diagnostic Program

----------------------------------------------------------------

Title: <Enter> during cable MAU loopback causes error

Summary: Hitting Enter during Cable/MAU loopback test is causing

TRDIAG to generate an incorrect message.

Advice: Ignore the message.

Title: TRDIAG Cable/MAU Loopback test.

Summary: The TRDIAG Cable/MAU Loopback test F6 "set

parameter function" has an unneeded arrow key function;

there is no reference to the ESC key to get out of the

help screen.

----------------------------------------------------------------

Problem Summary for: Token-Ring NetWare 286 IPX Server Driver

----------------------------------------------------------------

Title: NetWare External Router stack overflow

Summary: When an IPX load or read/write file test is performed

across an external router, a

'stack overflow-system halted' message is received.

Error message appears on the Router.

Advice: Use a 386 DX or better for External Router.

Title: Fails the Read/Write Test in Dedicated mode.

Summary: During the Read/Write File test, the workstations

lose their ring connections.  This happens

before the one hour duration of the test.  The error

message is "Network Error on Server DEV22: Error receiving

from network".

Advice: This is not expected to affect users.

----------------------------------------------------------------

Problem Summary for: Token-Ring NDIS Driver

----------------------------------------------------------------

Title: Refers to 8003 in error message

Summary: If unable to initialize the adapter the driver

generates the error message: "Error: No 8003 Adapters

Specified in PROTOCOL.INI Present".

Advice: Interpret the reference as '8115'.

Title: Lost connections with OS/2 v2.0 and LAN Server v2.0

Summary: Workstations lose their connections to the server after

an extended period of performing intense file operations.

IBM's Token-Ring adapters perform the same way.

Advice: Use OS/2 v1.3.2 on the server.

Title: NDIS driver did not initialize the second adapter properly

Summary: NDIS driver failed to initialize the second

adapter correctly in DOS and OS/2 environments.  In both

cases the driver failed to bind to the second adapter

correctly.

Advice: Manually edit the configuration files.

Title: NDIS driver hangs when cable isn't connected to adapter

Summary: When the cable is not connected to the configured port of

the adapter, driver hangs after the NET START command

is issued.

Advice: Don't run applications with the adapter cable disconnected.

Title: MS LAN Manger Server failure

Summary: When running a four-node endurance test using NDIS,

the server failed after running for one day.

Advice: This is not expected to affect users.

Title: OS/2 NDIS driver error under DEC Pathworks

Summary: NDIS OS/2 driver is causing DEC Pathworks V2.0

to generate error messages when "Net Start Workstation"

command gets executed.

Advice: Call SMC Technical Support.

----------------------------------------------------------------

Problem Summary for: LAN Support Program (LSP) for DOS

and

LAN Adapter Protocol Support (LAPS) Program

for OS/2

----------------------------------------------------------------

Title: Does not install two SMC8100.OS2 (.DOS) properly.

Summary: Does not install two DEVICE=\SMC8100.OS2 (.DOS) in the

CONFIG.SYS file when using it to configure two adapters

and the NDIS driver.

Advice: Manually edit the CONFIG.SYS and PROTOCOL.INI files to

enter the proper commands.

Title: Installed wrong "DriverName=" for second card.

Summary: When using LAPS or LSP to configure 2 adapters in the

system, generates a "DriverName=SMC81002$" in the

PROTOCOL.INI entry for the second card instead of

"DriverName=SMC8102$".

Advice: Manually edit the PROTOCOL.INI file to change "81002"

to "8102".

Title: Suggests that users remove "DEVICE=TRINIT.EXE (.SYS)" from

the CONFIG.SYS file.

Summary: When updating using LAPS or LSP, if you have the correct

statement "DEVICE=TRINIT.EXE (.SYS) " in the CONFIG.SYS

file, program will suggest that the user remove that

statement.

Advice: Ignore the suggestion to remove TRINIT.

----------------------------------------------------------------

Problem Summary for: DOS PC AGENT/SNMP

----------------------------------------------------------------

Title: Won't load high if available memory is marginal

Summary: If I have a high memory block with approximately 28-45K

bytes free, PC AGENT/SNMP doesn't load at all and gives

a "cannot execute pcagent" error message.

Advice: Requires at least 55K free memory to load high, but

returns 30K to system after initialization;

otherwise load in low memory.

----------------------------------------------------------------

Problem Summary for: Token-Ring NetWare 386 3.11 ODI Server Driver

----------------------------------------------------------------

Title: Will not allow the removal of multicast addresses

Summary: Will not allow the removal of multicast addresses

from the multicast table.

Advice: Unload and reload driver to clear multicast address

table.

----------------------------------------------------------------

Problem Summary for: LAN Adapter Protocol Support (LAPS) Program

for OS/2

----------------------------------------------------------------

Title: LAPS did not install two SMC8100.OS2 properly.

Summary: LAPS did not install two DEVICE=\SMC8100.OS2 in the

CONFIG.SYS file when using it to configure two adapters

and the NDIS driver.

Advice: Manually edit the CONFIG.SYS and PROTOCOL.INI files to

enter the proper commands.

Title: LAPS installed wrong "DriverName=" for second card.

Summary: When using LAPS v1.3b to configure 2 adapters in the

system, LAPS generates a "DriverName=SMC81002$" in the

PROTOCOL.INI entry for the second card instead of

"DriverName=SMC8102$".

Advice: Manually edit the PROTOCOL.INI file to change "81002"

to "8102".

Title: LAPS suggests that users remove "DEVICE=TRINIT.EXE" from

the CONFIG.SYS file.

Summary: When updating using LAPS if you have the correct statement

"DEVICE=TRINIT.EXE" in the CONFIG.SYS file, LAPS will

suggest that the user remove that statement.

Advice: Ignore the LAPS suggestion.

TokenCard and SuperDisk are trademarks, and SMC and Standard

Microsystems are registered trademarks of Standard Microsystems

Corporation.  Other trademarks mentioned herein belong to their

respective companies.