README.DOC - Fax Software for OS/2

Copyright (c) 1992, 1993 by Keller Group Inc.

All rights reserved.

July 9, 1993

Topics contained in this README.DOC file:

MODEM CONFIGURATION SETTINGS

MULTI-LINE NOTES

SUPPORT FOR GAMMALINK'S GAMMAFAX PRODUCTS

NEW FEATURES

MODEM CONFIGURATION SETTINGS

----------------------------

Tested modems are listed below.  Unless the manufacturer has changed

the product since the time of our testing, you should use the

following information to determine correct Options/Configure values

for your fax hardware.  (Many other modems will also work, but the

vendors have not provided us with evaluation units for testing.)

MANUFACTURER  MODEL                          CONFIGURATION

Adtech Micro  Externally Yours! (ROM v2.2)   Class-2, might need 'No DLE esc'

AT&T Paradyne DataPort Fax Modem             Class-1

BIT Blitzer   XM124S/RFAX                    Class-2

Brooktrout    TR112 (with OS/2 BTLIB 1.04+)  uses FaxBT.adp as Fax.adp

Complete PC   Complete Modem Plus            Class-1

CompuCom      Speedmodem Combo (ROM v2.77)   Class-2

CPI           VIVA 14.4k internal            Class-1,9600bps,Spec cmd &C1&D3

Dataplex      DPX-223-55 2400/9600 rom5.42   Class-2

Dataplex      DPX-225-01 14,400 (ROM 1.00)   Class-2

Digicom (DSI) Scout Plus, Spirit II          Class-1

Dove          DoveFax (ROM v4.2)             Class-1

Dr. Neuhaus   FURY 9600, 9600 TI, 14.4 TI    Class-2

Dr. Neuhaus   FURY 2400 POCKET/FAX, PC/FAX   Class-2

Dr. Neuhaus   FURY 2400 PC JUNIOR            SendFax

Dr. Neuhaus   FURY 2402 TI/FAX               SendFax

Elsa          MicroLink 14.4T (ROM 2.01)     Class-2, Spec cmd $K1&R7

Everex        EverFax 24/96 (COM1/2 only)    Cls2,Alt bit order,Xon recv start

GVC           FAXMODEM 9624Vbis              Class-1 or Class-2

Hayes         Optima 24/96                   Class-1

Hayes         Optima 96/96                   Class-1

Hayes         Optima 14.4/14.4               Class-1

IBM           L40SX laptop modem             Class-1, No BPS s/d,Skip TCF read

IBM           ThinkPad Fax Modem             Class-1

Image Comm    TWINCOM 24/96                  Class-2

Image Comm    TWINCOM Commuter 24/96         Class-1

Intel         SatisFAXtion classic           uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/200               uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/400  (internal)   uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/400e (external)   Class-1

Intel         SatisFAXtion/100               Class-1

Megahertz     96/96 FAX/Modem (P296FMV)      Class-1, Special cmd \J0\Q1

MICC          9642                           Class-2

MultiTech     MultiFaxPC MT932               Class-2, Alt bit order

MultiTech     Multi Modem MT932EAF           Class-2, Alt bit order

MultiTech     Multi Modem II                 Class-2, Alt bit order

MultiTech     Multi Modem V.32 Fax           Class-2, Alt bit order

NetComm       M4F or M7F (ROM rev K)         Cls-2, Special cmd +FDCC=,3,0,2,0

Practical     2400FX96                       Class-2

Practical     2400 SendFAX                   SendFax

Practical     14400FXSA V.32bis              Class-1, Check TCF signal

Practical     14400FXMT V.32bis              Class-1 or Class-2

Prodigy       Eagle                          Class-1

QuickCom      Spirit II                      Class-1

Supra         SupraFAXmodem V.32bis          Class-1 or Class-2

TwinCom       TwinCom v1.13C                 Class-2

UDS           FasTalk Fax32                  Class-1

USRobotics    Courier HST Dual Std w/Fax     Cls-1,Spec cmd &H2,Recv EOP delay

USRobotics    Sportster 14,400               Cls-1,Spec cmd &H2,Recv EOP delay

USRobotics    Worldport 2496 V.42bis         SendFax

USRobotics    Worldport 9696 MNP5 FAX        SendFax

USRobotics    Worldport 9696 V.42 w/Fax      Class-1, Special cmd &H2

USRobotics    Worldport 14400 V.42 w/Fax     Class-1, Special cmd &H2

ZyXEL         U-1496 (ROM 5.05 & 6.01 good)  Class-2 (ROM 6.00 is bad)

Zoltrix       ZX1896 or Zofax 96/24          SendFax

Zoltrix       9624 Class 2 (regular)         Class-2

Zoltrix       Class 2 with V.42 MNP          Class-2, might need 'No DLE esc'

Zoom          FaxModem FC 9624 (internal)    Class-2

Zoom          FaxModem FX 9624 (external)    Class-2

Zoom          V32bis (ROMs 1.56D,2.0 tested) Class-1, 9600bps, Spec cmd &C1&D3

(or Class-2)

Class 1 modem note: For most Class 1 modems, using the 'Check TCF

signal' on the Options/Configure screen will provide enhanced speed

downgrading.  This may improve the quality of received faxes.

The Everex EverFax 24/96 internal is supported, but because the board

does not allow IRQ selection, it must generally be used as COM1 or COM2.

Some users have reported that the following modems work

best when used in Class-1 mode, 9600bps, Special cmd of &C1&D3.

Boca          Bocamodem 14.4Kbps (ROM v1.27) ROM update was requested

Modems that failed our compatibility testing:

Calculus      EZ-FAX                         Proprietary fax commands

Complete PC   certain models                 Proprietary fax commands

DSI           digicom 9624LE                 SendFax non-spec

Formosa       MoFax MF-96P                   SendFax train failure

Frecom        Fax96                          Non-standard chip set

Intel         Connection CoProcessor         Proprietary fax commands

Omnitel       9624+S/R                       SendFax recv failures

Omnitel       Omnitel v1.10                  Class-2 recv failures

MULTI-LINE NOTES

----------------

If you have purchased a multi-line version of the program, you can

install multiple fax hardware devices and fax telephone lines, and

then do simultaneous fax sending and/or receiving on these lines.

Different versions of the fax software are available to support a

maximum of 2, 4, 8, 16 or 32 lines, but the practical limit may be

determined by your type of fax hardware.

On a given CPU, the multiple fax hardware devices must be of the same

type.  A single Options/Configure screen controls the configuration

for all the fax hardware devices.  The Options/Ports screen is used

to configure the individual fax lines for sending, receiving or both.

The selected lines are automatically used for simultaneous

sending/receiving.

Fax modems - It is reasonable to use two Class-1 fax modems or up to

four Class-2 fax modems.  Either internal or external fax modems can

be used, and while the modems do not need to be identical, they must

all use exactly the same values on the Options/Configure screen.

Each fax modem must be on a separate COM port in your system, and the

COM ports must be standard hardware ports (although the ports need

not be at the conventional COM port address and IRQ settings since

you can specify these settings on the FMD.SYS line in your CONFIG.SYS

file).  Serial boards that provide simulated ports using a special

COM.SYS device driver are not supported (e.g., ARTIC or Digiboard).

Serial boards that provide direct hardware ports should work fine

(e.g., the STB 4-COM card, or other serial port cards that provide

multiple 8250-compatible or 16550A-compatible UARTs).

SatisFAXtion/200 or SatisFAXtion/400 boards - You can use up to eight

internal boards.  Be sure to read about SatisFAXtion boards in the

appendix of the User's Manual before proceeding.  All of the

SatisFAXtion boards must be of the same type (all /200, or all /400)

since the same DOWNLOAD.* and LOADER.* file is used for all boards.

Each board must be set to a unique I/O Port Address.  As described in

the appendix, you use either Intel's SETUP.EXE program (on an ISA bus

machine) or a Reference Disk (on a MCA bus machine) to change the I/O

Port Address.  Since Intel's SatisFAXtion board installation

procedures only work with one board at a time, you must insert one

board, power up your machine and do the necessary board I/O Port

Address changes to place the board at a unique address, power down

your machine and remove the board, then repeat this procedure for the

next board.  After all boards have been separately configured for

unique I/O Port Addresses, you can then insert all the boards in your

system.

Brooktrout boards - If you have properly installed the OS/2 runtime

software from Brooktrout, the Brooktrout ports will appear in the

Options/Ports list.  It is theoretically possible to run up to 32

lines using Brooktrout fax hardware.  At this time, Brooktrout ships

OS/2 software only for their TR112 fax board (two-line board), but we

will be working with Brooktrout to support their TR114 (four-line)

and other boards.

GammaLink boards - The GammaFax ports will appear in the

Options/Ports list.  The fax program interfaces with the GammaFax

boards via the GammaFax Dispatcher, so it can operate simultaneously

with many other GammaFax software products which also use the

GammaFax Dispatcher.  See the separate installation notes for

GammaLink hardware (below).

SUPPORT FOR GAMMALINK'S GAMMAFAX PRODUCTS

-----------------------------------------

The GammaFax adapter allows the operation of intelligent GammaFax

boards, such as the CPi, CP, CP MC, XPi, XP and XP MC.  Since it

interfaces with the board via the GammaFax Dispatcher, it will

operate simultaneously with many other GammaFax software products

which also use the GammaFax Dispatcher.  The GammaFax adapter

disables reception on the fax channels which it uses when it is not

waiting for a fax.  Therefore, if an application assumes that

incoming faxes will be automatically answered by the Dispatcher, it

may not function correctly on a line that is enabled for use by the

adapter.  Otherwise, applications should operate normally when

sending or when assigned separate lines.

Install the GammaFax OS/2 software as directed in GammaLink's

instructions.  It is recommended that the GammaFax software be

installed in a directory different than the fax program.  For our

examples that follow, we are assuming that the GammaFax software is

installed in the directory "C:\GAMMA", while our fax software is

installed in the directory "C:\FAXWORKS".  Be sure to modify the

pathnames if you use different directories.

Edit the command scripts \GAMMA\DISPATCH.CMD and \GAMMA\MENUS.CMD to

reflect the fax program's installation directory.  For example, if

the fax program is installed in C:\FAXWORKS and the GammaFax OS/2

software is installed in C:\GAMMA, the you must edit the first line

of both the C:\GAMMA\DISPATCH.CMD and C:\GAMMA\MENUS.CMD files to be

"set GFAX=C:\FAXWORKS".

Edit your CONFIG.SYS file to include the line "set GFAX=C:\FAXWORKS".

(You will need to reboot your system so that the changes to your

CONFIG.SYS file will take effect.)

Edit the configuration file, \GAMMA\GFAX.$DC, to include the lines

below, in addition to, or replacing, the lines provided by the

GammaFax installation program.  The last 4 lines should be repeated

for each logical and physical channel which will be controlled by the

fax adapter.  The first 3 lines specify timeout values in seconds

which may be adjusted, as desired.  For lower disk usage, increase

these values.  For quicker response, decrease them.

controlt 10

queuet 15

statust 30

channel <1> <0> <GFAX1.01>	*see note a below

gfxshutdown <1> 2		*see note b below

gfxform <1> 3			*see note b below

gfxwait <1> 15			*see note b below

Notes:

a. The value <1> should be replaced with the desired logical

channel number, such as 1.  The value <0> should be replaced

with the corresponding physical channel number. The name

GFAX1.01 should reflect the current chassis number (the first

"1"), and the logical line number (the "01").

b. The value <1> should be replaced with the desired logical

channel number.

c. Refer to chapter 17, "Queue Manager Configuration" of the

"GammaFax Reference Manual" for more information about

the configuration file.

If necessary, see the examples of complete configuration files that

are included at the end of this section.  Refer to the second example

if you wish to use DTMF routing with the LAN fax software.

Start \GAMMA\DISPATCH.CMD in an OS/2 window.  If desired, you can add

\GAMMA\DISPATCH.CMD to your STARTUP.CMD or create a program object

with the Path and File Name set to C:\GAMMA\DISPATCH.CMD and place

the object in your startup folder.

Start the fax program and set the ports to be controlled using the

Options/Ports screen.  Using the Options/Configure screen, set the

adapter for the proper phone line type (recognizable dial tone or

not) and dialing style (tone or pulse).

The Q File option on the Options/Configure screen defaults to

"GFAX.$QU", which will cause the program to look for the GammaFax

GFAX.$QU file in the fax program directory (which is C:\FAXWORKS in

our example).  If desired, you can explicitly set the full pathname

(such as "C:\FAXWORKS\GFAX.$QU") of the GammaFax Q file.

Set the Q Poll Time variable on the Options/Configure screen as

desired.  This is generally set to the same value as the "queuet"

configuration value.

----------

Example 1 - Here is a example of a complete configuration file for a

single line GammaFax installation.

chassis 1

buffers 2

numchan 1

controlt 10

queuet 15

updatet 300

channel 1 0 GFAX1.01

country 1 1

init 1

boardtype 1 0x80

load 1 C:\GAMMA\GFXCP.BIN

csid 1-612-555-5555

modemctrl 1 1024 1

gfxshutdown 1 2

gfxform 1 3

gfxwait 1 15

statust 30

Example 2 - This is an example of a multiline configuration file, and

also demonstrates the use of DTMF input for routing incoming faxes.

This configuration is for a 3-line system, with line 1 being a CP

board, and lines 2 and 3 being CPi boards.  (From what we understand,

the CPi and XPi boards are like the CP and XP, but with built-in DTMF

reception - the CP board requires an optional daughter board for DTMF

input).

To enable DTMF input, lines like the following must be added to the

GFAX.$CD file:

gfxdtmftimeout 2 6 15

gfxdigits 2 10 0

gfxdtmftone 2 440 500

These lines are added as part of each card's configuration.  The "2"

in each of these lines refers to the line number, so it changes for

each card.  The first line specifies an interdigit timeout (6

seconds) and a total timeout (15 seconds).  The second line specifies

the maximum number of DTMF digits to be entered (10 digits) and the

terminating digits (0 for none).  The third line specifies the tone

that will be used to promt the caller for entering the DTMF digits

(440Hz for 500ms).  See the GammaFax release notes for a complete

description of the values for DTMF input.

chassis  1

buffers  2

numchan  3

controlt 10

queuet 15

updatet 300

statust 30

channel 1 0 GFAX1.01

country 1  1

init 1

boardtype 1 0x8O

load 1 C:\GAMMA\GFXCP.BIN

csid 1 612-555-5555

modemctrl 1 1039 0

gfxshutdown 1 0

gfxform 1 3

gfxwait 1 15

gfxspeaker 1 3

gfxtwod 1 0

channel 2 1 GFAX1.02

country 2 1

init 2

boardtype 2 0x283

load 2 C:\GAMMA\GFXCX.BIN

csid 2 612-555-5555

gfxshutdown 2 0

gfxform 2 3

gfxwait 2 15

modemctrl 2 1039 0

gfxspeaker 2 3

gfxtwod 2 0

gfxdtmftimeout 2 6 15

gfxdigits 2 10 0

gfxdtmftone 2 440 500

channel 3 2 GFAX1.03

country 3 1

init 3

boardtype 3 0x283

load 3 C:\GAMMA\GFXCX.BIN

csid 3 612-555-5555

gfxshutdown 3 0

gfxform 3 3

gfxwait 3 15

modemctrl 3 1039 0

gfxspeaker 3 3

gfxtwod 3 0

gfxdtmftimeout 3 6 15

gfxdigits 3 10 0

gfxdtmftone 3 440 500

NEW FEATURES

------------

The following is a partial summary of new features that are included

in this release.  The number indicates the program version that first

included the feature.

1.16 - When a received fax terminates in an error status, pages that

were received before the error are retained.

1.16 - Text line spacing is now 120% of the current font point size.

Previously, line spacing was derived from the font, so the number of

lines per inch in a given point size could vary based on the font.

1.16 - Added a "No BPS stepdown" check box on the Options/Configure

screen for the L40SX modem.  This fax modem is still unable to send

to fax machines that require stepping down to slower baud rates, but

if this check box is used, the program will retry without stepping

down, which may allow the machines to communicate in some

circumstances.

1.16 - Enhanced capturing with the Utilities/Capture to clipboard

command.  Colored icons, including OS/2 buttons such as

minimize/maximize, now map to a pattern rather than turning black.

1.16 - Added support for the Intel Hand Scanner with SatisFAXtion

boards (see the section in this README.DOC file for details).

1.22 - For the Printer Driver Configuration screen: The program now

sets the fax directory for the printer driver, so the "Fax directory"

field has been removed.  The "No log entry" action has been removed.

A "Tones" field has been added to enable/disable the beeping tones

during printer driver operation.

1.22 - The Send Fax screen now includes a "To" field that shows a

list of all selected destinations.  You can remove a destination by

double-clicking on its line.

1.22 - The "Cover bitmap" pull-down list on the Fax Send screen now

includes BMP files from your fax directory and your working directory.

1.22 - Current call sending is now started by pushing the "Send

current" button on the Send Fax screen.  If you select multiple

destinations, the Send button changes to indicate the number of

destinations.

1.22 - The "Fax local id" field on the Options Settings screen now

supports alphanumeric characters.  This allows you to send text as

your fax id.  (This only works if your fax hardware allows it... some

fax hardware will only accept numerics.)

1.22 - The field widths in the log display have been adjusted to

compensate for OS/2 system font changes in the OS/2 2.0 Service Pack

update (and OS/2 2.1).

1.22 - For the IBM L40SX laptop and other machines with a "suspend"

mode, the program can now receive a fax when the machine is

"resumed".  This did not work in previous releases because the L40SX

fax modem is reset by the suspend/resume procedure.  If the program

is left in receive mode, it will now reinitialize the modem and be

able to answer fax calls when the machine resumes.  Since a suspended

machine is automatically resumed by an incoming call, this means that

a suspended machine can receive faxes.  After the fax has been

received, the L40SX can automatically suspend itself again after a

time delay (which is set on the L40SX by the L40's "PS2 SET POwer off

minutes" command).

1.22 - Also of interest to L40SX users, the "No BPS Stepdown"

check box on the Options/Configure screen now applies only to

sending.  The L40SX fax modem can correctly receive at slower baud

rates, but does not correctly support sending at speeds slower than

9600 baud.

1.23 - Tab and cursor key support is improved in the INSTALL program.

1.23 - Alt-F4 now works as a short-cut key for closing the application.

1.23 - Cover sheets now use proportional fonts for enhanced

appearance.  The order of preference (using the first one that is

available on your system) is Helvetica, Times Roman, then Courier.

1.23 - A "Utilities/Edit phone books" command has been added to

permit phone book editing without having to use the Fax/Send command.

1.23 - The application now shuts down fax modems upon program exit,

thereby leaving the modem reset for use by data comm programs even if

you were in fax receive mode.

1.23 - Current call sending has been enhanced.  If a cover sheet is

enabled, you will be prompted for the name and company so that this

information can be included on the cover sheet.  The application will

then construct the fax document and provide a pop-up dialog box when

it is ready for you to dial the call.  This allows the application to

immediately begin sending upon your request.

1.23 - For LAN workstation use, the workstation slave adapter is now

called the workstation adapter and is significantly modified as a

result of improvements for the enhanced LAN version of the product.

1.23 - Enhanced printer support for DOS and Windows printing is

included.  The printer object's job properties can now be either

normal or fine resolution when printing from DOS or Windows (rather

than requiring fine resolution), so you can create either normal or

fine resolution fax documents from DOS and Windows applications.  For

DOS printing, the IBM Proprinter X24 emulation is improved to support

underlining and fine-tune the character spacing.

(WordPerfect 5.1 DOS users... When printing to the fax printer device

from WordPerfect using WordPerfect's IBM Proprinter X24 printer

driver, you can now generate very high-quality output.  Try printing

WordPerfect's printer test file for an example.  If you need even

more font support for printing fax documents from WordPerfect DOS,

you might want to consider the PrimeType product as discussed in the

User's Manual.)

1.26 - Phone book displays now show the fax number on the main

selection list and on the Utilities/Edit phone books list.

1.26 - For DOS printing, the Proprinter X24 emulation now shows

emphasized and double-strike characters in Bold.  Note that the

Proprinter X24 printer does not provide italic.

1.26 - Enhanced Class-1 support, including support for new Practical

Peripherals 14400FXSA v1.21 ROMs, Megahertz and new USR Worldports.

Improvements for USR HST receiving, ZyXEL sending, and more...

1.26 - The "Recv EOP delay" check box (for Class-1 only) is added to

the Fax modem Options/Configure screen.  Use this option for

USRobotics Courier and Sportster Class-1 modems to correct a USR

quirk when receiving from certain fax machines and modems.

1.26 - The "Check TCF signal" check box (for Class-1 only) is added

to the Fax modem Options/Configure screen.  Use this option for

enabling interpretation of TCF data for enhanced negotiation of

automatic speed downgrading.  This helps overcome problems with

receiving noisy faxes if you telephone company uses a line

concentrator (which may require speed downgrading to 7200bps).

1.27 - Revised INSTALL program.

1.27 - Support for 16550 UARTs.  The 'Use HW FIFO' setting on the fax

modem Options/Configure screen allows the FMD.SYS driver to take

advantage of the high-performance buffering features of UART (serial

I/O chips, used for your COM port) that are compatible with the

NS16550A chips.

1.28 - For LAN version, added support for DTMF/Touch-Tone automatic

routing (in addition to DID automatic routing) with Brooktrout

boards.

1.28 - Fixed current call sending for SatisFAXtion boards.

1.28 - The printer driver now supports the FAX.GGS file for providing

printer driver commands to the next print job.  Applications can

place the >> printer driver commands in this file rather than

directly in the printer data stream.  This can be used to provide fax

sending and/or fax merge features from other applications that cannot

otherwise insert the printer driver commands directly into the

printer data stream.

1.28 - The printer driver now supports a configuration item for

selecting the number of gray levels.  For applications that send a

color image to the printer, the printer driver can convert the colors

to "gray" patterns.  Using gray areas will increase fax transmission

time, but can provide very nice looking graphics.  Experiment with

various settings on the printer driver configuration screen to see

what works best for your images.

1.28 - A program (FxRcv.EXE) is now provided on the distribution

diskette as a alternative way of turning receive mode on and off.

See the section above called "CONTROLLING RECEIVE MODE FROM COMMAND

LINE/COMMAND FILE".  This can be used in command files for

temporarily taking the fax program out of receive mode so that you

can make a data communications call with a fax modem.

1.29 - (LAN version) - Private mode, which provides LAN support for

user privacy and routing, now supports up to 5000 workstations per

server.  This was unintentionally limited to only 200 workstations in

previous beta test releases.

1.29 - Paste from Clipboard now provides color metafile support.

1.29 - "Color filled area" printing is further enhanced in the

printer driver.  You can now configure the printer driver for either

"Black & white", "Fax 8-level" or "System dithered" processing.

"Black & white" forces all colors to black.  "Fax 8-level" maps

colors to patterns that are custom-designed to allow efficient fax

transmission and support resizing and printing without aliasing.

"System dithered" uses standard system color dithering which can

produce nice looking images, but the images are less efficient for

fax transmission and the gray areas may alias (distort) when resized

or printed.  Experiment with various settings on the printer driver

configuration screen to see what works best for your images.

1.29 - For some strange reason, the OS/2 Enhanced Editor opens and

closes the printer driver when you select the printer.  This formerly

caused an "empty" print job.  The printer driver now ignores print

jobs that don't actually print anything.

1.30 - Proprinter emulation in the printer driver is improved

to avoid "jaggy" vertical lines and characters when printing

from Windows applications through the Proprinter WIN-OS/2 printer

driver.  This was caused by the WIN-OS/2 printer driver using

"mixed resolution" positioning commands, such as moving the print

head with 120dpi commands while printing in 180dpi graphics mode.

1.30b - For fax modems, the FMD.SYS device driver now checks the port

interrupt.  This allows the program to display only the useable ports

in the Options/Ports list.  In special cases where this might be a

problem, the port checking can be turned off by adding the 'N'

parameter to the CONFIG.SYS line which installs FMD.SYS.

1.30b - Fixed Fax/Export/DCX to always write fine mode (square

aspect ratio), and to write smaller DCX headers.

1.30b - Added Intel SatisFAXtion DTMF support (for SatisFAXtion/200

and SatisFAXtion/400), which can be used in the LAN version (private

mode) for automatic routing of incoming fax documents.  (We already

support both DTMF and DID automatic routing with Brooktrout TR112

fax boards).

1.30b - Event logging to the VOUT file when the -v parameter is used.

1.30b - Added a "Skip TCF read" check box on the Options/Configure

screen for the L40SX modem.

1.30b - SVGA displays - Due to problems in the SVGA drivers, the

algorithm for the Utilities/Capture to clipboard commands is changed.

Previously, snapshots on SVGA displays may have appeared "dark".

1.30b - 8514A displays - Problems in the OS/2 2.1 32-bit 8514A driver

impact the Utilities/Capture to clipboard command.  To work around

this problem, you can add the line "SET FXSNAP=BW" to your config.sys

file.  This environment variable setting will force the fax program

to do black-and-white screen/window capture rather than color

mapping.

1.31 - The Options menu now has a separate "Ports" command to

configure ports for sending/receiving.  The port selection was

previously part of the Options/Configure screen.

1.31 - First beta release of multi-line versions.  If you have

a multi-line version of the program, the Options/Ports screen

allows you to select multiple ports for simultaneous use, and

to specify each port as send, receive or send/receive.

1.31 - For Brooktrout fax hardware, implemented support for the

"Kill" feature on the status window.  (This already worked for

all other fax hardware... select a line in the status window and

press the "Kill" button to abort the activity.)

1.32 - Initial test support for the GammaLink fax boards.

1.34 - Enhanced the phone book handling in the printer driver pop-up

and the fax program.  Any individual phone book can now have up to

2000 entries (which is a size limit imposed by OS/2 list box issues),

and of course, you can have multiple phone books.

1.34 - An "All" button is added to the phone book.  This makes it

easier to use a phone book as a distribution list since you can

easily select all members of the current phone book without using a

Group name.

1.35 - Installation (and updating) is now simplified.  The INSTALL

program copies the files, helps you modify your CONFIG.SYS file (if

you want it to do so), and instructs you to reboot your system when

necessary.  But you no longer need to run the INSTALL program a

second time to select your ports and set your modem configuration.

The fax program now "knows" when you've run INSTALL and automatically

gives you the option of setting ports/configuration the next time

that you run it.

1.35 - A separate "Fax data directory" is now supported by the

INSTALL program.  If desired, you can specify separate directories as

the "Install to" directory for the program files, "Fax data

directory" for the program's data files (FAX.LOG, FAX.IDX, *.FAX fax

document files, *.PBK phone book files, and *.BMP bitmap files), and

"Working directory" as the starting working directory for your

import/export/paste file operations.  The separate data directory

makes it easier for you to backup your work or keep your fax

documents on a different drive.  (If you change your fax data

directory after initial installation and want to retain your past fax

activity, you must manually move your FAX.LOG, FAX.IDX, *.FAX, *.PBK

and *.BMP files to the new fax data directory.)

1.35 - The INSTALL program now remembers the locations of your fax

program directory, fax data directory and working directory.  This

makes it easier to update your software.

1.36 - The fax program now supports a new "-L newdir" command line

parameter for specifying the fax data directory.  The "newdir"

parameter is the pathname of the drive and directory that contains

the FAX.LOG file and other fax data files (FAX.IDX, *.FAX, *.PBK).

If you move your fax directory, you can use this parameter to tell

the program where to find the fax data files (the program remembers

the location, so the parameter is only needed the first time that you

run the program after moving your fax data directory).  If you have

multiple fax data directories you can use this parameter in the

"optional parameter" field of your program object to force the

program to use the correct fax data directory for various situations.

1.37 - The fax printer driver includes modifications to eliminate

clipping the edge of the output from certain OS/2 applications that

print all the way to the very edge of the page.

1.37 - Disk space monitoring is added.  To ensure proper logging of

activity, sending/receiving is disabled if there is less than 10

kbytes of available disk space.  To prevent answering the phone and

attempting to receive when the disk is full, sending/receiving is

also disabled if a receive operation fails due to a disk-related

error (syserror).  A pop-up dialog box will notify you when this

occurs.  Sending/receiving is enabled again when you use Fax/Receive

to turn receive mode back on or when you visit the Fax/Send dialog

box.

1.37 - Fixed a problem with fax modems that do not support the S7=100

S register setting (such as for USRobotics modems in the UK).  These

modems will use their default setting for the S7 register value

rather than produce an error.

1.37b - Upon starting the fax program, if the fax log contains faxes

that have been "received" (or, for the LAN version, "routed") but not

yet read, the status window will be displayed and will indicate the

number of received faxes.

1.37b - The OS/2 LAN workstation program now includes a status window

(like the stand-alone version).  When you start the program, the

status window will show a count of received fax documents (if any).

The status window also shows background printing activity and allows

you to kill print jobs by selecting the print line and pressing the

Kill button.  If you are using the LAN Private mode with the

workstation's notification method set to "OS/2 mailslot", the status

window's receive count will be dynamically updated as faxes are

received.

1.37e - The "Busy retries" setting on the Options/Settings screen can

now be a maximum of 99 (previously the maximum was 9).