Chapter 1

About the MCI VISCA Driver

MCI VCR Services

Windows provides videocassette recorder (VCR) services through a device driver that is based on the MCI command-set for VCRs. This chapter describes the functions, structures, and messages of the MCI Video System Control Architecture (VISCA) driver, and explains how to use them to control a VCR.

The MCI VISCA driver controls Sony VISCA-compatible videotape recorders, such as the CVD-1000 VDeck. The VISCA driver controls the parts of a VCR: the tape transport, channel tuners, and VCR input and output channels. To play the content of a videotape on your PC, you must connect the video and audio outputs from your VCR to the video capture or overlay card of your PC, and to the audio inputs of your PC. You might also need an MCI digital-video device driver installed on your PC.

The rest of this section describes the following features of the VISCA driver:

·	Controlling the tape transport

·	Setting other parts of the VCR

·	Retrieving VCR status information

·	Searching and positioning with a VCR

·	Capturing frames

·	Selecting inputs

·	Selecting recording tracks

·	Recording and editing

·	Using the clock while editing

## Controlling the Tape Transport

The tape transport moves videotape within a VCR for playing, positioning, and recording. Most of the transport commands in the VISCA driver can operate the tape transport either in the forward or reverse direction.

Transport commands affect tape movement in a VCR by controlling the mode (state) of the VCR tape transport. Transport commands are unique in that only one command can be running at any given time on a single VCR. This category of commands includes playing, recording, seeking, pausing, stopping, and stepping.

If a VCR receives a new transport command while another transport command is executing, the VCR accepts the new command and either stops or supersedes the transport command in progress. (A command is stopped when it cannot be carried to completion as originally specified. A command is superseded if a new command of the same type is issued.)

The following examples show transitions from several transport commands. The examples assume one transport command is in progress when the VCR receives a new transport command. The examples also use the notify flag to return information about the transition to the new command.

| Command in Progress | New Command | Return Notification |
| --- | --- | --- |
| play myvcr from 8 to 20 notify | play myvcr to 20 notify | Superseded |
| play myvcr to 10 reverse notify | play myvcr to 20 notify | Stopped (transport direction change) |
| play myvcr to 10 notify | play myvcr to 20 notify | Stopped (position change) |
| play myvcr from 20 to 10 notify | play myvcr to 10 reverse notify | Superseded |
| stop myvcr notify | play myvcr notify | None |

## Setting Other Parts of a VCR

In addition to the tape transport, the VISCA driver controls other portions of a VCR, including video channels, audio channels, and tuner channels used for playing and recording. The set commands control the state of these items and can be issued at any time.

The set commands are synchronous, and notifications from these commands don’t interfere with outstanding (that is, not yet received) transport-command notifications, providing that the notifications are not performed from the same MCI VISCA-driver instance (each driver instance can have only one pending notification). For example, you can issue the settuner channel number notify command while the VCR is performing a seek command without stopping or superseding the seek command.

The set commands perform the following functions.

| Command | Description |
| --- | --- |
| set | Sets the state of the following items: assemble recording, clock, counter, counter format, door, index, pause timeout, postroll duration, power, preroll duration, recording format, speed, tape length, time mode, and tracking. |
| setaudio | Sets the audio channel for playing, recording, and monitoring. |
| settimecode | Sets the timecode state for recording or playing, or turns off timecode monitoring. |
| settuner | Sets the VCR tuner(s) to specific channels. |
| setvideo | Sets the video channel for playing, recording, and monitoring. |

## Retrieving VCR Status Information

A third group of commands, status commands, retrieve information about the VCR. Like the set commands, the status commands are synchronous, and notifications from the status commands don’t interfere with outstanding (that is, not yet received) transport-command notifications, providing that the notifications are not performed from the same instance (each driver instance can have only one pending notification). For example, you can issue the status position notify command while the VCR is seeking without stopping or superseding the seek command.

The status commands perform the following functions.

| Command | Description |
| --- | --- |
| status | Can report the state of any of the following items: assemble record, audio monitor, audio-monitor number, audio record, audio source, audio-source number, channel, clock, clock ID, clock increment rate, counter, counter format, counter resolution, current track, frame rate, index, index on, length, media present, media type, mode, number of tracks, number of audio tracks, number of video tracks, play format, position, power, preroll duration, postroll duration, pause timeout, ready-to-receive-a-play command, record format, speed, time format, time mode, time type, timecode present, timecode record, timecode type, video monitor, video-monitor number, video record, video source, video-source number, write protected. |
| list | Reports the number and type of all video and audio input channels. |
| capability | Reports capabilities found in a specific VCR, including the following: detect length, eject, freeze, monitor sources, play, preroll, preview, record, reverse, save, test, device type, has audio, has clock, has timecode, video, maximum number of marks, and seek accuracy. |

## Searching and Positioning with a VCR

The VISCA driver supports two methods of tracking videotape movement within the VCR tape transport: using timecode information and using tape counters.

Timecode information is timing information that has been recorded on the videotape in addition to the audio and video information. Timecode information is recorded as a non-negative increasing value.) Most VCRs allow timecode to be recorded on a tape without destroying audio and video tracks.

The alternative to using timecode information is using tape counters. Tape counters estimate the amount of videotape (footage) that travels past the tape head to obtain a position.

Both timecode information and tape counters increase as the tape moves from beginning to end. Because of its accuracy, using timecode information to position a videotape is almost always preferable to using tape counters.

The MCI command parameters for specifying positioning information are expressed as time dependencies: time format, duration, from, to, and seek. (Also, the status position command returns its time value in the current-time format.)

The VISCA driver uses the set time mode command to select the type of positioning to use with a videotape. When the time mode is set to timecode, the status position and set time format commands use the timecode on the tape. When the time mode is set to counter, the status position and set time format commands use counters.

If an application doesn’t care that there might be two sources of position information (such as the Microsoft Media Player), it can set the time mode to detect. When in detect mode, the VISCA driver will use timecode information for positioning when any of the following conditions occur:

·	If the timecode is present when the driver is opened.

·	If you change a tape with the set door open command and have timecode present.

·	If the set time mode command is re-issued.

If timecode cannot be found, the driver uses the tape counters.

To determine the current positioning method, issue the status time type command, which returns either timecode or counter. You can also identify the current positioning mode by using the status time mode command, which returns “timecode,” “counter,” or “detect.”

The status counter command retrieves the current tape counter value, regardless of the current positioning method. However, you can only use this counter reading with the set counter command.

The VISCA driver can retrieve the native timecode format recorded on a videotape by using the status timecode type, and status frame rate commands together. For example, if timecode type is smpte and frame rate is 25, the native timecode format recorded on the tape is smpte 25.

The VISCA driver can also retrieve the counter resolution by using status counter resolution command, which will return “seconds” or “frames.” The counter format might still be set to smpte 30, but the return value will only return a frame of 0. If the current time type is counter, then this resolution also applies to the value returned by status position.

## Capturing Frames

Frame-capturing commands provide still images for a frame-capture device. The VISCA driver supports the freeze command to stabilize a still image for capturing. Also, the unfreeze command can be used to restart the tape transport following a freeze command.

The freeze command provides a high-quality (non-jittery, stabilized, time-base corrected) image for a frame-capture device. (A frame-capture device is a separate piece of hardware capable of “reading” and storing the video image.) The basic concept behind the freeze command is that a device might not always deliver its maximum-quality output image and that such a video image is not suitable for capturing.

The unfreeze command unlocks the tape transport and resumes the transport mode in effect prior to the freeze command.

When your application needs to record a video image on the VCR, as is common with software-animation applications, use the freeze input command or the cue command to record the image. For more information about the cue command, see “Recording and Editing,” later in this document.

## Selecting Inputs

The following illustration identifies the VCR inputs and outputs supported by the VISCA driver. Arrows on the left side indicate external inputs that connect to the VCR. Inputs that are surrounded by a box (tuner and timecode) are internal inputs that need no external connections. Arrows on the right side indicate external outputs that can connect to playback and capture equipment.

µ §

The VISCA driver supports three input types: video, audio, and timecode. The video inputs include two standard channels (line 1 and line 2), an SVideo channel, an auxiliary channel, and a channel from an internal tuner. The audio inputs include two standard channels (line 1 and line 2) and a channel from an internal tuner. The timecode input is internal to the VCR.

The normal outputs carry the currently selected inputs when the VCR is recording or when the transport is stopped, and carry the contents of the videotape when the tape transport is playing or paused. The monitored outputs carry the same information as the normal outputs plus current timecode and channel information.

The back of your VCR might have several output plugs, but usually the same signal will appear on all of them. In this way, they can be viewed as a single logical output.

Note

Assuming the appropriate external inputs are connected to your VCR and you have decided what you want to record, you can select the inputs to be recorded. For example, to record or view from the “svideo” video and the “line 1” audio inputs, you would use the setvideo and setaudio commands to select these input sources. You can verify these selections by using the status command.

By default, the monitor shows exactly what appears as the output. However, sometimes you might want to “view” one source, while recording from another. This is a common practice using the tuner. For example, you might want to watch channel 4 while you record channel 7. In this case, you have two “logical” tuner inputs. You could set up the VCR by using the following commands:

1.	Use the settuner command to select the channels to watch and record.

2.	Use the setvideo command select the video-recording source.

3.	Use the setaudio command to select the audio-recording source.

4.	Use the setvideo command to route the channel-4 video input to the monitored output to display it on screen.

5.	Use the setaudio command to route the channel-4 audio input to the monitored output to play the audio.

6.	Verify your selections by using the status command.

The VISCA driver also supports a special input type for audio and video called “mute.” Mute allows the selection of “no input,” which is useful when recording a blank signal.

Note

## Selecting Recording Tracks

The recording tracks of a videotape are of three types: video, audio, and timecode. The general use of the tracks on a videotape include the following:

·	One video track

·	One or more audio tracks

·	One timecode track

When recording audio onto multiple tracks, use the following conventions:

·	Track 1 is always the main audio track

·	Track 2 is the second track

The VISCA driver supports two operating modes: assemble and insert. In assemble mode, all tracks are selected to be recorded. In insert mode, tracks can be independently selected for recording. The default mode is “assemble record on.”

## Recording and Editing

The record command provides simple recording and is accurate to approximately one second of the starting position. To record more accurately or if you expect to editing the video content while simultaneously operating multiple decks, you should use the cue command.

The cue command prepares the VCR for recording or playing. In MCI terms, recording is expressed as “input,” and playing is expressed as “output.” Therefore, “cue input” means prepare for recording. The cue command is separate from play and record because an application must know when the VCR is ready to perform the command (and because it can take several minutes to prepare for a play or record command).

The VCR prepares itself for recording or playing by seeking to the “in-point,” which is specified by the from parameter in the cue command. However, if the preroll flag is given (use capability with can preroll to check for support), the VCR will position itself the preroll distance from the “in-point.” The preroll flag also indicates that the VCR uses any applicable “editing mode,” so it’s important that you use preroll, especially when you want the most accurate recording.

When recording with “from” and “to” positions, the “from” position is included in the edit, and the “to” position is not.

Note

## Using the Clock While Editing

When editing, you might want to record specific segments from one VCR to another. To begin recording at a specific time and position on one VCR when another VCR begins playing at the same time and position, you must specify the following items for each VCR:

·	An action (play or record)

·	A position

·	A time

Both VCRs must use the same clock for this type of editing; the clock helps synchronize both devices. As a shared resource, the clock can be connected to multiple VCRs. The VISCA driver supports only one shared clock.

You can determine if two VCRs share the same clock by using the status command with the clock id parameter to query each VCR. If the IDs (identification numbers) returned from the status command are the same, they use the same clock.

You can also determine clock resolution by using the status command with the clock increment rate parameter. This command returns the number of increments the clock supports per second. For example, if the clock is updated every millisecond, 1000 is returned as the clock increment rate. The advantage of using the increment rate is that the rate is expressed as an integer; otherwise, the increment would be a (single- or double-precision) floating-point value. As an integer, manipulating the increment rate is a simple operation void of round-off error. You can reset the clock by using the set command with the clock 0 (zero) parameter.

When issuing a play, record, or seek command, you can specify when the command is to be executed. Exactly when to start each VCR involved in the edit depends on the characteristics of the VCRs being used. The timing must account for the amount of preroll each device requires and the amount of time needed to complete the MCI commands used to set up the edit session. The basic technique consists of the following steps:

1.	Retrieve the clock time.

2.	Add a waiting interval (5 to 10 seconds) to the clock time.

The waiting interval must be long enough to let the preroll and any outstanding MCI commands complete.

You might place the record command as the last command in the script, then check the time immediately before the record command to be sure the waiting interval is large enough. (And restart the play command if the interval is too small.) Alternatively, you could check the time immediately after the last command of the script to verify that there is enough time to send and complete all of the commands.

# Using the MCI VISCA Driver

This section explains how to perform the following tasks:

·	Capturing a still-frame image from videotape

·	Selecting and verifying VCR inputs

·	Selecting recording tracks when a VCR is in insert mode

·	Using a clock with tape-to-tape editing

## Capturing Frames

The following example shows how an application can use the freeze and unfreeze commands:

open vcr alias myvcr 
set myvcr time format smpte 30
capability myvcr can freeze	; return true
seek myvcr to 00:10:10:10 wait	; capture frame at: 10 minutes, 
	; 10 seconds, 10 frames
freeze myvcr wait	; wait might take ~8 seconds 
	; on some VCRs
.
.	; frame is ready to capture
.
unfreeze myvcr wait

## Selecting and Verifying Inputs

This section includes two examples of selecting and verifying VCR inputs. The first example shows a basic setup using default values for the commands. The second example is more complex in that it shows one command sequence that you could use to watch and listen to one channel while recording audio and video from another channel.

The following example sets the “svideo” video input and the “line 1” audio input for recording, and verifies the settings:

; Set video and audio inputs using defaults
setvideo myvcr source to svideo		; Number 1 is by default
setaudio myvcr source to line		; Number 1 is by default

; Verify current input settings
status myvcr video source		; Returns "line"
status myvcr video source number 		; Returns 1
status myvcr audio source		; Returns "line"
status myvcr audio source number 		; Returns 1

By default, the monitor shows exactly what appears as the output. However, sometimes you might want to “view” one source while recording from another. This is a common practice using the tuner. For example, you might want to watch channel 4 while you record channel 7. In this case, you have two “logical” tuner inputs. (The previous illustration included one logical tuner.) You could use the following commands to accomplish this task:

settuner myvcr channel 4 number 1	; Set channel on tuner 1
settuner myvcr channel 7 number 2	; Set channel on tuner 2

	; Recording inputs
setvideo myvcr source to tuner number 2	; Video input 
setaudio myvcr source to tuner number 2	; Audio input 

	; Map output channels
setvideo myvcr monitor to tuner number 1	; Video output to tuner 1
setaudio myvcr monitor to tuner number 1	; Audio output to tuner 1

	; Verify the selections
status myvcr channel tuner number 1	; Returns 4
status myvcr channel tuner number 2	; Returns 7
status myvcr video source	; Returns "tuner"
status myvcr video source number	; Return 2
status myvcr video monitor 	; Returns "tuner"
status myvcr video monitor number	; Return 1

## Selecting Recording Tracks

The following are the track settings to select the recording of the video and PCM tracks:

setaudio myvcr record off track 1
status myvcr audio record
status myvcr audio record track 2	; For Hi8 VCRs, this 
	; corresponds to the PCM track

## Recording and Editing

The following example uses the cue and the freeze commands to perform accurate frame recording (which is useful when creating animation output):

set myvcr time format frames
cue myvcr input from 1000 to 1001 preroll wait
.
.	; your output
.	; device is prepared
.
freeze myvcr input wait
record myvcr

When recording with “from” and “to” positions, the “from” position is included in the edit and the “to” position is not. In this example, one frame is recorded.

Note

## Using the Clock while Editing

The following example shows how a time-driven edit with two VCRs could be performed:

open vcr alias myplay	;Source VCR to play content
open vcr1 alias myrec	;Recording VCR

;Set timecode format for source and recording VCRs
set myplay time format smpte 30
set myrec time format smpte 30

;Cue the videotapes in the VCRs to appropriate positions
cue myplay output from 00:10:00:00 to 00:11:05:00 preroll wait
cue myrec input from 00:45:02:03 to 00:46:02:03 preroll wait

;Start both decks
status myplay clock
play myplay at 100000
record myrec at 100000 wait

# Reference

## Command-Message Reference

This reference section describes the messages and associated structures for the VISCA video cassette recorder (VCR) device driver. You can use these messages and associated structures with the mciSendCommand function that supports the MCI command-message interface. For information about using command messages, see the Multimedia Programmer’s Reference in the Microsoft Windows 32 Software Development Kit.

This command set is taken from the VCR command specification for MCI. Any device-specific behavior affecting the MCI commands is also noted in this document.

### About MCI Command Messages

All MCI commands use two parameters to pass information to the MCI VCR driver. The first parameter, lParam1, contains flags associated with each command that is sent. The second parameter is a long pointer to a data structure containing information needed to carry out a command, or information returned by the command.

All commands accept the optional flags MCI_NOTIFY, MCI_WAIT, and MCI_TEST flags as described in the VCR MCI command set.

The VCR driver uses the VCR keyword to identify the device type. The constant MCI_DEVTYPE_VCR is defined for this device type.

### MCI_CUE

This message prepares a device instance for playback or recording with minimum delay.

When cueing for playback (using CUE with MCI_VCR_CUE_OUTPUT), issuing the MCI_PLAY command with the MCI_FROM, MCI_TO, or MCI_VCR_PLAY_REVERSE flag cancels the MCI_CUE command.

Notes

When cueing for recording (using CUE with MCI_VCR_CUE_INPUT), issuing the MCI_RECORD command with the MCI_FROM, MCI_TO, or MCI_VCR_RECORD_INITIALIZE flag cancels the MCI_CUE command.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_CUE_PREROLL

Cue the device to current position, or the dwFrom position, minus the preroll duration. This will allow the device to get “up to speed” before entering record or playback mode.

MCI_VCR_CUE_REVERSE

The direction of the upcoming play or record command is reverse.

MCI_VCR_CUE_INPUT

Prepare for recording (=input).

MCI_VCR_CUE_OUTPUT

Prepare for playing (=output). If neither MCI_VCR_CUE_INPUT nor MCI_VCR_CUE_OUTPUT is specified, MCI_VCR_CUE_OUTPUT is assumed.

MCI_FROM

dwFrom contains the starting position specified in the current-time format.

MCI_TO

dwTo contains the ending (pausing) position specified in the current-time format.

LPMCI_VCR_CUE_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_CUE_PARMS data structure.

### MCI_FREEZE

This message freezes the input or output image of the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_FREEZE_INPUT

Freeze the input image (used for recording).

MCI_VCR_FREEZE_OUTPUT

Freeze the output image (used with frame capture).

MCI_VCR_FREEZE_FIELD

Freeze only one field of the current frame.

MCI_VCR_FREEZE_FRAME

Freeze both fields of the current frame (=frame).

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_GENERIC_PARMS

The MCI_GENERIC_PARMS structure contains parameters used by the following messages for digital-video cassette recorders: MCI_FREEZE, MCI_INDEX, MCI_MARK, MCI_PAUSE, MCI_RESUME, MCI_STOP, and MCI_UNFREEZE. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_GENERIC_PARMS {
	DWORD	dwCallback;

} MCI_GENERIC_PARMS;

The MCI_GENERIC_PARMS structure has the following field:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

### MCI_GETDEVCAPS

This message obtains static information about a device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_GETDEVCAPS_ITEM

Specifies that the dwItem field of the data structure contains a constant specifying which device capability to obtain. The following constants are recognized:

MCI_GETDEVCAPS_CAN_RECORD

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_CLOCK_INCREMENT_RATE

Sets dwReturn to the number of increments per second.

MCI_GETDEVCAPS_HAS_AUDIO

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_HAS_VIDEO

Sets dwReturn to TRUE.

MCI_VCR_GETDEVCAPS_HAS_TIMECODE

Sets dwReturn to TRUE is device has timecode capability or if this capability is unknown.

MCI_GETDEVCAPS_USES_FILES

Sets dwReturn to FALSE.

MCI_GETDEVCAPS_COMPOUND_DEVICE

Sets dwReturn to FALSE.

MCI_GETDEVCAPS_DEVICE_TYPE

Sets dwReturn to MCI_DEVTYPE_VCR.

MCI_GETDEVCAPS_CAN_EJECT

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_CAN_PLAY

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_CAN_SAVE

Sets dwReturn to FALSE.

MCI_VCR_GETDEVCAPS_CAN_PREROLL

Sets dwReturn to TRUE

MCI_VCR_GETDEVCAPS_CAN_PREVIEW

Sets dwReturn to FALSE.

MCI_VCR_GETDEVCAPS_CAN_MONITOR_SOURCES

Sets dwReturn to FALSE.

MCI_VCR_GETDEVCAPS_CAN_DETECT_LENGTH

Sets dwReturn to TRUE.

MCI_VCR_GETDEVCAPS_CAN_FREEZE

Sets dwReturn to if the device is capable of freezing the output image.

MCI_VCR_GETDEVCAPS_SEEK_ACCURACY

Sets dwReturn to the seek accuracy of the device.

MCI_VCR_GETDEVCAPS_HAS_CLOCK

Sets dwReturn to TRUE if the device supports an external clock.

MCI_VCR_GETDEVCAPS_CAN_REVERSE

Sets dwReturn to TRUE.

MCI_VCR_GETDEVCAPS_NUMBER_OF_MARKS

Sets dwReturn to 99.

MCI_VCR_GETDEVCAPS_CAN_TEST

Sets dwReturn to TRUE.

LPMCI_GETDEVCAPS_PARMS   lParam2

Specifies a far pointer to the MCI_GETDEVCAPS_PARMS data structure.

### MCI_GETDEVCAPS_PARMS

The MCI_GETDEVCAPS_PARMS structure contains parameters used by the MCI_GETDEVCAPS message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_GETDEVCAPS_PARMS {
	DWORD	dwCallback;
	DWORD	dwReturn;
	DWORD	dwItem;

} MCI_GETDEVCAPS_PARMS;

The MCI_GETDEVCAPS_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwReturn

Contains the return information on exit.

dwItem

Identifies the capability being queried.

### MCI_INDEX

This message turns the on-screen display on or off. The information presented in the on-screen display is controlled by the set index command.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_SET_ON

Turns on-screen display on.

MCI_SET_OFF

Turns on-screen display off.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_INFO

This message obtains static string information from the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_INFO_PRODUCT

Sets lpstrReturn to point to the product name, and also sets dwRetSize equal to the length of the string pointed to.

MCI_VCR_INFO_VERSION

Sets lpstrReturn to point to the version number, and also sets dwRetSize equal to the length of the string pointed to.

LPMCI_INFO_PARMS   lParam2

Specifies a far pointer to the MCI_INFO_PARMS data structure.

### MCI_INFO_PARMS

The MCI_INFO_PARMS structure contains parameters used by the MCI_INFO message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_INFO_PARMS {
	DWORD	dwCallback;
	LPSTR	lpstrReturn;
	DWORD	dwRetSize;
} MCI_INFO_PARMS;

The MCI_INFO_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

lpstrReturn

Specifies a long pointer to a user-supplied buffer for the return string.

dwRetSize

Specifies the size in bytes of the buffer for the return string.

### MCI_LIST

This message obtains information about the number and types of inputs available to the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_LIST_VIDEO_SOURCE

Inquiry is for video inputs or types.

MCI_VCR_LIST_AUDIO_SOURCE

Inquiry is for audio inputs or types.

MCI_VCR_LIST_COUNT

Set dwReturn to the total number of video or audio inputs.

MCI_VCR_LIST_NUMBER

Set dwReturn to the type of the video or audio input specified by dwNumber.

LPMCI_VCR_LIST_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_LIST_PARMS data structure.

### MCI_MARK

This message records or erases marks that can be used with MCI_SEEK for high-speed searches.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_MARK_WRITE

Write a mark at the current location.

MCI_VCR_MARK_ERASE

Erase a mark at the current location if one exists.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_PAUSE

This message pauses the current action of the device.

DWORD   lParam1

Parameters

Specifies the MCI_NOTIFY, MCI_WAIT, and MCT_TEST flags.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_PLAY

This message plays the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_FROM

dwFrom contains the starting play location specified in the current-time format.

MCI_TO

dwTo contains the ending (pausing) play location specified in the current-time format.

MCI_VCR_PLAY_REVERSE

The play direction is reverse.

MCI_VCR_PLAY_SCAN

Play as fast as possible while maintaining video output.

MCI_VCR_PLAY_AT

dwAt contains a time when the entire command begins, or if the device is cued, when the device reaches the from position given by the cue command.

LPMCI_VCR_PLAY_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_PLAY__PARMS data structure.

### MCI_RECORD

This message causes the device to start recording.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_FROM

dwFrom contains the starting play location specified in the current-time format.

MCI_TO

dwTo contains the ending (pausing) play location specified in the current-time format.

MCI_RECORD_INSERT

Returns error.

MCI_RECORD_OVERWRITE

This is assumed by default, and therefore is not needed.

MCI_VCR_RECORD_INITIALIZE

Seek the device to the start of the media, begin recording blank video and audio, and record timecode, if possible.

MCI_VCR_RECORD_AT

dwAt contains a time when the entire command begins, or if the device is cued, when the device reaches the from position given by the cue command.

LPMCI_VCR_RECORD_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_RECORD_PARMS data structure.

### MCI_RESUME

This message resumes a play, record, or seek command that was interrupted by a pause.

DWORD   lParam1

Parameters

Specifies the MCI_NOTIFY, MCI_WAIT, and MCT_TEST flags.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_SEEK

This message positions and cues the workspace to the specified position.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_SEEK_TO_START

Seek to the start of the media (=tape).

MCI_SEEK_TO_END

Seek to the end of the media.

MCI_TO

dwTo contains the position, in the current-time format, to seek to.

MCI_VCR_SEEK_REVERSE

Seek direction is reverse; this is only used with the mark flag.

MCI_VCR_SEEK_MARK

dwMark contains the numbered mark to search for.

MCI_VCR_SEEK_AT

dwAt contains a time when the entire command begins.

LPMCI_VCR_SEEK_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SEEK_PARMS data structure.

### MCI_SET

This message sets device information or state.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_SET_TIME_FORMAT

dwTimeFormat contains a constant specifying the new time format to be used with play, record, seek, cue, and status position commands. The following are valid constants:

MCI_FORMAT_MILLISECONDS

Time format is milliseconds.

MCI_FORMAT_FRAMES

Time format is frames.

MCI_FORMAT_SMPTE_30DROP

Time format is smpte 30 drop.

MCI_FORMAT_SMPTE_30

Time format is smpte 30 non-drop.

MCI_FORMAT_SMPTE_25

Time format is smpte 25.

MCI_FORMAT_SMPTE_24

Time format is smpte 24.

MCI_FORMAT_HMS

Time format is hours, minutes, seconds.

MCI_FORMAT_TMSF

Time format is tracks, minutes, seconds, frames.

MCI_FORMAT_MSF

Time format is minutes, seconds, frames.

MCI_SET_AUDIO

Unsupported.

MCI_VCR_SET_TIME_MODE

dwTimeMode contains a constant indicating the new positional time mode. The following constants are valid:

MCI_VCR_TIME_DETECT

Each time a new tape (=media) is inserted into the device, or the mode changes from not ready to ready, attempt to determine if there is timecode available, on the tape. If timecode is available then use timecode in all subsequent commands that specify positions. Otherwise, use the counter.

MCI_VCR_TIME_TIMECODE

Forces the device to use timecode exclusively.

MCI_VCR_TIME_COUNTER

Forces the device to use counter exclusively.

MCI_SET_DOOR_OPEN

Ejects the tape.

MCI_VCR_SET_ASSEMBLE_RECORD

Sets the device to record in assemble or insert modes (when assemble is off, insert is on, and viceversa).

MCI_SET_ON

Sets to assemble record, insert is turned off. Records all video, audio and timecode tracks.

MCI_SET_OFF

Sets assemble record off, and turn insert record on. When assemble record is off, individual tracks of video, audio, and timecode can be selected for recording.

MCI_VCR_SET_RECORD_FORMAT

dwRecordFormat contains a constant describing the record speed, which must be one of the following:

MCI_VCR_FORMAT_SP

Records at standard speed.

MCI_VCR_FORMAT_LP

Records at medium-slow speed.

MCI_VCR_FORMAT_EP

Records at slow speed.

MCI_VCR_SET_SPEED

dwSpeed contains the new speed setting, where 1000 is normal speed, 2000 is double, etc., and 500 is 1/2 speed, 250 is 1/4, etc.

MCI_VCR_SET_TAPE_LENGTH

dwTapeLength contains the new length of the tape, provided that the length of the tape is undetectable.

MCI_VCR_SET_CLOCK

dwClock contains the new clock time.

MCI_VCR_SET_COUNTER_FORMAT

dwCounterFormat contains a constant specifying the new counter-time format to be used by the status counter. For a list of valid constants, see MCI_VCR_SET_TIME_FORMAT.

MCI_VCR_SET_COUNTER_VALUE

dwCounterValue contains the new counter value.

MCI_VCR_SET_INDEX

dwIndex contains a constant indicating the contents of the on-screen display (for more information, see MCI_INDEX), and must be one of the following:

MCI_VCR_INDEX_TIMECODE

Displays timecode.

MCI_VCR_INDEX_COUNTER

Displays counter.

MCI_VCR_INDEX_DATE

Displays date.

MCI_VCR_INDEX_TIME

Displays time.

MCI_VCR_SET_POWER

Sets the power on or off, and must be used with one of the following flags:

MCI_SET_ON

Turns power on.

MCI_SET_OFF

Turns power off.

MCI_VCR_SET_PREROLL_DURATION

dwPrerollDuration contains the videotape length, in the current-time format, needed to stabilize the VCR output.

MCI_VCR_SET_POSTROLL_DURATION

dwPostrollDuration contains the videotape length, in the current-time format, needed to brake the VCR transport when a stop or pause command is issued.

MCI_VCR_SET_PAUSE_TIMEOUT

dwPauseTimeout contains the maximum duration, in milliseconds, of a pause command.

MCI_VCR_SET_TRACKING

Tune the speed of the VCR transport with a fine adjustment, and must be used with one of the following flags:

MCI_VCR_PLUS

Increases the transport speed.

MCI_VCR_MINUS

Decreases the transport speed.

MCI_VCR_RESET

Returns the tracking adjustment to zero.

LPMCI_VCR_SET_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SET_PARMS data structure.

### MCI_SETAUDIO

This message sets values associated with audio recording, monitoring, and input selection.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETAUDIO_RECORD

Sets the audio recording to on or off, which is used in conjunction with one of following flags:

MCI_SET_ON

Audio recording on.

MCI_SET_OFF

Audio recording off. It might be necessary to first turn off the assemble recording (using set with the MCI_VCR_SET_ASSEMBLE_RECORD flag set to off) before the audio recording can be turned off.

MCI_TRACK

dwTrack specifies which track is affected by the command.

MCI_VCR_SETAUDIO_SOURCE

Sets the audio source, and must be used with the MCI_VCR_SETAUDIO_TO flag.

MCI_VCR_SETAUDIO_MONITOR

Sets the audio source monitor, and must be used with the MCI_VCR_SETAUDIO_TO flag.

MCI_VCR_SETAUDIO_TO

dwTo contains a constant describing the type of input or monitored input, and must be one of the following:

MCI_VCR_SRC_TYPE_TUNER

Type is tuner.

MCI_VCR_SRC_TYPE_LINE

Type is line.

MCI_VCR_SRC_TYPE_AUX

Type is auxiliary.

MCI_VCR_SRC_TYPE_GENERIC

Type is generic.

MCI_VCR_SRC_TYPE_MUTE

Type is mute. This can only be used with the MCI_VCR_SETAUDIO_SOURCE flag.

MCI_VCR_SRC_TYPE_OUTPUT

MCI_VCR_SETAUDIO_NUMBER

dwNumber contains the audio input (of the type specified in the dwTo field) to use.

LPMCI_VCR_SETAUDIO_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SETAUDIO_PARMS data structure.

### MCI_SETTIMECODE

This message enables or disables timecode recording for a VCR.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETTIMECODE_RECORD

Sets the timecode track recording to on or off. This flag is used in combination with one of the following additional flags.

MCI_SET_ON

Timecode recording on.

MCI_SET_OFF

Timecode recording off.

LPMCI_VCR_SETAUDIO_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_SETTUNER

This message sets the current channel on the tuner.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETTUNER_CHANNEL_UP

Increments the tuner channel.

MCI_VCR_SETTUNER_CHANNEL_DOWN

Decrements the tuner channel.

MCI_VCR_SETTUNER_CHANNEL_SEEK_UP

Searches in the increasing channel-number direction for a valid channel.

MCI_VCR_SETTUNER_CHANNEL_SEEK_DOWN

Searches in the increasing channel-number direction for a valid channel.

MCI_VCR_SETTUNER_CHANNEL

dwChannel contains the new channel number.

MCI_VCR_SETTUNER_NUMBER

dwNumber specifies which logical tuner to affect with this command.

LPMCI_VCR_SETTUNER_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SETTUNER_PARMS data structure.

### MCI_SETVIDEO

This message sets values associated with video recording, monitoring, and input selection.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETVIDEO_RECORD

Sets the video recording to on or off, and is used in conjunction with one of following flags:

MCI_SET_ON

Video recording on.

MCI_SET_OFF

Video recording off. It might be necessary to first turn off the assemble recording (using set with the MCI_VCR_SET_ASSEMBLE_RECORD flag set to off) before the video recording can be turned off.

MCI_TRACK

dwTrack specifies which track is affected by the command.

MCI_VCR_SETVIDEO_SOURCE

Sets the video source, and must be used with the MCI_VCR_SETVIDEO_TO flag.

MCI_VCR_SETVIDEO_MONITOR

Sets the video source monitor, and must be used with the MCI_VCR_SETVIDEO_TO flag.

MCI_VCR_SETVIDEO_TO

dwTo contains a constant describing the type of input or monitored input. For a description of the applicable constants, see the MCI_SETAUDIO flag under MCI_VCR_SETAUDIO.

MCI_VCR_SRC_TYPE_TUNER

MCI_VCR_SRC_TYPE_LINE

MCI_VCR_SRC_TYPE_AUX

MCI_VCR_SRC_TYPE_GENERIC

MCI_VCR_SRC_TYPE_MUTE

MCI_VCR_SRC_TYPE_OUTPUT

MCI_VCR_SRC_TYPE_RGB

MCI_VCR_SETVIDEO_NUMBER

dwNumber contains the video input (of the type specified in the dwTo field) to use.

LPMCI_VCR_SETVIDEO_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SETVIDEO_PARMS data structure.

### MCI_STATUS

This message obtains information about an instance of an MCI device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_STATUS_ITEM

Specifies that the dwItem field of the data structure contains a constant indicating which information to obtain. The following constants are recognized:

MCI_STATUS_LENGTH

Sets dwReturn to the length of the media (=tape).

MCI_STATUS_POSITION

Sets dwReturn to the current device position in the current-time format.

MCI_STATUS_NUMBER_OF_TRACKS

Sets dwReturn to 1.

MCI_VCR_STATUS_NUMBER_OF_VIDEO_TRACKS

Sets dwReturn to the number of video tracks that are independently selectable.

MCI_VCR_STATUS_NUMBER_OF_AUDIO_TRACKS

Sets dwReturn to the number of audio tracks that are independently selectable.

MCI_STATUS_MODE

Sets dwReturn to a constant indicating the current mode of the device. The mode will be one of the following:

MCI_MODE_NOT_READY

MCI_MODE_STOP

MCI_MODE_PLAY

MCI_MODE_RECORD

MCI_MODE_SEEK

MCI_MODE_PAUSE

MCI_MODE_OPEN

MCI_STATUS_MEDIA_PRESENT

Sets dwReturn to TRUE if the media is present.

MCI_VCR_STATUS_TIMECODE_PRESENT

Sets dwReturn to TRUE if timecode is present at the current position on the media.

MCI_STATUS_TIME_FORMAT

Sets dwReturn to the current-time format. For a description of the time formats, see MCI_SET flag under MCI_SET_TIME_FORMAT.

MCI_VCR_STATUS_TIME_MODE

Sets dwReturn to one of the following (for more information, see MCI_SET, flag MCI_VCR_SET_TIME_MODE):

MCI_VCR_TIME_TIMECODE

MCI_VCR_TIME_COUNTER

MCI_VCR_TIME_DETECT

MCI_STATUS_READY

Sets dwReturn to TRUE if the device is ready to accept a play or record command.

MCI_STATUS_CURRENT_TRACK

Sets dwReturn to 1.

MCI_VCR_STATUS_FRAME_RATE

Sets dwReturn to the current native frame rate of the device.

MCI_VCR_STATUS_SPEED

Sets dwReturn to the current speed. (For more information, see MCI_SET, flag MCI_SET_SPEED).

MCI_VCR_STATUS_MEDIA_TYPE

Sets dwReturn to one of the following:

MCI_VCR_MEDIA_8MM

MCI_VCR_MEDIA_HI8

MCI_VCR_MEDIA_VHS

MCI_VCR_MEDIA_SVHS

MCI_VCR_MEDIA_BETA

MCI_VCR_MEDIA_EDBETA

MCI_VCR_MEDIA_OTHER

MCI_VCR_STATUS_RECORD_FORMAT

Sets dwReturn to one of the following:

MCI_VCR_FORMAT_SP

MCI_VCR_FORMAT_LP

MCI_VCR_FORMAT_EP

MCI_VCR_FORMAT_OTHER

MCI_VCR_STATUS_PLAY_FORMAT

Sets dwReturn to one of the following:

MCI_VCR_FORMAT_SP

MCI_VCR_FORMAT_LP

MCI_VCR_FORMAT_EP

MCI_VCR_FORMAT_OTHER

MCI_VCR_STATUS_PREROLL_DURATION

Sets dwReturn to length of tape, in the current-time format. Needed to stabilize the VCR output.

MCI_VCR_STATUS_POSTROLL_DURATION

Sets dwReturn to length of tape, in the current-time format. Needed to brake the VCR transport from a stop or pause command.

MCI_VCR_STATUS_PAUSE_TIMEOUT

Sets dwReturn to the maximum duration, in milliseconds, of a pause command. The return value of zero indicates that no time-out will occur.

MCI_VCR_STATUS_AUDIO_SOURCE_NUMBER

Sets dwReturn to the number of the currently selected audio-source type.

MCI_VCR_STATUS_AUDIO_SOURCE

Sets dwReturn to a constant, indicating the current audio-source type.

MCI_VCR_STATUS_VIDEO_SOURCE_NUMBER

Sets dwReturn to the number of the currently selected video-source type.

MCI_VCR_STATUS_VIDEO_SOURCE

Sets dwReturn to a constant indicating the currently selected video-source type.

MCI_VCR_STATUS_AUDIO_MONITOR_NUMBER

Sets dwReturn to the number of the currently selected audio-monitor type.

MCI_VCR_STATUS_AUDIO_MONITOR

Sets dwReturn to a constant, indicating the currently selected audio-monitor type.

MCI_VCR_STATUS_VIDEO_MONITOR_NUMBER

Sets dwReturn to the number of the currently selected video-monitor type.

MCI_VCR_STATUS_VIDEO_MONITOR

Sets dwReturn to a constant, indicating the currently selected video-monitor type.

MCI_VCR_STATUS_INDEX_ON

Sets dwReturn to TRUE if the on-screen display is on.

MCI_VCR_STATUS_INDEX

Sets dwReturn to a constant, describing the current contents of the on-screen display, and will be one of the following:

MCI_VCR_INDEX_TIMECODE

MCI_VCR_INDEX_COUNTER

MCI_VCR_INDEX_DATE

MCI_VCR_INDEX_TIME

MCI_VCR_STATUS_COUNTER_FORMAT

Sets dwReturn to a constant describing the current counter format. (For more information, see MCI_SET flag, MCI_SET_TIME_FORMAT).

MCI_VCR_STATUS_COUNTER_RESOLUTION

Sets dwReturn to a constant describing the resolution of the counter, and will be one of the following values:

MCI_VCR_COUNTER_RES_SECONDS

Counter has resolution of seconds.

MCI_VCR_COUNTER_RES_FRAMES

Counter has resolution of frames.

MCI_VCR_STATUS_TIMECODE_TYPE

Sets dwReturn to a constant, describing the type of timecode which is natively supported by the device, and will be one of the following:

MCI_VCR_TIMECODE_TYPE_SMPTE

This device uses smpte timecode.

MCI_VCR_TIMECODE_TYPE_SMPTE_DROP

This device uses smpte drop timecode.

MCI_VCR_TIMECODE_TYPE_OTHER

MCI_VCR_TIMECODE_TYPE_NONE

MCI_VCR_STATUS_COUNTER_VALUE

Sets dwReturn to the current counter reading, in the current counter-time format.

MCI_VCR_STATUS_TUNER_CHANNEL

Sets dwReturn to the current channel number.

MCI_VCR_STATUS_NUMBER

dwNumber contains the logical-tuner number this command applies to.

MCI_VCR_STATUS_WRITE_PROTECTED

Sets dwReturn to TRUE if the media is write protected.

MCI_VCR_STATUS_TIMECODE_RECORD

Sets dwReturn to TRUE if the timecode will be recorded when the next record command is given.

MCI_VCR_STATUS_ASSEMBLE_RECORD

Sets dwReturn to TRUE if assemble mode is on.

MCI_VCR_STATUS_VIDEO_RECORD

Sets dwReturn to TRUE if video will be recorded when the next record command is given.

MCI_TRACK

dwTrack contains the track this inquiry applies to.

MCI_VCR_STATUS_AUDIO_RECORD

Sets dwReturn to TRUE if audio will be recorded when the next record command is given.

MCI_TRACK

dwTrack contains the track this inquiry applies to.

MCI_VCR_STATUS_TIME_TYPE

Sets dwReturn to a constant describing the current time type in use (used by play, record, seek, etc.), and will be one of the following:

MCI_VCR_TIME_TIMECODE

Timecode is in use.

MCI_VCR_TIME_COUNTER

Counter is in use.

MCI_VCR_STATUS_POWER_ON

Sets dwReturn to TRUE if the power is on.

MCI_VCR_STATUS_CLOCK_ID

Sets dwReturn to a number which uniquely describes the clock in use.

MCI_VCR_STATUS_CLOCK

Sets dwReturn to the current clock value, in total clock increments.

MCI_STATUS_START

Sets dwReturn to 0.

LPMCI_VCR_STATUS_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_STATUS_PARMS data structure.

### MCI_STEP

This message steps the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_STEP_FRAMES

dwFrames contains the number of frames to step by.

MCI_VCR_STEP_REVERSE

The step direction is reverse.

LPMCI_VCR_STEP_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_STEP_PARMS data structure.

### MCI_STOP

This message stops the device.

DWORD   lParam1

Parameters

Specifies the MCI_NOTIFY, MCI_WAIT, and MCT_TEST flags.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_UNFREEZE

This message unfreezes the input or output and indicates that the device resumes displaying normal output or receiving normal input.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_UNFREEZE_INPUT

Unfreeze the input.

MCI_VCR_UNFREEZE_OUTPUT

Unfreeze the output.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_VCR_CUE_PARMS

The MCI_VCR_CUE_PARMS structure contains parameters used by the MCI_CUE message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_CUE_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrom;
	DWORD	dwTo;
} MCI_VCR_CUE_PARMS;

The MCI_VCR_CUE_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrom

Specifies the position to cue from.

dwTo

Specifies the position to cue to.

### MCI_VCR_LIST_PARMS

The MCI_VCR_LIST_PARMS structure contains parameters used by the MCI_LIST message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_LIST_PARMS {
	DWORD	dwCallback;
	DWORD	dwReturn;
	DWORD	dwNumber;
} MCI_VCR_LIST_PARMS;

The MCI_VCR_LIST_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwReturn

Contains the return information on exit.

dwNumber

Specifies the number of video or audio inputs supplied with the VCR.

### MCI_VCR_PLAY_PARMS

The MCI_VCR_PLAY_PARMS structure contains parameters used by the MCI_PLAY message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_PLAY_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrom;
	DWORD	dwTo;
	DWORD	dwAt;
} MCI_VCR_PLAY_PARMS;

The MCI_VCR_PLAY structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrom

Specifies the beginning play position specified in the current-time format.

dwTo

Specifies the ending play position specified in the current-time format.

dwAt

Specifies a time when the play command begins, or when a device is cued, when the device reaches the dwFrom position given with the MCI cue command.

### MCI_VCR_RECORD_PARMS

The MCI_VCR_RECORD_PARMS structure contains parameters used by the MCI_RECORD message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_RECORD_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrom;
	DWORD	dwTo;
	DWORD	dwAt;
} MCI_VCR_RECORD_PARMS;

The MCI_VCR_RECORD_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrom

Specifies the beginning play position specified in the current-time format.

dwTo

Specifies the ending play position specified in the current-time format.

dwAt

Specifies a time when the play command begins, or when a device is cued, when the device reaches the dwFrom position given with the MCI cue command.

### MCI_VCR_SEEK_PARMS

The MCI_VCR_SEEK_PARMS structure contains parameters used by the MCI_SEEK message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SEEK_PARMS {
	DWORD	dwCallback;
	DWORD	dwTo;
	DWORD	dwMark;
	DWORD	dwAt;
} MCI_VCR_SEEK_PARMS;

The MCI_VCR_SEEK_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTo

Specifies a position to seek to in the current-time format.

dwMark

Specifies a numbered mark to search for.

dwAt

Specifies a time when the seek command begins.

### MCI_VCR_SET_PARMS

The MCI_VCR_SET_PARMS structure contains parameters used by the MCI_SET message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SET_PARMS {
	DWORD	dwCallback;
	DWORD	dwTimeFormat;
	DWORD	dwAudio;
	DWORD	dwTimeMode;
	DWORD	dwRecordFormat;
	DWORD	dwCounterFormat;
	DWORD	dwIndex;
	DWORD	dwTracking;
	DWORD	dwSpeed;
	DWORD	dwLength;
	DWORD	dwCounter;
	DWORD	dwClock;
	DWORD	dwPauseTimeout;
	DWORD	dwPrerollDuration;
	DWORD	dwPostrollDuration;
} MCI_VCR_SET_PARMS;

The MCI_VCR_SET_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTimeFormat

Specifies the time format used with the play, record, seek cue, and status commands.

dwAudio

Not used.

dwTimeMode

Contains a constant that specifies the timing source used by the device. The timing source is either a timecode recorded on tape, or the counters in the device that sense tape movement.

dwRecordFormat

Specifies the recording rate used by the device.

dwCounterFormat

Specifies the format of a new counter-time value.

dwIndex

Specifies the contents of the on-screen display.

dwTracking

Specifies the speed adjustment used when tracking the VCR playback rate.

dwSpeed

Specifies the playback rate used by the device as an integer. Normal playback speed is 1000, double speed is 2000, and half speed is 500.

dwLength

Specifies the tape length when the tape length is undetectable by the device.

dwCounter

Specifies a new counter value.

dwClock

Specifies a new clock time.

dwPauseTimeout

Specifies a new timeout value for the pause command.

dwPrerollDuration

Specifies the videotape length needed to stabilize the VCR output.

dwPostrollDuration

Specifies the videotape length needed to brake the VCR transport when a stop or pause command is issued.

### MCI_VCR_SETAUDIO_PARMS

The MCI_VCR_SETAUDIO_PARMS structure contains parameters used by the MCI_SETAUDIO message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SETAUDIO_PARMS {
	DWORD	dwCallback;
	DWORD	dwTrack;
	DWORD	dwTo;
	DWORD	dwNumber;
} MCI_VCR_SETAUDIO_PARMS;

The MCI_VCR_SETAUDIO_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTrack

Specifies the track that is affected by the command.

dwTo

Specifies the type of input or monitored input.

dwNumber

Specifies the audio input (of the type specified in the dwTo field) to use.

### MCI_VCR_SETTUNER_PARMS

The MCI_VCR_SETTUNER_PARMS structure contains parameters used by the MCI_SETTUNER message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SETTUNER_PARMS {
	DWORD	dwCallback;
	DWORD	dwChannel;
	DWORD	dwNumber;
} MCI_VCR_SETTUNER_PARMS;

The MCI_VCR_SETTUNER_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwChannel

Specifies the new channel number.

dwNumber

Specifies the logical tuner that the MCI_SETTUNER command affects.

### MCI_VCR_SETVIDEO_PARMS

The MCI_VCR_SETVIDEO_PARMS structure contains parameters used by the MCI_SETVIDEO message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SETVIDEO_PARMS {
	DWORD	dwCallback;
	DWORD	dwTrack;
	DWORD	dwTo;
	DWORD	dwNumber;
} MCI_VCR_SETVIDEO_PARMS;

The MCI_VCR_SETVIDEO_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTrack

Specifies the track that is affected by the command.

dwTo

Specifies the type of input or monitored input.

dwNumber

Specifies the video input (of the type specified in the dwTo field) to use.

### MCI_VCR_STATUS_PARMS

The MCI_VCR_STATUS_PARMS structure contains parameters used by the MCI_STATUS message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_STATUS_PARMS {
	DWORD	dwCallback;
	DWORD	dwReturn;
	DWORD	dwItem;
	DWORD	dwTrack;
	DWORD	dwNumber;
} MCI_VCR_STATUS_PARMS;

The MCI_VCR_STATUS_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwReturn

Specifies the value returned by the MCI status command. The return value varies according to the inquiry of the status command. For additional information, see the description of the MCI_STATUS message.

dwItem

Specifies the type of information the status command supplies.

dwTrack

Specifies the audio or video track that will store information during the next recording. This field is used to return information when the status command inquires about the video or audio recording status.

dwNumber

Specifies the logical tuner that the current channel is associated with. This field is used to return information when the status command inquires about the current channel number.

### MCI_VCR_STEP_PARMS

The MCI_VCR_STEP_PARMS structure contains parameters used by the MCI_STEP message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_STEP_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrames;
} MCI_VCR_STEP_PARMS;

The MCI_VCR_STEP_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrames

Specifies the number of frames to jump (the length of a single step) as the step command steps forward or backward through the media.

## Command-String Reference

This reference section describes the MCI command strings for the VISCA video-cassette recorder (VCR) device driver that you can use with applications that support the MCI command-string interface. (Applications with this interface send the command strings to MCI using the mciSendStr function.) For more information about using command strings and the command-string interface, see the Multimedia Programmer’s Reference in the Microsoft Windows 32 Software Development Kit.

This command set is taken from the digital-video command set for MCI. Any device-specific functionality affecting the MCI commands is also noted in this section.

### Additional MCI String Options

All commands accept the optional flags notify, wait, and test flags as described in the VCR MCI command set.

The VCR driver uses the VCR keyword to identify the device type. The constant MCI_DEVTYPE_VCR is defined for this device type.

### capability

capability devicename parameter

Syntax

The capability command requests information about a particular capability of the VCR and driver.

The following optional items modify capability:

Parameters

can detect length

Returns “true” if the VCR can detect the length of media in the VCR.

can eject

Returns “true” if the VCR can control tape ejection using software commands.

can freeze

Returns “true” if the VCR supports freeze and unfreeze.

can monitor sources

Returns “true” if the VCR is capable of passing an input (source) to the monitored output, independent of the current input selection.

can play

Returns “true” if the VCR is capable of playing.

can preroll

Returns “true” if the VCR supports the preroll flag with the cue command.

can preview

Returns false.

can record

Returns “true” if the VCR is capable of recording.

can reverse

Returns “true” if the VCR can play in reverse.

can save

Returns “false.” This must be supported for compatibility with other MCI devices.

can test

Returns “true” if the VCR supports the test flag.

clock increment rate

Returns the number of subdivisions the external clock supports per second. If the external clock is a millisecond clock, the return value is 1000. If the return value is 0, then no clock is supported.

compound device

Returns “false.”

device type

Returns the device-type short name (“VCR” for VCRs).

has audio

Returns “true” if the VCR has audio output.

has clock

Returns “true” if the VCR has a clock.

has timecode

Returns “true” if the VCR is capable of supporting timecode, or if it is unknown.

has video

Returns “true” if the VCR has video output.

number of marks

Returns the maximum number of marks that can be used; 0 indicates that marks are unsupported.

seek accuracy

Returns the expected accuracy of a search in frames; 0 indicates that the VCR is frame accurate, 1 indicates that the VCR expects to be +/- 1 frame from the indicated seek position, etc.

uses files

Returns “false.”

### cue

cue devicename parameter

Syntax

Prepare for the next play or record command.

When cueing for playback (using cue output), issuing the play command with the from, to, or reverse flag cancels the cue command.

Notes

When cueing for recording (using cue input), issuing the record command with the from, to, or initialize flag cancels the cue command.

The following optional items modify cue:

Parameters

input

Prepare the VCR for recording.

output

Prepare the VCR for playing. If neither input nor output is specified, output is assumed.

from position

Indicates where to start.

to position

Indicates where to stop.

reverse

Play direction is in reverse (backward). This flag cannot be used with the input flag.

preroll

Move the preroll distance from the “in-point.” The “in-point” is the current position, or the position specified by the from flag.

### freeze

freeze devicename parameter

Syntax

Freezes either video input or video output on the VCR. This command is intended for use with frame-grabbing cards. If neither input or output is specified, output is assumed.

The following optional items modify freeze:

Parameters

output

Freezes the output image. If the VCR is playing when this command is issued, then the current frame is frozen, and the VCR is paused. If the VCR is paused when freeze is issued, the current frame is frozen. The frozen image remains on the output device until an unfreeze command is issued.

input

Freezes the input image.

field

Freezes the first field. Field is assumed by default (if neither frame or field is specified).

frame

Freezes the entire frame, displaying both fields.

### index

index devicename parameter

Syntax

Control the on-screen display.

The following optional items modify index:

Parameters

on

Turns on the on-screen display. The item to be displayed is specified by set index.

off

Turns off the on-screen display.

### info

info devicename parameter

Syntax

Fills a user-supplied buffer with a string containing information about the VISCA driver.

The following optional items modify info:

Parameters

product

Returns the manufacturer of the hardware and if possible, VCR model information.

version

Returns the release level of the driver and hardware.

### list

list devicename parameter

Syntax

Used to determine the number and types of video and audio inputs. Either video source or audio source must be specified with either the count or number flags. If count is given, the total number of inputs of video or audio is returned. If number is specified, the driver returns a type corresponding to the input. The type can be any one of the following: “tuner,” “line,” “svideo,” “aux,” or “generic.” Typically, you first query the VCR for the count and then use the count as the range for the number flag. The source numbers start from 1.

The following optional items modify list:

Parameters

audio source count

Returns total number of audio inputs.

audio source number index

Returns the type of audio input of source index.

video source count

Returns total number of video inputs.

video source number index

Returns the type of video input of source index.

### mark

mark devicename parameter

Syntax

Controls recording and erasing of marks on the tape. Marks are defined to be special signals written to the media that can be detected by the VCR during high-speed searches. For this reason, marks are VCR specific.

The following optional items modify mark:

Parameters

write

Write a mark at the current position. The VCR might need to be in play or record mode for this command to succeed.

erase

Erase a mark at the current position, if one exists. To erase a mark, first seek to the mark and then issue the mark erase command.

### pause

pause devicename

Syntax

Pauses the VCR, leaving the video displayed. Issuing a pause command when the VCR is already paused will have no effect. Issuing pause while the VCR is stopped places the VCR in pause state.

### play

play devicename parameter

Syntax

Begin playing at the current speed setting (from last set speed command). The direction is reverse if the reverse flag is given, or if the to parameter is specified as a value less than the from parameter. If the from parameter is not given, then the current position is taken as the starting point. The to and reverse parameters cannot be used together.

The following optional items modify play:

Parameters

from position

Indicates where to start playing.

to position

Indicates where to stop playing.

reverse

Indicates that the play direction is in reverse; for example, backward.

at time

Indicates when the VCR begins performing this command, or if the device has been cued (for more information, see the cue command), when the cued command begins.

scan

Play as rapidly as possible while maintaining video output.

### record

record devicename parameter

Syntax

Begin recording. The tracks to be recorded are specified by the set timecode record, set assemble record, setvideo record, and setaudio record commands.

The following optional items modify record:

Parameters

from position

An optional parameter that specifies the position to start recording. The default is to start recording from the current position.

to position

An optional parameter that specifies where to stop recording.

initialize

Initialize the tape (media), which involves recording timecode (if possible) for blank video and audio. This command might take several hours if the entire tape must be initialized.

at time

For information, see the play at command.

### resume

resume devicename

Syntax

Resume a play, record, or seek command that has been placed in the pause state using the pause command. If currently playing or recording, the resume command will be ignored.

### seek

seek devicename parameter

Syntax

Change position as fast as possible. If neither the reverse nor the to parameter are specified, then the VCR will move forward to the end of the tape as fast as possible.

The following optional items modify cue:

Parameters

to position

Moves the tape to position.

to start

Moves to the beginning of the tape.

to end

Moves to the end of the tape.

mark mark_num

Seeks to the relative mark indicated by mark_num, which must be a positive integer value. Marks are defined to be signals, written to the tape using the mark command, and are used for high-speed relative searching.

reverse

Seek direction is backward. The reverse parameter is not valid if the to parameter is specified.

at time

For information, see the play at command.

### set

set devicename parameter

Syntax

Sets the state of a variable for the driver instance.

The following optional items modify set:

Parameters

clock time

Sets time on the external clock to time. The format is specified as a long unsigned integer.

time mode detect

Sets the position information mode based on detection of timecode information on the tape. If timecode information is detected, the time type is set to timecode; otherwise, the time type is set to counter.

Detect is a special mode. Whenever the driver is opened, a new tape is inserted, or the time mode command is issued, the driver checks the current time mode available on the tape and sets time type to either timecode or counter. Once time type is set, the driver doesn’t change it until one of the above conditions occurs again.

time mode timecode

Sets the position information mode to use timecode information on the tape.

time mode counter

Sets the position-information mode to use the VCR counters.

counter format

Set the time format for the counter, as returned by status counter. For information about applicable types, see the set time format command.

time format milliseconds

time format ms

Set the time format, as returned by status position, to milliseconds (ms). The driver round at the thousandths place when returning millisecond times; for example, .06666666 equals .0667.

time format frames

Instructs the driver to use the absolute frame number (from start of tape) to reference the position/time on the tape.

time format smpte 24

time format smpte 25

time format smpte 30

Sets the time format to hh:mm:ss:ff, where the legal values are 00:00:00:00 through 23:59:59:xx, and xx is one less than the frames per second as specified by the number after the smpte token. On input, colons (:) are required to separate the components. The least significant units can be omitted if they are 00; for example, 02:00 is the same as 02:00:00:00.

time format smpte 30 drop

Same as smpte 30, except certain timecode positions are dropped from the format to have the recorded timecode positions for each frame (at the NTSC frame rate of 29.97 fps) correspond to real time (at 30 fps). Timecode positions that are dropped are as follows: two every minute, on the minute, for the first nine of every ten minutes of recorded content. For example, at 01:04:59:29, the next timecode position would be 01:05:00:02, not 01:05:00:00.

time format hms

Indicates the time format is hh:mm:ss. Positioning defaults to any frame at the specified time offset (second).

time format tmsf

Indicates the time format is tt:mm:ss:ff.

time format msf

Indicates the time format is mm:ss:ff.

door open

Ejects a tape automatically (open equals eject).

door closed

Loads a tape automatically.

record format SP

record format LP

record format EP

Sets the recording mode for the VCR to one of the following values:

SP (standard play)
EP (extended play)
LP (long play)

These values are not intended to be VHS specific. They map to any three appropriate modes with other tape formats; for example, SP maps to the fastest, highest quality recording.

speed factor

Sets the relative speed of playback from the VCR. Factor is a value indicating the desired speed relative to the normal speed, which is 1000 (500 is half normal speed, 2000 is twice normal speed, etc.).

length duration

Sets the user-specified length of the tape in the VCR. This length is returned by status length and is provided for compatibility with applications that require status length to return a valid length.

index timecode

index counter

index date

index time

Sets the current display screen on the VCR.

counter value

Sets the VCR counter to the specified value. The value must be specified in the current counter format (for more information, see the set counter format command).

power on

power off

Sets the VCR power to on or off.

assemble record on

assemble record off

In assemble mode, all tracks are recorded as defined by the VCR. Most VCRs will be in assemble mode by default.

preroll duration duration

Sets the length, in the current-time format, needed to stabilize the VCR output.

postroll duration duration

Sets the length, in the current-time format, needed to brake the VCR transport when a stop or pause command is issued.

pause timeout

Sets the maximum duration, in milliseconds, of a pause command. A timeout value of 0 indicates that no time out will occur.

tracking plus

tracking minus

tracking reset

Finely adjusts the speed of the videotape transport. Use the tracking parameters when a noisy picture is obtained from a VCR. Tracking plus increases the transport speed. Tracking minus decreases the transport speed. Tracking reset returns the tracking adjustment to zero.

### setaudio

setaudio devicename parameter

Syntax

Sets the audio track input (source = input), record, and monitor settings.

The following optional items modify setaudio:

Parameters

source to type number number

Selects the audio source to be recorded on the tape. Type must be “tuner,” “line,” “svideo,” “aux,” “generic,” or “mute.”

record on

record track track_number on

Selects the audio source to be recorded with the next record command. Track allows independent track selection. Track 2 corresponds to the PCM track in Hi8. If track is not specified, a default of 1 is assumed.

record off

record track track_number off

Clears the audio-source selection so that no audio will be recorded with the next record command. Track allows independent track selection. Track 2 corresponds to the PCM track in Hi8. If track is not specified, a default of 1 is assumed.

monitor to type number number

Controls which source input will be passed to the VCR output, without changing the recording source input selection. Type can be “output,” or one of the valid input sources. If number is not specified, then the first input of that type will be chosen.

### settimecode

settimecode devicename parameter

Syntax

Enables or disables timecode recording for a VCR.

The following optional items modify settimecode:

Parameters

record on

record off

Record on sets the VCR to record timecode. Record off disables timecode recording.

### settuner

settuner devicename parameter

Syntax

Changes the current tuner or the channel setting of the current tuner.

The following optional items modify settuner:

Parameters

channel up

channel down

Increments or decrements the tuner channel. You might not be able to change the channel while recording, depending on the VCR. The tuner wraps to channel 1 when the maximum channel number is exceeded. Similarly, when seeking down, the tuner wraps to the maximum channel.

channel channel

Sets the tuner to channel. You might not be able to change the channel while recording, depending on the VCR. A channel larger than the maximum sets the tuner to the maximum channel.

channel seek up

channel seek down

Seeks the next channel with a valid signal. Seek up increments the channel number in its search. Seek down decrements the channel number while in its search. The tuner wraps to channel 1 when the maximum channel number is exceeded. Similarly, when seeking down, the tuner wraps to the maximum channel.

number number

Specifies the tuner to be affected by the settuner command. If number is not given, the default of 1 is assumed.

### setvideo

setvideo devicename parameter

Syntax

Video track input (source = input), record, and monitor settings.

The following optional items modify setvideo:

Parameters

source to type number number

Selects the video source to be recorded on the tape. Type must be “tuner,” “line,” “svideo,” “aux,” “generic,” “mute,” or “rgb.”

record on

record track track_number on

Selects the video source to be recorded with the next record command. Track allows independent track selection. Track 2 corresponds to the PCM track in Hi8. If track is not specified, a default of 1 is assumed.

record off

record track track_number off

Clears the video-source selection so that no audio will be recorded with the next record command. Track allows independent track selection. If track is not specified, a default of 1 is assumed. It might be necessary to first issue set assemble record off before the video recording can be turned off.

monitor to type number number

For more information, see the setaudio command.

### status

status devicename parameter

Syntax

Returns status information about the VCR.

The following optional items modify status:

Parameters

position

position track track_num

Returns the current position. The driver returns an error if the current position is unknown.

position start

Returns the position of the start of the usable media.

counter

Returns the counter position, in the current counter format.

length

length track track_num

Returns the tape length or the length of the specified track.

number of tracks

This returns the total number of tracks on the media. For VCRs, the return value is usually 1 because the term “tracks” in MCI refers to consecutive, random-accessible media sequences (such as tracks of music on a compact disc), not the individual signals recorded on the tape (for example, an audio “track,” or a video “track,” etc.).

number of video tracks

Returns the total number of video tracks on the media.

number of audio tracks

Returns the total number of audio tracks on the media.

clock

Returns the external time. The time must be an unsigned long integer expressing total increments. For more information, see the status clock increment rate command.

clock id

Returns a unique integer identifying the clock.

ready

Returns one of “true” or “false,” indicating whether or not the VCR is ready to receive a play command.

counter format

Returns the current counter format. For more information, see the set counter format command.

counter resolution

Returns “frames” or “seconds,” indicating the counter’s resolution. This is not the same as accuracy.

time format

Returns the current-time format (for more information, see the set time format command). The defined values are “milliseconds,” “frames,” “hms,” “tmsf,” “msf,” “smpte 24,” “smpte 25,” “smpte 30,” or “smpte 30 drop.”

timecode type

Returns “smpte,” “smpte drop,” “other,” or “none.” Note the frames per second can be obtained from the status frame rate command, and the accuracy of the device can be returned by the capability seek accuracy command.

time format

Returns the current-time format (for more information, see the set time format command). The defined values are “milliseconds,” “frames,” “hms,” “tmsf,” “msf,” “smpte 24,” “smpte 25,” “smpte 30,” or “smpte 30 drop.”

current track

Returns 1 (one).

media present

Returns “true” if a tape is inserted in the VCR.

timecode present

Returns “true” if timecode has been recorded at the current position on the tape. The timecode must advance from the current position. A VCR might need to be played to check this condition.

frame rate

Returns the number of frames per second that the device will use by default. NTSC returns 30, PAL 25, etc.

speed

Returns the last set-speed factor. (This applies to playing only.)

media type

Returns the type of media being used. The current return types are “8mm,” “vhs,” “svhs,” “beta,” “Hi8,” “edbeta,” or “other.”

record format

Returns a code indicating the format that the tape will be recorded in. The current return types are “lp,” “ep,” “sp,” or “other.” These formats are not VHS specific, but instead can be applied to any VCR that has multiple selectable recording formats. The “sp” type is the fastest, highest quality recording format, and is also used as default on single format VCRs.

play format

Returns a code indicating the format that the tape will be played back in, if detectable. The return values are “lp,” “ep,” “sp,” or “other.” For more information, see the record format command.

mode

The mode command returns the current status of the VCR, as follows:

“stopped”	Returned if the VCR is stopped.
“opened”	Returned if there is no tape in the unit.
“recording”	Returned if the VCR is currently recording.
“paused”	Returned if the VCR is paused.
“seeking”	Returned if the VCR is seeking.
“playing”	Returned if the VCR is playing, regardless of speed.
“not ready”	Returned if the VCR is in a state not enumerated above.

audio source

Returns the audio-source type (for more information, see the setaudio command).

video source

Returns the video-source type (for more information, see the setvideo command).

audio monitor

Returns “output,” or one of the valid source-input types (for more information, see the setaudio monitor command).

video monitor

Returns “output,” or one of the valid source-input types (for more information, see the setvideo monitor command).

audio source number

Returns the audio-source number of the type returned by status audio source (for more information, see the setaudio command).

video source number

Returns the video-source number of the type returned by status video source (for more information, see the setvideo command).

audio monitor number

Returns the monitored-video number of the type specified by status audio monitor (for more information, see the setaudio command).

video monitor number

Returns the monitored-video number of the type returned by status video monitor (for more information, see the setvideo command).

index on

Returns “true” if the index is on.

index

Returns the current index display (for more information, see the set index command).

channel

Returns the integer value of the channel set on the tuner.

channel tuner number number

If tuner number is given, then the currently selected channel on the logical tuner number will be returned. Note that there can be several logical tuners.

write protected

Returns “true” if the VCR detects that it cannot record (that is, the write protect is on). If it can record, or if it is unable to determine whether or not it can record (without actually writing), the driver returns “false.”

timecode record

Returns “true” if the VCR is set to record timecode.

assemble record

Returns “true” if the VCR is set to assemble mode recording.

video record

video record track track_number

Return “true” if the VCR is set to record video. If no track number is given, the default track 1 is assumed.

audio record

audio record track track_number

Return “true” if the VCR is set to record audio. If no track number is given, the default track 1 is assumed.

time mode

Returns the current-position time mode. It can be “detect,” “timecode,” or “counter.”

time type

Returns the current-position time in use: “timecode” or “counter.”

power on

Returns “true” if the VCR’s power is on.

preroll duration duration

Returns the length of tape, in the current-time format, needed to stabilize the VCR output.

postroll duration duration

Returns the length of tape, in the current-time format, needed to brake the VCR transport when a stop or pause command is issued.

pause timeout timeout

Returns the maximum duration, in milliseconds, of a pause command. A timeout value of 0 indicates that no time-out will occur.

### step

step devicename parameter

Syntax

Step one or more frames forward or reverse. If no parameters are used in the command, the default action is to step one frame forward.

The following optional items modify step:

Parameters

by num_frames

Step by num_frames frames in the specified direction; for example, step by 10 would step ahead 10 frames (possibly displaying the frames in between). The num_frames value must be a positive number.

reverse

Step in reverse.

### stop

stop devicename

Syntax

Stop playing or recording and remain at the current location. No video or audio is displayed.

### unfreeze

unfreeze devicename parameter

Syntax

Return to displaying or inputting normal video.

The following optional items modify unfreeze:

Parameters

output

Unfreeze the output image. If neither input or output is given, output is assumed.

input

Unfreeze the input image.

## Archaic String Commands

These commands will be supported by all videotape recorder drivers for compatibility with other MCI classes. These functions should be accessed through the setaudio and setvideo commands.

### set

set devicename parameter

Syntax

Sets the state of various control items.

The following optional items modify set:

Parameters

audio all on

audio all off

Enables or disables all audio output.

audio left on

audio left off

Enables or disables audio output to the left channel.

audio right on

audio right off

Enables or disables audio output to the right channel.

video on

video off

Enables or disables video output.