Chapter 1

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Overview of the Telephony SPI

# What Telephony Is

Telephony is a technology that integrates computers with the telephone network. With telephony, people can use the capabilities of their computers to take advantage of a wide range of sophisticated communications features over a telephone line.

The focus of the Windows Telephony API (TAPI) is to provide users of the Windows environment with "personal telephony" by enabling telephony functions within desktop applications. TAPI supports both speech and data transmission, allows for a variety of terminal devices, and supports complex connection types and call-management techniques such as conference calls, call waiting, and voicemail. TAPI allows all elements of telephone usage, from the simple dial-and-speak call to international e-mail, to be controlled via the intuitive Windows graphical user interface.

# Using Telephony in Applications

Telephony capabilities help people get the most from telecommunications systems. Users can not only efficiently manage their voice calls, but also data-transfer operations, which makes telecommuting more productive. These advanced functions will become available to users in the familiar form of Microsoft Windows applications.

The Telephony API and SPI give developers a consistent set of tools for creating these Windows-based telephony applications. Although the sophistication of applications will increase over time, we can already predict some of the functionalities that will soon be incorporated into them. The following examples describe various possibilities, all programmable with TAPI:

·	Personal information managers that offer features such as automatic dialing, voicemail sorting, and collaborative computing over telephone lines.

·	Strengthened information applications through the connection of database managers, spreadsheets, and word processors directly into the telephone network.

·	Electronic directory managers that let users manage complicated telephone functions such as setting up conference calls. It could be possible, for example, to initiate a conference by dragging three or four names from an electronic directory into a "conference box" and clicking "connect." In the future, video as well as audio conference calls will be available at the desktop, supported by TAPI.

·	Icon-driven control of data transmission. With drag and drop, users can send files from a local disk to that of a remote colleague, specifying first whether to transmit the document as a file, a facsimile, or electronic mail. When ISDN connections become more widespread, users can send data to the recipient simultaneously with a voice call to the same person.

·	Functions making use of caller-ID. Users can identify an inbound call while on another line without putting anyone on hold. The user could then send the second caller a prerecorded personalized message such as "Hold on, Bill; I'll be right with you," or route the inbound call directly to voicemail.

·	Remote-control applications let users operate their PCs from a distance over public telephone lines. Developers can use TAPI to write both the host and the caller modules of such remote-control applications.

·	Access to information services through TAPI applications. Although all of today's information services have their own front ends, developers can write new interfaces, standardized or customized for specific environments.

Adding the power of the computer will no doubt encourage the development of as-yet unimagined uses for telephone communications.

## Telephone Network Services

The Telephony SPI provides an abstraction of telephony services that is independent of the underlying telephone network and the configuration used to connect the PC to the switch and phone set. And as seen from the other side, it provides independent abstractions of the PC connections to the switch or network and the phone set. The connection may be realized in a variety of arrangements including pure client-based wired or wireless connections, or client/server configurations using some sort of local area network.

Currently, most telephone connections in the world are of the type POTS, or "Plain Old Telephone Service." Most POTS connections are analog, carrying either human speech or data that has been (temporarily) converted by a modem from digital to analog format. The Telephony API and its documentation support the development of applications using, but not restricted to, POTS. Using Telephony for POTS is straightforward because POTS is comparatively simple. It normally uses only one media mode per call, supports one channel per line, and so on. The vast majority of uses for TAPI are still POTS, and most telephony programmers will use TAPI for POTS applications only.

But TAPI also lets you make connections over other types of networks. More advanced kinds of data transmission methods are being developed, refined, and installed. For example, one important digital service is ISDN (Integrated Services Digital Network), which is expected to grow significantly in availability. ISDN networks have the following advantages when compared to POTS:

·	ISDN is completely digital. Because data travels from one end of the ISDN network to the other in digital format, error rates are lower than with analog transmission.

·	Speeds of up to 128 Kbps are possible on BRI-ISDN lines (and much higher on PRI-ISDN lines), compared with today's maximum dial-up modem signaling rates of 28.8 Kbps.

·	Each ISDN line, depending on its transmission rate, provides at least three channels (two for voice or data and one strictly for data or signaling information) and as many as 32 channels, for simultaneous, independently operated transmission of voice and data.

·	The ISDN standard is international.

TAPI can be used effectively for ISDN, but the complexities of ISDN makes the use of TAPI for ISDN less simple than for POTS. For example, the function lineGetDevCaps returns information about the capabilities of the current line. For ISDN, using this information is both more complex and more powerful because of ISDN's support for multiple channels on one line. In this document, the descriptions of TAPI's functions tells which of its elements are best used with, or logically restricted to, calls carried by ISDN networks, and how to take advantage of those elements.

Besides ISDN, TAPI can be used with other digital networks such as T1/E1 and Switched 56 service. With Switched 56, some local and long-distance telephone companies provide signaling at 56 Kbps over dial-up telephone lines. Switched 56 is quickly becoming available throughout the U.S. and in many other countries. It requires special equipment, and though its connection capabilities are limited to calls to other specially equipped facilities, its high speed and newly lowered pricing make it a reasonable choice for many data communications needs. In TAPI, Switched 56 will be modeled as a data-bearer service with certain defined capabilities. The capabilities will probably be limited, because Switched 56 is used for data calls only.

The way Switched 56 lines are used, though dependent on the implementation provided by the service provider, would be transparent to TAPI-based applications. Applications would use standard methods for placing calls (such as the function lineMakeCall), and the service provider does whatever it needs to give the phone number to the phone network. To the application, the Switched 56 line would simply appear to be a channel that supports digital bearer service at the 56 Kbps rate. The media mode of calls it carries would be "digital data."

TAPI can also be used with other services such as CENTREX, which provides a set of centralized network services (such as conferencing) without the need to install special equipment. With CENTREX, you pay for the use of telephone-company equipment over regular telephone lines. In addition, TAPI can be used with digital PBXs (Private Branch Exchanges) and key systems.

µ §

TAPI provides a means of controlling calls and the lines they use over analog and digital networks of various types.

# Telephony is a WOSA Component

Windows Telephony is a component of WOSA (Windows Open Services Architecture), and as such consists of an application programming interface (API) used by applications and a service provider interface (SPI) implemented by service providers.

The WOSA model defines an abstraction layer to heterogeneous computing resources through the WOSA set of APIs. The front-end application and back-end service don’t need to speak each other's language in order to communicate, as long as they both know how to talk to the WOSA interface. An application needs to know only the definition of the API, not its implementation.

Telephony is designed to function within the WOSA model. Between TAPI and the Telephony SPI is the Windows operating system itself, in which telephony services are executed by the Telephony DLL, called TAPI.DLL. This relationship is illustrated below:

µ §

The Telephony API and SPI within the WOSA model

In this diagram, the first Service Provider (SP) is labeled "Default." This SP was created by Microsoft and ships with TAPI. It includes the Unimodem driver, which supports data transmission via modem, and it implements a number of the functions of Basic Telephony that work with standard Hayes-compatible modems. This default SP was not intended to be a fully functioning service proivder, but rather a sample application on which third-party developers can base their development of more extensive service providers. For example, it can be used only for outgoing calls.

The "Custom" SPs can be written by third parties to extend Telephony's services beyond those of the Basic and Supplementary sets. These SPs can also provide non-modem functionality or Basic (or parts of Supplementary) Telephony for non-Hayes-compatible modem environments.

Note that multiple Telephony applications can operate simultaneously, all making use of an SP. Also, multiple service providers can be used, as long as they access different hardware devices. For example, a data-transfer application may need to use the Comm driver in Windows (COMM.DRV, used for communicating with the serial port) in order to send a file. For this to work, the SP's device driver would need to be specified by the user. Some service providers may actually contain a device-driver component, but in general, service providers are separate and "above" the drivers, as shown in the illustration.

#### Support for Serial Ports

The Microsoft Virtual Communications Driver (VCOMM) provides support for the different kinds of custom serial port hardware now available. To do this, it acts as a communications link between service providers and the various serial port device drivers.

For more information about VCOMM and the complex mechanisms used at the service-provider and device-driver levels, please refer to Part 9: Communication Drivers in the Microsoft Windows Device Drivers Programmer's Reference. This book is a part of the Microsoft Windows DDK. Part 9's chapters are Communications Overview, Virtual Communications Driver, Communication Port Drivers, Port Virtualization Devices, and Port Configuration Libraries.

#### Multiple Service Providers and TAPI.DLL

All calls to any service provider (default or custom) pass through TAPI.DLL. TAPI.DLL acts as a broker, which routes calls from the application to the appropriate service provider, several of which may be running at once. It performs this routing by knowing which service provider is associated with the handle it has just received from the application as a parameter for a function call.

Using different service providers works as long as each one controls different hardware from the others. An application could not, for example, use different SPs to access the same modem, but it could use one SP that simultaneously handles a fax board, an ISDN card, a telephone, and a modem. All active SPs know only about TAPI, and not about the application or applications from which function calls originate.

## The Advantages of WOSA

As shown in the diagram on the previous page, WOSA provides a single-level interface for connecting front-end applications (shown at the top) with back-end services (shown at the bottom). These are some of the advantages that WOSA provides:

·	WOSA provides an extensible framework in which applications can seamlessly access information and resources in a distributed environment.

·	Application developers and users needn't worry about conversing with numerous services, each with its own protocols and interfaces. Instead, applications access many different services through a single, consistent interface. Applications don't have to be modified in any way to enjoy this support, as long as they are designed to operate in the absence of features that the service provider is treating as optional (and thus does not support).

·	WOSA allows application developers and vendors of back-end services to mix and match applications and services to build solutions that shield programmers and users from the underlying complexity of the system.

·	Applications written to the WOSA APIs have access not only to all the various computing environments supported today, but also to all additional environments as they become available. Because this set of APIs is extensible, new services and their corresponding APIs can be added as needed. (Telephony and its API are an example.)

·	WOSA uses a Windows dynamic-link library (DLL) that allows software components to be linked at runtime. In this way, applications can connect to services dynamically.

### The Telephony API and the Telephony SPI

Because Telephony services under Windows follow the WOSA model, they are implemented at three complementary levels:

·	The Telephony API, an interface provided to applications by TAPI.DLL, gives application programmers access to telephony services.

·	The Telephony Dynamic Link Library (DLL) is part of the Windows operating system. Its main function is to connect applications to the service providers by mapping applications' function calls to their implementations in the service provider(s).

·	The Telephony SPI is the interface provided by TAPI.DLL to service providers (and thus to the service-provider vendors who write them).

The Telephony API is the subject of this document. The Telephony SPI is documented in the Telephony Service Provider Guide which is part of the Windows Telephony Development Kit.

### Other APIs Needed for Media-Stream Control

TAPI by itself provides control only for line and phone devices, and does not give access to the information exchanged over a call. To manage this media stream itself, developers need to integrate functions from other APIs into telephony applications. In this manner, TAPI works in conjunction with other Windows-based services such as Windows multimedia wave audio, MCI, or data APIs. This separation of control guarantees maximum interoperability with existing audio or data applications.

For example, an application that provides an interface for managing fax or data transmission would use TAPI functions to control and monitor the line over which bits are sent, but the actual transmission of data is accomplished with functions from another API, such as the communications functions (known collectively as the Comm API) of the Windows SDK.

For added control, the Modem Configuration Extensions (MCX), an extension to the Microsoft Virtual Communications Driver will be available in a future version of Windows. It defines a standard way for applications to configure features of modems, and is included in the Virtual Communications Driver documentation of the Microsoft Windows 4.0 DDK.

Note

In the same manner, the media stream in a speech call is produced and controlled not by TAPI, but by one human talking to another. However, the line on which that call is established and monitored, and the call itself, remain in control of the TAPI application. Generally, an API (such as the Comm API) is associated with a particular media stream. For example, the WAV API can handle the voice stream that is transmitted over a TAPI-controlled line.

µ §

Other APIs complement the functionality provided by TAPI to control voice and data communications.

### Special Hardware Needed for Full Functionality

Some of the more advanced capabilities of TAPI require that an application be able, for example, to retrieve data from telephones. Currently, most telephones cannot be connected directly to computers, and thus are incapable of supporting Telephony functions beyond the passive role they play in POTS. In the future, users will install and configure telephone sets like other peripheral devices. They will be accompanied by cards that control their information flow and respond to transmission or monitoring requests by TAPI and data-transmission APIs.

## Independent Functioning

TAPI.DLL does not know how requests are carried out. This action is up to the implementation offered by the service provider. The TSPI is independent of both the telephone network that performs the actual switching (analog, PBX, ISDN, wireless, and so forth) and the connectivity scheme used to link the PC to the telephony world.

### Connectivity Examples

These models are reflected in a variety of physical arrangements, or connectivity schemes, between the desktop and the telephone network. The following examples show a selection of configurations that could be supported, or modeled, by a service provider.

#### Example 1: The Phone-centric Model

A single POTS line in the phone-centric or PC-centric connection models would, in the most straightforward manner, be modeled as a single line device with one channel.

The illustration below shows a phone-centric connection. The PC is connected to the switch through the desktop phone set. Such phone sets typically connect to the PC through one of its serial ports. The functions of TAPI are first channeled through TAPI.DLL and converted into telephony commands by the service provider. These commands ,which are often based on the Hayes AT command set (ANSI/TIA/EIA-602), are then sent over the serial connection to the telephone.

µ §

Example 1: The phone-centric connection model

A limitation of the above model is that it generally provides control only, offering no media-stream access to the PC.

#### Example 2: The PC-Centric Model

The second example uses a PC add-in card (which could also be an external add-on box) that is connected to both the switch and the phone set.

µ §

Example 2: The PC-centric connection model

Benefits of the PC-centric model are ease of integrating modem and fax functions and the possibility of using the telephone as an audio I/O device.

#### Example 3: BRI-ISDN Models

A single BRI-ISDN line could fit both the phone-centric or PC-centric models. The service provider can treat this situation in a number of ways, some of which are described below:

·	Model the BRI line as a single line device with a pool of two channels, allowing both channels to be combined for establishing 128 Kbps calls.

·	Model each B-channel as a separate line device and disallow the combination of the two channels into a single 128 Kbps channel.

·	Model the BRI connection as two separate line devices, each drawing up to 2 channels from a shared pool of 2 B-channels.

·	Model as three line devices: one for each of the two B-channels and one for the combination.

In the latter two models, resources may be assigned to different line devices at different times.

#### Example 4: A Client-server Network (LAN)

In client-server networks, a pool of telephone ports attached to a server may be shared among multiple client PCs via the LAN. The ports may be configured to let a maximum number of line devices (the quota) be assigned to each client workstation. It is not unusual for the sum of all quotas to exceed the total number of lines.

Also, the assignment of lines (via ports) is dynamic. For example, a given client PC with a quota of 2 may use ports 1 and 2 at one time and ports 7 and 11 at a later time.

The service provider for the pool may model this arrangement by providing each client workstation access to two line devices. This implies that the (fixed) device IDs for each client are 0 and 1. If the application later calls lineGetDevCaps for device 0 and again for device 1, it can assume that the device capabilities for each device is constant, since that is the Windows device model. For server-based devices that are pooled as described in the example above, this holds only for line devices that have identical device capabilities.

#### Example 5: LAN with Multiple Connections to the Switch

This example shows a LAN-based server with multiple telephone-line connections to the switch. SPI operations invoked at any of the client PCs are forwarded over the LAN to the server. The server uses third-party call control between the server and the switch to implement the client's call-control requests.

µ §

Example 5: LAN-based server with multiple connections to the switch

A potential advantage of this model is a lowered cost per PC (if the LAN is already in use), but a limitation would be the lack of media-stream access by the PCs.

#### Example 6: LAN with a Switch-to-host Link

This example shows a LAN-based host connected to the switch via a switch-to-host link. SPI operations invoked at any of the client PCs are forwarded over the LAN to the host. The host uses a third-party switch-to-host link protocol to implement the client's call-control requests.

µ §

Example 6: LAN-based server connected to the switch with a switch-to-host link

#### Example 7: Wireless Connection

The PC in use need not be a desktop PC. It can also be a notebook or portable PC connected to the telephone network via a wireless connection. This scheme is illustrated in the example below:

µ §

Example 7: A wireless connection

#### Example 8: Shared Telephony Connections

Finally, TAPI.DLL and its clients should be aware that the PC's connection may be shared by other telephony equipment, such as the telephone set shown below. For an application to operate properly in this arrangement, TAPI.DLL, its client, and the service provider must not assume that there are no other active agents on the line.

µ §

Example 8: The PC's connection to the switch is shared by other telephony equipment

The above examples are different ways that a service provider may choose to implement the mapping of line-device behavior onto a physical realization, and should not be construed as representing the only ways to provide PC-to-telephony connectivity.

Some of the hardware required by some of these schemes is not yet widely available.

Note

Most POTS calls are now transmitted digitally except while in the local loop, the part of the telephone network between the telephone and the telephone company's central switching office.

ISDN lines of the BRI (Basic Rate Interface) standard provide two 64-Kbps "B" channels (B channels carry voice or data) and one 16-Kbps "D" channel (D channels carry signalling information or packet data). The PRI (Primary Rate Interface) standard for the U.S., Canada, and Japan has twenty-three 64-Kbps B channels and one 64-Kbps D channel. The European PRI standard offers thirty B channels and two D channels.

Basic, Supplementary, and Extended Telephony Services are described later in this chapter.

Please note that voice consists of any signal that can travel over a 3.1 kHz-bandwidth channel, whereas speech refers exclusively to human speech.