Chapter 1

General and Syntactic Comments

Video Cassette Recorder Commands for MCI

In the following text, a bold font indicates ASCII strings that must be typed precisely as shown (except case is ignored). An italic font indicates information supplied by the user.

# Device Driver Instances

Like other MCI devices, a video-cassette-recorder (VCR) device is opened using the open command. If necessary, this loads the device driver into memory. Each open command creates an instance of the device driver. The device name acts as a handle to the instance of the device driver created. A device-driver instance (or more simply, device instance) is simply the collection of status information that describes the net result of all the operations that have been performed using a single device name. Commands use the device name so that they can be issued in a specific context.

The VCR device class lets applications open multiple device instances simultaneously. Each instance has its own context. For example, an application might operate two VCRs, one playing and the other recording. Each device must be opened with a unique device name. (MCI returns an error when an application attempts a second open with the same device name.)

Hardware or software constraints of VCR devices can limit the number of simultaneously open device instances, or what they can do simultaneously. Application programmers can determine the device constraints by using the capability, info version, and info product commands, and by using the test flag with a command.

## A Typical VCR

The VCR device category displays a wide range of devices, including "low end" home machines (VCRs), as well as high-end devices including professional editing equipment (VTRs). The following illustration identifies the inputs and outputs of a “typical” VCR that includes a base set of features.

In the following illustration, arrows on the left side indicate external inputs that connect to the VCR. Inputs that are surrounded by a box (tuner and timecode) are internal inputs that need no external connections. Arrows on the right side indicate external outputs that can connect to playback and capture equipment.

µ §

This VCR supports three input types: video, audio, and timecode. The video inputs include two standard channels (line 1 and line 2), an SVideo channel, an auxiliary channel, and a channel from an internal tuner. The audio inputs include two standard channels (line 1 and line 2) and a channel from an internal tuner. The timecode input is internal to the VCR.

The normal outputs carry the currently selected inputs when the VCR is recording or when the transport is stopped, and carry the contents of the videotape when the tape transport is playing or paused. The monitored outputs carry the same information as the normal outputs plus current timecode and channel information.

The back of your VCR might have several output plugs, but usually the same signal will appear on all of them. In this way, they can be viewed as a single logical output.

Note

The MCI VCR command set also supports a special input type for audio and video called “mute.” Mute allows the selection of “no input,” which is useful when recording a blank signal.

# Controlling the Tape Transport

The tape transport mechansim moves the tape in a videocasette forwards or backwards for playing and positioning. MCI VCR commands set includes several transport commands to control the tape transport (and the videocassette loaded in it) for playing, recording, seeking, pausing, stopping, and stepping.

Transport commands are special in that only one transport command can run at any given time on a single VCR. If another transport command is started before the current command completes, the current command will either be aborted or superseded. A command is aborted if it cannot be carried to completion as originally specified. A command is superseded if another command is issued that is the same as the currently running command.

The seek, play, cue, and freeze commands all leave the VCR in a paused mode when they complete.

## Setting Other Parts of a VCR

In addition to the tape transport commands, the VCR command set includes commands to control other portions of a VCR, such as video channels, audio channels, and tuner channels used for playing and recording. The set commands control the state of these items and can be issued at any time.

The set commands are synchronous, and notifications from these commands don’t interfere with outstanding (not yet received) transport-command notifications, providing that the notifications are not performed from the same driver instance. Each driver instance can have at most one pending notification. For example, you can issue the settuner channel number notify command while the VCR is performing a seek command without aborting or superseding the seek command.

The VCR set commands do the following:

| Command | Description |
| --- | --- |
| set | Sets the state of the following items: clock, time mode, counter format, door, recording format, speed, tape length, index, counter, power, preroll duration, postroll duration, pause timeout, tracking, and assemble recording. |
| setaudio | Sets the audio channel for playing, recording, and monitoring. |
| settimecode | Sets the timecode state for recording or playing, or turns off timecode monitoring. |
| settuner | Sets the VCR tuner(s) to specific channels. |
| setvideo | Sets the video channel for playing, recording, and monitoring. |

## Retrieving VCR Status Information

A third group of commands, status commands, retrieve information about the VCR. The status commands must be synchronous commands, and notifications from the status commands must not interfere with outstanding (not-yet received) transport-command notifications from the same driver instance. Each driver instance can have at most one pending notification. For example, you can issue the status position notify command while the VCR is seeking without aborting or superseding the seek command.

The status commands do the following:

| Command | Description |
| --- | --- |
| capability | Reports capabilities found in a specific VCR, including the following. detect length, eject, freeze, monitor sources, play preroll, preview, record, reverse, save, test, device type, has audio, has clock has timecode, video, maximum number of marks, and seek accuracy. |
| status | Can report the state of any of the following items: assemble record, audio monitor, audio monitor number, audio record, audio source, audio source number, channel, clock, clock ID, clock increment rate, counter, counter format, counter resolution, current track, frame rate, index, index on, length, media present, media type, mode, number of tracks, number of audio tracks, number of video tracks, play format, position, power, preroll duration, postroll duration, pause timeout, ready to receive a play command, record format, speed, time format, time mode, time type, timecode present, timecode record, timecode type, video monitor, video monitor number, video record, video source, video source number, write protected. |
| list | Reports the number and type of all video and audio input channels. |

## Searching and Positioning with a VCR

The VCR command set supports two methods of tracking videotape movement within the VCR tape transport: using timecode information and using tape counters.

Timecode information is timing information that has been recorded on videotape, in addition to the audio and video information. Timecode information is recorded as a non-negative increasing value. Most VCRs allow timecode to be recorded on a tape without destroying audio and video tracks.

The alternative to using timecode information is using tape counters. Tape counters estimate the footage of videotape that travels past the tape head to obtain a position.

Both timecode information and tape counters increase as the tape moves from beginning to end. Because of its accuracy, using timecode information to position a videotape is almost always preferable to using tape counters. Your driver implementation of the VCR command set should support both timecode information and tape counters, if possible.

The MCI command parameters for specifying positioning information are expressed as time dependencies: time format, duration, from, to, and seek. (Also, the status position command returns its time value in the current time format.)

The set time mode command controls the type of positioning to use with a videotape. When the time mode is set to timecode, the status position and set time format commands use the timecode on the tape. When the time mode is set to counter, the status position and set time format commands use counters.

If an application doesn’t care that there might be two sources of position information (such as the Microsoft Media Player), it can set the time mode to detect. When in detect mode, your MCI VCR driver should use timecode information for positioning when any of the following conditions occur:

·	If the timecode is present when the driver is opened.

·	If you change a tape with the set door open command and have timecode present.

·	If the set time mode command is re-issued.

If timecode cannot be found, your VCR driver should use tape counters to determine the tape position.

The status time type command returns the current positioning method in use and returns timecode or counter. Your VCR driver should also support the status time mode command to identify the current positioning mode, and return “timecode,” “counter,” or “detect.”

The status counter command retrieves the current tape counter value, regardless of the current positioning method. The However, you can only use this counter reading with the set counter command.

Together, the status timecode type and status frame rate commands of the VCR command set retrieve the native timecode format recorded on a videotape. For example, if the native timecode format recorded on the tape is smpte 25, retrieving the timecode type returns the value “smpte” and retrieving the frame rate returns the value “25”.

The status counter resolution command of VCR command must return the counter resolution as “seconds” or “frames.” The counter format might still be set to smpte 30, but the return value will only return a frame of 0. If the current time type is counter, then this resolution will apply to the value returned by status position.

## Capturing Frames

Frame-capturing commands provide still images for a frame-capture device. The VCR command set includes the freeze command to stabilize a still image for capturing. It also includes the unfreeze command to resume displaying normal output.

The freeze command provides a high-quality image for a frame-capture device. A high-quality image is stable, non-jittery, and correct from a timing perspective. A frame-capture device is a separate piece of hardware capable of “reading” and storing the video image. The justification for the freeze command is that a device might to not always deliver its maximum-quality output image and that such an image is not suitable for capturing.

## Selecting Inputs

The setvideo and setaudio commands of the VCR command set the VCR inputs to use in recording if the appropriate external inputs are connected to your VCR. For example, to record or view from the “svideo” video and the “line 1” audio inputs, use setvideo and setaudio to select these input sources. You can verify these selections by using the status command.

By default, the monitor shows exactly what appears as the output. However, sometimes you might want to “view” one source, while recording from another. This is a common practice using the tuner. For example, you might want to watch channel 4 while you record channel 7. In this case, you have two “logical” tuner inputs. You could set up the VCR by using the following commands:

1.	Use the settuner command to select the channels to watch and record.

2.	Use the setvideo command select the video-recording source.

3.	Use the setaudio command to select the audio-recording source.

4.	Use the setvideo command to route the channel-4 video input to the monitored output to display it on screen.

5.	Use the setaudio command to route the channel-4 audio input to the monitored output to play the audio.

6.	Verify your selections by using the status command.

Your implementation of a VCR driver might also support a special input type for audio and video called “mute.” that allows the selection of “no input,” to record a blank signal.

Note

## Selecting Recording Tracks

The recording tracks of a videotape are of three types: video, audio, and timecode. The general use of the tracks on a videotape include the following:

·	One video track

·	One or more audio tracks

·	One timecode track

When recording audio onto multiple tracks, use the following conventions:

·	Track 1 is always the main audio track

·	Track 2 is the second track

Your implementation of the VCR command set might support two operating modes: assemble and insert. In assemble mode, all tracks are selected to be recorded. In insert mode, tracks can be independently selected for recording. The default mode is “assemble record on.”

## Recording and Editing

If you expect to edit the video content while simultaneously operating multiple decks, you should include the cue command in your implementation of the VCR command set.

The cue command prepares the VCR for recording or playing. In MCI terms, recording is expressed as “input,” and playing is expressed as “output.” Therefore, “cue input” means prepare for recording. The cue command is separate from play and record because an application must know when a VCR is ready to perform the command (and because it can take several minutes to prepare for a play or record command). To cue effectively, follow the cue command with either the play command with the at flag or the  record command with the at and preview flags. Using other flags with the play and record commands should negate the effects of the cue command.

The VCR should prepare itself for recording or playing by seeking to the “in-point,” specified by the from parameter in the cue command. However, if the preroll flag is used (use can preroll to check for support), the VCR should position itself the preroll distance from the “in-point.” The preroll flag also indicates that the VCR should use any applicable “editing mode”.

When recording with “from” and “to” positions, include the “from” position in the edit, but not the “to” position.

Note

## Using the Clock While Editing

When editing, you might want to record specific segments from one VCR to another. Both VCRs must use the same clock for this type of editing; the clock helps synchronize both devices. The clock, as a shared resource, can be connected to multiple VCRs.

To begin recording at a specific time and position on one VCR when another VCR begins playing at the same time and position, you must specify the following items for each VCR:

·	An action (play or record)

·	A position

·	A time

You can determine if two VCRs share the same clock by using the capability command with the clock id parameter to query each VCR. If the IDs (identification numbers) returned from the status command are the same, they use the same clock.

You can also determine clock resolution by using the status command with the clock increment rate parameter. This command returns the number of increments the clock supports per second. For example, if the clock is updated every millisecond, 1000 is returned as the clock increment rate. You can reset the clock by using the set command with the clock 0 (zero) parameter.

When issuing a play, record, or seek command, you can specify when the command is to be executed. Exactly when to start each VCR involved in the edit depends on the characteristics of the VCRs being used. The timing must account for the amount of preroll each device requires and the amount of time needed to complete the MCI commands used to set up the edit session. The basic technique consists of the following steps:

1.	Cue the record device using the wait or notify flag.

2.	Cue the play device using the wait or notify flag.

3.	Retrieve the clock time.

4.	Issue play at for the device providing the recording content.

5.	Issue record at for the recording device with the same time as in the previous step.

Verify that both VCR devices are running from the same clock. You can use the status clock id command to query each device. If the return values for both devices are identical, they share the same clock.

Notes

Also, before using time values from the clock, be sure to establish the clock increment rate.