Chapter 9

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Programming with Phone Devices

Using Full Telephony is, in essence, using its two device classes: the line device class and the phone device class. Any physical telephonic device recognized by TAPI can be abstracted into one of these two classes, and can thus be treated in a consistent and reliable manner. Please note that none of Telephony's phone functions are accessible through Simple Telephony.

This chapter describes the role that TAPI's phone functions play. Because there are many functions that work similarly for both for line devices and phone devices, you will notice similarities between the material in this chapter and descriptions of line devices in earlier chapters.

You may wish to refer to the chapter The TAPI Programming Model, which explains the strengths of abstracting devices into classes, one of which is the phone device class.

# The Phone Device

TAPI defines a device that supports the phone device class as a device that contains some or all of the following elements (or additional elements):

·	Hookswitch/Transducer  This is a means for audio input and output. The Telephony API recognizes that a phone device may have several transducers, which can be activated and deactivated (taken offhook or placed onhook) under application or manual user control. TAPI identifies three types of hookswitch devices common to many phone sets:

·	Handset  The traditional mouth-and-ear piece combination that must be manually lifted from a cradle and pressed against the user's ear.

·	Speakerphone  Enables the user to conduct calls hands-free. The hookswitch state of a speakerphone can usually be changed both manually and by the API. The speakerphone may be internal or external to the phone device. The speaker part of a speakerphone allows multiple listeners.

·	Headset  Enables the user to conduct calls hands-free. The hook switch state of a headset can usually be changed both manually and by the API.

A hookswitch must be offhook to allow audio data to be sent to and/or received by the corresponding transducer.

·	Volume Control/Gain Control/Mute  Each hookswitch device is the pairing of a speaker and a mic component. The API provides for volume control and muting of speaker components and for gain control or muting of mic components.

·	Ringer  This is a means for alerting human users, usually a bell of sorts. A phone device may be able to ring in a variety of modes or patterns.

·	Display  This is a mechanism for visually presenting messages to the user. A phone display is characterized by its number of rows and columns.

·	Buttons  This is an array of buttons. Whenever the user presses a button on the phone set, the API reports that the corresponding button was pressed. Button-Lamp IDs identify a button and lamp pair. Of course, it is possible to have button-lamp pairs with either no button or no lamp. Button-lamp IDs are integer values that range from 0 to the maximum number of button-lamps available on the phone device, minus one. Each button belongs to a button class. Classes include call appearance buttons, feature buttons, keypad buttons, and local buttons.

·	Lamps  This is an array of lamps (such as LEDs) individually controllable from the API. Lamps can be lit in different modes by varying the on and off frequency. The button-lamp ID identifies the lamp.

·	Data Areas  These are memory areas in the phone device where instruction code or data can be downloaded to and/or uploaded from. The downloaded information would affect the behavior (or in other words, program) the phone device.

TAPI allows an application to monitor and/or control elements of the phone device. The most useful elements for an application are probably the hookswitch devices. The phone set can act as an audio I/O device (to the PC) with volume control, gain control and mute, a ringer (for alerting the user), data areas (for programming the phone), and perhaps a display, though the PC's display is more capable. The application writer is discouraged from directly controlling or using phone lamps or phone buttons, since lamp and button capabilities can vary widely among phone sets, and applications can quickly become tailored to specific phone sets.

There is no guaranteed core set of services supported by all phone devices as there is for line devices (the Basic Telephony services). Therefore, before an application can use a phone device, the application must first determine the exact capabilities of the phone device. Telephony capability varies with the configuration (client versus client-server), the telephone hardware and service-provider software. Applications should make no assumptions as to what telephony capabilities are available. An application determines the device capabilities of a phone device by calling the function phoneGetDevCaps.

A phone's device capabilities indicate which of these elements exist for each phone device present in the system and what their capabilities are. Although strongly oriented towards real-life telephone sets, this abstraction can provide a meaningful implementation (or subset thereof) for other devices as well. Take as an example a separate headset directly connected and controllable from the PC and operated as a phone device. Hookswitch changes can be triggered by detection of voice energy (offhook) or a period of silence (onhook); ringing can be emulated by the generation of an audible signal into the headset; a display can be emulated via text-to-speech conversion.

Note that a phone device need not be realized in hardware, but can instead be emulated in software, using a mouse or keyboard driven graphical command interface and the PC's speaker or sound system. Such a "soft phone" can be an application of TAPI. It can also be a service provider, which can be listed as a phone device available to other applications via the API¾it is assigned a phone device ID.

Depending on the environment and configuration, phone sets can be shared devices between the application and the switch. Some minor provision is made in the API where the switch may temporarily suspend the API's control of a phone device (see further).

# Phone Device Functions

The remainder of this chapter describes TAPI's phone functions. For convenience, they are grouped into general functional categories.

## Initialization and Shutdown of Phone Functions

For an application to use any of TAPI's 30 supplementary phone functions, it needs a connection to TAPI.DLL, through which it can receive messages. The application establishes this connection with a process known as initialization, using either the lineInitialize or, specifically for phone functions, the phoneInitialize function. One of the parameters passed to TAPI.DLL in these function calls is a pointer to the application's callback function. After initialization, TAPI uses this pointer to notify the application of the asynchronous completion of functions.

The phoneInitialize function returns two pieces of information to the application: an application handle, and the number of phone devices. The application handle represents the application's usage of TAPI. The TAPI functions that use phone handles do not require the application handle, as this handle is derived from the specified phone handle.

The second piece of information returned by phoneInitialize is the number of phone devices available to the application via the Telephony API. Phone devices are identified by their device identifier (device ID). Valid device IDs range from zero to the number of phone devices - 1. For example, if phoneInitialize reports that there are two phone devices in a system, then valid phone device IDs are 0 and 1. Once an application is finished using the phone functions of TAPI, it invokes phoneShutdown, passing its application handle to shut down its usage of TAPI. This allows TAPI to free any resources assigned to the application.

Both phoneInitialize and phoneShutdown operate synchronously. That is, these functions either return a success or failure indication, and never return an asynchronous Request ID.

## Opening and Closing Phone Devices

After determining the capabilities of a phone device, an application must open the device before it can access functions on that phone. After a phone device has been successfully opened, the application is returned a handle representing the open phone. A phone device can be opened in different modes, thus providing a structured way of sharing a phone device, as describer later.

The function phoneOpen opens the specified phone device in order to provide the application with access to functions on the phone. A phone device is identified to phoneOpen by means of its device ID, passed as the parameter dwDeviceID.

#### Operating Modes and Privileges

The application can request one of two operating modes when opening a phone device. These modes reflect the privileges the application requests for the device:

·	Opening a phone for monitor privileges lets the application determine the status of the phone device. Messages are sent to the application's callback function when status changes on the phone device are detected.

·	An application that opens a phone device for owner privileges can use operations that modify the state of the phone device. Owner privilege automatically includes full monitor privileges as well. At any time, a given phone device can be open only once for owner privileges, but multiple times for monitor privileges. All phoneSet operations require owner privileges, while all phoneGet operations require only monitor privileges.

#### Device IDs

Other TAPI phone functions use a handle to an open phone device to identify the open phone device. The only functions for phone devices that take a phone device ID parameter (as opposed to a phone handle) are the phoneGetDevCaps and phoneOpen functions.

An application can retrieve the phone's device ID by using the function phoneGetID with the phone handle as a parameter.

An application can also obtain device IDs for various Windows device classes associated with an opened phone by invoking phoneGetID. This function takes a phone handle and a device class description. It returns the device ID for the device of the given device class that is associated with the open phone device. If the device class is phone, the device ID of the phone device is returned. If the device class is mci waveaudio, the device ID of an mci waveaudio device is returned (if supported) that allows such functions as recording and playback of audio over the phone.

To subsequently open a phone device for audio playback using the waveform API, an application calls the waveOutOpen function. The implementation of the waveOutOpen call is device-specific, and each implementation has a number of options for implementing this function. If the phone's handset is on hook when invoking the function, for example, the waveOutOpen driver may return an error code message to the application that is further mapped in a request (dialog box) to the user to "pick up the phone." This is similar to requesting a CD to play back audio when no compact disc is in the player, and the existing definition of the waveform APIs can deal with it.

In contrast with line devices, for which the basic line services provide the equivalent of POTS, no minimum guaranteed set of functions is defined for phone devices. While each phone device provides at least the functions and messages described in this section, they do not offer any actual operations on the physical phone device.

#### Closing the Phone Device

The function phoneClose closes the specified phone device. After the open phone device has been successfully closed, the implementation sends a PHONE_CLOSE message to the application. Note that these messages can also be sent unsolicited as a result the phone device being reclaimed in some other manner. An application should therefore be prepared to handle unsolicited PHONE_CLOSE messages. At the time the phone device is closed, any outstanding asynchronous replies pertaining to that device are suppressed.

## Hookswitch Devices

A phone device may have multiple hookswitch devices. The Telephony API defines three types of hookswitch devices for a phone: handset, speakerphone, and headset. Each hookswitch device has a speaker and a mic component, and operates in one of four hookswitch modes:

·	Onhook  The hookswitch device is onhook, and both its mic and speaker are disabled.

·	Mic only  The hookswitch device is offhook, its mic is enabled, and its speaker is mute.

·	Speaker only  The hookswitch device is offhook, its mic is mute, and its speaker is enabled.

·	Mic and speaker  The hookswitch device is offhook, and both mic and speaker are enabled.

AThis function phoneSetHookSwitch is used to set the hookswitch mode of one or more of the hookswitch devices of an open phone device. For example, to mute or unmute the mic or speaker component of a hookswitch device, use phoneSetHookSwitch with the appropriate hookswitch mode. The function phoneGetHookSwitch can be used to query the hookswitch mode of a hookswitch device of an open phone device.

When the mode of a phone's hookswitch device is changed manually, for example by lifting the handset from its cradle, a PHONE_STATUS message is sent to the application's callback to notify the application about the state change. Parameters to this message provide an indication of the change.

The volume of the speaker component of a hookswitch device can be set with the function phoneSetVolume. Note that volume setting is different from mute in that muting a speaker and later unmuting it will preserve the volume setting of the speaker. The function phoneGetVolume can be used to return the current volume setting of a hookswitch device's speaker of an open phone device.

The mic component of a hookswitch device can also be gain controlled. Note that gain setting is different from mute in that muting a mic and later unmuting it will preserve the gain setting of the mic. Use the function phoneSetGain to set the gain of a hookswitch device's mic of an open phone device, and the function phoneGetGain to return the gain setting of a hookswitch device's mic of an opened phone.

When the volume or gain of a phone's hookswitch device is changed, a PHONE_STATUS message is sent to the application's callback to notify the application about the state change. Parameters to this message provide an indication of the change.

## Display

The Telephony API provides access to a phone's display. The display is modeled as an alphanumeric area with rows and columns. A phone's device capabilities indicate the size of a phone's display as the number of rows and the number of columns. Both these numbers are zero if the phone device does not have a display. Use the function phoneSetDisplay to write information to the display of an open phone device, and phoneGetDisplay to have the current contents of a phone's display returned.

When the display of a phone device is changed, a PHONE_STATUS message is sent to the application's callback to notify the application about the state change. Parameters to this message provide an indication of the change.

## Ring

A single phone may be able to ring with different ring modes. Given the wide variety of ring modes available, ring modes are identified via their ring mode number. A ring mode number ranges from zero to the number of available ring modes minus one.

The functions an application would use to control a phone device's ring modes are phoneSetRing, which rings an open phone device according to a given ring mode, and phoneGetRing, which returns the current ring mode of an opened phone device.

When the ring mode of a phone device is changed, a PHONE_STATUS message is sent to the application's callback to notify the application about the state change. Parameters to this message provide an indication of the change.

## Buttons

The Telephony API models a phone's buttons and lamps as button-lamp pairs. A button with no lamp next to it or a lamp with no button is specified using a "dummy" indicator for the missing lamp or button. A button with multiple lamps is modeled by using multiple button-lamp pairs.

Information associated with a phone button can be set and retrieved. When a button is pressed, a PHONE_BUTTON message is sent to the application's callback function. The parameters of this message are a handle to the phone device and the button-lamp ID of the button that was pressed. The keypad buttons '0' through '9', '*', and '#" are assigned the fixed button-lamp IDs 0 through 11.

The functions associated with buttons are phoneSetButtonInfo, which sets the information associated with a button on a phone device, and phoneGetButtonInfo, which returns information associated with a button on a phone device. The message PHONE_BUTTON is sent to an application's callback when a button on the phone is pressed.

The information associated with a button does not carry any semantic meaning as far as TAPI is concerned. It is useful for (device-specific) applications that understand the meaning of this information for a given phone device, or for display to the user (such as online help).

## Lamps

The lamps on a phone device can be lit in a variety of different lighting modes. Unlike ringing patterns, lamp modes are more uniform across phone sets of different vendors. A common set of lamp modes is defined by the API. A lamp identified by its lamp-button ID can be lit in a given lamp mode. A lamp can also be queried for its current lamp mode.

The TAPI functions used for lamps are phoneSetLamp, which lights a lamp on a specified open phone device in a given lamp lighting mode, and phoneGetLamp, which returns the current lamp mode of the specified lamp.

When a lamp of a phone device is changed, a PHONE_STATUS message is sent to the application's callback function to notify the application about the state change. Parameters to this message provide an indication of the change.

## Data Areas

Some phone sets support the notion of downloading data from or uploading data to the phone device, which can allow the phone set to be programmed in a variety of ways. The Telephony API models these phone sets as having one or more download or upload areas. Each area is identified by a number that ranges from zero to the number of data areas available on the phone minus one. Sizes of each area may vary. The format of the data itself is device specific.

The TAPI function phoneSetData downloads a buffer of data to a given data area in the phone device, and the function phoneGetData uploads the contents of a given data area in the phone device to a buffer.

When a data area of a phone device is changed, a PHONE_STATUS message is sent to the application's callback to notify the application about the state change. Parameters to this message provide an indication of the change.

## Status

Most of the Get and Set operations described in the earlier sections of this chapter deal with one component of information only. The operation phoneGetStatus returns complete status information about a phone device to an application.

As mentioned earlier, whenever a status item changes on the phone device, a PHONE_STATUS message is sent to the application's callback function. This message's parameters include a handle to the phone device and an indication of the status item that changed.

An application can use phoneSetStatusMessages to select the specific status changes for which it wants to be notified via the callback. Correspondingly, the function phoneGetStatusMessages returns the status changes for which the application wants to be notified.

## Error Codes for Phone Devices

Most of the Telephony-API functions used for phone devices may return a negative error code. The only function that does not return an error code is the phoneGetNumDevs function. This function returns zero if no phones are present in the system, or if the function encounters an error.

## Extended-Telephony Phone Functions

The Extended Phone Services (or Device-Specific Phone Services) include all extensions to the Telephony API defined by the service provider. TAPI defines a mechanism that enables service-provider vendors to extend TAPI using device-specific extensions. TAPI defines only the extension mechanism, and by doing so provides access to device-specific extensions. The Telephony API does not define their behavior, which is completely defined by the service provider.

TAPI consists of scalar and bit-flag data constant definitions, data structures, functions, and callback messages. Procedures are defined that enable a vendor to extend most of these as follows:

#### Scalar Data Constants

For extensible scalar data constants, a service-provider vendor may define new values in a specified range. As most data constants are DWORDs, the range 0x00000000 through 0x7FFFFFFF is typically reserved for common future extensions, while 0x80000000 through 0xFFFFFFFF is available for vendor-specific extensions. The assumption is that a vendor would define values that are natural extensions of the data types defined by the API.

#### Bit-Flag Data Constants

For extensible bit-flag data constants, a service-provider vendor may define new values for specified bits. As most bit-flag constants are DWORDs, a specific number of the lower bits are usually reserved for common extensions, while the remaining upper bits are available for vendor-specific extensions. Common bit flags are assigned from bit zero up, and vendor-specific extensions should be assigned from bit 31 down. This scheme provides maximum flexibility in assigning bit positions to common extensions, as opposed to vendor-specific extensions. A vendor is expected to define new values that are natural extensions of the bit flags defined by the API.

Extensible data structures have a variably sized field that is reserved for device-specific use. Because the field is variably sized, the service provider decides the field's amount of information and interpretation. A vendor that defines a device-specific field is expected to make these natural extensions of the original data structure defined by the API.

#### Functions and Messages

The phoneDevSpecific operation and its associated PHONE_DEVSPECIFIC message enable an application to access device-specific phone features that are unavailable through the common Telephony services for phones. In other words, phoneDevSpecific is the device-specific escape function that allows vendor dependent extensions, and PHONE_DEVSPECIFIC is the device-specific message that is sent to the application's callback function.

The parameter profile of the phoneDevSpecific function is generic in that little interpretation of the parameters is made by the Telephony API. Rather, the interpretation of the parameters is defined by the service provider and must be understood by an application that uses them. The parameters are simply passed through by the Telephony DLL from the application to the service provider. An application that relies on device-specific extensions will not generally work with other service providers, but applications written to the common telephony phone services will work with the extended service provider.

A hookswitch is the switch that connects or disconnects a device from the phone line. On a telephone, for example, this is the switch that is automatically activated when a user lifts the receiver from the cradle to get a new dial tone.