Chapter 5

Please note: The information contained in this documentation is preliminary and subject to change without notice.

The Phone Device Class

## Basic Phone Services

### Phone SPI Initialization

As parts of the phone device abstraction defined by the TSPI, TAPI.DLL and the service provider must first undergo basic initialization. The first of these steps is interface-version negotiation, which TAPI.DLL performs by calling the TSPI_phoneNegotiateTSPIVersion function. This function is normally used to negotiate on behalf of an individual phone device because different phone devices within the same service provider may operate according to different interface versions. TAPI.DLL passes a special reserved device ID value, INITIALIZE_NEGOTIATION, to indicate that it is negotiating an overall interface version for early initialization functions that affect all phone devices.

The result of this negotiation is passed to subsequent procedures until a phone device is opened. At that time, the phone device becomes committed to a particular interface version. This interface version is implicit until the phone is closed, and does not need to be passed to subsequent functions that operate on an opened phone.

Another minor function that may be called early in initialization is TSPI_phoneGetProcTable. This procedure retrieves pointers to all of the phone-related procedures defined by the TSPI and implemented by the service provider. In principle, TAPI.DLL should load all of these procedure pointers by name using the usual library-management procedures supplied by Windows to do so. In fact, the service provider is required to EXPORT all of its entry point procedures to support such an approach. In practice, this may be a very slow operation, so TSPI_phoneGetProcTable is supplied as an alternate optimization.

The next major step in initialization is TSPI_phoneSetDeviceIDBase. Devices of the Telephony "phone" device class are identified by integers starting from zero. This range of identifiers is contiguous across the full range of phone devices. Since there may be multiple service providers managing phone devices in a single system, each service provider gets a contiguous portion of the total range. The TSPI_phoneSetDeviceIDBase function tells the service provider the lowest value in its portion of the range. The service provider, rather than TAPI.DLL, has the responsibility for mapping this variable range to its own internal device identifiers. This gives the service-provider vendor sufficient flexibility to use device IDs in device-specific extensions if it so desires. Since the service provider "knows" what device IDs appear in the TAPI-defined parameters and data structures, it can use consistent device IDs in device-specific extension parameters and data structures.

All of the above functions operate strictly asynchronously.

### Phones

The Telephony SPI defines a device that supports the phone device class as one containing some or all of the following elements (or additional elements):

·	Hookswitch/Transducer  this is a means for audio input and output. The SPI recognizes that a phone device may have several transducers, which can be activated and deactivated (taken offhook, placed onhook) under application or manual user control. The Telephony SPI identifies three types of hookswitch devices common to many phone sets:

·	Handset  is the traditional mouth-and-ear piece combination that must be manually lifted from a cradle and pressed against the user's ear.

·	Speakerphone  enables the user to conduct calls hands-free. The hookswitch state of a speakerphone can usually be changed both manually and by the SPI. The speakerphone may be internal or external to the phone device. The speaker part of a speakerphone allows multiple listeners.

·	Headset  enables the user to conduct calls hands-free. The hook switch state of a headset can usually be changed both manually and by the SPI.

A hookswitch must be offhook to allow audio data to be sent to and/or received by the corresponding transducer.

·	Volume Control/Gain Control/Mute  each hookswitch device is the pairing of a speaker and a mic component. The SPI provides for volume control/mute of speaker components and for gain control/mute of mic components.

·	Ringer  this is a means for alerting human users, usually a bell of sorts. A phone device may be able to ring in a variety of modes or patterns.

·	Display  this is a mechanism for visually presenting messages to the user. A phone display is characterized by its number of rows and columns.

·	Buttons  this is an array of buttons. Whenever the user presses a button on the phone set, the SPI reports that the corresponding button was pressed. Button/Lamp IDs identify a button and lamp pair. Of course, it is possible to have button/lamp pairs with either no button or no lamp. Button/lamp IDs are integer values that range from 0 to the maximum number of button/lamps available on the phone device, minus one. Each button belongs to a button class. Classes include call appearance buttons, feature buttons, keypad buttons, and local buttons.

·	Lamps  this is an array of lamps (such as LEDs) individually controllable from the SPI. Lamps can be lit in different modes by varying the on and off frequency. The button/lamp ID identifies the lamp.

·	Data Areas  these are memory areas in the phone device where instruction code or data can be downloaded to and/or uploaded from. The downloaded information would affect the behavior (or in other words, program) the phone device.

The Telephony SPI allows TAPI.DLL to monitor and/or control elements of the phone device on behalf of its client applications. Probably the most useful elements for an application to use are the hookswitch devices (as when the phone set is used as an audio I/O device to the PC) with volume control, gain control and mute, the ringer (for alerting the user), the data areas (for programming the phone), and perhaps the display (the PC's display is certainly a lot more capable). The application writer is discouraged from directly controlling or using phone lamps or phone buttons, since lamp and button capabilities can vary widely among phone sets, and applications can quickly become tailored to specific phone sets.

There is no guaranteed core set of services supported by all phone devices as there is for line devices with the Basic Telephony Services. Therefore, before an application can use a phone device, the application must first determine the exact capabilities of the phone device. Telephony capability varies with the configuration (client versus client/server, for example), the telephone hardware, and the service-provider software. Applications should make no assumptions as to what telephony capabilities are available. TAPI.DLL determines a phone's device capabilities on behalf of an application via TSPI_phoneGetDevCaps.

A phone's device capabilities indicate which of these elements exist for each phone device present in the system and what their capabilities are. Although strongly oriented towards real-life telephone sets, this abstraction can provide a meaningful implementation (or subset thereof) for other devices as well. Take as an example a separate headset directly connected and controllable from the PC and operated as a phone device. Hookswitch changes can be triggered by detection of voice energy (offhook) or a period of silence (onhook). Ringing can be emulated by the generation of an audible signal into the headset. A display can be emulated via text-to-speech conversion.

Note that a phone device need not be realized in hardware, but can instead be emulated in software, using a mouse- or keyboard-driven graphical command interface and the PC's speaker or sound system. Such a "soft phone" can be an application of the Telephony API. It can also be a service provider, which can be listed as a phone device available to applications via the SPI (that is, it is assigned a phone device ID).

Depending on the environment and configuration, phone sets can be shared devices between the application and the switch. Some minor provision is made in the SPI where the switch may temporarily suspend the SPI's control of a phone device (see further).

### Opening and Closing Phone Devices

After having performed basic initialization and possibly having retrieved device capabilities, TAPI.DLL must open the phone device before it can access functions on that phone. After a phone device has been successfully opened, TAPI.DLL is returned a handle representing the open phone. The function TSPI_phoneOpen opens the specified phone device prior to providing access to functions on the phone. TSPI_phoneClose closes a specified phone device.

A phone device is identified to TSPI_lineOpen by means of its device ID. This function returns a handle to an open phone. A handle to an open phone device is used in other operations to identify the open phone device. Note that the only functions on phone devices that take a phone device ID parameter are the early initialization functions such as TSPI_phoneGetDevCaps, TSPI_phoneNegotiateTSPIVersion, TSPI_phoneNegotiateExtVersion, TSPI_phoneSetDeviceIDBase, and TSPI_phoneOpen functions. All other functions take phone handles.

Given a phone handle, the phone's device ID can be retrieved via the function TSPI_phoneGetID.

TAPI.DLL's client application can obtain device IDs for various Windows device classes associated with an opened phone by invoking TSPI_phoneGetID via the TAPI function phoneGetID. This function takes a phone handle and a device class description. It returns the device ID for the device of the given device class that is associated with the open phone device. If the device class is phone, the device ID of the phone device is returned. If the device class is mci waveaudio, the device ID of an mci waveaudio device (if supported) is returned that allows the manipulation (such as recording or playback) of audio over the phone.

To subsequently open a phone device for audio playback using the waveform API, an application calls the waveOutOpen function. The implementation of the waveOutOpen call is device-specific, and each implementation has a number of options for implementing this function. If the phone's handset is on hook when invoking the function, for example, the waveOutOpen driver may return an error code message to the application that is further mapped in a request (dialog box) to the user to "pick up the phone." This is similar to requesting a CD to play back audio when no compact disc is in the player, and the existing definition of the waveform APIs can deal with it.

In contrast with line devices, for which the basic line services provide the equivalent of POTS, no minimum guaranteed set of functions is defined for phone devices. While each phone device provides at least the functions and messages listed in this section, they do not offer any true operations on the physical phone device.

## Supplementary Phone Services

### Hookswitch Devices

A phone device may have multiple hookswitch devices. The SPI defines three types of hookswitch devices for a phone: handset, speakerphone and headset. Each hookswitch device has a speaker and a mic component. Each hookswitch device is in one of four hookswitch modes:

·	Onhook  The hookswitch device is onhook, and both its mic and speaker are disabled.

·	Mic only  The hookswitch device is offhook, its mic is enabled and its speaker is mute.

·	Speaker only  The hookswitch device is offhook, its mic is mute and its speaker is enabled.

·	Mic and speaker  The hookswitch device is offhook, both mic and speaker are enabled.

TAPI.DLL can set and query hookswitch mode of an opened phone's hookswitch devices. To mute/unmute the mic or speaker component of a hookswitch device, the operation TSPI_phoneSetHookSwitch is used with the appropriate hookswitch mode. This function sets the hookswitch mode of one or more of the hookswitch devices of an open phone device. The function TSPI_phoneGetHookSwitch queries the hookswitch mode of a hookswitch device of an open phone device.

When the mode of a phone's hookswitch device is changed manually, for example by lifting the handset from its cradle, a PHONE_STATUS message is sent to TAPI.DLL's callback to notify it about the state change. Parameters to this message provide an indication of the change.

The speaker component of a hookswitch device can be volume controlled. Note that volume setting is different from mute in that muting a speaker and later unmuting it preserves the volume setting of the speaker. The function TSPI_phoneSetVolume sets the volume of a hookswitch device's speaker of an open phone device. TSPI_phoneGetVolume returns the volume setting of a hookswitch device's speaker of an open phone device.

The mic component of a hookswitch device can be gain controlled. Note that gain setting is different from mute in that muting a mic and later unmuting it preserves the gain setting of the mic. TSPI_phoneSetGain sets the gain of a hookswitch device's mic of an open phone device, and TSPI_phoneGetGain returns the gain setting of a hookswitch device's mic of an opened phone.

When the volume or gain of a phone's hookswitch device is changed, a PHONE_STATUS message is sent to TAPI.DLL's callback to notify TAPI.DLL about the state change. Parameters to this message provide an indication of the change.

### Display

The SPI provides access to a phone's display. The display is modeled as an alphanumeric area with rows and columns. A phone's device capabilities indicate the size of a phone's display as number of rows and number of columns. Both these number are zero if the phone device does not have a display.

The function TSPI_phoneSetDisplay writes information to the display of an open phone device. TSPI_phoneGetDisplay returns the current contents of a phone's display.

When the display of a phone device is changed, a PHONE_STATUS message is sent to TAPI.DLL's callback to notify it about the state change. Parameters to this message provide an indication of the change.

### Ring

A single phone may be able to ring with different ring modes. Given the wide variety of ring modes available, ring modes are identified via their ring mode number. A ring mode number ranges from zero to the number of available ring modes minus one.

TSPI_phoneSetRing rings an open phone device according to a given ring mode.TSPI_phoneGetRing returns the current ring mode of an opened phone device.

When the ring mode of a phone device is changed, a PHONE_STATUS message is sent to TAPI.DLL's callback to notify it about the state change. Parameters to this message provide an indication of the change.

### Buttons

The SPI models a phone's buttons and lamps as button/lamp pairs. A button with no lamp next to it, or a lamp with no button is specified using a "dummy" indicator for the lamp, resp. button. A button with multiple lamps is modeled by using multiple button/lamp pairs.

Information associated with a phone button can be set and retrieved. When a button is pressed, a PHONE_BUTTON message is sent to TAPI.DLL's callback. Parameters of this message are a handle to the phone device and the button/lamp ID of the button that was pressed. The keypad button '0' through '9', '*', and '#" are assigned fixed button/lamp IDs 0 through 11.

The function TSPI_phoneSetButtonInfo sets the information associated with a button on a phone device. TSPI_phoneGetButtonInfo returns information associated with a button on a phone device. PHONE_BUTTON is a message is sent to TAPI.DLL's callback when a button on the phone is pressed.

The information associated with a button does not carry any semantic meaning as far as the SPI is concerned. It is useful for device-specific applications that understand the meaning of this information for a given phone device, or for display to the user (such as in online help).

### Lamps

The lamps on a phone device can be lit in a variety of different lighting modes. Unlike ringing patterns, lamp modes are more uniform across phone sets of different vendors. A common set of lamp modes is defined by the SPI. A lamp identified by its lamp/button ID can be lit in a given lamp mode. A lamp can also be queried for its current lamp mode.

The lamp functions are TSPI_phoneSetLamp, which lights a lamp on a specified open phone device in a given lamp lighting mode, and TSPI_phoneGetLamp, which returns the current lamp mode of the specified lamp.

When a lamp of a phone device is changed, a PHONE_STATUS message is sent to TAPI.DLL's callback to notify it about the state change. Parameters to this message provide an indication of the change.

### Data Areas

Some phone sets support the notion of downloading or uploading data to or from the phone device. This may allow the phone set to be programmed in a variety of ways. The SPI models these phone sets as having one or more download and/or upload areas. Each area is identified by a number that ranges from zero to the number of data areas available on the phone minus one. Sizes of each area may vary. The format of the data itself is device specific.

The function TSPI_phoneSetData downloads a buffer of data to a given data area in the phone device. TSPI_phoneGetData uploads the contents of a given data area in the phone device to a buffer.

When a data area of a phone device is changed, a PHONE_STATUS message is sent to TAPI.DLL's callback to notify it about the state change. Parameters to this message provide an indication of the change.

### Status

Most of the Get and Set operations described earlier deal with one component of information only. The operation TSPI_phoneGetStatus allows TAPI.DLL to obtain complete status information about a phone device. As mentioned, whenever a status item changes on the phone device, a PHONE_STATUS message is sent to TAPI.DLL's callback. The message parameters include a handle to the phone device and an indication of the status item that changed. TAPI.DLL can use TSPI_phoneSetStatusMessages to select status changes for which it wants to be notified via the callback. The function TSPI_phoneGetStatusMessages returns the status changes for which TAPI.DLL wants to be notified.

## Extended Phone Services

Extended Phone Services (or Device Specific Phone Services) include all service-provider defined extensions to the SPI. The SPI defines a mechanism that enables service-provider vendors to extend the Telephony SPI using device-specific extensions. The SPI only defines the extension mechanism, and by doing so provides access to device-specific extensions, but the SPI does not define their behavior. Behavior is completely defined by the service provider.

The Telephony SPI consists of scalar and bit flag data constant definitions, data structures, functions, and callback messages. Procedures are defined that enable a vendor to extend most of these as follows:

For extensible scalar data constants, a service-provider vendor may define new values in a specified range. As most data constants are DWORDs, typically the range 0x00000000 through 0x7FFFFFFF is reserved for common future extensions, while 0x80000000 through 0xFFFFFFFF are available for vendor-specific extensions. The assumption is that a vendor would define values that are natural extensions of the data types defined by the SPI.

For extensible bit flag data constants, a service-provider vendor may define new values for specified bits. As most bit flag constants are DWORDs, typically a specific number of the lower bits are reserved for common extensions while the remaining upper bits are available for vendor-specific extensions. Common bit flags are assigned from bit zero up, and vendor-specific extensions should be assigned from bit 31 down. This provides maximum flexibility in assigning bit positions to common extensions versus vendor-specific extensions. A vendor is expected to define new values that are natural extensions of the bit flags defined by the SPI.

Extensible data structures have a variable sized field that is reserved for device-specific use. Being variable sized, the service provider decides the amount of information and the interpretation. A vendor that defines a device-specific field is expected to make these natural extensions of the original data structure defined by the SPI.

The TSPI_phoneDevSpecific operation and associated PHONE_DEVSPECIFIC message enable TAPI.DLL's client application to access device-specific phone features that are unavailable via the common Telephony Services on phones. The parameter profile of the TSPI_phoneDevSpecific function is generic in that little interpretation of the parameters is made by the SPI. Device-handle parameters have TSPI-defined meanings and are translated appropriately between the application and the service provider. Generic parameters are simply passed through unmodified. The interpretation of the generic parameters is defined by the service provider and must be understood by any applications that use them. An application that relies on device-specific extensions will not generally work with other service providers. However, applications written entirely to the common telephony phone services will work with the extended service provider.

Regarding handle translation, the pass-through nature of the generic parts of device-specific extensions has an imporant consequence. A service provider has no way to relate the handles used at the TSPI level to those at the TAPI level except by passing them through the predefined handle parameters and fields. Any handle put into the generic extension area is untranslated by TAPI.DLL as it is passed between application and service provider. The designer of a service-provider extension should generally not define extensions that pass handles in this way.

The appropriate approach when defining a device-specific extension that needs to refer to specific devices without using handles is to refer to them using their absolute device identification. The device ID used in opening a phone at the TAPI level, for example, is strictly the same value that is used at the TSPI level to open the phone. This is the motivation for the TSPI_phoneSetDeviceIDBase procedure. It adjusts the service provider's device ID numbering to be the same as the numbering used at the TAPI level. This gives the service-provider designer the flexibility to use device IDs to identify devices in device-specific extensions.