Appendix C

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Call States and Events

A connection is not fully established until both parties are communicating. To reach that point, the establishment of the call goes through several stages, as does the clearing (termination) of the call. A call's events cause it to transition through call states as it comes into existence, is used to exchange information, and terminates. These call-state transitions result from both solicited and unsolicited events. A solicited event is one caused by the application controlling the call (as when it invokes TAPI operations), while unsolicited events are caused by the switch, the telephone network, or the actions of the remote party. Some operations on line devices, addresses, and calls may first require that the line, address, or call upon which they operate be in certain specific states.

Different call states indicate that connections exist to different parts of the switch. For example, a dial tone is a particular state of a switch that means the PC is ready to receive digits.

Whenever a call changes state, TAPI reports the new state to the application in a callback. This programming model, therefore, is one in which the application reacts to the events reported to it, as opposed to a rigid call-state model. In other words, call-state notification tells the application what the call's new state is instead of reporting the transitions between two states.

Some of the call states and events defined by TAPI are exclusive to inbound or outbound call processing, while others occur in both cases. Several of these call states provide additional information that can be used by the application. For example, the busy state signifies that a call cannot be completed because a resource between the originator and the destination is unavailable, as when an intermediate switch has reached its capacity and cannot handle an additional call. Information supplied with the busy state includes station busy or trunk busy. Station busy means that the destination's station is busy (the phone is offhook), while trunk busy means that a circuit in the switch or network is busy. The call states defined by TAPI are listed below:

·	Idle  This corresponds to the "null" state:  No "activity exists" on the call, which means that no call is currently active.

·	Offering (inbound)  When the switch informs the PC of the arrival of a new incoming call, that call is in the offering state. Note that offering is not the same as causing a phone or PC to ring. When a call is offered, the PC is not necessarily instructed to alert the user.

·	Example: An incoming call on a shared call appearance is offered to all stations that share the appearance, but typically only the station that has the appearance as its primary address is instructed to ring. If that station does not answer after some amount of time, the bridging stations may be instructed to ring as well.

·	Accepted (inbound)  An application has taken responsibility for an incoming call. In ISDN, this has the side effect of alerting the users at both sides of the call: the caller's and the called party's. An incoming call can always be answered without first being accepted.

·	Dial tone (outbound)  The call is receiving a dial tone from the switch. The presence of a dial tone indicates that the switch is ready to receive a dialable number. Additional information includes:

— Normal dial tone  The "normal" or everyday dial tone, usually a continuous tone.

— Special dial tone  A special dial tone is often used to signal certain conditions such as message-waiting. This is usually an interrupted dial tone.

·	Dialing (outbound)  The originator is dialing digits on the call. The dialed digits are collected by the switch.

·	Proceeding (outbound)  The call is proceeding through the network. This occurs after dialing is complete and before the call reaches the dialed party, as indicated by ringback, busy, or answer.

·	Special info (outbound)  The call is receiving a special information signal. A special information signal precedes a prerecorded announcement indicating why a call cannot be completed. Such announcements can be of these types:

— No circuit  A no-circuit or emergency announcement.

— Customer irregularity  This typically means that the dialed number is not correct.

— Reorder  A reorder or equipment-irregularity announcement.

·	Busy (outbound)  The call is receiving a busy signal. Busy indicates that some resource is not available and the call cannot be normally completed at this time. Additional information consists of:

— Station busy  The station at the other end is off-hook.

— Trunk busy  The network is congested. This usually produces a "fast busy" signal.

·	Ringback (outbound)  The station to be called has been reached, and the destination's switch is generating a ring tone back to the originator. A ringback means that the destination address is being alerted to the call.

·	Connected (inbound and outbound)  Information is being exchanged over the call.

·	On hold (inbound and outbound)  The call is currently held by the switch. This frees up the physical line, which allows another call to use the line.

·	Conferenced (inbound and outbound)  The call is a member of a conference call and is logically in the connected state (to the conference bridge). A call in the conferenced state refers to a conference call (in the connected, onHold, ... state).

·	On hold pending conference (inbound and outbound)  The conference call is currently on hold and waiting for the user to add another party.

·	On hold pending transfer (inbound and outbound)  The call is on hold in preparation of being transferred.

·	Disconnected (inbound and outbound)  The call has been disconnected by the remote party.

·	Unknown (inbound and outbound)  The call exists, but its state is currently unknown. This may be the result of poor call progress detection.

Although under normal circumstances an outbound call is likely to transition from idle to connected via a number of intermediate states (dial tone, dialing, proceeding, ringback), other paths are often possible. For example, the ringback state may be skipped, as when a hot phone (or other non-dialed phone) transitions directly from idle to connected.

An application should always process call-state event notifications. Call-state transitions valid for one switch or configuration may be invalid for another. For example, consider a line from the switch that (via a simple Y-connector) physically terminates both at the PC and at a separate phone set, creating a party line configuration between the PC and the phone set. The PC termination and, therefore, the application using TAPI, may not know of the activities on the line handled by the phone set. That is, the line may be in use without the service provider being aware of it. An application that wants to make an outbound call will succeed in allocating a call appearance from the API, but ends up sharing the active call on the line. In this case, blindly sending a DTMF dial string without first checking for a dial tone may not result in intended (or polite) behavior.

### Obtaining Call State Information

Using the function lineGetCallStatus, an application can receive complete call status information for the specified call as a data structure of type LINECALLSTATUS. The function lineGetCallInfo returns mostly constant information about a call as a data structure of the type LINECALLINFO. The message LINE_CALLSTATE is sent unsolicited to an application to notify it about changes in a call's state.

The call-information data structure maintained for each call contains an application-specific field that applications can read from and write to. This field is not interpreted by TAPI. Applications can use it to "tag" calls in application-specific ways. Writing to this field so that the change is visible to other applications can be done with the function lineSetAppSpecific.