LANtastic (R) LANCHECK (R) Program Documentation

------------------------------------------

(C) Copyright 1992, 1993 ARTISOFT Inc.

(Revised 4/93)

When you install the LANtastic software, the LANCHECK program is copied

to your LANtastic installation directory.  LANCHECK is a LANtastic

network diagnostic program that allows you to determine if the adapters

in all of your network computers are communicating properly.

LANCHECK can test up to six adapters per computer and display a maximum

of 1000 computer names.  You can run LANCHECK any time after bringing up

the NETBIOS, even if other network software is running.

RUNNING LANCHECK

================

To run the LANCHECK program, complete the following steps:

1.  Run your network startup batch file (usually STARTNET.BAT) on all

the computers you want to test.  Be sure to run the network software

at approximately the same time on all the computers.  If you receive

an error message, refer to your LANtastic manuals for further

information.

2.  Run LANCHECK on each network computer to be tested.  At the DOS

prompt, type:

LANCHECK

and press Enter.  After a message and a short pause, you will see

the LANCHECK Main screen, which is shown later in this file.

LANCHECK will display this computer's information using the name

assigned to the computer on the REDIR command line.

If you have run the NETBIOS for your adapter(s) but have not yet run

the REDIR program, issue the command:

LANCHECK unique-name

where "unique-name" is a name (up to 16 characters) for this

computer.  For example:

LANCHECK JOANN

If the LANCHECK Main screen does not appear clearly on your monitor,

you may want to use the MONO switch when you run the program.  Type:

LANCHECK/MONO

or

LANCHECK/MONO unique-name

where "unique-name" is a name (up to 16 characters) for this

computer.

NOTE:  Do not attempt to run LANCHECK from within a Windows DOS virtual

machine while Windows is running in enhanced mode.  This will cause

your computer to lock up.  If Windows is running, exit Windows

before running LANCHECK.

LANCHECK Main Screen

======================

LANtastic (R) LANCHECK (R) Version 4.xx   (C) Copyright 1992 Artisoft Inc.

���������������������������������������������������������������������������ͻ

� A#  NAME         NODE NUMBER   MINUTES RUNNING   STATUS    ERROR-INDEX    �

���������������������������������������������������������������������������͹

� 0   BOB          00006EA00029          9         (local)     2%  (5%)     �

� 0   JOAN         00006EA00045          9         active      2%  (5%)     �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

�                                                                           �

���������������������������������������������������������������������������ͼ

Enter-Select, Ins-Enter, Space-Update, R-Refresh, F10-File, Esc-Exit, F1-Help

OPTIONS ON THE LANCHECK MAIN SCREEN

=====================================

HELP (F1)

=========

At any time while the LANCHECK program is running you may press the F1

key to receive more information about the fields presented in the

screen.  You can make the Help window double in size by pressing the

letter Z on the keyboard.  Pressing Z a second time changes the Help

screen back to its original size.  Use the arrow keys to scroll the text

up and down, or use the PgUp or PgDn keys to move the text up or down a

page.

Display Update Interval

=======================

The Adapter Status screen can be set to update automatically at any

interval between 0 and 3600 seconds.  By default the screen is updated

every 10 seconds.  You can update it manually by pressing the spacebar.

By adjusting the interval from the Adapter Status screen you can watch

the number of Good Transmissions and Good Receives as they change.  The

adjustment also can be made from the LANCHECK screen.

To change the update interval from the Adapter Status screen:

1.   Press Enter from the LANCHECK screen.

2.   Press R to bring up the Refresh Period screen.

3.   Type in a time between 0 and 3600 seconds and press Enter.

To change the rate from the LANCHECK screen:

1.   Press R to bring up the Refresh Period screen.

2.   Type in a time between 0 and 3600 seconds and press Enter.

Manual Display Update

=====================

Press the spacebar to update the LANCHECK Main screen display.

Arrow Keys

===========

Use the arrow keys to move up and down the list of adapters.  Use these

keys to select an adapter that you want information about.

Page Up and Page Down

=====================

Press either the Page Up or Page down keys to scroll up or down the

LANCHECK Main screen.  Use this option with the arrow keys to select an

adapter when you want more information about it.

Printing

========

Press F10 to print or copy the LANCHECK Main screen to a file.

Statistics

==========

Select an adapter and press Enter to view detailed statistics of the

adapter in the Adapter Status screen.  Explanations of these

statistics and the Adapter Status screen appear later in this file.

Fields on the LANCHECK Main Screen

==================================

A# (Adapter)

============

Tells you which adapter you are receiving information on.  The

first adapter is numbered "0," the second "1" and so on.  Up to six

adapters may be tested per computer.

Name

====

Refers to the unique node name you specified with the LANCHECK command

or on the REDIR command line.  Make sure that this table lists all the

adapter cards in each PC that is running LANCHECK.  If any adapters are

missing, you may have a cabling problem or faulty adapter.

Node Number

===========

Denotes the unique network number assigned to the adapter.

Minutes Running

===============

Shows how long the adapter has been running since you installed the

NETBIOS software.

Status

======

Tells you whether the adapter being tested is active or inactive and

which adapters are "local," (located on your computer).  If a computer

is sending packets but no other computers are sending packets in return,

the status is given as "*waiting*."  If an adapter that should be active

is listed as inactive, there may be a problem with the adapter or the

cabling to it.  If you see an adapter with a status that keeps

switching between "active" and "inactive,"  there is probably an

intermittent problem with the cabling.

Error Index

------------

Presents you with two statistics to help you deduce the relative

performance of each adapter in the sending and receiving of network

data. The first figure shows the current error rate.  The second figure,

in parentheses, gives the maximum error rate for the entire session.

NOTE: The values given in the sample LANCHECK screen should not be

considered average or ideal values for the adapters you are

evaluating.  Look instead for adapters that have ERROR-INDEX values

that are either much higher or lower than the other adapters in

your network.

*  An adapter with high ERROR-INDEX might have a cabling problem

or other hardware problem.  If it is an end network node, it may

not be terminated correctly.

*  Low values might suggest the adapter is able to receive data

correctly but is not properly transmitting data to the rest of

the network.  This will increase the other adapters' ERROR-INDEX

scores because they will be receiving bad transmissions from the

faulty adapter.  When this occurs, try an adapter that has been

functioning correctly in a different computer to see if the

problem happens again.  If so, contact Artisoft Technical

Support.

*  If you see an unusually high or low ERROR INDEX value for an

adapter, disconnect its cable so it is bypassed on the network.

When using thin Ethernet cable, simply disconnect the T-connector

from the adapter's BNC connector, and the rest of the network can

still communicate.  If it cannot, the T-connector may need to be

replaced.  With Artisoft 2Mbps or A2Mbps adapters, simply

disconnect the cables and reconnect the two ends.  If the other

adapters' ERROR INDEX values decrease dramatically, the adapter

could be at fault.  If this is the case, contact your dealer or

Artisoft Technical Support.  The adapter may need service.

*  If the other adapters' ERROR INDEX values are not affected by

removing the suspected adapter from the network, reconnect the

network cabling to include the suspected adapter.

SPOTTING AND SOLVING PROBLEMS ON THE LANCHECK MAIN SCREEN

==========================================================

Make the following checks on the LANCHECK Main screen.

1.  Check the A# field.  Make sure all the NETBIOS adapters for each

computer appear on the LANCHECK screen.  If an adapter does not

appear, look for the following problem conditions:

*  Adapter does not appear on the screen.

*  Adapter status listed as "inactive" while LANCHECK is

running on the computer.

*  Adapter status repetitively switches between active

and inactive.

If any of these conditions exist, try the following suggestions:

*  Check your hardware manual for cable specifications.

Make sure that the cable you are using is the approved

type for your network.

*  Check the cable connections between the computers.

*  Make sure you have not mixed cable types (even supported

types).

*  If you are using thick or thin Ethernet cable, be sure

you have placed terminators and T-connectors on the open

cable connectors of the end nodes.  With Artisoft 2Mbps

or A2Mbps adapters, you also must make sure you have

placed terminators on the end nodes.

*  Place a different section of cable between the adapter

that does not appear on the LANCHECK Main screen and the

next adapter(s).  It is best to use a section of cable

that has already successfully supported communications

between two computers.  Systematically move it into

different locations until you find the faulty section of

cable.

*  If the problem persists after completing the steps above,

contact your dealer or Artisoft Technical Support.  The

adapter may need service.

To further evaluate the efficiency of a network node, use the

arrow keys to move the highlight bar to the desired node and press

Enter.  This screen appears:

LANtastic (R) LANCHECK (R) Version 4.XX   (C) Copyright 1992 Artisoft Inc.

���������������������������������������������������������������������������ͻ

� A#  ��������������������������������������������������������������ͻ-INDEX�

�����ͺ               Adapter Status of: JOAN                        ������͹

� 0   ��������������������������������������������������������������͹      �

� 0   �                  Adapter Number: 0                           �      �

�     �                     Node Number: 00006EA00045                �      �

�     �                Software Version: 4.xx                        �      �

�     ��������������������������������������������������������������ͺ      �

�     �  TRAFFIC AND ERROR STATISTICS  � ADAPTER RESOURCE STATISTICS �      �

�     ��������������������������������������������������������������ĺ      �    Minutes Running: 9          �     NCBs Available: 26      �      �

�     �         CRC Errors: 222        �     NCBs Allocated: 32      �      �

�     �   Alignment Errors: 320        �  Max NCBs Possible: 128     �      �

�     �         Collisions: 9          �    Active Sessions: 4       �      �

�     �  Bad Transmissions: 65         � Sessions Allocated: 32      �      �

�     � Good Transmissions: 3928       �  Sessions Possible: 128     �      �

�     �      Good Receives: 23103      �    Max Packet Size: 2461    �      �

�     �    Retransmissions: 65         �    Number of Names: 8       �      �

�     �   Resource Exhaust: 838        �                             �      �

�     � Last Packet Received: 9:49 AM  � Last Packet Sent: 9:50 AM   �      �

�     �                                                              �      �

�     ��������������������������������������������������������������ͼ      �

�                                                                           �

�                                                                           �                                                                           �

���������������������������������������������������������������������������ͼ

Space-Update Data, R-Refresh Rate, F10-To File,  Esc-Exit, F1-Help

The information at the top of the screen tells you the name of the

computer, adapter number and node number being tested and the version of

the LANCHECK software you are using.

The bottom left portion of the screen provides you with Traffic and

Error Statistics that may help you diagnose a hardware problem with

your network.  To the right of this are the Adapter Resource Statistics

that provide information concerning your NETBIOS configuration.

Detailed descriptions of each field are given later in this file.

The values presented in the example above are not necessarily the same

as those you should see for your network.  Look for values that are

out of the average range for the rest of your adapters.

To check a second computer, press Esc to exit to the previous screen

then use the arrow keys to select the computer you want to test and

press Enter.

FIELDS IN THE ADAPTER STATUS SCREEN

=====================================

This screen contains two categories of information: Traffic and Error

Statistics and Adapter Resource Statistics.

Traffic and Error Statistics

============================

The following fields appear on the left side of the Adapter Status

screen.

Minutes Running

---------------

Refers to the number of minutes since the NETBIOS was run.

CRC Errors

----------

Shows the number of times a packet with a CRC error was received.

Cyclical Redundancy Check (CRC) errors occur when a packet of data

arrives at a computer in a corrupt form.  Before a computer sends a

packet, it mathematically evaluates the contents of the packet and

places the CRC information in it.  The receiving computer performs the

same calculation on the data.  It then matches its results against those

of the sending computer's evaluation.  If the results do not match, the

receiving computer asks the sending computer to retransmit the packet.

A high value in the CRC field could indicate that an adapter is not

correctly transmitting data.  It also might indicate that faulty cabling

or electrical noise is corrupting the data in transit.

Alignment Errors

-----------------

Shows the number of times that bits of data arrived out of the 8-bit

group in which they were sent.  These bits must then be regrouped into

their original bytes by the receiving node.  While this does not usually

degrade network performance, a high value in this field could suggest

cabling or hardware problems.

Collisions

-----------

Displays the number of times two or more nodes attempted to access the

network simultaneously.  A high value here usually indicates a busy

network.  Occasionally, however, a malfunctioning adapter or cable or a

section of the network that is not terminated results in a higher

number of collisions.  In some rare instances, a malfunctioning adapter

continuously transmits noise, resulting in a high number of collisions.

Bad Transmissions

-----------------

Shows the number of unsuccessful transmissions sent by this adapter.

Busy networks have more unsuccessful transmissions than networks

with less frequent use.  A high number of bad transmissions usually

indicates cabling or hardware problems.

Good Transmissions

------------------

Indicates the number of successful transmissions by this adapter.  A low

number in this field could indicate that an adapter is not efficiently

receiving network data.  In such a case, test this adapter with only a

few other adapters to make sure this statistic is not the result of poor

transmissions from another network adapter.

Good Receives

-------------

Shows the number of data packets successfully received by this adapter.

A low value in this field could indicate that an adapter is not

receiving network data reliably.  In such cases, it is best to test this

adapter with only a few other adapters and make sure this low value is

not a result of poor transmissions from an adjoining network node.

Retransmissions

---------------

Displays the number of times data had to be re-sent due to unsuccessful

transmissions.  Values that are significantly higher than the rest of

the adapters could indicate a faulty adapter.

Resource Exhaust

----------------

Shows the number of times the NETBIOS exhausted its supply of buffers.

When this occurs, it can no longer receive data from the network.  The

Resource Exhaust field keeps count of each time this happens.  If your

computer is continually running out of buffers, set your NETBIOS buffers

higher to accommodate network traffic better.  Refer to your network

adapter's documentation for information.

Last Packet Received

--------------------

Displays the time at which the last packet was received from another

computer.

Adapter Resource Statistics

============================

The statistics given on the right side of the Adapter Status screen can

help you evaluate how well your software configuration meets your

network needs.

NCBs Available

--------------

Displays the number of Network Control Blocks (NCBs) the NETBIOS has

available.  If the NCBs Available field has a value less than eight,

consider raising the number of NCBs allowed for this adapter.  Without

enough NCBs network performance will suffer.  There is also the

possibility that some network software will not run.  You can set this

with the NETBIOS command line switches.  To estimate the number of NCBs

you need, add eight to the number of log-ins you will need.

Max NCBs Allocated

------------------

Tells you how many Network Control Blocks (NCBs) are allocated in the

NETBIOS command line.  You can increase the number of NCBs by using the

appropriate driver software switch.

Max NCBs Possible

------------------

Displays the maximum number of Network Control Blocks (NCBs) the adapter

can support.

Active Sessions

---------------

Shows the number of active NETBIOS sessions.  A session is created any

time a connection between two computers is made or attempted.  If the

number given in the Active Sessions field seems out of proportion with

what you feel it should be, check your software to make sure the number

of sessions configuration is correct.  The maximum number of available

sessions depends on the NETBIOS.

Sessions Allocated

------------------

Displays the maximum number of sessions supported by the adapter as

configured by the NETBIOS RESET command.

Sessions Possible

-----------------

Shows the maximum number of sessions the adapter can support.

Max Packet Size

---------------

Indicates the largest size data packet that this adapter can either send

or receive.  The maximum size of a data packet depends on the size of

the buffers set for the adapter.

Number of Names

----------------

Shows to the number of network names currently in use.  An adapter with

a much larger value than the other adapters may be running a program

that is erroneously creating a large number of names.

Last Packet Sent

----------------

Displays the time the sending machine sent its last packet (using the

sending machine's clock).

SPOTTING AND SOLVING PROBLEMS ON THE ADAPTER STATUS SCREEN

===========================================================

Make the following checks in the Adapter Resource Statistics section

of the Adapter Status screen:

1.  If the NCBs Available field has a value of less than eight, consider

raising the number of NCBs allowed for this adapter.  If you are

using an Artisoft adapter, you can set this number with the

LANBIOS, LANBIOS2 and LANBIOS3 NCBS= switch, or with the AILANBIO

MAX_NCBS= and NCBS= switches.  If you are using a non-Artisoft

NETBIOS that has no provision for setting the number of NCBs, use

the NBSETUP program described in your LANtastic manuals.  A good way

to estimate the necessary number of NCBs is to add eight to the

number of log-ins you will need.

2.  Check the Active Sessions field.  You should have at least one

session for each user logged in to the server and one or more

sessions for each NETBIOS application you are currently running.  If

you have too few sessions, you may need to allocate more.  If you

are using an Artisoft adapter, you can set this number with the

LANBIOS, LANBIOS2 and LANBIOS3 SESSIONS= switch, or with the

AILANBIO MAX_SESSIONS= and SESSIONS= switches.

3.  Evaluate the Number of Names field.  An adapter with a much larger

value than other adapters may be running a program that is

erroneously creating a large number of names.

CHECKING OTHER NETWORK COMPUTERS

================================

To perform a networkwide adapter check, you do not have to run

LANCHECK on each computer.  You can check the adapter status of any

network node by completing the following steps:

1.  From the LANCHECK Main screen, press Ins.  You are prompted to

enter the name of the computer you want to test.

2.  Type in the machine name in capital letters, then press Enter.

The Adapter Status screen for the desired node appears.

3.  Repeat this process for each adapter you want to evaluate.

4   To exit LANCHECK, press the Esc key from each screen.

PRINTING FROM A LANCHECK SCREEN

===============================

The Error Statistics display may be copied to a file and printed.  You

can print from either the LANCHECK screen or the Adapter Status screen.

1.   Press F10 from the LANCHECK Main screen.

2.   Select the short list to copy or print the LANCHECK Main screen.

Choose the long list to copy or print the Adapter Status screen

for each adapter listed on the Main screen.  If you press F10

from the Adapter Status screen, only the current Adapter Status

screen will be copied to a file or printed.

3.   Enter the DOS path to a file or printer device.  For example,

if you enter LPT1, the screens will be output on the local or

network printer connected to your LPT1 port.  If you enter the

filename C:\LANCHECK.TXT, the output will be sent to the file

LANCHECK.TXT in the root directory of your C: drive.

4.   Press Enter.

TROUBLESHOOTING HINTS

=====================

The most important test for your network adapters is whether they all

appear on the screen when LANCHECK runs.  If an adapter is not listed on

the LANCHECK screen, it is probably not communicating with the rest of

the network.  There may be several reasons for this.  The most obvious

cause is that the cabling is faulty or not connected correctly.  There

also may have been a hardware failure on the adapter.

It is recommended that you start up all your network nodes

simultaneously to ensure that they are evaluated for performance during

the same period.  LANCHECK evaluates each node's performance from when

it first comes up on the network, not just from the time LANCHECK

executes.

If you suspect that there is a hardware problem, and an adapter has a

higher than average value in one or more of the fields in the LANCHECK

Main screen, remove the node from the network and see how network

performance changes.  If performance goes up dramatically, the adapter

could be at fault.