README.DOC                                   updated November 1, 1993

Backup Exec for NetWare

---------------------------------------------------------------------

CAREFULLY FOLLOW THE INSTRUCTIONS IN SECTIONS 1-3 BELOW TO COMPLETE

THE INSTALLATION PROCESS.

YOU MAY WISH TO PRINT A COPY OF THIS README.DOC FILE, WHICH IS

LOCATED IN YOUR SERVER'S SYS:CONNER SUBDIRECTORY.  FOR EXAMPLE:

NPRINT [ insert server name ]/SYS:CONNER\README.DOC

=====================================================================

TABLE OF CONTENTS

=================

Section#  Topic

--------  ------------------------------------------------------

1.   Installation

2.   Novell NLMs

3.   Server Configuration Issues

4.   Compatibility Information for Drives and Controllers

5.   Windows Client Information

6.   DOS Client Information

7.   Limitations

8.   Commonly Asked Questions

9.   Changes to Documentation

10.   Contents of Installation Disks

=====================================================================

1.  INSTALLATION

============

The Backup Exec for NetWare software is installed directly on

the server that you intend to use as your backup server.  You

may choose a server running NetWare version 3.11 or later.

The Backup Exec installation process consists of several steps:

(1)  Install the Backup Exec software onto the file server.  At

the end of the installation you are asked to update the

Novell NLMs on other servers that Backup Exec will access.

(2)  DOWN the file server and power it off.

(3)  Install any new backup-related hardware into the file

server.  For example:  install an Adaptec SCSI controller

and an internal tape drive.

(4)  Turn on the file server and restart.

(5)  After the server finishes booting, at the server console

prompt ':' type the following command: BESTART.

BESTART is an .NCF file located in your backup server's

SYS:SYSTEM subdirectory which is used to load the Backup

Exec Job Manager and associated NLMs.  You may wish to add

this command to your AUTOEXEC.NCF in order to automatically

load the Backup Exec Job Manager when your server boots.

(6)  Install clients and/or agents on workstations so users

can submit jobs.

For more information, refer to the Backup Exec for NetWare

Installation Manual.

2.  NOVELL NLMS

===========

Backup Exec utilizes several of Novell's NLMs to process its

NetWare-specific requests and to communicate with other servers

and workstations.  The installation program prompts for a

confirmation when it attempts to install the proper version of

each NetWare NLM that is needed.

Some NetWare NLMs must also be installed (or updated) on remote

servers that Backup Exec will be accessing for backup or

restore operations.  The install program also prompts for

confirmation as it installs these NLMs.

If Backup Exec is not run with the specific versions of the

NetWare NLMs that it needs, undesirable results may occur.

Therefore, we STRONGLY recommend that you allow Backup Exec to

install the recommended versions of these NetWare NLMs.

3.  SERVER CONFIGURATION ISSUES

===========================

NOTE:  The required changes to server configuration files listed

below are done as part of the Backup Exec installation.

However, if you later make changes to these files and you

encounter problems running Backup Exec, you may need to

refer to this information.

a.  STARTUP.NCF Changes Required:

---------------------------------

Servers with greater than 16 megabytes of memory:

-------------------------------------------------

If you have more than 16 megabytes of memory in your file

server, the number of reserved buffers that are below the

16 megabyte boundary need to be increased from the default

(NetWare defaults to 16 of these buffers).  Backup Exec

allocates its tape buffers from this reserved buffer pool,

which makes it necessary to increase the amount of

reserved buffers.

To increase the number of reserved buffers below 16

megabytes, use the following SET command in your

STARTUP.NCF file:

SET RESERVED BUFFERS BELOW 16 MEG = 200

The server must be downed and re-started for this change

to take effect.

Loading controller level drivers:

---------------------------------

It is recommended that you load ALL controller level

drivers in your server's STARTUP.NCF file.  Examples of

controller level drivers are:  AHA1540.DSK, AHA1640.DSK,

AHA1740.DSK, ASPITRAN.DSK, etc.

b.  AUTOEXEC.NCF Changes Recommended:

-------------------------------------

BESTART.NCF

-----------

Backup Exec installs a file (BESTART.NCF) which loads

the Job Manager and other required NLMs.  You may wish to

add BESTART at the end of your AUTOEXEC.NCF file so that

the Job Manager will load each time your server is booted.

Running BESTART.NCF loads the device driver for the

type of controller you chose during installation.  If you

later decide to use a different controller, you may wish

to modify your BESTART.NCF file to load MS_SCSI instead of

AD_ASPI or vice-versa.

4.  COMPATIBILITY INFORMATION FOR DRIVES AND CONTROLLERS

====================================================

a.  SCSI Controller Support

Vendor           Model       Notes

--------------------------------------------------------

Adaptec          1510        Must disable Fast File Access

Exabyte 10i not supported

Adaptec          152x        Must disable Fast File Access

Exabyte 10i not supported

Adaptec          154x

Adaptec          1640

Adaptec          1740

Adaptec          2740

Adaptec          2740T

BusLogic         BT542       * (see below)

BusLogic         BT646       * (see below)

BusLogic         BT747S      * (see below)

Conner (Maynard) 16-bit      Cipher T860-DLT not supported

Cipher T860-LDLT not supported

Conner (Maynard) PS/2        Cipher T860-DLT not supported

Cipher T860-LDLT not supported

DPT              2011

DPT              PM2021/90   With & without cache

Exabyte 10i not supported

DPT              PM2021/95   With & without cache

Exabyte 10i not supported

DPT              2012B

DPT              PM2122/90   With & without cache

DPT              PM2122/95   With & without cache

UltraStor        14F

UltraStor        24F         Exabyte 10i not supported

* Use BT311.DSK dated 02/02/93 for NetWare 3.11 servers;

Use BT312.DSK dated 05/04/93 for NetWare 3.12 servers;

Use BT40.DSK dated 06/09/93 for NetWare 4.x servers.

------------------------------------------------------------

Additional Notes on Controller Support:

o  Adaptec Controllers

AHA-1510, 152x, 154x, 1640, 1740 Controllers:

----------------------------------------------------

Backup Exec v4.0 was compatibility tested with

Adaptec's ASW-1440 Version 3.3 NetWare 386 SCSI

Drivers.  The specific driver versions are as follows:

Driver:        Version:       Module Date:

AHA1510.DSK    1.22           11/13/91

AHA1520.DSK    1.22           11/13/91

AHA1540.DSK    2.22           07/26/91

AHA1640.DSK    2.22           07/26/91

AHA1740.DSK    1.20           07/01/91

ASPITRAN.DSK   1.0            07/31/90

AHA-2740, 2740T Controllers:

----------------------------

Backup Exec v4.0 was compatibility tested with

Adaptec's ASW-1240 Version 1.3 NetWare 386 SCSI

Drivers.  The specific driver versions are as follows:

Driver:        Version:       Module Date:

AIC7770.DSK    1.05           06/28/93

ASPITRAN.DSK   1.10           04/13/93

Recommended Drive Selection Settings

------------------------------------

Conner Software recommends that, when used with an

Adaptec controller, the SCSI ID for the tape drive

be set at address 2 for the first tape drive.

Addresses 0 and 1 are reserved for hard disks.

Synch Negotiation Changes Required

----------------------------------

If you are using a Conner DAT or DAT/DC tape

drive connected to an Adaptec 164x or 174x

controller, you must disable the controller's Synch

Negotiation (i.e., the Adaptec controller must not

initiate a synch negotiation).  Use the PS/2 setup

or EISA configuration utility provided with your

system to change this option.

Fast File Access problem with Adaptec 1510/1520

------------------------------------------------

The Adaptec 1510/1520 controllers were tested with

AHA1510.DSK and AHA1520.DSK drivers both dated

12/03/91.  Unlike the drivers for the 154x, 1640 or

174x controllers, Backup Exec's Fast File Access

feature will not work with these 1510/1520 drivers.

Until the problem is fixed in the supplied Adaptec

drivers, you must disable Backup Exec's Fast File

Restore feature.  Simply edit the BEMGR.CFG

configuration file located in the SYS:CONNER

directory on your server and set the parameter

'Use_fast_file_access' equal to 0.  (Do not modify

the BEMGR.CFG file while the Job Manager is running.)

Recommendation for loading AHA1510.DSK

--------------------------------------

When loading the AHA1510.DSK driver, Adaptec recommends

that you explicitly specify all of the load options

available, for example:

load aha1510 port=aaa verbose=y fixed_disk=off

int=i id=z sync=off

where aaa = port address of 1510 controller card

i   = interrupt setting of 1510 card

z   = SCSI address of the 1510 card

o  BusLogic Controllers

Backup Exec for NetWare v4.0 was compatibility tested

with the following BusLogic SCSI drivers:

Driver:        Version:       Module Date:

BT311.DSK      2.20           02/02/93

BTASPI.DSK     1.0            07/23/92

Using a BT311.DSK dated earlier than indicated

can cause the Job Manager application to "hang"

during its initialization process.

o  DPT Controllers

Backup Exec for NetWare v4.0 was compatibility tested

with the following DPT SCSI drivers:

DPT2011 and DPT2012B Host Adapters

Driver:        Version:       Module Date:

PM12NW31.DSK   1.82           02/03/93

PM11NW31.DSK   1.80           05/11/92

DPTDDL.NLM     1.80           05/11/92

DPT2021 and DPT2122 Host Adapters

Driver:        Version:       Module Date:

PM11NWS31.DSK  3.B            9/23/93

PM11NWS40.DSK  3.B            9/23/93

ASPITRAN.DSK   1.01           9/23/93

DPTSIM.DSK     3.B            9/23/93

Firmware versions:

Host Adapter   Firmware       BIOS ROM

DPT2011        3G             1J9

DPT2012        3G             1J9

DPT2021        2.15           2B

DPT2122        2.10           2B

o  UltraStor Controllers

Exabyte 10i Loader

------------------

The Exabyte 10i Loader is not supported on the

UltraStor 24F controller.

Synch Negotiation Changes Required

----------------------------------

If you are using a Conner DAT or DAT/DC tape

drive connected to an UltraStor U14F controller

you must disable the controller's Synch Negotiation

(i.e. the UltraStor controller must not initiate a

synch negotiation).

Backup Exec for NetWare v4.0 was compatibility tested

with the following UltraStor SCSI drivers:

Driver:        Version:

U14_31X.DSK    2.00

U24_31X.DSK    2.00

U14_312.DSK    2.02

U24_312.DSK    2.02

U14_40X.DSK    1.02

U24_40X.DSK    1.02

o  Conner (Maynard) Controllers

The Cipher T860 family of drives is not supported on

the Conner or Maynard SCSI controllers.

b.  SCSI Device Support

Vendor    Drive                    Firmware Certified

------------------------------------------------------

Conner    250                      21247-011

Conner    525                      25582-003,

25582-004,

25462-007

Conner    2750                     28077-003-v6.0,

28077-902-v6.0

Conner    2800                     ANCDA 2800 28889-900

Conner    DAT                      25588-005-2.19,

25588-006-2.66,

25588-007-2.96

Conner    DAT DC                   27807-003-3.58,

27807-005-4.04,

27807-005-4.61

Conner    Turbo DAT DC             28388-XXX 4.28,

28388-XXX 4.59,

28388-XXX 4.61

Conner    Diamondback DAT Loader   28849-XXX-4.59 with

2.93D EPROM,

28849-XXX-4.61 with

2.93D EPROM

Cipher    T860-DLT                 430A T860S

4314

Cipher    T860L-DLT                430A L860S

K5 EPROM

28 Loader EPROM

Exabyte   8200SX                   EXB-8200-2638

Exabyte   8500                     EXB-8500-0340, 0415, 04SO

Exabyte   10i Loader w/8500        2.10 (loader) with

EXB-8500-0415,

3.024 (loader) with

EXB-8500-04SO

Exabyte   4200 DAT                 107, 137A

HP        DAT 35470A               7.09

HP        DAT DC 35480A            7.09

IBM       4100                     I05Y0621F8719

IBM       35480A                   7.09

IBM       8505                     47DB

Tandberg  3600                     08:03, 08:15

Tandberg  3800                     04:08, 03:07

Tandberg  4100                     05:17, 05:10

Tandberg  4200                     04X35

Teac      600                      0233-06R.V.A-191C, 191D,

0233-06R.V.D-239E

WangDAT   DAT 3100                 2.0

WangDAT   DAT DC 3200              2.0

------------------------------------------------------

Additional SCSI Device Support notes:

o  Conner DAT

DO NOT attempt to access a tape created by a Conner

DAT DC if your DAT drive has firmware versions

1.36 or 2.19!  In this situation, the DAT drive

will appear hung for up to 10 minutes before

returning control to Backup Exec for NetWare.  This

problem was fixed in the 2.66 firmware revision.

o  WangDAT

The WangDAT drives have a switch which enables/disables

Software Eject.  It is set to "DISABLE" by default.  If

the feature is "ENABLED", Backup Exec's Utility "Eject

Tape" feature will operate.  If this switch is left

in the disable position, Backup Exec will only rewind

the tape when the "Eject Tape" function is selected.

o  Conner Diamondback DAT Loader

If you are using a Conner Diamondback DAT Loader,

make sure that a magazine is loaded in the drive prior

to starting the Backup Exec Job Manager (BEMGR.NLM).

This loader supports magazines with 4, 8 or 12 tapes

inserted and Backup Exec determines the number of tape

slots supported during initialization.  If a magazine

is not inserted in the drive when Backup Exec

initializes, the software reports that no tape drives

have been found.  If this happens, simply exit Backup

Exec, insert the magazine in the Diamondback and

re-load Backup Exec.  In addition, once Backup Exec

is loaded and running, changing the size of the

inserted magazine is not supported.

o  Conner 525

Writing to non-native format tapes is not supported

on the Conner 525 drives with 25582-003 and

25582-004 firmware.

5.  Windows Client Information

==========================

a.   The Backup Exec DOS Agent (also used for Windows) does not

support the Microsoft-supplied MSIPX.COM driver which comes

standard with Windows for Workgroups version 3.1.  See

LIMITATIONS item (g) for more information.

b.   Do not use the double buffering feature on SMART DRIVE

while a workstation is being restored.  Enabling the

double buffering of the Smart Drive utility causes

errors to occur during the restore.  Note that the

double buffering capability of Smart Drive is usually

associated with the use of SCSI hard drives.

6.  DOS Client Information

======================

a.   The DOS client should be run with at least 512K free

memory.

b.   Do not use the double buffering feature of SMART DRIVE

while a workstation is being restored.  Enabling the

double buffering of the Smart Drive utility causes

errors to occur during the restore.  Note that the

double buffering capability of Smart Drive is usually

associated with the use of SCSI hard drives.

c.   Running the DOS client in a Windows DOS box is not

supported.

7.  LIMITATIONS

===========

a.   When accessing servers with older versions of the STREAMS

and TLI NLM modules, the Backup Exec clients or the

Job Manager may hang during the scan for SMDRs.  See

the Commonly Asked Questions section for more information

if you are encountering this problem.

b.   If there is a logical disconnect between the Backup

Server and a remote server during a backup, you may

experience errors ranging from a job with a constant

active status to a server abend.  This is a result of the

SMS component failing to cleanly deal with the

connection loss.  For example, if you were to

dismount a volume while it was being backed up, you

would run into this problem.

c.   If you are using the Backup Exec Windows client and your

screen "blacks out" followed by a workstation hang, you

need to get updated IPX drivers from Novell's NetWire

Compuserve forum.  Updated files are found in the

BSDUP1.EXE self-extracting archive.

d.   When using Backup Exec agent software, you may want to

first type CASTOFF ALL.  If a message is sent to a

workstation that is running Backup Exec agent software, the

Job Manager or Client software may lose its connection to

the agent or performance may be severely degraded.

e.   NetWare 2.x servers are not yet supported by this product

because there is no Novell TSA available for NetWare 2.x

servers.

f.   Running the Backup Exec Job Manager within a Novell SFT III

server is not supported yet because there is no Novell

SMDR available for SFT III servers.

g.   The Backup Exec DOS Agent (also used for Windows) does not

support the Microsoft-supplied MSIPX.COM driver which comes

standard with Windows for Workgroups version 3.1.  In order

to use the Backup Exec DOS agent under Windows for

Workgroups, you must install Novell's ODINSUP.COM driver

instead of the MSIPX.COM driver.  The ODINSUP driver does

the translation between Novell's ODI and Microsoft's NDIS.

An outline of the procedure to modify a Windows for

Workgroups environment to use the ODINSUP driver is found

in the README.TXT file that is installed on the

workstation as part of the Backup Exec Windows workstation

SETUP program. If there are Windows for Workgroups

workstations that will run the Backup Exec Dos Agent, the

network administrator should review the procedure outlined

and create a customized procedure for the network users.

h.   If you are running the Backup Exec Job Manager on an

Adaptec controller and the tape drive is powered off

during an operation, you may need to DOWN your file

server to re-use the Job Manager.  Adaptec's ASPI manager

does not correctly handle this problem and may result

in the Job Manager appearing to be "hung" trying to

complete an operation.  The file server will continue to

function properly, but you will need to DOWN your file

server and re-boot in order to use Backup Exec again.

i.   A problem with the Novell supplied DSAPI.NLM may result

in a server console error message indicating a "Read from

a non present page frame".  This error has been seen in

instances of rather complex NetWare 4.01 NDS structures

with multiple organizations and multiple countries.  If

your NetWare 4.01 server is configured for "Read Fault

Emulation" then this error will be reported to the server

console and the server will continue functioning.

However, if you have not configured your server for "Read

Fault Emulation" then the 4.01 server will ABEND in this

circumstance.

j.   There is a problem with the 3Com 3C503 card that causes it

to hang the driver if it experiences heavy network traffic

for an extended period of time.  This manifests itself

in the Backup Exec for NetWare product by generating

"LAN receive buffer limit reached.  All buffers are in use"

errors while performing remote server backups.  If you

encounter this error, you should upgrade your network card.

k.   If the Job Manager (BEMGR.NLM) or the NetWare Client

(BE.NLM) displays an "out of memory" error and you see a

message at the NetWare console "Alloc short term memory

allocator requests exceeded the configuration limit", you

will need to give NetWare more memory to work with.

Failure to do so may cause memory corruption or a server

abend.

For 3.11 Servers: Type

"set maximum alloc short term memory=xxxxxxxx"

where xxxxxxxx is 50kb to 16mb.  It is set to

2mb by default.

For 3.12 servers Type

"set maximum alloc short term memory=xxxxxxxx"

where xxxxxxxx is 50kb to 32mb.

It is set to 8mb by default.

l.   Appending to backup tapes that were created with version

3.1 of Backup Exec for NetWare, MaynStream NLM or

Archive Information Manager NLM is not supported.  These

tapes can be cataloged and restored by Backup Exec for

NetWare v4.0 or they can be overwritten by backup jobs.

8.  COMMONLY ASKED QUESTIONS

========================

Q:   The Backup Exec Job Manager reports that it cannot find

any tape drives connected to the controller.  Why is

this and how do I fix this?

A:   There are several reasons you could get this message.

All of them have to do with the setup of the hardware

and software that make up and control the SCSI bus.

If you are having this problem, try the following:

1) Make sure that you have your SCSI bus terminated

properly.  There must be a terminator at either

end of the SCSI bus, and NO terminators in the

middle of the SCSI bus.

2) Make sure that none of the connections on the SCSI

bus are loose.

3) Check the SCSI bus IDs for each device on the SCSI

bus.  Each device should have a unique ID.  Don't

forget the SCSI bus adapter in your system. Typically

it is set to SCSI bus ID 7.

4) If you are using an ASPI compatible controller, make

sure that you load your disk (.DSK) drivers in the

STARTUP.NCF.  If you must load your disk drivers in

the AUTOEXEC.NCF, make sure that the "mount all"

command follows the load of the disk drivers.

5) Make sure that the tape drive you have connected to

the SCSI bus is supported by Backup Exec for NetWare.

See the COMPATIBILITY INFORMATION FOR DRIVES AND

CONTROLLERS section for a complete list of the

supported drives and controllers.

Q:   My client application starts up, and hangs while scanning

for network agents.  What is wrong?

A:   There is a problem with the scanning of the Novell agents

that causes the application to hang.  This usually

occurs when a server is being downed.  There are two

factors that affect the probability of your seeing this

problem:

1) The more servers you have on your network running the

SMS SMDRs, the more likely you are to have the problem.

2) The slower your system is, the more you will encounter

this problem.

You can reduce or prevent this problem by reducing the

number of SMS SMDRs that you are using by creating a

SMDR Include file.  See page A - 2 of the Backup Exec

for NetWare 4.0 Administrator's Manual for details

about the SMDR Include file.

Q:   Whenever I attempt to perform a tape operation, Backup

Exec reports an "Out of Memory" error.  In addition, my

file server reports:

"[date time] 1.1.38 Cache memory allocator exceeded

minimum cache buffer left limit"

on the system console.  Why is this and what can I do to

fix it?

A:   If you have more than 16 megabytes of memory in your file

server, you will need to increase the number of reserved

buffers that are below the 16 megabyte boundary (NetWare

defaults to 16 of these buffers).  If your server is

configured with more than 16 megabytes of memory,

Backup Exec will allocate its tape buffers from this

reserved buffer pool, which makes it necessary to increase

the amount of reserved buffers.

Normally, the Backup Exec for NetWare Install process

does this for you automatically.  You should only need

to change this if you had removed the setting from your

STARTUP.NCF file, or if you added memory after installing

Backup Exec for NetWare.

To increase the number of reserved buffers below 16

megabytes, use the following SET command in your

STARTUP.NCF file:

SET RESERVED BUFFERS BELOW 16 MEG = 200

The server must now be downed and re-started for this

change to take effect.

Q:   Why would loading Backup Exec for NetWare, slow my

server down more than I'd expected?

A:   NetWare supplies memory for all NLMs from a pool of file

cache buffers.  If you are loading several NLMs on your

file server, your server could reduce the number of

available file cache buffers to its minimum (the default

is 10) greatly reducing server performance.  To protect

the server's performance, use the following SET command

at the fileserver console:

SET MINIMUM FILE CACHE BUFFERS = XXXX

where XXXX is a number from 20 to 1000.  Setting this

value will cause the server to report an out of memory

error when any NLM requests memory that would allocate

from the minimum file size cache.  REMEMBER, Backup Exec

for NetWare requires TWO MEGABYTES of memory over that

of your normal operating environment.

Q:   My server constantly reports:

"[date time] Primary Interrupt Controller Detected

a lost Hardware Interrupt"

when I run Backup Exec for NetWare.  All server and Backup

Exec functions appear to be working correctly. What does

this mean?

A:   This message indicates that the server "thinks" that a

defective controller card is installed, or that there is

an interrupt conflict in the system. Verify that there

are no interrupt conflicts in the system.  If the above

message persists, this message can be suppressed by using

the following SET command at the fileserver console:

SET DISPLAY LOST INTERRUPT ALERTS = OFF

Q:   When I try to use RCONSOLE to control the NetWare Client

remotely, I can't use the <+> or <-> keys for Quick File

selections.  Instead of producing the include/exclude

windows, my screen scrolls through the server's console

screens.  How can I create Quick File selections within

the NetWare Client while using RCONSOLE?

A:   In NetWare 3.11 and 3.12, Novell's RCONSOLE reserves the

<+> and <-> keys on the numeric keypad for stepping through

the server's console screens.  Use the <+> and <-> keys on

the main keyboard to make Quick File selections from within

the NetWare Client.  (In NetWare 4.x, the <Alt+F4> and

<Alt+F3> keys are used to step through the server's console

screens.  Therefore, you may use the <+> and <-> keys on

the numeric keypad for Quick File selections in NetWare

4.x.)

Q:   I was running Backup Exec for NetWare and an "Assertion

Failed" error message similar to this appeared:

Assertion Failed in xxx.c

<Press any key to close screen>

What does this mean, and why did it happen?

A:   Backup Exec for NetWare continuously checks the integrity

of the memory allocated to it.  If Backup Exec finds a

memory corruption situation that could cause the server to

abend (possibly due to another NLM modifying Backup Exec's

memory), the above message is displayed and the Backup

Exec for NetWare NLM is unloaded.  You need to down the

server and re-boot to clear the condition.  Use <Alt+Esc>

to return to your system console ":" prompt and down the

file server.  Bring the server back up and re-start the

Backup Exec software.

9.  CHANGES TO DOCUMENTATION

========================

Administrator's Manual

-----------------------------------------------------------

Page A - 2

ADDITION:

You can use the wildcard character "*" in the SMDRINCL.DAT

file to indicate multiple SMDRs.  The wildcard can only

preceed or succeed the name.  For example:

TEST*     Would include TEST, TEST1, TEST_3

*TEST     Would include 1_TEST, 2TEST

-----------------------------------------------------------

Page C - 8

EXISTING TEXT:

Step 8.  Establish a search path to the local drive by

typing: search add c: at the server's console

prompt (":").

CHANGE TO:

This step should be performed AFTER step 11 instead of

step 8.  If it is done as step 8, the process of mounting

the NetWare volumes will delete the search path.

-----------------------------------------------------------

Page C - 10

EXISTING TEXT:

Step 27. Following the successful restoration of the

Backup Server's bindery submit additional restore jobs

corresponding to all of the Backup Server's volumes.

CHANGE TO:

Step 27. Following the successful restoration of the

Backup Server's bindery exit both the Job Manager and

-----------------------------

the Client NLMs.  Restart both applications and submit

-----------------------------------------------

additional restore jobs corresponding to all of the

Backup Server's volumes.  Note that since you have

------------------------

restored the binderies, when the NetWare client is started

----------------------------------------------------------

you should supply it with the name and password that

----------------------------------------------------

you were using prior to the disaster.

-------------------------------------

-----------------------------------------------------------

10. CONTENTS OF INSTALLATION DISKS

==============================

This section outlines the files that can be found on each of

the Backup Exec for NetWare installation diskettes.

Backup Exec for NetWare v4.0

3.5" 1.44MB and 5.25" 1.2MB Diskettes

----------------------------------------------------------

Disk 1:

!CNR1600.CFG        EISA Config file for the Conner (Maynard)

SCSI adapter

@568D.ADF           ADF for Conner (Maynard) Microchannel SCSI

adapter

1DISKSEQ            Installation control file

BE.REQ              Installation control file

BE40INST.NLM        Install program's link to Directory Services

BEINSTL.NLM         Backup Exec install program

BEINSTL.RES         Resource file for the install program

BEZIP.SIZ           Installation control file

BE_DISK.1           Compressed data file for software

CONNER.SPG          Installation control file

README.DOC          This file

SMS_WS.ZIP          Contains SMS TSAs for DOS and OS2

Disk 2:

2DISKSEQ            Installation control file

BE_DISK.2           Compressed data file for software

Disk 3:

3DISKSEQ            Installation control file

BE_DISK.END         Compressed data file for software