Artisoft AE-1/T, AE-2 And AE-3 Ethernet Adapter Information File (AEX.DOC)

----------------------------------------------------

(C) Copyright 1992 ARTISOFT Inc.

(Revised 2/15/92)

CHANGE IN THE DEFAULT IRQ JUMPER SETTING

The default jumper setting for Artisoft AE-3 and AE-2 adapters has changed

from IRQ3 to IRQ15.  Artisoft AE-2 adapters will now use the AEX.EXE

driver instead of AE2.EXE.  If you place an AE-2 or AE-3 adapter in an

8-bit slot, you must set the adapter to use IRQ 2, 3, 4, 5, 6 or 7.  You

must also include the corresponding command line switches on the AEX.EXE

command line.  For example, if you place an AE-3 adapter in an 8-bit slot

and set the IRQ jumper to 3, you would use the following command line to

run the AEX.EXE driver:

AEX/IRQ=3

Consult your Artisoft AE-3 Ethernet Adapter User's Manual for more

information on using the AEX.EXE command line switches.  If you are using

an AE-2 adapter, consult the LANtastic AE-2 Ethernet Adapter User's Manual

for information on using command line options.  The AE2.EXE and AEX.EXE

use the same command line switches with the exception of the two switches

described in the following sections.

AEX.EXE COMMAND LINE OPTIONS NOT AVAILABLE PREVIOUSLY TO AE2.EXE USERS

AEX.EXE uses the same switches as the AE2.EXE driver; however, AEX.EXE

allows you to use these two additional command line options:

--------

REMOVE

This switch removes the AEX program from memory.  You must remove

Terminate and Stay Resident (TSR) programs in the opposite order they were

loaded.  For example, if you loaded AEX and then AILANBIO, you must first

remove AILANBIO from memory before you can remove AEX.  Further, if you

have more than one invocation of AEX and AILANBIO you must use the MPX=

switch on the command line to specify which copy of AEX (but not AILANBIO)

you want to unload.  For example, if you load two copies of AEX and

AILANBIO:

AEX/MPX=C7

AILANBIO/ADAPTER=0/MPX=C7

AEX/MPX=D7

AILANBIO/ADAPTER=1/MPX=D7

To unload these copies of AEX and AILANBIO, type:

AILANBIO/REMOVE

AEX/MPX=D7/REMOVE

AILANBIO/REMOVE

AEX/MPX=C7/REMOVE

The REMOVE option will remove the most recently run copy of AILANBIO.

-------

TRANSMIT_BUFFERS= [20] Range 2 to 80 decimal

This option allows you to select the number of buffers used to transmit

data to a remote node.  Increasing the number of transmit buffers may help

increase performance, but will use more memory.

W11 PEROM/ROM SELECTION JUMPER

On Artisoft AE-2 Rev. E and higher adapters, you will find a new W11

jumper designed to allow the use of Programmable, Erasable, Read Only

Memory (PEROM) chips.  These chips allow you to update the software

stored in the PEROM chip as Artisoft releases new versions of its ROM

image.  You will find jumper W11 near the top of the board, towards the

center.  This two-position jumper allows you to select between PEROM ("A"

position) and ROM ("B" position).

���Ŀ           ���Ŀ

B� - �          B� + �

� + �           � + �

A� + �          A� - �

�����           �����

PEROM (Default)        ROM

#