Network Monitor Remote Agent:  Concepts and Use

Lisa Dusseault

Table of Contents

Concepts

Overview

Using Network Monitor

Architecture

Remote Agent components and functions

Network Monitoring Agent Service

Manager and Agent

Interface to NetBIOS

Control Panel Applet for Agent

Agent Status Tool

Communication

Using the Network Monitoring Agent

Installation

How to install the Network Monitoring Agent

Where to install the Network Monitoring Agent

Capturing with the Network Monitoring Agent

Connecting

Suspending the Agent

Reconnecting

Disconnecting

Agent Status

Triggers

Trigger Behavior with Suspend

Connecting over RAS

Capturing on a different Subnet

Capturing on a router

Router separates two physical subnets

Router separates two virtual subnets

When to use Suspend

Over an expensive link

Using an uncertain link

Capturing on multiple agents

Security

Troubleshooting

Event Logging

Network Monitoring Agent User Interface

Connection Problems

# Concepts

## Overview

Network Monitor is a network analysis tool for the Windows operating systems.  Although Network Monitor is entirely implemented in software, it has a full range of network analysis capabilities, including:

- Capturing frames from the network

- Displaying the parsed frames

- Filtering both on capture and display, to give the user the information they need

- Using triggers to control the progress of a capture session

- Capturing on a remote machine

This last capability, remote capturing, is explored in this document.  Remote capturing allows the user to connect to another machine, possibly on another subnet, and capture network frames as seen by that remote machine.

Remote capturing is useful in many network administration and troubleshooting scenarios.  Without remote capturing capability, the user is limited to capturing frames that their machine sees directly.  This can hinder problem resolution when the problem is on the other side of a router or a bridge.  With remote capturing capability, a network card on a machine running the Network Monitoring Agent can be used as a network resource.  The user can capture traffic on any subnet that has a machine with the Network Monitoring agent available to them, including subnets available over RAS (Remote Access Service).

Think of the process as the local instance of Network Monitor using the remote network card as if the card were local.  Information is sent across the network and displayed in the local user interface as usual.  Filters and triggers can be used normally.  If the capture is stopped and viewed, then frame contents will be sent across the network and displayed as if the information was stored locally.

The Network Monitoring Agent service can be installed and started on any Windows NT platform.  Once the Network Monitoring Agent service is started, remote capturing is enabled, as long as the network card on the machine works with Network Monitor.  With recent builds of NT, the remote machine need not have the full installation of Network Monitor in order to be a remote agent.

The “manager”, or local machine, can be any Windows NT machine with a full Network Monitor installation.  The manager need not have the Network Monitoring Agent service started in order to connect to a remote machine to capture.  Also, the manager does not need to have a network card that supports Network Monitor and capturing.  The manager and agent must have at least one protocol in common, and the corresponding NetBIOS interface for it.  As long as the manager can communicate with the remote agent, either over RAS or over the LAN, the manager can perform remote captures.

In this document, the machine with the remote agent installed and the Network Monitoring Agent Service started is called the “remote machine”.  The machine with the full Network Monitor installation, where the user sits and performs the connection and capture, is called the “local machine” or “manager”.  The full installation allows the user to capture frames, view frames, and connect to an agent, whereas on the agent-only installation the user can only change the password and stop and start the service.

### Using Network Monitor

Network Monitor is fairly simple to use, because it has a standard Windows user interface and full on-line help.  Here is a brief guide to using Network Monitor:

- To start Network Monitor: Double-click on the Network Monitor icon in the Program Manager.

- To capture:  Once a capture filter has been set (see “To set a capture filter” below), select “Start” from the Capture menu or use the Start button on the toolbar.  When finished capturing, select “Stop” from the Capture menu or use the Stop button on the toolbar.

- To view the captured frames:  Select “Display Captured Data” from the Capture menu.  This will open a Frame Viewer window showing a list of all frames kept during the last capture.

- To view the contents of a frame:  If the Frame Viewer window only shows a list of frames, double-click on any line in this list or press the Zoom button on the toolbar to show the contents of the frame in two formats.  In the Details pane (middle) you will see the protocol headings and properties (protocol fields and values) of the frame.  In the Hex pane (bottom) the contents of the frame are displayed in hex and ASCII format.

- To set a capture filter:  Capture filters are useful for limiting the number of frames kept by Network Monitor during capture.  Any frames that do not meet the filter specifications are discarded immediately instead of taking up memory.  Choose “Filter” from the Capture menu in the Capture window and specify the frames you are interested in by protocol, source address, destination address, or by character patterns within the frame data.

- To set a display filter:  Display filters are useful to limit the amount of information displayed on the screen.  Any frames that do not match the filter are not displayed, but are not discarded unless you save the capture with the filter applied.  Choose “Filter” from the Display menu in the Display window and specify the frames you are interested in by protocol, source address, destination address, or property value.

- To set a trigger:  Choose “Trigger” from the Capture menu in the Capture window.  Select the type of trigger, the pattern match required or the amount of frames to capture, and the action to be performed when the trigger condition is met.

For more information on using Network Monitor, use the on-line Help files.  The help files are available by selecting the “Contents” item in the Help menu.

## Architecture

The Network Monitor Architecture is highly modular, as shown in the diagram below (local machine only).  The User Interface (UI) presents information to the user.  The kernel, or driver, manages the data and abstracts the other modules for the UI.  The Network Abstraction Layer, or NAL, provides an abstraction for the layer below it, which can be a NDIS 2.0 or NDIS 3.0 network card, or a remote connection.  The kernel sends frames to the parsers for interpretation:  the parsers identify the protocol headers and break up the headers into properties with associated values.  The support modules provide functionality such as complex filters.

1.  Network Monitor Architecture

When the local machine is connected to a remote machine for a capture, the diagram becomes more complicated. The Remote Network Abstraction Layer, or RNAL, has two components in the same DLL:  the manager and the server, which are peer-to-peer.  The Manager RNAL communicates with the Server RNAL over NetBIOS, either on the LAN or over RAS.

2.  Network Monitor Architecture with Remote Machine connected over LAN

When the local machine is connected to the remote machine over the LAN, and the remote machine has only one network card, the capture is performed on the same card that the RNALs communicate through.  If another card is available, then when the connection is established, the user can choose to capture on the same card that the RNALs communicate through, or to capture on another card, perhaps on another network.

3.  Network Monitor connected to a Remote Agent which is a RAS Server

When the local machine is connected via RAS to a RAS server that is also a remote agent, the server must have a network card that supports promiscuous mode connected to the network.  Then the capture can take place on the network at the remote machine.

4.  Network Monitor connected to a Remote machine through a RAS Server

If the local machine is connected via RAS to a RAS Server which does not have the Network Monitoring Agent, the user can still capture if there is a remote agent available on the RAS Server’s network.  When the connection to the RAS Server is set up, the local machine sees the RAS Server’s network and can connect to the remote machine by specifying its name.  Then the remote machine can either capture on the same network card that it uses to communicate with the RAS Server, or on another card on the same network or on a different network.

## Remote Agent components and functions

### Network Monitoring Agent Service

The Remote Agent Service, or Network Monitoring Agent Service, is NMAGENT.EXE.  When the user has started this service with “net start nmagent” or through the control panel Services applet, it responds to start and stop connection messages from other machines.

### Manager and Agent

Most of the communication between the manager and the agent is done via the Remote Network Abstraction Layer DLL, RNAL.DLL.  It has the same APIs as the other NALs, plus some calls specific to the remote agent.  Communication is peer-to-peer, even though the manager and agent have different roles.

### Interface to NetBIOS

The DLL BHNETB.DLL services NetBIOS requests.

### Control Panel Applet for Agent

The Control Panel Applet allows the user at the remote machine to set the password to restrict access to the remote agent.  Eventually, this applet may also allow the user to specify the protocol to be used and to configure the agent.

### Agent Status Tool

AGENTUI.EXE:  This application will be useful on the agent machine for checking the current status of the agent.  It will be made available separately because it is an unsupported utility.

## Communication

The manager communicates with the remote agent using RMB over NetBIOS.  RMB stands for RNAL Message Block.  It is much like SMB, having a static header followed by variable fields.  Commands are recorded as sequences of ASCII strings, so if RMB packets are captured by Network Monitor, the fields can be read using the Hex pane.  No parser currently exists for decoding RMB.

The decision was made to run over NetBIOS because of the limitations of many OS/2 RAS Servers currently in use, which do not support other protocols.  In the future, the remote agent communication will be more flexible.

# Using the Network Monitoring Agent

## Installation

### How to install the Network Monitoring Agent

Installing the Network Monitoring Agent varies depending on how Network Monitor was obtained.

If a recent version of Windows NT or the full Network Monitor UI is installed and running on a machine, then the Network Monitoring Agent is already installed, and the agent service can be started.

If you have SMS, you can create a package and send a job to install the Network Monitoring Agent on any NT machine.  A special Network Monitoring Agent installation program has been written to make installation over the network easy. See the SMS documentation to learn how to do this.

### Where to install the Network Monitoring Agent

For Network Administrators:  to get full coverage of the machines supported, install the Network Monitoring Agent at least once on each subnet. With SMS, you can determine which machines are on which subnet by querying all machines either by subnet mask or by default gateway.  Otherwise, track the subnet information manually and determine where to put the agent.

Alternatively, if you install the agent on each router or bridge in the network, this would ensure coverage.  This may be confusing, because if the router connects two real subnets and has two network cards attached to two different networks, then you need to know which network card to capture on.  If the router connects two virtual subnets and has only one network card, then it will receive each packet and resend it on the same card.  When you capture using that card each message will appear twice, and it may be unclear which messages are coming and which are going.

It would be a good idea to install the agent on machines where you have administrative privileges so that the event viewer can be accessed remotely.  This ability is useful in troubleshooting (see  Troubleshooting).

## Capturing with the Network Monitoring Agent

Capturing with the agent looks just the same as capturing locally, except that statistics are updated less frequently.  Once the connection is established, filters and triggers can be set, buffer size and frame size chosen, and each of these items will function as if you were activating them locally.

### Connecting

In order to capture on a remote agent, the user has to connect to the remote agent and specify the remote agent as the network to capture on.

Choose Networks from the Capture menu in the Capture window.  A dialog will appear with a line for every network available plus a line for a remote connection.  Choose the remote line and press “OK”.  In the Connect To Network Monitoring Agent dialog, enter the name of a remote machine (e.g. \\AGENT) with the Network Monitoring Agent service running, enter a comment, and press Connect.

If connection succeeds, Network Monitor will offer a choice of network cards on the remote agent.  If the remote agent has more than one network card, capturing may be performed on any card that supports Network Monitor.  Choose a network card.

The Network Information dialog will return; press OK and a capture can be performed on the remote machine.  If the connection is not successful (the specified machine may not exist, may not have the Network Monitoring Agent, or could refuse the connection) then an error message will appear.

Network Monitor can connect to the remote agent by the LAN or by RAS.  Connecting to the remote agent by RAS is explained in “Connecting over RAS” below.

#### Setting a comment

Whenever a connection is established  to a remote agent, it is good practice to enter as a comment your name and a way to reach you, such as your phone number.  This can come in handy in several different scenarios.  If a user wants to connect to a remote machine that you are currently connected to, they will see a message with your comment, and can get in touch with you to ask you to disconnect.  Or, if somebody connects to an agent that you have suspended, and inadvertently collects all the data, they can save the data and contact you.

#### Buffer size

The capture buffer is the temporary memory assigned to hold the frame data during and after the capture.  The default buffer size is 1 MB, but it can be changed through the Capture Buffer Settings dialog (choose Buffer Settings from the Capture menu in the Capture window).  When you start a new capture, the old capture in the buffer will be dropped unless you are currently viewing that data.  The same buffer size is always allocated on both machines.  If you set a buffer size on the local machine, then connect to a remote machine that can’t handle that buffer size, you will be prompted at this point to change it.  Frames are always stored in the temporary capture buffer on the remote machine until the user saves the capture on the local machine.

### Suspending the Agent

To suspend the remote machine, choose Networks from the Capture menu in the Capture window.  In the Network Information dialog, choose Suspend.  This option is only available if a capture is currently running on the remote machine.  When you choose Suspend, you will disconnect from the remote machine and lose any unsaved views of captured data, but the capture will not be stopped and the agent will not go through its normal cleanup procedure.  Instead, the agent will continue capturing until somebody reconnects to it, or until it hits a ‘stop capture’ trigger.  If the remote machine stops capturing due to a trigger while it is suspended, then it will hold onto the current capture until somebody connects to it.

### Reconnecting

When you reconnect to a suspended machine, you will receive a message including the connection comment of the last person to connect to that machine.

Reconnect will always reconnect you to the  suspended capture, even if you did not start the capture. If you accidentally connect to a suspended agent with a capture currently running, it would be polite to suspend the connection again so that the original manager can reconnect and continue with their work.  If you connect to a suspended agent where the capture has stopped due to a trigger, then the capture information will be sent to you.  Save the capture and notify the person who last connected to the remote machine.  If the capture is not saved, the data will be lost when you disconnect (suspend is not available because the capture is already stopped).

If the manager machine connecting to a suspended machine has a buffer size set too large for the remote machine, then the buffer size will be reduced on the local machine to match the remote buffer size.  If the manager machine has a buffer size set too small for the remote machine and the manager cannot allocate a buffer large enough, connection will be refused.

### Disconnecting

When the agent is disconnected, if any views of unsaved data are open, you are prompted to save the data.  If you do not save the data, the views will be lost:  this data was stored on the remote agent, not locally, so after disconnection it is no longer available.  In fact, when the agent is disconnected, it goes through its cleanup procedure and deallocates all buffers, so the data is gone.  Any data which was already brought over can still be saved.

### Agent Status

The network monitoring agent has three different states from the point of view of the local machine:  It can be connected, suspended, or disconnected.

In the connected state, the local machine has control of the remote agent.  Nobody else can connect to the agent.  The local machine cannot open another connection to the remote agent:  only one connection per agent is allowed at any time.  The manager can now start a capture to get information from the remote machine’s subnet.

To suspend a connection means to disconnect from the agent without stopping the capture, so the local user can only suspend the agent if a capture is currently being performed.  The capture continues on the remote agent until a manager connects to it and stops the capture, or until a trigger stops the capture.  When a manager connects to a suspended agent, the session information (capture statistics) is sent from the agent.  Any manager can connect to a suspended agent if they know the password.

The agent is disconnected if the connection is lost or if the manager disconnects from the remote agent.  When the agent is disconnected, capturing is stopped and the agent performs its cleanup.  Capture data is stored in a capture buffer, temporary memory set aside for the capture, and when the agent is disconnected this buffer is freed and the data is lost.

## Triggers

Triggers can be set through the Capture Trigger dialog (choose Trigger from the Capture menu in the Capture window).  The trigger will fire whether the connection is active or suspended.

When capturing locally, the trigger action occurs locally, but when capturing on a remote machine, the trigger action (stop capture or execute command) takes place on the remote machine only, not on the local machine.  When specifying a command to execute, make sure that the executable is available on the remote machine.

### Trigger Behavior with Suspend

If you set a trigger on a remote machine that executes a command or does nothing, and the connection is suspended, when you reconnect the remote machine will still be capturing.  Alternatively, you could set a trigger to stop the capture, but if the trigger condition is never met, when you reconnect the remote machine will still be capturing. The capture can then be stopped normally.

If a ‘stop capture’ trigger fires while the connection is suspended, you will not be notified.  The agent will stop capturing and wait for reconnection.  When any manager reconnects, it will send the last set of statistics gathered.  The data can then be viewed normally.

## Connecting over RAS

To connect to a remote machine over RAS, first establish the connection to a RAS server separately using the RAS user interface.  Then the machines available from the RAS server can be used for remote capturing if they have the network monitoring agent installed.  Just enter the name of the machine with the network monitoring agent in the Network Information dialog (choose Networks from the Capture menu in the Capture window). You may wish to select the “Slow Link” option in this dialog.  This will greatly increase the time-out value, the amount of time the remote machine waits before it thinks the connection has been lost and performs its cleanup procedures.

The RAS Server may also serve as the remote capturing machine if it has the network monitoring agent installed and the service running.

## Capturing on a different Subnet

If you are capturing on one subnet (either locally or on a remote machine), and looking at frames going to and from a machine on a different subnet, you may not be capturing all the frames that machine sees.  You may only be able to capture connection traffic, and only that connection traffic which must pass through the subnet you are capturing in.  Broadcasts and multicasts will not usually be forwarded by routers or bridges.

To get all the traffic going to and from a target machine, try to find a network monitoring agent on a machine on the same subnet as the target machine.

With SMS, the user can choose which agent to capture on to get the frames desired.

- Determine which machine is having a problem:  target machine

- Determine subnet of target machine by IP address and subnet mask

- From list of available agents, choose one on the same subnet

If there is not an agent available on the same subnet, try determining if the problem occurs when the target machine communicates with a particular other machine, for example if the problem occurs during print jobs while the target machine communicates with the print server.  Repeat steps 1 to 3 with the print server instead of the target machine.

Even if a capture is performed on the same logical subnet as the target machine, you still may not see all of the traffic if learning bridges are in place.  To be absolutely certain of capturing all traffic to or from a target machine, perform the capture on the target machine itself.  As long as the network card on the target machine supports loopback, you will be able to capture all the frames it sends and receives.

If you suspect you are not seeing all the frames you are interested in, make sure the capture filter and display filter are set correctly.

## Capturing on a router

If you install the Network Monitoring Agent on a router, you will be able to capture traffic on either side of the router, or both sides.  However, interpreting the frames captured on a router can be difficult, and there are two different cases to keep in mind.

### Router separates two physical subnets

Having a Network Monitoring Agent installed here would be very useful.  If you know which network card on the router is connected to which physical network, then you can connect to the router to start a capture, and capture on either network.

There is no way to capture on both of the router’s network cards simultaneously from another machine.  The Network Monitoring Agent only supports one connection.  To capture on both sides of a router simultaneously to compare information, capture on different machines on each subnet.  (To do this from one local machine, launch two instances of Network Monitor, and connect each instance to a different remote machine).  Set the capture trigger to stop the capture at the same time on both remote machines.  Alternatively, you could install the full Network Monitor UI on the router, and launch two instances of Network Monitor on the router, capturing on a different network with each instance of Network Monitor.

### Router separates two virtual subnets

This is a router with only one network card.  It routes frames that are all on the same physical subnet, but the router is still needed:  the machines can’t see each other because of their addresses.  If you capture on the router, then all frames passing over the router should be there twice:  once coming in, once going out.  This may be confusing, because it is difficult to tell which subnet the frame is on. To keep the frames on each subnet separate, perform two separate captures on one machine in each subnet.

## When to use Suspend

### Over an expensive link

If you are capturing over an expensive link, such as RAS over a long-distance telephone line, suspend the remote connection to save money.  Reconnect to the remote machine when the capture is done to save it to disk.

### Using an uncertain link

- Suspend the connection during capture

If you are using an uncertain link, suspend the remote machine as soon as possible after capturing begins. In this way, the agent will keep on capturing. If you lose the connection by accident, the agent will stop capturing and clean up, losing all data.

- Save as soon as possible after capture

If you are viewing data and lose the connection by accident, the data will be lost unless you have already saved the capture.  Therefore, when the capture finishes on the remote agent, save it immediately, then view it.

## Capturing on multiple agents

It is not possible to have more than one connection or more than one capture on the same agent.  Subsequent connections are refused, and the Network Monitor User Interface does not allow a second capture.  However, captures can be performed on different agents simultaneously.  To do this on the same local machine, launch multiple instances of Network Monitor, one for each capture.

## Security

The Network Monitoring Agents have password security:  a password can be set on the remote agent only, and will be asked for when any manager tries to establish a connection.  This password is originally set null, but can be changed using the Network Monitoring Agent control panel applet.

Because of security concerns, we have disabled the transmit frame functionality on remote machines.  There are two reasons for this decision.  First, users should not be able to transmit frames into somebody else’s subnet.  Second, if users transmit too many frames, they could flood the net and be unable to stop the transmission.

# Troubleshooting

This section concerns debugging problems with the Network Monitoring Agent itself, either connection problems or status problems.

## Event Logging

The easiest way to find out what the Network Monitoring Agent is doing is to view the event log.  If you have administrative privileges on the remote machine, you can view the remote event log on your local machine.  Otherwise, there must be somebody physically present at the remote machine to view the event log.

No setup is required to start event logging.  The Network Monitoring Agent will detect if the registry does not have the correct keys, and will insert them.

There are nine messages sent to the event log.

- The Network Monitoring Agent service has successfully started.

- The Network Monitoring Agent service has successfully stopped.

- The Network Monitoring Manager <manager_machine> has connected.

- The Network Monitoring Manager has disconnected; the currently pending capture operation is continuing.

- The Network Monitoring Manager has disconnected; any currently pending operations have been stopped.

- The Network Monitoring Agent has failed to start.  The Agent found no local network drivers to bind to.

- Network Monitoring is not installed correctly.

- The Network Monitoring Agent has failed to start.  The Agent failed registration.  The return code is <Transport-specific return code>.

- Note:  This return code may be used by Microsoft Product Support in determining the cause of the problem.

- The Network Monitoring Agent has failed to start.  The Agent could not find the NalAgentProc() entry point in RNAL.DLL

## Network Monitoring Agent User Interface

There is a small utility, AGENTUI.EXE, which may be helpful in troubleshooting the Network Monitoring Agent, or just to find out the status of the agent and its connections.  This utility will not be shipped with the product but will be made available separately as an unsupported utility.

The utility is run from the command line on the remote agent, and will report on the current status of the Agent's connection and capture contexts. The report contains at least three lines of information:

- Connection

The connection status of the agent can be either IDLE or ACTIVE.

- Last User Name

The ‘Last User Name’ field will contain the name of the agent machine if nobody has connected since the agent service started.  Otherwise, the field will contain the machine name of the last machine that connected.

- Capture

Capture status may be “Capturing” or “Not Capturing or Capture Stopped”.  This has no relationship to the status of the connection.  If the capture status is “Capturing” and the connection status is “IDLE”, then the agent has been suspended.

In addition there may be a fourth line of information indicating whether there is a trigger pending or whether a trigger has fired.

## Connection Problems

If there are problems connecting to the remote machine, there are several items to verify and several remedial actions to take.

- If the connection is refused because another manager machine is already connected to the remote machine, you will be notified when you try to connect and will see the comment entered by the currently connected manager.

- If the connection is refused because you do not know the password for the remote machine, you will see an error message.

- Check the name of the remote machine.  Be sure that this machine exists, and has the Network Monitoring Agent installed.

- Verify that the Network Monitoring Agent is running as a service on the remote machine.  Type ‘net view <agent>’ on the command line from the manager machine.  This will also verify that the local machine can see the remote machine over the network.

- Or, type ‘net start’ on the remote machine to see if the agent is in the list of started services.  If it is not, type ‘net start nmagent’ to start the service, and check again to verify that it started correctly.

- If  you can’t see the agent service running on the remote machine, check the event log on the remote machine to see if the agent failed registration.

- Do a NetBIOS Adapter status on the agent to see if the name of the remote agent is registered correctly.  You should see the name of the agent machine plus at least one special character.

- If you can ‘net view’ the agent but can’t connect through Network Monitor, the problem might be with the LAN Adapter lists.  The manager and remote agent only use the first four LAN Adapters listed.  Check the LAN Adapter list on both the local machine and the remote machine, using the Control Panel Networks applet.  Make sure that the lists have at least one LAN Adapter entry in common between 0 and 3.

Use Network Monitor to sniff the connection.  Open two instances of Network Monitor at once on the local machine.  With one instance, set a filter to look at any frames coming into or leaving your machine:  “ANY <--> local_machine“, and start the capture.  With the other instance, try to connect to the agent.  Look at the capture to see if you can isolate the problem.  This will only work if your network card driver supports loopback.  If the network card driver does not support loopback, the instance of Network Monitor performing the capture will have to be on another machine.