========  NOTES ON RELEASE 620B  ========

Include versions

PS620B.U24 & PS620B.U25: for S+ model with 2M EPROM

MS620B.U24 & MS620B.U25: for S model with 1M EPROM or MS model

PE620B.U24 & PE620B.U25: for E+ model with 1M EPROM

ME620B.U24 & ME620B.U25: for E  model with 1M EPROM

PB620B.U24 & PB620B.U25: for B+ model with 1M EPROM

MB620B.U24 & MB620B.U25: for B  model with 1M EPROM

CP620B.U12 & CP620B.U13: for U-1496P with 1M EPROM (or flash ROM)

How to modify U-1496 modem to accomodate 2Mbit EPROM's ?

. The 1Mbit EPROM has 32 pins the same as 1Mbit EPROM does.

In order for the existing U-1496 series modem to accept 2Mbit EPROMs,

Singal connections on pin 30 and pin 31 of U24 and U25 need to be

modified as follows :

. Pin 30 of the orginal socket accomodating 1Mbit EPROM needs to

be removed.

. Install a 32 pin socket onto the top of the orginal socket.

. Solder a wire to connect pin 30 of new socket to pin 46 of

U26 (Mc68000).

. Solder a jumper-wire to short pin 31 and pin 16 of the socket.

. Put the new firmware on the add-on socket and do a power-on test.