DigiBoard

6400 Flying Cloud Drive

Eden Prairie, MN  55344

(612) 943-9020

Installation Guide

and

Reference Manual

for

DigiCHANNEL C/X System

Novell AIO Driver

DigiBoard, DigiCHANNEL and C/X are trademarks of DigiBoard.  NetWare, NetWare

Access and Novell are registered trademarks of Novell, Incorporated.  All

other brand and product names are the trademarks of their respective holders.

Copyright c Digi International, Inc. 1992

All Rights Reserved

Information in this document is subject to change without notice and does not

represent a commitment on the part of DigiBoard.

DigiBoard provides this document "as is", without warranty of any kind, either

expressed or implied, including, but not limited to, the particular purpose.

DigiBoard may make improvements and/or changes in this manual or in the prod-

uct(s) and/or the program(s) described in this manual at any time.

This product could include technical inaccuracies or typographical errors.

Changes are periodically made to the information herein; these changes may be

incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used, disclosed,

and/or copied only as permitted under such license agreement.  Any copy must

contain the above copyright notice and this restricted rights notice.  Use,

copying, and/or disclosure of the programs is strictly prohibited unless

otherwise provided in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to restrictions

as set forth in sub-paragraph (c)(1)(ii) of the Rights in Technical Data and

Computer Software clause of DFARS 52.227-7013.

1.0 Introduction

1.1 Overview

DigiBoard's support for Novell compatible asynchronous applications is

provided through a universal driver conforming to the "AIO" (Asynch-

ronous Input/Output) specification.  Use the AIO driver when DigiBoard

hardware is being installed in the file server (or machine with run-time

NetWare 3.X). When DigiBoard hardware is installed in a machine other

than the server (for instance NetWare Access Server V1.2) use the spec-

ific driver for that application.

AIO drivers are loaded on the file server from the system console (or

RCONSOLE) command line or the AUTOEXEC.NCF file.  Installation is simply a

matter of copying of the driver files AIODGCX.NLM and AIODGMEM.NLM, and

the configuration utility, AIOCXCFG.NLM.

1.2 Compatibility

The DigiBoard AIO driver supports the ISA, EISA and MCA versions of the

DigiCHANNEL C/X system and therefore can be used on all three platforms.

The driver will support up to 4 DigiCHANNEL C/X host adapters on ISA and

EISA bus servers.  Each host adapter may have up to eight DigiCHANNEL

C/CON-16 concentrators attached, for a total of up to 128 ports per host

adapter. These DigiCHANNEL host adapters may be configured to use the

same memory address ("stacked") or may be configured for unique address-

es.

On MCA bus servers the driver will support up to 4 DigiCHANNEL C/X host

adapters, however, your server may be limited to 2 boards due to confl-

icts with the network adapter card and ESDI hard disk controller.

Concentrators may be connected locally (direct wiring), or remotely (via

high-speed synchronous modems).  This driver supports one modem pair

per synchronous line, and the modems must be installed between the host

adapter and the first concentrator (i.e. local and remote concentrators

may not be combined on a single synchronous line).

2.0 Installation and Configuration

2.1 Copying the Files

To install the DigiBoard AIO driver, simply copy the relevant files to

your file server.  The DigiBoard AIO files (AIODGCX.NLM, AIOCXCFG.NLM

and AIODGMEM.NLM) should be copied to the system directory of the file

server, typically called F:\SYSTEM.  Copy the files from a workstation

to the file server with the commands:

COPY A:\AIODGCX.NLM F:\SYSTEM

COPY A:\AIOCXCFG.NLM F:\SYSTEM

COPY A:\AIODGMEM.NLM F:\SYSTEM

2.2 Configuring the AIO Driver

Before the AIO driver can be loaded for the first time, it must be con-

figured to match the DigiCHANNEL C/X System hardware you have installed

in your server.  The configuration program, AIOCXCFG, rewrites the driver

file, AIODGCX.NLM, with the new configuration data.  The new configura-

tion becomes permanent, and can only be changed by re-running the config-

uration utility.

To configure the AIO Driver, enter the following command from the server:

load aiocxcfg

You will see the banner for the configuration utility:

+---------------------------------------------------------------------------+

| DigiBoard C/X AIO Driver Configuration Utility                     V 1.00 |

+---------------------------------------------------------------------------+

Below the banner, the current driver configuration will be displayed. On

ISA and EISA servers, the configuration fields will be empty, as shown

below:

+---------------------------------------------------------------------------+

|                                                                           |

|                      DigiBoard Driver Configuration                       |

|                                                                           |

|   Brd Type     I/O Addr     Mem Addr     #Concs     Brd#     Brd Name     |

|                                                                           |

|                   AIODGCX.NLM configured for 0 Boards                     |

|                                                                           |

+---------------------------------------------------------------------------+

On Micro Channel servers, which read board configuration information

directly from the POS, the configuration fields will already be filled

in. The number of concentrators will be automatically set to a default

value of 1, and the board name will be set to "DigiCX00" (if multiple

host adapters are present, they will be named "DigiCX01", "DigiCX02"

and "DigiCX03"):

+---------------------------------------------------------------------------+

|                                                                           |

|                      DigiBoard Driver Configuration                       |

|                                                                           |

|   Brd Type     I/O Addr     Mem Addr     #Concs     Brd#     Brd Name     |

|                                                                           |

|    MCA         Port 108h    0D8000h         1        0       DigiCX00     |

|                                                                           |

+---------------------------------------------------------------------------+

Below the configuration display a box will appear listing the available

configuration options:

+----------------------------+

|   Available Options        |

+----------------------------+

| >| Add Board               |

|  | Delete Board            |

|  | Change Board Parameters |

|  | Configure Synch Lines   |

|  | Set Altpin Option       |

|  | Set AIO Version         |

|  | Exit                    |

+----------------------------+

(On Micro Channel servers, the "Add Board" and "Delete Board" options

will be absent, since the driver configuration program takes its infor-

mation directly from the POS.)

2.2.1 Adding Boards

Step 1.

To add an ISA or EISA host adapter to the configuration, use

the up and down arrow keys to position the pointer next to "Add

Board" and press <Enter>.  The configuration program will put up

a series of menus requesting information about the host adapter

and synchronous line configuration.  These are the same menus

that come up if you select "Change Board Parameters" and

"Configure Sync Lines" from the main "Available Options" menu.

To add a Micro Channel host adapter, simply install the board

(follow the procedure in the Installation Guide and Reference

Manual for the Micro Channel host adapter) and run the Automatic

Configuration program on the IBM reference diskette.  Skip steps

2 and 3, below, and proceed to step 4.

Step 2. I/O Options (ISA Host Adapters)

You will first be asked to specify the board's I/O port address

as set on the board's DIP switches):

+---------------+

|  I/O Options  |

+---------------+

| >| Port 108h  |

|  | Port 118h  |

|  | Port 128h  |

|  | Port 208h  |

|  | Port 228h  |

|  | Port 308h  |

|  | Port 328h  |

+---------------+

The following table shows the DIP switch settings for the I/O

addresses:

Address  SW1 SW2 SW3

-------  --- --- ---

108h     OFF OFF ON

118h     OFF ON  OFF

128h     OFF ON  ON

208h     ON  OFF OFF

228h     ON  OFF ON

308h     ON  ON  OFF

328h     ON  ON  ON

If you are installing an ISA host adapter in an EISA server, you

will also be asked for the number of the slot in which the board

resides.

Step 2. I/O Options (EISA Host Adapters)

To add an EISA host adapter to the configuration, use the up and

down arrow keys to position the pointer next to "Add Board" and

press <Enter>.  You will now be asked to specify the board's slot

number:

+---------------+

|  I/O Options  |

+---------------+

| >| Slot 1     |

|  | Slot 2     |

|  | Slot 3     |

|  | Slot 4     |

|  | Slot 5     |

|  | Slot 6     |

|  | Slot 7     |

|  | Slot 8     |

+---------------+

Step 3. Memory Options (ISA and EISA)

Now you will be asked to select a starting address for the

board's shared memory.  Multiple C/X host adapters may use the

same starting address.  Note that while there are 14 address

options, many of these will be unavailable due to other memory

and peripheral considerations.  The addresses most likely to be

available are 0D8000h and 0D0000h.

+----------------+

| Memory Options |

+----------------+

| >| 080000h     |

|  | 088000h     |

|  | 090000h     |

|  | 098000h     |

|  | 0A0000h     |

|  | 0A8000h     |

|  | 0B0000h     |

|  | 0B8000h     |

|  | 0C0000h     |

|  | 0C8000h     |

|  | 0D0000h     |

|  | 0D8000h     |

|  | 0E0000h     |

|  | 0E8000h     |

+----------------+

Step 4. Number of Concentrators

On ISA and EISA servers, the configuration program will now ask

for the number of concentrators attached to the host adapter.

Select 1 to 8 concentrators by moving the pointer to the desired

number and press <Enter>.

+--------------------+

|   Concentrators    |

+--------------------+

| >| 1 Concentrator  |

|  | 2 Concentrators |

|  | 3 Concentrators |

|  | 4 Concentrators |

|  | 5 Concentrators |

|  | 6 Concentrators |

|  | 7 Concentrators |

|  | 8 Concentrators |

+--------------------+

If the server is a Micro Channel machine, select "Change Board

Parameters" from the main ("Available Options") menu, then select

"# Concentrators" to get this screen.

Step 5. Host Adapter Sync Line Configuration

Now you need to set up the host adapter's synchronous line

parameters.  These include the wiring mode (4 or 8 wire),

Transmit Clock mode (Internal or External) and the baud rate.

The following screen will come up automatically for ISA and EISA

servers; for Micro Channel servers, select "Change Board

Parameters" from the main ("Available Options") menu, then select

"Config Sync Lines", and finally, select "Host Adapter":

+--------------------------------------------+

|  C/X Host Adapter Sync Line Configuration  |

+--------------------------------------------+

|                                            |

|  Line     Mode     TX Clock     Speed      |

|                                            |

|    1   > 8-Wire    Internal     1.2M       |

|    2     8-Wire    Internal     1.2M       |

|                                            |

|                              <ESC> to Quit |

+--------------------------------------------+

Step 5a. Wiring Mode

To change the wiring mode, use the up, down, left and right arrow

keys to place the pointer next to the wiring mode you wish to

change, then press <Enter>.  For maximum throughput under

moderate to heavy loads, 8-Wire mode is recommended.  If the

concentrators are to be located at a remote site and connected to

the host adapter via high-speed synchronous modems, 8-Wire mode

is required.

Step 5b. Transmit Clock Mode

To change the Transmit Clock mode, use the up, down, left and

right arrow keys to place the pointer next to the TX Clock mode

you wish to change, then press <Enter>.  Select Internal for

local (hard-wired) concentrators, and External for remote

concentrators (connected via high-speed synchronous modems).

Step 5c. Line Speed Selection

Now choose the baud rate for the synchronous lines.  Use the up,

down, left and right arrow keys to place the pointer next to the

Line Speed you wish to change, then press <Enter>.  If you chose

8-Wire, Internal clocking above, the recommended Line Speed is

1.2 megabaud ("1.2M").  If you chose 4-Wire, Internal clocking,

the maximum baud rate is 460K.  If you are using high-speed

synchronous modems to connect to remote concentrators, use the

fastest baud rate supported by your modems.

+-------------------+

| Select Line Speed |

+-------------------+

|  | 2400           |

|  | 4800           |

|  | 9600           |

|  | 19.2K          |

|  | 38.4K          |

|  | 57.6K          |

|  | 76.8K          |

|  | 115K           |

|  | 230K           |

|  | 460K           |

|  | 920K           |

| >| 1.2M           |

+-------------------+

Step 6. Concentrator Sync Line Configuration

Next, you will be asked to set up the concentrator's sync line

parameters (this will not affect the sync line parameters you set

for the host adapter).  You will see a screen similar to the

following:

+--------------------------------------------------+

|     C/X Concentrator Sync Line Configuration     |

+--------------------------------------------------+

|                                                  |

|           Line     Mode     TX Clock     Speed   |

|                                                  |

|  Conc 1  > 1      8-Wire    Internal     1.2M    |

|  Conc 2    1      8-Wire    Internal     1.2M    |

|  Conc 3    1      8-Wire    Internal     1.2M    |

|  Conc 4    1      8-Wire    Internal     1.2M    |

|  Conc 5    2      8-Wire    Internal     1.2M    |

|  Conc 6    2      8-Wire    Internal     1.2M    |

|  Conc 7    2      8-Wire    Internal     1.2M    |

|  Conc 8    2      8-Wire    Internal     1.2M    |

|                                                  |

|                                    <ESC> to Quit |

+--------------------------------------------------+

Note that only the Line number and Speed can be changed, and that

the speed of the last concentrator on each line will always

reflect the host adapter's speed setting for that line.

Step 6a. Sync Line Selection

By default, concentrators 1 through 4 will be placed on Line 1,

and concentrators 5 through 8 will be placed on Line 2.  To

redistribute the concentrators, use the up, down, left and right

arrow keys to move the pointer to the Line number of the

concentrator you wish to change, and press <Enter>.  Note that

configuring any concentrator for Line 1 will automatically change

all lower-numbered concentrators to Line 1, and setting any

concentrator to Line 2 will automatically change all higher-

numbered concentrators to Line 2.

Step 6b. Line Speed Selection

By default, all concentrators will be assigned the maximum line

speed for their wiring mode (1.2 megabaud for 8-wire and 460K for

4-wire), except the last concentrator in the daisy chain, which

will be set for the same line speed as the host adapter line to

which the concentrators are attached.  Under normal circum

stances, this setting should not be changed (changing the line

speed will degrade the performance of all concentrators connected

to this host adapter synchronous line).

Step 7. Changing the C/X Host Adapter's Name

You will now be given the oportunity to change the name of the

host adapter.  By default, host adapters are named "DigiCX00"

through "DigiCX03".  For Micro Channel servers, select "Change

Board Parameters" from the main ("Available Options") menu, then

select "Host Adapter Name" to get this screen:

+------------------------------------+

|    Change C/X Host Adapter Name    |

+------------------------------------+

|                                    |

|   Current Name   : DigiCX00        |

|                                    |

|   Enter New Name :                 |

|                                    |

|   <ENTER> to Accept Current Name   |

|                                    |

+------------------------------------+

To keep the default name, simply press <Enter>.

Now press <Esc> to return to the main ("Available Options") menu.

This completes the procedure for adding host adapters to the config-

uration.

2.2.2 Removing Boards

To remove an ISA or EISA host adapter from the configuration, select

"Delete Board" from the main ("Available Options") menu.  Then, in the

Driver Configuration box, move the pointer to the board you wish to

remove, and press <Enter>.

+---------------------------------------------------------------------------+

|                                                                           |

|                     DigiBoard Driver Configuration                        |

|                                                                           |

|   Brd Type     I/O Addr     Mem Addr     #Concs     Brd#     Brd Name     |

|                                                                           |

|  > ISA         Port 108h    0D8000h         8        0       DigiCX00     |

|    ISA         Port 118h    0D8000h         8        1       DigiCX01     |

|    ISA         Port 208h    0D8000h         8        2       DigiCX02     |

|    ISA         Port 218h    0D8000h         8        3       DigiCX03     |

|                                                                           |

+---------------------------------------------------------------------------+

The "Delete Board" option is not available for Micro Channel servers,

since the driver configuration program takes its information directly

from the POS.  To delete a board from a Micro Channel server, remove

the board from the computer and run the configuration utility on your

IBM reference diskette, then reload the DigiBoard AIO driver config-

uration utility.  The board you removed should no longer appear in the

driver configuration box.

2.2.3 Enabling and Disabling the AltPin Feature

This feature internally swaps the signals DCD (Data Carrier Detect)

and DSR (Data Set Ready).  This is useful for concentrator models that

are equipped with RJ-45 modular jacks for the serial connections.  The

DCD signal is normally on pin 10 of the RJ-45 jack, which makes it

inaccessible to 8-pin RJ-45 plugs.

When the AltPin feature is enabled, pin 2 of the 10-pin RJ-45 connector

will be interpreted by the driver as the DCD signal.  This corresponds

to pin 1 of an 8-pin RJ-45 connector.  Pin 10 then becomes DSR.  Be

sure that your cabling reflects this configuration.

To configure the AIO driver to use the alternate pin assignments, sel-

ect "Set AltPin Option" from the main ("Available Options") menu, then

select the board from the Driver Configuration box:

+---------------------------------------------------------------------------+

|                                                                           |

|                     DigiBoard Driver Configuration                        |

|                                                                           |

|   Brd Type     I/O Addr     Mem Addr     #Concs     Brd#     Brd Name     |

|                                                                           |

|  > ISA         Port 108h    0D8000h         8        0       DigiCX00     |

|    ISA         Port 118h    0D8000h         8        1       DigiCX01     |

|    ISA         Port 208h    0D8000h         8        2       DigiCX02     |

|    ISA         Port 218h    0D8000h         8        3       DigiCX03     |

|                                                                           |

+---------------------------------------------------------------------------+

Now you will be shown a list of the concentrators attached to that

host adapter.  Choose the concentrator you wish to configure for

AltPin by positioning the pointer and press <Enter>.

+---------------------------------------------------+

| Select C/X Concentrator for AltPin Configuration. |

+---------------------------------------------------+

|                                                   |

|            Line     Mode     TX Clock     Speed   |

|                                                   |

| > Conc 1     1     8-Wire    Internal     1.2M    |

|   Conc 2     1     8-Wire    Internal     1.2M    |

|   Conc 3     1     8-Wire    Internal     1.2M    |

|   Conc 4     1     8-Wire    Internal     1.2M    |

|   Conc 5     2     8-Wire    Internal     1.2M    |

|   Conc 6     2     8-Wire    Internal     1.2M    |

|   Conc 7     2     8-Wire    Internal     1.2M    |

|   Conc 8     2     8-Wire    Internal     1.2M    |

|                                                   |

|                                     <ESC> to Quit |

+---------------------------------------------------+

You will now see a list of the ports on the selected concentrator, and

their AltPin status.  Use the up, down, left and right arrow keys to

position the pointer by the port you wish to change.  Press <Enter> to

toggle the AltPin status of the selected port.

+---------------------------------------------------+

|                Port AltPin Status                 |

+---------------------------------------------------+

|                                                   |

|          Host Adapter 0  Concentrator 1           |

|                                                   |

|   > Port  1 : Enabled      Port  2 : Disabled     |

|     Port  3 : Disabled     Port  4 : Disabled     |

|     Port  5 : Disabled     Port  6 : Disabled     |

|     Port  7 : Disabled     Port  8 : Disabled     |

|     Port  9 : Disabled     Port 10 : Disabled     |

|     Port 11 : Disabled     Port 12 : Disabled     |

|     Port 13 : Disabled     Port 14 : Disabled     |

|     Port 15 : Disabled     Port 16 : Disabled     |

|                                                   |

| <ENTER> to toggle Status            <ESC> to Quit |

|                                                   |

+---------------------------------------------------+

2.2.4 Selecting the AIO Version

The AIO driver needs to be configured for the version of AIO you are

using on your server (the version number is displayed in a banner

message when you load AIO on the server).  From the main ("Available

Options") menu, select "Set AIO Version".  You will see the following

menu:

+----------------------------+

|     AIO Version : V1.02    |

+----------------------------+

| >| V1.02 and earlier       |

|  | V1.03 and later         |

+----------------------------+

Move the pointer to the appropriate line and press <Enter>.

NOTE:  Depending upon the AIO version, the board number ("Board #" in

the Driver Configuration box) will vary.  In versions 1.02 and

earlier, board numbers are assigned sequentially in the order

in which the boards are added to the configuration (in Micro

Channel servers, this is the order in which they appear in the

POS).  In versions 1.03 and later, the board numbers are de-

pendent upon the I/O port addresses (Port 108 = Board 0, Port

118 = Board 1, etc.), and therefore may not appear in sequent-

ial order in the Driver Configuration box.

2.2.5 Saving the Configuration

Once all parameters have been set as desired, select "Exit" from the

main ("Available Options") menu.  When asked if you want to save the

configuration, answer "Y" to save the changes, or "N" to leave the

driver unchanged.

2.3 Loading the AIO Driver

To load the configured AIO driver, enter the following command on the

server's console:

load aiodgcx

Upon successfully loading the driver, the following banner message (or

similar) will appear:

NetWare 3.X AIO DigiBoard C/X Driver

Version 1.00    July 1, 1992

Copyright 1992, Digi International, Inc.  All rights reserved.

Board 0 :  Port 208, Mem D0000 - Initialized successfully.

The driver is now ready for use.

2.4 Changing the AIO Driver's Configuration

To reconfigure the AIO driver, enter the following command on the

server's console:

load aiocxcfg

You will be shown the current configuration, and the "Available Options"

menu.  Make the desired changes (the various menus are described in

section 2.2 of this manual) and save the new configuration.

3.0  Error Messages

If there is a problem with the installation, one or more of the following

error messages may appear after you load the driver.

*Error: Driver Already Loaded.

This message will appear if the driver is presently loaded. To reload

the driver you must first unload the present memory copy of the driver

by entering "unload aiodgcx" at the server console. Then you may reload

the driver.

*Error: Driver Configured for 0 Boards. Configure Driver with AIOCXCFG.NLM.

This message indicates that the driver is configured for 0 boards. You

must "load aiocxcfg" to configure the driver to support at least one

board.

*Error: AllocateIOResourceTag failed.

*Error: Allocate Resource Tag failed.

*Error: AllocateAESResourceTag failed.

*Error: Allocation of PortConfig memory failed.

*Error: Allocation of PortStats memory failed.

These messages indicate that the driver encountered difficulties while

trying to obtain resources from AIO.  This usually indicates that the

system is out of resources.  Unload any unnecessary NLMs and reload the

driver.

*Error: AIORegisterDriver failed (code=xx).

This message indicates that another driver has already registered the C/X

hardware type (possibly illegally).  Unload other AIO drivers and reload

the C/X AIO driver.

Board 0 : Port 208, Mem D0000 - Port 208 not found.

This message indicates that the driver could not find a board with port

208h. Check to insure that you have specified the correct I/O port or

that you have properly configured the board's port address.

Board 0 : Port 208, Mem D0000 - Hardware Reset fails.

This  message indicates that the driver could not place the board into a

reset state.   This is most likely the result of an I/O port address

conflict or a faulty  board.   Be sure that there are no other I/O ports

in your system at the indicated I/O port address.

Board 0 : Port 208, Mem D0000 - AIORegisterBoard failed (code=xx).

This message indicates that an error occured while trying to register the

board with AIO.  This failure is usually the result of no system resources,

or a duplicate board number.  Check the driver configuration for duplicate

board numbers.  If none exist, unload unnecessary NLMs to free system

resources.

Board 0 : Port 208, Mem D0000 - Failed Hardware Registration.

This message indicates that an error occured while trying to register

the board's specific hardware addresses with AIO.

The following error messages are usually the result of memory and/or I/O

address conflicts with other devices within your system.  Insure that no

such conflicts exist.  If errors persist, your board may be defective.

Board 0 : Port 208, Mem D0000 - Memory not found error x.

This  message indicates that the driver could not find the board's memory.

x indicates the number of memory errors found in a 256 byte block.

Board 0 : Port 208, Mem D0000 - Memory error x.

This  message indicates that the driver has found the board's memory, but

it appears to be faulty.  x indicates the number of errors found in a 256

byte block.

Board 0 : Port 208, Mem D0000 - Memory fails disable x.

This message indicates that the board's memory could not be disabled.

Board 0 : Port 208, Mem D0000 - BIOS fails initialization.

This message indicates that the board's on-board BIOS did not initialize

properly.

Board 0 : Port 208, Mem D0000 - NMI Fails during FEPOS execution.

This message indicates a failure during transfer of the on-board operating

system code.

Board 0 : Port 208, Mem D0000 - FEPOS Fails Initialization.

This message indicates that the on-board operating system did not

initialize successfully.

Board 0 : Port 208, Mem D0000 - Command <x> failure during Initialization.

This message indicates that the on-board operating system encountered a

failure during initialization of the asynchronous channels.

Board 0 : Port 208, Mem D0000 - Memory Fails Window Test.

This message indicates that the board fails to window it's memory pages

correctly.