AIOESP.NLM Version 1.00

Hayes Microcomputer Products, Inc.

(January 14, 1993)

This driver allows AIO applications to use the serial ports

on a Hayes ESP Communications Accelerator installed in the server.

Up to two boards, or four ports, are supported. Each board requires

its own interrupt. The driver supports a maximum of 57,600 bps.

1. ESP-AT adapter instructions

To load the AIOESP NLM for use with an ESP-AT adapter, use:

"load aioesp port=xxx int=x dma=x [name=x] [node=x]"

The brackets indicate optional parameters.

REQUIRED load parameters (prompted for if not on command line)

port        Specifies I/O address (in hex) used by ESP adapter. This

address is referred to as the "Enhanced Mode Address"

in the ESP Technical Reference. Valid choices are:

180, 188, 250, 258, 330, 338, 350, 358.

int         Specifies interrupt used by ESP adapter. This interrupt

is referred to as the "Enhanced Mode Interrupt" in the

ESP Technical Reference. Valid choices are: 3, 4, 5, 9.

dma         Specifies DMA channel used by ESP adapter. Valid choices

are: 1, 3.

OPTIONAL load parameters:

name        The user-defined name which correlates to the board loaded.

node        Specifies board number used by ESP adapter. If this

parameter is specified, the driver will assign one based

on I/0 address. Valid choices are: 0-7.

2.  ESP-MCA instructions (MicroChannel)

To load the AIOESP NLM for use with an ESP-MCA adapter, use:

"load aioesp [port=xxx] [name=x] [node=x]"

The brackets indicate optional parameters.

OPTIONAL load parameters

port        Specifies I/O address (in hex) used by ESP adapter. This

address is referred to as the "Enhanced Mode Address"

in the ESP Technical Reference. Using this parameter

means you want to use a specific adapter, the one with the

I/O address specified. If no "port" parameter is specified

the driver will load the next available ESP adapter.

Valid choices are: 2230, 2238, 3230, 3238, 4230, 4238,

5230, 5238.

name        The user-defined name which correlates to the board loaded.

node        Specifies board number used by ESP adapter. If this

parameter is specified, the driver will assign one based

on I/0 address. Valid choices are: 0-7.