Please see READODI.ME for installation instructions using ODIPKT.COM

with ICE.TCP.

James River Group, Inc.

**********************************************************************

ODIPKT(8)                (February 2, 1992)             ODIPKT(8)

NAME

odipkt - translate Open Data-Link Interface to Packet Driver

interface

SYNOPSIS

odipkt [ logical_board [ vector ] ]

DESCRIPTION

ODIPKT.COM provides a Packet Driver API over Novell's Open

Data-Link Interface, allowing a single network adapter to

service IPX, tcp/ip, and other protocol stacks which use

packet drivers.  ODIPKT.COM understands Ethernet, and

ARCnet frame types.  Ethernet and frames are passed unchanged

to packet driver applications, but ARCnet frames are internally

mapped to Ethernet.  Thus, packet driver applications do

not need specific knowledge of ARCnet in order to operate

in such environments.

The logical_board argument indicates which logical adapter

odipkt should bind to.  Under ODI, there is one logical

adapter per frame type.  There may be multiple physical

adapters and multiple frame types per physical adapter.  The

logical adapter number starts at zero and increases by one

for each frame type.  For example, if a system contained two

physical adapters with two frame types each, there would be

four logical adapters numbered 0-3.  The frame types are

specified in the NET.CFG file which must be in the current

directory (or on the APPEND path) when the ODI drivers are

loaded.

Typically, the frame type used by default by Netware on a

given adapter will not be appropriate for tcp/ip.  The

correct frame type for tcp/ip on on Ethernet is almost

always ETHERNET_II.  Assuming an existing installation

with one adapter running the default Netware frame type,

it will usually be necessary to define a second frame type

(as listed above) and specify the logical_board argument

as 1.  An exception to this rule is ARCnet which uses only

one frame type.  Below is a NET.CFG entry for a 3c501

using the default address and interrupt.  In many cases,

this is the only entry required in NET.CFG.  Other

Ethernet adapters would use similar configurations:

Link driver 3c501

envelope type ETHERNET_802.3

envelope type ETHERNET_II

Given the above entry, the logical_board argument of odipkt

would be 1.

Page 1

ODIPKT(8)                (February 2, 1992)             ODIPKT(8)

ARCnet installations require no frame-type

definitions and should use a logical_board of 0.

Vector is the packet driver interrupt at which to provide

service.  The default is 105 and the parameter is decimal.

Odipkt.com should be loaded after lsl.com and the adapter-

specific driver(s).  It must, of course, be loaded before

the packet driver applications which use it.  The Netware

shell and ipxodi.com can be loaded before or after the

odipkt.com program.

AUTHOR

Dan Lanciani

185 Atlantic Road

Gloucester, MA 01930

Page 2