AT Command Reference Manual
for RC32ACW, RC32ACL, and RC96V24AC Modem Families
(Preliminary)

Rockwell International
Digital Communications Division

© 1993, Rockwell International	Order No. 883, Revision 1
All Rights Reserved	July 2, 1993
Printed in U.S.A.

NOTICE

Information furnished by Rockwell International Corporation is believed to be accurate and reliable. However, no responsibility is assumed by Rockwell International for its use, nor any infringement of patents or other rights of third parties which may result from its use. No license is granted by implication or otherwise under any patent rights of Rockwell International other than for circuitry embodied in Rockwell products. Rockwell International reserves the right to change circuitry at any time without notice. This document is subject to change without notice.

ConfiguACE is a trademark of Rockwell International.
MNP is a registered trademark of Microcom, Inc. 
Hayes is a registered trademark of Hayes Microcomputer Products, Inc.

PREFACE

This document is an update to the Advance Copy, dated November 10, 1992. Currently in the review process, this document has not been approved for release by System Engineering.

This document is prepared in Microsoft Word 5.1 for Macintosh.

This page is intentionally blank.

Table of Contents

1.	INTRODUCTION	1-1

1.1.	OVERVIEW	1-1

1.1.1.	Command Syntax	1-1

1.1.2.	Command Descriptions	1-1

1.1.3.	Call Progress and Blacklisting Parameters	1-1

1.1.4.	ConfigurACE II	1-1

1.2.	REFERENCE DOCUMENTATION	1-1

2.	COMMAND SYNTAX	2-1

2.1.	DTE/DCE INTERCHANGE CIRCUITS	2-1

2.2.	COMMAND SYNTAX AND GUIDELINES	2-1

2.2.1.	DTE Commands	2-1

2.2.2.	DTE Command Lines	2-1

2.3.	AT COMMAND GUIDELINES	2-1

2.3.1.	Basic Command Syntax	2-1

2.3.2.	Extended Command Syntax	2-1

3.	AT COMMAND SET	3-1

3.1.	AT COMMAND GUIDELINES	3-1

3.1.1.	AT Commands, DTE Adaption	3-1

3.1.2.	AT Command Format	3-1

3.1.3.	Escape Code Sequence	3-1

3.2.	AT COMMAND SET	3-2

3.2.1.	AT Commands	3-2

A/ - Re-execute Command	3-2

AT= x - Write to Selected S-Register	3-2

AT? - Read Selected S-Register	3-2

A - Answer	3-2

Bn - CCITT or Bell	3-2

Cn - Carrier Control	3-3

Dn - Dial	3-3

En - Command Echo	3-4

Fn - Select Line Modulation	3-4

Hn - Disconnect (Hang-Up)	3-5

In - Identification	3-5

Ln - Speaker Volume	3-6

Mn - Speaker Control	3-6

Nn - Automode Enable	3-6

On - Return to On-Line Data Mode	3-6

P - Set Pulse Dial Default	3-7

Qn - Quiet Results Codes Control	3-7

Sn - Read/Write S-Register	3-7

T - Set Tone Dial Default	3-8

Vn - Result Code Form	3-8

Wn - Error Correction Message Control.	3-8

Xn - Extended Result Codes:	3-8

Yn - Long Space Disconnect	3-9

Zn - Soft Reset and Restore Profile	3-9

3.2.2.	AT& Commands	3-9

&Cn - RLSD (DCD) Option	3-9

&Dn - DTR Option	3-11

&F - Restore Factory Configuration (Profile)	3-11

&Gn - Select Guard Tone	3-11

&Jn - Telephone Jack Control	3-12

&Kn - Flow Control	3-12

&Ln - Leased Line Operation	3-12

&Mn - Asynchronous/Synchronous Mode Selection	3-12

&Pn - Select Pulse Dial Make/Break Ratio	3-13

&Qn - Sync/Async Mode	3-13

&Rn - RTS/CTS Option	3-14

&Sn - DSR Override	3-14

&Tn - Test and Diagnostics	3-14

&V - Display Current Configuration and Stored Profiles.	3-15

&Wn - Store Current Configuration	3-15

&Xn - Select Synchronous Clock Source	3-16

&Yn - Designate a Default Reset Profile	3-16

&Zn=x - Store telephone number	3-16

3.2.3.	AT% Commands	3-16

%En - Enable/Disable Line Quality Monitor and Auto-Retrain or 
Fallback/Fall Forward	3-16

%L - Line Signal Level	3-17

%Q - Line Signal Quality	3-17

%TTn - PTT Testing Utilities	3-17

3.2.4.	AT\ Commands	3-18

\Gn - Modem-to-Modem Flow Control (XON/XOFF)	3-18

\Kn - Break Control	3-18

\Nn - Operating Mode	3-19

3.3.	ERROR DETECTION AND DATA COMPRESSION COMMANDS	3-20

3.3.1.	AT% Commands	3-20

%C - Enable/Disable Data Compression	3-20

3.3.2.	AT\ Commands	3-20

\An - Select Maximum MNP Block Size	3-20

\Bn - Transmit Break to Remote	3-20

\Ln - MNP Block/Stream Mode Select	3-20

3.4.	MNP 10 COMMANDS	3-21

3.4.1.	AT) Commands	3-21

)Mn - Enable Cellular Power Level Adjustment	3-21

3.4.2.	AT- Commands	3-21

*Hn - Link Negotiation Speed	3-21

-Kn - MNP Extended Services	3-21

-Qn - Enable Fallback to V.22 bis/V.22	3-21

@Mn - Initial Cellular Power Level Setting	3-22

:E - Compromise Equalizer Enable Command	3-22

3.5.	W-CLASS COMMANDS	3-23

3.5.1.	AT% Commands	3-23

%Fn - Split-Speed Direction Select	3-23

%Mn - AUXCTL Output Line Control	3-23

3.5.2.	AT* Commands	3-24

*B - Display Blacklisted Numbers	3-24

*C - Remote Configuration Password	3-24

* D - Display Delayed Numbers	3-24

*E - Exit Remote Configuration Mode	3-24

*L - Display Secure Access (Callback) Directory.	3-25

*NCnn - Country Select	3-26

*P - Store Callback Password	3-27

*R - Request Remote Configuration Mode	3-27

*Zn - Change Dial Codes:	3-27

3.5.3.	AT\ Commands	3-28

\F - Display Telephone Directory	3-28

\S - Report Active Configuration	3-28

\Wn - Split-Speed Operation	3-28

3.6.	CALLER ID COMMANDS	3-29

3.6.1.	AT#CID Command	3-29

#CIDn - Caller ID	3-29

Inquiries	3-29

Formatted Form Reporting	3-29

Example of Formatted Form Reporting	3-30

Unformatted Form Reporting	3-30

Example of Unformatted Form Reporting	3-30

3.7.	AT COMMAND RESULT CODES	3-31

00- OK	3-31

01- CONNECT	3-31

02- RING	3-31

03- NO CARRIER	3-31

04- ERROR	3-31

05- CONNECT 1200	3-31

06- NO DIALTONE	3-31

07- BUSY	3-31

08- NO ANSWER	3-32

09- CONNECT 0600	3-32

10- CONNECT 2400	3-32

11- CONNECT 4800	3-32

12- CONNECT 9600	3-32

13- CONNECT 7200	3-32

14- CONNECT 12000	3-32

15- CONNECT 14400	3-32

16- CONNECT 19200	3-32

17- CONNECT 38400	3-32

18- CONNECT 57600	3-32

22- CONNECT 75TX/1200RX	3-32

23- CONNECT 1200TX/75RX	3-32

24- DELAYED	3-32

32- BLACKLISTED	3-33

-33 FAX	3-33

35- DATA	3-33

40- CARRIER 300	3-33

44- CARRIER 1200/75	3-33

45- CARRIER 75/1200	3-33

46- CARRIER 1200	3-33

47- CARRIER 2400	3-33

48- CARRIER 4800	3-33

49- CARRIER 7200	3-33

50- CARRIER 9600	3-33

51- CARRIER 12000	3-33

52- CARRIER 14400	3-33

66- COMPRESSION: CLASS 5	3-33

67- COMPRESSION: V.42 bis	3-33

69- COMPRESSION: NONE	3-34

76- PROTOCOL: NONE	3-34

77- PROTOCOL: LAPM	3-34

80- PROTOCOL: ALT	3-34

81- PROTOCOL: ALT-CELLULAR	3-34

+FC - FCERROR	3-34

4.	S REGISTERS	4-1

4.1.	FACTORY DEFAULTS	4-1

4.2.	S REGISTER DEFINITIONS	4-4

S0 - Number of Rings to Auto-Answer	4-4

S1 - Ring Counter	4-4

S2 - Escape Character	4-4

S3 - Carriage Return Character	4-4

S4 - Line Feed Character	4-4

S5 - Backspace Character	4-4

S6 - Wait Time for Dial Tone Before Blind Dialing, or After “W” Dial Modifier
(W-Class Models)	4-5

S7 - Wait Time For Carrier After Dial, For Silence, or For Dial Tone After “W” Dial Modifier (US Models)	4-5

S8 - Pause Time For Dial Delay	4-5

S9 - Carrier Detect Response Time	4-5

S10 - Lost Carrier To Hang Up Delay	4-5

S11 - DTMF Tone Duration	4-5

S12 - Escape Code Guard Time	4-6

S13 - Reserved	4-6

S14 - General Bit Mapped Options	4-6

S15 - Reserved	4-6

S16 - General Bit Mapped Test Options	4-6

S17 - Reserved	4-7

S18 - Test Timer	4-7

S19 - AutoSync Registers	4-7

S20 - AutoSync HDLC Address or BSC Sync Character	4-8

S21 - V.21/General Bit Mapped Options	4-8

S22 - Speaker/Results Bit Mapped Options	4-9

S23 - General Bit Mapped Options	4-10

S24 - Sleep Inactivity Timer	4-10

S25 - Delay To DTR	4-10

S26 - RTS to CTS Delay	4-10

S27 - Bit Mapped Option	4-11

S28 - Bit Mapped Options	4-12

S29 - Flash Dial Modifier Time	4-12

S30 - Disconnect Inactivity Timer	4-12

S31 - Bit Mapped Options	4-13

S32 - XON Character	4-13

S33 - XOFF Character	4-13

S34-S35 - Reserved	4-13

S36 - LAPM Failure Control (Applies to ECC Models Only)	4-13

S37 - Desired Line Connection Speed	4-14

S38 - Delay Before Forced Hang Up	4-14

S39 - Flow Control	4-14

S40 - General Bit Mapped Options	4-15

S41 - General Bit Mapped Options	4-16

S46 - Data Compression Control	4-16

S48 - V.42 Negotiation Action	4-16

S80 - Soft-Switch Functions	4-17

S82 - Break Handling Options	4-17

S86 - Call Failure Reason Code	4-18

S91 - PSTN Transmit Attenuation Level	4-18

S92 - Fax Transmit Attenuation Level	4-18

S95 - Extended Result Codes	4-19

S99 - Leased Line Transmit Level	4-19

S201 - Cellular Transmit Level	4-19

S202 - Remote Access Escape Character	4-19

5.	V.25 BIS COMMAND SET	5-1

5.1.	DTE/DCE INTERCHANGE CIRCUITS	5-1

5.2.	FORMATS FOR COMMANDS AND INDICATIONS	5-1

5.2.1.	Synchronous Signalling	5-1

HDLC Frames	5-1

BSC Frames	5-2

5.2.2.	Asynchronous Signalling	5-2

Command/Indication Exchange Protocol	5-2

DTE Adaption	5-2

5.2.3.	V.25 bis Escape Code Sequence	5-2

5.3.	STANDARD V.25 BIS COMMANDS	5-3

CIC - Connect Incoming Call	5-3

CRI - Call Request with Number and Identification	5-3

CRN - Call Request with Number	5-3

CRS - Call Request with Memory Address	5-4

DIC - Disregard Incoming Call	5-4

PRI - Program Identification	5-4

PRN - Program Number	5-4

RLD - List Request of Delayed Call Numbers	5-4

RLF - List Request of Forbidden Numbers	5-4

RLI - List Request of Identification Numbers	5-5

RLN - List Request of Stored Numbers (Dial Strings)	5-5

5.4.	EXTENDED V.25 BIS COMMANDS	5-5

CNL - Local Configuration	5-5

5.5.	STANDARD V.25 BIS INDICATIONS	5-6

CFI - Call Failure Indication	5-6

CNX - Connect	5-6

DLC - Delayed Call	5-6

INC - Incoming Call	5-6

INV - Invalid Command	5-6

VAL - Valid Command	5-6

6.	FAX CLASS 1 COMMANDS	6-1

6.1.	FAX I/O PROCESSING	6-1

6.1.1.	DTE-to-Modem Transmit Data Stream	6-1

6.1.2.	Modem-to-DTE Receive Data Stream	6-1

6.1.3.	Fax Mode Selection	6-1

6.1.4.	Fax Origination	6-1

6.1.5.	Fax Answering	6-2

6.1.6.	Fax Control Transmission	6-2

6.1.7.	Fax Control Reception	6-2

6.1.8.	Fax Data Transmission	6-3

6.1.9.	Fax Data Reception	6-3

6.2.	COMMANDS	6-4

+FCLASS=n - Select Service Class	6-4

+F<command>? - Report Active Configuration	6-4

+F<command>=? - Report Operating Capabilities	6-4

+FAE=n - Data/Fax Auto Answer	6-4

+FTS=n - Stop Transmission and Wait	6-4

+FRS=n - Receive Silence	6-4

+FTM=n - Transmit Data	6-5

+FRM=n - Receive Data	6-5

+FTH=n - Transmit Data with HDLC Framing	6-6

+FRH=n - Receive Data with HDLC Framing	6-6

6.3.	EXAMPLES	6-6

7.	FAX CLASS 2 COMMANDS	7-1

7.1.	COMMAND SYNTAX AND GUIDELINES	7-1

7.1.1.	DTE Commands	7-1

DTE Command Lines	7-1

Basic Command Syntax	7-1

Extended Command Syntax	7-1

7.1.2.	Serial Port Speed and Flow Control	7-3

Data Stream Termination	7-3

DTE to DCE Streams	7-3

DCE to DTE Streams	7-3

7.1.3.	Auto Answer	7-3

7.1.4.	Identification of T.30 Options	7-3

7.1.5.	Session Status Reporting	7-4

7.1.6.	Procedure Interrupt Negotiation	7-4

7.2.	SERVICE CLASS 2 IDENTIFICATION AND SELECTION	7-4

7.2.1.	+FMFR?, Request Manufacturer Identification	7-4

7.2.2.	+FMDL?, Identify Product Model	7-4

7.2.3.	+FREV?, Identify Product Revision	7-4

7.3.	SERVICE CLASS 2 ACTION COMMANDS	7-4

7.3.1.	ATD, Originate a Call	7-5

7.3.2.	ATA, Answer a Call	7-5

Manual Call Answer	7-5

Automatic Answer	7-5

Connection as a Data Modem	7-5

7.3.3.	+FDT, Data Transmission	7-6

Initiate Page Transmission	7-6

Continue a Page	7-6

Phase C Data Framing	7-6

Phase C Data Format	7-6

<CAN>, Escape from Transmission	7-6

7.3.4.	+FET, Transmit Page Punctuation	7-8

End a Page	7-8

7.3.5.	+FDR, Begin or Continue Phase C Receive Data	7-9

Initiate Document Reception	7-9

Continue Document Reception	7-9

Phase C Data Framing	7-10

Phase C Data Format	7-10

<CAN>, Escape from Reception	7-10

7.3.6.	+FK, Session Termination	7-10

7.4.	SERVICE CLASS 2 DCE RESPONSES	7-12

7.4.1.	+FCON, Facsimile Connection Response	7-12

7.4.2.	+FDCS:, Report Current Session Capabilities	7-12

7.4.3.	+FDIS:, Report Remote Station Capabilities	7-12

7.4.4.	+FCFR, Indicate Confirmation to Receive	7-12

7.4.5.	+FTSI:, Report the Transmit Station ID	7-12

7.4.6.	+FCSI:, Report the Called Station ID	7-12

7.4.7.	+FPTS:, Receive Page Transfer Status	7-12

7.4.8.	+FET:, Post Page Message Response	7-13

7.4.9.	+FPTS:, Transmit Page Transfer Status	7-13

7.4.10.	+FHNG:, Call Termination with Status	7-13

7.5.	SERVICE CLASS 2 PARAMETERS	7-14

7.5.1.	+FDCC, DCE Capabilities Parameters	7-14

7.5.2.	+FDIS, Current Sessions Capabilities Parameters	7-14

7.5.3.	+FDCS, Current Session Results Parameters	7-15

7.5.4.	+FLID=, Local ID String	7-16

7.5.5.	+FCR, Capability to Receive	7-16

7.5.6.	+FPTS=, Page Transfer Status	7-16

7.5.7.	+FCQ, Copy Quality Checking	7-16

7.5.8.	+FPHCTO, DTE Phase C Response Time-out	7-16

7.5.9.	+FAXERR, T.30 Session Error Report	7-17

7.5.10.	+FBOR, Data Bit Order	7-17

7.5.11.	+FAA, Answer Parameter	7-17

7.5.12.	+FBUF?, Buffer Size	7-18

7.6.	EXAMPLE SESSIONS	7-18

8.	VOICE COMMANDS	8-1

8.1.	VOICE SUBMODES	8-1

8.1.1.	Online Voice Command Mode	8-1

8.1.2.	Voice Receive Mode	8-1

8.1.3.	Voice Transmit Mode	8-1

8.2.	VOICE CAPABILITIES	8-1

8.2.1.	Call Establishment - Originate	8-1

Directed Originate (Dial as a specific modem type)	8-1

Adaptive Originate (Dial with Voice/Data/Fax Discrimination)	8-2

8.2.2.	Call Establishment - Answer	8-2

Directed Answer (Answer as a specific modem type)	8-2

Adaptive Answer (Answer with Voice/Data/Fax Discrimination)	8-2

8.2.3.	Voice Data Transfer	8-3

8.2.4.	Tone and Status Monitoring– Shielded <DLE> Statuses	8-3

8.2.5.	Shielded <DLE> Commands from the DTE	8-5

8.2.6.	Voice Record	8-5

8.2.7.	Voice Playback	8-6

8.2.8.	Voice Call Termination	8-6

Local Disconnect	8-6

Remote Disconnect Detection	8-6

8.2.9.	Mode Switching	8-7

Voice to Fax	8-7

Unsuccessful Fax Connection Attempt to Voice	8-7

Voice to Data	8-7

Unsuccessful Data Connection Attempt to Voice	8-7

8.2.10.	Caller ID	8-7

8.3.	AT VOICE COMMAND SUMMARY	8-7

8.3.1.	Global AT Command Set Extensions	8-7

ATA - Answering in Voice	8-7

ATD - Dial Command in Voice	8-8

ATH - Hang Up in Voice	8-8

ATZ - Reset from Voice Mode	8-9

#BDR - Select Baud Rate (Turn off Autobaud)	8-9

#CID - Enable Caller ID Detection and Select Reporting Format	8-10

#CLS - Select Data, Fax, or Voice	8-10

#MDL? - Identify Model	8-11

#MFR? - Identify Manufacturer	8-11

#REV? - Identify Revision Level	8-11

8.3.2.	AT#V Commands Enabled Only in Voice Mode (#CLS=8)	8-11

#VBQ? - Query Buffer Size	8-11

#VBS - Bits Per Sample (Compression Factor)	8-11

#VBT - Beep Tone Timer	8-12

#VCI? - Identify Compression Method	8-12

#VLS - Voice Line Select	8-12

#VRA - Ringback Goes Away Timer (Originate)	8-13

#VRN - Ringback Never Came Timer (Originate)	8-14

#VRX - Voice Receive	8-14

#VSD - Silence Deletion Tuner (Voice Receive)	8-14

#VSK - Buffer Skid Setting	8-15

#VSP - Silence Detection Period (Voice Receive)	8-15

#VSR - Sampling Rate Selection	8-15

#VSS - Silence Sensitivity Tuner (Voice Receive)	8-16

#VTD - Tone Reporting Capabilities	8-16

#VTS - Play Tone String (Online Voice Command)	8-17

#VTX - Voice Transmit	8-17

8.4.	S-REGISTERS	8-18

S30 - Deadman (Inactivity) Timer	8-18

8.5.	RESULT CODES FOR VOICE OPERATION	8-18

8.6.	EXAMPLES OF VOICE OPERATION	8-18

A.	AT COMMAND SET SUMMARY	A-1

A.1.	BASIC AT COMMANDS	A-1

A.2.	ECC COMMANDS	A-3

A.3.	MNP 10 COMMANDS	A-4

A.4.	W-CLASS COMMANDS	A-4

A.5.	CALLER ID COMMANDS	A-4

A.6.	V.25 BIS COMMANDS	A-5

A.7.	FAX CLASS 1	A-5

A.8.	FAX CLASS 2	A-5

A.9.	VOICE COMMANDS	A-6

B.	COMMON CONFIGURATION SETUP STRINGS	B-1