README.DOC                                      updated March 3, 1994

Backup Exec for NetWare - Evaluation Version

---------------------------------------------------------------------

This is a 30 DAY EVALUATION VERSION of Backup Exec for NetWare.

The evaluation period will begin the first time that the Job

Manager NLM is loaded.  The software will be fully functional

for the first 30 days.  Once the evaluation period has expired,

jobs will no longer be processed by the Job Manager.  You will,

however, be able to use all of the other functions in the

software.  When you purchase and re-install Backup Exec, all of

the user options, jobs, catalogs and job histories from the

evaluation version will remain.

Please contact Arcada Software at 800-3ARCADA or 407-262-8000

to inquire about purchasing Backup Exec for NetWare.

Continuing with the installation of this product means that

you understand and agree with the following terms and

conditions:

1.  This evaluation package ceases to provide backup and restore

capability following the 30 day evaluation period.

2.  Arcada Software is not liable for any damages resulting from

the use of this product; including but not limited to, lost

time or revenue as a result of the disabling of backup and

restore capabilities once the evaluation period has expired.

3.  This software may be installed for evaluation purposes only.

---------------------------------------------------------------------

CAREFULLY FOLLOW THE INSTRUCTIONS IN SECTIONS 1-3 BELOW TO COMPLETE

THE INSTALLATION PROCESS.

YOU MAY WISH TO PRINT A COPY OF THIS README.DOC FILE, WHICH IS

LOCATED IN YOUR SERVER'S SYS:BKUPEXEC SUBDIRECTORY.  FOR EXAMPLE:

NPRINT [ insert server name ]/SYS:BKUPEXEC\README.DOC

=====================================================================

TABLE OF CONTENTS

=================

Section#  Topic

--------  ------------------------------------------------------

1.   Installation

2.   Novell NLMs

3.   Server Configuration Issues

4.   Compatibility Information for Drives and Controllers

5.   Windows Client Information

6.   DOS Client Information

7.   Limitations

8.   Commonly Asked Questions

9.   Changes to Documentation

10.   Notes for Autoloader Users

11.   Contents of Installation Disks

=====================================================================

1.  INSTALLATION

============

The Backup Exec for NetWare software is installed directly on

the server that you intend to use as your backup server.  The

Single Server Edition of Backup Exec supports a single NetWare

server licensed for up to 25 users.  You may choose a server

running NetWare version 3.11 or later.

The Backup Exec installation process consists of several steps:

(1)  Install the Backup Exec software onto the file server.  If

you purchased the Enterprise Edition of Backup Exec, at

the end of the installation you will be asked to update the

Novell NLMs on other servers that Backup Exec will access.

(2)  DOWN the file server and power it off.

(3)  Install any new backup-related hardware into the file

server.  For example:  install an Adaptec SCSI controller

and an internal tape drive.

(4)  Turn on the file server and restart.

(5)  After the server finishes booting, at the server console

prompt ':' type the following command: BESTART.

BESTART is an .NCF file located in your backup server's

SYS:SYSTEM subdirectory which is used to load the Backup

Exec Job Manager and associated NLMs.  You may wish to add

this command to your AUTOEXEC.NCF in order to automatically

load the Backup Exec Job Manager when your server boots.

(6)  Install clients and/or agents on workstations so users

can submit jobs.

Although Backup Exec provides the necessary Novell system files,

Novell updates these files on a regular basis.  Please contact

Novell for information on obtaining the latest versions of

these files.

For more information, refer to the Backup Exec for NetWare

Installation Manual.

2.  NOVELL NLMS

===========

Backup Exec utilizes several of Novell's NLMs to process its

NetWare-specific requests and to communicate with other servers

and workstations.  The installation program prompts for a

confirmation when it attempts to install the proper version of

each NetWare NLM that is needed.

If you purchased the Enterprise Edition of Backup Exec, some

NetWare NLMs must also be installed (or updated) on remote

servers that Backup Exec will be accessing for backup or

restore operations.  The install program also prompts for

confirmation as it installs these NLMs.

If Backup Exec is not run with the specific versions of the

NetWare NLMs that it needs, undesirable results may occur.

Therefore, we STRONGLY recommend that you allow Backup Exec to

install the recommended versions of these NetWare NLMs.

3.  SERVER CONFIGURATION ISSUES

===========================

NOTE:  The required changes to server configuration files listed

below are done as part of the Backup Exec installation.

However, if you later make changes to these files and you

encounter problems running Backup Exec, you may need to

refer to this information.

a.  STARTUP.NCF Changes Required:

---------------------------------

Servers with greater than 16 megabytes of memory:

-------------------------------------------------

If you have more than 16 megabytes of memory in your file

server, the number of reserved buffers that are below the

16 megabyte boundary need to be increased from the default

(NetWare defaults to 16 of these buffers).  Backup Exec

allocates its tape buffers from this reserved buffer pool,

which makes it necessary to increase the amount of

reserved buffers.

To increase the number of reserved buffers below 16

megabytes, use the following SET command in your

STARTUP.NCF file:

SET RESERVED BUFFERS BELOW 16 MEG = 200

The server must be downed and re-started for this change

to take effect.

Loading controller level drivers:

---------------------------------

It is recommended that you load ALL controller level

drivers in your server's STARTUP.NCF file.  Examples of

controller level drivers are:  AHA1540.DSK, AHA1640.DSK,

AHA1740.DSK, ASPITRAN.DSK, etc.

b.  AUTOEXEC.NCF Changes Recommended:

-------------------------------------

BESTART.NCF

-----------

Backup Exec installs a file (BESTART.NCF) which loads

the Job Manager and other required NLMs.  You may wish to

add BESTART at the end of your AUTOEXEC.NCF file so that

the Job Manager will load each time your server is booted.

Running BESTART.NCF loads the device driver for the

type of controller you chose during installation.  If you

later decide to use a different controller, you may wish

to modify your BESTART.NCF file to load MS_SCSI instead of

AD_ASPI or vice-versa.

c.  Macintosh support for NetWare 4.01 servers:

(Enterprise Edition of Backup Exec only)

-----------------------------------------------

If you are getting a message indicating that ADSP.NLM

can't be found on a 4.01 server, you probably have not

installed the Macintosh support on the server.

Before Backup Exec for NetWare can support Macintosh

workstations on a NetWare 4.01 server, the NetWare server

must be configured to support Macintosh workstations.  To

add Macintosh workstation support to the server, perform

the following operations:

o  From the System Console, type "LOAD INSTALL"

o  Choose "Maintenance/Selective Install"

o  Choose "Other Options"

o  Choose "Install NetWare for Macintosh"

o  Follow the prompts and perform any necessary

configuration.

4.  COMPATIBILITY INFORMATION FOR DRIVES AND CONTROLLERS

====================================================

a.  SCSI Controller Support

Vendor           Model       Notes

--------------------------------------------------------

Adaptec          1510        Must disable Fast File Access

Exabyte 10i not supported

Adaptec          152x        Must disable Fast File Access

Exabyte 10i not supported

Adaptec          154x

Adaptec          1542CF

Adaptec          1640

Adaptec          1740

Adaptec          2740

Adaptec          2740T

Adaptec          2742AT

Adaptec          2842VL

BusLogic         BT542       * (see below)

BusLogic         BT646       * (see below)

BusLogic         BT747S      * (see below)

Conner (Maynard) 16-bit      DEC T860-DLT not supported

DEC T860-LDLT not supported

Conner 12L DAT not supported

Conner (Maynard) PS/2        DEC T860-DLT not supported

DEC T860-LDLT not supported

Conner 12L DAT not supported

Dell             DSA

DPT              2011

DPT              2022        With & without cache

DPT              PM2021/90   With & without cache

Exabyte 10i not supported

DPT              PM2021/95   With & without cache

Exabyte 10i not supported

DPT              2012B

DPT              PM2122/90   With & without cache

DPT              PM2122/95   With & without cache

UltraStor        14F

UltraStor        24F         Exabyte 10i not supported

UltraStor        34F

* Use BT311.DSK dated 02/02/93 for NetWare 3.11 servers;

Use BT312.DSK dated 05/04/93 for NetWare 3.12 servers;

Use BT40.DSK dated 06/09/93 for NetWare 4.x servers.

------------------------------------------------------------

Additional Notes on Controller Support:

o  Adaptec Controllers

AHA-1510, 152x, 154x, 1640, 1740 Controllers:

----------------------------------------------------

Backup Exec v4.01 was compatibility tested with

Adaptec's ASW-1440 Version 3.3 NetWare 386 SCSI

Drivers.  The specific driver versions are as follows:

Driver:        Version:       Module Date:

AHA1510.DSK    1.22           11/13/91

AHA1520.DSK    1.22           11/13/91

AHA1540.DSK    2.22           07/26/91

AHA1640.DSK    2.22           07/26/91

AHA1740.DSK    1.20           07/01/91

ASPITRAN.DSK   1.0            07/31/90

AHA-1542CF Controller:

----------------------

Backup Exec v4.01 was compatibility tested with

the following Adaptec drivers.  The specific driver

versions are as follows:

Driver:        Version:       Module Date:

AHA1540.DSK    2.22           07/26/91

ASPITRAN.DSK   1.0            07/31/90

AHA-2740, 2740T Controllers:

----------------------------

Backup Exec v4.01 was compatibility tested with

Adaptec's ASW-1240 Version 1.3 NetWare 386 SCSI

Drivers.  The specific driver versions are as follows:

Driver:        Version:       Module Date:

AIC7770.DSK    1.05           06/28/93

ASPITRAN.DSK   1.10           04/13/93

AHA-2742AT Controller:

----------------------

Backup Exec v4.01 was compatibility tested with

the following Adaptec drivers.  The specific driver

versions are as follows:

Driver:        Version:       Module Date:

AIC7770.DSK    2.10           12/14/93

ASPITRAN.DSK   1.0            04/13/93

AHA-2842VL Controller:

----------------------

Backup Exec v4.01 was compatibility tested with

the following Adaptec drivers.  The specific driver

versions are as follows:

Driver:        Version:       Module Date:

AIC7770.DSK    2.10           12/14/93

ASPITRAN.DSK   1.0            04/13/93

Recommended Drive Selection Settings

------------------------------------

It is recommended that, when used with an Adaptec

controller, the SCSI ID for the tape drive be set at

address 2 for the first tape drive.  Addresses 0 and

1 are reserved for hard disks.

Synch Negotiation Changes Required

----------------------------------

If you are using a Conner DAT or DAT/DC tape

drive connected to an Adaptec 164x or 174x

controller, you must disable the controller's Synch

Negotiation (i.e., the Adaptec controller must not

initiate a synch negotiation).  Use the PS/2 setup

or EISA configuration utility provided with your

system to change this option.

Fast File Access problem with Adaptec 1510/1520

------------------------------------------------

The Adaptec 1510/1520 controllers were tested with

AHA1510.DSK and AHA1520.DSK drivers both dated

12/03/91.  Unlike the drivers for the 154x, 1640 or

174x controllers, Backup Exec's Fast File Access

feature will not work with these 1510/1520 drivers.

Until the problem is fixed in the supplied Adaptec

drivers, you must disable Backup Exec's Fast File

Access feature.  Simply edit the BEMGR.CFG options

file located in the SYS:BKUPEXEC directory on your

server and set the parameter 'Use_fast_file_access'

equal to 0.  (Do not modify the BEMGR.CFG file while

the Job Manager is running.  The Job Manager must be

loaded again before any changes to this file will take

effect.)

Recommendation for loading AHA1510.DSK

--------------------------------------

When loading the AHA1510.DSK driver, Adaptec recommends

that you explicitly specify all of the load options

available, for example:

load aha1510 port=aaa verbose=y fixed_disk=off

int=i id=z sync=off

where aaa = port address of 1510 controller card

i   = interrupt setting of 1510 card

z   = SCSI address of the 1510 card

o  BusLogic Controllers

Backup Exec for NetWare v4.01 was compatibility tested

with the following BusLogic SCSI drivers:

Driver:        Version:       Module Date:

BT311.DSK      2.20           02/02/93

BTASPI.DSK     1.0            07/23/92

Using a BT311.DSK dated earlier than indicated

can cause the Job Manager application to "hang"

during its initialization process.

o  Dell Controllers

Backup Exec for NetWare v4.01 was compatibility tested

with the following SCSI drivers:

Driver:        Version:       Module Date:

DSA311.DSK     1.12           08/06/93

AHA1540.DSK    2.34           08/13/93

ASPITRAN.DSK   1.10           04/13/93

Using a BT311.DSK dated earlier than indicated

can cause the Job Manager application to "hang"

during its initialization process.

o  DPT Controllers

Backup Exec for NetWare v4.01 was compatibility tested

with the following DPT SCSI drivers:

DPT2011 and DPT2012B Host Adapters

Driver:        Version:       Module Date:

PM12NW31.DSK   1.82           02/03/93

PM11NW31.DSK   1.80           05/11/92

DPTDDL.NLM     1.80           05/11/92

DPT2021, DPT2022, and DPT2122 Host Adapters

Driver:        Version:       Module Date:

PM11NWS31.DSK  3.B            9/23/93

PM11NWS40.DSK  3.B            9/23/93

ASPITRAN.DSK   1.01           9/23/93

DPTSIM.DSK     3.B            9/23/93

Firmware versions:

Host Adapter   Firmware       BIOS ROM

DPT2011        3G             1J9

DPT2012        3G             1J9

DPT2021        2.15           2B

DPT2022        5.B0           2B

DPT2122        2.10           2B

Note:  If you are using the combination of a DPT

2021, 2022, or 2122 host adapter and a DEC DLT tape

drive, you must use the following versions of the

DPT SCSI drivers.

Driver Date    Driver Ver.

PM12NW31.DSK   01/15/94       V3.C2

PM11NW31.DSK   01/15/94       V3.C2

PM12NW40.DSK   01/15/94       V3.C2

PM11NW40.DSK   01/15/94       V3.C2

DPTSIM.DSK     01/15/94       V3.C2

The firmware on the host adapter should be version 5C,

and the BIOS should be version 002C.

o  UltraStor Controllers

UltraStor 14F Controller:

--------------------------

Backup Exec for NetWare v4.01 was compatibility tested

with the following UltraStor SCSI drivers:

Driver:        Version:

U14_31X.DSK    2.00

U14_312.DSK    2.02

U14_40X.DSK    1.02

UltraStor 24F Controller:

--------------------------

Backup Exec for NetWare v4.01 was compatibility tested

with the following UltraStor SCSI drivers:

Driver:        Version:

U24_31X.DSK    2.00

U24_312.DSK    2.02

U24_40X.DSK    1.02

UltraStor 34F Controller:

------------------------------------

Backup Exec for NetWare v4.01 was compatibility tested

with the following UltraStor SCSI drivers:

Driver:        Version:         Module Date:

U14_31X.DSK    2.04.02 v2.04b   10/18/93

USPI_MGR.DSK   1.00j            12/30/92

U14_40X.DSK    1.02             6/15/93

Exabyte 10i Loader

------------------

The Exabyte 10i Loader is not supported on the

UltraStor 24F controller.

Synch Negotiation Changes Required

----------------------------------

If you are using a Conner DAT or DAT/DC tape

drive connected to an UltraStor U14F controller

you must disable the controller's Synch Negotiation

(i.e. the UltraStor controller must not initiate a

synch negotiation).

o  Conner (Maynard) Controllers

When installing this controller, you must make sure

that the DMA channel the card is using does not

conflict with other hardware in your system.  Backup

Exec can not detect DMA conflicts for this card.

On an EISA system, we recommend that you use the

supplied !CNR1600.CFG configuration file for

configuring this controller.

On a Microchannel system, we recommend that you use

the supplied @568D.ADF configuration file for

configuring this controller.

The DEC T860 family of drives is not supported on

the Conner or Maynard SCSI controllers.

The Conner 12L DAT loader is not supported on the

Conner or Maynard SCSI controllers.

b.  SCSI Device Support

Please note that the firmware versions listed in the

following table indicate the firmware with which the tape

drive was tested.  If you have a different version of

firmware, it most likely will work with Backup Exec, even

though it is not listed as the version that was certified

with the tape drive.  You should contact the manufacturer

or reseller of your tape drive for more information about

firmware differences.

Vendor               Drive                    Firmware

------------------------------------------------------

Conner               Viper 2150S              21247-011

Conner MS525Q        Viper 2525S              25582-003

Conner               Viper 2525S              25462-007

Conner               Viper 2525S              25462-011

Conner MS525Q        Viper 2525S              25582-004

Conner               Anaconda 2750S           28077-902 v6.0

Conner MS1350Q       Anaconda 2750S           28077-003 v6.0

Conner               Anaconda 2800S           28889-900

Conner               Anaconda 2800S           28077-003 v6.0

Conner MS2000DAT     Python DAT 4320          2.19

Conner MS2000DAT     Python DAT 4320          2.66

Conner MS2000DAT     Python DAT 4320          2.96

Conner               Python DAT 4320          2.50

Conner MS4000DAT     Python DAT DC 4322       3.58

Conner MS4000DAT     Python DAT DC 4322       4.04

Conner MS4000DAT     Python DAT DC 4322       4.61

Conner               Python Turbo DAT 4324    4.28

Conner               Python Turbo DAT 4324    4.59

Conner               Python Turbo DAT 4324    4.04

Conner MS4000DAT     Python Turbo DAT 4324    4.61

Conner MS4000DAT+    Python DDS-2 4326        4.98

Conner MS6000DLT     DEC C860S DLT            V10 CI-1 (430A)

Conner MS6000DLT     DEC C860S DLT            V22 C0-6 (4316)

Conner MS20DLT       DEC DLT2000              V5 OEM1 (9105)

Conner 4L-DLT        Diamondback Loader 4584  4.59 (drive)

2.93D (loader)

Conner 4L-DLT        Diamondback Loader 4584  4.61 (drive)

2.93D (loader)

Conner 7L-DLT        DEC L7A-42               V10 CI-1 (430A)

(loader = 28)

Conner 12L-DAT       ADIC-DAT Autochanger     Drive 4.61

Loader 4.51

Conner MS2200HS+     8200SX                   2638

Conner MS2200HS+     8200SX                   261Z

Conner MS5000HS      8500                     0340

Conner MS5000HS      8500                     0415

Conner MS5000HS      8500                     04S0

Conner 10L-HS        Exabyte 10i Loader       3.024 (loader)

8500 drive               04S0 (drive)

Conner 10L-HS        Exabyte 10i Loader       2.10 (loader)

8500 drive               0415 (drive)

ADIC-DAT Autochanger ADIC 12L DAT             Drive 4.61

Loader 4.51

DEC                  T860DLT                  430A

DEC                  T860DLT                  4314

DEC                  DLT2000                  V5 OEM1

DEC                  T860L-DLT                430A (Drive)

28 (Loader)

Exabyte              8200SX                   2638

Exabyte              8200SX                   261Z

Exabyte              8500                     0340

Exabyte              8500                     0415

Exabyte              8500                     04S0

Exabyte              8505                     0472

Exabyte              8505                     05B0

Exabyte              8505                     06J1

Exabyte              10i Loader /8500         2.10 (loader)

0415 (drive)

Exabyte              10i Loader /8500         3.024 (loader)

04S0 (drive)

Exabyte              DAT (4200)               107

Exabyte              DAT (4200)               137A

Exabyte              DAT DC (4200C)           096

Exabyte              DAT DC (4200C)           132

HP                   DAT (35470A)             7.09 Rev A

HP                   DAT(35480A)              7.09 Rev A

HP                   DDS-2 (1533A)            1.95

IBM                  4100                     Wangtek I05Y06

IBM                  DAT                      Conner C6.79251

IBM                  35480A                   7.09

IBM                  8505                     47DB

IBM                  DDS-2 DAT                4.73

IBM                  DDS-2 DAT                4.82

Iomega               4000 DAT                 1.35

Iomega               4000 DAT                 1.38

Sony                 SDT 2000                 2.09

Sony                 SDT 4000                 2.09

Sony                 SDT 2000                 3.02

Sony                 SDT 4000                 3.02

Sony                 SDT 5000                 3.02

Tandberg             3660 (250MB)             08:03

Tandberg             3660 (250MB)             08:15

Tandberg             3820 (525MB)             04:08

Tandberg             3820 (525MB)             03:07

Tandberg             4120 (1GB)               05:10

Tandberg             4120 (1GB)               05:17

Tandberg             4220 (2GB)               05:10

Tandberg             4220 (2GB)               05:17

Teac                 600CS                    0233-06RV.A-191C

Teac                 600CS                    0233-0RV.A-191D

Teac                 600CS                    0233-06RV.D-239E

Teac                 600CS                    (MAYNARD)

MT-2ST/F50b Rev D

WangDAT              DAT (3100)               2.0

WangDAT              DAT (3200)               2.0

Wangtek              250                      FA14

Wangtek              525                      73J

Wangtek              51000                    7J3

------------------------------------------------------

Additional SCSI Device Support notes:

o  Conner Python DAT 4320

DO NOT attempt to access a tape created by a Conner

Python DAT DC 4322 if your Python DAT drive has

firmware versions 1.36 or 2.19!  In this situation,

the DAT drive will appear hung for up to 10 minutes

before returning control to Backup Exec for NetWare.

This problem was fixed in the 2.66 firmware revision.

o  WangDAT

The WangDAT drives have a switch which enables/disables

Software Eject.  It is set to "DISABLE" by default.  If

the feature is "ENABLED", Backup Exec's Utility "Eject

Tape" feature will operate.  If this switch is left

in the disable position, Backup Exec will only rewind

the tape when the "Eject Tape" function is selected.

o  Conner 4L-DLT (Diamondback DAT Loader)

If you are using a Conner 4L-DLT (Diamondback DAT

Loader, make sure that a magazine is loaded in the

drive prior to starting the Backup Exec Job Manager

(BEMGR.NLM).  This loader supports magazines with 4, 8

or 12 tapes inserted and Backup Exec determines the

number of tape slots supported during initialization.

If a magazine is not inserted in the drive when Backup

Exec initializes, the software reports that no tape

drives have been found.  If this happens, simply exit

Backup Exec, insert the magazine in the loader and

re-load Backup Exec.  In addition, once Backup Exec

is loaded and running, you may insert a different size

magazine and re-initialize Backup Exec for the new

size by performing the Options/Controller/Test

operation in the Job Manager main menu.

Tape magazines should not be manually ejected from

the Diamondback loader while the Job Manager is

running.  Doing this may cause the Job Manager to hang

upon exit.  To change or remove the magazine, first

perform the Utilities/Eject Tape operation in the Job

Manager main menu.  Then proceed to change or remove

the magazine.

o  Conner MS525Q

Writing to non-native format tapes is not supported

on the Conner MS525Q drive with 25582-004 firmware.

5.  Windows Client Information

==========================

a.   The Backup Exec DOS Agent (also used for Windows) does not

support the Microsoft-supplied MSIPX.COM driver which comes

standard with Windows for Workgroups version 3.1.  See

LIMITATIONS item (g) for more information.

b.   Do not use the double buffering feature on SMART DRIVE

while a workstation is being restored.  Enabling the

double buffering of the Smart Drive utility causes

errors to occur during the restore.  Note that the

double buffering capability of Smart Drive is usually

associated with the use of SCSI hard drives.

6.  DOS Client Information

======================

a.   The DOS client should be run with at least 512K free

memory.

b.   Do not use the double buffering feature of SMART DRIVE

while a workstation is being restored.  Enabling the

double buffering of the Smart Drive utility causes

errors to occur during the restore.  Note that the

double buffering capability of Smart Drive is usually

associated with the use of SCSI hard drives.

c.   Running the DOS client in a Windows DOS box is not

supported.

7.  LIMITATIONS

===========

a.   Backup Exec will always back up volume restrictions but

will NOT restore them.  If it is imperative that you get

back the volume restrictions, you may set the Special_word

token in the BEMGR.CFG file to 80 (i.e. Special_word = 80)

and volume restrictions WILL be restored.  (Do not modify

the BEMGR.CFG file while the Job Manager is running.  The

Job Manager must be loaded again before any changes to

this file will take effect.)

NOTE:  Remember to reset Special_word back to 0 before

doing any other restore operations.

b.   When accessing servers with older versions of the STREAMS

and TLI NLM modules, the Backup Exec clients or the

Job Manager may hang during the scan for SMDRs.  See

the Commonly Asked Questions section for more information

if you are encountering this problem.

c.   If there is a logical disconnect between the Backup

Server and a remote server during a backup (this applies

only to the Enterprise Edition of Backup Exec), you may

experience errors ranging from a job with a constant

active status to a server abend.  This is a result of the

SMS component failing to cleanly deal with the

connection loss.  For example, if you were to

dismount a volume while it was being backed up, you

would run into this problem.

d.   If you are using the Backup Exec Windows client and your

screen "blacks out" followed by a workstation hang, you

need to get updated IPX drivers from Novell's NetWire

CompuServe forum.  Updated files are found in the

DOSUP8.EXE and WINUP8.EXE self-extracting archive.

e.   When using Backup Exec agent software, you may want to

first type CASTOFF ALL.  If a message is sent to a

workstation that is running Backup Exec agent software, the

Job Manager or Client software may lose its connection to

the agent or performance may be severely degraded.

To turn off broadcasts on a workstation logged into a 4.x

server, use SEND /A=N instead of CASTOFF ALL.

f.   NetWare 2.x servers are not yet supported by this product

because there is no Novell TSA available for NetWare 2.x

servers.

g.   Running the Backup Exec Job Manager within a Novell SFT III

server is not supported yet because there is no Novell

SMDR available for SFT III servers.  However, you may back

up an SFT III server as a remote server.

h.   The Backup Exec DOS Agent (also used for Windows) does not

support the Microsoft-supplied MSIPX.COM driver which comes

standard with Windows for Workgroups version 3.1.  In order

to use the Backup Exec DOS agent under Windows for

Workgroups, you must install Novell's ODINSUP.COM driver

instead of the MSIPX.COM driver.  The ODINSUP driver does

the translation between Novell's ODI and Microsoft's NDIS.

An outline of the procedure to modify a Windows for

Workgroups environment to use the ODINSUP driver is found

in the README.TXT file that is installed on the

workstation as part of the Backup Exec Windows workstation

SETUP program. If there are Windows for Workgroups

workstations that will run the Backup Exec DOS Agent, the

network administrator should review the procedure outlined

and create a customized procedure for the network users.

If you are using Workgroups for Windows version 3.11,

you should see the NETWORKS.WRI file in the Windows

directory for information about upgrading your network

interface to use the ODI drivers.

i.   If you are running the Backup Exec Job Manager on an

Adaptec controller and the tape drive is powered off

during an operation, you may need to DOWN your file

server to re-use the Job Manager.  Adaptec's ASPI manager

does not correctly handle this problem and may result

in the Job Manager appearing to be "hung" trying to

complete an operation.  The file server will continue to

function properly, but you will need to DOWN your file

server and re-boot in order to use Backup Exec again.

j.   A problem with the Novell supplied DSAPI.NLM may result

in a server console error message indicating a "Read from

a non present page frame".  This error has been seen in

instances of rather complex NetWare 4.01 NDS structures

with multiple organizations and multiple countries.  If

your NetWare 4.01 server is configured for "Read Fault

Emulation" then this error will be reported to the server

console and the server will continue functioning.

However, if you have not configured your server for "Read

Fault Emulation" then the 4.01 server will ABEND in this

circumstance.

NOTE:  This problem has been corrected with newer versions

of DSAPI.NLM ( version 4.01c or later ) which are available

on CompuServe.

k.   There is a problem with the 3Com 3C503 card that causes it

to hang the driver if it experiences heavy network traffic

for an extended period of time.  This manifests itself

in the Backup Exec for NetWare product by generating

"LAN receive buffer limit reached.  All buffers are in use"

errors while performing remote server backups (this applies

only to the Enterprise Edition of Backup Exec).  If you

encounter this error, you should upgrade your network card.

l.   If the Job Manager (BEMGR.NLM) or the NetWare Client

(BE.NLM) displays an "out of memory" error and you see a

message at the NetWare console "Alloc short term memory

allocator requests exceeded the configuration limit", you

will need to give NetWare more memory to work with.

Failure to do so may cause memory corruption or a server

abend.

For 3.11 Servers: Type

"set maximum alloc short term memory=xxxxxxxx"

where xxxxxxxx is 50kb to 16mb.  It is set to

2mb by default.

For 3.12 servers Type

"set maximum alloc short term memory=xxxxxxxx"

where xxxxxxxx is 50kb to 32mb.

It is set to 8mb by default.

m.   Appending to backup tapes that were created with version

3.1 of Backup Exec for NetWare, MaynStream NLM or

Archive Information Manager NLM is not supported.  These

tapes can be cataloged and restored by Backup Exec for

NetWare v4.01 or they can be overwritten by backup jobs.

n.   When you run out of disk space on your catalog volume, you

should move your catalogs to a different volume.  If you

do this and encounter the "Unable to initialize job" on

the Job Manager screen, you will need to go into the

<volume>:\BKUPEXEC\HISTORY directory and manually delete

any 0 byte 0000000?.00? files that exist.  This should

resolve the problem.

8.  COMMONLY ASKED QUESTIONS

========================

Q:   I have upgraded my NetWare to support more users.  The

Backup Exec - Single Server Edition Job Manager now

reports a warning that my license limit has been exceeded.

Will I still be able to back up and restore my data?

A:   You will be able to back up and restore your data, however

the Single Server Edition of Backup Exec supports a single

NetWare server licensed for up to 25 users.  If your

NetWare license supports more than 25 users, you will

receive a warning while the Job Manager is running.  You

should contact Arcada Software to purchase an upgrade in

order to comply with your Backup Exec for NetWare license

agreement.

Q:   The Backup Exec Job Manager reports that it cannot find

any tape drives connected to the controller.  Why is

this and how do I fix this?

A:   There are several reasons you could get this message.

All of them have to do with the setup of the hardware

and software that make up and control the SCSI bus.

If you are having this problem, try the following:

1) Make sure that you have your SCSI bus terminated

properly.  There must be a terminator at either

end of the SCSI bus, and NO terminators in the

middle of the SCSI bus.

2) Make sure that none of the connections on the SCSI

bus are loose.

3) Check the SCSI bus IDs for each device on the SCSI

bus.  Each device should have a unique ID.  Don't

forget the SCSI bus adapter in your system. Typically

it is set to SCSI bus ID 7.

4) If you are using an ASPI compatible controller, it

it strongly recommended that you load your disk (.DSK)

drivers in the STARTUP.NCF instead of in the

AUTOEXEC.NCF file.

5) Make sure that the tape drive you have connected to

the SCSI bus is supported by Backup Exec for NetWare.

See the COMPATIBILITY INFORMATION FOR DRIVES AND

CONTROLLERS section for a complete list of the

supported drives and controllers.

Q:   My client application starts up, and hangs while scanning

for network agents.  What is wrong?

A:   There is a problem with the scanning of the Novell agents

that causes the application to hang.  This usually

occurs when a server is being downed.  There are two

factors that affect the probability of your seeing this

problem:

1) The more servers you have on your network running the

SMS SMDRs, the more likely you are to have the problem.

2) The slower your system is, the more you will encounter

this problem.

You can reduce or prevent this problem by reducing the

number of SMS SMDRs that you are using by creating a

SMDR Include file.  See page A - 2 of the Backup Exec

for NetWare 4.0 Administrator's Manual for details

about the SMDR Include file.

Q:   Whenever I attempt to perform a tape operation, Backup

Exec reports an "Out of Memory" error.  In addition, my

file server reports:

"[date time] 1.1.38 Cache memory allocator exceeded

minimum cache buffer left limit"

on the system console.  Why is this and what can I do to

fix it?

A:   If you have more than 16 megabytes of memory in your file

server, you will need to increase the number of reserved

buffers that are below the 16 megabyte boundary (NetWare

defaults to 16 of these buffers).  If your server is

configured with more than 16 megabytes of memory,

Backup Exec will allocate its tape buffers from this

reserved buffer pool, which makes it necessary to increase

the amount of reserved buffers.

Normally, the Backup Exec for NetWare Install process

does this for you automatically.  You should only need

to change this if you had removed the setting from your

STARTUP.NCF file, or if you added memory after installing

Backup Exec for NetWare.

To increase the number of reserved buffers below 16

megabytes, use the following SET command in your

STARTUP.NCF file:

SET RESERVED BUFFERS BELOW 16 MEG = 200

The server must now be downed and re-started for this

change to take effect.

Q:   Why would loading Backup Exec for NetWare, slow my

server down more than I'd expected?

A:   NetWare supplies memory for all NLMs from a pool of file

cache buffers.  If you are loading several NLMs on your

file server, your server could reduce the number of

available file cache buffers to its minimum (the default

is 10) greatly reducing server performance.  To protect

the server's performance, use the following SET command

at the fileserver console:

SET MINIMUM FILE CACHE BUFFERS = XXXX

where XXXX is a number from 20 to 1000.  Setting this

value will cause the server to report an out of memory

error when any NLM requests memory that would allocate

from the minimum file size cache.  REMEMBER, Backup Exec

for NetWare requires TWO MEGABYTES of memory over that

of your normal operating environment.

Q:   My server constantly reports:

"[date time] Primary Interrupt Controller Detected

a lost Hardware Interrupt"

when I run Backup Exec for NetWare.  All server and Backup

Exec functions appear to be working correctly. What does

this mean?

A:   This message indicates that the server "thinks" that a

defective controller card is installed, or that there is

an interrupt conflict in the system. Verify that there

are no interrupt conflicts in the system.  If the above

message persists, this message can be suppressed by using

the following SET command at the fileserver console:

SET DISPLAY LOST INTERRUPT ALERTS = OFF

Q:   When I try to use RCONSOLE to control the NetWare Client

remotely, I can't use the <+> or <-> keys for Quick File

selections.  Instead of producing the include/exclude

windows, my screen scrolls through the server's console

screens.  How can I create Quick File selections within

the NetWare Client while using RCONSOLE?

A:   In NetWare 3.11 and 3.12, Novell's RCONSOLE reserves the

<+> and <-> keys on the numeric keypad for stepping through

the server's console screens.  Use the <+> and <-> keys on

the main keyboard to make Quick File selections from within

the NetWare Client.  (In NetWare 4.x, the <Alt+F4> and

<Alt+F3> keys are used to step through the server's console

screens.  Therefore, you may use the <+> and <-> keys on

the numeric keypad for Quick File selections in NetWare

4.x.)

Q:   I was running Backup Exec for NetWare and an "Assertion

Failed" error message similar to this appeared:

Assertion Failed in xxx.c

<Press any key to close screen>

What does this mean, and why did it happen?

A:   Backup Exec for NetWare continuously checks the integrity

of the memory allocated to it.  If Backup Exec finds a

memory corruption situation that could cause the server to

abend (possibly due to another NLM modifying Backup Exec's

memory), the above message is displayed and the Backup

Exec for NetWare NLM is unloaded.  You need to down the

server and re-boot to clear the condition.  Use <Alt+Esc>

to return to your system console ":" prompt and down the

file server.  Bring the server back up and re-start the

Backup Exec software.

Q:   Can one user access data placed on tape by another user?

A:   No.  Each user sees only the data that he has placed on

tape.  However, the administrator can access all data on

all tapes, no matter who backed it up.  This "virtual tape

system" allows users to share the tape system as if they

were the only one using it, and extends network security

to the tape system.

Q:   How do I unload the Backup Exec for NetWare Job Manager?

A:   Change to the Job Manager screen on the server console, and

type a <CTRL-X> to terminate the Job Manager.  To restart

the Job Manager, type BESTART at the console prompt ":".

Q:   Does Backup Exec for NetWare verify the data I place on

tape?

A:   During all tape write operations (backup, file grooming,

etc.), Backup Exec generates and records checksums for the

data placed on tape.  These checksums are a mathematical

validation for the data on tape.  During all tape read

operations (restore, verify, etc.), these checksums are

recalculated from the data read from the tape, and compared

to the original.  If there is an error in the data, the

checksums will not match and the software will report an

error.

Q:   How can I calculate the performance (MB/minute) for my

backup system?

A:   The Job History log for each job shows the MB/minute

average throughput for the job.  Note that these results

may vary depending on the type of data, network traffic,

and other factors.

9.  CHANGES TO DOCUMENTATION

========================

Administrator's Manual

-----------------------------------------------------------

Page 3 - 6

The function 'Update Agent List' has been added to the

Commands menu in the NetWare and DOS clients. Selecting

this item updates the list of agents on the network which

are available for Backup Exec operations.  When the

'Update Agent List' item is selected, agents that are no

longer on the network are deleted from the list. The agent

list is used when making Backup and File Grooming

selections, and when selecting an alternate restore

target device.

-----------------------------------------------------------

10. Notes for Autoloader Users

==============================

Backup Exec for NetWare treats all tapes in the tape magazine as

a single "family" of tapes, and will automatically switch to the

next tape as required for backup operations.  When an empty slot

in the magazine or the end of the tape magazine is encountered,

Backup Exec for NetWare will report an error.  Remember that all

operations (erase, overwrite, append, etc) will apply to ALL

tapes in the magazine.

Note that it is not necessary to keep a full magazine in the

autoloader.  However, you should make sure enough tapes are in

the autoloader to meet your backup needs until the magazine is

changed, and that there are no gaps between the tapes in the

magazine.

Backup Exec places a unique identifier on each tape during the

backup process, and will number each tape as it is created.

Therefore, no pre-formatting of blank tapes is required.

To load a cleaning cartridge, first perform the

Utilities/Eject Tape operation in the Job Manager main menu.

After the tape is returned to the magazine, remove the magazine

(if needed for access to the drive) and manually insert the

cleaning tape into the drive.  After the cleaning cycle is

complete, the cleaning tape will be ejected from the drive.

After it is ejected, reload the tape magazine to resume normal

operation.

Backup Exec for NetWare will prompt you to insert a tape

"family" (i.e. magazine) when a job executes to restore files

that are not present on the currently loaded magazine.  When

this is requested, select the Utilities/Eject Tape operation in

the Job Manager main menu, and then remove the current magazine

and insert the requested magazine.  The restore operation will

then proceed automatically.  Backup Exec for NetWare will select

the needed tape, insert it into the drive, and perform the

restore operation.

11. CONTENTS OF INSTALLATION DISKS

==============================

This section outlines the files that can be found on each of

the Backup Exec for NetWare installation diskettes.

Backup Exec for NetWare v4.01

3.5" 1.44MB and 5.25" 1.2MB Diskettes

----------------------------------------------------------

Disk 1:

!CNR1600.CFG        EISA Config file for the Conner (Maynard)

SCSI adapter

@568D.ADF           ADF for Conner (Maynard) Microchannel SCSI

adapter

1DISKSEQ            Installation control file

BE.REQ              Installation control file

BE40INST.NLM        Install program's link to Directory Services

BEDIAG.NLM          Backup Exec diagnostics utility

BEINSTL.NLM         Backup Exec install program

BEINSTL.RES         Resource file for the install program

BEZIP.SIZ           Installation control file

BE_DISK.1           Compressed data file for software

BKUPEXEC.SPG        Installation control file

README.DOC          This file

SMS_WS.ZIP          Contains SMS TSAs for DOS and OS2

Disk 2:

2DISKSEQ            Installation control file

BE_DISK.2           Compressed data file for software

Disk 3:

3DISKSEQ            Installation control file

BE_DISK.END         Compressed data file for software