REFTEST, Version 1.0      6/25/94       by Gold Dragon

DESCRIPTION:

REFTEST will test your machine to determine the best RAM refresh time

for your computer and create a short .COM program to set the refresh

counter at this rate.  This can result in a performance increase of 5

to 25%.

SYNTAX:

REFTEST T[filename] | Vn | Mn

T         : Test to find optimum counter value

Tfilename : Logs results to named file

Vn        : Tests using specific value n only

Mn        : Makes QFRESH.COM using value n

"n" is a decimal integer value (1 to 65535), usually between 20 and

1024. "filename"'is any legal DOS filename; will overwrite existing

files.

BACKGROUND:

Dynamic RAM essentially acts like a tiny capacitor.  Periodically, it

has to be "recharged", or refreshed, to retain its contents.  All

PC's have a counter that keeps track of when this needs to be done.

When the counter counts down to zero, a refresh is triggered.  This

counter is adjustable, and is defaults to a more or less arbitrary

number.

During the refresh cycle, the CPU is essentially suspended, so no

"work" gets done during a refresh.  This is known as "overhead", and

usually eats up somewhere between 5 and 25% of your computer's

processing time.

Most RAM used in computers today doesn't need to be refreshed near as

often as the default specifies.  By adjusting the counter value, you

can regain much of that lost overhead time.

The down side is that if you reduce the refresh rate too far, your

RAM will lose its charge before a refresh gets done, and some data

will be lost.  This will invariably result in a PARITY CHECK error,

and you will have to reboot your computer.  This has absolutely no

harmful effect on your computer!  It just means that your RAM needs a

faster refresh rate.

OPERATION:

Run "REFTEST" alone for a brief description of the program.  Then run

"REFTEST Tref.log" to start testing.  (This will create "REF.LOG",

which will be a duplicate of what comes up on the screen.)  Every 15

to 30 seconds, a line will appear that says,

Value : nnn    Performance increase : xx%

Higher values represent a longer delay between refresh cycles, and

thus greater performance increases.  (The test will actually start

with a refresh rate 6 times FASTER than the default, which will give

a (probably large) NEGATIVE performance increase).

Eventually, the percentage performance increase will level out, and

will not rise any higher.  At this point, stop the program using

Control-C (it won't really stop until it finishes a test cycle).

Pick the LOWEST value that corresponds to the highest percentage

increase reached.  This will usually be somewhere between 100 and

1000.  This is the "n" value you want to use in the next step.

Now run "REFTEST Mn" where n is the lowest number that resulted in

the highest performance increase.  This will create QFRESH.COM

optimized for your system, and will report the percentage increase it

provides.  You should put QFRESH in your AUTOEXEC.BAT.

NOTES:

If you let "REFTEST T" run long enough, your system will eventually

crash with a parity error.  Just cold-reboot.  Take a look at the log

file to see where it died.  If it died before the performance

increase leveled off, this will show the highest "n" value you can

get away with.  If you take exception to your computer locking up

with a parity error in the middle of your work, running your machine

right on the ragged edge of the refresh cycle is probably not a great

idea.

Note that "n" can be any integer value, not just the ones that show

up during testing.

The default "n" value is 18.  Values higher than this will speed up

your machine; values lower will slow it down.   In some instances

this may be desirable: if you have a flakey RAM chip somewhere that

results in a PARITY CHECK even at the default refresh rate, you can

use a small value of "n"--between 1 and 17--and increase the refresh

cycle.  This could get you through an important project until you

have time to isolate and replace the defective chip.

For some reason, some computers will crash with a "n" value of 1.

IMPORTANT NOTE: The parity error is your safety net when playing with

refresh cycles.  If you get a PARITY CHECK, you've gone too far; if

not, you're OK.  Unfortunately, not all IBM "compatibles" utilize

parity.  A few less expensive clones--in particular, Tandy's lower-

end models, like the 1000's--don't.

The easiest way to tell if your computer uses parity checking is so

see how many RAM chips it takes to add a bank of RAM.  If there are 9

chips per bank, you have parity; if there are only 8, you don't.

If your machine doesn't have parity, you could go past the allowable

refresh cycle time and not know it.  Nothing would go wrong until you

happened to execute code that happened to reside at a RAM location

that needs a faster refresh.  Then you will undoubtably get an abrupt

crash.

If you have one of these machines without parity, your best bet is to

thoroughly test the RAM at the new refresh rate you choose.

TECHNICAL INFO:

Modifying the refresh cycle on a PC is really quite simple.  Here is

the assembly source for QFRESH.COM, which is created by REFTEST:

MOV     AL, 74

OUT     43, AL          ; Tell timer to accept 2 byte value

MOV     AL, <ll>        ; <ll> is low order byte

OUT     41, AL

MOV     AL, <hh>        ; <hh> is high order byte

OUT     41, AL

INT     20              ; Exit to DOS

The motivation for this program comes from the July 1988 issue of PC

Magazine, which has an excellent discussion of RAM refresh cycles.

See it for further information.

----------------------------------------------------------------------------

I would be interested in hearing your results with this program,

especially if you get a performance improvement to of over 15%.

If you would like to let me know how it went, please send a

description of your system (type of computer, clock speed, amount of

memory, etc.) and a copy of the resulting REFTEST log file to me.

Look for US!