3Com EtherLink II Adapter Installation Notes

============================================

1. IMPORTANT NOTES

---------------

If you use this MLID, 3C503.COM is the only driver you need. You should

NOT install or load any other programs supplied by 3Com for your

EtherLink II adapter. (You may have to delete the line in your

CONFIG.SYS that loads 3Com's ETH503.SYS ("DEVICE=ETH503.SYS") or its

equivalent.)  This MLID will not work if another driver for the same

adapter is loaded.  Make sure to run the setup program 3C503.EXE that

comes with the card, you must use it to set the IRQ for the card first.

2. GENERAL INFORMATION

-------------------

There are three methods of transferring data to and from the EtherLink

II adapter: memory-mapped (or shared memory) mode, DMA mode, and

programmed (or port ) I/O mode. You may not choose all three modes at

the same time.

If you have an IBM PC AT or compatible, you may choose memory-mapped

(or shared memory) mode or programmed (or port) I/O mode by changing

the memory jumper setting. If you disable the memory jumper setting by

selecting "No Memory" or "Disable", you are using programmed (or port)

I/O mode . If you select any other setting except the "No Memory" or

"Disable" setting , you are using memory-mapped (shared memory) mode.

If you have an IBM PC, IBM PC XT, or compatible, you may use DMA mode

or memory-mapped (or shared memory) mode by changing the memory jumper

setting. If you disable the memory jumper setting by selecting "No

Memory" or "Disable", you are using DMA mode. If you select any other

setting except the "No Memory" or "Disable" setting , you are using

memory-mapped  (shared memory) mode.

Note that, in all cases, the actual jumper settings on the card and the

parameters  specified in the INSTALL program must match or the MLID

will not work.

The EtherLink II adapter will work on either thin Ethernet cabling

(connected via a BNC connector) or thick Ethernet cabling (connected

via a DIX connector).

For more information about installing and configuring your adapter,

please refer to the "3Com EtherLink II Adapter Guide" that comes with

your  adapter.