IBM Token-Ring 16/4 Adapter (16 bit microchannel card) Installation Notes

=========================================================================

1. IMPORTANT NOTES

---------------

You must load IBM's Token-Ring driver BEFORE loading the TOKEN.COM file.

The IBM's Token-Ring driver may be either:

(1) one or more .SYS files (supplied with the IBM Local Area Network

Support Program) that you must specify in your CONFIG.SYS, or

(2) the TOKREUI.COM file (supplied with most IBM Token-Ring Network PC

Adapters). You may have to edit your AUTOEXEC.BAT file AFTER you run

INSTALL to make sure that TOKREUI.COM is loaded before TOKEN.COM.

The IBM Token-Ring driver must also be loaded before ASHARE.COM, even if

TOKEN.COM is loaded after ASHARE.COM.

If you are going to be using NETBIOS on a token-ring network, you must

load TOKEN.COM before running any application that uses the NETBIOS

interface. (The NETBIOS driver may be loaded by CONFIG.SYS, but the

adapter will not be initialized until an application sends a command to

the NETBIOS interface. The TOKEN MLID must initialize the adapter.)

Most adapter open parameters are set to the default values. If your

NETBIOS application runs before TOKEN.COM, the open parameters it sets

will prevent TOKEN.COM from running.

2. GENERAL INFORMATION

-------------------

There are one or two configuration menus for the primary and secondary

memory addresses used by the adapter. (There are two for the Adapter/A,

and one for the non-MicroChannel adapters.) The primary memory address

is the address of the ROM on the Token-Ring Adapter. This ROM is usually

8K in size. The secondary memory address is the address of the RAM to be

used for buffer pools on the adapter. This RAM is usually 8K or 16K in

size. (The primary and secondary memory addresses correspond to the MEM

#1 and MEM #2 parameters in NET.CFG, respectively .)

For instructions on installing the IBM Token-Ring driver, consult your

"Guide to Operations" or other documentation that comes with your Token

-Ring adapter or Local Area Network Support Program.