3Com EtherLink II/16 TP Adapter Installation Notes

==================================================

1. IMPORTANT NOTES

---------------

If you use this MLID, 3C503.COM is the only driver you need. You should

NOT install or load any other programs supplied by 3Com for your EtherLink

II/16 TP adapter. (You may have to delete the line in your CONFIG.SYS

that loads 3Com's ETH503.SYS ("DEVICE=ETH503.SYS") or its equivalent.)

This MLID will not work if another driver for the same adapter is loaded.

2. GENERAL INFORMATION

-------------------

There are three methods of transferring data to and from the EtherLink

II/16 TP adapter: memory-mapped (or shared memory) mode, DMA mode, and

programmed (or port ) I/O mode. You may not choose all three modes at the

same time.

If you have an IBM PC AT or compatible, you may choose memory-mapped

(or shared memory) mode or programmed (or port) I/O mode by changing the

memory jumper setting. If you disable the memory jumper setting by

selecting "No Memory" or "Disable", you are using programmed (or port) I/O

mode . If you select any other setting except the "No Memory" or

"Disable" setting , you are using memory-mapped (shared memory) mode.

If you have an IBM PC, IBM PC XT, or compatible, you may use DMA mode or

memory-mapped (or shared memory) mode by changing the memory jumper

setting If you disable the memory jumper setting by selecting "No Memory"

or "Disable", you are using DMA mode. If you select any other setting

except the "No Memory" or "Disable" setting , you are using memory-mapped

(shared memory) mode.

Note that, in all cases, the actual jumper settings on the card and the

parameters  specified in the INSTALL program must match or the MLID will

not work.

The EtherLink II/16 TP adapter will work on either Twisted Pair Ethernet

cabling (connected via an RJ-45 connector) or thick Ethernet cabling

(connected via a DIX connector).

For more information on installing and configuring your adapter, please

refer to "3Com EtherLink II/16 TP Adapter Guide" that comes with your

adapter.