IBM Token-Ring Network 16/4 Adapter Installation Notes

======================================================

1. IMPORTANT NOTES

---------------

If you will be using NETBIOS on a token-ring network, you must load

TOKEN.COM before running any application that uses the NETBIOS

interface.  (The NETBIOS driver may be loaded by CONFIG.SYS, but the

adapter will not be initialized until an application sends a command to

the NETBIOS interface.  The TOKEN MLID must initialize the adapter.)

Most adapter open parameters are set to the default values.  If your

NETBIOS application runs before TOKEN.COM, the open parameters it sets

will prevent TOKEN.COM from running.

2. GENERAL INFORMATION

-------------------

There are one or two configuration menus for the primary and secondary

memory addresses used by the adapter.  (There are two for the

Adapter/A, and one for the non-MicroChannel adapters.)  The primary

memory address is the address of the ROM on the Token-Ring Adapter.

This ROM is usually 8K in size.  The secondary memory address is the

address of the RAM to be used for buffer pools on the adapter.  This

RAM is usually 8K or 16K in size.  (The primary and secondary memory

addresses correspond to the MEM #1 and MEM #2 parameters in NET.CFG,

respectively.)

For instructions on installing the IBM Token-Ring driver, consult your

"Guide to Operations" or other documentation that comes with your

Token-Ring adapter or Local Area Network Support Program.