6.	FAX CLASS 1 COMMANDS

6.1.	FAX I/O PROCESSING

The fax I/O interface supports asynchronous serial and parallel interfaces. The interface rate is 19200 bps. The character format is 8 bits data, no parity, and 1 stop bit. Start and stop elements are removed from the transmit data and added to the receive data. Both transmit and receive data are buffered. Flow control using XON/XOFF (DC1/DC3) or RTS/CTS is provided.

Unique control character strings are identified, filtered, or reinserted into the I/O data stream. These control characters and their resultant action are described below.

6.1.1.	DTE-to-Modem Transmit Data Stream

Characters Detected	Action Taken

<DLE><data>	Delete <DLE><data> characters.

<DLE><ETX>	Recognize as a string terminator and take appropriate action.

<DLE><DLE>	Replace with single <DLE> character.

6.1.2.	Modem-to-DTE Receive Data Stream

Characters Detected	Action Taken

<DLE>	Insert extra <DLE> ahead of <DLE>.

The modem also identifies the end of a frame by inserting <DLE><ETX> into the data stream after the FCS bytes.

6.1.3.	Fax Mode Selection

Fax class 1 commands are identified in Table 6-1. Fax modes and rates are determined by the AT+F commands as described in Section 6.2.

Table 6-1.  Fax Class 1 Commands

Command

Function

Service Class ID

+FCLASS=

Service Class

Fax Class 1 Commands

+FAE=n

Data/Fax Auto Answer

+FTS=n

Stop Transmission and Wait

+FRS=n

Receive Silence

+FTM=n

Transmit Data

+FRM=n

Receive Data

+FTH=n

Transmit Data with HDLC Framing

+FRH=n

Receive Data with HDLC Framing

6.1.4.	Fax Origination

Origination of fax calls is made using the ATD command. Upon completion of the dial function, a calling tone at 1100 Hz is transmitted, with a cadence of 0.5 seconds on and 3 seconds off. The modem automatically enters mode +FRH=3 and sends the CONNECT message to the DTE when FSK flags are detected from the remote.

6.1.5.	Fax Answering

Answering of fax calls is identical to answering of data calls with the exception that the modem enters the fax handshaking mode instead of the data handshaking mode after going off-hook. If +FAE=0, the modem, after sending answer tone, automatically enters fax mode (+FTH=3), sends the CONNECT message to the DTE, and starts sending FSK flags. If +FAE=1, the modem determines whether the caller is a data modem or fax modem and sends the DATA or FAX result code, respectively, to the DTE.

6.1.6.	Fax Control Transmission

Fax control transmission is initiated by the +FTH=n command. After this command is issued, the modem generates the CONNECT message and transmits carrier in the modulation scheme specified by the parameter n. The modem then transmits HDLC flags for a minimum of 1 second. The modem continues to transmit the HDLC flags until it receives a character from the DTE.

When characters are received from the DTE, the modem adds start and end flags, performs zero-bit insertion, generates FCS, and deletes <DLE><chr> character pairs before transmitting the characters to the remote fax machine. Each <DLE><DLE> sequence is transmitted as a single <DLE>. <DLE><ETX> is considered as the end of frame marker and is not transmitted. All data received from the DTE after <DLE><ETX> is ignored by the modem until the modem generates either the CONNECT, OK, or ERROR result code.

If no more data is in the transmit buffer and the final bit was a 1 (bit 4 of the second byte received from the DTE), the modem generates the OK result code and returns to the command mode. If the final bit was a 0, the modem generates the CONNECT message and waits for further data from the DTE while transmitting HDLC flags. If no more data is received before 5 seconds elapse, the modem drops carrier, goes on-hook, and generates the ERROR result code.

6.1.7.	Fax Control Reception

Fax control reception is initiated using the AT+FRH=n command. After this command is issued, the modem looks for carrier in the modulation scheme specified by the parameter n. If no carrier is detected before the period of time specified by register S7 expires, the modem generates the NO CARRIER message and returns to command mode. If carrier is detected that is not the one specified by the parameter n, the modem generates the +FCERROR message and returns to the command mode. If the specified carrier is detected, the modem generates the CONNECT message and enters the HDLC receive mode.

In HDLC receive mode, the modem receives HDLC frames from the remote fax machine, strips the flags, performs zero-bit deletion, performs error checking, and handles <DLE><chr> character pairs before passing the data to the DTE. The modem prefixes each <DLE> character with another <DLE> character before sending it to the DTE. After the last byte in the frame, the modem sends <DLE><ETX> to the DTE marking the end of the frame. The modem then generates either the OK message if no errors were detected or the ERROR message if errors were detected (FCS is incorrect), and returns to command mode.

While in command mode, the modem continues to receive data in the selected modulation scheme, and sends the data after sending the CONNECT message to the DTE when the DTE reissues the +FRH command with the same parameter. If the DTE issues the +FRH command with a different parameter, the modem clears all buffers and proceeds as described previously.

If carrier is lost while in command mode and the DTE reissues the +FRH command with the same parameter, and there is no data in the buffer, the modem sends the ERROR result code to the DTE and returns to the command mode. If there is data in the buffer, the modem sends the next frame of buffered data to the DTE, followed by <DLE><ETX>, and either the ERROR result code if errors were detected or the OK result code if no errors were detected. The modem then returns to command mode.

The modem concludes an error is detected if carrier is lost for any period of time during or before the reception of a frame. If carrier is lost for a time period longer than the time specified by the register S10, the modem finishes delivering the data in the receive buffer (if any) to the DTE, sends <DLE><ETX>, generates the ERROR message, and returns to command mode. All subsequent data received from the remote is discarded.

If the modem detects a frame abort sequence (seven consecutive ones with no zero insertion) while it is waiting for a frame beginning flag (it was receiving HDLC flags), it will wait for the HDLC flags again until either carrier is lost, or the DTE aborts the process by sending an abort character or by dropping DTR with &D2 in effect. If the frame abort sequence is detected while the modem is receiving a frame it finishes delivering the data in the receive buffer (if any) to the DTE, sends <DLE><ETX>, generates the ERROR message, and returns to command mode. The modem keeps looking for HDLC flags followed by more data from the remote, with the selected modulation scheme.

If the modem detects a receive buffer overflow condition, it concludes that there was an FCS error in that frame. The modem will receive more frames only if a starting flag is detected and there is room in the receive buffer. If a starting flag is detected and there is no room in the buffer, the modem discards all data in that frame.

If the modem receives any character from the DTE after the +FRH command (except flow control characters if software flow control is in effect), or if the modem detects a high-to-low transition of the DTR signal while &D1 is in effect, it sends <DLE><ETX> to the DTE, generates OK result code, and returns to command mode. The receive buffer is cleared and all data received from the remote is discarded. If the modem detects a DTR drop while &D2 is in effect, it goes on-hook, sends <DLE><ETX> to the DTE, generates OK result code, and returns to command mode. If the modem detects a DTR drop while &D3 is in effect, the modem performs a warm reset.

6.1.8.	Fax Data Transmission

Fax data transmission is initiated by the AT+FTM=n command. After this command is issued, the modem generates the CONNECT message and transmits carrier in the modulation scheme specified by the parameter n. The modem then transmits constant 1 bits for a minimum of one second and continues to transmit constant 1 bits until it receives a character from the DTE.

When data is received from the DTE, the modem deletes start and stop bits and deletes all <DLE><chr> character pairs before transmitting the data to the remote. Each <DLE><DLE> sequence is transmitted as a single <DLE>. <DLE><ETX> is considered as the end of stream marker, and is not transmitted. All data received from the DTE after the <DLE><ETX> is ignored by the modem until the modem generates either the CONNECT, OK, or ERROR result code.

If no more data is in the transmit buffer, and the last transmitted character was not an ASCII NULL, the modem generates the OK result code and returns to the command mode. If the last character transmitted was an ASCII NULL, the modem generates the CONNECT message to the DTE and waits for further data from the DTE while transmitting NULLs to the remote. If more data is received before five seconds elapse, the modem continues to transmit the data as described in the previous paragraph. If five seconds elapse and no data is received from the DTE, the modem drops carrier, goes on-hook, and generates the ERROR result code.

6.1.9.	Fax Data Reception

Fax data reception is initiated using the AT+FRM=n command. After this command is issued, the modem looks for carrier in the modulation scheme specified by the parameter n. If no carrier is detected before the period of time specified by register S7 expires, the modem generates the NO CARRIER message and returns to command mode. If carrier is detected that is not the one specified by the parameter n, the modem generates the +FCERROR message and returns to the command mode. If the specified carrier is detected, the modem generates the CONNECT message and enters the data receive mode.

While in data receive mode, the modem receives data from the remote, adds start and stop bits, and handles <DLE><chr> character pairs before passing the data to the DTE. The modem prefixes each <DLE> character with another <DLE> character before sending it to the DTE.

If carrier is lost for a time period longer than the time specified by the register S10, the modem finishes delivering the data in the receive buffer (if any) to the DTE, sends <DLE><ETX>, generates the NO CARRIER result code, and returns to the command mode. All subsequent data received from the remote is discarded.

If the modem detects a receive buffer overflow condition, it stops receiving from the remote until there is room in the receive buffer. The modem informs the DTE of the buffer overflow after it sends to the DTE the last character that was stored in the buffer before the overflow occurred.

If the modem receives any character from the DTE after the +FRM command (except flow control characters if software flow control is in effect), or if the modem detects a high-to-low transition of the DTR signal while &D1 is in effect, it sends <DLE><ETX> to the DTE, generates the OK result code, and returns to command mode. The receive buffer is cleared and all data received from the remote is discarded. If loss of DTR is detected while &D2 is in effect, the modem goes on-hook, sends <DLE><ETX> followed by OK result code to the DTE, and returns to command mode. If the modem detects a DTR drop while &D3 is in effect, the modem performs a warm reset.

6.2.	COMMANDS

+FCLASS=n - Select Service Class

+FCLASS=n command sets the active service class.

Parameters: 0-2

Command options:

+FCLASS=0	Select Data Mode (Default.)

+FCLASS=1	Select Facsimile Class 1

+FCLASS=2	Select Facsimile Class 2

+F<command>? - Report Active Configuration

+F<command>? interrogates the modem to determine the active configuration.

Typical responses are:

+FAE?	0 if auto answer is disabled; 1 if auto answer is enabled

+FCLASS?	0 if in data mode; 1 if in fax class 1; 2 if in fax class 2

+F<command>=? - Report Operating Capabilities

+F<command>=? can be used to determine the operating capabilities of the modem.

Typical responses are:

+FAE=?	0 or 1

+FCLASS=?	0, 1, or 2

+FTM=?	24, 48, 72, 96 (9600 bps version)

+FRM=?	24, 48, 72, 96 (9600 bps version)

+FTM=?	24, 48, 72, 73, 74, 96, 97, 98, 121, 122, 145, 146 (14400 bps version)

+FRM=?	24, 48, 72, 73, 74, 96, 97, 98, 121, 122, 145, 146 (14400 bps version)

+FAE=n - Data/Fax Auto Answer

+FAE=n allows the DTE to either restrict answering to class 1, or to automatically detect whether the calling station is a fax class 1 modem or data modem, and answer accordingly.

Parameters: n = 0 or 1

Command options:

n = 0	Disable data/fax auto answer mode. The modem answers as a fax modem only. (Default).

n = 1	Enable data/fax auto answer mode. The modem answers as a fax or data modem.

After a data (not fax) connection is acheived (indicated by the DATA result code), the DTE must issue an ATO command to cause the modem to go on-line.

+FTS=n - Stop Transmission and Wait

+FTS=n causes the modem to terminate a transmission and wait for n 10-ms intervals before responding with the OK result code. An ERROR response code results if this command is issued while the modem is on-hook.

+FRS=n - Receive Silence

+FRS=n causes the modem to report back to the DTE with an OK result code after n 10 ms-intervals of silence have been detected on the line. This command is aborted if any character is received from the DTE. The modem discards the aborting character and issues an OK result code. An ERROR response code results if this command is issued while the modem is on-hook.

+FTM=n - Transmit Data

+FTM=n causes the modem to transmit data using the modulation defined below. An ERROR response code results if this command is issued while the modem is on-hook.

Parameters: See Command options

Command options:

+FTM=24	V.27 ter 2400 bps
	+FTM=48	V.27 ter 4800 bps
	+FTM=72	V.29 7200 bps
	+FTM=73	V.17 7200 bps long
	+FTM=74	V.17 7200 bps short 
	+FTM=96	V.29 9600 bps
	+FTM=97	V.17 9600 bps long
	+FTM=98	V.17 9600 bps short
	+FTM=121	V.17 12000 bps long
	+FTM=122	V.17 12000 bps short
	+FTM=145	V.17 14400 bps long
	+FTM=146	V.17 14400 bps short

+FRM=n - Receive Data

+FRM=n causes the modem to enter the receiver mode using the modulation defined below. An ERROR response code results if this command is issued while the modem is on-hook.

Parameters: See Command options

Command options:

+FRM=24	V.27 ter 2400 bps
	+FRM=48	V.27 ter 4800 bps
	+FRM=72	V.29 7200 bps 
	+FRM=73	V.17 7200 bps long 
	+FRM=74	V.17 7200 bps short 
	+FRM=96	V.29 9600 bps 
	+FRM=97	V.17 9600 bps long
	+FRM=98	V.17 9600 bps short
	+FRM=121	V.17 12000 bps long
	+FRM=122	V.17 12000 bps short
	+FRM=145	V.17 14400 bps long
	+FRM=146	V.17 14400 bps short

+FTH=n - Transmit Data with HDLC Framing

+FTH=n causes the modem to transmit data using HDLC protocol and the modulation defined below. An ERROR response code results if this command is issued while the modem is on-hook.

Parameters: See Command options.

Command options:

+FTH=3	V.21 channel 2 300 bps
	+FTH=24	V.27 ter 2400 bps
	+FTH=48	V.27 ter 4800 bps
	+FTH=72	V.29 7200 bps
	+FTH=73	V.17 7200 bps long 
	+FTH=74	V.17 7200 bps short 
	+FTH=96	V.29 9600 bps
	+FTH=97	V.17 9600 bps long
	+FTH=98	V.17 9600 bps short
	+FTH=121	V.17 12000 bps long
	+FTH=122	V.17 12000 bps short
	+FTH=145	V.17 14400 bps long
	+FTH=146	V.17 14400 bps short

+FRH=n - Receive Data with HDLC Framing

+FRH=n causes the modem to receive frames using HDLC protocol and the modulation defined below. An ERROR response code results if this command is issued while the modem is on-hook.

Parameters: See Command options.

Command options:

+FTH=3	V.21 channel 2 300 bps
	+FRH=24	V.27 ter 2400 bps
	+FRH=48	V.27 ter 4800 bps
	+FRH=72	V.29 7200 bps
	+FRH=73	V.17 7200 bps long 
	+FRH=74	V.17 7200 bps short 
	+FRH=96	V.29 9600 bps
	+FRH=97	V.17 9600 bps long
	+FRH=98	V.17 9600 bps short
	+FRH=121	V.17 12000 bps long
	+FRH=122	V.17 12000 bps short
	+FRH=145	V.17 14400 bps long
	+FRH=146	V.17 14400 bps short

6.3.	EXAMPLES

Examples of calling (transmitting) and answering (receiving) one page using fax class 1 commands are shown in Tables 6-2 and 6-3, respectively.

Table 6-2. Fax Class 1 Calling Sequence (One Page)

DTE Commands	DCE Responses	
	(Host)	(Modem)	Remote Fax	Notes

(1)	AT+FCLASS=1	(2)	OK			Set to Class 1

PHASE A

(3)	ATDT6163	(4)	Dials	(5)	Answers	+FRH=3 implied by Dialing
			(6)	CONNECT	(7)	Send HDLC flags

PHASE B

(8)	Send NSF frame
			(9)	<NSF>,OK
	(10)	AT+FRH=3	(11)	CONNECT
					(12)	Send CSI frame
			(13)	<CSI>,OK
	(14)	AT+FRH=3	(15)	CONNECT
					(16)	Send DIS frame	Last frame bit = 1
			(17)	<DIS>,OK	(18)	Drop carrier
	(19)	AT+FTH=3	(20)	Send HDLC flags	(21)	Receive flags
			(21)	CONNECT
	(22)	<TSI>	(23)	Send TSI frame	(24)	Receive TSI	Last frame bit = 0
			(25)	CONNECT
	(26)	<DCS>	(27)	Send DCS frame	(28)	Receive DCS	Last frame bit = 1
			(29)	Detect last frame bit
			(39)	OK, drop carrier
	(31)	AT+FTS=8	(32)	OK, wait 80 ms
	(33)	AT+FTM=96	(34)	Send V.29
			(35)	CONNECT
	(36)	<TCF>	(37)	Send TCF data	(38)	Receive & check
			(39)	OK
	(40)	AT+FRH=3	(41)	CONNECT
			(42)	Send CFR frame		Last frame bit = 1
	(43)	<CFR>,OK	(44)	Drop carrier
			(45)	OK

PHASE C

(46)	AT+FTM=96	(47)	Send V.29
			(48)	CONNECT
	(49)	age data	(50)	Send page data	(51)	Receive data
			(52)	OK
	(53)	AT+FTH=3	(54)	Send HDLC flags	(55)	Receive flags
			(56)	CONNECT

PHASE D

(57)	<EOP>	(58)	Send EOP frame	(59)	Receive EOP	Last frame bit = 1
			(60)	OK, drop carrier
	(61)	AT+FRH=3	(62)	CONNECT
					(63)	Send MCF frame	Last frame bit = 1
			(64)	<MCF>, OK
	(65)	AT+FTH=3	(66)	Send HDLC flags	(67)	Receive flags
			(68)	CONNECT
	(69)	<DCN>	(70)	Send DCN frame	(71)	Receive DCN	Last frame bit = 1
			(72)	OK, drop carrier

PHASE E

(73)	ATH0	(74)	OK, hang up	(75)	Hang up

Table 6-3. Fax Class 1 Answering Sequence (One Page)

DTE Commands	DCE Responses		
	(Host)	(Modem)	Remote Fax	Notes

(1)	AT+FCLASS=1	(2)	OK			Set to Class 1

PHASE A

(3)	FAX machine dials
			(4)	RING
	(5)	ATA	(6)	Modem answers
			(7)	Send HDLC flags	(8)	Receive flags	+FTH=3 implied by
							answering
			(9)	CONNECT

PHASE B

(10)	<NSF>	(11)	Send NSF frame	(12)	Receive NSF	Last frame bit = 0
			(13)	CONNECT
	(14)	<CSI>	(15)	Send CSI frame	(16)	Receive CSI	Last frame bit = 0
			(17)	CONNECT
	(18)	<DIS>	(19)	Send DIS frame 	(20)	Receive DIS	Last frame bit = 1
			(21)	OK, drop carrier
	(22)	AT+FRH=3	(23)	CONNECT
					(24)	Send TSI frame	Last frame bit = 0
			(25)	<TSI> , OK
	(26)	AT+FRH=3	(27)	CONNECT
					(28)	Send DCS frame	Last frame bit = 1
			(29)	<DCS>, OK	(30)	Drop Carrier
	(31)	AT+FRM=96
					(32)	Send V.29
			(33)	CONNECT	(34)	Send TCF frame
			(35)	<TCF>	(36)	Drop carrier
			(37)	NO CARRIER
	(38)	AT+FTH=3	(39)	CONNECT
	(40)	<CFR>	(41)	Send CFR frame	(42)	Receive CFR	Last frame bit = 1
			(43)	OK, drop carrier

PHASE C

(44)	AT+FRM=96
					(45)	Send page data
			(46)	<page data>	(47)	Drop carrier
			(48)	NO CARRIER

PHASE D

(49)	AT+FRH=3	(50)	CONNECT
					(51)	Send EOP frame	Last frame bit = 1
			(52)	<EOP>, OK
	(53)	AT+FTH=3	(54)	CONNECT
	(55)	<MCF>	(56)	Send MCF frame	(57)	Receive MCF	Last frame bit = 1
			(58)	OK, drop carrier
	(59)	AT+FRH=3	(60)	CONNECT
					(61)	Send DCN frame 	Last frame bit = 1
			(62)	<DCN>, OK

PHASE E

(73)	ATH0	(74)	OK, hang up	(75)	Hang up

7.	FAX CLASS 2 COMMANDS

The fax class 2 commands are summarized in Table 7-1.

7.1.	COMMAND SYNTAX AND GUIDELINES

7.1.1.	DTE Commands

The ISO 646 character set (CCITT T.50 International Alphabet 5, American Standard Code for Information Interchange) is used for the issuance of commands and responses. Only the low-order 7 bits of each character are used for commands or parameters; the high order bit is ignored. Upper case characters are equivalent to lower case characters.

For Phase C data transmission or reception, all 8 bits are needed.

DTE Command Lines

A command line is a string of characters sent from a DTE to the DCE while the DCE is in a command state. Command lines have a prefix, a body, and a terminator. The prefix consists of the ASCII characters “AT” (065, 084) or “at” (097, 116). The body is a string of commands restricted to printable ASCII characters (032 - 126). Space characters (ASCII 032) and control characters other than CR (ASCII 013) and BS (ASCII 010) in the command string are ignored. The default terminator is the ASCII <CR> character. Characters that precede the AT prefix are ignored.

Basic Command Syntax

Characters within the command line are parsed as commands with associated parameter values. The basic commands consist of single ASCII characters, or single characters preceded by a prefix character (e.g., “&"), followed by a decimal parameter.

Missing decimal parameters are evaluated as 0.

Extended Command Syntax

The facsimile commands use extended syntax. They are preceded by the “+F” characters, and they are terminated by the semicolon “:” character (ASCII 059) or by the <CR> that terminated the command line.

AT+FAE=0	; +FCR 1 <CR>

This command instructs the DCE to automatically answer a data or fax call and also enables reception.

AT+FCLASS=0 <CR>	for Data Mode

AT+FCLASS=1 <CR>	for Service Class 1 Fax

+FCLASS=2 <CR>	for Service Class 2 Fax

In Class 2, the DCE makes and terminates calls, manages the communication session and negotiates (T.30 protocol) and transports the image date to DTE. The T.4 protocol management of image data, etc., is done by DTE.

The response to

AT+FCLASS= 0<CR>	in Data Mode

or

AT+FCLASS=2<CR>	in Service Class 2 Fax

is

OK

The service class may be set by the DTE from the choices available using the “+FCLASS=<VALUE>” command.

General Rules

1.	+Fnnn commands must be entered completely, otherwise an ERROR response is sent.

2.	All response messages are preceded and followed by <CR><LF>. Multiple response commands (e.g., +FDIS: +FCSI: and +FDCS:) will, therefore, appear to a have a blank line between them.

3.	Fax Class 2 commands can be separated by the “:” character. The “;” character can be omitted if desired. Note that non-data commands cannot be separated by the “;” which is allowed as a dial modifier.

Table 7-1.  Fax Class 2 Commands

Command

Function

Service Class ID

+FCLASS=

Service Class

Class 2 Action Commands

D

Originate a Call

A

Answer a Call

+FDT

Data Transmission

+FET=N

Transmit Page Punctuation

+FDR

Begin or Continue Phase C Receive Data

+FK

Session Termination

Class 2 DCE Responses

+FCON

Facsimile Connection Response

+FDCS:

Report Current Session

+FDIS:

Report Remote Identification

+FCFR

Indicate Confirmation to Receive

+FTSI:

Report the Transmit Station ID

+FCSI:

Report the Called Station ID

+FPTS:

Page Transfer Status

+FET:

Post Page Message Response

+FHNG

Call Termination with Status

Class 2 Session Parameters

+FMFR?

Identify Manufacturer

+FMDL?

Identify Model

+FREV?

Identify Revision

+FDCC=

DCE Capabilities Parameters

+FDIS=

Current Sessions Parameters

+FDCS=

Current Session Results

+FLID=

Local ID String

+FCR

Capability to Receive

+FPTS=

Page Transfer Status

+FCR=

Capability to Receive

+FAE

Adaptive Answer

+FBUF?

Buffer Size (Read Only)

+FPHCTO

Phase C Time Out

+FAXERR

Fax Error Value

+FBOR

Phase C Data Bit Order

4.	All Class 2 commands are assumed to be the final command on a command line. Additional characters will be ignored.

5.	An ERROR message will be generated if any of the following conditions occur:

a.	A Class 1 command is received while in Class 2.

b.	A Class 2 command is received while in Class 1.

c.	A Class 1 or Class 2 action command is received while in data modem mode.

d.	A Class 2 read-only parameter is given the “=” form of a +F command (e.g., AT+FAXERR=5).

e.	A Class 2 action command is given the inappropriate “=” or “=?” form (e.g., AT+FDR=?).

7.1.2.	Serial Port Speed and Flow Control

During fax mode, the DTE-DCE port speed is 19200 bps.

The DCE provides a speed buffer of 1024 bytes and provides the DC1/DC3 (XON/XOFF) or RTS/CTS method of controlling the data into the buffer. This flow control is controlled by the &K3 or &K4 command.

This method of data flow control is available only for DTE to DCE direction of data. There is no provision for data flow control from DCE to DTE.

Data Stream Termination

The DCE exchanges streams of data with the DTE while executing data transfer commands. These data streams use the termination described in Section 3.2/ISO 2111.

The ASCII <DLE> character (016) is used as a special character to shield special characters. The <DLE><ETX> character pair (<106><003>) is used to mark the end of a stream. The following patterns are used:

any data...<DLE><ETX>	end of stream

any data...<DLE><DLE>	single <DLE> in data

any data...<DLE><any byte>	delete <DLE><any byte>

DTE to DCE Streams

The DCE filters the data stream from the DTE, and removes all character pairs beginning with <DLE>. The DCE recognizes <DLE><ETX> as the stream terminator. The DCE recognizes <DLE><DLE> and reinserts a single <DLE> in its place.

The DTE must filter stream data to the DCE, and insert extra <DLE> characters ahead of data.

DCE to DTE Streams

The DTE must filter the data stream from the DCE, and remove all character pairs beginning with <DLE>. The DTE must recognize <DLE><ETX> as the stream terminator. The DTE must recognize <DLE><DLE> and reinsert a single <DLE> in its place.

The DCE filters stream data to the DTE, and inserts extra <DLE> characters ahead of data.

7.1.3.	Auto Answer

The DCE can answer as a data DCE or as a fax DCE. It can answer the call adaptively, i.e., it can determine whether the call is 'data' or 'fax'. The +FAA parameter controls this feature.

AT+FAA=1	; Auto answer as a facsimile or a data modem depending on the call

CONNECT XXXX	; DCE status response if data call

+FCON	; DCE status response if fax call

7.1.4.	Identification of T.30 Options

Group 3 devices negotiate session parameters in DIS, DCS, and DTC frames. These parameters are defined in Table 7-2.

7.1.5.	Session Status Reporting

The DCE provides reports to the DTE on the status of a session. The DCE provides the following status reports:

1.	Connection and hang up status:

+FCON

+FHNG:<0-255>

2.	Requested DIS session parameters +FDIS: <string> reports remote facsimile capabilities.

Syntax: +FDIS: VR, BR, WD, LN, DF, EC, BF, ST

3.	+FDCS: <string> reports the negotiated parameters

4.	Phase C prompts:

XON, XOFF, DC2

5.	Phase C base status reports: depending upon copy quality and related end-of page status:

+FPTS: <1-5>

7.1.6.	Procedure Interrupt Negotiation

CCITT allows a station to request a procedure interruption at the end of a page. This request is passed between stations by the PIP, PIN, and PRI-Q messages.

7.2.	SERVICE CLASS 2 IDENTIFICATION AND SELECTION

Three commands report identification and selection information. Each of these three commands cause the DCE to send a message to the DTE. The messages are programmable using ConfigurACE.

7.2.1.	+FMFR?, Request Manufacturer Identification

The +FMFR? command causes the DCE to send a message identifying the DCE product manufacturer. The default message is:

ROCKWELL
OK

7.2.2.	+FMDL?, Identify Product Model

The +FMDL? command causes the DCE to send a message identifying the DCE product model. The default message is similar to:

RC96XXXAC
OK

7.2.3.	+FREV?, Identify Product Revision

The +FREV? command causes the DCE to send a message identifying the DCE product model revision number. The default message is similar to:

TRXX-XXXX-XXX 
OK

7.3.	SERVICE CLASS 2 ACTION COMMANDS

These commands transfer data, and punctuate sessions. They also release specific T.30 messages. All action commands must be the last command on a command line as indicated by the terminating <CR>.

All action commands initiate processes. The modem will not accept other commands from the DTE until the modem issues a final result code (e.g., OK, CONNECT). The modem will abort the process if it receives any character before the final result code is issued.

7.3.1.	ATD, Originate a Call

Syntax:	ATD....<CR>

The DCE can support a DTE command to originate a call using the ATD command (see Section 3.2).

If this command is unsuccessful, the DCE reports an appropriate failure or error type result code such as NO CARRIER, NO DIALTONE, or BUSY (see Section 3.3).

If this call is successful, the typical DCE response is:

ATDnn..nn	(go off-hook, dial, get CED)
+FCON	(DCE detects flags)
[+FCSI:<remote ID string>]
+FDCS:<T.30 subparameter string>
OK

The DCE dials, detects call progress, and generates the CNG tone. It then waits for a DIS frame. On detection of the first Phase B preamble (V.21 ch. 2 modulated by 300 bps HDLC flags), it reports the “+FCON” message to the DTE. The DCE then switches to 19200 bps.

The DCE generates a DCS frame based on the received DIS frame and on the previously set +FDIS parameter. A +FDT command from the DTE releases the DCE to transmit that DCS frame.

The DCE reports the initial received T.30 negotiation messages, including the DIS frame and the optional CSI ID string. The +FDIS: report is followed by the OK final result code.

7.3.2.	ATA, Answer a Call

The DCE can support a DTE command to answer an incoming call using the ATA command (see Section 3.2).

The DTE may issue an Answer command in response to an incoming ring.

If the Answer command is unsuccessful, the DCE will report an appropriate failure or error type result code, such as NO CARRIER (see Section 3.3).

Manual Call Answer

If this call is successful, the typical DCE response (answer and receive) is:

+FCON
[+FTSI:<remote ID string>]
+FDCS:<T.30 subparameter string>
OK
(DTE should issue +FDR command here)

Upon receipt of an Answer command from the DTE, the DCE answers and generates the CED tone. The DCE then generates a DIS frame (derived from the +FDIS parameter) and hunts for the first T.30 negotiation frames. Upon detection of the first Phase B preamble (V.21 ch 2 modulated by 300 bps HDLC flags), it reports the “+FCON” message to the DTE.

The DTE should report the initial received T.30 negotiation messages, including the DCS frame. The +FDCS: report will be followed by the OK final result code.

Automatic Answer

The modem provides for automatic answering of incoming calls. If configured for automatic answer, the modem answers an incoming call in compliance with T.30, and reports the same messages as described for manual answer.

Connection as a Data Modem

If configured to do so by the +FAA parameter, the DCE will adaptively answer as a facsimile DCE or as a data DCE. If the DCE answers as a data DCE, it resets the +FCLASS parameter to 0 and issues the appropriate final result code (e.g., CONNECT, or NO CARRIER) to the DTE.

7.3.3.	+FDT, Data Transmission

Syntax: 	+FDT <CR>

The +FDT command prefixes Phase C data transmission. When the DCE is ready to accept Phase C data, it issues the negotiation responses and the CONNECT result code to the DTE.

In Phase B, the +FDT command releases the DCE to proceed with negotiation, and releases the DCS message to the remote station. In Phase C, the +FDT command resumes transmission after the end of a prior transmit data stream.

Initiate Page Transmission

Phase B DCE polled response:

[+FCSI:<remote ID string>]	- if new CSI received
[+FDIS:<subparameters from remote station>]
	- if new DIS received
+FDCS:<T.30 subparameter string>
CONNECT
<XON>	- when ready for data

After placing a call, or after finishing a document exchange, the DTE may command the DCE to re-enter T.30 Phase B to attempt to negotiate a document transmission.

Continue a Page

CONNECT
<XON>

The DTE may issue more than one +FDT command for a given page, so that different files may be concatenated together. These files must have the same format.

Phase C Data Framing

Phase C data must be presented to the DCE in stream mode. The DCE expects Phase C data to follow until it detects <DLE><ETX> termination characters. The DCE will filter the stream as described in Section 7.1.2.

The DCE will acknowledge the end of the data by returning the OK result code to the DTE.

If there is data underrun before the next +FDT or +FET= command, the DCE will zero-fill the pad as per T.4 until the Phase C timeout (+FPHCTO) is reached, or until more data is received. The DCE appends an RTC pattern to the transmit data after an +FET= command is received from the DTE.

Phase C Data Format

The Phase C data will be of the format specified by the negotiated T.30 DCS frame. The +FDCS:<string> response is defined in Section 7.4.2. The subparameter values are described in Table 7-2.

The DCE will use the negotiated minimum Scan Time parameter from the DCS frame, and insert sufficient fill bits to pad each line to the minimum scan time. This is reported in the +FDCS:ST subparameter.

If the DCE finds more than one consecutive EOL in Phase C data (e.g., RTC), it will send only one EOL.

1.	Phase C data must conform to T.4 specifications.

2.	The DTE need not include a final RTC, since the DCE will append an RTC in response to an FET= command.

3.	Some facsimile machines may treat two EOLs as an RTC.

<CAN>, Escape from Transmission

The DCE may request the DTE to halt Phase C transmission, by sending a cancel <CAN> character (024) to the DTE. In this case, the DTE should terminate Phase C transmission, issue <CAN>, and wait for the OK response code from the DCE.

Table 7-2.  T.30 Session Subparameter Codes

Label

Function

Value

Description

VR

Vertical Resolution

0

Normal, 98 lpi

1

Fine, 196 lpi

BR

Bit Rate

0

2400 bps V.27 ter

(See Note 1)

1

4800 bps V.27 ter

2

7200 bps V.29 or V.17

3

9600 bps V.29 or V.17

4**

12000 bps V.33 or V.17

5**

14400 bps V.33 or V.17

WD

Page Width

0

1728 pixels in 215 mm

1

2048 pixels in 255 mm

2

2432 pixels in 303 mm

3*

1216 pixels in 151 mm

4*

864 pixels in 107 mm

LN

Page Length

0

A4, 297 mm

*1

B4, 364 mm

*2

unlimited length

DF

Data Compression Format

0

1-D modified Huffman

1

2-D modified Read

2*

2-D uncompressed mode

3*

2-D modified modified Read

EC

Error Correction

0

Disable ECM

(Annex A/T.30)

1*

Enable ECM, 64 bytes/frame

2*

Enable ECM, 256 bytes/frame

BF

Binary File Transfer

0

Disable BFT

1*

Enable BFT

ST

Scan Time/Line

VR = normal

VR = fine

0

0 ms

0 ms

1

5 ms

5 ms

2

10 ms

5 ms

3

10 ms

10 ms

4

20 ms

10 ms

5

20 ms

20 ms

6

40 ms

20 ms

7

40 ms

40 ms

Notes: 	1. CCITT T.30 does not provide for the answering station to specify all speeds exactly using the DIS frame. Implementation of some BR codes (e.g., code 2) by an answering DCE is manufacturer specific.
	* Not supported.
	** Available only if modem supports 12000 and 14400 bps.

7.3.4.	+FET, Transmit Page Punctuation

Syntax:	+FET=<ppm>[,<pc>,<bc>,<fc>]

DCE response:

+FPTS:<ppr> 	- when receive from remote OK

This command is used to punctuate page and document transmission after one or more +FDT commands. This command generates T.30 Post Page Messages selected by the <ppm> code (Table 7-3).

The +FET=<ppm> command indicates that the current page is complete; no more data will be appended to it. The value indicates whether there are any additional pages are to be sent and, if so, whether there is a change in any of the document parameters.

The DTE can command the DCE to generate PRI-Q messages with the +FET=<ppm> command using ppm codes 4-6 (see Table 7-3).

This command must be sent within the time out specified by +FPHCTO after sending Phase C data, or else the DCE will end the page and document transmission. If the Phase C timeout is reached, the DCE sends an EOP post page message and terminates the session.

The remote facsimile station should respond to the post page message with a post page response. The DCE will report this using the +FPTS:<ppr> response (Table 7-4).

End a Page

The +FET= command causes the DCE to append an RTC (6 EOL) pattern as needed and enter Phase D by sending the selected T.30 Post Page message.

The +FET=1 (EOM) command signals the remote station that the next document will have a new DCS negotiated; this causes the session to re-enter Phase B.

Table 7-3.  T.30 Post Page Message Codes

ppm Code

Mnemonic

Description

0

[PPS-]MPS

Another page next, same document

1

[PPS-]EOM

Another document next

2

[PPS-]EOP

No more pages or documents

3

PPS-NULL

Another partial page next

4

[PPS-]PRI-MPS

Another page, procedure interrupt

5

[PPS-]PRI-EOM

Another doc., procedure interrupt

6

[PPS-]PRI-EOP

All done, procedure interrupt

=8+ppm

Post Page Message (ppm code)

Table 7-4.  T.30 Post Page Response Messages

ppr Code

Mnemonic

Description

1

MCF

Page good

2

RTN

Page bad; retrain requested

3

RTP

Page good; retrain requested

4

PIN

Page bad; interrupt requested

5

PIP

Page good; interrupt requested

7.3.5.	+FDR, Begin or Continue Phase C Receive Data

Syntax:	+FDR <CR>

Default value:	3 seconds in some places

The +FDR command initiates transition to Phase C data reception. This can occur after answering, dialing, a document is received, or a page is received.

The DCE reports the negotiated T.30 parameters, with the remote ID information if available. When the DCE is ready to commence data transfer, it issues a CONNECT response code. If the DCE cannot resume data transfer because there is no more data, it responds OK. When the DTE is ready to accept data, it issues an <DC2> character (018) to the DCE.

If the DTE issues an <XOFF> character to the DCE for flow control, the DCE signals the DTE when its buffers are empty by sending a <DLE><DC2> (<016><018>) character pair.

When the DCE delivers the last byte of a page, the DCE reports the Page Transfer Status via the +FPTS:<ppr> response (Table 7-4).

After a Page Transfer Status Report, the DCE reports the post page message from the remote facsimile station via the +FET:<ppm> response (Table 7-3) which signals the intentions of the remote station.

The DCE holds the post page response message to the remote facsimile station (MCF, etc.), represented in the +FPTS parameter until the next +FDR command. The DTE may modify the +FPTS parameter before issuing the +FDR command which releases that message. The DTE must issue a +FDR command to release Post Page Messages.

Initiate Document Reception

The +FDR command may be issued in Phase B after an answer command, or in Phase B after a previous document.

The DCE response in stream mode is:

+FCFR 	 when CFR sent
[+FTSI:<remote ID string>]	 if new TSI received
+FDCS:<T.30 subparameter string>]	 if new DCS

CONNECT
(<DC2> needed from DTE here)
<Phase C data stream>
<DLE><ETX>
+FPTS:<ppr>, <lc> [,<blc>, <cblc>]
+FET:<ppm>
OK
(DTE must issue +FDR command to release post page response)

Continue Document Reception

The DTE may issue a +FDR command in Phase D, which releases the post page message, and indicates readiness to receive another page after receipt of a Multipage (+FET:0) or PPS-NULL (+FET:3) message. The DCE response will be:

CONNECT
(<DC2> needed from DTE here)

<Phase C data stream)
<DLE><ETX>
+FPTS:<ppr>, <lc> [,<blc>, <cblc>]
+FET:<ppm>
OK
(DTE must issue +FDR command to release post page response)

If done receiving:

+FHNG: <hangup cause code>
OK

Continue page reception

Phase C Data Framing

Phase C data may be presented to the DTE in stream mode. The DCE will transfer a stream of data to the DTE, followed by the <DLE><ETX> stream termination characters. The DCE will filter the stream as described in 7.1.2.

Phase C Data Format

The received data format is negotiated under T.30 reported by the +FDCS:VR,BR,WD,LN,DF,EC,BF,ST response.

The DCE will delete the terminating RTC (6 EOLs) patterns.

<CAN>, Escape from Reception

From the +FDR command until the end of Phase D Date, the DCE is in a data transfer state, and will not respond to DTE command characters. The DCE will respond to three ASCII control characters: <DC1> (017) and <DC3> (019) flow control characters, and cancel <CAN> (024).

Upon receipt of the <CAN> character, the DCE will terminate the reporting of received data by sending trailing <DLE><ETX> characters to the DTE, and will then execute an implied +FK command in order to conduct an orderly disconnection.

7.3.6.	+FK, Session Termination

Syntax:	+FK

The +FK command causes the DCE to terminate the session in an orderly manner. In particular, the DCE will send a DCN message at the next opportunity and hang up. At the end of the termination process, the DCE will report the +FHNG response with result code (Table 7-5).

This operation can be invoked by using the cancel <CAN> character during Phase C data reception (see prior section)

The DCE will wait until the current page completes, unless the reception is of unlimited length; in that case, the DCE may halt reception and terminate the session at any time.

Table 7-5. Hang Up Status Codes

Code

Cause Description

0-9

Call Placement and Termination

0

Normal and proper end of connection

1

Ring Detect without successful handshake

2

Call aborted, from +FK or AN

3

No Loop Current

10-19

Transmit Phase A & Miscellaneous Errors

10

Unspecified Phase A error

11

No Answer (T.30 T1 timeout)

20-39

Transmit Phase B Hangup Codes

20

Unspecified Transmit Phase B error

21

Remote cannot receive or send

22

COMREC error in transmit Phase B

23

COMREC invalid command received

24

RSPEC error

25

DCS sent three times without response

26

DIS/DTC received 3 times; DCS not recognized

27

Failure to train at 2400 bps or +FMINSP value

28

RSPREC invalid response received

40-49

Transmit Phase C Hangup Codes

40

Unspecified Transmit Phase C error

43

DTE to DCE data underflow

50-69

Transmit Phase D Hangup Codes

50

Unspecified Transmit Phase D error

51

RSPREC error

52

No response to MPS repeated 3 times

53

Invalid response to MPS

54

No response to EOP repeated 3 times

55

Invalid response to EOP

56

No response to EOM repeated 3 times

57

Invalid response to EOM

58

Unable to continue after PIN or PIP

70-89

Receive Phase B Hangup Codes

70

Unspecified Receive Phase B error

71

RSPREC error

72

COMREC error

73

T.30 T2 timeout, expected page not received

74

T.30 T1 timeout after EOM received

90-99

Receive Phase C Hangup Codes

90

Unspecified Receive Phase C error

91

Missing EOL after 5 seconds

92

Unused code

93

DCE to DTE buffer overflow

94

Bad CRC or frame (ECM or BFT modes)

100-119

Receive Phase D Hangup Codes

100

Unspecified Receive Phase D errors

101

RSPREC invalid response received

102

COMREC invalid response received

103

Unable to continue after PIN or PIP

120-255

Reserved Codes

Note: Some codes are not implemented.

7.4.	SERVICE CLASS 2 DCE RESPONSES

The DCE sends information responses to the DTE as a facsimile session proceeds. They indicate the state of the facsimile session and convey needed information. These messages are solicited messages generated in execution of DTE action commands described in Section 7.3.

The DCE precedes and follows the information responses with <CR><LF>.

The DCE provides the on-line status of several session parameters when they are available during T.30 handshaking. These include the remote ID string and the DIS/DCS parameters. These responses report the T.30 session parameter frames. The subparameters are described in Table 7-2.

7.4.1.	+FCON, Facsimile Connection Response

+FCON indicates connection with a fax machine. It is released by detection of HDLC flags in the first received frame. +FCON is generated in response to an Originate or Answer command.

7.4.2.	+FDCS:, Report Current Session Capabilities

Syntax:	+FDCS:VR,BR,WD,LN,DF,EC,BF,ST

+FDCS:<string> reports the negotiated parameters. Phase C data will be formatted as described by the subparameters. This message may be generated in execution of +FDT or +FDR commands before the CONNECT result code if new DCS frames are generated or received. (See Table 7-2.)

7.4.3.	+FDIS:, Report Remote Station Capabilities

Syntax:	+FDIS:VR,BR,WD,LN,DF,EC,BF,ST

+FDIS:<string> reports remote facsimile station capabilities and intentions. The parameters are provided in ASCII notation. (See Table 7-2.)

This message is generated in execution of Originate, Answer, +FDT, or +FDR commands.

7.4.4.	+FCFR, Indicate Confirmation to Receive

Syntax:	+FCFR

The DCE sends a +FCFR response to the DTE upon reception of an acceptable TCF training burst and a valid DCS signal from the remote machine. This indicates that the DCE will receive Phase C data after the remote station receives the local DCE's CFR message. The +FCFR message is generated in execution of a +FDR command.

7.4.5.	+FTSI:, Report the Transmit Station ID

Syntax:	+FTSI: “<TSI ID string>” Transmit Station ID

This response reports the received transmit station ID string, if any. This message is generated in execution of Originate, Answer, +FDT, or +FDR commands.

7.4.6.	+FCSI:, Report the Called Station ID

Syntax:	+FCSI: “<CSI ID string>” Called Station ID

This response reports the received called station ID string, if any. This message is generated in execution of Originate, Answer, +FDT, or +FDR commands.

7.4.7.	+FPTS:, Receive Page Transfer Status

Syntax:	+FPTS:<ppr>, <lc> [,<blc>, <cblc>]

The +FPTS:<ppr> is generated by the DCE at the end of Phase C data reception in execution of a +FDR command.

The <ppr> is generated by the DCE; it depends on the DCE capabilities at T.4 error checking. See Table 7-4 for <ppr> values.

The receiving DCE will count the lines. These values are;

<lc> = line count

<blc = bad line count

<cblc> = <consecutive bad line count

A receiving DTE may inspect <ppr> and write a modified value into the +FPTS parameter. The DCE will hold the corresponding Post Page Response message until released by a +FDR command from the DTE.

7.4.8.	+FET:, Post Page Message Response

Syntax:	+FET:<ppm>

The +FET:<post page message> response is generated by a receiving DCE after the end of Phase C reception on receipt of the post-page message from the transmitting station. The +FET:<ppm> response is generated in execution of a +FDR command. The <ppm> codes respond to the T.30 post page messages (Table 7-3).

7.4.9.	+FPTS:, Transmit Page Transfer Status

Syntax:	+FPTS:<ppr>

The +FPTS: response reports a <ppr> number representing the copy quality and related post-page message responses received from the remote DCE. The valid <ppr> values are defined in Table 7-4.

The +FPTS:<ppr> response is generated in execution of a +FET=<ppm> command.

7.4.10.	+FHNG:, Call Termination with Status

Syntax:	+FHNG:<hangup status code>

+FHNG indicates that the call has been terminated. The hangup cause is reported and stored in the +FAXERR parameter for later inspection. The <hangup status code> values are described in Table 7-5.

+FHNG:<hsc> is a possible intermediate result code to any DTE action command described in Section 7.3. It is always followed by the OK final result code.

Upon termination of a call, the DCE determines the cause of termination and reports it as part of the FHNG:<hsc> response. It also stores this <hsc> code in the +FAXERR parameter for later inspection.

The hangup values are organized according to the phases of the facsimile transaction as defined by T.30. In the Figure A/T.30 flow charts, there are decision boxes labelled “Command Received?"; this is referred to as COMREC in the table. Similarly, decision boxes labelled `Response Received?' are referred to as RSPREC in the table. A COMREC error or RSPREC error indicates that one of two events occurred: 1) a DCN (disconnect) signal was received, or 2) an FCS error was detected and the incoming signal was still present after 3 seconds. (See Figure A/T.30).

The table values are in decimal notation. Leading zero characters are optional.

7.5.	SERVICE CLASS 2 PARAMETERS

All Service Class 2 parameters can be read, written, and tested for range of legal values by the DCE. The general syntax is described in Section 7.1.

Group 3 FAX devices negotiate session parameters in DIS, DCS, and DTC frames. The following parameters are provided to condition the facsimile DCE for the capabilities it will offer and to report the session settings negotiated.

The three primary T.30 session parameters are +FDCC, +FDIS and +FDCS. They are compound parameters, using values listed in Table 7-2. Figure 7-1 illustrates their relationships.

Figure 7-1.  T.30 Session Parameter Relationships

7.5.1.	+FDCC, DCE Capabilities Parameters

Write syntax:	+FDCC=VR,BR,WD,LN,DF,ED,BF,ST

Valid values:	See Table 7-2

Default values:	0,3,0,2,0,0,0,0 (9600 bps version)

0,5,0,2,0,0,0,0 (14400 bps version)

+FDCC allows the DTE to sense and constrain the capabilities of the facsimile DCE from the choices defined in CCITT T.30 Table 2. When +FDCC is modified by the DTE, the DCE copies +FDCC into +FDIS.

7.5.2.	+FDIS, Current Sessions Capabilities Parameters

Write syntax:	+FDIS=VR,BR,WD,LN,DF,EC,BF,ST

Valid values:	See Table 7-2

Default values:	0,3,0,2,0,0,0,0 (9600 bps version)

0,5,0,2,0,0,0,0 (14400 bps version)

The +FDIS parameter allows the DTE to sense and constrain the capabilities used for the current session. The DCE uses +FDIS to generate DIS or DTC messages directly, and uses +FDIS and received DIS messages to generate DCS messages.

The DCE initializes the +FDIS parameter from the +FDCC parameter on initialization, when +FDCC is written, and at the end of a session.

7.5.3.	+FDCS, Current Session Results Parameters

Read syntax:	+FDCS?

DCE response:	+FDCS=VR,BR,WD,LN,DF,EC,BF,ST

or 	+FDCS=

Valid values:	See Table 7-2.

Default values:	0,0,0,0,0,0,0,0

The +FDCS parameter is loaded with the negotiated T.30 parameters for the current session. A transmitting DCE generates DCS; a receiving DCE gets DCS from the remote station. The DTE may read this parameter.

The +FDCS parameter is initialized 0,0,0,0,0,0,0,0 upon initialization and at the beginning of a session. If the DTE issues a +FDCS? command in the initial state, the DCE reports:

<CR><LF>0,0,0,0,0,0,0,0<CR><LF>

The contents of +FDCS are spontaneously reported during execution of +FDR or +FDT commands by the +FDCS:VR,BR,WD,LN,BF,EC,BF,ST response using the same compound parameter format.

Several commands and responses reference T.30 session negotiated parameters. These are described by a set of common subparameters. These subparameters are described in Table 7-2 with notes. Optional parameter values are marked with an *. Unspecified parameter values are reserved for future use.

The +FDCC, +FDIS, and +FDCS compound parameters use these session parameters (On writes, unspecified subparameters are unchanged.)

The +FDIS:, +FDCS:, and +FDTC: session report responses use these subparameters.

For test response, ranges of values are reported for each subparameter enclosed in parentheses characters. For example, a DCE response to +FDCC=? could report:

<CRLF> (0,1), (0-3), (0-3), (0-2), (0-2), (0-2), (0), (0-7) <CRLF> (9600 bps version)

or

<CRLF> (0,1), (0-5), (0-3), (0-2), (0-2), (0-2), (0), (0-7) <CRLF> (14400 bps version)

Example:

+FDIS=0,3,0,2,0,0,0,1 =

VR = 0	98 dpi vertical resolution, 
BR = 3	9600 bit/s (9600 bps version), 
BR = 5	14400 bit/s (14400 bps version), 
WD = 0	1728 pixels, 
LN = 2	unlimited length
DF = 0	1-D modified Huffman coding, 
EC = 0	no ECM, 
BF = 0	no BFT,
ST = 1	5 ms scan time.

7.5.4.	+FLID=, Local ID String

Write syntax:	+FLID="<local ID string>"

Valid value:	20-character ASCII string

Default value:	Empty

If FLID is not a null string, it generates a TSI or CSI frame. Table 3/T.30 includes digits 0-9, “+” and space.

If the DCE supports use of Table 3/T.30 only, the response to a +FLID=? command is “(20) (32, 43, 48-57).” If the DCE supports printable ASCII <, the response is: “(20) (32-127)<CRLF>.” The first “(20)” represents string length: the second (character values) field reports supported string values.

1.	The string is saved in RAM.

2.	Non-numeric characters are not filtered out.

3.	The string is right justified.

7.5.5.	+FCR, Capability to Receive

Write syntax:	+FCR=<value>

Valid values:	0 or 1

Default value:	0

+FCR=0 indicates that the DCE will not receive message data. This can be used when the DTE has insufficient storage. The DCE can send and can be polled for a file.

+FCR is sampled in CCITT T.30 Phase A and Phase D.

7.5.6.	+FPTS=, Page Transfer Status

Write Syntax:	+FPTS=<ppr>

Valid values:	1, 2, 3, 4, 5

Default value:	0

The +FPTS parameter contains a value representing the post page response, including copy quality and related end-of-page status. These values correspond to post page response messages defined in T.30. The receiving DCE sets this parameter after it receives a page of Phase C data. The transmitting DCE sets this parameter with the status reported by the receiving station. The DTE may inspect or modify this parameter.

The set of <ppr> values is defined in Table 7-4. These values are also reported in the +FPTS response to the +FDR command.

7.5.7.	+FCQ, Copy Quality Checking

Write syntax:	+FCQ=<value>

Valid values:	0

Default value:	0

This parameter controls Copy Quality checking by a receiving facsimile DCE.

The DCE returns +FCQ=0 which indicates the DCE does no quality checking. The DCE will generate Copy Quality OK (MCF) responses to complete pages, and set +FPTS=1.

7.5.8.	+FPHCTO, DTE Phase C Response Time-out

Write syntax:	+FPHCTO=<value>

Valid values:	0 - 255, 100 millisecond units.

Default value:	30

The +FPHCTO command determines how long the DCE will wait for a command after reaching the end of data when transmitting in Phase C. When this time-out is reached, the DCE assumes there are no more pages and no documents to send. It then sends the T.30 EOP response to the remote device.

7.5.9.	+FAXERR, T.30 Session Error Report

Read syntax:	+FAXERR= <table value>, read only

Valid values:	0 - 255, see Table 7-5 for meaning

This read-only parameter indicates the cause of the hangup. Table 7-5 shows the valid values for this parameter as well as the meaning of the each value. +FAXERR is set by the DCE at the conclusion of a fax session. The DCE resets +FAXERR to 0 at the beginning of Phase A off-hook time.

7.5.10.	+FBOR, Data Bit Order

Write syntax:	+FBOR=<value>

Valid values:	0 or 1

Default value:	0

This parameter controls the mapping between PSTN facsimile data and the DTE-DCE link. There are two choices:

Direct: The first bit transferred to each byte on the DTE-DCE link is the first bit transferred on the PSTN data carrier.

Reversed: The last bit transferred of each byte on the DTE-DCE link is the first bit transferred on the PSTN data carrier.

There are two data types to control:

This command controls Phase C data (T.4 encoded data) transferred during execution of +FDT or +FDR commands.

The following two codes are supported:

+FBOR=0	Selects direct bit order for Phase C data.

+FBOR=1	Selects reversed bit order for Phase C data.

Note that this parameter does not affect the bit order of control characters generated by the DCE.

7.5.11.	+FAA, Answer Parameter

Write syntax:	+FAA=<value>

Valid values:	0 or 1

Default value:	0

+FAA=0	Constrains the DCE to answer as set by +FCLASS.

+FAA=1	Indicates that the DCE can answer and automatically determine whether to answer as a Class 2 facsimile DCE or as a data modem. If the DCE automatically switches, it modifies FCLASS appropriately.

Class 2 adaptive answer is implemented as follows:

First, a data mode handshake is attempted. If the DCE has been configured for automode detection (using the ATN1 command), the DCE may try several protocols before terminating attempts to make a data mode connection. This can take as long as 6-8 seconds.

If the data mode connection attempt fails, a facsimile Class 2 connection is assumed. When a connection is made as a result of the adaptive answer, the DCE issues the DATA or FAX result code before the CONNECT or +FCON message to inform the DTE of the connection type. After making a Class 2 connection, the DCE stays on-line rather than going into the command mode as with a Class 1 connection.

7.5.12.	+FBUF?, Buffer Size

Read syntax:	+FBUF?

DCE response syntax:	<bs>, <xoft>, <xont>, <bc>

where:

<bs> = total buffer size

<xoft> = XOFF threshold

<xcont> = XON threshold

<bc> = current buffer byte count

Example:

+FBUF?

512, 506, 500, 0

The +FBUF parameter allows the DTE to determine the characteristics of the DCE's data buffer. Data buffers are used for flow control. Use of the reported values allow the DTE to transfer data without provoking XOFF.

7.6.	EXAMPLE SESSIONS

Tables 7-6 and 7-7 show the typical command and responses for sending and receiving two pages, respectively.

Table 7-6.  Send Two Pages, 1-D, No Errors

DTE Command

DCE Response

Local DTE Action

Remote Station Action

AT+CLASS=2

OK

Set Class 2

x

AT+FLID="<local ID>"

OK

Set local ID

AT<dial string>

+FCON
[+FDIS:"<csi>"]
+FDIS: <dis codes>
OK

Off hook, dial
Send CNG
Detect flags
[Get CSI]
Get DIS

Answer,
Send [CED]
V.21 flags
[CSI]
DIS

AT+FDT





<1st page data>

<DLE><ETX>

+FDCS:<DCS codes>
CONNECT
<XON>



OK

[Send TSI]
Send DCS
Send TCF
Get CFR
Send carrier

Send page data

[Get TSI]
Get DCS
Get TCF
Send CFR
Receive carrier

Receive page data

AT+FET=0

+FPTS:1
OK

Send RTC
Get MPS
Get MCF

Get RTC
Get MPS
Send MCF

AT+FDT

<2nd page data>
<DLE><ETX>

CONNECT
<XON>

OK

Send carrier

Send page data

Receive carrier

Receive page data

AT+FET=2

+FPTS:1
+FHNG:0
OK

Send RTC
Send EOP
Get MCF
Send DCN
Hang up

Get RTC
Get EOP
Send MCF
Get DCN
Hang up

Table 7-7.  Receive Two Pages, 1-D Data, No Errors

DTE Command

DCE Response

Local DTE Action

Remote Station Action

AT+FCR=1

OK

Enable reception

AT+FLID="<local ID>"

OK

Set local ID

RING

Detect ring

Dials[, Send CNG]

ATA

+FCON
[+FTSI:"<tsi>"]
+FDCS:<dis codes>
OK

Off hook
Send CED
Send CSI
Send DIS
Detect flags
[Get TSI]
Get DCS
Begin TCF receive

Get CED
Get CSI
Get DIS
Send V.21 flags
[Send TSI]
Send DCS
Start TCF

AT+FDR



<DC2>

+FCFR
[+FDCS:"<dcs codes>]
CONNECT

<page data stream>
<DLE><ETX>
+FPTS:1, <lc>
+FET:0
OK

Accept TCF
Send CFR

Get page carrier
Get page data

Detect RTC

Get MPS

Finish TCF
Get CFR

Send pager carrier
Send page data

Send  RTC
Drop carrier
Send MPS

AT+FDR

<DC2>

CONNECT

<page data stream>
<DLE><ETX>
+FPTS:1, <lc>
+FET:2
OK

Send MCF
Get page carrier
Get page data

Detect RTC

Get EOP

Get MCF
Send page carrier
Send page data

Send RTC
Drop carrier
Send EOP

AT+FDR

+FHNG:0
OK

Send MCF
Get DCN

Get MCF
Send DCN