The following are the known density codes for various tape formats.  A

specific format can be forced by specifying "-d<code>" on the SCSI2SEQ

driver command line.  The <code> is in hex.

Exabyte density codes

90   Exabyte 8200c/8205 (8200 w/ compression)

8C   Exabyte 8500c/8505 (8500 w/ compression)

15   Exabyte 8500       (5 GB)

14   Exabyte 8200       (2.3 GB)

DAT density codes

93   WangDAT 2600 (proprietary Group Compression)

24   DAT          (DDS-2 standard)

13   DAT          (DDS standard)

Quarter Inch Cartridge (QIC) density codes

22   QIC-2000

16   QIC-2100

15   QIC-1000

12   QIC-1350

11   QIC-525

10   QIC-150

0F   QIC-120

0E   QIC-150 with ECC (Tandberg only)

0D   QIC-120 with ECC (Tandberg only)

05   QIC-11

04   QIC-24

Miscellaneous density codes

92   Everex Excel 1G

0C   HI-TC1 (Patriot and DDC Pertec CS1200)

08   Teac (60 MB cassette)

The following table shows the various tape capacities what will be used for

drives which don't report the capacity of the installed tape.  "Standard"

is used by default.  The "short" capacity is used if the "-s" option is

specified, and the "long" capacity is used if the "-l" option is specified.

These options apply to the SCSI2SEQ and SCSIQIC drivers only.

Cartridge   Short Tape  Standard    Long Tape

Type        Capacity    Capacity    Capacity

=========   ==========  ========    =========

DDS-2          1866       3732         3732

DAT            1244       1866         1866

QIC-525         320        525          525

QIC-150          37        150          250

QIC-120          30        120          200

HI-TC1          240        240          400

QIC-11/24        15         60          100

For DAT cartridges, long is a 90-meter tape.  Standard is 60-meter.

For DDS-2, short is a 90-meter tape.  Standard and long are 120m.

For QIC cartridges, short is a 150 foot tape, standard is a

600-foot tape, and long is a 1,000-foot tape.

Log of changes to the TapeVol package.

Changes in version 3.20 - 08 JUL 1994:

1. AHA1XXXN now recognizes the AHA-154xCF.

2. SCSI2SEQ now proprly recognizes Exabyte 8mm drives which have an

inquiry string that begins with "EXABYTE EXB8".  It previously

recognized only "EXABYTE 8" and "EXAYBTE EXB-8".  The driver will now

accept any drive which begins with EXABYTE and has "8" as the first

digit after that as an 8mm drive.

3. SCSICHGR has been added to the package to support SCSI autochanger

devices.

4. The SHOWCHGR utility has been added to the package.  This shows all

installed auto changers devices, the changer number that the software

will use to address each device as well as each device's physical unit

number, and details about the device.

5. NULLTAPE now accepts the "-m" parameter to specify the simulated tape

capacity, in megabytes.

6. FDCQIC now supports the Colorado Memory Systems FC-10 Jumperless Tape

Controller with the "-j" option.

7. FDCQIC will now automatically detect that some floppy controllers are

capable of 1Mbps transfer rates.  If the "-r" option is not specified,

FDCQIC will default to 1Mbps when detected.

8. FDCQIC now displays the selected transfer rate.

9. FDCQIC will not use 1Mbps transfer rates for QIC-40 tape drives or

QIC-40 tapes in QIC-80 tape drives.  It will use 500Kbps instead.

10. TAPERASE now displays the number of bad sectors while formatting.

11. TAPECHK now supports the /WAIT option to wait for command competion.

12. TAPECOPY now supports the /TAPENAME option to change the name of the

output tape.

Changes in version 3.11 - 01 FEB 1994:

1. AHA1XXX now handles LocalBus cards which do not have a DMA channel.

2. AHA1XXX now detects that the required Adaptec host adapter "autosense"

function has been disabled and gives error 10C to indicate this problem.

3. AHAASPIN now prevents operations to its own SCSI bus ID.  This gets

around problems in some ASPI managers, such as the Always IN-2000

driver.

4. SCSI2SEQ now indicates that the Exabyte EXB-2501 requires formatting

before use.

5. FDCQIC.EXE has been added to the package.  This supports quarter inch

mini-cartridge drives which conform to the QIC-117 specification.  At

this time, this includes Colorado Memory Systems, Conner (Archive),

and Mountain drives which connect to the floppy disk controller.

6. The increased timeouts added in 3.10 have been fixed to work properly

for timeouts greater than one hour.

7. TAPETEST with the /R option now automatically recognizes the format

in which the tape was written and, if a byte count is specified, will

check against the contents of the tape.

8. TAPERASE now supports the /FORMAT option.

Changes in version 3.10 - 05 NOV 1993:

1. Version has been synchronized with POWERsave.

2. AHA1XXXN now recognizes the AHA-154xC.  At this time, the BIOS must be

disabled or AHA1XXXN will not recognize the card.

3. SCSI drivers now have different timing code which allows timeouts of

greater than one hour.

4. SCSI2SEQ now recognizes the DDS-2 density code.

5. drivers now have increased timeouts to better accommodate long searches

in POWERsave restore.

6. TAPECOPY now has a /PROMPT option which will prompt before beginning

the copy.  Commas have been added for better readability and tape header

display has been improved.

Changes in version 1.2.L - 23 JUN 1993:

3. SCSI2SEQ now supports the Exabyte EXB-2501 with minimum firmware level

19.

Changes in version 1.2.K - 10 JUN 1993:

1. All modules have been changed to DOS 5 standard help screen and option

format.  Options are accepted in both -option=value and /option:value

formats, so this change should not cause any upgrade problems.

4. SCSI2SEQ has slightly improved some of the threshold parameters used

for high-end Exabyte drives.

5. SCSI2SEQ now introduces a delay before retrying a command when the

drive is in the process of loading the tape.  This prevents the HP

DAT drive from hanging.

6. SHOWTAPE now displays hardware compression information if the -c option

is specified.

Changes in version 1.2.J - 08 DEC 1992:

1. SCSI2SEQ now defaults to 90 meter tape capacity for DAT drives which

cannot report the true capacity (e.g. Wangtek).  The default capacity

is now 1866 MB.  If the "-s" option is used, the 60 meter tape capacity

of 1244 MB will be used.

2. SCSI2SEQ now handles some additional Exabyte drives.  This does not

imply that these drives are supported, only that the code handles

their peculiarities.

3. SCSI2SEQ now recognizes some additional cartridge types and density

codes used by high-end QIC drives (i.e. QIC-1000 and above).

4. SCSIQIC now obtains error statistics for some Wangtek drives under

more conditions.

Changes in version 1.2.I - 20 NOV 1992:

1. EV8XX, MOUNTAIN, and ARCHIVE have been changed so that status

information may be retrieved during an I/O operation.  This change is

necessary for proper operation with POWERsave 3.0.

Changes in version 1.2.H - 20 OCT 92:

1. WD7000N now handles the -p option correctly.

2. SCSI2SEQ now supports the Tandberg TDC 4200 with firmware 04:35

or later.

3. SCSI2SEQ now determines the soft error count on DAT drives by obtaining

the DDS tape log and reporting the total number of read-after-write

retries plus the total C3 ECC retries for the life of the current tape.

This allows the correct soft error count to be reported on HP drives.

4. TAPECOPY, TAPERASE, and TAPETEST now support the new POWERsave header

block which allows tracking statistical information over the life of

a tape.

5. TAPETEST now displays the last SCSI CDB and Sense data when an error

occurs if the -v option was specified.  This information is useful only

for debugging purposes.

Changes in version 1.2.G - 05 AUG 1992:

1. AHA1XXX now uses the adapter's scatter/gather mode to eliminate the

need for any external buffering.  Because of this change, the old,

original AHA-1540 is no longer supported when base memory has been

filled beyond 640K by a 386 memory manager.  Note that this board was

never in widespread use and should not be confused with the 1540A or

1540B boards.  The AHA-1540 used through-hole technology whereas all

of the newer Adaptec boards use surface mounted parts.

2. VCPI support has been replaced by VDS support for bus mastering host

adapters.  This allows operation in Windows 3.0 enhanced mode.

Because of this change, environments which provided VCPI support but

not VDS support, such as QEMM version 4.23, are no longer supported.

QEMM version 5.10 or later is required to provide VDS support.  Also,

operation with WD7000 may require a 16K DMA buffer to be provided.

This is accomplished with the DMA=16 parameter on the memory manager

command line.  The "-v" option has thus been removed from all drivers

as it is no longer needed.  Note that this does not affect non-bus

master adapters, such as the Adaptec AHA-152x and AHA-1510.

3. SCSI2SEQ now supports the Sankyo CP525 with firmware 5_64 or later.

4. SCSI2SEQ now supports the Tandberg TDC 4100 with firmware 04:08

or later.

5. TAPETEST recognizes drives which do not perform multiple passes over

the tape and does not increment the track count, but instead reports

of any unusual delays in the data stream.

6. TAPETEST now writes two file marks at the end of the test pattern in

order to better simulate NETSAVE activity.  Also, TAPETEST ensures

that blank media exists after the second file mark during the read

pass.

7. SHOWTAPE now always reports the compression status for drives with

hardware data compression.  This allows a drive which is capable of

compression, but currently has it disabled, to be identified.

Changes in version 1.2.F - 20 MAR 1992:

1. Support for control of data compression has been added.  TAPETEST shows

the compression ratio achieved, TAPECHK allows compression to be turned

on or off, and SHOWTAPE tells whether a drive has compression enabled.

Note that this applies only to drives which support hardware data

compression such as the Hewlet Packard 35480A DAT drive.

2. SCSI2SEQ now obtains the soft error and underrun count from an Exabyte

EXB-8500.

3. SCSI2SEQ now supports SCSI-2 drives which report ANSI version 1.

4. SCSI2SEQ now supports the Cipher ST150S with firmware 2.3F or later on

non-intelligent Adaptec host adapters ONLY (e.g. 1510, 1520).  Note that

this drive cannot use short tapes (e.g. DC6037, DC615) and will return

error 63 (end of user area) if such a tape is written to the end.  Also,

rewrite and underrun statistics are not available for this drive.

5. SCSI2SEQ now disables advanced commands for all types of Wangtek drives

so that the "-a" option is no longer necessary for these drives.

6. SCSI2SEQ now supports the Wangtek 5525ES in SCSI-2 mode.

7. EV8XX, ARCHIVE, and MOUNTAIN now allow an application to obtain status

while performing I/O.  This is necessary for versions of POWERsave later

than 1.42/2.22 so that the tape format may be properly determined under

all conditions.

8. NULLTAPE now accepts the "-n" parameter to specify the number of null

drives to be created.  Also, NULLTAPE no longer allows itself to be

loaded multiple times (broken in 1.2.E).

9. TAPETEST no longer occasionally flashes a byte count of all zeroes and

now properly counts tracks while reading (both broken in 1.2.E).

10. SHOWTAPE now shows the tape capacity in bytes for drives with more

than 4 GB.

Changes in version 1.2.E - 15 NOV 1991:

1. All drivers help screens can now be redirected to a file.  In addition,

many of the utilities can now write the dynamic driver initialization

messages to a file.  This allows information such as board revision and

firmware levels to be written to a file.

2. All drivers now recognize an attempt to append to a tape which was

previously completely filled by a backup which spans multiple tapes.

3. WD7000N and AHA1XXXN correct a problem which could randomly cause the

driver to hang during initialization.

4. EXABYTE now recognizes the Contemporary Cybernetics Group CY-8200 drive.

5. SCSI2SEQ now supports the Exabyte EXB-8500 with firmware versions

03F2 or later.

6. SCSI2SEQ now obtains capacity information from WangDAT drives.

7. SCSI2SEQ now supports the WangDAT 2600.  The "-f -d93" options can be

used to force a tape to be reformatted when it is written, even if

formatting is not necessary.  This can be used to reformat a previously

written DDS-format tape in group compression format.  To use this

feature, specify these options and use the TAPERASE command on tapes

which should have group compression format.  Then, remove the driver

options for normal operation.  This is because reformatting takes

approximately one minute and also destroys the cartridge load count.

Note that this option is not needed when a new, previously unwritten

tape is used because a blank tape is always reformatted.

8. SCSI2SEQ now supports the Tandberg TDC 3800 drives in SCSI-2 mode

with firmware versions later than 3:71.

9. SCSI2SEQ now allows the -S and -L options to inform the drive that short

or long tapes are being used.  If, for example, a QIC-150 tape is used

with no options, the capacity will be 150 MB.  With the -S option, a

capacity of 37 MB would be assumed, and with the -L option, a capacity

of 250 MB would be assumed.  While this option is used primarily with

QIC drives, it can also be specified when 90 meter DAT tapes are being

used for drives which do not report the tape capacity.  The following

table shows various tape cartridge types, their capacities with the

short tape option (-S), with no option, and with the long tape option

(-L).  All capacities are in megabytes, or 1,048,576 bytes.  Note

that these are the default capacities.  If a drive is capable of

reporting the exact cartridge capacity, it will override the default.

Cartridge   Short Tape  Standard    Long Tape

Type        Capacity    Capacity    Capacity

=========   ==========  ========    =========

DAT            1244       1244         1866

QIC-525         320        525          525

QIC-150          37        150          250

QIC-120          30        120          200

HI-TC1          240        240          400

QIC-11/24        15         60          100

For DAT cartridges, long is a 90-meter tape.  Standard is 60-meter.

For QIC cartridges, short is a 150 foot tape, standard is a

600-foot tape, and long is a 1,000-foot tape.

10. GIGATAPE now supports the VersaDAT and MultiDAT drives.  The default

tape capacity will be set to 1001 megabytes if a standard drive is used

or 2504 megabytes if a compression drive is detected.  The -m option has

been added to allow the tape capacity to be manually specified.

11. SCSIQIC supports the -S and -L options to allow the tape length to be

specified as for SCSI2SEQ.

12. TAPETEST adds the "-pattern" option to specify the data pattern type

to use.  This may be random, sequential, or a fixed binary value.  This

allows evalutation of data compression systems.  The default is random,

which foils most data compressors.

13. TAPETEST now handles tape capacities larger than 4 GB.

14. TAPERASE now selects the maximum density available for the drive before

erasing the tape.

Changes in version 1.2.D - 15 JAN 1991:

1. AHAASPI no longer prints garbage after the ASPI manager identification.

2. SCSI2SEQ no longer gets error 10B, controller has failed, on host

adapters which do not support auto sense, such as the WD7000.  This was

broken in 1.2.C.

Changes in version 1.2.C - 13 DEC 1990:

1. Drivers no longer display the "already loaded" message.  If the driver

already been loaded, completely silent operation is now produced.

2. Normal initialization messages have been changed to remove English-

language for better acceptibility in overseas markets.  For example,

the "Controller n is..." message has been changed to "ID n:..."

3. Fixed a problem with multiple host adapters such that scanning would skew

an extra ID between each adapter.

4. This package now allows very large tape capacities (more than four GB)

to be reported.  Versions of POWERsave prior to 1.27 will not be able to

report such large capacities.

5. Uncommon flags have been removed from the "tape flags" and moved to a

new set of extended flags.  This should eliminate some confusion by

customers.

6. All tape drivers will now automatically assume -v16384 when loaded if a

386 expanded memory manager has backfilled DOS memory beyond 640K, the

-v option was not explicitly specified, and a host adapter which uses

DMA is involved.  This will automatically eliminate error 16 (data

synchronization error) without having to manually specify the -v option.

7. WD7000N now allows specification of I/O port, interrupt level, and DMA

channel values through command-line options.  The first -p option sets

the I/O port of the first board and sets the number of boards to 1.

The second option specifies the I/O port for the second board, and so on.

Like -p, the -l and -c options set the interrupt level and DMA channel,

respectively, for the boards if they differ from the default values.

8. AHAASPIN has been added to the package.  This allows PT tape drivers to

operate through the Adaptec ASPI Manager software.

9. EXABYTE & SCSI2SEQ now detect the drive being reset after tape

operations have begun and reports error 29 (device has been reset) in

this case.

10. EXABYTE now sets the drive cartridge type as specified by the -p5,

-p6, or -pI options.  If none of these options is specified, the

drive's default switch setting will be used.

11. EXABYTE will now detect drive identification strings which begin with

either "EXABYTE EXB-8200" or "EXABYTE 8200".

12. SCSI2SEQ now performs SCSI-2 Log Sense commands to obtain retry

statistics, total and remaining cartridge capacities, and cartridge

load count.  The "-a" option can be used to disable the use of these

advanced commands if problems with older drives occur.  This may be

necessary with certain versions of Wangtek firmware which cause the

drive to hang when these commands are issued.

13. SCSI2SEQ will ignore errors when attempting to reformat a tape cartridge

if the "-i" option is specified.  Normally, this driver attempts to

reformat any dual-partition tape as a single partition whenever writing

occurs at the beginning of the tape.  This might occur if a tape were

used which was written by software which uses dual partitions.  Some

firmware versions of Sony-H/P drives will not accept the command to

reformat the tape.  If an error occurs when trying to write from the

beginning of a tape, this option can be included and the tape will be

used as-is, but with the capacity reduced by the size of the second

partition.

14. SCSI2SEQ and SCSIQIC can now append to the end of the tape on drives

which do not support reverse spacing.  Also, all drivers now allow

appending to a completely blank tape.

15. SCSIQIC has been fixed to work with very old Wangtek firmware versions

which report an identification string of WANGTEK SCSI-36.

16. SCSIQIC now fully supports the Archive 2525 and Wangtek 5525 tape drives.

17. GIGATAPE now supports the -f option which gets around a problem with

some firmware versions of this drive.  If TAPETEST shows that more data

was read from the tape than was written, try using this option.

18. GIGASTOR has been added to the package to support the Digi-Data

Gigastore series tape drives which use standard VHS cartridges to store

over 5 GB.

19. EV8XX has been fixed to return error 100 instead of hanging when a

read/write operation times out.

20. EV8XX has been improved to support retension without wait for completion.

21. EV8XX now unloads properly when no other PT drivers are loaded.

22. EV8XX now supports the -a option to be better able to support old drives.

This option will disable using advanced commands on the drive.  This

means that tape appends will always be performed by sequential search.

23. EV8XX now supports the -o option which will prevent the ONLINE signal

from being used to terminate commands and rewind the tape.  If the drive

does not rewind properly, then try this option.

24. ARCHIVE has been added to the package to support the Archive SC499/SC402

tape controllers for QIC-36/QIC-02 tape drives.

25. MOUNTAIN has been added to the package to support the Mountain QIC-02

tape drive controller.

26. TAPECOPY has been added to the package.  This allows direct tape to

tape copies to be made.  This utility will not work properly unless

at least one of the drives is connected through SCSI.

27. TAPETEST has various aesthetic enhancements including comma separators

in large numbers for better readability.

28. TAPETEST will no longer hang if aborted while reading from a QIC-36 drive

with certain Everex firmware versions.

29. TAPETEST now displays the tape function and drive number when displaying

a tape error.  The drive number is now displayed while checking the tape

header.

Changes in version 1.2.B - 28 JAN 1990:

1. All drivers no longer display copyright or other verbose messages if

they are loaded dynamically (PTTAPE.CFG).

2. WD7000N and AHA1XXXN now properly erase the "delaying n seconds" message

when the -d option is specified.

3. WD7000N and AHA1XXXN now display an error message if they are loaded and

the "disk" version of the driver is not loaded and SCSIROMs are

installed.

4. WD7000N and AHA1XXXN no longer reset the SCSI bus if they are loaded

from PTTAPE.CFG.

5. AHA1XXX now works properly on AHA-1640 boards when the last hex digit of

the port address is non-zero.

6. AHA1XXXN no longer turns on the disk activity light on micro channel

machines during non-disk I/O.

7. AHA1XXXN now supports the -p option to specify the adapter base port

address.  A -p option should be specified for each adapter to be

supported.  Default is -p330 -p334 -p130 -p134.

8. EXABYTE now supports the remaining tape capacity feature.

9. EXABYTE now sets certain tape motion and reconnection threshold

parameters in the drive for more optimum performance.  Independent

parameters are set for both reading and writing.

10. EXABYTE now returns error 44, target internal hardware error, any time a

media error is reported and the drive also indicates a buffer parity

error.

11. EXABYTE now flushes the drive's buffer after the first write to tape.

This eliminates the effect whereby the drive buffers large amounts of

data and then appears to hang for extended periods while it prepares

to begin writing.  Instead, the initial delay will now occur on the

first write.

12. SCSI2SEQ has been added to the package.  This driver supports sequential-

access devices conforming to the SCSI-2 specification.  The only drives

tested and supported at this time are the Wangtek 6130FS and the

WangDAT model 1300 DAT drives.

13. GIGATAPE has been added to the package.  This driver supports the

GigaTrend GIGA 1230 series SCSI DAT tape drives.

14. EV8XX no longer gives "illegal command" when attempting to append to the

end of the tape on some drives.

15. EV8XX support has been completed for the Archive multi-QIC series drives.

16. EV8XX now supports the -s option to disable the scan for multiple drives

per controller.  This allows operation with Wangtek QIC-02 drives which

do not properly support multiple drive selection.

17. EV8XX now supports the -t option which enables DMA terminal count

interrupt processing.  This feature improves performance with

controllers which allow this.  The only controller known to support

this feature at this time is the EV-817.

18. EV8XX now issues read file mark commands to the controller.  This allows

drives such as the QIC-02 DAT to perform high-speed file mark searches.

On some very old Everex 60 MB controllers, the read file mark command is

not supported.  If the -r option is specified, file mark searching will

be performed by reading blocks, as in previous versions.

19. EV8XX now properly appends on Archive and Tandberg QIC-150 drives.

20. All drivers now support the long block space option.  Also, a new tape

flag, FastSkip, has been added to indicate drives which can skip blocks

at high speed.  These features will be used internally for faster file

restore on drives which support this capability, such as DAT.

21. All tape utilities now allow specification of up to 36 drives, 0 through

9 and A through Z.  The drivers still do not impose any limit on the

number of possible drives.

22. TAPETEST now supports the -nobuffer option which allows testing with

buffered mode disabled.

23. TAPETEST allows forcing the buffer transfer size with the -b option and

the tape block size with the -f option.

24. TAPETEST now displays average bytes per second when the -v option is

specified and the total transfer took at least 30 seconds.  The transfer

time clock is not started until 1,500,000 bytes have been transferred.

25. TAPECHK now displays information for all selected drives and returns

the highest errorlevel of all drives tested.

26. TAPECHK now supports the -l option which sends a load command to the

drive.  Currently, none of the supported drives require this command.

27. TAPERASE now writes only a file mark at the beginning of the tape if the

-secure option is not specified.  This provides a short erase function

for drives which do not support short erase, such as the Exabyte.  Also,

this can be used to prepare a new tape for use with POWERsave's "append"

option.

Changes in version 1.2.A - 18 MAY 1989:

1. AHA1540.EXE has been renamed AHA1XXX.EXE because it now supports the

AHA-154xA and the AHA-1640.  The base I/O ports for AHA-1xxx boards have

been changed to 330 for card 0, 334 for card 1, 130 for card 2, and 134

for card 3.

2. The WD7000 and AHA1XXX drivers have been replaced by WD7000N and

AHA1XXXN, respectively.  These versions of the SCSI board drivers are

much smaller, but do not support disk drives.  They may also be loaded

by the dynamic driver load mechanism.  Since these drivers are now

specific to the this package, they will bear the version level of the

this package instead of the MultiVol package.

3. WD7000N & AHA1XXXN now support the Virtual Control Program Interface

(VCPI) for physical address translation if it is installed.

4. TAPETEST always displays summary statistics if the -l option is given.

5. TAPETEST average bytes/track is now correct for tapes not completely

written.

6. TAPETEST will no longer have strange results if aborted while reading

from a QIC-36 drive with certain Everex firmware versions.

7. SCSIQIC support has been completed for the Archive Viper series drives.

8. SCSIQIC now supports the new Wangtek 5xxx firmware with the new

identification.

9. EXABYTE, PATRIOT, SCSIQIC, and EV8XX can now be installed either as

permanent drivers through CONFIG.SYS or as temporary drivers which are

loaded when necessary and unloaded when no longer required.  To use this

new feature, a text file called PTTAPE.CFG is created in any directory

accessible in the current directory or the PATH specification.  This

file contains one or more records where each record contains the normal

driver command line, just as it would be specified in CONFIG.SYS.  The

drivers specified in this file will be loaded before any tape software

begins and will be unloaded after the tape software terminates.  The

proper version of POWERsave must also be used for this feature to be

available.

10. EXABYTE, PATRIOT, SCSIQIC, and EV8XX now support the tape append

function.

11. EXABYTE, PATRIOT, and SCSIQIC now support the -v<n> option.  This

specifies the maximum block size, in bytes, that can be processed under

VCPI when the requestor's buffer is not physically contiguous.

12. EXABYTE, PATRIOT, and SCSIQIC handle the enhanced error reporting

mechanism of the SCSI drivers.  If using a SCSI board driver from

MultiVol, version 2.1.B or later should be used or "index mark not

found" will be reported instead of the correct error code.

13. EV8XX now uses VCPI for physical address translation if it is installed.

14. TAPERASE has been added to the package.  This utility allows the tapes

in specified drives to be erased.  The default mode is to erase the tape

such that it cannot be read by any tape software.  The -secure option

causes the entire length of the tape to be erased.

Changes in version 1.1.K - 18 OCT 1988:

1. EV8XX no longer gives spurious errors on unloading tape cartridge.

Changes in version 1.1.J - 13 OCT 1988:

1. EV8XX will now accept hardware interrupt level specifications of 2

through 15, excluding 8 and 13.  DMA channel specifications of 0 through

7 excluding 4 are also allowed.

2. EV8XX no longer gives a write protect error when reading a write-

protected tape.

3. EV8XX no longer writes shortened blocks without giving an error.

4. EV8XX no longer returns zero soft errors and underruns after normal tape

reading or writing.

5. EV8XX always selects the correct drive.

6. EV8XX no longer indicates that a tape cartridge is write protected when

it is first inserted.

7. EV8XX now supports retensioning.

8. SCSIQIC now has a longer timeout on drive busy conditions.  Generally, 5

minutes are allowed for any operation.

9. SCSIQIC now supports the Archive Viper drive.

10. SCSIQIC now supports retensioning.

11. PATRIOT now supports retensioning.

12. TAPECHK now has the "retension" option.  All drives selected will be

retensioned if the "-r" option is specified.

13. TAPETEST error totals (with -v) are now correct.

14. TAPETEST now displays the actual number of bytes to be written to each

drive when the -v option is specified.

15. TAPETEST statistics have been improved.

16. SHOWTAPE now shows the tape drive number in hexadecimal, as it is used

by all other tape programs including POWERsave.

Changes in version 1.1.I - 30 AUG 1988:

1. EXABYTE has been changed to accomodate firmware versions 4.20 and later.

2. EXABYTE now supports European P5 type cartridges.  This is accomplished

with the "-p5" option in CONFIG.SYS.

3. EXABYTE has an option to disable SCSI bus parity checking.

4. SCSIQIC has been added to the package to support quarter-inch tape

drives with embedded SCSI controllers.  Currently supported drives are

all of the Wanktek SCSI series (60, 120, and 150 MB) with either Wangtek

or Datapoint firmware.

5. EXABYTE, PATRIOT, and SCSIQIC have been changed so that device unit

numbers are the same as the devices' SCSI bus IDs.

6. EXABYTE, PATRIOT, and SCSIQIC have their "busy wait" code improved.

This allows the drive to be in a "busy" state for up to five minutes

before a device busy error is returned.

7. EV8XX has been changed to correctly handle Wangtek QIC-02 drives.

8. EV8XX no longer occasionally hangs on slow machines.

9. TAPETEST now has the "loop" option and will display long-term statistics

if the "-v" option is specified.

10. TAPETEST now has the "tracks" option.  This limits the maximum number

of tracks to be written.

11. TAPETEST now uses all available base memory for buffer space.  This

provides better error detection on systems which may have a problem with

DMA into certain areas of memory.

12. The TAPECHK utility has been added to the package.  This utility is

intended mainly for use in batch files to determine whether or not a

cartridge is loaded in a drive or to unload the cartridge from a drive.

13. The TAPEDUMP utility has been added to the package.  This can be used to

display blocks of data from a device and is used mainly for diagnostic

purposes.

14. The SHOWTAPE utility has been added to the package.  This shows all

installed backup devices, the drive number that the software will use to

address each device as well as each device's physical drive number, and

each device's current capacity, in bytes.

15. Copyright messages have been made consistent throughout.

Changes in version 1.1.H - 27 APR 1988:

1. TAPETEST no longer waits for the final rewind to complete.

2. TAPETEST no longer occasionally hangs when the output is redirected.

3. TAPETEST now displays average bytes/track and average bytes/second if

the -v option is specified.  Bytes/track is only displayaed if at least

two tracks were transferred, and bytes/second is only displayed if more

than 5,000,000 bytes were transferred.  Also, if the tape is written to

the end, and the -v option is specified, the difference in the expected

and actual capacity is reported.  If the actual capacity is less than

expected, the average number of bytes lost per rewrite is also

displayed.

5. EV8XX has been much improved and now works properly with QIC-02 North

Atlantic drives.

Changes in version 1.1.G - 22 APR 1988:

1. Initial release.