DayStar Digital LT200 PC LocalTalk Card Installation Notes

==========================================================

1. IMPORTANT NOTES

---------------

To use this MLID, DSTARLT.COM is the only driver you need to run

PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The LT200 PC card uses I/O addresses 200H to 34FH.  The card does not

use any DMA channels, interrupts, or shared memory.

The card's I/O address can be set by changing the JB1 jumper block (in

the lower center of the card).  If you choose to install this card, the

INSTALL program will show a picture of the JB1 jumper block.  Use the

arrow keys to make the screen match JB1.  The INSTALL program allows the

following possible values for I/O ports:

340H, 330H, 320H, 310H, 300H, 240H, 230H, 220H, 210H, 200H

If none of these values are acceptable because of conflicts with other

cards it is possible to use any port value from 100H to 3F0H (the last

digit must be a zero).  To use another value, choose the default

settings when running the install program, and then edit the PORT #1

line in the Link Driver DSTARLT section of the NET.CFG file after

completing the installation process.

Note that in all cases, the actual jumper settings on the card and the

parameters specified in the INSTALL program must match or the MLID will

not work.

For more details about configuring and installing this card, please

refer to the "LT200 Connection Interface Board Installation/User's

Manual" that comes with your adapter.