Eagle/Novell NE2000T Ethernet Adapter Installation Notes

========================================================

1. IMPORTANT NOTES

---------------

If you use this adapter card, NE2000.COM is the only driver you need.

The INSTALL program does not provide all the possible Base I/O Address

and Interrupt Line settings which are specified in the manual that

comes with your adapter.  If you need to use a different setting which

is listed in the manual but not shown during the installation

procedure, you should choose any setting and continue with the

installation.  After the installation is completed you may edit the

NET.CFG file to change the Interrupt Line and Base I/O Address values

to suit your need.

2. GENERAL INFORMATION

-------------------

The NE2000T board may contain two jumper blocks to configure the

Interrupt Line and Base I/O Address.  Otherwise the configuration

software that came with the card should be used.

During this installation, the installer will prompt different Base I/O

Addresses and Interrupt Lines for your selection.  In all cases, the

actual settings on the card and the parameters specified in the

PhoneNET PC INSTALL program must match or the MLID will not work.

The Eagle/Novell NE2000T Ethernet Adapter will only work on Twisted

Pair Ethernet cabling (connected via an RJ-45 connector).

For more information about installing and configuring your adapter,

please refer to the manual that comes with the adapter.