# Hardware Notes

Windows NT supports a wide range of hardware — including support for over 2300 x86-based uniprocessor systems, over 35 x86-based multiprocessor systems, as well as support for 15 RISC-based systems (for details, see the Hardware Compatibility List). The following notes can help you with problems you might encounter. For information about specific printer types, see “Printing Notes” later in these Release Notes.

Some items on the Hardware Compatibility List are supported with device drivers available in the Windows NT Driver Library (those marked with “footnote 1”). These drivers are available on the Windows NT compact disc on \DRVLIB\STORAGE.

Note

## General Hardware Notes

For general hardware information, see Chapter 2, “Troubleshooting,” in the Installation Guide, which contains general troubleshooting information about interrupt conflicts, unsupported peripherals, network adapter cards, SCSI drives, and video display adapters.

## Specific Computer Systems

The following list presents some notes on specific computer systems.

Hardware Notes on Specific Computer Systems

| Computer system | Comment |
| --- | --- |
| ACER AcerFrame 3000 MP | Before installing Windows NT, disable shadow RAM Video BIOS. |
| Ambra Video Display | When 16 colors are selected on Ambra computers, the video image has some scattered imperfections painting windows. Switch to 256 colors to get a clean image. |
| AST® EISA Memory | Enable access to memory above 16 MB, even if the computer has only 16 MB of RAM. To enable memory, run the EISA configuration utility supplied with the computer. |
| AST® Premmia | Certain BIOS versions of the Premmia are incompatible with Windows NT. To work with the onboard IDE drive, you must have either BIOS version 1.04, or the beta version of 2.4b. If your BIOS version is in between these two, you must to install BIOS version 1.04. |
| DELL® | If you experience problems updating the Dell Omniplex system to this release of Windows NT 3.5, you may need a BIOS update. A BIOS level of 5xx_a08 is required. This BIOS update can be obtained from Dell's BBS at (512)728-8528. Some Dell models, for example the 486D/50, 433DE EISA, and 25/50 MHz ISA systems, may be unable to recognize more than 16 MB of RAM on Windows NT. Contact DELL Product Support for a BIOS upgrade to version A11 or greater. Before installing this Release Candidate release on a Dell Omniplex, you must run the MONITOR.EXE program supplied by Dell. If you are using in 65,000 color mode, set the refresh rate to 60 Hz. |

Hardware Notes on Specific Computer Systems (continued)

| Computer system | Comment |
| --- | --- |
| DFI 9800T | To install Windows NT on the DFI 9800T system, select Custom Setup, and then press S to skip SCSI detection. If a SCSI host adapter is present on your system, you can add it manually by selecting the appropriate entry. |
| Digital Alpha AXP | Users with firmware version 2.x must upgrade to version 3.0 before installing this Release Candidate release. |
| Gateway 2000 and Micronics Gemini 486 VESA Local Bus Motherboards | Windows NT may stop running because of a timing issue on systems that use the UltraStor U34F Local Bus controller with the Micronics Gemini 486 VESA Local Bus motherboard. Some older Gateway systems include this combination of hardware. Disabling the motherboard’s external cache should allow Windows NT to run. The problem does not occur with ISA bus controllers from UltraStor. This problem does not occur on Gateway 2000 or Micronics VESA Local Bus motherboards with a blue OverDrive socket. If you encounter this problem, you can resolve it by obtaining an upgrade offered by Micronics for the Gemini 486 VESA Local Bus motherboard. The motherboard will need to be returned to Micronics for the upgrade. The Micronics reference number for this is VLBA03. Micronics can be reached at 510-651-2300 for additional information. If you have a Gateway computer, contact Gateway 2000 at 800-846-2301 for upgrade information. |
| Gateway Nomad and TI® TravelMate™ Notebooks | To successfully install Windows NT on these computers, you must choose Custom Setup. If you encounter problems running Setup while scanning for SCSI adapters, you can skip SCSI detection to solve the problem. To properly use the floppy drive with Windows NT, configure the system using its setup utility. Set the Advanced Operating System option to Auto. When power management drivers for these computers are available from Texas Instruments®, using the Auto option will not be necessary. The built-in InPort/QuickPort mouse adapter requires a mouse driver from Texas Instruments to work properly under Windows NT. You can use a serial mouse until this driver is available from Texas Instruments. |

Hardware Notes on Specific Computer Systems (continued)

| Computer system | Comment |
| --- | --- |
| Gateway Nomad and TI® TravelMate™ Notebooks (continued) | To use the internal modem for these computers, you will need a power management driver and software from Texas Instruments. |
| IBM® PS/2® | On some IBM PS/2 systems, only planar memory is recognized when booting from the Windows NT CD-ROM boot disk or Setup Boot Disk of the floppy disk installation set. This is a memory card problem. You may encounter this problem if your system contains a card with the following part numbers: FRU-88F0075 Option 87F9856 or 87F9860 Newer model cards are available that correct this problem. Part numbers for the newer cards are: FRU-87F9916 or 34F2825 Option 34F3077 or 34F3011 Using Kingston expansion memory can also correct this problem. It is recommended that before you install Windows NT, you update your system configuration using the PS/2 Reference Diskette version 1.12 or later. |
| IBM PS/2 models 53, 76, 76i, 77, and 77i. IBM ThinkPad models 700c and 720c | These systems require an ABIOS patch to install and run Windows NT. Copy the following files from the IBM Reference Diskette into the C:\ directory: ABIOS.SYS *.BIO |
| IBM ThinkPad portables | These portables can only use 640x480 resolution using 16 colors. |
| Olivetti® LSX5030 | Disable all ROM shadowing. |
| Toshiba® T4400SX Monochrome | This display is not supported when an external monitor is connected. |
| Toshiba portables | Setup using floppy disks cannot be performed on Toshiba laptop computers for this Release Candidate release. |

Network Adapter Cards

This section lists information on specific network cards. For more information on network cards, including assigning IRQs, I/O port addresses, and memory buffer addresses, see Chapter 2, “Troubleshooting,” in the Installation Guide.

### Installing an ISA Nework Adapter Card On a PCI Bus Computer

If you install an ISA network adapter card on a PCI bus computer and the network is not recognized, you must set the BusNumber parameter for your network adapter card from 1 to 0 for the card to be recognized. The registry path is HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\[NIC driver or NIC driver1]\Parameters. The entry is BusNumber.

Hardware Notes on Specific Network Adapter Cards

| Network card | Comment |
| --- | --- |
| 3Com® EtherLink® II® | If you have an older version of this card (rev. 01-0c or earlier), you might find that it is not reliable on fast, high-end computers. These cards may cause nonmaskable interrupts (NMIs). This is a documented problem with Microsoft LAN Manager 2.1 as well. The best solution is to upgrade your network card. If you have two or more COM ports on your computer, you might find that the EtherLink II card will conflict with one port. Two common symptoms are that the client fails to start and that an error attributed to the EtherLink II card is logged in Event Viewer. To solve conflicts between an EtherLink II card and your COM ports, use the Network option in Control Panel. Double-click the EtherLink II entry in the list of Adapter Cards. In the configuration dialog box, change the interrupt number from 3 to another interrupt, such as 5. Make sure that the interrupt you choose is not being used by another device. For the 3Com EtherLink II/16 TP card on a 486/50 or faster computer, we suggest that you use the shared-memory mode of this adapter with Windows NT. There is a known issue with regard to the 3Com EtherLink II card and COM2. If an interrupt conflict exists on IRQ 3, between the preferred default on the card and COM2, the system will silently disable COM2, because the network adapter card loads first. Make sure there are no conflicts before you run Setup if you want to use COM2. |

Hardware Notes on Specific Network Adapter Cards (continued)

| Network card | Comment |
| --- | --- |
| 3Com Etherlink MC and Older 486 Computers | An intermittent problem can arise when using this network card in computers containing a 486 C step CPU or earlier. When the problem occurs, the computer stops executing all running software, including Windows NT itself. To solve this problem, you can either replace the Etherlink MC card with another type of network card or upgrade your 486 processor to the current stepping. |
| Digital EtherWorks® LC, Turbo, and Turbo/TP | These network adapters might use conflicting memory addresses. For example, addresses might conflict with Adaptec® or Future Domain SCSI adapters. This requires reconfiguring the hardware by changing jumpers. |
| Intel® EtherExpress 16 LAN Adapter | In some cases, selecting Early in the I/O Channel Ready box can stop the network adapter card from functioning correctly and may require reconfiguring with the Intel SoftSet utility. If you are unsure whether your system can take advantage of this network setting, consult your hardware vendor. |
| Ungermann-Bass® EOTP | If problems occur with booting Windows NT when this card is configured on an EISA bus system, run the configuration utility and then remove the configuration information for this card. |
| NE2000 | The best configuration for the NE2000 Network Adapter under Windows NT is at IRQ 5 and I/O Port 340H. |

## Video Display Drivers and Adapters (x86-Based Computers)

Windows NT supports over 45 video display adapters, including excellent 
high-end support. This section presents some detailed notes on video support 
for Windows NT. The information in this section is applicable for x86-based computers only.

This release supports the VGA standard video cards plus some newer video cards. If your video card is not supported with an enhanced driver, VGA will be selected automatically.

Testing the Configuration

If you want to change your display settings, you should always use the Test button in the Display dialog box to view what the video display will look like. Several things might not be working correctly that Windows NT cannot detect. For example, with this Release Candidate version, 65536-color modes are supported for the ET4000, Cirrus, and S3 drivers. However, not all cards with these chip sets are capable of displaying in 65536-color mode. Some S3 cards, for example, actually support 32768-color modes, not 65536 colors. Adapters that support 32768 colors will show two or more shades or red, green, and blue. If this occurs, do not select this mode.

Caution

If you install a new video card before using the Display option in Control Panel for the new configuration, your computer may not restart correctly. In such a case, you can either install the original card (if it is available) and revise the configuration using the Display option, or select the VGA boot option on the Boot Loader startup screen. If you use the latter method, you must manually disable the video driver that was running.

Updating to Windows NT 3.5 Video Drivers

It is strongly recommended that you use the latest video drivers for Windows NT 3.5. You can continue to use video drivers from Windows NT 3.1, but you will be missing the benefits of performance enhancements and the new functionality provided by the Display option in Control Panel. Also, numerous bug fixes have been made to video drivers.

If you are using a video driver that was not provided by Microsoft, you should be able to continue to use that driver. We have not tested many of these drivers with Windows NT 3.5. If you have problems, please contact Microsoft as described in the Release Candidate release cover letter, and also contact the provider of the driver.

### Troubleshooting Video Display Configuration

If you encounter problems, read the manual for your video display adapter again for proper settings. You may have overlooked something in the manual that pertains to Windows NT if you previously installed the card under Windows 3.1.

In general, Windows NT video drivers do not use interrupts. Many VGA cards use jumpers to enable IRQ 2 interrupt for EGA compatibility. When possible, you should disable the IRQ 2 interrupt.

During Setup, Windows NT may allow you to set the vertical refresh rate for your card. Read your monitor manual to see what settings you can use. If you are unsure, set up at the lower resolution, and then later try higher resolutions.

Caution

Some monitors may be physically damaged by wrong video settings. Check your monitor’s manual carefully before choosing a new setting.

If after changing resolution settings and rebooting, you find that your display is not operating correctly, reboot again before logging on and use the Last Known Good feature to restore your previous video settings.

Minimum video memory requirements for Windows NT depend on the specific adapter card.

The following list presents information about specific video display types. See also “ATI Driver Notes” later in these Release Notes.

Hardware Notes on Specific Video Displays

| Display type | Comment |  |  |
| --- | --- | --- | --- |
| 8514/A and Compatible Cards | When installing Windows NT on a computer that has an 8514/A or compatible graphic card (for example, IBM 8514/A or ATI 8514Ultra), and when the computer has an older version of BIOS, Custom Setup may display your video card as “Unknown.” You can change the video driver from “Unknown” to the Standard VGA (640x480, 16 colors) driver during Windows NT Setup. If you have an ATI 8514 Ultra graphic card, after Windows NT Setup is completed, you can use the Display option in Control Panel to select the appropriate ATI video driver. Express Setup will default the video driver to the Standard VGA (640x480, 16 colors) when an unknown video card is detected. |  |  |
| Cirrus and ET4000 | Only some systems support 64K VGA, and 64K VGA supports only 640x480 resolution. For 800x600 resolution, 1 MB of video memory is required at 8 bpp. |  |  |
| Metheus | For this Release Candidate release, the refresh frequency option is not supported for this video adapter. |  |  |
| S3 VGA | The vertical refresh rate for many S3-based video is set by DIP switches on the card or by running an MS-DOS–based utility that programs the adapter’s ROM. For these systems, select the default setting in Windows NT Setup. The MS-DOS–based setup program for these cards must be run under MS-DOS. If you have a Dell computer, Number Nine card, or Metheus, you may use the other settings when listed. If you experience difficulties using the S3-based video card on your Hewlett-Packard® Vectra® computer, call Hewlett-Packard technical support to upgrade your BIOS EEPROMS. The Windows NT S3 driver supports only 256, 32768, 65536, and true-color resolutions, but not 24 bpp. The modes that are accelerated depend on the S3 chip, the DAC type, and the amount of memory on the adapter. See your S3 manual for details. |  |  |

Hardware Notes on Specific Video Displays (continued)

| Display type | Comment |
| --- | --- |
| VESA VGA | The VESA VGA driver uses the newer standard for detecting whether a card can handle the 800x600 Super VGA mode. If you have a card that uses the old standard or a card that requires an MS-DOS – based, memory-resident program to implement Super VGA functionality, only standard VGA mode is supported. |
| VGA | If you experience a problem with the standard VGA driver, try the alternative VGA driver in the \SYSTEM32\DRIVERS directory that programs the VGA directly. The standard VGA driver uses the VGA display adapter’s BIOS to initialize the card. To use the alternate driver, rename the standard driver VGA.SYS to VGA.SAV. Then copy VGA_ALT.SYS to VGA.SYS and restart your computer. |

### ATI Driver Notes

The ATI drivers support the following boards:

8514/Ultra
Graphics Ultra
Graphics Ultra Pro
Graphics UltraPlus
Graphics Vantage
Graphics Wonder

You should run the ATI-supplied Install program before setting up Windows NT. You need to do this only once. When you run this program, select the monitor that your computer is using. The Install program sets the vertical refresh rate that should work with your selected monitor at every resolution. Some versions of the Install program did not do this correctly for all monitors. In such cases, you can select the Custom monitor option, and then set the refresh rate for every resolution. This appears to work correctly in every version of Install that has been tried at Microsoft.

All Graphics Wonder boards with 1MB of graphics memory and Graphic Ultra Pro and Graphics Ultra Plus cards with a -006 Mach32 chip will support 800x600 
16 bpp mode.

The Graphics Ultra Pro, Graphics UltraPlus, and Graphics Wonder on an ISA bus computer with the aperture disabled support the following resolutions:

640x480, 800x600, 1024x768, 1280x1024	8 bpp 	[256 colors]
640x480, 800x600, 1024x768	16 bpp 	[65,536 colors]

The supported resolutions and pixel depths assume enough memory on the card. All cards require a minimum of 1 MB of graphics memory. The pixel depth depends on the DAC type on the display adapter. If you choose a resolution that is not supported or that your card has not been installed for, the driver substitutes a supported resolution.

The Graphics Ultra Pro, Graphics Ultra Plus, and the Graphics Wonder with aperture enabled will support

640x480     24 bpp
800x600     24 bpp (requires 2MB RAM and ATI originally specified DAC

AST Premmia users should select ATI AST for all AST supplied ATI video adapters.

If your ATI card does not initialize properly for the selected mode, restart your computer using MS-DOS and run the ATI card Install program to set the card’s parameters for a monitor that can handle high-resolution graphics.

### Weitek P9000-based Video Cards

Video cards based ont he Weitek P9000 chip will not automatically be detected during setup. After Windows NT has been installed, use the Display option in Control Panel to select the Weitek P9000 Graphics Adapter.

### Switching Between a Window and Full Screen

When running MS-DOS – based graphics applications using 256 colors, you may encounter a problem switching between a window and the full screen with the following video boards:

ATI 8514 Ultra
ATI Graphics Ultra Plus
COMPAQ QVision
IBM XGA2
STB Powergraph X-24
STB Powergraph VL-24
STB Wind/X HC
Trident 8900c
Trident 9000a
Video Seven VRAM(1)

When switching back to the full screen, the display can become scrambled. Although the application is still running, the graphics screen is incorrect. At this point, close the application. If this is not possible, switch back to Windows NT and use the Task List to stop the application.

The IBM XGA2 also exhibits this problem with 16-color, MS-DOS – based graphic applications. Both problems only occur in 640x480 mode on the XGA2 system.

## Disk Drive Information

The following presents issues for hard disks and floppy disk drives.

### Unrecognized Disks

In a few cases, Windows NT may not recognize a hard or floppy disk even though MS-DOS recognizes the disk. Windows NT requires strict conformance to proper partition table formats, boot blocks, and so on. This enhances protection against viruses. If you suspect that a disk’s partition table is incorrect, take the following steps:

1.	Back up any data you need, including the files required to run MS-DOS.

2.	For a hard disk, use a recent version of MS-DOS FDISK to delete and then recreate all partitions.

3.	Format the hard or floppy disk.

### Volume Sets Created via the BIOS

On controllers other than the UltraStor 124f, Windows NT does not support disk controllers that create volume sets via the BIOS (that is, two different drives merged into a single drive letter).

### Caching Disk Controllers and Drives

Windows NT supports caching disk controllers and drives only if one of the following is true:

u	The controller has a driver that specifically supports caching (several of these exist, for example, DPT and IBM SCSI Caching Adapter).

u	Write-through is active (that is, write caching is disabled) either by setting it manually or by default when the computer starts.

u	The entire cache option is off, either by setting it manually or by default when the computer starts.

Using a caching controller or drive in other situations risks loss of data if the computer is rebooted or a power failure occurs before the cache has flushed itself. You can minimize risk by waiting at least one minute after all system activity stops before you reboot. (You can wait a shorter time if you know the maximum time the cache waits before flushing itself.) The disk activity light is a good indicator of how write caching is being done on your system. This applies to the initial Windows NT installation as well as shutdown.

Notice that unexpected power failure at any time can potentially corrupt data on the disk, possibly to the extent that the disk becomes unreadable. To reduce this risk, make the smallest possible time for the delay time that Write data is cached before it is flushed to disk.

### Installing Windows NT on an ESDI Disk Drive with More Than 1024 Cylinders

In some cases, Windows NT installation proceeds normally until the first boot from the hard drive where Windows NT is installed. The Windows NT Boot loader will load various files and then produce a Fatal System Error: 0x0000006b with the message that Phase 1 Process Initialization failed. Following this message will be some type of hexadecimal dump, and the system will be locked up. If you experience this difficulty, the following information can help.

Testing has been done on the following ESDI controllers using a hard drive with a capacity exceeding 516 MB (MB=1,048,576 bytes) formatted. The MS-DOS limit of 1024 cylinders creates a situation where special BIOS mapping on the controller is used to change the geometry of the drive. ESDI drives are capable of being prepared with various values of sectors per track (spt) such as 53 or 63 spt geometry during a low-level format. For example, this is for a drive capacity using a different spt format.

1024 cylinders x 15 heads x 53 spt x 512 bytes per sector = 
	398 MB formatted capacity

– Or –

1024 cylinders x 15 heads x 63 spt x 512 bytes per sector =
	472 MB formatted capacity

Thus, using 63 spt will yield 74 MB more space. Windows NT is perfectly compatible with either geometry and, depending on the drive or controller, can access the remaining cylinders beyond 1024. This space can be partitioned and formatted but not accessed by MS-DOS as the operating system. However, some controllers can successfully remap the remaining cylinders beyond 1024 so that either MS-DOS or Windows NT can use the entire capacity. For example:

1632 cylinders x 15 heads x 53 spt x 512 bytes per sector = 
	634 MB formatted capacity

Testing has been done on the following ESDI controllers with their Windows NT compatibility results.

Notes on ESDI Controllers

| Controller | Comment |
| --- | --- |
| Data Technology Corp (DTC) Model 6282-24 | Maximum Windows NT-compatible geometry is to use 63 spt and a limit of 1024 cylinders under MS-DOS. Windows NT will be able to access the cylinders beyond 1024. Do not perform a low-level format on the drive using Head Mapping mode. |
| DTC Model 6290-24 | Maximum Windows NT-compatible geometry is to use 63 spt and a limit of 1024 cylinders under MS-DOS. Since this card does not have an onboard BIOS, Windows NT cannot access the remaining cylinders beyond 1024. |
| DTC Model 6290 SEZ (Dual SCSI/ESDI Controller) | Maximum Windows NT-compatible geometry is to use BIOS translation in the EISA configuration, using 63 spt and a limit of 1024 cylinders under MS-DOS. Windows NT can access the remaining cylinders beyond 1024. |
| DTC Model 6295-24 | Maximum Windows NT-compatible geometry is to use 63 spt and a limit of 1024 cylinders under MS-DOS. This model does have an onboard BIOS, so Windows NT can access the remaining cylinders beyond 1024. Do not perform a low-level format on the drive using Head Mapping mode. |

Notes on ESDI Controllers (continued)

| Controller | Comment |
| --- | --- |
| Adaptec Model 2322D | The methods that can achieve maximum drive capacity are as follows: 1. Disable drive translation and the on-board controller BIOS. Then use a user-defined drive type with the actual drive parameters. For example: Drive Type in CMOS=48 Cylinders=1632 Heads=15 SPT=53 This will give 634 MB capacity. MS-DOS will still be limited to the first 1024 cylinders, which makes 398 MB available. Windows NT will be able to access the cylinders beyond 1024, yielding another 236 MB. 2. This option allows both MS-DOS and Windows NT to access the entire drive. Set jumpers on the controller for Drive Splitting. Disable drive translation and the on-board controller BIOS. Set up the first physical drive in the CMOS as: Cylinders=1024 Heads=15 SPT=53 which will give a drive capacity of 398 MB. Set up the second drive (it appears as a physical drive) in the CMOS as: Cylinders=606 Heads=15 SPT=53 This will yield another 236 MB. Windows NT will actually see the drive as two physical drives. |
| UltraStor with PROM versions less than #####-009 | Maximum Windows NT compatible geometry is to use 63 spt and a limit of 1024 cylinders under MS-DOS. Windows NT will be able to access the cylinders beyond 1024. |
| UltraStor with PROM versions equal to or greater than #####-009 | Maximum Windows NT compatible geometry is to use Track Mapping during low-level formatting. Both MS-DOS and Windows NT will have access to entire drive capacity. |

SCSI Devices

The following presents issues for SCSI devices.

### Adaptec BIOS Detection

This release of Release Candidate includes new BIOS detection code from Adaptec. The detection code helps identify BIOS conflicts in the system with other network or video adapters. If BIOS conflicts are detected, you may encounter a number of problems that might be interpreted as a driver problem. If Windows NT detects a BIOS conflict, the system will write information in the event log to describe the conflict and identify the adapters in conflict. Please consult the vendor’s manual to adjust the location of the BIOS to eliminate the conflict.

### Installing Windows NT with a Proprietary Adapter Driver

Some SCSI and proprietary CD-ROM adapter drivers are provided in the Windows Driver Library on the Windows NT compact disc. These drivers are contained in a directory tree below the \DRVLIB directory. Some of the drivers will have a README file containing additional configuration information.

Please read this information before installing.

Û	To install Windows NT using a proprietary adapter driver

1.	Using your current operating system, access the Windows NT CD-ROM, and change to the directory containing the driver that you want to use.

2.	Copy all files in that directory to a blank, formatted floppy disk placed in drive A (Windows NT Setup requires the driver to be on a disk in drive A). Label the disk “Driver Disk.”

3.	Shut down your current operating system (if necessary), and reboot with the Windows NT CD-ROM Setup Boot Disk in drive A.

4.	When Setup asks for Custom or Express Setup, choose Custom.

5.	When choosing an adapter, type s to select from additional SCSI adapters.

6.	Choose the Other (requires disk provided by hardware manufacturer) option.

7.	Setup prompts for the driver disk. Insert “Driver Disk” in drive A, and then choose OK.

Replacing SCSI Adapters

If you want to replace a SCSI adapter in your hardware configuration with a new adapter, you must enable the new adapter miniport driver and disable the old adapter.

Û	To enable a new boot device adapter

1.	Using the Devices option in Control Panel, set the new adapter Startup value to Boot.

2.	Change NTBOOTDD.SYS, the loader environment miniport driver.

This is a read-only, hidden system file called NTBOOTDD.SYS in the root of the drive containing NTLDR. Usually this is the C drive, but it may not be if you use sticky drive letters and changed it to some other drive letter.

Use the following commands at the command prompt to copy the new miniport driver over the top of the existing drive:

attrib -r -s -h c:\ntbootdd.sys
copy %systemroot%\system32\drivers\buslogic.sys c:\ntbootdd.sys
attrib +r +s +h c:\ntbootdd.sys

The final attrib statement protects users from removing the file by mistake.

3.	Restart the computer.

Notes on Specific SCSI Devices

The following presents issues for specific SCSI devices.

| SCSI drive | Comment |
| --- | --- |
| Adaptec | An Adaptec adapter might use memory addresses that conflict with other cards such as network adapters. This requires reconfiguring the hardware by changing jumpers. |
| Adaptec AHA-1542C and Denon DRD-253 | This adapter and drive are incompatible together under Windows NT. The AHA-1542C requires active termination and the Denon DRD-253 CD-ROM has built-in passive termination. |
| Adaptec 1542x | There is a known problem using Micronics VL bus motherboards with these adapters. |
| Adaptec 154x | DMA speed cannot be set on an Adaptec 154x SCSI controller. Currently the AHA154X.SYS driver supports a DMA transfer rate of 5.0 MB. The Adaptec AHA154xC card is extremely sensitive to termination and cabling. Systems with this card should use SCSI-II cables and/or SCSI-II active terminators. SCSI-II cables are available from Amphenol, Quintec, and Icontec. If these recommendations are not followed, unreliable operation, including data corruption, is possible. The 1988 version 3.08 Adaptec 154x BIOS has a problem with the Scatter/Gather feature. This problem is detected by the Adaptec driver, and the feature is disabled. If you have this BIOS version, a message informs you that the Scatter/Gather feature has been disabled. If you get this message, you can contact the Adaptec Technical Support at 408-945-2550 for information on the most recent version, 3.20, of the BIOS. BIOS versions after 3.08 do not have this problem. The latest 154x BIOS also supports drives that are larger than 1 GB under both BIOS/MS-DOS and Windows NT. Windows NT will be able to access drives larger than 1 GB even without this upgrade, provided an extended partition is used. |

| SCSI drive | Comment (continued) |
| --- | --- |
| Adaptec 1640 Micro Channel® adapter | This adapter does not support the Maynard 2000 or 1300 DAT drives in this release. The 1640 adapter is not supported on IBM PS/2 Model 70 computers. |
| Adaptec AHA164x | The AHA164x driver logs unnecessary errors. These occur while NTBackup is used with a tape connected to the AHA164x. The entries look like this: 7/10/93 2:41:40 AM AHA154x Error None 11 N/A [ComputerName] The driver detected a controller error on \Device\ScsiPort1. These entries can be ignored. Also, when too many of these entries are generated, a popup message indicates the error log is full. This can be avoided by setting the System event log settings to overwrite the events as needed. |
| Adaptec 1740 | If this card is configured for Standard mode, Windows NT detects the card as an AHA-154x. If you change the card to Enhanced mode, you must use Windows NT Setup to remove the AHA-154x driver and install the AHA-1740 driver. |
| Buslogic | When using a Buslogic 747 SCSI adapter with an AST Manhattan SMP computer, you must disable the BIOS on the adapter. For more information on disabling the BIOS, refer to the adapter’s documentation. |
| COMPAQ® Smart SCSI Array | This adapter must be configured to use any possible interrupt other than IRQ 2 to function properly under Windows NT. |
| Dell Drive Array | For this Release Candidate release, this controller is not supported with the supplied drivers. Please check the BETAWINNT forum on CompuServ for the driver for this adapter. Once you have installed the updated driver, the Adaptec emulation mode on the Dell Drive Array (DDA) must be disabled for compatibility with the hardware’s Windows NT driver. |

| SCSI drive | Comment (continued) |
| --- | --- |
| Future Domain | This adapter might use conflicting memory addresses. This requires reconfiguring the hardware by changing jumpers. The Windows NT driver that supports the Future Domain 845, 850, 860, and 885 adapters assumes operation on IRQ 5. If the adapter is set for another interrupt, the Future Domain driver will continue to work, but in a polling fashion that may slow down operation of the system. Further, the Future Domain driver will register to the system claiming the use of IRQ 5, and this may disrupt operation of another device that is actually configured for IRQ 5. If you are using one of the above mentioned Future Domain adapters, please complete the Windows NT installation normally, then change the IRQ information for the driver in the Registry. If the IRQ is incorrect, the driver will log an event that you can view using Event Viewer. If you have a 16-bit Future Domain card or an 8-bit M series card configured with an external SCSI device, make sure that SCSI termination is correct. You must set a jumper on the card for this setting. Check the card’s documentation for details. |
| IBM PS/2 SCSI Adapter | To configure an IBM SCSI adapter with a BIOS dated before 1991 for use with Windows NT, use a PS/2 Reference Diskette version 1.21 or later. The IBM PS/2 SCSI Adapter does not support Logical Unit Numbers (LUNs). The adapter views them as IDs instead of identifying them as LUNs. This may cause SCSI ID conflicts with other devices if more than one SCSI devices is connected to this adapter. |
| Maynard SCSI Adapter | You cannot use Express Setup to install Windows NT onto a hard disk attached to a WD33C9C SCSI host adapter (Maynard SCSI adapter), because Setup cannot identify this adapter. However, you can use Custom Setup to install this adapter card, or after Setup you can use Windows NT Setup in the Main Program group to install the adapter card. |
| Mylex DCE376 and Tangent 48933 USA | The Tangent and Zeos 433 EISA system comes standard with a Mylex 376 controller, which operates in WD1003 emulation mode by default. This will no longer be necessary when a SCSI driver for this card becomes available for Windows NT. |

| SCSI drive | Comment (continued) |
| --- | --- |
| NEC Intersect CD-ROM | Readers for this device have a switch that enables disconnects during accesses to the reader. This switch is off by default, disabling disconnects. Because of this, while your CD-ROM is being read, no other devices on that SCSI bus can be accessed. Since reads from a reader can take a significant amount of time, having this switch off can degrade system performance noticeably. This slow-down may occur even if you are not using the CD-ROM reader. To avoid this problem, set switch number 5 to On. The switch is located with switches labeled SW1. |
| Plextor (Texel) DM-5024 CD-ROM | A firmware level of 1.10C is required to make the DM-5024 CD-ROM compatible with Windows NT. Contact Plextor at 800-886-3935 for information on this upgrade. |
| ProAudio Spectrum 16 TMU1 | This adapter defaults to IRQ 15 and may conflict with other adapters. If are not using this CD-ROM drive but you experience problems during Windows NT Setup, you must remove the adapter and restart Windows NT Setup. After Windows NT is installed, reinstall the adapter. |
| Trantor | The Windows NT driver that supports the Trantor T-128 and T-130B adapters assumes operation on IRQ 5. If the adapter is set for another interrupt, the Trantor driver will continue to work, but in a polling fashion that may slow down operation of the system. Also, the Trantor driver will register to the system claiming the use of IRQ 5, and this may disrupt operation of another device actually configured for IRQ 5. If you are using one of these Trantor adapters, complete the Windows NT installation normally, and then change the IRQ information for the driver in the Registry. If the IRQ is incorrect, the driver will log an event that you can view using Event Viewer. In addition, these adapters are SCSI-termination sensitive. If Windows NT hangs upon booting, or if Windows NT Setup cannot find devices attached to a Trantor adapter, verify that the SCSI termination occurred and that one of the SCSI devices attached to the Trantor adapter is providing termination power. |

UltraStor SCSI Controllers

If you experience a problem in tape functionality with a supported 4mm DAT Tape Device and an UltraStor 14F or 24F controller, contact UltraStor for an upgrade to resolve this.

If you have an UltraStor 34F controller installed in a Micronics VLB motherboard, it is recommended that you turn the motherboard external cache off.

The UltraStor 124 adapter is compatible with removable media drives but does not allow the user to remove and replace media. For this reason, Windows NT does not support removable media drives when used with this controller.

The combination of the UltraStor 14F and DECTalk™ speech synthesizer card can cause Windows NT Setup to fail because of an I/O port conflict. To avoid this problem, either set the Digital I/O to 350, or delete the file ULTRA14F.SYS from the boot floppy while leaving the DECTalk card at default settings.

## Tape Devices

Windows NT supports over 50 tape drives.

### DEC DLT2000 Tape Drives

For this Release Candidate release, users of the DEC DLT2000, DEC TZ86 and DEC TZ87 tape drives should start the NTBackup program with the /missingtape option to prevent the tape drives from misreading the tape catalog.

### QIC 40/80 Tape Devices

QIC 40/80 tape devices should be attached to your computer’s native floppy controller. Proprietary floppy tape controller cards are not supported. Also, hardware data compression is not supported on QIC 40/80 floppy tape drives.

### MiniQIC Tape drives (Exabyte 2501)

For this Release Candidate release, users of this tape drive should format all tapes that will be used in a multi-tape backup before starting the backup operation.

Non-SCSI CD-ROM Drives

Windows NT supports over 7 non-SCSI CD-ROM drives.

### Creative Labs CD-ROM Drives

Please see the section on Panasonic CD-ROM Drives.

### Mitsumi CD-ROM Drives

Windows NT supports the Mitsumi CRMC-FX001D.

To use this drive, please select the Mitsumi CD-ROM entry under Add SCSI Adapter in Setup. This drive will not be auto-detected during the installation of Windows NT. You must select the driver manually.

The Mitsumi CD-ROM drive is supported on the Creative Labs Sound Blaster 16 MultiCD and the Mitsumi 74-1881A 16-bit interface card.

If you already have the board should consult your user-manual to ensure you have the correct interface.

Or customers may match them to the board we tested in the following way: (Please orient the card so that you are able to look at the chips, with the metal bracket to your right, and the gold fingers of the edge connector towards the bottom.) Along the bottom, just above the gold fingers, "74-1881A" will appear in white lettering.

Audio is not supported on the Mitsumi CD-ROM drive with this release of Windows NT.

### Panasonic CD-ROM Drives

Windows NT supports the Panasonic CR-521, CR-523, CR-562, and CR-563.

These drives may be labeled on the top or bottom of the drive by Panasonic, Creative Technology (Creative Labs), or Matsushita-Kotobuki.

Some of these drives may be labeled on the front bezel with the SoundBlaster logo.

To use these drives, please select the Panasonic CD-ROM entry under Add SCSI Adapter in Setup. These drives will not be auto-detected during the installation of Windows NT. You must select the driver manually.

The Panasonic CD-ROM drives are supported on the Creative Labs Sound Blaster Pro, Sound Blaster 16, Sound Blaster 16 MultiCD, and the Panasonic (Matsushita-Kotobuki) LMEP0084B 8-bit interface card.

If you already have the board should consult your user-manual to ensure you have the correct interface.

Or customers may match them to the board we tested in the following way: (Please orient the card so that you are able to look at the chips, with the metal bracket to your right, and the gold fingers of the edge connector towards the bottom.) Turn the board over so that you are able to see the solder side of the board.  A white label should be attached to the board with the model number and the FCC ID: IU09TB016CRI.

This driver can take a considerable length of time to initialize the hardware during boot, and your system may appear to hang. This initialization period can take several minutes. An updated driver will be available through the Windows NT Driver Library when this problem is resolved. Please see the Windows NT Hardware Compatibility List for more information on the Windows NT Driver Library.

### Sony proprietary CD-ROM Drives

Windows NT supports the Sony CDU31A and the CDU33A.

To use these drives, please select the Sony CD-ROM entry under Add SCSI Adapter in Setup.

The Sony CD-ROM drives are supported on the Creative Labs Sound Blaster 16 MultiCD and the Sony CDB-334 8-bit interface card.  The Sony CDB-334 is sold contained in a Sony CD-ROM interface kit. Sony has two kits, one for external drives, and one for internal drives. The Sony part numbers for these kits are 	OPA-474E (external) and OPA-474I (internal).

If you already have the board should consult your user-manual to ensure you have the correct interface.

Or customers may match them to the boards we tested in the following 3 ways; (Please orient the card so that you are able to look at the chips, with the metal bracket to your right, and the gold fingers of the edge connector towards the bottom.) 1)  Along the bottom, just above the gold fingers, "FCC ID: AK8CDB334" will appear in white lettering. 2)  Just above and to the right of the lowest chip in the lower left corner, from top to bottom "DEC-15 V0" will appear in white lettering. 3)  In the upper right corner of the board, next to the audio connector, "SE-S0-9202-3" will appear in white lettering.

## Removable Media

Windows NT supports over 10 SCSI removable media systems.

Removable media drives (such as Bernoulli and Syquest®) are supported for use with the NTFS and FAT file systems only.

If you install Windows NT or any portion of Windows NT onto removable media, be sure to shut down Windows NT before removing the media. (That is, choose Shutdown from the File menu of Program Manager.) Do not remove the media until after Windows NT has completed its shutdown process.

### Installing from CD-ROM with Floptical Drives Present

Users with configurations that have a 21MB floptical drive configured as drive A: will not be able to perform a floppy setup of Windows NT 3.5 without modifying the TXTSETUP.SIF file.

To install from CD-ROM in a configuration that has a 21MB floptical drive as drive A:, edit the TXTSETUP.SIF file on disk 1. Under the section [SetupData], add the line ForceOEMSCSI=1. This will disable SCSI adapter detection, and the appropriate driver must be chosen manually.

Multiport Serial Adapters

Windows NT supports over 10 multiport serial adapters. Support for the intelligent Digiboard Adapters is incorporated in Windows NT. If you want to use a dumb multiport serial adapter or a Comtrol Hostess or Stargate Technologies adapter, you will either need a driver from your hardware vendor or you will need to modify the Registry to support your card. If you need to modify the Registry, follow these steps.

Û	To configure multiport serial adapters for Windows NT

1.	Determine the correct IRQ and port address values for your adapter.

2.	Install the adapter according to the manufacturer’s instructions.

3.	Using Registry Editor, open the key:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services
	\Serial\Parameters

For more information on using the Registry Editor, see “Editing Registry Values” in README.WRI.

4.	From the appropriate table below, add the Serial* keys under Parameters, and add the corresponding values for each Serial* key.

The actual values for your adapter may be different. Also, if you have only one serial port on your computer, you can start with the Serial1 and Com2 keys. If no serial ports are built into your computer, you can start with Serial0 and Com1.

5.	If there are no serial ports built into your computer, the following Values need to be added under the key:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\Serial

| Add value name | Data type | Data |
| --- | --- | --- |
| ErrorControl | REG_DWORD | 1 |
| Group | REG_SZ | Extended base |
| Start | REG_DWORD | 2 |
| Type | REG_DWORD | 1 |

6.	Quit Regedt32, and enter the following two commands at the command prompt to activate the changes:

net stop serial
net start serial

The serial ports should now work. Restarting Windows NT will also activate the changes.

Notice that COM1 through COM9 can be directly referenced just like a filename from programs and from the command line. However, COM10 and above must be referenced with the following syntax:

\\.\com10

The command line MODE command does not recognize ports above COM9.

The samples below all assume that there are two serial ports already available that will be listed as Serial0 and Serial1 in the Registry and assigned to COM1 and COM2 respectively. The additional ports will start at COM3. For more than four port adapters, simply continue the pattern for as many ports as needed. The following values will work even if one of the first two ports has a serial mouse attached.

Hostess 4-Port Adapter at IRQ 2, Starting Port Address 500, Starting at COM3

| Add key (no class) | Value names | Data type |  | Data |  |
| --- | --- | --- | --- | --- | --- |
| Serial2 | PortAddress | REG_DWORD |  | 0x500 |  |
|  | Interrupt | REG_DWORD |  | 2 |  |
|  | DosDevices | REG_SZ |  | COM3 |  |
|  | PortIndex | REG_DWORD |  | 1 |  |
|  | InterruptStatus | REG_DWORD |  | 0x507 |  |
| Serial3 | PortAddress | REG_DWORD |  | 0x508 |  |
|  | Interrupt | REG_DWORD |  | 2 |  |
|  | DosDevices | REG_SZ |  | COM4 |  |
|  | PortIndex | REG_DWORD |  | 2 |  |
|  | InterruptStatus |  | REG_DWORD |  | 0x507 |
| Serial4 | etc. | etc. |  | etc. |  |

DigiBoard 4-Port Adapter at IRQ 2, Starting Port Address 100, Starting at COM3

| Add key (no class) | Value names | Data type | Data |
| --- | --- | --- | --- |
| Serial2 | PortAddress | REG_DWORD | 0x100 |
|  | Interrupt | REG_DWORD | 2 |
|  | DosDevices | REG_SZ | COM3 |
|  | PortIndex | REG_DWORD | 1 |
|  | InterruptStatus | REG_DWORD | 0x140 |
|  | Indexed | REG_DWORD | 1 |
| Serial3 | PortAddress | REG_DWORD | 0x108 |
|  | Interrupt | REG_DWORD | 2 |
|  | DosDevices | REG_SZ | COM4 |
|  | PortIndex | REG_DWORD | 2 |
|  | InterruptStatus | REG_DWORD | 0x140 |
|  | Indexed | REG_DWORD | 1 |
| Serial4 | etc. | etc. | etc. |
| Serial5 | etc. | etc. | etc. |

Stargate 4-Port Adapter at IRQ 2, Starting Port Address 180, Starting at COM3

| Add key (no class) | Value names | Data type | Data |
| --- | --- | --- | --- |
| Serial2 | PortAddress | REG_DWORD | 0x180 |
|  | Interrupt | REG_DWORD | 2 |
|  | DosDevices | REG_SZ | COM3 |
|  | PortIndex | REG_DWORD | 1 |
|  | InterruptStatus | REG_DWORD | 0x580 |
| Serial3 | PortAddress | REG_DWORD | 0x188 |
|  | Interrupt | REG_DWORD | 2 |
|  | DosDevices | REG_SZ | COM4 |
|  | PortIndex | REG_DWORD | 2 |
|  | InterruptStatus | REG_DWORD | 0x580 |
| Serial4 | etc. | etc. | etc. |
| Serial5 | etc. | etc. | etc. |

Mice and Other Pointing Devices

Windows NT supports over 40 mice and other pointing devices. Windows NT includes four mouse port drivers that support various types of mice.

### Serial Mouse (driver name: sermouse.sys)

The serial mouse connects to COM1 or COM2. This driver also provides 
support for serial ballpoints (the default right-handed ballpoint orientation and primary/secondary buttons are supported). A ballpoint-specific driver that supports the selection of right/left orientation and primary/secondary buttons may be released via the Windows Drivers Library.

On x86-based computers, support for the serial mouse is automatic. On RISC-based computers, the serial mouse may not be detected but is supported.

To enable a serial mouse on a RISC-based computer, add one of the following to the Registry:

OverrideHardwareBitstring:REG_DWORD:0x1

– Or –

OverrideHardwareBitstring:REG_DWORD:0x2

Use the value 0x1 when the mouse is connected to COM1 and the value 0x2 when the mouse is connected to COM2. The value is located in the following Registry key:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Services\Sermouse\Parameters

For more information on editing the Registry, see “Editing Registry Values” in README.WRI.

### Microsoft InPort Bus Mouse (driver name: inport.sys)

The Microsoft InPort® bus mouse is supported on x86-based computers. It connects to the Microsoft InPort Adapter via a round 9-pin connector (the female connector is on the adapter). The mouse itself is labeled InPort Mouse or Bus Mouse. However, the InPort bus mouse is not the same as the bus mouse supported by BUSMOUSE.SYS, as described later in this section.

Jumpers must be set manually on the InPort adapter. A typical jumper setting for Windows NT is as follows (notice that we choose IRQ 2, since IRQs 3 or 4 conflict with the COM ports, and IRQ 5 is often used by network cards):

J2  NORMAL (v. XT SLOT 8)
J3  PRI. INPORT (v. SEC. INPORT)
J4  INTERRUPT 2 (v. 3 or 4 or 5)

### Bus Mouse (driver name: busmouse.sys)

The bus mouse is supported on x86-based computers. It connects to a Microsoft or Logitech® bus mouse adapter via a rectangular 9-pin connector (the female connector is on the adapter). The Microsoft bus mouse is an older version with green buttons and a steel ball. This mouse is not the same as the InPort bus mouse. The Logitech bus mouse is typically labeled “MouseMan Bus.”

One jumper must be set manually on the bus mouse adapter to select the IRQ. Typically, the IRQ selected for Windows NT is 2, since IRQs 3 and 4 conflict with the COM ports, and IRQ 5 is often used by network cards.

### Mouse Port or “PS/2” Mouse (driver name: i8042prt.sys)

The Mouse Port mouse (sometimes called the PS/2 mouse) is supported on both x86-based and RISC-based computers. It connects to a round, 6-pin connector built into the computer as the Intel 8042 (or Intel 8742) auxiliary device. The keyboard is also connected to the Intel 8042 and is supported by the same driver.

The I8042PRT.SYS driver also provides support for Mouse Port ballpoints. The default right-handed ballpoint orientation and primary/secondary buttons are supported. A ballpoint-specific driver that supports the selection of right/left orientation and primary/secondary buttons may be released via the Windows Drivers Library.

Support for the Mouse Port mouse is automatic on both x86-based and RISC-based systems. It is the standard pointing device on RISC-based computers.

### Support for Multiple and Alternative Pointing Devices

Windows NT can support the simultaneous use of more than one mouse or pointing device. The use of multiple pointing devices is not, however, supported by default. Windows NT Setup enables only one pointing device driver to load when starting.

Windows NT does not include drivers for alternative pointing devices, such as tablets, touch screens, and so on. Contact your hardware vendor for Windows NT drivers.

## Sound

Sound devices are configured after running Windows NT Setup. This section describes how to install Sound Blaster™ and the Windows Sound System to work with Windows NT.

When using a sound card, you may encounter an IRQ or DMA channel conflict with another device. Use the Drivers tool in Control Panel to reset if necessary. The recommended settings for the sound card are the following:

DMA1 (configurable; must be 1 for Sound Blaster)
IRQ 2, 7, or 10 (LPT1 also often uses IRQ 7)
Port 220h

### Sound Blaster

If your Sound Blaster card does not operate correctly, you may need to install the Creative Labs driver supplied on the Windows NT Setup CD. However, there are some 486 systems and some chip sets that are not fully compatible with the Sound Blaster/AudioPro card.

There may be an IRQ conflict with another device. Either change the IRQ setting on the other device or on the sound card. The default settings for the sound card are DMA 1, IRQ 7, and port 220h (notice that LPT1 also uses IRQ 7). Try physically changing the Sound Blaster card to IRQ 2 and then using the Control Panel to reconfigure the Creative Labs Sound Blaster 1.x driver to IRQ 2.

(Annie-- I changed the following paragraph, according to knowledgebase doc error Q103941, on the second page of the knowledgebase doc errors you gave me. What now reads “MIDI Synthesizer driver” used to read “AD-LIB MIDI driver”)

If you can play .WAV files but not .MID files, you need to install the MIDI Synthesizer driver. You can install this driver using the Drivers icon in Control Panel.

Û	To install and configure a Sound Blaster driver

1.	Start Windows NT and log on with Administrator rights.

2.	Run Control Panel, and then double-click the Drivers icon.

3.	In the Drivers dialog box, choose the Add button.

4.	In the Add Driver dialog box, select Creative Labs Sound Blaster 1.x, and then choose the OK button.

5.	In the configuration dialog box, select the correct port and interrupt number.

The typical default setting for the interrupt number is IRQ 7, and the port setting is usually 220. However, you might need to change these settings on your card to avoid conflicts with other hardware on your computer. For example, if your computer has an LPT1 port, you cannot use your Sound Blaster card on IRQ 7.

6.	When the message asks if you want to restart your computer, choose Restart Now.

### Pro AudioSpectrum-16

The Pro AudioSpectrum-16 includes an option for Sound Blaster emulation. This emulation is not supported by Windows NT. When installing the Pro AudioSpectrum-16, do not use the same interrupt value as that option. Using the same interrupt may cause unpredictable system behavior.

On earlier cards, the default for this interrupt is 5, and jumpers can be set to be 2, 3, 5, or 7. On later cards without jumpers, the default is usually 7. Please check the documentation for the card before installing it under Windows NT.

To perform optimally on the Digital DECpc AXP 150, the Pro AudioSpectrum-16 card should be installed using one of the higher DMA channels (channels 5-7).

If you do not use the Sound Blaster emulation under MS-DOS or Windows 3.1, you should disable it. For more information, see “Changing I/O Address for Sound Blaster Compatibility” in Appendix A of the Pro AudioSpectrum-16 
User Guide.

To install and configure the Pro AudioSpectrum-16, follow the steps above for installing the Sound Blaster.

Windows Sound System

Windows NT 3.5 supports the Windows Sound System and its utilities. In addition, Windows NT includes versions of Sound Recorder and Volume Control in the Accessories Group in Program Manager.

Û	To install and configure the Windows Sound System driver

1.	Start Windows NT and log on with Administrator rights.

2.	Run Control Panel, and then double-click the Drivers icon.

3.	In the Drivers dialog box, choose the Add button.

4.	In the Add Driver dialog box, select Windows Sound System, and then choose the OK button.

5.	Insert the Windows NT Setup CD-ROM, or insert the appropriate floppy disk in drive A.

6.	Accept the default configuration settings suggested by Windows NT. These settings depend upon your actual hardware, which may be improperly detected by Windows NT Setup in rare circumstances.

7.	When the message asks if you want to restart your computer, choose Restart Now.

### Sound on RISC-based Computers

Some RISC-based systems have built-in sound capabilities. For example, sound drivers are available for the following systems:

Olivetti M700-10
Acer ARC1
MIPS ARCSystem Magnum or Millennium PC-50
MIPS ARCSystem Magnum or Millennium SC-50

Follow the same steps as previously described to install one of these drivers. However, you do not have to change any configuration settings.

Printing Notes

This section contains information not available in the System Guide or in online Help for printers. Some of the topics in this document are specific to particular printer models or printer types, such as PostScript or PCL printers, and some are more general, such as instructions on how to print extended characters.

## Digital PrintServer Printers

Digital Equipment Corporation PrintServer printers are “smart” devices that can be soft-loaded from your Windows NT workstation or server by using bootp and tftp.

If your printer is configured but will not softload, and if ?54 appears on the front panel of the PrintServer, try the following.

u	Configure the proper hardware address for the PrintServer printer in Print Manager. Select the PrintServer printer in Print Manager. From the Printers menu, choose Properties. Choose the Settings button. Choose the Options button. Choose the Configure button.

If you are not sure of the hardware address, check the PrintServer's front panel after starting. The panel will display the hardware ethernet address.

u	The PrintServer printer and the Windows NT system must bee in the same TCP/IP subnet to softload the printer. You may print to a PrintServer printer from anywhere on your network, regardless of subnet.

u	Check your network hardware: Some types of hardware (bridges, for example) may be configured to filter bootp packets. To solve this problem softload the PrintServer printer from a Windows NT system that is on the same side of the bridge as the printer, or have your network administrator disable bootp packet filtering on that bridge.

## Printing a Document with Overlapping Text and Graphics

By default, device fonts are enabled and TrueType fonts are downloaded when printing a document. If you have a document with overlapping text and graphics, the document will not print correctly unless you disable device fonts and prevent TrueType fonts from downloading using Print Manager.

Û	To disable device fonts and prevent TrueType fonts from downloading

1.	Select the printer in Print Manager.

2.	From the Printer menu, choose Properties.

3.	Choose the Details button.

4.	In the Printer Details dialog box, choose the Job Defaults button.

5.	In the Document Properties dialog box, choose the Options button.

6.	In the Advanced Document Properties dialog box, select Print Text As Graphics and choose the OK button in all dialog boxes until you return to the Print Manager main window.

This option may also need to be set in the Advanced Document Properties dialog box when printing from an application.

## Using Beta 2 Print Drivers

Beta2 print drivers are incompatible with the Beta 1 release. All Windows NT 3.5 computers that want print to a Beta2 print server should upgrade to Beta2. Note that Windows NT 3.1 workstations may print to beta 2 servers the same way they did in beta1 (this only affects beta 1 computers connecting to beta 2 computers).

## Printing from Thinkpad 7xx Computers

If printing from the Thinkpad 7xx results in illegible output, you may need to change Parallel port arbitration in the computer’s system setup to Disabled in the BIOS setup program. This will force the parallel port to no-DMA ISA computer-compatible mode and it will resolve data handshaking error with some printers.

Û	To change parallel port arbitration in the computer’s system setup to disabled

1.	Turn on the ThinkPad.

2.	Press CTRL+ALT+INS after memory count up Enter password if it's set. IBM logo will appear.

3.	Press Enter.

4.	Select Set Configuration and press Enter.

5.	Select Change Configuration and press Enter.

6.	Select Parallel Port Arbitration Level. Press F6 several times until Disabled appears.

7.	Press F10 to save the current setting. Press Enter.

8.	Press F3 twice to exit the setup program, then press Enter to start the operating system.

You can also try changing the parallel port arbitration. If it does not solve the problem, contact Xircom for a driver update.

## Windows for Workgroups Clients Not Printing Simultaneously

There is a know problem when simultaneously printing from multiple Windows or Windows for Workgroups clients to the same Windows NT print server. A fix for this problem is available on CompuServe in the BETAWINNT forum.

## Allowing Users to Print to Local Printers When Windows NT is Installed on an NTFS Drive

If Windows NT is installed on an NTFS partition, users without administrator privileges will not be able to print to local printers, unless the following steps are performed once. Alternatively, a fix for this problem is available on CompuServe in the BETAWINNT forum.

Û	To allow any user logged onto the system to print to the attached local printer

1.	Log on as Administrator.

2.	Start File Manager.

3.	Change to the systemroot\SYSTEM32\SPOOL\PRINTERS directory.

4.	From the Security menu, choose Permissions.

5.	In the Directory Permissions dialog box, select Everyone. In the Type of Access box, select Add.

6.	Choose the OK button.

7.	Log off.

Printing to a File from an Application Using the Connect To Printer Command

An output file may not be generated when printing to a file from an application if the printer has been set up using the Connect To Printer command. To solve this, in the Printer Properties dialog box, change the Print To destination to FILE:.

## Printing from Windows 3.1-based Applications

Some Windows 3.1-based applications expect the printer driver to exist on the local computer when you are printing to a network printer. Some applications may fail when the network printer is not available. To avoid these problems, install an appropriate printer driver on your computer.

You may also experience problems with printers with names of 32 characters or greater. To prevent this problem, use less than 32 characters for printer names.

## Problems with Printer Name Over 32 Characters

Windows NT can create long printer names, however some applications cannot print to printer with names longer than 32 characters. If you experience problems, create a printer with 31 or fewer characters in the name.

## Changing the Default Paper Form

Changing the country setting using the International option in Control Panel sets the default paper form used by non-PostScript printers only. The default form for all PostScript printers is determined by the printer driver when you install the printer.

## Printing Extended or International Characters

In addition to the 128 standard ASCII characters you can type by using your keyboard, you can use extended or international characters by using the Character Map application. For more information, see Help for Character Map.

When Windows NT prints a file, each character you typed while using your application is translated from the Windows NT character to the appropriate character on your printer. If your printer supports the same character, the character prints. Otherwise, some other character such as a period or other filler character prints instead. Check your printer manual and experiment with your printer to determine which extended characters are supported.

This limitation applies only to the printer’s hardware fonts. Fonts provided by Windows NT do print extended characters.

Note

Troubleshooting for Printing of Incomplete Pages

When printing complex documents from Windows 3.1 or MS-DOS – based applications using a slower computer, Windows NT may begin printing before a complete page has been processed. When this occurs, portions of a page may be printed on a series of pages. You can resolve this problem by increasing the time that Windows NT waits before starting to print.

To increase the time that Windows NT waits, edit the following value in the Registry:

LPT_timeout:REG_SZ:15

The value is located in the following Registry key:

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Control\WOW

By default, the value is 15 seconds. Because raising this value slows the speed of printing from all MS-DOS and Windows 3.1-based applications, do not raise the value higher than necessary. You may want to try a value of 60 seconds first.

For more information on using the Registry Editor, see “Editing Registry Values” in README.WRI.

## Printing Colored Text on Non-Postscript Printers

If text is not one of the primary colors (black, red, green, blue, cyan, magenta, yellow or white) the color may get mapped to white and you won't see any text. In other cases it may get mapped to another color.

## Configuring Your Printer’s DIP Switch Settings

The following printer models require certain DIP switch settings in order to work properly in Windows NT. Make sure you configure your printer’s DIP switch settings before you install the printer.

Printer DIP Switch Settings

| Printer type | Comment |
| --- | --- |
| Canon® Bubble-Jet BJ-10e, BJ-10ex, and BJ-130e | All DIP switches should be set to the factory-default position. For the BJ-10e and BJ-130e models, this is the OFF position. |
| Digital 9-pin and 24-pin printers supported by Windows NT | These printers should be set to Emulation (IBM ProPrinter®) mode. See your printer manual for details on how to set the emulation. |
| Digital laser printers supported by Windows NT | Printers listed that have an “(LJ)” after the model name must have a HP LaserJet emulation cartridge installed. The printer must be set to LaserJet mode. See your printer manual for details on how to set the emulation. |
| Epson® 9-Pin and 24-Pin printers supported by Windows NT | The following DIP switch settings are required for all Epson 9-pin and 24-pin printers: Auto LineFeed: OFF Skip Over Perf: OFF |
| Fujitsu® 9-pin and 24-pin printers supported by Windows NT | The following DIP switch settings are required for all Fujitsu 9-pin and 24-pin printers: Color: AUTOSEL LF Code: LF Only CR Code: CR Only Emulate: DPL24/DPL24C |
| Mannesmann Tally® 9-pin and 24-pin printers supported by Windows NT | The emulation on the MT 130/9, MT 131/9, MT 230/9, MT 230/18, and MT 340 models should be set to ANSI + IBM ProPrinter. See your printer manual for details on how to make this change. The emulation on the MT 130/24, MT 131/24, MT 230/24, and MT 330 models should be set to ANSI + Epson LQ2500 emulation. See your printer manual for details on how to make this change. |
| Olivetti 9-pin and 24-pin printers supported by Windows NT | These printers should be set to Epson mode. See your printer manual for details on how to set the emulation. |
| Seikosha LT-20 | The following DIP switch setting is required: AGM mode: ON Code page: 850 |

Notes on Specific Printers and Font Packages

### Notes on Hewlett-Packard, Canon, and PCL Printers

This section contains information specific to printing on Hewlett-Packard, Canon, and PCL printers.

Notes on Hewlett-Packard, Canon, and PCL Printers

| Issue | Comment |
| --- | --- |
| PCL printers — setting printer memory | When you create a printer for a PCL printer, make sure that the memory specified in the Printer Setup dialog matches the memory your printer contains. Specifying an incorrect amount of memory can cause the printer to print incorrectly. For more information on setting printer memory, choose the Help button in the Printer Setup dialog box. |
| HP® LaserJet® IID | If a job requires different sources for different pages, then the control panel "TRAY=" menu selection should be set to "UPPER". This setting is also a prefered setting. |
| HP LaserJet IIIsi — language switching | System language switching must be set ON (SYS SWITCH=ON*) when printing to LaserJet IIIsi printers. If language switching is set OFF, the printer may not print. |
| HP LaserJet IIIsi in PostScript mode | If you are using a LaserJet IIIsi printer in PostScript mode, make sure the PRT PS ERRS option on your printer panel menu is turned off (the factory-default setting), unless you are writing PostScript programs and need error information for debugging. If this option is on, your print jobs pause, and a message appears on your printer panel each time a PostScript error is encountered. In most cases, your documents still print, but the printing process is interrupted. |
| HP LaserJet Series III — upgrading printer drivers | The HP LaserJet III printer driver included with Windows NT is named HPPCL5MS.DRV. In earlier versions of Windows, this driver was named HPPCL5A.DRV. If you upgrade to Windows NT, the Setup program updates the printer driver for the HP LaserJet III printer. However, some applications store the information about the printer-driver file with the document, and if you try to print a document that you created before you upgraded to Windows NT, you may receive a message notifying you that the printer driver cannot be found. If this happens, choose the Printer Setup command in your application to update the driver information stored in the document. |

Notes on Hewlett-Packard, Canon, and PCL Printers (continued)

| Issue | Comment |
| --- | --- |
| HP LaserJet 4 — memory | Although LaserJet 4 printers have a maximum memory size of 32 MB, Windows NT supports a maximum of 17 MB. This restriction does not apply to the LaserJet 4 in PostScript mode. |
| HP DeskJet® 550c — printing envelopes | Because of a driver limitation, Windows NT prints envelopes upside down on the HP DeskJet 550C when you load envelopes following the printer’s diagram. To avoid this problem, load envelopes upside down relative to the diagram on the printer. |
| HP PaintJet® XL — margins | You should set all margins to at least 0.5 inch for the PaintJet XL. If clipping occurs, increase the margin on the clipped side. |

### Notes on Additional Printers and Font Packages

This section contains information specific to font packages and printing on dot-matrix, 24-pin, Epson, and IBM printers.

Notes on Additional Printers and Font Packages

| Issue | Comment |
| --- | --- |
| Printing graphics on Canon Bubble-Jet BJ – 10e or BJ –130e | If you are printing in 360x360 dpi graphics mode and parts of the graphics images in your documents are missing, make sure the DIP switch to control graphics image density is set to HIGH. |
| Fonts with Epson LQ –510, LQ –850, and LQ –1050 printers | The Epson LQ –510, LQ –850, and LQ –1050 printer drivers support the full set of fonts provided with the latest versions of these printer models. Earlier versions of these printer models only support the Roman and Sans Serif fonts. |
| TrueType® fonts on Kyocera® F-series printers | To print TrueType fonts on Kyocera F-Series printers, make sure that the Download TrueType Fonts check box is cleared in the Advanced Document Properties dialog box when configuring your printer. Otherwise, TrueType fonts may not print correctly. For more information on setting the Download TrueType Fonts check box, see “Setting Job Defaults,” in Chapter 6, “Print Manager,” in the System Guide. |

Notes on Additional Printers and Font Packages (continued)

| Issue | Comment |
| --- | --- |
| Soft fonts on Mannesmann Tally MT –92C | Only HP LaserJet compatible fonts can be downloaded and only if the 256K memory card is fitted. HP DeskJet compatible fonts will not work. |
| Cut-Sheet Feeder on NEC Pinwriter P7 printer | The Paper Source option, Cut Sheet Feeder, does not work properly with the NEC Pinwriter P7 printer. To use the cut-sheet feeder as the paper source, you must specify a left margin of about three inches for your document so that text prints correctly on the page. |
| Feeding paper on Fujitsu 9-pin dot-matrix printers | The manual-feed option is not supported on Fujitsu 9-pin dot-matrix printers. All paper is automatically fed to the printer from the roller. |
| Changing printer settings with Bitstream® Facelift™ Version 1.0 | With some applications, you can change the settings for your printer on a per-page basis. You cannot use this feature in Bitstream Facelift 1.0 when printing multiple-page documents. |
| Using separator pages | If you are using one of the following printer models and the printer is a local printer (attached to your computer), do not use the separator-page feature in Print Manager, because this feature is not supported. Canon LBP-4, LBP-8 II, and LBP-8 III IBM Laser Printer 4019 Olivetti DM 109 and DM 309 |

Application Notes

The following sections provide general information about running applications on Windows NT that were created for Windows 3.1 or MS-DOS. Windows NT has excellent support for Windows 3.1 and MS-DOS – based applications. If you do encounter a problem, this section provides some solutions.

To ensure your system reliability and provide you with security, Windows NT does not run MS-DOS –based and Windows 3.1-based applications that attempt to directly access system hardware.

Note

## Using MS-DOS-based Disk Utilities

The new long file name format introduced in Windows NT 3.5 uses file elements on FAT partitions differently than MS-DOS. When using MS-DOS--based disk utilities on FAT partitions used by Windows NT 3.5 files can be deleted or long file names can be lost. Microsoft recommends that you only run disk utilties specifically designed for Windows NT 3.5, or when running MS-DOS, the chkdsk or scandisk utilities.

To prevent the Windows NT 3.5 FAT file system from using long names, you must set to 1 the following registry entry :

HKEY_LOCAL_MACHINE\SYSTEM\CurrentControlSet\Control\FileSystem\Win31FileSystem

After changing the entry, you must restart your computer for the change to take effect.

CAUTION

PCTools' diskfix utility is known to delete all files on the FAT partition. If you use MS-DOS-based disk utilities on a FAT disk partition used by Windows NT 3.5, the utility may cause the loss of long file names or delete files on the FAT partition unless this registry entry is set to 1. You should backup the FAT partition before running any disk utility other than chkdsk.

## Running SNA Server

During the course of testing SNA Server 2.0 on the Windows NT 3.5 Beta 2, two bugs were discovered in SNA Server 2.0.  These bugs were uncovered as a result of the improved parameter checking and fault detection included in Windows NT 3.5 and do not affect the operation of SNA Server 2.0 on Windows NT 3.1.

The symptoms of these bugs are SDLC and X.25 SNA connections will not go active and 802.2 SNA connections will cause an exception in SNALINK.EXE.  Fixes for these problems are available in the SNA directory of your Windows NT 3.5 Beta 2 CD and are also available in the Library 2 section of the MSSNA forum on CompuServe.

If you have SNA Server 2.0 installed on your system, please copy the contents of the SNA directory to your <snaroot>\SYSTEM directory after you have upgraded your system to Windows NT 3.5.|

The contents of the SNA directory are:

IBMSDLC.DLL
IBMX25.DLL
SNADLC.DLL

Although these patches are openly available, Microsoft recommends that you use SNA Server 2.1 Beta on all Windows NT 3.5 systems.  If you would like to join the SNA Server 2.1 Beta program please send email to MSSNA@MICROSOFT.COM and include your name, company name and shipping address. Please refer to the MSSNA forum on Compuserve for any additional updates or notices.

## Running Serial Communication Applications

For reliable data transfer at the highest baud rate, use communication protocols 
that support end-to-end flow control. If the protocol you use does not support 
flow control, you may need to reduce the serial port baud rate for reliable communications. The following table shows which protocols support flow control:

| Serial flow control protocol | Flow control |
| --- | --- |
| ASCII (Xon/Xoff) | No |
| XMODEM | Yes |
| XMODEM-1K | Yes |
| YMODEM | Yes |
| YMODEM-G1 | No |
| ZMODEM2 | No |
| Kermit | Yes |
| CompuServe B+ | Yes |
| 1 The “-G” option disables flow control. 2 Some applications allow flow control with ZMODEM. For example, you can enable flow control with Procomm Plus 2 by setting the Transmit Method to “4K WINDOW.” |  |

Windows NT supports 16-bit Windows communications programs using the communications driver COMM.DRV. Other communications drivers are not supported and may not function reliably. You can check or change the communications driver using the Registry Editor. The value should be:

comm.drv:REG_SZ:comm.drv

The value is located in the following Registry key:

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows NT
	\CurrentVersion\WOW\boot

For more information on using the Registry Editor, see “Editing Registry Values” in README.WRI.

To run multiple 16-bit communications applications simultaneously, or to run a 16-bit communications application in the background or minimized and still maintain a reliable connection, it is suggested that you modify the Tasking on your system to Foreground and Background Applications Equally Responsive. To do this, in Control Panel, choose the System option. Choose the "Tasking" button. In the Tasking dialog box select Foreground and Background Applications Equally Responsive and choose the OK button.

### Procomm Plus for Windows

The Procomm Plus implementation of the XMODEM protocols for uploads may yield less-than-expected throughput when running NT on slower 386-based machines.

Generally, you shouldn't experience any problems using the ZMODEM protocol for transfers on 486-based computers. On slower computers, try using 16-bit CRC and 4K sliding windows for ZMODEM downloads to increase the throughput and robustness.

Using ZMODEM for uploads and XMODEM for downloads on slower computers will give the expected performance at higher baud rates.

Running MS-DOS–Based Remote Control Applications

Remote control applications control a remote computer from a local computer through a serial connection. The local computer runs a program that sends keystrokes to the remote computer while displaying a copy of its screen. The remote computer runs a memory-resident program that allows the local computer to control it. If the remote computer is running Windows NT, you must create custom startup files and a custom PIF to set up the MS-DOS environment for the memory-resident program.

Û	To create the startup files

·	Edit the AUTOEXEC.NT and CONFIG.NT files to change the following items, and then save the files under new names:

u	In the custom AUTOEXEC file, add the name of the memory-resident program if you want to start it when you run the PIF.

u	In the custom CONFIG file, add the commands dosonly and echoconfig. The dosonly command ensures that no programs can be started that can disable the memory-resident program.

Û	To create the PIF

1.	Double-click PIF Editor in the Main program group, and select the _DEFAULT.PIF file.

2.	In the PIF Editor dialog box, type command.com in the Program Filename box.

3.	In the Advanced Options dialog box, clear the Allow Fast Paste check box.

4.	In the Windows NT Options dialog box, specify the names of the custom startup files you created.

5.	Save the file under a new name.

For more information on creating PIFs and custom startup files, see the “Other Application Environments” chapter in the System Guide.

## Running MS-DOS–Based Multimedia Applications

All MS-DOS–based (and many 16-bit Windows) applications that play multimedia do so by directly accessing the hardware involved. Direct access of most multimedia hardware (sound cards, and so on) is not supported. Consequently, any application that uses such methods to access hardware will be unsuccessful. Results may vary according to the application’s handling of error conditions. Typically the application will execute without the multimedia portions working but may be slow.

## Limits to Application Support on RISC-based Computers

On RISC-based computers, Windows NT provides emulation for the 80286 instruction set. Applications that rely on the 80386 instruction set are not supported. The following applications or programs are included in this group:

| Approach 2.1 AutoCAD® for Windows Autodesk® 3D Studio Borland C++ 4.0 Charisma 4.0 Claris Works 2.0 Designer 4.0 FoxPro for Windows 2.5 FrameMaker 4.0 Gupta SQL Windows 4.0 Harvard Graphics for Windows 2.0 Lotus® Improv for Windows 2.0 Mathematica 2.2 Microsoft Bookshelf 1994 Microsoft C 7.0 Microsoft Developers Network Disk 3 Microsoft Flight Simulator 5.0 | Microsoft FoxPro® for Windows Microsoft Profit 1.0 for Windows Microsoft Visual Basic Professional Programming System for MS-DOS 1.0 (LINK.EXE requires 80386 support) Microsoft Visual C++ 1.5 Microsoft Works for Windows 3.0 MS-Cobol 5.0 (LINK.EXE requires 80386 support) Paradox® for Windows Photoshop 2.5 for Windows PowerBuilder 3.0 Quark Express QuattroPro® for Windows 5.0 Ventura Publisher 4.1.1 for Windows Visio 2.0 WordPerfect for Windows 6.0 |
| --- | --- |

## Information on Specific Windows 3.1 and MS-DOS–Based Applications

This section provides application-specific information about running MS-DOS–based and Windows 3.1-based applications on Windows NT. It describes problems you might encounter and provides solutions where possible. Applications are listed in alphabetical order.

Notes on Applications Running Under Windows NT

| Application | Comment |
| --- | --- |
| 1-2-3 version 2.4 and greater | Lotus 1-2-3® can be configured to print in either parallel or LPT mode. For optimal printing performance with Windows NT, configure the application in LPT mode. |
| After Dark | Pressing Ctrl+Esc to display the Task List dialog box disrupts the “journal record” in After Dark so it never recognizes keyboard or mouse activity, causing it to start the screen saver even when the computer is in use. If you press Ctrl+Esc, close and restart After Dark. |
| Agenda | Start Lotus Agenda® using the forcedos command. For more information, see the forcedos topic in the Command Reference section of Windows NT Help. |

Notes on Applications Running Under Windows NT (continued)

| Application | Comment |
| --- | --- |
| Aldus® PageMaker® | When printing from PageMaker, always have Print Directly To Ports checked in the Printer Details dialog box. You may need to create a printer first in order to check this option. |
| AutoCAD Release 11 286 | The Autolisp extensions (EXTLISP.EXE) for AutoCAD are not supported because they require VCPI. |
| AutoCAD Release 12 | To correctly redraw and perform other graphic operations, such as cut and paste, you must use the correct AutoCAD video driver. The default video driver when you install AutoCAD is Windows Accelerated Display Driver ADI 4.2 - by Autodesk, Inc (#1). If you use this driver you will have problems with cut and paste and with redraw operations. Use the Windows driver - by Autodesk, Inc (#2), and these operations will function correctly. |
| Autodesk 3D Studio, Release 2 | 3D Studio is not supported. The application fails when attempting to run the processor in protected mode. |
| Autodesk Generic CADD 6.0 | The Generic CADD 6.0 screen is limited to 640x480 VGA resolution on Windows NT. To use this application, select the “IBM VGA (Video Graphics Array) & compatibles” video graphics display from the Generic CADD 6.0 configuration utility (CONFIG.EXE). |
| Borland® C++ 3.0 | The 16-bit character-mode debugger (TD.EXE) is supported; however, the 16-bit Windows-based debugger (TDW.EXE) is not supported. |
| Borland Pascal with Objects 7.0 | Turbo Debugger and WinSight are not supported. |
| Borland Turbo Pascal® for Windows | The 16-bit character-mode debugger is supported; however, the 16-bit Windows-based debugger is not supported. |
| Broderbund® KidPix for Windows v. 1.0 | The application may fail when printing to a printer whose port name is greater than 12 characters. To avoid this problem, before you print use Print Manager to make sure that the name of the printer’s destination shown in the Print To box in the Printer Properties dialog box is less than 12 characters. |
| cc:Mail™ | The PATH environment variable cannot exceed 143 characters or a General Protection Fault in cc:Mail will occur.The path used by Windows NT is created by using the Windows NT path settings and appending the path in the computer’s AUTOEXEC.BAT file. Reduce the length of your path and restart the computer. While installing cc:Mail for Windows, a General Protection Fault in MSCONSTF.DLL will occur if you do not have a CONFIG.SYS file. Although CONFIG.SYS is required by Windows, it is not required for Windows NT. The cc:Mail setup program looks for CONFIG.SYS and the fault occurs if the file doesn't exist. Shutdown Windows NT and create a file named CONFIG.SYS in the root directory of your boot drive. The content of the file is irrelevant. Using the system mouse pointer during the installation of cc:Mail can lock the keyboard. To work around this problem, you can either hide the mouse pointer or, on x86-based computers only, run the installation program using the full screen. To hide the mouse pointer, follow the instructions in “Using an Application’s Own Mouse Cursor” earlier in these Release Notes. |
| Chessmaster 3000 | Because of copy protection on the Chessmaster distribution disks, Windows NT does not support installation of this application. |
| Commute | Start Commute using a custom PIF and custom startup files. Be sure to clear the Allow Fast Paste check box in the Advanced Options when creating the PIF. For more information, see “Running MS-DOS–Based Remote Control Programs” earlier in these Release Notes. |

Notes on Applications Running Under Windows NT (continued)

| Application | Comment |
| --- | --- |
| CompuServe Information Manager (CIM) | The MS-DOS–based version of CIM attempts to access COM1 through COM4 when it starts. If any of these ports is not installed or is already in use, you may see the following error message: “The system cannot open COM port requested by the application.” You can safely ignore this message unless your modem is installed on the unavailable port. |
| Corel® Draw version 4.0 | When choosing Save As from the File menu, the error “Not enough diskspace” may appear, because the application is trying to load its own 16-bit driver, which is not supported under Windows NT. To solve this, select None or Only Mono Formats for the Image Header option. |
| Eicon Access | For information on running Eicon Access, see “Running Windows Mainframe Connectivity Software,” in Chapter 2 of the Installation Guide. |
| Extra! for Windows | For information on running Extra!, see “Running Windows Mainframe Connectivity Software,” in Chapter 2 of the Installation Guide |
| F-19 Stealth Fighter | This application is not supported. |
| Fastback 6.0 for DOS | If you experience display and/or mouse problems, try to disable the 3-D by changing the Screen Mode to Flat Color and change Show Mouse to Block. |
| File Maker Pro | Printers used by File Maker Pro must have names of 32 characters or less. |
| Filewalk | FILEWALK.EXE (version 1.0A CDRM 566200), included on the Microsoft Multimedia Development Kit CD-ROM, attempts direct disk access and is not supported. A working version (S/B 1.0, CDRM 837500) of this application is available from Microsoft. |
| LapLink V | File transfer between parallel ports and between serial ports is not supported. You can transfer files using modems. |
| LHX Attack Chopper | Windows NT does not allow applications to directly adjust the computer’s timer. In this case, that results in sound that runs slowly. |
| Lotus Notes | Lotus Notes depends on quick carrier detect notification, which the simulated modem interrupt in Windows NT sometimes does not fast enough when run at a lower priority. A fix will be available for this shortly on CompuServe. |
| Micrografx® Windows Draw 3.0 | Version 3.0 (720K) release disks may fail during installation. For more information, see “Floppy Disk Failure When Installing an Application” earlier in this document. |

Notes on Applications Running Under Windows NT (continued)

| Application | Comment |
| --- | --- |
| Microsoft C 7.00 | CodeView for Windows (CVW.EXE) is not supported. To debug 16-bit Windows-based applications, use the Windows NT debugger, WINDBG.EXE. The character-mode version of CodeView is supported. |
| Microsoft Dinosaurs | To view the video clients in Microsoft Dinosaurs, edit MSDINOS.INI to change VideoInWindow value to 1. Microsoft Dinosaurs does not install the Macromind Multimedia Player (MMP) when installed on a Windows NT computer. You must copy MMCIMMP.DRV and MMP.DLL from the \XFILES\DRIVERS directory on the Microsoft Dinosaurs CD to the \systemroot\SYSTEM directory. After copying the files you must add the following entries to the registry. Add the REG_SZ value MMMovie=MCIMMP.DRV to the key: HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\ Windows NT\CurrentVersion\MCI Add the REG_SZ value mmm=MMMOVIE.DRV to the key: HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\ Windows NT\CurrentVersion\MCI Extensions |
| Microsoft Encarta 1994 | If you have a RISC-based Windows NT computer you must setup and run Encarta on a hard disk so you can replace two files. Copy ENC94IMG.DLL and POEMWINC.DLL from the \ENCARTA\NT directory on the Encarta CD to the root directory of your Encarta installation (the default is C:\ENCARTA). |
| Microsoft Excel version 5.0 | Windows NT does not support computer-based tutorial (CBT) files. The Help files use CBT files for the Examples, Demos and Quick Previews. When viewing sections of help that use the CBT files, you will receive a pop-up box saying “you cannot run the CBT with this operating system” or “CBT directory not found.” |
| Microsoft Entertainment Pack, Idlewild | To use Idlewild screen savers with Windows NT, the full path to the screen savers must be added to the system path. Otherwise, the system will be unable to locate the screen savers and the screen will blank when a screen saver is supposed to start. You can either add the screen savers’ directory to the path in your AUTOEXEC.BAT file or use the System option in Control Panel to add an environment variable containing the path. |
| Microsoft PowerPoint | The GeniGraphics® driver included with PowerPoint is not supported. |

Notes on Applications Running Under Windows NT (continued)

| Application | Comment |
| --- | --- |
| Microsoft Word for Windows version 6.0a | Windows NT does not support computer-based tutorial (CBT) files. The Help files use CBT files for the Examples, Demos and Quick Previews. When viewing sections of help that use the CBT files, you will receive a pop-up box saying “you cannot run the CBT with this operating system” or “CBT directory not found.” |
| Microsoft Word, version 5.5 | When using Word with a foreign language keyboard, start the application using the forcedos command. This enables “dead keys” used when composing accented characters with two keystrokes. For more information, see the forcedos topic in the Command Reference section of Windows NT Help. |
| Microsoft Works 3.0 | Windows NT does not support computer-based tutorial (CBT) files. The Help files use CBT files for the Examples, Demos and Quick Previews. When viewing sections of help that use the CBT files, you will receive a pop-up box saying “you cannot run the CBT with this operating system” or “CBT directory not found.” |
| Multiplan version 4.2 for DOS | The MP.PIF file has the Windows 2.11 format which forcedos doesn't support. To fix this problem, open MP.PIF using the Windows NT PIF editor and save it. This will save it in the proper format. |
| Norton Utilities™ 6.01 | Utilities that directly address system hardware are not supported. This includes, for example, disk caching, virus checking, and error diagnostics utilities. |
| ORACLE® DBMS for DOS | Although Windows NT clients can access data on Oracle servers, because Oracle for MS-DOS uses VCPI memory management, the application is not supported. |
| OzCIS | If OzCIS loses characters when transmitting data, try using a lower baud rate. |
| PackRat 5.011 for Windows | On RISC-based computers, the PackRat setup program generates illegal instructions when trying to modify the path. Select No when the PackRat setup program asks if you would like to add PackRat directory to your path. After installation, add the directory to your path manually. |
| pcANYWHERE III | Start pcANYWHERE using a custom PIF and custom startup files. Be sure to include the dosonly command in the custom configuration file. For more information, see “Running MS-DOS –based Remote Control Programs” earlier in these Release Notes. |
| Photoshop, Adobe® | Other windows may not repaint properly when you are running Photoshop using 256 colors and an operation, such as restoring a file, changes the color palette. Loading pattern files (*.AI) in Photoshop can cause the application to fail. |

Notes on Applications Running Under Windows NT (continued)

| Application | Comment |
| --- | --- |
| Quattro Pro for Windows, version 1.0 | Multiple copy printing is not supported. |
| Rumba | For information on running Rumba, see “Running Windows Mainframe Connectivity Software,” in Chapter 2 of the Installation Guide |
| SimAnt, Maxis | The application can fail when the mouse is moved if sound is enabled for the computer’s speaker. To prevent this, disable the sound. You can either edit SIMANT.CFG and change the setting for Sound Mode to 0, or install the application and set the setting for Sound Driver to None. |
| Smartcom Windows Communication Software, v. 1.0A | Use the Smartcom Custom utility program to configure Smartcom to use the Windows NT communications driver COMM.DRV. |
| SQLWindows 4.0/4.1 for Windows | At the end of installation, always choose Upgrade Existing Program Group, even on first installation. If you choose to create a new program group, any name used will be considered invalid. |
| SuperPrint, v. 2.2 | The application is not supported because it relies on incompatible 16-bit device drivers. |
| US Atlas, version 3.1.0, The Software Toolworks | The application may fail when printing to a printer whose port name is greater than 12 characters. To avoid this problem, before printing, use Print Manager to make sure that the name of the printer’s destination shown in the Print To box in the Printer Properties dialog box is less than 12 characters. |
| Ventura Publisher® | On RISC-based computers, the application can fail when attempting to open more than one document or when some mathematical formulas are entered. |
| Delrina Winfax™ Pro, Version 2.0 | The application is not supported because device drivers are not yet available. |
| WordPerfect Windows 6.0a | The tutorial is not supported. |