===============================( README.DOC )===============================

README.DOC for Cakewalk

$Date:   29 Nov 1991 12:02:00  $

Cakewalk is copyright (c) Greg Hendershott, 1987, 1988, 1989, 1990.

All rights reserved.

Cakewalk is a trademark of Twelve Tone Systems, Inc.

============================================================================

=====( Installing on top of an existing Twelve Tone Systems product )=======

If you are installing on top of an existing Twelve Tone Systems product,

just follow the normal instructions in the User's Guide.  But note that

this will replace your existing MIDI driver (.DRV) files with the new

ones.  If you've run Setup to change the MIDI interface settings to other

than their factory default, you'll need to do so again after installing.

-------

================================( DRIVERS )=================================

-------

Following is a list of the Installable Port Drivers supplied with this

release:

Driver file	Device

-----------	-----------------------------------------------------------

MPU401.DRV	MPU-401 compatible (including Music Quest PC MIDI Card)

MQBASIC.DRV	Music Quest Basic MIDI Card

MQX16.DRV	Music Quest MQX-16

MQX16S.DRV	Music Quest MQX-16S

MQX32.DRV	Music Quest MQX-32 or MQX-32M

IBMPS1.DRV	IBM PS/1 Audio Card and Joystick Option

KEY101.DRV	Key (KEE) MIDIator MS-101

CMS101.DRV	Computer Music Supply CMS-101

OPTRON.DRV	Optronics Basic MIDI I/O card

ADLIB.DRV	AdLib or Sound Blaster FM sounds

SBFMMI.DRV	Sound Blaster FM sounds and MIDI

SBMI.DRV	Sound Blaster MIDI only (no sounds)

BEEP.DRV	Plays one tone at a time on the PC speaker

M_ADL.DRV	MPU-401 compatible + AdLib/Sound Blaster FM sounds

| The above product names may be trademarks or registered

| trademarks of their respective manufacturers.

+-------------------------------------------------------------------------+

NOTE: These drivers are copyrighted material just like Cakewalk itself.

Your license does NOT include the right to copy them (except for

personal back-up purposes) or the right to modify them.

+-------------------------------------------------------------------------+

Note: For space reasons, we have limited the supplied drivers to those

that cover the most popular interfaces for this particular product.

Our Supplemental Drivers Diskette contains all single and multiple board

drivers developed so far by Twelve Tone Systems.  New releases of the

Supplemental Drivers Diskette are made as new drivers are created.

Contact Twelve Tone Systems for the latest information on this diskette.

If you desire support for an unsupported interface (or combination of

interfaces), send your request in writing to us ("ATTN: Driver Requests")

for consideration.

=====( Installing the right driver for your MIDI interface )===============

(1) IMPORTANT: Each Twelve Tone Systems product has been initially set up

with the most likely MIDI interface driver for that product.  But that

may not match the MIDI interface you have.  Copy the appropriate driver

file to a file named MIDIPORT.DRV.  Cakewalk needs to find MIDIPORT.DRV

when it starts; otherwise you will get an error message.

For example, if you are using an MPU-401 compatible, you need to enter the

following at the DOS Prompt:

CD \CAKEWALK

COPY MPU401.DRV MIDIPORT.DRV

(2) Then, if you are using a MIDI interface with other than its

factory-default settings -- for example, an MPU-401 compatible on other

than IRQ 2 and base port 330 -- you must run Setup.  Start Cakewalk by

entering at the DOS Prompt:

CAKEWALK -S

Select choice (3), "MIDI Interface Settings", and enter the appropriate

information.

If you ever recopy the original .DRV file to MIDIPORT.DRV, you will need

to run Setup again.

Tip: If you want to determine which driver is currently installed, just

run Setup as described above.  The interface type is displayed on the

same screen as its current settings.

=====( If you are upgrading from 3.0 )======================================

If you are using an interface with other than its out-of-the-box,

factory-default settings, you must run Setup again.  The settings you

previously made used to be stored in CAKEWALK.INI, but Cakewalk no longer

looks in CAKEWALK.INI for these values.  It looks in MIDIPORT.DRV.  And

when you run Setup, the values you set are now stored in MIDIPORT.DRV.

If you ever recopy the original .DRV file to MIDIPORT.DRV, you will need

to run Setup again.

=====( IBM PS/1 Owners Only:  Using the IBM PS/1's Thru/Out Port )==========

The IBM PS/1 Audio/Joystick Card has a MIDI Thru/Out port which can be set

to either Thru or Out.  When it's set to Thru, it outputs a copy of what

comes into the MIDI In port.  This is true even when Cakewalk is not

running.  When it's set to Out, it outputs a copy of what the MIDI Out port

is sending.  Cakewalk must be running to generate this output.

When the IBM PS/1 starts up, the MIDI Thru/Out port is set to Out.  We have

supplied two programs you can run to set this the way you prefer.  At the

DOS prompt, type PS1THRU to set it to Thru and PS1OUT to set it to Out.  You

may want to put one of these commands in your AUTOEXEC.BAT file, so it gets

set right automatically every time your PS/1 starts up.  Note that

Cakewalk sets it to Out while running, and restores the original state

when done.

Hint: You may want to put PS1THRU in your AUTOEXEC.BAT file, and then use

the MIDI Thru/Out Port instead of the MIDI Out Port.  This way, when

Cakewalk is running you get what the program sends, and when the program

is not running you get an echo of MIDI In.

=====( Key MIDIator MS-101/103 Notes )======================================

If Cakewalk can't find your Key MIDIator, first make sure you've selected

the right IRQ number and address for your serial port.  Run Setup as

described above and select "MIDI Interface Settings".  For COM1, use IRQ 4

and address 3F8.  For COM2, use IRQ 3 and address 2F8.  If that's okay,

try a different serial cable.  It's critical to get a properly wired

serial cable that connects all the required pins.  The MIDIator uses some

pins that most applications don't need.

If you notice occasional or constant garbling of MIDI output, try

adjusting the CPU speed compensation in the driver.  Run Setup as

described above and select "MIDI Interface Settings".  By default, this

parameter is set up to auto-set itself, but this automatic method does not

work for some computers.  Start at 1 and try higher and higher values till

it works, then add 1 or 2 to make sure you'll avoid even a rare glitch.

If it's set too low, MIDI output will be garbled.  If it's set too high,

it will unnecessarily rob performance from your computer.  The faster the

machine, the higher the value required.  One 20Mhz 386 that we know of

uses a value of 21.  If you change your CPU speed, serial port, or

MIDIator, this value might be affected.

If after this you still notice garbling of MIDI output, there may be a

problem with the serial port in your computer.  Even if your serial port

works fine for other software, the speed or hardware timing may be off

enough that it won't work with the Key MIDIator, which requires exact

conformance.  Call Key technical support at 817-560-9751.  They will

provide you with a way to test your serial port with the Key interface and

get it working if possible.  Please note that Key is much better equipped

to help you with this kind of hardware problem than we are, since this is

not a problem with Cakewalk.

=====( AdLib and Sound Blaster Notes )======================================

Twelve Tone Systems supplies several drivers that use the FM sound

capability that is identical on the AdLib and Sound Blaster cards.

VOICE MODES: These cards have two modes of operation.  In the default

mode, you can play up to 9 instrumental notes simultaneously.  The special

percussion mode sacrifices some of these instrument notes to create

percussion sounds: in this mode, you get 5 percussion sounds on MIDI

channel 10, and only 6 instrument sounds on the other MIDI channels.

To put the card into percussion mode, use a patch change of 126 on MIDI

channel 10; to take it out of percussion mode, use patch change 127.

Probably you will want certain song files to use one mode or the other,

depending on whether the song needs percussion.  You can use Cakewalk's

"Pat" parameter to set up the patch change of 126 or 127 on MIDI channel

10.  This patch change will get saved in your Cakewalk song file.  See the

manual for more information.

PATCHES (SOUNDS): Built into the driver are 128 patches (sounds) following

the "General MIDI" standard.  A list of General MIDI patch numbers and

names follows.

The driver can also load ".IBK" files.  The Sound Blaster .IBK file format

is a standard already used by other programs to save patches for the AdLib

and Sound Blaster FM sounds.  To use one of these .IBK files, put it in

the Cakewalk startup directory and name it ADLIB.IBK.  The driver will

find this file and use its patches instead of the built-in defaults.  (Note

that we provide GENMIDI.IBK, which is identical to the General MIDI

patches built into the driver.)

Pat  Name        Pat  Name         Pat  Name        Pat  Name

---  --------    ---  --------     ---  --------    ---  --------

0  ACGPIANO     32  ACOUBASS      64  SOPRANO      96  FX1

1  ACPIANO      33  ELECBASS      65  ALTOSAX      97  FX2

2  ELGPIANO     34  PICKBASS      66  TENORSAX     98  FX3

3  HONKTONK     35  FRETLESS      67  BARISAX      99  FX4

4  ELPIANO1     36  SLAPBASS      68  OBOE        100  FX5

5  ELPIANO2     37  WRDBASS       69  ENGLHORN    101  FX6

6  HARPSCHD     38  SYNBASS1      70  BASSOON     102  FX7

7  CLAVICHD     39  SYNBASS2      71  CLARINET    103  FX8

8  CELESTA      40  VIOLIN        72  PICCOLO     104  SITAR

9  GLOCK        41  VIOLA         73  FLUTE       105  BANJO

10  MUSICBOX     42  CELLO         74  RECORDER    106  SHAMISEN

11  VIBES        43  CONTRA        75  PIPES       107  KOTO

12  MARIMBA      44  STRINGS1      76  BOTTLE      108  KALIMBA

13  XYLOPHON     45  PIZZ          77  SHAKA       109  BAGPIPE

14  TUBEBELL     46  STRINGS2      78  WHISTLE     110  FIDDLE

15  PIANOBEL     47  TIMPANI       79  WOOD        111  ETHNIC

16  BARORGAN     48  EXCELSTR      80  LEAD1       112  HANDBELL

17  PRCORGAN     49  STRSECT       81  LEAD2       113  AGOGO

18  ROCKORGN     50  SYNSTR1       82  LEAD3       114  STEELDRM

19  PIPEORGN     51  SYNSTR2       83  LEAD4       115  WOODBLOC

20  REEDORGN     52  CHORUS        84  LEAD5       116  TAIDRUM

21  ACCORDN      53  WUZZLE1E      85  LEAD6       117  MELDRUM

22  HARMNICA     54  VOXSYNTH      86  LEAD7       118  SYNSNAR

23  TANGO        55  ORCHHIT       87  LEAD8       119  REVCYMB

24  NYLON        56  TRUMPET1      88  PAD1        120  CHICKEN

25  ACOUST       57  TROMB1        89  PAD2        121  BREATH

26  JAZZGTR      58  TUBA3         90  PAD3        122  SEASHORE

27  ELGUITAR     59  SOFTRUMP      91  PAD4        123  INSECTS

28  ELGMUTE      60  FRHORN1       92  PAD5        124  OINKEY

29  OVERDRIV     61  BRASSECT      93  PAD6        125  HELICPTR

30  HVYMETAL     62  SYNBRASS      94  PAD7        126  CROWD

31  DTGUITAR     63  BRASSOFT      95  PAD8        127  SHOT

The following table describes the percussion MIDI key assignments.  These

have been chosen to correspond as closely as possible with the General

MIDI percussion map.

Key  Perc Snd    Key  Perc Snd    Key  Perc Snd    Key  Perc Snd

---  ---------   ---  ---------   ---  ---------   ---  ---------

35  kick drum    47  tom          59  cymbal       71  high hat

36  kick drum    48  tom          60  tom          72  high hat

37  high hat     49  cymbal       61  tom          73  tom

38  snare        50  tom          62  tom          74  high hat

39  high hat     51  cymbal       63  tom          75  tom

40  snare        52  cymbal       64  tom          76  tom

41  kick drum    53  high hat     65  tom          77  tom

42  high hat     54  high hat     66  tom          78  high hat

43  kick drum    55  cymbal       67  tom          79  high hat

44  high hat     56  high hat     68  tom          80  high hat

45  tom          57  cymbal       69  high hat     81  cymbal

46  high hat     58  snare        70  high hat

Sound Blaster owners: Twelve Tone Systems also supplies a driver that

supports the Sound Blaster MIDI interface option.  But note that your

Sound Blaster must have DSP ROM revision 2.0 or later, otherwise Cakewalk

will not find the MIDI interface.  Contact Creative Labs, Inc. to get this

ROM update.

===( Using Music Quest MQX Chase Lock Sync (CLS) )=========================

When using MQX Chase Lock Sync (CLS) you must run the Cakewalk Setup each

time you want to switch between two modes.  One mode is for striping a tape

(which you do infrequently).  The other mode is for playing in sync to a

striped tape (which is what you'll do most of the time).  So, you'll normally

leave Cakewalk configured to SYNC to CLS.  Whenever you need to stripe a new

tape, you need to run Setup to select STRIPING mode, stripe the tape, then

run Setup again to leave the mode set back to SYNC to CLS.  Following are

step-by-step instructions for each of these two procedures.

Note:| If you use SMPTE sync with Cakewalk Professional, none of this

| applies to you.

-----( Striping Chase Lock Sync )-----

When you wish to stripe a tape with CLS, you'll need to run the Setup

option to configure the driver and MQX card for striping mode:

-1- Start Cakewalk with the " -S" flag to run Setup.

-2- Pick the "MIDI Interface" choice.

-3- Set the sync choice "Stripe CLS", and press ENTER to go back to

the main setup menu.

-4- If you'll finish all your striping during this one Cakewalk session,

it will be easiest NOT to save this configuration.  Don't press "S" to save,

just press "C" to run Cakewalk with these new settings.

-5- Follow the normal procedure for striping a tape.  Here's a quick

summary of the information you should already be familiar with from the

Cakewalk and MQX Card manuals:

Connect an audio cable from the card's Tape Out jack to your tape

recorder's input.  Don't use noise reduction on the track with CLS.

Load your work file, and set Settings/Clock to "Internal".  Start the tape

recording, wait a few seconds, and then play your work from start to finish.

-----( Syncing to Chase Lock Sync )-----

Note:| Normally you'll need to do steps (1)-(5) only when you've just

| been striping a tape, and now want to switch the mode to

| syncing.  If you've already switched the mode to syncing to CLS

| and saved it, you won't need to run Setup each time.

-1- Start Cakewalk with the " -S" flag to run Setup.

-2- Pick the "MIDI Interface" choice.

-3- Set the sync choice "Sync to CLS", and press ENTER to go back

to the main Setup menu.

-4- Press "S" to save these settings. in the driver file.  You'll probably

want to do this because most of the time you'll be syncing to CLS, not

striping a tape with CLS.

-5- Press "C" to run Cakewalk now with these new settings.

-6- Follow the normal procedure for syncing to CLS.  Here's a quick summary

of the information you should already be familiar with from the Cakewalk

and MQX Card manuals:

Connect an audio cable from your tape recorder's sync output to the card's

Tape In jack.  Don't use noise reduction on the track with CLS.

Load your work file.  Set Settings/Clock to "MIDI Sync", if you haven't

already done so for this file.

Set Cakewalk to record or play (use Chase Mode if you like), and start

your tape playing wherever you like.  Cakewalk will chase to the right

location and begin record/playback.

=====( Warning About SCSI Drives )==========================================

Some SCSI hard drive controller cards use IRQ 2.  Do not attempt to use a

MIDI interface set to IRQ 2 with such a hard drive: loss of data may

result.

If you are not sure what kind of hard drive controller you have, then you

*probably* don't have a SCSI controller.  They generally cost more than

ST-506 or ESDI controllers and usually don't come as standard equipment.

---------------------

=======================( ADDITIONS/CORRECTIONS )===========================

---------------------

===( Disabling MIDI Metronome )============================================

To disable the MIDI metronome, you put a zero in the *Velocity* field, NOT

the Duration field as some editions of the manual say.

===( -D flag and Sysx Meta-Events )========================================

System Exclusive banks can be transmitted three ways in Cakewalk:

* You press the "Send" button in "View/System Exclusive".

* You set a bank as "Auto", so that whenever the file is

loaded, the bank is automatically sent.

* You insert a "Sysx" event in the Event View which triggers

transmission of the bank at that time during playback.

The third method has some restrictions.  As the manual notes, these

messages can't be very long, because MIDI doesn't allow much else to

happen while a sysx message is in progress.  For similar reasons, the "-D"

command line flag -- which slows down transmission of system exclusive

data -- has no effect on banks which are sent as "Sysx" meta-events during

playback.  Instead, Cakewalk sends the message as fast as possible so as

to have the least-disruptive effect on playback.  This means, however,

that if one of your instruments can't receive sysx at the full MIDI

bandwidth, you won't be able to send it sysx during playback.  You'll need

to use one of the first two methods.

=====( Problems fixed in 4.0A )============================================

If currently have Cakewalk 4.0 (with no final letter), you should make

sure you install this maintenance update, because it eliminates problems

that some 4.0 users have had (including one where playback occasionally

froze).  These problems were fixed as of 4.0A.

=====( Receiving system exclusive data with slow PC's )====================

As of 4.0B, we increased the efficiency of Sysx/Receive to help prevent

slow PC's (8088 4.77Mhz) from falling behind when receiving very large

system exclusive messages at the full MIDI rate.

=====( Loading MIDI Files that are not 120 PPQ )===========================

A problem converting times of Standard MIDI Files with timebases other

than 120 PPQ was fixed in 4.0C.

=====( DOS 5.0, Tandy )====================================================

Starting with 4.0C, we have changed the method used to compress our

executable programs so they fit on the diskette.  This should allow

DOS 5.0 users to avoid the "Packed file is corrupt" error message

without using LOADFIX.  Also, some users owning Tandy machines have

complained of extremely slow start-up and other problems, which may

be fixed by this change.

=====( Using the '?' wildcard in Edit/Interpolate )========================

One other problem fixed in 4.0C had to do with the Edit/Interpolate

function.  In the manual, we explain how you can use the '?' wildcard

to change all E's to Eb's.  This stopped working in Cakewalk 4.0 due

to a problem with a new compiler version, but we have worked around the

problem in this release.

=====( Disabling the Metronome PC Speaker Beep )===========================

As of 4.0C, it is now possible to prevent Cakewalk's metronome from beeping

at the PC speaker.  Simply add the line "MetroBeep=0" to your 12TONE.INI in

the [Cakewalk] section.

=====( Simultaneous Sysx Send and MIDI Thru )==============================

As of 4.0D, there are no longer any problems associated with playing on

your MIDI keyboard during Auto-send system exclusive transmission.

[][][]===============================================================[][][]

Please remember to return your product Registration Card within 30 days of

purchase.  Doing so qualifies you for technical support, upgrades,

newsletters, offers on MIDI products available only to registered

users...and more!

****************************************************************************

End of README.DOC

****************************************************************************