IBM STREAMER FAMILY ODI NETWARE LAN DRIVER, VERSION 1.19

________________________________________________________

About This File  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  2

Supported Platforms  . . . . . . . . . . . . . . . . . . . . . . . . . .  2

IBMMPCO LAN Driver Installation Procedure  . . . . . . . . . . . . . . .  3

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  3

Configurable IBMMPCO LAN Driver Parameters   . . . . . . . . . . . . . .  5

Sample IBMMPCO.CFG Configuration File for NetWare Server   . . . . . . .  7

IBMMPCO LAN Driver Performance Tuning  . . . . . . . . . . . . . . . . .  8

NetWare Server   . . . . . . . . . . . . . . . . . . . . . . . . . . .  8

IBM Streamer Family Adapter Custom Statistic Counters  . . . . . . . . .  9

Messages for the IBMMPCO LAN Driver  . . . . . . . . . . . . . . . . .   13

(C) Copyright IBM Corp. 1993                                                1

ABOUT THIS FILE

This file describes the use of the IBM Streamer Family ODI NetWare LAN Driver

(IBMMPCO.LAN) with any of the IBM Streamer Family Adapters listed below:

IBM LANStreamer MC 32 Adapter

IBM LANStreamer MC 16 Adapter

IBM EtherStreamer MC 32 Adapter

The term "IBM Streamer Family Adapter" is used in this document to refer to

any of the adapters listed above.  This file is an extension of the IBM

STREAMER FAMILY ADAPTER INSTALLATION AND TESTING INSTRUCTIONS that are

shipped with the respective adapter.

This file describes the installation and configuration of the IBM Streamer

Family Adapter Open Datalink Interface (ODI) NetWare IBMMPCO LAN driver.  It

contains information about the following topics:

o   Supported platforms

o   Software installation procedures

o   Configurable IBMMPCO LAN driver parameters

o   Performance tuning the IBMMPCO LAN driver

o   IBM Streamer Family Adapter Custom Statistic Counters

o   Messages for the IBMMPCO LAN driver

SUPPORTED PLATFORMS

The IBMMPCO LAN driver is supported in any of the hardware environments

supported by the IBM Streamer Family Adapters.  See the IBM STREAMER FAMILY

ADAPTER INSTALLATION AND TESTING INSTRUCTIONS for information about supported

hardware.

The following operating system environments are supported:

o   Novell NetWare 3.1x

o   Novell NetWare 4.0

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  2

IBMMPCO LAN DRIVER INSTALLATION PROCEDURE

This section describes how to install the IBMMPCO LAN driver on the NetWare

Server platform.

See the IBM STREAMER FAMILY ADAPTER INSTALLATION AND TESTING INSTRUCTIONS

that is shipped with the adapter for instructions on how to install the IBM

Streamer Family Adapter.

NetWare Server

This section describes how to install the IBMMPCO LAN driver on a NetWare

Server.

To install the IBMMPCO LAN driver on a NetWare Server, perform the following

steps:

1.  Choose a target directory path on the diskette or the fixed disk (for

example, C:\NW) for installation of the IBMMPCO LAN driver and its

associated files.  All files (in step 2, below) should be copied to this

target directory.

2.  Copy the following files to the target directory:

o   PATCHMAN.NLM

o   LSLENH.NLM

o   IBMMPCO.LAN

o   MSM.NLM

o   MSM31X.NLM

o   TOKENTSM.NLM

o   ETHERTSM.NLM

3.  Add the following lines to the STARTUP.NCF file:

Set Minimum Packet Receive Buffers = 48

NOTES:

1.  If an IBM Streamer Family Adapter is installed in a 16-bit

slot in your computer or the adapter is a 16-bit adapter, add

Set Reserved Buffers Below 16 Meg = 48

2.  If more than one IBM Streamer Family Adapter is installed in your

computer, increase the value for the above paramters

by 48 times the number of installed IBM Streamer Family Adapters.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  3

4.  Add the following lines to the AUTOEXEC.NCF file:

LOAD C:\NW\PATCHMAN.NLM

LOAD C:\NW\LSLENH.NLM

LOAD C:\NW\IBMMPCO.LAN @IBMMPCO.CFG

NOTES:

a.  @IBMMPCO.CFG is an optional command line parameter (see step 5,

below).

b.  IBMMPCO.LAN automatically loads the MSM.NLM (MSM31X.NLM if running

Novell NetWare 3.1x) TOKENTSM.NLM and ETHERTSM.NLM modules.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information about the

AUTOEXEC.NCF and STARTUP.NCF files.

5.  If you need to change any IBMMPCO LAN driver parameter values from their

defaults, these parameters can be specified on the command line when the

IBMMPCO LAN driver is loaded, or they can be specified by creating an

IBMMPCO.CFG configuration file.  If the optional @IBMMPCO.CFG command

line parameter is used to specify an IBMMPCO.CFG configuration file, then

the configuration file must be located in the same directory as the

IBMMPCO.LAN file.  You can use any text editor to create or edit the

IBMMPCO.CFG file.  See "Sample IBMMPCO.CFG Configuration File for NetWare

Server" for a sample IBMMPCO.CFG file.

6.  For your convenience, a copy of the LSLENH.NLM, PATCHMAN, MSM.NLM,

MSM31X.NLM, TOKENTSM.NLM, ETHERTSM.NLM, and MONITOR.NLM have been shipped

on the adapter options diskette.  Please contact Novell to ensure you

have the latest copy of these files.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  4

CONFIGURABLE IBMMPCO LAN DRIVER PARAMETERS

The following parameters can be configured at the NetWare Server by setting

keywords on the command line or in the IBMMPCO.CFG file.  A sample

IBMMPCO.CFG file is shown in "Sample IBMMPCO.CFG Configuration File for

NetWare Server."

SLOT=

This keyword defines the slot number of the corresponding IBM Streamer

Family Adapter in the computer.  To avoid being prompted to enter a

slot number when the IBMMPCO LAN driver is loaded, specify this

keyword in the AUTOEXEC.NCF file.  This keyword must be within the

range of 1-8.

FRAME=

This keyword defines the frame type to be used with the IBM Streamer

Family Adapter when loading the IBMMPCO LAN driver.

For token-ring IBM Streamer Family Adapters the default value for this

keyword is TOKEN-RING.  The value for this keyword must be one of the

following ascii strings:

o   TOKEN-RING

o   TOKEN-RING_SNAP

For ethernet IBM Streamer Family Adapters the default value for this

keyword is is ETHERNET_802.2 on versions of NetWare later than 3.11

and ETHERNET_802.3 for NetWare 3.11 and earlier.  The value for this

keyword must be one of the following ascii strings:

o   ETHERNET_802.2

o   ETHERNET_802.3

o   ETHERNET_II

o   ETHERNET_SNAP

NODE=

This keyword is used to set a locally administered address for the IBM

Streamer Family Adapter.  A local address must contain 12 hexadecimal

characters and can be specified in either MSB (Most Significant Bit)

or LSB (Least Significant Bit) format.

To specify a MSB node address the letter "M" immediately follows the

last digit of the node address.  If an "M" or "L" is not specified

after the last digit of the node address, the default format for the

address is MSB for token-ring and LSB for ethernet IBM Streamer Family

Adapters.  This keyword is specified on the command line or in the

IBMMPCO.CFG as NODE for the NetWare Server.  See "Sample IBMMPCO.CFG

Configuration File for NetWare Server" for a sample IBMMPCO.CFG file.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  5

If a node address is specified in MSB format, the characters must be

within the range 400000000000-7FFFFFFFFFFF (for example,

400000000001M).  This same address specified in LSB format would be

020000000080L with an "L" optionally appended to the end.

NOTES:

1.  Do not assign the same local address to 2 or more stations.

Severe station communication or network failures may occur when 2

or more stations are assigned the same address.

2.  If you do not assign a locally administered address, the adapter

uses the universally administered address permanently embedded on

the adapter.

RXBUFFERS=

This keyword defines the number of receive buffers used by the IBMMPCO

LAN driver.

The value for this keyword must be: 4, 8, 16, 32, or 64; the default

is 32.  (Receive buffer size is defined by the server SET command.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information.)

TXBUFFERS=

This keyword defines the number of transmit buffers used by the

IBMMPCO LAN driver.

The value for this keyword must be: 1, 2, 4, 8, or 16; the default is

16. (Maximum transmit frame size is defined by the server SET command.

See NOVELL NETWARE SYSTEM ADMINISTRATION for more information.)

ENET20UTP

This keyword will enable the IBM EtherStreamer MC 32 Adapter to

operate in full duplex mode.  This keyword is only valid if the

adapter is connected to a switched ethernet network with UTP cable.

Full duplex mode enables the IBM EtherStreamer MC 32 Adapter transmit

and receive channels to simultaneously operate at 10 Mbps.

TXINTSOFF

This keyword will reduce CPU utilization in server environments when

using the standard IPX protocol.  Reducing CPU utilization (shown on

the monitor screen) may improve performance if CPU utilization is very

high (above 80% utilized).  This option should not be used if Novell's

packet burst protocol (PBURST.NLM) is loaded, as performance on packet

burst links may be significantly degraded.  Use of this option is

recommended only for servers using the standard IPX protocol.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  6

SAMPLE IBMMPCO.CFG CONFIGURATION FILE FOR NETWARE SERVER

; The following keywords are generic ODI keywords for IBMMPCO

;  when using the IBM LANStreamer MC Adapter:

;

NODE=400000002222M

FRAME=TOKEN-RING_SNAP

; The following keywords are generic ODI keywords for &frame.

;  when using the IBM EtherStreamer MC 32 Adapter:

;

NODE=020000000001L

SLOT=2

FRAME=ETHERNET_802.2

TXBUFFERS=16

RXBUFFERS=32

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  7

IBMMPCO LAN DRIVER PERFORMANCE TUNING

This section describes how to get the best performance from the IBM Streamer

Family Adapter and IBMMPCO LAN driver.

NetWare Server

The maximum frame size that can be sent or received by a token-ring IBM

Streamer Family Adapter configured at a 4-Mbps data rate is 4494 bytes and at

a 16-Mbps data rate is 17954 bytes.  The maximum frame size that can be sent

or received by an ethernet IBM Streamer Family Adapter is 1518.

Since frame sizes are typically negotiated between the NetWare server and

workstations, it is best to configure receive buffer sizes at the server that

equal the maximum transmit buffer size at workstations.  For this reason,

increasing your receive buffer size to a value that is too big only increases

the amount of memory that is used by NetWare and may not increase the

performance of the IBM Streamer Family Adapter or NetWare.

Since the IBMMPCO.LAN driver uses NetWare receive buffers to receive frames,

the size of receive buffers used by the IBMMPCO.LAN driver is configurable by

changing the size of the NetWare receive buffers.  To obtain the best

performance from the IBM Streamer Family Adapter edit the STARTUP.NCF file

and modify the following NetWare parameter according to your configuration as

indicated below:

If a token-ring and/or ethernet IBM Streamer Family Adapter is installed

in your NetWare Server increase the following parameter as shown

below:

Set Maximum Physical Receive Packet Size = 4202

If only an ethernet IBM Streamer Family Adapter is installed in your

NetWare Server increase the following parameter as shown

below:

Set Maximum Physical Receive Packet Size = 1518

If the above parameter is not found in the STARTUP.NCF, use the NetWare set

command to view system parameters and ensure that the default for this

parameter is greater than the values approriate for your configuration.  If

it is less, specify the above parameter in the STARTUP.NCF.  See

"Configurable IBMMPCO LAN Driver Parameters" regarding the TXINTSOFF keyword

that may improve performance of the NetWare server.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  8

IBM STREAMER FAMILY ADAPTER CUSTOM STATISTIC COUNTERS

The custom statistic counters give the user information that is specific to

the IBM Streamer Family Adapter and IBMMPCO LAN driver.  There follow

descriptions of the custom statistic counters and what action, if any, you

should take based on these counters.

Token-ring Utilization Percentage:

DESCRIPTION:  This counter indicates the percentage of the token-ring

bandwidth that is used.  This counter is updated every 30

seconds.

CAUSE/ACTION: None.

MAC Frame Count:

DESCRIPTION:  This counter indicates the number of token-ring Media Access

Control (MAC) frames sent by the IBMMPCO driver.

CAUSE/ACTION: None.  These MAC frames were built and requested to be sent

by a protocol stack.  This counter is only valid for

token-ring IBM Streamer Family Adapters.

Receive Collisions:

DESCRIPTION:  This counter indicates the number of frames that experienced

a collision while being received by an ethernet IBM Streamer

Family Adapter.

CAUSE/ACTION: If this counter steadily increases, it may indicate a

problem with your ethernet IBM Streamer Family Adapter or

your ethernet network.  Run the adapter diagnostic tests

located on the Option Diskette that were shipped with the

IBM Streamer Family Adapter.  If there are no problems with

the adapter, reload the IBMMPCO LAN driver.  If this problem

persists, contact your network administrator.

Receive Frames Discarded:

DESCRIPTION:  This counter indicates the number of frames that were

received by the IBM Streamer Family Adapter but were

discarded by the IBMMPCO LAN driver because they were

received with an error.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  9

CAUSE/ACTION: If this counter steadily increases, it may indicate a

problem with your computer or the IBM Streamer Family

Adapter.  Run the adapter diagnostic tests located on the

Option Diskette that were shipped with the IBM Streamer

Family Adapter.  If there are no problems with the adapter,

reload the IBMMPCO LAN driver.  If this problem persists,

contact your network administrator.

Transmit Frames Discarded:

DESCRIPTION:  This counter indicates the number of frames that could not

be transmitted by the IBM Streamer Family Adapter and were

discarded by the IBMMPCO LAN driver.

CAUSE/ACTION: Possible explanations for this problem are as follows:

The IBM Streamer Family Adapter is not responding.  Run the

adapter diagnostic tests located on the Option Diskette that

was shipped with the IBM Streamer Family Adapter.  If there

are no problems with the adapter, reload the IBMMPCO driver.

If a problem was found, follow the instructions supplied

with the diagnostic tests to correct the problem.

If your IBM Streamer Family Adapter is connected to a

token-ring network, the token-ring network may be

experiencing problems and, therefore, the IBM Streamer

Family Adapter could not transmit these frames.  If this

problem persists, contact your network administrator.

Adapter Hardware Errors:

DESCRIPTION:  This counter indicates the number of hardware errors

reported by the IBM Streamer Family Adapter.

CAUSE/ACTION: The IBM Streamer Family Adapter and IBMMPCO LAN driver may

recover from these errors.

If this counter continues to increase, Run the adapter

diagnostic tests located on the Option Diskette that was

shipped with the IBM Streamer Family Adapter.  If there are

no problems with the adapter, reload the IBMMPCO LAN driver.

If a problem was found, follow the instructions supplied

with the diagnostic tests to correct the problem.

Adapter Detected Parity Errors:

DESCRIPTION:  This counter is incremented when the IBM Streamer Family

Adapter detects a parity error while data is being

transferred between the IBM Streamer Family Adapter and the

computer.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  10

CAUSE/ACTION: If this counter continues to increase, it may indicate a

problem with your computer or the IBM Streamer Family

Adapter.  Run the adapter diagnostic tests located on the

Option Diskette that was shipped with the IBM Streamer

Family Adapter.  If there are no problems with the adapter

reload the IBMMPCO LAN driver.  If a problem was found,

follow the instructions supplied with the diagnostic tests

to correct the problem.

Adapter RAM Access Errors:

DESCRIPTION:  This counter is incremented when the IBM Streamer Family

Adapter reports a RAM access violation.

CAUSE/ACTION: If this counter continues to increase, and you are

experiencing problems with the IBM Streamer Family Adapter,

run the adapter diagnostic tests located on the Option

Diskette that was shipped with the IBM Streamer Family

Adapter.  If there are no problems with the adapter, reload

the IBMMPCO LAN driver.  If this problem still exists,

contact your network administrator.

Micro Channel Errors:

DESCRIPTION:  This counter indicates the number of Micro Channel bus

errors that were detected.

CAUSE/ACTION: The IBM Streamer Family Adapter detected an error with the

Micro Channel bus while accessing or attempting to access

the bus.

If this counter continues to increase, run the diagnostics

on the reference diskette that came with your computer to

test for problems with the Micro Channel bus.

Receive Hardware Mismatch Errors:

DESCRIPTION:  This counter is incremented when the IBMMPCO LAN driver

detects a difference between the hardware reported frame

length and the frame length field in the IPX, IP, ARP or

RARP protocol header.

CAUSE/ACTION: If this counter continues to increase, contact your network

administrator or IBM support center.

Receive Overrun Errors:

DESCRIPTION:  This counter is incremented when the IBM Streamer Family

Adapter receives data from the network faster than it can be

moved into computer memory.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  11

CAUSE/ACTION: If this counter continues to increase, run the diagnostics

on the reference diskette that came with your computer to

test for problems with the system bus.  If this problem

still persists, contact your network administrator.

Transmit Underrun Errors:

DESCRIPTION:  This counter is incremented when the IBM Streamer Family

Adapter attempts to transmit a frame to the network but

cannot move the data fast enough from computer memory to the

IBM Streamer Family Adapter.

CAUSE/ACTION: If this counter continues to increase, run the diagnostics

on the reference diskette that came with your computer to

test for problems with the system bus.  If this problem

still persists, contact your network administrator.

Transmit Checksum Errors:

DESCRIPTION:  This counter is incremented when the transmission of a frame

completes and the checksum value for the frame is incorrect.

CAUSE/ACTION: If this counter continues to increase, contact your network

administrator.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  12

MESSAGES FOR THE IBMMPCO LAN DRIVER

IBMMPCO-xx-071: The matching virtual adapter could not be found.

Cause:  An error occurred while trying to load another copy

of the IBMMPCO LAN driver to support an additional

frame type (FRAME= parameter) on an

IBM Streamer Family Adapter.  The MSM could not find

a previous copy of the IBMMPCO LAN driver required

to load the specified frame type.

Action: Obtain the most current copy of the MSM.NLM or

MSM31X.NLM (if running NetWare version 3.1x), and then

reload the IBMMPCO LAN driver for the desired frame

types.  If this error persists, contact your

network administrator.  Refer to the

NOVELL NETWARE SYSTEM ADMINISTRATION manual

for information about loading and unloading LAN

drivers.

IBMMPCO-xx-072: A resource tag is unavailable.

Cause:  The NetWare operating system was unable to allocate

memory for a resource tag.  The IBMMPCO LAN driver

did not load because required internal resources

were not available.  There may be too many NetWare

Loadable Modules (NLMs) running in the NetWare Server.

Action: Unload the NLMs that are loaded but are not being used,

and then load the IBMMPCO LAN driver.  If this error

persists, down, exit and then restart the server.  Load

the IBMMPCO LAN driver before loading NLMs.  This may

require you to edit the AUTOEXEC.NCF file before restarting

the server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION

for information about the AUTOEXEC.NCF file.

IBMMPCO-xx-073: Unable to allocate memory.

Cause:  The NetWare operating system was unable to allocate

memory for the IBMMPCO LAN driver.  The IBMMPCO

LAN driver did not load.

Action: Unload the NetWare Loadable Modules (NLMs) that are

loaded but are not being used, and then load the IBMMPCO

LAN driver.  If this error persists, down, exit and then

restart the server.  Load the IBMMPCO LAN driver before

loading NLMs.  This may require you to edit the

AUTOEXEC.NCF file before restarting the server.  Refer to

NOVELL NETWARE SYSTEM ADMINISTRATION for information

about the AUTOEXEC.NCF file.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  13

IBMMPCO-xx-074: The hardware interrupt cannot be set.

Cause:  The NetWare operating system was unable to set the

hardware interrupt level requested by the IBMMPCO

LAN driver.  The IBMMPCO LAN driver did not load.

Action: Try to load the IBMMPCO LAN driver again.  If the error

persists, contact your network administrator.

IBMMPCO-xx-075: The MLID cannot be registered with the LSL.

Cause:  The IBMMPCO LAN driver could not be registered with the

Link Support Layer (LSL) module.  The maximum number of

LAN drivers have already been loaded or there was not

enough memory to load the IBMMPCO LAN driver.  The IBMMPCO

LAN driver did not load.

Action: Unload LAN drivers that are not being used and then

load the IBMMPCO LAN driver.  If this error persists,

down, exit and then restart the server.  Load the IBMMPCO

LAN driver before loading other LAN drivers.  This may

require you to edit the AUTOEXEC.NCF file before restarting

the server.  Refer to NOVELL NETWARE SYSTEM ADMINISTRATION

for information about the AUTOEXEC.NCF file.

IBMMPCO-xx-076: The polling procedure cannot be added.

Cause:  Too many polling processes are registered with

the NetWare operating system.  The IBMMPCO LAN

driver did not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

IBMMPCO LAN driver again.  If this error persists,

contact your network administrator.

IBMMPCO-xx-077: The event notification routine cannot be registered.

Cause:  The NetWare operating system was unable to allocate

memory to register the IBMMPCO LAN driver event

notification routine.  The IBMMPCO LAN driver did

not load.

Action: Unload LAN drivers or NetWare Loadable Modules that

are loaded but are not being used.  Try to load the

IBMMPCO LAN driver again.  If this error persists,

contact your network administrator.

IBMMPCO-xx-078: The firmware file cannot be read.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  14

Cause:  The Netware operating system was unable to load the

IBMMPCO LAN driver.  The IBMMPCO LAN driver was not

loaded.

Action: Obtain another copy of the IBMMPCO LAN driver from

the original distribution diskette.  Reload the IBMMPCO

LAN driver.  If this error persists, contact your network

administrator.

IBMMPCO-xx-084: Unable to allocate memory below 16 Megabyte boundary.

Cause:  The Netware operating system ran out of memory buffers

below 16 megabytes.  The IBMMPCO LAN driver was not

loaded.

Action: Set the Reserved Buffers Below 16 Meg parameter in the

STARTUP.NCF to a larger number.

IBMMPCO-xx-086: The driver parameter block is too small.

Cause:  The IBMMPCO.LAN file is designed to run with an older

version of the MSM.NLM.  The IBMMPCO LAN driver was

not loaded.

Action: Obtain an older version of the MSM.NLM or a newer

version of the IBMMPCO.LAN file.

IBMMPCO-xx-087: The media parameter block is too small.

Cause:  The TOKENTSM.NLM NetWare Loadable Module (NLM) is

designed to run with an older version of the MSM.NLM

or MSM31X.NLM if running NetWare Version 3.1x.

Action: Obtain an older version of the MSM.NLM (MSM31X.NLM

if running NetWare Version 3.1x) or a new version of

the TOKENTSM.NLM.

IBMMPCO-xx-091: The hardware configuration conflicts.

Cause:  The IBMMPCO LAN driver requested use of a hardware

resource that was already being used by another

hardware device.  The IBMMPCO LAN driver did not load.

Action: Change the hardware configuration of the

IBM Streamer Family Adapter or the hardware

configuration of the conflicting hardware device.

IBMMPCO-xx-092: Cannot schedule AES without an HSM routine.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  15

Cause:  The IBMMPCO.LAN file is damaged.

Action: Obtain another copy of the IBMMPCO LAN driver from

the original distribution diskette.  Reload the IBMMPCO

LAN driver.  If this error persists, contact your network

administrator.

IBMMPCO-xx-093: Cannot schedule interrupt time call back without an

HSM routine.

Cause:  The IBMMPCO.LAN file is damaged.

Action: Obtain another copy of the IBMMPCO LAN driver from

the original distribution diskette.  Reload the IBMMPCO

LAN driver.  If this error persists, contact your network

administrator.

IBMMPCO-xx-094: Cannot set hardware interrupt without an HSM routine.

Cause:  The IBMMPCO.LAN file is damaged.

Action: Obtain another copy of the IBMMPCO LAN driver from

the original distribution diskette.  Reload the IBMMPCO

LAN driver.  If this error persists, contact you network

network administrator.

IBMMPCO-xx-095: Cannot add polling without an HSM routine.

Cause:  The IBMMPCO.LAN file is damaged.

Action: Obtain another copy of the IBMMPCO LAN driver from

the original distribution diskette.  Reload the IBMMPCO

LAN driver.  If this error persists, contact your network

administrator.

IBMMPCO-xx-126: The group bit in the node address override was cleared.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The group

address bit for the node address specified had been

set.  The group address bit was cleared by the IBMMPCO

LAN driver.  The IBMMPCO LAN driver was loaded.

Action: Correct the node address parameter to specify a locally

administered address according to the IBM TOKEN-RING

NETWORK ARCHITECTURE REFERENCE.  On a DOS or

OS/2 workstation, change the node address parameter in

the NET.CFG file.  On a NetWare Server, change the node

address parameter specified on the command line when the

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  16

IBMMPCO LAN driver was loaded, or change the node address

specified in the IBMMPCO.CFG configuration file.

IBMMPCO-xx-127: The local bit in the node address override was set.

Cause:  The address specified for the node address parameter

is an incorrect adapter node address.  The node address

specified does not have the locally administered address

bit set.  The locally administered address bit was set

by the IBMMPCO LAN driver.  The IBMMPCO LAN driver was

loaded.

Action: Correct the node address parameter to specify a locally

administered address according to the IBM TOKEN-RING

ARCHITECTURE REFERENCE.  On a DOS or OS/2

workstation, change the node address parameter in the

NET.CFG file.  On a NetWare server change the node

address parameter specified on the command line when the

IBMMPCO LAN driver was loaded, or change the node address

specified in the IBMMPCO.CFG configuration file.

IBMMPCO-xx-200: Initializing the IBM Streamer Family Adapter ... please wait.

CAUSE: The IBM Streamer Family Adapter initialization process has

started.

ACTION: None.

IBMMPCO-xx-201: Connecting the IBM Streamer Family Adapter to the network.

CAUSE: The IBM Streamer Family Adapter is connecting to an ethernet

or token-ring network.

ACTION: None.

IBMMPCO-xx-202: The IBM Streamer Family Adapter is closed.

CAUSE: The IBMMPCO LAN driver could not connect the IBM Streamer

Family Adapter to the network due to previously reported

hardware problems.  The IBMMPCO LAN driver has closed the

adapter and has generated an alert after unsuccesfully trying

to connect the adapter to the network after several times.

ACTION: Follow the recommended action for correcting any error

messages displayed by the IBMMPCO LAN driver before this one.

IBMMPCO-xx-204: The IBM Streamer Family Adapter requires a computer with a

Micro Channel bus.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  17

CAUSE: The IBM Streamer Family Adapter will work only in a computer

that supports the IBM Micro Channel Architecture.

ACTION: Install the IBM Streamer Family Adapter in a computer that

supports the IBM Micro Channel Architecture.

IBMMPCO-xx-205: The IBMMPCO LAN driver was not loaded because of an

incorrect keyword value.

CAUSE: The value for one of the keywords specified on the command

line or in the IBMMPCO.CFG file was not within the allowable

range.  The IBMMPCO LAN driver did not load.

ACTION: Correct the value of the keyword displayed in the message

that was displayed above this error message, and reload the

IBMMPCO LAN driver.

IBMMPCO-xx-207: The IBM Streamer Family Adapter could not be reset.

CAUSE: The IBMMPCO LAN driver timed out while waiting for the IBM

Streamer Family Adapter to complete a hardware reset.  The

IBMMPCO LAN driver did not load.

ACTION: Restart your computer and try to load the IBMMPCO LAN driver

again.  If this error persists, contact your network

administrator.

IBMMPCO-xx-209: The IBMMPCO LAN driver failed to initialize the

IBM Streamer Family Adapter.

CAUSE: There was a problem while initializing the IBM Streamer

Family Adapter.  There may be previous error messages above

this one or the IBM Streamer Family Adapter may not be

responding.

ACTION:

Correct any problems with error messages displayed above this

error message, and then reload the IBMMPCO LAN driver.

If this is the only error message that is displayed, restart

your computer and check the adapter by running the adapter

diagnostic tests located on the Option Diskette that was

shipped with the IBM Streamer Family Adapter.  Then, reload

the IBMMPCO LAN driver.  If this error persists, contact your

network administrator.

IBMMPCO-xx-210: Could not find an IBM Streamer Family Adapter in this

computer.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  18

CAUSE: There is no IBM Streamer Family Adapter installed in this

computer.  The IBMMPCO LAN driver did not load.

ACTION: Install an IBM Streamer Family Adapter in this computer if it

has a 80386 or higher microprocessor, and then reload the

IBMMPCO LAN driver.

IBMMPCO-xx-211: The IBM Streamer Family Adapter was not found in the

specified slot.

CAUSE: There is no IBM Streamer Family Adapter in the slot specified

by the SLOT keyword.

ACTION: Correct the SLOT keyword so that it indicates a slot number

in which an IBM Streamer Family Adapter is located, and then

reload the IBMMPCO LAN driver.

If there is an IBM Streamer Family Adapter in the specified

slot, contact your network administrator.

IBMMPCO-xx-212: Insufficient memory available for configured transmit and

receive buffers.

CAUSE: During initialization of the IBMMPCO LAN driver, there was

insufficient memory available for the configured number of

transmit and receive buffers.

ACTION: Increase the number of buffers that the server operating

system can allocate by editing the STARTUP.NCF file on the

server.  Increase the Minimum Packet Receive Buffers server

parameter in the STARTUP.NCF file by the total number of

transmit and receive buffers that the IBMMPCO LAN driver is

using.  Down, exit, and then restart the NetWare Server.

If the IBM Streamer Family Adapter is installed in a 16-bit

slot in your computer add or increase the following parameter

in the STARTUP.NCF file:

Set the Reserved Buffers Below 16 Meg = total transmit and

receive buffers used by the IBMMPCO LAN driver.

If this error still persists, unload the LAN drivers and

NetWare Loadable Modules (NLMs) that are loaded but are not

being used at the NetWare Server.

If this error still persists, decrease the number of buffers

that are being used by the IBMMPCO LAN driver by specifying

the transmit and receive keywords for the IBMMPCO LAN driver.

See "Configurable IBMMPCO LAN Driver Parameters" for

information about specifying keywords for the operating

system that you are using.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  19

IBMMPCO-xx-213: The slot to use for the IBM Streamer Family Adapter

was not specified.

CAUSE: The IBMMPCO LAN driver prompted the user to select a slot

number that has an IBM Streamer Family Adapter installed but

no choice was made.

ACTION: None.  The IBMMPCO LAN driver was not loaded.

IBMMPCO-xx-215: The value specified for RXBUFFERS is not valid.

The value for RXBUFFERS has been changed to xx.

VARIABLE DATA: Valid RXBUFFERS values (xx) are 4, 8, 16, 32 or 64.

CAUSE: An incorrect value was specified for RXBUFFERS either on the

command line or in the IBMMPCO.CFG file for the server.  The

IBMMPCO LAN driver changed the value for RXBUFFERS to the

value given in this error message.

ACTION: None.  However, to keep this message from being displayed

when the IBMMPCO LAN driver is loaded, change the value

specified by the RXBUFFERS keyword to one of the supported

values displayed above.  See "Configurable IBMMPCO LAN Driver

Parameters" for the supported values.

IBMMPCO-xx-216: The value specified for TXBUFFERS is not valid.

The value for TXBUFFERS has been changed to xx.

VARIABLE DATA: Valid TXBUFFERS values (xx) are 1, 2, 4, 8, 16.

CAUSE: An incorrect value was specified for TXBUFFERS either on the

command line or in the IBMMPCO.CFG file for the server.  The

IBMMPCO LAN driver changed the value for TXBUFFERS to the

value given in this error message.

ACTION: None.  However, to keep this message from being displayed

when the IBMMPCO LAN driver is loaded, change the value

specified by the TXBUFFERS keyword to one of the supported

values for your operating system.  See "Configurable IBMMPCO

LAN Driver Parameters" for the supported values.

IBMMPCO-xx-219: The IBM Streamer Family Adapter reported an internal error

when it was opened.

CAUSE: The open adapter command issued by the IBMMPCO LAN driver to

connect to the netork failed.  An alert was generated by the

IBMMPCO LAN driver.

ACTION: Restart the computer and try to load the IBMMPCO LAN driver

again.  If this message continues to occur, check the adapter

by running the adapter diagnostic tests located on the Option

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  20

Diskette that was shipped with the IBM Streamer Family

Adapter.  If there is a problem with the adapter hardware,

contact your network administrator.  If there are no problems

with the adapter hardware, report this problem to your

support center.

IBMMPCO-xx-221: The IBM Streamer Family Adapter received a disconnect frame

from another adapter on the network.

CAUSE: The IBM Streamer Family Adapter received a disconnect frame

request that was sent by another adapter at a remote station.

The IBM Streamer Family Adapter installed in this computer

disconnected from the network.

ACTION: To connect the IBM Streamer Family Adapter back to the

network, unload the IBMMPCO LAN driver and then reload it.

If this error persists, contact your network administrator.

IBMMPCO-xx-223: The IBM Streamer Family Adapter failed to connect to the

network; another adapter is using the same node address.

CAUSE: The IBM Streamer Family Adapter could not connect to the

network because another station is using the same value that

was specified for the node address parameter at your station.

ACTION: Change the node address parameter value specified for your

station.  This parameter is specified as NODE= at the NetWare

Server.  See "Configurable IBMMPCO LAN Driver Parameters" for

more information about configuration parameters.

IBMMPCO-xx-224: The IBM Streamer Family Adapter detected a wire fault error

and closed the adapter.

CAUSE: The connection between the IBM Streamer Family Adapter and

the LAN has been broken.  The adapter or the adapter cable

may have developed a problem.

ACTION: Check the connection between the adapter and cable by running

the adapter diagnostic tests located on the Option Diskette

that was shipped with the IBM Streamer Family Adapter.

IBMMPCO-xx-225: The token-ring network is beaconing.

CAUSE: The token-ring LAN segment to which the IBM Streamer Family

Adapter is connected is beaconing.  The ring is experiencing

an error condition detected by the adapter when there is

either a signal loss (possibly caused by a broken line) or no

token circulating the ring within the predefined time limit.

ACTION: Contact your network administrator.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  21

IBMMPCO-xx-228: A Micro Channel error occurred; the adapter was reset.

CAUSE: An error was detected with the Micro Channel bus while data

was being transferred between the IBM Streamer Family Adapter

and the computer.  This condition is reported as an alert.

ACTION: The adapter will retry the data transfer, and the IBMMPCO LAN

driver Micro Channel Error statistics counter will be

incremented.

If this error persists, check the adapter by running the

adapter diagnostic tests located on the Option Diskette that

was shipped with the IBM EtherStreamer MC 32 Adapter, and

check your computer by running the hardware diagnostic tests.

If this error still persists, contact your support center or

place of purchase of the IBM Streamer Family Adapter.

IBMMPCO-xx-229: The IBM Streamer Family Adapter reported a hardware error.

CAUSE: The IBM Streamer Family Adapter reported an adapter hardware

error.  The adapter was closed.

ACTION: The IBMMPCO LAN driver will increment the Adapter Hardware

Error counter, issue an alert, and try to reset the adapter.

The IBM EtherStreamer MC 32 Adapter and IBMMPCO LAN driver

may recover from this error.  If not, run the adapter

diagnostic tests located on the Option Diskette that was

shipped with the IBM Streamer Family Adapter.  If there are

no problems with the adapter, reload the IBMMPCO LAN driver.

If a problem was found, follow the instructions supplied with

the diagnostic tests to correct the problem.

IBMMPCO-xx-231: The alert condition was corrected.

CAUSE: The IBM Streamer Family Adapter, IBMMPCO LAN driver, or

network has recovered from a previously reported alert

condition.

ACTION: None.

IBMMPCO-xx-232: The IBM Streamer Family Adapter detected a parity error.

CAUSE: The IBM Streamer Family Adapter detected a parity error while

data was being transferred between the IBM Streamer Family

Adapter and the computer.

ACTION: The IBMMPCO LAN driver will increment the Adapter Detected

Parity Error counter, and this condition is reported as an

alert.  The IBM Streamer Family Adapter and IBMMPCO LAN

driver may recover from this error.  If not, run the adapter

diagnostic tests located on the Option Diskette that was

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  22

shipped with the IBM Streamer Family Adapter.  If there are

no problems with the adapter, reload the IBMMPCO LAN driver.

If a problem was found, follow the instructions supplied with

the diagnostic tests to correct the problem.

IBMMPCO-xx-233: The IBM Streamer Family Adapter reported a RAM access

violation.

CAUSE: The IBM Streamer Family Adapter reported a RAM access

violation.  The IBMMPCO LAN driver may have tried to access

RAM on the IBM EtherStreamer MC 32 Adapter that was not

accessible.

ACTION: The IBMMPCO LAN driver will increment the Adapter RAM Access

Error counter, and this condition is reported as an alert.

The IBM Streamer Family Adapter and IBMMPCO LAN driver may

recover from this error.  If not, run the adapter diagnostic

tests located on the Option Diskette that was shipped with

the IBM Streamer Family Adapter.  If there are no problems

with the adapter, reload the IBMMPCO LAN driver.  If a

problem was found, follow the instructions supplied with the

diagnostic tests to correct the problem.

IBMMPCO-xx-237: Check the connection of the adapter cable to the

IBM Streamer Family Adapter.

CAUSE: The IBM Streamer Family Adapter detected an error when

connecting to the network  This error can occur if there is

no adapter cable connected to the IBM Streamer Family Adapter

or there is an error in the network cabling.

ACTION: Check the connection of the adapter cable to the adapter and

ethernet network.  If the IBM Streamer Family Adapter is

connected to the network by a coaxial cable, check with your

network administrator to make sure the network cabling is

properly terminated.

IBMMPCO-xx-238: Check the connection of the UTP adapter cable to the

IBM Streamer Family Adapter.

CAUSE: The IBM Streamer Family Adapter is configured to use the

unsheilded twisted pair (UTP) connector on the adapter.  A

UTP cable is not connected to the adapter UTP connector or

the UTP cable is not connected to the network.

ACTION: If there is no UTP cable connected to the IBM Streamer Family

Adapter UTP connector, connect one and then reload the

IBMMPCO LAN driver.  If there is already a UTP cable

connected to the IBM Streamer Family Adapter, check with your

network administrator to make sure that the UTP cable is

properly connected to the network.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  23

IBMMPCO-xx-239: Check the connection of the STP adapter cable to the

IBM Streamer Family Adapter.

CAUSE: The IBM Streamer Family Adapter is configured to use the

sheilded twisted pair (STP) connector on the adapter. A STP

cable is not connected to the adapter STP connector or the

STP cable is not connected to the network.

ACTION: If there is no STP cable connected to the IBM Streamer Family

Adapter STP connector, connect one and then reload the

IBMMPCO LAN driver.  If there is already a STP cable

connected to the IBM Streamer Family Adapter, check with your

network administrator to make sure that the STP cable is

properly connected to the network.

IBMMPCO-xx-240: Check the connection of the AUI adapter cable to the

IBM Streamer Family Adapter.

CAUSE: The IBM Streamer Family Adapter is configured to use the AUI

connector on the adapter.  An AUI adapter cable is not

connected to the adapter AUI connector or the AUI cable is

not connected to the network.

ACTION: If there is no AUI cable connected to the IBM Streamer Family

Adapter AUI connector, connect one and then reload the

IBMMPCO LAN driver.  If there is already a AUI cable

connected to the IBM Streamer Family Adapter, check with your

network administrator to make sure that the STP cable is

properly connected to the network.

IBMMPCO-xx-241: Check the connection of the coax adapter cable to the

IBM Streamer Family Adapter.

CAUSE: The IBM EtherStreamer MC 32 Adapter is configured to use the

coax connector on the adapter.  A coax adapter cable is not

connected to the adapter coax connector or the coax cable is

not connected to the network.

ACTION: If there is no coax cable connected to the IBM EtherStreamer

MC 32 Adapter coax connector, connect one and then reload the

IBMMPCO LAN driver.  If there is already a coax cable

connected to the IBM EtherStreamer MC 32 Adapter, check with

your network administrator to make sure that the coax cable

is properly connected to the network.

IBM Streamer Family ODI NetWare LAN Driver, Version 1.19  24