TECHNICAL INFORMATION FOR PROGRAMMERS

-------------------------------------

2M access always directly to NEC765 diskette controller chip and 8237

DMA controller. But you haven't to do the same!.  The standard DOS and BIOS

functions still works when 2M is installed, supporting new diskette formats

as standard ones.  If you need to format a 2M diskette from your high-level

applications, this is possible also via standard INT 13h function 5.

When 2M is installed on memory,  INT 13h  service 5 becomes increased

its functionality. To find 2M resident on memory:

Load AL with 0.

Load ES:DI with 1492h:1992h.

Load AH with a value from 0C0h to 0FFh.

Call INT 2Fh.

When INT 2Fh returns, if AL=0FFh and ES:DI points a "CiriSOFT:2M:2.1"

null terminated string...  2M is in memory!.   Note that ':' is a delimiter

and not always CiriSOFT will be  the  manufacturer  (I recomend search only

for the ":2M:" and ":2MX:" substrings). You can begin scanning with AH=0C0h

and, if not found 2M or 2MX there,  continue with AH=0C1h and so on.  If 2M

has been loaded from booting a 2M disk,  format functions are not available

(and 2M installation check will correctly fail).

When you had detected 2M on memory,  you can call to INT 13h function

5 to format a diskette track.  This  is easiest than standard BIOS service:

you don't need to select the disk density,  media,  or  last  sector in the

track. Of course, the standard BIOS function is still available but you can

call INT 13h as bellow:

Load AH with 5.

Load AL with 7Fh.

Load CH with track number.

Load DH with head number.

Load DL with drive number.

Load SI with 324Dh signature ("2M").

Load ES:BX pointing to boot sector of future 2M diskette.

Note that in standard BIOS call, AL sets the number of sectors (never

is 7Fh) and SI is not necesary equal to 324Dh.  This lets 2M to detect your

intention. You have not to fill big tables on ES:BX, simply point to a boot

sector  of  future diskette. This boot sector can be obtained from standard

2M diskette already formatted. New INT 13h format function return the sames

codes in AH that the BIOS standard routines.  Since 2.0 version of 2M,  the

track 0  head 0 contains a virtual BOOT sector in the first physical sector

of FAT2, and in high and extra-high density diskettes the SuperBOOT code in

the second-sixth sectors of FAT2.  The FAT2 is emulated using FAT1 instead.

The real BOOT sector is never shown in BIOS normal functions, and SuperBOOT

code also can not be accessed through BIOS standard functions.  The virtual

boot sector is equal to real one when 2MF formats a diskette,  but  can  be

altered by SYS command, for example. If you desire to format a 2M diskette,

and you want to make this diskette capable of SuperBOOT, you must write the

real BOOT sector, the virtual BOOT sector *AND* the SuperBOOT code. This is

possible using a little trick: when calling standard BIOS functions (read &

write), select the 128 or 129 head (instead 0 and 1 ones).  This lets 2M to

know  your  intention, and physical sectors will be read or written without

emulations.  Note that first track of 2M diskette has less sectors than the

rest,  so you must access no more than 1 (boot) + N (FAT1) + 1 (the virtual

BOOT) + 5 (SuperBOOT code) = 7 + N sectors ... and with double density only

must be accessed boot, FAT1 and virtual BOOT (no SuperBOOT code!).

If disk is formatted using the STV technology (when offset 65 of boot

record is equal to 1)  the completely track must be written after format to

avoid future CRC errors on read.  This  does not is necessary on cylinder 0

side 0. In cylinder 0 side 1 the track must be write using head 81h instead

head 1. STV technology is used by 2MF if /M switch is present.

You don't need to build your own boot  sectors,  buf if you decide to

modify something, you must know:

- 2M diskettes have always  TWO FAT  copies,  for hardware reasons. Do

not try to alter this specification (yes, for *hardware* reasons).

- Modifing system ID probably makes 2M not able to detect the diskette,

and standard BIOS will report sector-not-found errors.

- Yoy can modify the jump instruction at the begin  of  boot,  and the

boot code located after destination of this jump.  But  data located

from offset 63 to last byte before boot code is covered by checksum,

and you have to compute it if you modify something (the arithmetic 8

bits add off all bytes must be zero, use checksum byte to force it).

- Resuming: you can only modify offsets from 0 to 2, from 11 to 61 (it

is not recommended) and the boot code.

- If you try to write a boot sector with 2M installed, you will really

write the virtual boot sector, instead the real one.  With 2M loaded

on memory, the real boot sector can be modified only invocating BIOS

write function in head 128 or 129 (instead 0-1).  The  virtual  boot

sector is located on first sector of the second FAT copy (when 2M is

loaded, all BIOS access to second copy of FAT are emulated using the

first copy instead).  This  trick  makes  2M compatible with DOS SYS

command. And  much  more, the first track of 2M diskette has the DOS

standard format, with less space free where don't have room to place

the completely FAT2.

- If  the  boot  sector has a invalid checksum, a 'seek error' will be

reported.

- The virtual boot sector can be modify without problem, inside of the

reason limits.  Note  that  with 2M installed you can access to this

sector via standard DOS functions (sector 0) or BIOS (track 0 head 0

and sector 1), never through FAT2 ... this is *emulated* by 2M (this

emulation is only skipped while using head 128-129 instead 0-1).

Here are the format of 2M diskettes, in order to meet your curiosity;

2M is not a secret or magic program!.  See  the  source code files for more

information (if you don't understand the source code comments, you may come

to Spain next summer :) ).

* BOOT SECTOR OF 2M DISKETTE (COMMON TO ALL FORMATS).

Assembler                        Comment                 Offset

---------------------  ----------------------------------  ----------

JMP   BootP           ; 3 bytes                                0

DB    "2M-STV08"      ; system ID of 2M 1.80M diskette         3

DW    512             ; bytes per sector                      11

DB    1               ; sectors per cluster                   13

DW    1               ; sectors reserved at beginning         14

DB    2               ; number of FAT copies                  16

DW    224             ; root directory entries                17

DW    3772            ; total sectors on disk                 19

DB    0F0h            ; media descriptor                      21

DW    11              ; sectors ocupied by each FAT           22

DW    23              ; sectors per track                     24

DW    2               ; number of heads                       26

DD    0               ; special sectors reserved              28

DD    0               ; number of sectors (32 bit drives)     32

DB    0               ; physical drive                        36

DB    0               ; reserved                              37

DB    29h             ; DOS >= 4.0 diskette                   38

DD    4B368A0Eh       ; serial number (random)                39

DB    "NO NAME    "   ; disk label                            43

DB    "FAT12   "      ; FAT type                              54

DB    0               ; reserved by 2M                        62

DB    0               ; 2M checksum (if used)                 63

DB    7               ; format version (>=7 if virtual BOOT)  64

DB    1               ; 1 if write track after format         65

DB    0               ; data transfer rate on track 0         66

DB    0               ; data transfer rate on track<>0        67

DW    BootP           ; offset to boot program                68

DW    Infp0           ; T1: information for track 0           70

DW    InfpX           ; T2: information for track<>0          72

DW    InfTm           ; T3: tracks sectors size table         74

* NORMAL FORMATS (820/1476/984/1804K). FOR EXAMPLE: 1804K.

Infp0 DB    19, 70          ; number of sectors / GAP 3 for format

DB    1,2,3,4,5,6,7,8 ; sectors ordered (20..22 does not exist)

DB    9,10,11,12,13,14

DB    15,16,17,18,19

InfpX DB    11, 40          ; number of sectors / GAP3 for format

DB    3               ; sector size

DB    1, 2            ; sector slidding (override by 2MF /X /Y)

InfTm DB    3,3,3,3,3,3     ; size of sectors 1, 2, 3,...

DB    3,3,3,3,3

BootP:...                   ; SuperBOOT loader

*  MAXIMUM CAPACITY FORMATS (902/1558/1066/1886K). FOR EXAMPLE: 1886K.

Infp0 DB    19, 70          ; number of sectors, GAP3 for format

DB    1,2,3,4,5,6,7,8 ; sectors ordered (20..23 does not exist)

DB    9,10,11,12,13,14

DB    15,16,17,18,19

InfpX DB    64, 3           ; number of sectors / GAP3 for preformat

DB    7               ; number of sectors to renum

DB    128+1,  4, 4    ; renum table:

DB    128+12, 1, 4    ; number of sector, new number, size

DB    128+23, 5, 4

DB    128+34, 2, 4

DB    128+45, 6, 3

DB    128+51, 3, 4

DB    128+62, 7, 2

InfTm DB    4,4,4,4,4,3,2   ; size of sectors 1, 2, 3,...

BootP:...                   ; SuperBOOT loader

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

2M format function in Ralf Brown Interrupt List format:

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

--------B-13057FSI324D-----------------------

INT 13 - 2M - FORMAT TRACK

AX = 057Fh

SI = 324Dh ("2M")

CH = track number

DH = head number

DL = drive number

ES:BX -> boot sector of future 2M diskette

Return: CF set on error

CF clear if successful

AH = status (see AH=01h)

Program: 2M is a TSR developed by Ciriaco Garcia de Celis to support

non standard diskettes with 820-902/1476-1558K (5.25 DD/HD)

and 984-1066/1804-1886K/3608-3772K (3.5 DD/HD/ED)

Notes:  it is not necessary to call AH=17h/AH=18h first (will be ignored)

diskette format must begin always on cylinder 0 head 0

the installation check for 2M must search a "CiriSOFT:2M:2.1" string or

similar (recomended ":2M:" substring) in CiriSOFT TSR interface

the boot sector can be obtained from a 2M diskette already formatted if

reading (AH=02h) with head number 80h since 2M 2.0 (else use head 0)

since 2.0 version of 2M, the BOOT sector is emuled using first physical

sector of FAT2; the second-sixth physical sectors of FAT2 in HD or ED

diskettes store the SuperBOOT code. To skip the FAT2 emulation (using

FAT1) of 2M, in order to read the SuperBOOT code, head number must be

80h-81h instead 0-1 (bit 7 active) in standard read/write functions.

This lets diskcopy programs to format 2M target diskettes copying

SuperBOOT code. If target diskette is already 2MF formatted (provided

of boot code) this trick it is not necessary

when using STV technology (offset 65 of boot sector equal to 1) it is

necessary to write the full track before formatting (except track 0

side 0) to complete the format and skip future CRC errors on read; in

track 0 side 1 the head used must be 81h instead 1. Diskcopy programs

may do a format-write-verify secuential phases to improve performance

SeeAlso: AH=05h"FLOPPY",INT 2F"CiriSOFT TSR interface"