README.DOC - Fax Software for OS/2

Copyright (c) 1992-1994 by Keller Group Inc.  All rights reserved.

August 3, 1994

Topics contained in this README.DOC file:

MODEM CONFIGURATION SETTINGS

MULTILINE NOTES

SUPPORT FOR SERIAL PORT CARDS

SUPPORT FOR BROOKTROUT'S TR112, TR114 AND 3.4 DRIVER/API

SUPPORT FOR GAMMALINK'S GAMMAFAX PRODUCTS

NEW FEATURES

MODEM CONFIGURATION SETTINGS

----------------------------

Tested modems are listed below.  Unless the manufacturer has changed

the product since the time of our testing, you should use the

following information to determine correct configure values for your

fax hardware.  Make these settings on the Modem Type page in the fax

program's Settings notebook.

(Many other modems will also work, but the vendors have not provided

us with evaluation units for testing.)

Note that manufacturers are constantly changing their modem hardware

and firmware (EPROMs).  If you suspect a problem, please contact your

modem's manufacturer for assistance in determining the exact version

of your hardware and EPROM.  The manufacturer may ask you to use a

data communications program and issue the ATI3 command to obtain

modem version information.

MANUFACTURER  MODEL                          CONFIGURATION

------------- ------------------------------ ----------------------------------

Adtech Micro  Externally Yours! (ROM v2.2)   Class-2, might need 'No DLE esc'

Apex          PCMCIA modem                   Class-1

ATI           14,400 Internal Fax/Modem      Class-1

AT&T          Keep In Touch PCMCIA           Class-1

AT&T Paradyne DataPort Fax Modem             Class-1

Austin Cmptrs Desktop fax modem              Class-2, Special command &Q6

Best Data     Smart One Fax 14,400 Modem     Class-1

Boca          Bocamodem 14.4Kbps  user reported: Class-1, Special command &C1

BIT Blitzer   XM124S/RFAX                    Class-2

Brooktrout    TR112 and TR114                See special section below.

Calculus      EZ-FAX                         NOT COMPAT: Proprietary fax cmds

Complete PC   Complete Modem Plus            Class-1

Complete PC   certain models                 NOT COMPAT: Proprietary fax cmds

CompuCom      Speedmodem Combo (ROM v2.77)   Class-2

CPI           VIVA 14.4k internal            Class-1,Special command &C1

Dallas        Dallas Fax 14.4E Pro Plus      NOT COMPAT: Class 1 train failure

Try at 9600bps and special command &C1 or &C1\J0\Q3W0

Dataplex      DPX-223-55 2400/9600 rom5.42   Class-2

Dataplex      DPX-225-01 14,400 (ROM 1.00)   Class-2

DELL          Digitan (Dell desktop units)   Class-2,Xon recv start,no DLE esc

DELL          Datarace (Dell laptop units)   Class-1, Special command \J0\Q2

Digicom (DSI) Scout Plus, Spirit II          Class-1

Digicom (DSI) Digicom 9624LE                 NOT COMPAT: SendFax non-spec

Digicom (DSI) Digicom Connection 14.4+FAX    NOT COMPAT

Dove          DoveFax (ROM v4.2)             Class-1

Dr. Neuhaus   FURY 9600, 9600 TI, 14.4 TI    Class-2

Dr. Neuhaus   FURY 2400 POCKET/FAX, PC/FAX   Class-2

Dr. Neuhaus   FURY 2400 PC JUNIOR            SendFax

Dr. Neuhaus   FURY 2402 TI/FAX               SendFax

Elsa          2460, MicroLink 14.4T/TM/TL    Class-2, Special command \Q1

14.4T with ROM 2.01 also needs Special command &K1&R7

Everex        EverFax 24/96 (COM1/2 only)    Cls2,Alt bit order,Xon recv start

Formosa       MoFax MF-96P                   NOT COMPAT: SendFax train failure

Frecom        Fax96                          NOT COMPAT: Non-standard chip set

GammaLink     CPi, CP, CP MC, XPi            See special section below.

GammaLink     XP, XP MC                      NOT COMPAT: Not externally prog.

Gateway       Telepath (with recent EPROM)   Class-2

Supported only with recent EPROMs.  Some older EPROM versions

(1992, early 1993) are incompatible.

Gateway       Telepath II                    Same as U S Robotics Sportster

GVC           FAXMODEM 9624Vbis              Class-1 or Class-2

Hayes         Accura 14.4                    Class-1

Hayes         Optima 24/96                   Class-1

Hayes         Optima 96/96                   Class-1

Hayes         Optima 14.4/14.4               Class-1

IBM           L40SX laptop modem             Class-1, No BPS s/d, Skip TCF read

Tested and shows limited compatability potential.  Even after

engineering changes made by IBM, reception and sending of faxes

have been reported as spotty and line-sensitive. For

alternatives, see Microsource Fax42.L40.

IBM           9600 PCMCIA Fax Modem          Class-1

IBM           14400 PCMCIA Fax Modem         Class-1 (EPROM v028 or higher)

IBM           Microelectronics 14.4/14.4     Class-2

IBM           Fax Concentrator               NOT COMPAT: Proprietary fax cmds

Image Comm    TWINCOM 24/96                  Class-2

Image Comm    TWINCOM Commuter 24/96         Class-1

Image Comm    TWINCOM 14,400 v1.13C          Class-2

Infotel       1414 VQE                       Class-2

Intel         144/144e or 144/144i           Class-2 9600bps (failures at 14.4)

Intel         SatisFAXtion classic           uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/100               Class-1

Intel         SatisFAXtion/200               uses FaxIS.adp as Fax.adp

Intel         SatisFAXtion/300               Class-1

Intel         SatisFAXtion/400 (internal)    uses FaxIS.adp as Fax.adp

Units built in the fall 1993 timeframe require our fax software

build 1.39 or later.  Models made without a scanner port

require build 1.49 (1.39.10) or later.

Intel         SatisFAXtion/400e (external)   Class-1

Intel         Connection CoProcessor         NOT COMPAT: Proprietary fax cmds

Megahertz     96/96 FAX/Modem (P296FMV)      Class-1, Special command \J0\Q1

Megahertz     14400 Fax/Modem (XJ144)        Class-1

Megahertz     14400 Fax/Modem (XJ244)        Class-2,Xon recv start,No DLE esc

Megahertz     14400 Fax/Modem (XJ1144)       Class-1, Special command &K4

MICC          9642                           Class-2

MicroCom                      user reported: Class-1, Special command &C1

Microsource   Fax42.L40 (for IBM L40)        Class-2

Marketed as a replacement for the IBM L40 LAPTOP modem.

The modem has been tested and is compatible with the fax

software. (404 952-3336)

MultiTech     MultiFaxPC MT932               Class-2, Alt bit order

MultiTech     Multi Modem MT932EAF           Class-2, Alt bit order

MultiTech     Multi Modem II                 Class-2, Alt bit order

MultiTech     Multi Modem V.32 Fax           Class-2, Alt bit order

NetComm       M4F or M7F (ROM rev K)         Cls-2, Special cmd +FDCC=,3,0,2,0

Omnitel       9624+S/R                       NOT COMPAT: SendFax recv failures

Omnitel       Omnitel v1.10                  NOT COMPAT: Class-2 recv failures

P.C. Logic    Fax144EX                       Class-1 (recommended) or Class-2

P.C. Logic    Fax 24I                        Class-2, XON recv start

Practical     2400F MINI                     Class-1

Practical     2400FX96                       Class-2

Practical     2400 SendFAX                   SendFax

Practical     14400FXSA V.32bis              Class-1, Check TCF signal

Practical     14400FXMT V.32bis              Class-1 or Class-2

Prodigy       Eagle                          Class-1

PureData      14,400 PCMCIA V.17             Class-1 or Class-2

QuickCom      Spirit II                      Class-1

Supra         SupraFAXmodem V.32bis          Class-1 or Class-2

Silent Answer feature is supported.  The special command

+FAA=2 is required for operations as a Class 2 with Silent

Answer engaged, and +FEA=2 for Class 1 operations with Silent

Answer.  Recent versions of Supra modems respond more slowly

to certain commands, and therefore require fax software

build 1.50 (1.39.11) or later.

Supra         SupraFAXmodem V.32 bis Int.    Class-1

Supra         SupraFAXmodem V.32 bis LC      Class-1, (v1.39.11 or higher)

Texas Instr.  TI4000E laptop modem           Class-1, Special command \J0\Q3

Toshiba       laptop Megahertz modems        Same as Megahertz P296FMV

TwinCom       TwinCom v1.13C                 Class-2

UDS           FasTalk Fax32                  Class-1

USRobotics    Courier HST Dual Std w/Fax     Cls-1,Spec cmd &H2,Recv EOP delay

USRobotics    Courier 14400 MicroChannel     Cls-1,Spec cmd &H2,Recv EOP delay

See below for special note.

USRobotics    Sportster 14,400               Cls-1,Spc cmd &H2&I2,RecvEOPdelay

When sending to Pitney Bowes machines, use: Spc cmd &H2&I2S36=0

Do not use 'Check TCF signal' with this modem.

USRobotics    Courier 28800                  Cls-1,Spc cmd &H2&I2,RecvEOPdelay

USRobotics    V.terbo 21600 w/14400 Fax      Class-1, Recv EOP delay

USRobotics    Worldport 2496 V.42bis         SendFax

USRobotics    Worldport 9696 MNP5 FAX        SendFax

USRobotics    Worldport 9696 V.42 w/Fax      Class-1, Special command &H2

USRobotics    Worldport 14400 V.42 w/Fax     Class-1, Special command &H2

USRobotics    2400 PC / 2400 PC Fax          NOT COMPAT: SendFax non-spec

USRobotics    9600 Sportster                 NOT COMPAT: SendFax non-spec

VIVA          2496IF                         One model compatible

The VIVA board that is incompatible uses a slide switch

accessible from the rear of the computer to select the serial

port.  The VIVA board that is compatible uses on-card jumpers

to select the serial port setting.

ZyXEL         U-1496 (ROM 5.05 & 6.01)       Class-2 (ROM 6.00 is bad)

Zoltrix       ZX1896 or Zofax 96/24          SendFax

Zoltrix       9624 Class 2 (regular)         Class-2

Zoltrix       Class 2 with V.42 MNP          Class-2, might need 'No DLE esc'

Zoom          FaxModem FC 9624 (internal)    Class-2

Zoom          FaxModem FX 9624 (external)    Class-2

Zoom          V32bis (ROMs 1.56D,2.0 tested) Class-1 (or 2), Special cmd &C1

(ROM 1.6 bad)

SPECIAL NOTES:

CLASS 1 MODEM NOTE: For MOST Class 1 modems, using the 'Check TCF

signal' on the Modem Type page of the Settings notebook will provide

enhanced speed downgrading.  This may improve the quality of received

faxes.  On a few modems, engaging this switch will cause the modem

not to initialize properly.  Use this switch with care.  Never use

this switch with a US ROBOTICS SPORTSTER.

US ROBOTICS COURIER MicroChannel internal modem - has begun shipping

with a default setting loading during installation in the Reference

Diskette/ Partition that prevents return codes from reaching the fax

program.  THIS MUST BE CHANGED.  The user/installer must enter the

reference information area in the MicroChannel machine and ensure

that return code information is NOT blocked, otherwise the fax

program will NEVER be able to tell if your modem is responding

correctly, and will return an error message to you saying that it is

not responding correctly until you disable the return code blockage.

MULTILINE NOTES

----------------

If you have purchased a multiline version of the program, you can

install multiple fax hardware devices and fax telephone lines, and

then do simultaneous fax sending and/or receiving on these lines.

Upgrade versions of the fax software are available to support

various numbers of lines (2, 4, 8, 16, 32, 48, 96, etc.).

On a given CPU, the multiple fax hardware devices must be of the same

type.  The same configuration page in the Settings notebook controls

the configuration for all the fax hardware devices.  The Ports page

in the Settings notebook is used to configure the individual fax

lines for sending, receiving or both.  The selected lines are

automatically used for simultaneous sending/receiving.

Fax modems - Either internal or external fax modems can be used, and

while the modems do not need to be identical, they must all use

exactly the same values on the Modem Type page in the Settings

notebook.  Each fax modem must be on a separate COM port in your

system, and the COM ports must be standard hardware chips (8250,

16450, 16550, or 16550A equivalents); although the ports need not be

at the conventional COM port address and IRQ settings since you can

specify these settings on the FMD.SYS line in your CONFIG.SYS file.

See the next section for a discussion of serial boards for adding COM

ports.  For large port counts, we recommend using ports with 16550A

UARTs and Class 2 fax modems.

SatisFAXtion/200 or SatisFAXtion/400 boards - You can use up to eight

internal boards.  Be sure to read about SatisFAXtion boards in the

appendix of the Reference Manual before proceeding.  With build 1.47

and later, you can now use a mixture of various SatisFAXtion boards

types (/200, original /400, new /400, etc.) and the appropriate

DOWNLOAD.* and LOADER.* files will be used for each board.  Each

board must be set to a unique I/O Port Address.  As described in the

appendix, you use either Intel's SETUP.EXE program (on an ISA bus

machine) or a Reference Disk (on a MCA bus machine) to change the I/O

Port Address.  Since Intel's SatisFAXtion board installation

procedures only work with one board at a time, you must insert one

board, power up your machine and do the necessary board I/O Port

Address changes to place the board at a unique address, power down

your machine and remove the board, then repeat this procedure for the

next board.  After all boards have been separately configured for

unique I/O Port Addresses, you can then insert all the boards in your

system.

Brooktrout boards - If you have properly installed the OS/2 runtime

software from Brooktrout, the Brooktrout ports will appear on the

Ports page in the Settings notebook.  It is theoretically possible to

run up to 96 lines using Brooktrout fax hardware.  We now support the

four-line TR114 and the two-line TR112 boards.  The boards require

Brooktrout's version 3.4 OS/2 driver/API.  Contact Technical Support

if you need the Brooktrout version 3.4 OS/2 driver/API for use with

the fax software.  Updated installation instructions for the 3.4

driver/API are included below.

GammaLink boards - The GammaFax ports will appear on the Ports page

in the Settings notebook.  The fax program interfaces with the

GammaFax boards via the GammaFax Dispatcher, so it can operate

simultaneously with many other GammaFax software products which also

use the GammaFax Dispatcher.  See the separate installation notes for

GammaLink hardware below.

SUPPORT FOR SERIAL PORT CARDS (revised 11/03/93)

-----------------------------

The multiline versions of the fax software can support up to 96 fax

lines, but it can be difficult to find enough ports, interrupts or

slots to attach modems to your PC.

A serial port board might be the answer.  Boards that provide real

hardware UART ports (compatible with standard PC COM ports) are

supported, and the fax software's optimized device driver allows you

to use nonstandard port I/O addresses, nonstandard interrupts and

shared interrupts.  The more expensive "intelligent" serial boards

are not needed and are not supported.  Serial boards are commonly

available in 4-, 8- and 16-port versions for both ISA and MCA

machines, and we've tested many products including boards from

DigiBoard, STB, GTEK, and Quatech.  For large systems, we generally

recommend using boards with 16550A UARTs and Class 2 fax modems.

16550A UART Support - The fax software can take advantage of 16550A

UART chips if they are used in your PC serial port, internal fax

modem or serial port board.  The use of 16550A UARTs is not required,

but they will decrease the system load during fax sending and

receiving with fax modem hardware since the 16550A hardware buffering

substantially reduces the interrupt rate.

Multi-channel Serial Port Card Testing Info:

STB Systems, Inc.:

Tested: 4-COM

Description: 4-port (16550A compatible) serial card

Setup:  4 independent 16550A ports

i/o: 1A8, 1E8, 1F8, 2A8 (could be 2E8, 2F8, 3E8, 3F8)

int: 12 (could be 2, 3, 4, 5, 10, 11, 12, 15)

(ports can share interrupts but don't have to)

Notes:  Short DIN to 9-pin cables supplied.

GTEK, Inc.:

Tested: PCSS-8X

Description: 8-port (8250) serial card

Setup:  8 ports in a group

i/o: 280-28F (could be 2C0-2FF in COM2 compatibility mode)

int: 5 (could be 2,3,4,5,7)

(can also do DOS compatible port paging to COM1 or 2)

Notes:  16550A version of card would be lots better.  Cables

are not brought out back of card - they come out of top.

Other:

PCSS-4X: above with 4 ports

PCSS-8U: no port paging, i/o 180-18F and 1C0-2FF also

PCSS-4U: above with 4 ports

PCSS-8:  DOS compatible port paging only (no good)

PCSS-4:  above with 4 ports

Quatech, Inc.:

Tested: QS-100D

Description: 4-port (StarTech 16550A) serial card

Setup:  4 ports in a group

i/o: 0300-031F (could be anywhere: 0000-001F to FFE0-FFFF)

int: 5 (could be 2,3,4,5,6,7)

(new card version also supports 10,11,12,14,15)

(interrupts can be shared with other Quatech adapters but

not with other vendor's cards)

Notes:  Four port board uses octopus type cable -- could directly

attach to modems.  Eight port board has modular connectors.

16550A scratch port can be switch selected to interrupt

status port.

DigiBoard:

Tested: DigiCHANNEL MC/4

Description: Microchannel 4-port (16550A) serial card

Setup:  4 ports in a group, 4 blank ports, 1 interrupt status port

i/o: DB80-DBC0 (could be: DC00, BB80, BC00, AB80, AC00, CB80)

int: 3 (could be 3,4,5,7,10,11,12,15)

(interrupts can be shared with multiple adapters)

Notes:  Octopus type cable.  Could directly attach to modems.

MC/8 and MC/16 should work too.  The 4i, 8i, and 16i use

128K dual ported memory window and probably don't work.

AT bus version probably OK too, but untested.  Interrupt

status port cannot be disabled and takes up an additional

i/o port.

SUPPORT FOR BROOKTROUT'S TR112, TR114 AND 3.4 DRIVER/API (revised 5/11/94)

--------------------------------------------------------

Brooktrout's version 3.4 OS/2 driver/API is required for Brooktrout

TR112 and TR114 boards.  If you don't have the Brooktrout version 3.4

runtime files, you can obtain them from SofNet Technical Support or

Keller Group.

To install the Brooktrout fax hardware, first follow the instructions

in Brooktrout's hardware guide to install your Brooktrout fax boards,

then follow these instructions to install the Brooktrout version 3.4

OS/2 driver/API software.

[The destination directory for the Brooktrout software is assumed to

be C:\BFAX for the following steps.  Please change the commands as

needed for your installation directory.]

1) Copy BTDLLS.DLL into C:\OS2\DLL or another directory on your LIBPATH.

2) To create the install directory, type:

MKDIR C:\BFAX

3) Copy BFAX.SYS, FAXINIT.EXE, and *.CFG into C:\BFAX.  If a

directory other than C:\BFAX is used, BTCALL.CFG will need

to be edited and all four paths changed from C:\BFAX to the

proper directory.

4) Edit your C:\CONFIG.SYS to add an appropriate line like this one:

DEVICE=C:\BFAX\BFAX.SYS D1 I12

This line is for DMA channel 1, and interrupt 12.  These settings

should match the jumpers set on the board.  Modify the D1 or I12

as appropriate.  Please make sure any DEVICE= lines from older

Brooktrout BFAX drivers are removed from your CONFIG.SYS and that

there are no device conflicts with the DMA channel and interrupt

number you have chosen.

5) Edit your C:\STARTUP.CMD to add the following line:

C:\BFAX\FAXINIT

6) Install the OS/2 fax software as usual.  On the INSTALL screen,

select "Brooktrout BTDLLS version 3.4" as the fax hardware

type.

7) You will need to reboot for OS/2 to load the new BFAX.SYS driver.

8) For the Brooktrout configuration value in the fax program's

Settings notebook, specify the path:

C:\BFAX\BTCALL.CFG

Note that FAXINIT.EXE, BFAX.SYS, *.PEX, *.CFG, and BTCALLS.DLL are

from Brooktrout.  We will do our best to help out, but any real

problems should probably be addressed to Brooktrout.

We have noticed some things about Brooktrout's FAXINIT program.

There is an undocumented range of addresses that FAXINIT looks for.

Here is what we know about FAXINIT:

FAXINIT doesn't load firmware into the cards, it just finds them.

After it peeks around at the board addresses, it tells the BFAX.SYS

driver where they are.  (The driver then tells TR114 cards what DMA to

use.)  This means that FAXINIT needs to know where to look.  FAXINIT

will automagically find cards at the following addresses:

210 214 218 21C

220 224 228 22C

230 234 238 23C

240 244 248 24C

250 254 258 25C

260 264 268 26C

270 274 278 27C

If your card address is not on this list, it WILL NOT be found by

default.

If you choose non-standard addresses, you must create a configuration

file listing the port address of each card, even if some of the cards

have standard addresses.  Addresses must be in hex format.

For example, if you have 3 cards, with non-standard addresses at 300,

350, and 354, list them in your configuration file in the following

way:

addr 300

addr 350

addr 354

To execute FAXINIT when you are using non-standard addresses, use the

following command where filename is the name of your configuration file:

faxinit -n filename

SUPPORT FOR GAMMALINK'S GAMMAFAX PRODUCTS (revised 05/27/94)

-----------------------------------------

The GammaFax adapter allows the operation of intelligent GammaFax

boards, such as the CPi, CP, CP MC, and XPi.  The four port boards

should also work fine, including the CP4-LSI and the MLCP4 with

DTI-124 T1 interface board.  (XP and XP MC boards are NOT supported

since they are non-programmable and do not support API access.)

Since the program interfaces with the board via the GammaFax

Dispatcher, it will operate simultaneously with many other GammaFax

software products which also use the GammaFax Dispatcher.  The

GammaFax adapter disables reception on the fax channels which it uses

when it is not waiting for a fax.  Therefore, if an application

assumes that incoming faxes will be automatically answered by the

Dispatcher, it may not function correctly on a line that is enabled

for use by the adapter.  Otherwise, applications should operate

normally when sending or when assigned separate lines.

Install the GammaFax OS/2 software as directed in GammaLink's

instructions.  It is recommended that the GammaFax software be

installed in a directory different than the fax program.  For our

examples that follow, we are assuming that the GammaFax software is

installed in the directory "C:\FAX".  Be sure to modify the pathnames

if you use different directories.

Verify that the command script files (C:\FAX\DISPATCH.CMD and

C:\FAX\FAXMENUS.CMD with version 4.x, C:\FAX\GFQM.CMD and C:\QF.CMD

with version 5.x) correctly reflect the GammaFax software's

installation directory.  For example, if the GammaFax OS/2 software

is installed in C:\FAX, the first line of both the files should be

"SET GFAX=C:\FAX".

Verify that your CONFIG.SYS file includes the line "SET GFAX=C:\FAX".

(You will need to reboot your system so that the changes to your

CONFIG.SYS file will take effect.)

Edit the configuration file, C:\FAX\GFAX.$DC, to include the lines

below, in addition to, or replacing, the lines provided by the

GammaFax installation program.  The first three lines specify time out

values in seconds which may be adjusted, as desired.  For fewer disk

accesses, increase these values.  For quicker response, decrease

them.  The last three lines should be repeated for each GammaFax channel

to be controlled by the fax adapter and the value 1 should be

replaced with the logical channel number.

controlt 10

queuet 20

statust 30

gfxshutdown 1 3

gfxform 1 3

gfxwait 1 20

Please refer to the "GammaFax Reference Manual" for more information

about the configuration file (chapter 17 "Queue Manager

Configuration" in version 4.3.1, chapter 4 "Configuring the GammaFax

System" in version 5.x).

Examples of complete configuration files are included at the end of

this section.  Refer to the second example if you wish to use DTMF

routing with the LAN fax software.

Start C:\FAX\DISPATCH.CMD or C:\FAX\GFQM.CMD in an OS/2 window.  If

desired, you can add C:\FAX\DISPATCH.CMD or C:\FAX\GFQM.CMD to your

STARTUP.CMD or create a program object with the Path and File Name

set to C:\FAX\DISPATCH.CMD or C:\FAX\GFQM.CMD and place the object in

your startup folder.

Start the fax program and set the ports to be controlled using the

Ports page in the Settings notebook.  Also in the Settings notebook,

set the proper phone line type (recognizable dial tone or not) and

dialing style (tone or pulse).

Set the Q Poll Time variable in the fax program's Settings notebook

as desired.  This is generally set to the same value as the "queuet"

configuration value.

----------

Example 1 - Here is a example of a complete configuration file for a

single line GammaFax installation.

chassis 1

buffers 2

numchan 1

controlt 10             *changed from default install

queuet 20               *changed from default install

statust 30              *added this line

updatet 300

channel 1 0 GFAX1.01

country 1 1

init 1

boardtype 1 0x2c3

load 1 C:\FAX\GFXCX.BIN

csid 1 555-555-5555

modemctrl 1 1024 1

gfxecm 1 1 1

gfxshutdown 1 3         *changed from default install

gfxform 1 3             *added this line

gfxwait 1 20            *added this line

----------

Example 2 - This is an example of a multiline configuration file, and

also demonstrates the use of DTMF input for routing incoming faxes.

This configuration is for a 3-line system, with line 1 being a CP

board, and lines 2 and 3 being CPi boards.  (From what we understand,

the CPi and XPi boards are like the CP and XP, but with built-in DTMF

reception - the CP board requires an optional daughter board for DTMF

input).

To enable DTMF input, lines like the following must be added to the

GFAX.$CD file:

gfxdtmftimeout 2 6 15

gfxdigits 2 10 0

gfxdtmftone 2 440 500

These lines are added as part of each card's configuration.  The "2"

in each of these lines refers to the line number, so it changes for

each card.  The first line specifies an interdigit timeout (6

seconds) and a total timeout (15 seconds).  The second line specifies

the maximum number of DTMF digits to be entered (10 digits) and the

terminating digits (0 for none).  The third line specifies the tone

that will be used to promt the caller for entering the DTMF digits

(440Hz for 500ms).  See the GammaFax release notes for a complete

description of the values for DTMF input.

The lines in the following example were rearranged so all that

reference a channel are grouped together.  This does not affect the

operation.

chassis  1

buffers  6

numchan  3

controlt 10             *changed from default install

queuet 20               *changed from default install

statust 30              *added this line

updatet 300

channel 1 0 GFAX1.01

country 1  1

init 1

boardtype 1 0x80

load 1 C:\FAX\GFXCP.BIN

csid 1 555-555-5555

modemctrl 1 1039 0

gfxshutdown 1 3         *changed from default install

gfxform 1 3             *added this line

gfxwait 1 15            *added this line

channel 2 1 GFAX1.02

country 2 1

init 2

boardtype 2 0x283

load 2 C:\FAX\GFXCX.BIN

csid 2 555-555-5555

gfxshutdown 2 3         *changed from default install

gfxform 2 3             *added this line

gfxwait 2 15            *added this line

modemctrl 2 1039 0

gfxdtmftimeout 2 6 15   *added for DTMF LAN routing

gfxdigits 2 10 0        *added for DTMF LAN routing

gfxdtmftone 2 440 500   *added for DTMF LAN routing

channel 3 2 GFAX1.03

country 3 1

init 3

boardtype 3 0x283

load 3 C:\FAX\GFXCX.BIN

csid 3 555-555-5555

gfxshutdown 3 3         *changed from default install

gfxform 3 3             *added this line

gfxwait 3 15            *added this line

modemctrl 3 1039 0

gfxdtmftimeout 3 6 15   *added for DTMF LAN routing

gfxdigits 3 10 0        *added for DTMF LAN routing

gfxdtmftone 3 440 500   *added for DTMF LAN routing

NEW FEATURES

------------

The following is a partial summary of new features that are included

in each build.  The number indicates the program build that first

included the feature.

1.16 - When a received fax terminates in an error status, pages that

were received before the error are retained.  Text line spacing is

now 120% of the current font point size.  Previously, line spacing

was derived from the font, so the number of lines per inch in a given

point size could vary based on the font.  Added a "No BPS stepdown"

check box on the Options/Configure screen for the L40SX modem.  This

fax modem is still unable to send to fax machines that require

stepping down to slower baud rates, but if this check box is used,

the program will retry without stepping down, which may allow the

machines to communicate in some circumstances.  Enhanced capturing

with the Utilities/Capture to clipboard command.  Colored icons,

including OS/2 buttons such as minimize/maximize, now map to a

pattern rather than turning black.  Added support for the Intel Hand

Scanner with SatisFAXtion boards (see Appendix C in the User's Manual

for details).

1.22 - For the Printer Driver Configuration screen: The program now

sets the fax directory for the printer driver, so the "Fax directory"

field has been removed.  The "No log entry" action has been removed.

A "Tones" field has been added to enable/disable the beeping tones

during printer driver operation.  The Send Fax screen now includes a

"To" field that shows a list of all selected destinations.  You can

remove a destination by double-clicking on its line.  The "Cover

bitmap" pull-down list on the Fax Send screen now includes BMP files

from your fax directory and your working directory.  Current call

sending is now started by pushing the "Send current" button on the

Send Fax screen.  If you select multiple destinations, the Send

button changes to indicate the number of destinations.  The "Fax

local id" field on the Options Settings screen now supports

alphanumeric characters.  This allows you to send text as your fax

id.  (This only works if your fax hardware allows it... some fax

hardware will only accept numerics.)  The field widths in the log

display have been adjusted to compensate for OS/2 system font changes

in the OS/2 2.0 Service Pack update (and OS/2 2.1).  For the IBM

L40SX laptop and other machines with a "suspend" mode, the program

can now receive a fax when the machine is "resumed".  This did not

work in previous releases because the L40SX fax modem is reset by the

suspend/resume procedure.  If the program is left in receive mode, it

will now reinitialize the modem and be able to answer fax calls when

the machine resumes.  Since a suspended machine is automatically

resumed by an incoming call, this means that a suspended machine can

receive faxes.  After the fax has been received, the L40SX can

automatically suspend itself again after a time delay (which is set

on the L40SX by the L40's "PS2 SET POwer off minutes" command).  Also

of interest to L40SX users, the "No BPS Stepdown" check box on the

Options/Configure screen now applies only to sending.  The L40SX fax

modem can correctly receive at slower baud rates, but does not

correctly support sending at speeds slower than 9600 baud.

1.23 - Tab and cursor key support is improved in the INSTALL program.

Alt-F4 now works as a short-cut key for closing the application.

Cover sheets now use proportional fonts for enhanced appearance.  The

order of preference (using the first one that is available on your

system) is Helvetica, Times Roman, then Courier.  A "Utilities/Edit

phone books" command has been added to permit phone book editing

without having to use the Fax/Send command.  The application now

shuts down fax modems upon program exit, thereby leaving the modem

reset for use by data comm programs even if you were in fax receive

mode.  Current call sending has been enhanced.  If a cover sheet is

enabled, you will be prompted for the name and company so that this

information can be included on the cover sheet.  The application will

then construct the fax document and provide a pop-up dialog box when

it is ready for you to dial the call.  This allows the application to

immediately begin sending upon your request.  For LAN workstation

use, the workstation slave adapter is now called the workstation

adapter and is significantly modified as a result of improvements for

the enhanced LAN version of the product.  Enhanced printer support

for DOS and Windows printing is included.  The printer object's job

properties can now be either normal or fine resolution when printing

from DOS or Windows (rather than requiring fine resolution), so you

can create either normal or fine resolution fax documents from DOS

and Windows applications.  For DOS printing, the IBM Proprinter X24

emulation is improved to support underlining and fine-tune the

character spacing.

(WordPerfect 5.1 DOS users... When printing to the fax printer device

from WordPerfect using WordPerfect's IBM Proprinter X24 printer

driver, you can now generate very high-quality output.  Try printing

WordPerfect's printer test file for an example.  If you need even

more font support for printing fax documents from WordPerfect DOS,

you might want to consider the PrimeType product as discussed in the

User's Manual.)

1.26 - Phone book displays now show the fax number on the main

selection list and on the Utilities/Edit phone books list.  For DOS

printing, the Proprinter X24 emulation now shows emphasized and

double-strike characters in Bold.  Note that the Proprinter X24

printer does not provide italic.  Enhanced Class-1 support, including

support for new Practical Peripherals 14400FXSA v1.21 ROMs, Megahertz

and new USR Worldports.  Improvements for USR HST receiving, ZyXEL

sending, and more...  The "Recv EOP delay" check box (for Class-1

only) is added to the Fax modem Options/Configure screen.  Use this

option for USRobotics Courier and Sportster Class-1 modems to correct

a USR quirk when receiving from certain fax machines and modems.  The

"Check TCF signal" check box (for Class-1 only) is added to the Fax

modem Options/Configure screen.  Use this option for enabling

interpretation of TCF data for enhanced negotiation of automatic

speed downgrading.  This helps overcome problems with receiving noisy

faxes if you telephone company uses a line concentrator (which may

require speed downgrading to 7200bps).

1.27 - Revised INSTALL program.  Support for 16550A UARTs.  The 'Use

HW FIFO' setting on the fax modem Options/Configure screen allows the

FMD.SYS driver to take advantage of the high-performance buffering

features of UART (serial I/O chips, used for your COM port) that are

compatible with the NS16550A chips.

1.28 - For LAN version, added support for DTMF/Touch-Tone automatic

routing (in addition to DID automatic routing) with Brooktrout

boards.  Fixed current call sending for SatisFAXtion boards.  The

printer driver now supports the FAX.GGS file for providing printer

driver commands to the next print job.  Applications can place the >>

printer driver commands in this file rather than directly in the

printer data stream.  This can be used to provide fax sending and/or

fax merge features from other applications that cannot otherwise

insert the printer driver commands directly into the printer data

stream.  The printer driver now supports a configuration item for

selecting the number of gray levels.  For applications that send a

color image to the printer, the printer driver can convert the colors

to "gray" patterns.  Using gray areas will increase fax transmission

time, but can provide very nice looking graphics.  Experiment with

various settings on the printer driver configuration screen to see

what works best for your images.  A program (FxRcv.EXE) is now

provided on the distribution diskette as a alternative way of turning

receive mode on and off.  See the section above called "CONTROLLING

RECEIVE MODE FROM COMMAND LINE/COMMAND FILE".  This can be used in

command files for temporarily taking the fax program out of receive

mode so that you can make a data communications call with a fax modem.

1.29 - (LAN version) - Private mode, which provides LAN support for

user privacy and routing, now supports up to 5000 workstations per

server.  This was unintentionally limited to only 200 workstations in

previous beta test releases.  Paste from Clipboard now provides color

metafile support.  "Color filled area" printing is further enhanced

in the printer driver.  You can now configure the printer driver for

either "Black & white", "Fax 8-level" or "System dithered"

processing.  "Black & white" forces all colors to black.  "Fax

8-level" maps colors to patterns that are custom-designed to allow

efficient fax transmission and support resizing and printing without

aliasing.  "System dithered" uses standard system color dithering

which can produce nice looking images, but the images are less

efficient for fax transmission and the gray areas may alias (distort)

when resized or printed.  Experiment with various settings on the

printer driver configuration screen to see what works best for your

images.  For some strange reason, the OS/2 Enhanced Editor opens and

closes the printer driver when you select the printer.  This formerly

caused an "empty" print job.  The printer driver now ignores print

jobs that don't actually print anything.

1.30 - Proprinter emulation in the printer driver is improved

to avoid "jaggy" vertical lines and characters when printing

from Windows applications through the Proprinter WIN-OS/2 printer

driver.  This was caused by the WIN-OS/2 printer driver using

"mixed resolution" positioning commands, such as moving the print

head with 120dpi commands while printing in 180dpi graphics mode.

1.30b - For fax modems, the FMD.SYS device driver now checks the port

interrupt.  This allows the program to display only the useable ports

in the Options/Ports list.  In special cases where this might be a

problem, the port checking can be turned off by adding the 'N'

parameter to the CONFIG.SYS line which installs FMD.SYS.  Fixed

Fax/Export/DCX to always write fine mode (square aspect ratio), and

to write smaller DCX headers.  Added Intel SatisFAXtion DTMF support

(for SatisFAXtion/200 and SatisFAXtion/400), which can be used in the

LAN version (private mode) for automatic routing of incoming fax

documents.  (We already support both DTMF and DID automatic routing

with Brooktrout TR112 fax boards).  Event logging to the VOUT file

when the -v parameter is used.  Added a "Skip TCF read" check box on

the Options/Configure screen for the L40SX modem.  SVGA displays -

Due to problems in the SVGA drivers, the algorithm for the

Utilities/Capture to clipboard commands is changed.  Previously,

snapshots on SVGA displays may have appeared "dark".  8514A displays

- Problems in the OS/2 2.1 32-bit 8514A driver impact the

Utilities/Capture to clipboard command.  To work around this problem,

you can add the line "SET FXSNAP=BW" to your config.sys file.  This

environment variable setting will force the fax program to do

black-and-white screen/window capture rather than color mapping.

1.31 - The Options menu now has a separate "Ports" command to

configure ports for sending/receiving.  The port selection was

previously part of the Options/Configure screen.  First beta release

of multiline versions.  If you have a multiline version of the

program, the Options/Ports screen allows you to select multiple ports

for simultaneous use, and to specify each port as send, receive or

send/receive.  For Brooktrout fax hardware, implemented support for

the "Kill" feature on the status window.  (This already worked for

all other fax hardware... select a line in the status window and

press the "Kill" button to abort the activity.)

1.32 - Initial test support for the GammaLink fax boards.

1.34 - Enhanced the phone book handling in the printer driver pop-up

and the fax program.  Any individual phone book can now have up to

2000 entries (which is a size limit imposed by OS/2 list box issues),

and of course, you can have multiple phone books.  An "All" button is

added to the phone book.  This makes it easier to use a phone book as

a distribution list since you can easily select all members of the

current phone book without using a Group name.

1.35 - Installation (and updating) is now simplified.  The INSTALL

program copies the files, helps you modify your CONFIG.SYS file (if

you want it to do so), and instructs you to reboot your system when

necessary.  But you no longer need to run the INSTALL program a

second time to select your ports and set your modem configuration.

The fax program now "knows" when you've run INSTALL and automatically

gives you the option of setting ports/configuration the next time

that you run it.  A separate "Fax data directory" is now supported by

the INSTALL program.  If desired, you can specify separate

directories as the "Install to" directory for the program files, "Fax

data directory" for the program's data files (FAX.LOG, FAX.IDX, *.FAX

fax document files, *.PBK phone book files, and *.BMP bitmap files),

and "Working directory" as the starting working directory for your

import/export/paste file operations.  The separate data directory

makes it easier for you to backup your work or keep your fax

documents on a different drive.  (If you change your fax data

directory after initial installation and want to retain your past fax

activity, you must manually move your FAX.LOG, FAX.IDX, *.FAX, *.PBK

and *.BMP files to the new fax data directory.)  The INSTALL program

now remembers the locations of your fax program directory, fax data

directory and working directory.  This makes it easier to update your

software.

1.36 - The fax program now supports a new "-L newdir" command line

parameter for specifying the fax data directory.  The "newdir"

parameter is the pathname of the drive and directory that contains

the FAX.LOG file and other fax data files (FAX.IDX, *.FAX, *.PBK).

If you move your fax directory, you can use this parameter to tell

the program where to find the fax data files (the program remembers

the location, so the parameter is only needed the first time that you

run the program after moving your fax data directory).  If you have

multiple fax data directories you can use this parameter in the

"optional parameter" field of your program object to force the

program to use the correct fax data directory for various situations.

1.37 - The fax printer driver includes modifications to eliminate

clipping the edge of the output from certain OS/2 applications that

print all the way to the very edge of the page.

1.37 - Disk space monitoring is added.  To ensure proper logging of

activity, sending/receiving is disabled if there is less than 10

kbytes of available disk space.  To prevent answering the phone and

attempting to receive when the disk is full, sending/receiving is

also disabled if a receive operation fails due to a disk-related

error (syserror).  A pop-up dialog box will notify you when this

occurs.  Sending/receiving is enabled again when you use Fax/Receive

to turn receive mode back on or when you visit the Fax/Send dialog

box.  Fixed a problem with fax modems that do not support the S7=100

S register setting (such as for USRobotics modems in the UK).  These

modems will use their default setting for the S7 register value

rather than produce an error.

1.37b - Upon starting the fax program, if the fax log contains faxes

that have been "received" (or, for the LAN version, "routed") but not

yet read, the status window will be displayed and will indicate the

number of received faxes.  The OS/2 LAN workstation program now

includes a status window (like the stand-alone version).  When you

start the program, the status window will show a count of received

fax documents (if any).  The status window also shows background

printing activity and allows you to kill print jobs by selecting the

print line and pressing the Kill button.  If you are using the LAN

Private mode with the workstation's notification method set to "OS/2

mailslot", the status window's receive count will be dynamically

updated as faxes are received.

1.37e - The "Busy retries" setting on the Options/Settings screen can

now be a maximum of 99 (previously the maximum was 9).

1.39 - Based on user suggestions about the Import, Export and Paste

file commands, we've made the following changes to command names:

<old name>                    <new name>

Fax Import               -->  Fax Open file

Fax Export               -->  Fax Save file

Edit Paste file          -->  Edit Import

Utilities Save clipboard -->  Utilities Export from clipboard

A new "Edit Export" is command is added.  It saves a marked region

directly to a file.  This provides a faster way of creating cover

sheet bitmap files and other bitmap or PCX files.  Previously you

would need to mark a region, use Edit Copy to place it in the

Clipboard, and then use Utilities Save clipboard to save the

Clipboard contents to a bitmap file.  Much faster processing when you

select multiple documents for deletion from the fax log.  The

Options/Settings screen includes a new printer "Idle priority" check

box.  When printing fax documents to paper, this causes the print

thread to run at "idle" priority to avoid slowing system performance.

WARNING - DOS command windows and DOS applications run at normal

priority even when they are idle, and this prevents any "idle"

priority activity from running, so fax printing will pause when any

DOS processes are running.  IBM intends to correct this problem in a

future OS/2 release, but for now, DO NOT SET THE "IDLE PRIORITY"

CHECK BOX IF YOU WANT TO PRINT FAXES WHILE DOS PROCESSES ARE ACTIVE.

1.39b - The "Sent via" line is no longer generated on the cover sheet.

The default cover bitmap file includes a "sent via" line, but since

you can change the cover sheet bitmap, you can remove this if desired.

1.39c/d - Improved GammaFax support, install program network path

improvements, FxRte LAN routing fix, improved logging to VOUT file.

1.39d - Added "-D" option for disabling the available disk space check.

1.39e - Fixes "current page" OCR processing (if you have purchased

the OCR option) to correctly process the current page rather than the

first page of the document.

1.39f - Fixes a problem with the optional PCL printer driver which

could sometimes produce messy output files when printing from

AmiPro/Windows using the WIN-OS/2 LaserJet III printer driver with

the "Print ATM fonts as graphics" option (and perhaps in certain

other situations).  A new OS/2-based version of the PCL conversion

engine is also being used.  Improves page-oriented OCR processing of

normal resolution fax documents.  (But for best OCR results, you

should still use fine resolution fax documents whenever possible.)

1.39g - Improves support for Windows and DOS applications printing

through the Proprinter emulation which use line feeds rather than

page ejects for moving to the next page.

1.39h - Adds support for a new version of the SatisFAXtion/400 card

(the new version that provides 16550A emulation on it data modem port

changed an ID value that we check to identify the SatisFAXtion card).

1.39i - Fixes a couple minor problems of 1.39h

1.40a-h - Alpha releases for the added hooks for developer API.

(Contact Keller Group for information on the new developer API and

Enhanced Printer Driver toolkits.)

1.41 - Fixes a string problem with the WS adapter Adds new beta test

adapter for Brooktrout BTCALL 3.x Adds alpha test adapter for

Commetrex MultiFax

1.41b - Adds a count of spooled items to the status window.  Fixes a

program exit problem with cover sheet preview when it runs out of

system resources to create the bitmap.  Adds FxServerStatus to

developer API

1.41c - Fixes log print and printer driver help Adds new >>APPEND and

>>PASTE to Enhanced Printer Driver.  Adds FxWin calls to developer API

1.41d - Fixes IBM L40 modem support (fixed adapters for previous

builds were also released).  Fixes trap in some situations when

changing configuration values.

1.41e - Adds support for up to 1023 installed OS/2 fonts (limit was

274).  Adds new >>POSITION, >>LINE, >>BOX and >>ARC to Enhanced

Printer Driver and retry count to end of >>TO.

1.41f - Adds support for using a modem's stored numbers (S=0, S=1,

S=2 or S=3) within phone numbers.  Many modems ignore everything

after these strings in the phone number, but some (PPI 14400FXSA?)

allow you to use these stored numbers and append additional

characters.  The FMD.SYS device driver for fax modems will now

support up to 48 ports (previously the limit was four ports).

Several 4-port and 8-port serial boards have been tested, and it is

now possible to run large numbers of fax modems with the multiline

versions of the software.  For fax modems, the limitation of 40

characters for the dial string is now removed.  The dial string can

be as long as necessary, and the program will automatically block it

into multiple commands to get around the 40 character limit in most

fax modems.

1.42 - Corrects a memory problem with printer driver.  Supports

GammaFax version 5.0 software.

1.43 - Optimized submission of fax broadcasts from OS/2 workstations

(LAN version) - It's now much faster to send a document to multiple

recipients from an OS/2 workstation.  Non-'future spooled' fax

submission from an OS/2 workstation no longer will be delayed if

workstation clock is not synchronized with the server (blank

date/time in log means ASAP).  Close/Shutdown without prompt - When

closing the program, you will no longer be prompted if you are in

receive mode (but not CURRENTLY sending or receiving a fax).

1.43b - Removed debugging info from Fmd driver.

1.43c - Fixed time w/no date in log on future spooled faxes.

Improved shared LPT server alert for new Dos workstation interface.

1.44 - Updated Intel Satisfaxtion distinctive ring option (one long

ring) so no # key is required.  Some printer driver bug fixes (trap

after printing a page that included bitmaps, better forms querying).

Changed background fax thread to not wait for status window updates

(could cause problem on very busy system and class-1 modems).

1.44b - Changed printer driver to temporarily remove application

regions before clearing page bitmap.

1.45 - MultiFax adapter improvements (status window page updates,

answer rings configuration).  Added workaround for a bug in the 8514

video adapter that caused a 'disappearing' mouse pointer during erase.

1.45b&c - More MultiFax adapter improvements.  (MultiFax is special

firmware from Commetrex which allows Natural MicroSystems voice

boards to be used as fax boards.)

1.46 - Added support for 'new' Intel SatisFAXtion 400 (with 16550A

emulation, without any scanner port).  Intel changed chips on the

card which causes our adapter to get send failures.  Added support

for multiple Intel SatisFAXtions of different types (you can now mix

/200's, /400's, and new /400's in the same PC with the multiline

version of the fax software).

1.47 - Improved Intel SatisFAXtion board selection based on

information from Intel.  Fixed semaphore deadlock between the fax

software and FxAPI.DLL when using reporter functions that access the

log.  Fixed "drive-only" paths specified to install program, to you

can now correctly install into the root directory of virtual LAN

drives (such as G:).

1.48 - More MultiFax adapter improvements.  Added debug logging to

some background operations.  Fixed some temporary file names that can

cause problems in a shared LAN situation.  Added international

characters to font sample

1.49 (FW 1.39.10) - Fixed Intel SatisFAXtion sometimes cutting off

bottom 1/4" of page.  Changed FxRdr (LAN) to solve error 12 problem

on UNIX file server.  Fixed printer driver to handle PP X24E

'doublewide to end-of-line' graphics bug.  Adjusted printer driver

page lengths in PP X24E mode to closer match PP X24E.  Changed

printer driver to better handle TCP/IP NFS slow LAN.

1.50 (FW 1.39.11) - Fixed -L option problem when specifing a drive (e.g.

-Ln:).  Fixed encode problem in printer driver (broken in 1.49).

1.51 (FW 1.39.12) - Added semaphore to control resource allocation when

creating cover sheets in multi-line version.

1.52 (FW 1.39.13) - Fixed API dialer function line number problem.

2.00 - First 32-bit release... compiled with IBM C set++, changed

character handling for double-byte character sets, changed to use

'proper' country based collation order for sorts, changed to use a

re-sizable container for log display, install creates WP object for

program, all screens updated for 'new' look designed for better

multi-language support, printer driver renamed to 'FxPrint', all

options changed to 'drop-in', ...

2.01 - Updated for OS/2 2.1 Service Pack video problems and some

bugfixes from the first beta testers (Thanks!).

2.02 - Improved marking-moving-sizing-cutting-pasting tracking code.

Improved start-restart (starting a second copy just activates the

first, instead of giving error box).  Added all product options to

Beta test: multiline (up to 96), LAN, OCR, PCL printer driver,

Enhanced printer driver toolkit, API toolkit.

2.03 - Toolbar! You can add and delete tools, turn it off, make it

vertical, etc.  Just drag items from the menu to the bar to add, drag

them off the bar to delete, drag them around to rearrange. Also,

multiple page delete, multiple copy print.

2.04 - Beta test bugfix -- Thanks for your input!

2.05 - Drag and drop fax assembly!  Drag Tiff-F (FAX), DCX files and

drop in fax workspace to open, drop on viewed fax to append pages,

drop on any insert pages button to insert. Drop Tiff-F on print

button to print.  Drop on send button to send.  Drag BMP, PCX, and

text file and drop on fax to import.  Also various bugfixes.

2.06 - Fmd.sys now detects PCMCIA and automatically skips the port

tests.  Drag-and-drop fonts for message line, log, and phonebooks.

Right-button menus for fax manipulation.  New look for the LAN fax

redirector. Support for new Brooktrout BTDLLS 3.4, Intel SatisFAXtion

400 download 3.52, and Gammafax 5.1.

2.07 - No buttons on log and phonebook, just popup menus.  Optimized

log, phonebook, routing, viewer, fax deleting, and log cleanup.

Fixed Fmd 2.06 FIFO problem.  Fixed PCX paste of color files with

multiple bit planes.  Increased line length for paste ASCII file (was

1024, now 16k).  Cover page comment can now be formatted with

carriage returns and spaces.  Fixed XGA2 problems. Increased page

header length to 80 characters (from 40).  Added first letter search

to log, phonebook, and routing.  Added -p option to workstations to

retain previous ID rather than querying the system.

2.08 - Bugfixes from previous release (Thanks for your input!): Fixed

fax window to not popup on receive mode changes.  Fixed Open/Edit to

begin send on Spool log entry changes.  Fixed 'can't do send popup'

error from printer driver.  Also: Route added to log menu. Send notes

default added to settings notebook.  Cover sheet font choice added to

settings notebook.  Modifications to Class-1 14.4 Fax Modem handling

for better reception.

2.08b - Fixed fax-merge and log-entry through FxPrint printer driver

(broke in release 2.07).  Added cover sheet printing.

2.08c - Fixed FxPrint to return proper forms count to Gpi.  Changed

dial macro entry screen to trim leading and trailing spaces from

macros and check for duplicates.  Added 'letter dialing' for all fax

hardware types, using quotes to delimit, for example: 1-800-"FAXFAXF".

2.08d - Changed to use cover page font for page headers and allow

bitmap fonts to be used in cover page and page headers.

2.09 - Changed to real text filling for cover comment.  Changed to

increase cover comment length to 1000 characters.  Fixed a file name

conflict that affects multi-line (You would get SysErrs on send).

Added a page header option to add "To:" information to each page.

Added "Open" button to FxPrint popup.

2.10 - Added two new fields to phonebook, one for first name and one

for voice number.  First name and old name field are joined when

phonebook records are Spooled.  Voice number is added to log record

for easier display (with Open/Edit).  New print option to add time

stamp to printed faxes.  Bugfix in FxRdr to fix spurious BadSnd.

2.11 - On-line help system is now fully updated.  Log record name

field expanded to allow longer first+last names.  Other minor

bugfixes.

2.12 - Added SOM fax object to enable drap-and-drop fax printing.

Work around for stack quirk in OS/2 PM (affected builds 2.09-2.11).

Fixed FxRte auto-routing (broken in builds 2.06-2.11).

2.12b - Fixed another PM stack quirk.

2.12c - Added -f and -s options: -fm = minimize, -fx = maximize,

-fi = invisible, -sb = status in background, -si = status invisible.

2.12d - Fixed a couple minor bugs.  Added -ss = status show (to show

status box at startup even if no activity yet), and -s# = status nfax

(to set the maximum number of fax lines showing in the status window,

default is 8).

2.13 - Added page scrolling during text entry.  Fixed FxConvert in

API to properly close source files in converting multiple DCX files

to Tiff.  Fixed case mapping in log and phonebook search in

French-Canadian country code.  Fixed FxPrint trap in verbose debug

log with long >> commands.  Fixed Brooktrout 'empty fax' SysErr

(actually a HangUp).

2.13b - Fixed status window hide button (it didn't clear totals).

2.14 (FW 2.00.00) - Minor fixes for translated versions.

2.14b - FxRdr -v file now called 'rout'.  Install program improved

for auto-install and CDROM install.  PrnDrv program added to Enhanced

Printer Driver kit for easier install.  Added -c option to allow

country and code page specification (e.g. -c33,850 for France).

2.14c - Added multi LAN groups to FxRdr.  Old -c FxRdr option moved

to settings screen as copy/delete checkbox.  Added CID options and

return codes to install program and other code page fixes.

2.15 (FW 2.00.01) - Now supports AMT MM9424 8-line fax card.  Added

more CID options to install program.  Fixed FxPrint for new Describe

image handling.  Added separate notify commands to FxRdr groups.

NOTE: Make sure ALL THE PRODUCT PIECES (base product verses upgrade and

multiline files) you are using ARE OF THE SAME BUILD NUMBER.  They

are not always compatable between builds!