6400 Flying Cloud Drive

Eden Prairie, MN 55344

(612) 943-9020

DigiBoard, GmbH

Schmittgasse 64

51143 Koeln

Germany

49-2203-81083

Installation Guide

for

DigiBoard Intelligent

Asynchronous Communications Boards

Novell AIO Driver

DBI 92000115D

DigiBoard(R) is a registered trademark of Digi International Inc.  DigiWARE ,

MC/Xi , PC/Xi , 8Port , 4Port , 2Port , PC/8e , PC/4e , PC/2e ,

MC/8e , MC/4e  and MC/2e  are trademarks of Digi International Inc.  All other

brand and product names are the trademarks of their respective holders.

(C) Digi International, Inc.  1991-1993

All Rights Reserved

Information in this document is subject to change without notice and does not represent

a commitment on the part of DigiBoard.

DigiBoard provides this document "as is," without warranty of any kind, either ex-

pressed or implied, including, but not limited to, the particular purpose.  DigiBoard

may make improvements and/or changes in this manual or in the product(s) and/or the

program(s) described in this manual at any time.

This product could include technical inaccuracies or typographical errors.  Changes are

periodically made to the information herein; these changes may be incorporated in new

editions of the publication.

Introduction

Overview

DigiBoard's support for Novell compatible asynchronous applications is

provided through a universal driver conforming to the "AIO" (Asynchronous

Input/Output) specification.  Use the AIO driver when DigiBoard hardware is

being installed in the file server (or machine with run-time NetWare 3.X).  When

DigiBoard hardware is installed in a machine other than the server (for instance

NetWare Access Server v1.2) use the specific driver for that application.

AIO drivers are loaded on the file server from the system console (or

RCONSOLE) command line or the AUTOEXEC.NCF file.  Installation is simply

a matter of copying of the driver files AIODGXI.NLM and AIODGMEM.NLM.

Compatibility

The DigiBoard AIO driver supports the DigiBoard PC/Xi, PC/Xe, MC/Xe and

MC/Xi boards and therefore can be used on ISA, EISA and MCA servers.

The driver will support up to seven DigiBoard PC/Xe or PC/Xi boards on ISA

and EISA bus servers.  These boards may be configured to use the same memory

address ("stacked") or may be configured for unique addresses.  The driver

supports 8K, 64K and 128K versions, although your server may not have an

open memory space for a 128K PC/Xi board (PC/Xi boards may, however, be

configured to use 64K).

On MCA bus servers the driver will support up to four 32k DigiBoard MC/Xi

boards and/or seven MC/Xe boards.  Your server may be limited to two MC/Xi

boards due to conflicts with the network adapter card and ESDI hard disk

controller.

Installation

To install the DigiBoard AIO driver, simply copy the relevant files to your file

server (although the driver can be loaded from the file server floppy, this will not

be a typical way to set-up a system).  The DigiBoard AIO driver files

(AIODGXI.NLM and AIODGMEM.NLM) should both be copied to the system

directory of the file server, typically called F:\SYSTEM.  Copy the files  from a

workstation to the file server with the commands:

COPY A:\AIODGXI.NLM F:\SYSTEM

and

COPY A:\AIODGMEM.NLM F:\SYSTEM

PC/Xe and PC/Xi Installation

DigiBoard PC/Xe and PC/Xi boards may be installed in either ISA or EISA

platforms.  In either case, the driver must be loaded for each board you wish to

support.  The syntax for loading the driver is as follows:

load aiodgxi port=xxx mem=xxxxx [name=bname][node=0][size=8k]

[altp[=mask]]

port=xxx

This parameter is the board's 3 digit hex I/O port address.  For

example: port=320.  Valid port addresses are 100, 110,

120, 200, 220, 300 and 320.

Each board must have a unique I/O port address, which must

not be used by any other device in the system.

mem=xxxxx

This parameter is the 5 digit hex starting address of the board's

dual-ported memory.  For example: mem=d0000.  The device

driver supports 64K starting addresses (80000, 90000,

A0000, B0000, C0000, and D0000-typically C0000 and

D0000 will be the only 64K starting addresses available in

your server) and, on newer PC/Xe boards (which have a single

bank of four DIP switches in the end plate instead of two

banks on top of the board), 8K starting addresses.  There are

eight 8K starting addresses in each 64K region-for example,

in the 64K block beginning with D0000, valid 8K starting

addresses are D0000, D2000, D4000, D6000, D8000,

DA000, DC000 and DE000.

All PC/Xe and PC/Xi boards may share the same memory

addresses, since only one board's memory is turned on at a

time.

name=bname:

This parameter is optional and allows you to specify up to a 10

character name to identify the board.  bname may consist of

any combination of alphanumerical characters.  If you do not

specify a name, one will be assigned by the driver.  Following

is a list of the board's I/O port addresses and the associated

default names and board numbers.

Port Address   Default   Board

Name     Number

100h        DigiXi00    0

110h        DigiXi01    1

120h        DigiXi02    2

200h        DigiXi03    3

220h        DigiXi04    4

300h        DigiXi05    5

320h        DigiXi06    6

node=0

This parameter must be used if you are using AIO.NLM version

1.0.2 or earlier.  These versions of AIO need sequential board

numbers based on the load order of the driver.  (The first board

loaded will be assigned #0, the second board will be assigned #1

and so on.)  Using the node=0 parameter informs the driver to

assign board numbers sequentially.  If this parameter is excluded,

the driver will assign board numbers based on the I/O port address

as defined above.

size=8k

This parameter is optional and is effective only when used with

newer PC/Xe boards (which have a bank of four DIP switches in

the end plate instead of two banks on top of the board).  When this

parameter is specified, the driver configures the board to use an 8K

host memory window instead of the default 64K window.  If this

parameter is not specified, the newer PC/Xe boards will use a 64k

memory window.

altp:

The standard wiring for boards equipped with RJ-45 connectors

places the modem control lines RI (Ring Indicator) and DCD (Data

Carrier Detect) on pins 1 and 10, respectively, which makes them

inaccessible to an 8-pin RJ-45 plug.  While RI is seldom used,

DCD is necessary for most modem applications.  To facilitate the

use of 8-pin RJ-45 plugs, a software feature called "ALTPIN" has

been included with this device driver.  When ALTPIN is active,

DCD is swapped with DSR (Data Set Ready), which is on pin 1 of

an 8-pin RJ-45 plug.

This parameter is optional and allows you specify that you want to

activate the driver's ALTPIN option.  When this parameter is

entered as just "altp", all ports on the board will operate with the

ALTPIN option active.  When entered as "altp=mask" the driver

uses the hex mask to determine which ports will use the ALTPIN

feature.  For example :

altp=f0

This will activate ALTPIN on ports 5-8. The mask can be up to 4

hex digits long to activate ALTPIN on up to 16 ports.

IRQs on PC/Xe and PC/Xi boards should be disabled.  If

your board has two banks of DIP switches, turn all eight

switches in DS2 to OFF (away from the board).  If your

board has one bank of switches in the end plate, the

interrupts will automatically be disabled by the device

driver.

MC/Xe and MC/Xi Installation

The driver will also support up to four DigiBoard MC/Xi boards, and/or seven

MC/Xe boards on MCA (Micro Channel) bus servers.

Configuring the MC/Xi for 32K Operation

Due to DOS memory restrictions, the MC/Xi boards must be set for 32K

operation to be used with the Novell Access Server software.  This is

accomplished by changing a jumper on the board.

Please note that only the newer MC/Xi boards (Part #30001224) may be

configured in this manner.  The part number can be found next to the serial

number of the Processor board:

Jumper J1 needs to be set for 32K operation before installing in the system.

Refer to the manuals that came with your boards.

This jumper setting information applies only to the

newer MC/Xi boards (part number 30001224).

Loading the AIO Driver

There are two ways to load the driver on these servers:  You may load the driver

for one board at a time or you may autoload the driver for all boards in the

system (if you do not specify values for port, mem or name, the driver will

auto-load all MC/Xi and MC/Xe boards present in the system).  The syntax for

loading the driver is as follows :

load aiodgxi    [port=xxxx mem=xxxxx [name=bname]]

[node=0] [altp[=mask[,mask[,mask[,mask]]]]]

port=xxxx

This parameter is optional and specifies the MC/Xi

board's 4 digit hex I/O port address.

For example : port=F1F0

Valid port addresses are F1F0, F2F0, F4F0 and

F8F0 for MC/Xi boards, and 108, 118, 128, 208,

228, 308 and 328 for MC/Xe boards.

This parameter is used when you are loading the

driver to support just one specific board and must be

used with the parameter "mem=".  If you do not use

these two parameters, the driver will autoload support

for all MC/Xi or MC/Xe boards found in the system.

mem=xxxxx

This parameter is optional, and specifies the 5 digit

hex starting address of the board's dual-ported

memory.  For example: mem=d0000.  For MC/Xi

boards, the device driver supports four 32K starting

addresses (C0000, C8000, D0000 and D8000).

For MC/Xe boards, the device driver supports two

64K starting addresses (C0000, and D0000) and

sixteen 8K starting addresses.  There are eight 8K

starting addresses in each 64K region-for example,

in the 64K block beginning with D0000, valid 8K

starting addresses are D0000, D2000, D4000,

D6000, D8000, DA000, DC000 and DE000.

name=bname:

This parameter is optional when using the "mem="

and "port=" parameters, and allows you to specify

up to a 10 character name to identify the board.

bname may consist of any combination of

alphanumerical characters.  If you do not specify a

name, one will be assigned by the driver.  Following

is a list of the board's I/O port addresses and the

associated default names and board numbers.

Port Address   Default   Board

Name     Number

108        DigiXi00    0

118        DigiXi01    1

128        DigiXi02    2

208        DigiXi03    3

228        DigiXi04    4

308        DigiXi05    5

328        DigiXi06    6

F1F0h      DigiXi07    7

F2F0h      DigiXi08    8

F4F0h      DigiXi09    9

F8F0h      DigiXi10   10

node=0

This parameter must be used if you are using

AIO.NLM version 1.0.2 or earlier.  These versions of

AIO need sequential board numbers based on the load

order of the driver.  (The first board loaded will be

assigned #0, the second board will be assigned #1 and

so on.)  Using the node=0 parameter informs the

driver to assign board numbers sequentially.  If this

parameter is excluded, the driver will assign board

numbers based on I/O port address as defined above.

altp:

The standard wiring for boards equipped with RJ-45

connectors places the modem control lines RI (Ring

Indicator) and DCD (Data Carrier Detect) on pins 1

and 10, respectively, which makes them inaccessible

to an 8-pin RJ-45 plug.  To facilitate the use of 8-pin

RJ-45 plugs, a software feature called "ALTPIN" has

been included with this device driver.  When

ALTPIN is active, DCD is swapped with DSR (Data

Set Ready), which is on pin 1 of an 8-pin RJ-45 plug.

This parameter is optional and allows you specify

that you want to activate the driver's ALTPIN option.

When this parameter is entered as just "altp", all

ports on the board will operate with the altpin option

active.  When entered as "altp=mask" the driver

uses the hex mask to determine which ports will use

the ALTPIN feature.  For example :

altp=f0

This will activate ALTPIN on ports 5-8.

When autoloading support for all boards in the server,

you may specify up to four masks, one for each

possible board.  For example:

altp=1,8,10,80

In this example the first port on the first board, the

fourth port on the second board, the fifth port on the

third board and the eighth port on the fourth board

will use ALTPIN.  The mask can be up to 4 hex digits

long to activate ALTPIN on up to 16 ports.

When autoloading, the driver uses the MCA bus's

POS (Programmable Option Select) capabilities to

determine the MC/Xi boards that are installed in your

server, starting with slot 1.  The first ALTPIN mask

will activate ALTPIN on the board in the lowest

numbered slot, the second mask for the next lowest

slot, and so on.

Error Messages

The following error messages may occur while the driver is parsing the

parameters.

*Usage Error: AIODGXI port=XXXX mem=XXXXX [name=bname].

This error will appear if you load the driver with no parameters on ISA

and EISA bus servers.

*Usage Error: First parameter not 'port='.

The driver expects the first parameter to be port=xxxx, but this was

not the case.

*Error: Invalid Port Address 0x310.

This message indicates that an invalid port address was specified.

*Error: Port Address 0x320 is already supported.

This message indicates that the driver is already supporting a board at

this port address.

*Usage Error: Second parameter not 'mem='

The driver expects the second parameter to be mem=xxxxx, but this

was not the case.

*Error: Invalid Memory Address 0xF0000.

This message indicates that an invalid memory address was specified.

*Error: Memory Address 0xC0000 is already supported.

This message indicates that the driver is already supporting a board at

the specified memory address.  This message will only be seen on

MCA bus servers since MC/Xi boards must uses different memory

addresses.

The following error messages may occur while the driver is performing some

initial tests on the board.

Board 4: Port 220 Mem D0000 Port 220 not found.

This message indicates that the driver could not find a board with port

220h.  Check to insure that you have specified the correct I/O port or

that you have properly configured the board's port address.

Board 4: Port 220 Mem D0000 Hardware Reset Fails.

This message indicates that the driver could not place the board into a

reset state.  This is most likely the result of an I/O port address conflict

or a faulty board.  Be sure that there are no other I/O ports in your

system at the indicated I/O port address.

Board 4: Port 220 Mem D0000 PC/Xi Memory size too Large.

This message indicates that your PC/Xi board has more than 128k and

is not supported by the driver.

Board 4: Port 220 Mem D0000 Memory not found error x.

This message indicates that the driver could not find the board's

memory.  x indicates the number of memory errors found in a 256 byte

block.

Board 4: Port 220 Mem D0000 Memory error x.

This message indicates that the driver has found the board's memory,

but it appears to be faulty.  x indicates the number of errors found in a

256 byte block.

Board 4: Port 220 Mem D0000 Memory fails disable.

This message indicates that the board's memory could not be disabled.

This message will not occur on MC/Xi boards.

The following messages may occur during the drivers attempt to initialize the

board.

Board 4: Port 220 Mem D0000 BIOS fails initialization.

This message indicates that the board's on-board BIOS did not

initialize properly.

Board 4: Port 220 Mem D0000 BIOS passes init but reports 0 ports.

This message indicates that the BIOS initialized successfully, but could

not find any of the asynchronous channels.

Board 4: Port 220 Mem D0000 NMI Fails during FEPOS code move.

This message indicates a failure during transfer of the on-board

operating system code.

Board 4: Port 220 Mem D0000 NMI Fails during FEPOS execution.

This message indicates a failure during startup of the on- board

operating system.

Board 4: Port 220 Mem D0000 FEPOS Fails Initialization.

This message indicates that the on-board operating system did not

initialize successfully.

Board 4: Port 220 Mem D0000 Command failure during Initialization.

This message indicates that the on-board operating system encountered

a failure during initialization of the asynchronous channels.