Novell AIO Driver

for Stallion Technologies ONboard/Brumby boards.

Rev 1.0 Issue 1

Hardware Configuration

Before installing the Novell AIO Driver software, the board(s)

should be configured ready for installation. Up to four boards may

be installed into the system. EISA and Microchannel boards will be

configured automatically by your configuration utility, but sufficient

addresses and interrupts should be available for the number of boards

to be installed.

To configure your hardware:

1.      For each board, choose a memory address and an I/O address that

will avoid clashes with any other hardware you may have in your

system. Multiple ONboards may share the same memory address.

2.      Make note of any installation parameters set (ie. the  addresses

for each board), as they will also be required to install the

Novell AIO Driver software.

3.      ISA boards only:  Set the DIP switches on the board(s) according

to the chosen addresses, by consulting the tables in the section

titled 'DIP Switches and Jumper Settings'.

4.      If you are upgrading any of your ONboards for more ports (a 4 port

to an 8 port), install the expansion module. In most cases the

ONboard is supplied with the expansion module installed. See the

section entitled "ONboard Expansion Modules", for more information

on expansion module installation procedures.

TO PREVENT STATIC

DAMAGE AVOID EXCESSIVE

BOARD HANDLING

ONboard Expansion Modules

A key feature of the ONboard family architecture is its expandability.

An expansion module can upgrade each ONboard to handle up to 8 ports.

Up to 4 ONboards can be installed in any system.

The ONboard family currently supports a plug-on module to add four

extra serial I/O ports and memory.  This is the EXP4 module.  It

expands a basic 4 port card to handle 8 ports.

In most cases, an ONboard with eight serial ports is shipped with the

plug-on module already attached.

To upgrade an ONboard, plug the expansion module into the ONboard

base card connectors CN-2 and CN-3.  Polarizing pins prevent you from

installing the board with the wrong orientation, provided that only

minimal force is used.

Hardware Installation

The other step in the installation process is to install the hardware

into the machine.

1.      'Down' the server and power down the machine.  Consult the

computer system installation manual for the correct cover

removal procedures.

2.      For each board, locate an available slot.  Push the board

into the slot, making sure it is securely in place.

3.      Connect the I/O panel(s).

4.      Configure the board(s).

Note:

The following information is to be noted when installing ISA

boards in an EISA machine.

It is recommended to configure ISA boards in an EISA machine,

using the system's EISA Configuration Utility.  Hardware

conflicts can easily be detected and resolved using this

method.  If ISA boards are not configured, the AIO Driver

installation requires full board details to be entered when

loading the driver.  Refer to 'Installing the AIO Driver

Software' (step 11) for more details.

If ISA boards only are being installed, and they are not being configured,

proceed to step 5.

The Multiport Driver diskette contains configuration files required

by EISA machines to configure EISA and ISA boards, and by Microchannel

machines to configure MCA boards.

Configure your EISA/Microchannel machine to recognise the installed

board(s) using the respective system configuration utility. Consult

Hardware Vendor's manuals for instructions on this utility.

Follow these steps to reconfigure your EISA/Microchannel machine:

(a)     Power-on and boot-up your machine using the diskette containing

the system configuration utility supplied with your machine.

(b)     If you are not using a working copy of this utility diskette,

then make a working copy using the system configuration utility.

Restart the machine from the working copy.

(c)     Use the system configuration utility to copy the configuration

file found in the root directory of the installation diskette,

!STL0100.CFG    -       ONboard/E (EISA) in an EISA machine

!STL0120.CFG    -       ONboard (ISA) in an EISA machine

!STL0130.CFG    -       Brumby (ISA) in an EISA machine

@6091.ADF       -       ONboard2 (MCA) in a Microchannel machine

to your copy of the configuration utility diskette.

(d)     Save the new configuration and exit the utility.

(e)     Upon reboot to the system configuration utility, the configuration

file information will be updated and a valid DPRAM address and I/O

address will be selected automatically from the choices available.

(f)     Select 'disabled' in the interrupt field.

Note: The drivers are polling drivers and do not require an

interrupt request line (IRQ) to operate.  By selecting 'disabled',

available interrupts are left accessible to hardware that does

require an interrupt.

(g)     Save the new configuration, exit the system configuration utility,

and reboot your system.

5.      One message for every board installed will be displayed on your

screen during your machine's boot process.  These indicate that

the driver was loaded successfully and has found and booted the

board specified.

If the board(s) fail to be recognized, with the parameters

conforming to the requirements, the DIP switches may have been

set incorrectly, an address clash may have occurred, or the board

may be faulty.  Check your DIP switch settings, then ensure that

the addresses chosen are available for use.  If these values are

valid, use the board diagnostic test described in this chapter to

check for a faulty board.

Hardware Connections

Serial devices are attached to Stallion Technologies boards by various

I/O cables.  These cables are:

�       4 port DB25 cable               (Brumby 4-port)

�       8 port DB25 cable               (ONboard 4- and 8-port)

�       8 port RJ45 connector   (ONboard 4- and 8-port)

For peripheral cabling information, refer to Appendix C.

DIP Switches and Jumper Settings

The ISA architecture I/O boards, require that a number of DIP switches

and jumpers be set before board operation. The ISA architecture boards

include all Brumby boards and the ONboard ISA.

Brumby DIP Switch Settings

The Brumby board has one eight position dip switch (SW1) which sets the

dual port RAM buffer and I/O addresses. The RAM address is selectable on

16KB boundaries in the address range 0C0000 - 0DFFFF. The I/O address is

selectable on 16 byte boundaries (ie. the last digit must always be a zero)

in the address range 0300 - 03FF. The defaults for the first board (D0000

and 360) are highlighted in the following table.

Valid DIP Switch (SW1) Settings for Brumby Board

DIPSwitch #                             I/O     RAM

1       2       3       4       5       6       7       8     Address  Address

OFF     OFF     OFF     OFF                                     3F0

OFF     OFF     OFF     ON                                      3E0

OFF     OFF     ON      OFF                                     3D0

OFF     OFF     ON      ON                                      3C0

OFF     ON      OFF     OFF                                     3B0

OFF     ON      OFF     ON                                      3A0

OFF     ON      ON      OFF                                     390

OFF     ON      ON      ON                                      380

ON      OFF     OFF     OFF            Reserved for             370

ON      OFF     OFF     ON            Memory Address            360

ON      OFF     ON      OFF                                     350

ON      OFF     ON      ON                                      340

ON      ON      OFF     OFF                                     330

ON      ON      OFF     ON                                      320

ON      ON      ON      OFF                                     310

ON      ON      ON      ON                                      300

OFF     OFF     OFF     OFF             DC000

OFF     OFF     ON      OFF             D8000

OFF     ON      OFF     OFF             D4000

Reserved for            OFF     ON      ON      OFF             D0000

I/O Address             ON      OFF     OFF     OFF             CC000

ON      OFF     ON      OFF             C8000

ON      ON      OFF     OFF             C4000

ON      ON      ON      OFF             C0000

Switch 8 of SW1 should always be OFF

ONboard DIP Switch Settings

The I/O register DIP switch is at the bottom of the card. This is the

only switch that may need to be changed before installation. The memory

address and interrupt line (IRQ) are set-up automatically by the software.

The address of the ONboard's I/O register can be set by selecting one of

the dip switch arrangements listed in the following table.

Valid DIP Switch (SW1) Settings for ONboard

Dip Switch #

1               2               3               4              I/0 Addr

ON              ON              ON              ON              200

ON              ON              ON              OFF             210

ON              ON              OFF             ON              220

ON              ON              OFF             OFF             230

ON              OFF             ON              ON              240

ON              OFF             ON              OFF             250

ON              OFF             OFF             ON              260

ON              OFF             OFF             OFF             270

OFF             ON              ON              ON              280

OFF             ON              ON              OFF             290

OFF             ON              OFF             ON              2A0

OFF             ON              OFF             OFF             2B0

OFF             OFF             ON              ON              2C0

OFF             OFF             ON              OFF             2D0

OFF             OFF             OFF             ON              2E0

OFF             OFF             OFF             OFF             2F0

ONboard Jumper Settings

16-bit operation (J2)

To ensure data transfers between the system and the ONboard ISA are 16

bits wide, jumper J2 should be installed and the card should reside in

a 16 bit slot. This ensures that the host system can copy data to the

card more efficiently and improves performance by up to 10%. If the card

is in 8 bit mode (i.e. J2 removed), it will operate in either an 8 or 16

bit slot depending on the memory address entered by the user during the

software installation.

Advanced I/O channel timing (J3)

The ONboard (ISA) will also operate in PC AT systems that do not adhere

to original IBM PC AT I/O Channel timing. By installing jumper J3 the

ONboard (ISA) will respond to memory requests much earlier than the

standard bus timing. This is necessary in systems that operate the I/O

channel bus at greater than 8 MHz. It is imperative that the jumper J3

is never installed in IBM RT systems. Most 80386 systems should have J2

and J3 installed if 16 bit operation is required. In the default

configuration J2 and J3 are removed. Advanced bus timing is not necessary

for 8 bit operation.

Board Diagnostics

Diagnostic software, DIAG.EXE, has been supplied for the purpose of testing

the state of the board(s). This software tests the hardware functionality

of the board(s) and includes memory and loopback diagnostics tests.

DIAG can be found on the Multiport Driver diskette and is run under DOS.

INSTALL DIAG <target drive spec>

and run it from the \STALLION\DIAG directory.

To use the diagnostic facility:

1.      Boot your system with a DOS boot diskette.

2.      Insert the Multiport Driver diskette into the chosen floppy drive.

3.      To select the DIAG directory, enter the following at the DOS prompt

of the chosen drive:

CD \DIAG

4.      To start the board diagnostics, type:

DIAG

Note:

Several command line options are available to select I/O address

and memory address used by the board(s) during testing.

Command line options are:

-b      BRD_TYPE                board type menu option

-a      IO_ADDR         set I/O address of board

-r      RAM_ADDR                set the RAM address

For example, DIAG -b2 -a360 -rd0000 invokes diagnostics for an

ONboard at I/O address 360 and memory address D0000.

5.      Select the board type from the menu.

6.      The settings for the board are displayed on the screen.  If these

values are incorrect, change the configuration by entering the

configuration menu, type:

5

and set the configuration to the correct values. <ESC> to get back

to the Diagnostics menu.

7.      Run the required tests using diagnostic options 1 - 4.

8.      If the board fails diagnostics, it may only be suffering from an

address clash.  Check that the addresses chosen are available for

use.

9.      If the board still fails diagnostic tests, contact your supplier.

Installing the AIO Driver Software

The installation of the Asynchronous I/O (AIO) Driver software is executed

from the Novell NetWare Server prompt.

To install the AIO Driver, perform the following steps:

1.      Ensure that your Stallion Technologies Multiport board(s) are

configured and installed, using the section titled

"Hardware Procedures".

2.      If you have not done so already, start the NetWare Server from the

DOS prompt.

3.      At the server prompt, load the INSTALL NLM with the following

command:

load install

4.      If your NetWare software precedes version 4.0, proceed to step 5.

Select the Maintenance/Selective Install menu option.

5.      Select Product Options from the menu.  A list of currently installed

products is displayed.

6.      Press <INSERT> to install the AIO Driver.

7.      Place the Multiport Driver diskette in the floppy drive of your

choice.

8.      Select the drive which contains the diskette.

Pre-NetWare V4.0:   Enter the drive and/or path to the

product source media.

NetWare V4.0 onwards:   Select the default drive, or

change it as required.

A message, showing the driver installation process, is displayed

to the screen.

9.      Once the product is installed, exit from this task.

Pre-NetWare V4.0:       Exit back to main menu.

NetWare V4.0 onwards:   Exit back to the

"Maintenance/Selective Install" menu.

10.     Choose the options menu item to load the hardware.

Pre-NetWare V4.0:       Select the "System Options" menu

option.

NetWare V4.0 onwards:   Select the "NCF Files Options" menu

option.

11.     Add the entry to load the board.

(a)     Select the "Edit AUTOEXEC.NCF" menu option.

(b)     Choose the line(s) to be added depending on the system's

bus architecture.

Note: The command "LOAD AIOSTLN ?" entered at the server

prompt will give a usage message.

ISA machines (ONboard, Brumby)

LOAD AIOSTLN   [BTYPE=bbbbbb]  [NODE=n]  PORT=ppp

MEM=mmmmmm  [CACHE=c]  [DSRHIGH]

where:

BTYPE=bbbbbb

Defines the board type to be used.

By leaving the BTYPE parameter out,

the board type defaults to 'ONboard'.

To specify that the board is a Brumby,

set BTYPE=brumby.

NODE=n

Assigns a specified node ID to the board.

It is recommended for ID assignment to

begin at zero (0).  All successive boards

should be assigned ID values in increments

of one from the first board. For example:

Load AIOSTLN NODE=0 PORT=...

Load AIOSTLN NODE=1 PORT=...

Load AIOSTLN NODE=2 PORT=...

The value is entered in hexadecimal. If not

used, node IDs are assigned automatically.

PORT=ppp

Specifies the I/O port address used by the

board.  The default value is 240.  Valid

addresses range from 200 - 2F0 and are

entered in hexadecimal format.

ONboard only:  The value set must match

the DIP switch (SW1) settings on the board.

MEM=mmmmmm

Specifies the Dual Port RAM address used

by the board.  The default value is 0D0000,

or if using ONboards, the default is the

DPRAM address of the previously loaded

ONboard.  The value is entered in hexadecimal.

Brumby only:  The value set must match the

DIP switch settings on the board.

CACHE=c

Specifies the caching control used by the

machine.  The value is entered in decimal.

Note:   Memory addresses below 1Mb (0FFFFF) are not

subject to caching.  (If they are, they can

be configured not to be cached using the

machine's BIOS configuration utility.)

The default is no cache control.  A list of machines

directly supported follows:

c       Machine

1       Compaq Deskpro 386

2       Compaq Deskpro 386/20e

3       Compaq Deskpro 386/25

4       Compaq Deskpro 386/25e

5       Compaq Deskpro 386/33

6       Compaq Deskpro 486/25

7       Dell 310

8       Dell 325

9       ITT XTRA XL

10      Hewlett Packard RS-20c

11      Hewlett Packard RS-25c

12      Hewlett Packard QS-20

13      Everex Step 25

14      Everex Step 33

15      ALR Flex Cache

16      Mylex 386/20

17      Mylex 386/25

18      ASC 25

19      Acer 1100/25

20      Acer 1100/33

DSRHIGH

Forces DSR to always be reported as HIGH.

For example, a Brumby and two ONboards might be

configured as:

LOAD AIOSTLN BTYPE=brumby NODE=0 PORT=200 MEM=C8000

LOAD AIOSTLN NODE=1 PORT=240 MEM=D0000 DSRHIGH

LOAD AIOSTLN NODE=2 PORT=250 MEM=D0000 DSRHIGH

EISA machine (ONboard/E, ONboard, Brumby)

Note:

�       When using an EISA machine, ISA boards (ONboard,

Brumby) can be loaded in one of two ways.

The recommended method is to configure them the

same way that an EISA board would be configured

(ie. requiring board configuration using the

EISA Configuration Utility).  Refer to step 4 of

'Hardware Installation' for configuration details.

If ONboard (ISA) and Brumby boards are not configured

using the configuration utility, the LOAD AIOSTLN

statement for ISA machines (above) should be used.

�       If the SLOT parameter is not specified, only one

LOAD AIOSTLN command need be supplied for all

'configured' boards to be found.

LOAD AIOSTLN  [NODE=n]  [SLOT=s]  [DSRHIGH]

where:

NODE=n

Assigns a specified node ID to the board.

It is recommended for ID assignment to begin at

zero (0).  If the slot parameter is not specified,

each successive board is assigned ID values in

increments of one from the first board.  For example,

the load command,

Load AIOSTLN NODE=0

assigns a node ID = 0 to the first board, while

automatically setting the node values to 1 and 2

for the second and third boards. (Remember, the

second and third boards do not require any additional

LOAD AIOSTLN commands.  Refer to the note above.)

The value is entered in hexadecimal.  If not used,

node IDs are assigned automatically.

SLOT=s

Specifically states the slot number of the slot

that contains the board.  The value is entered in

decimal.  If this parameter is not specified for

each board, only one LOAD AIOSTLN command need be

entered for all Stallion boards to be located and

configured.

DSRHIGH

Forces DSR to always be reported as HIGH.

MCA machine (ONboard2)

LOAD AIOSTLN   [NODE=n]  PORT=ppp  MEM=mmmmmm  [DSRHIGH]

where:

NODE=n

Assigns a specified node ID to the board.  It is recommended

for ID assignment to begin at zero (0).  All successive

boards should be assigned ID values in increments of one

from the first board.  For example:

Load AIOSTLN NODE=0 PORT=...

Load AIOSTLN NODE=1 PORT=...

Load AIOSTLN NODE=2 PORT=...

The value is entered in hexadecimal.  If not used, node IDs

are assigned automatically.

PORT=ppp

Specifies the I/O port address used by the board.  The

default value is 780.  Valid addresses range from

780 - 7F0 and are entered in hexadecimal format.

MEM=mmmmmm

Specifies the Dual Port RAM address used by the board.

The default value is 0D0000, or the DPRAM address of the

previously loaded ONboard.  The value is entered in hex.

DSRHIGH

Forces DSR to always be reported as HIGH.

(c)     Add the line(s) to the AUTOEXEC.NCF file.  Order of

execution for each of the AUTOEXEC commands in this file

is important.  Refer to Chapter 6 (step 1) of the NetWare

Asynchronous Communications Services Administration Guide

for instructions on the position to place the LOAD AIOSTLN

entries in the AUTOEXEC.NCF file.  The Stallion Technologies

boards install in a similar way to the boards described.

12.     Exit and reboot the system in order for the changes to take effect.

Note:

For cabling information when installing the AIO Driver, refer to

the NetWare Asynchronous Communications Services Administration

Guide.

Driver Limitations

1.      A four (4) board maximum limit has been set.

2.      The following baud rates are not supported by the Novell NetWare

Drivers and are therefore mapped to comparable baud rates:

3600    mapped to 4800

>19200  mapped to 19200   (Brumby boards only)

>38400  mapped to 38400    (ONboards only)

3.      Not all combinations of data bits/stop bits are supported, so the

following stop bit mappings apply:

In conjunction with 5 data bits:

Stop Bits Selected      Stop Bits Provided

1                       1

1.5                     1.5

2                       1.5

In conjunction with 6-8 data bits:

Stop Bits Selected      Stop Bits Provided

1                       1

1.5                     2

2                       2

4.      The RS232 signal, DSR, is not supported by the boards.  By default,

the driver will report it to be the same as DCD. If the DSRHIGH

parameter is specified in the LOAD AIOSTLN command, it will always

be reported as high.