Copyright 1992 Gateway Communications, Inc.

WNIM-II AIO Driver Information

October 1993

Gateway Communications, Inc.

2941 Alton Avenue

Irvine, California 92714, USA

TEL: (800) 367-6555      FAX: (714) 553-1616

TEL: (714) 553-1555      BBS: (714) 863-7097

Copyright 1993 Gateway Communications, Inc.

All rights reserved.

INTRODUCTION

The AIOWNIM2.NLM is the AIO Driver for Gateway's Wide-area Network

Interface Module II (WNIM-II) adapter. This document contains the

necessary information to load the WNIM-II AIO Driver and lists

status/error messages that may display while loading and unloading the

AIO Driver.

I/O ADDRESS SWITCH SETTINGS

I/O Address:    WNIMs shall not share I/O addresses. Choice of 1 of 16

address blocks (SWITCH selectable)

In the following table. 0 is Switch on, and 1 is Switch

off.

Device Address range             SWITCH Positions

hexadecimal notation             4    2    3    1

280     -    28F                 0    0    0    0

290     -    29F                 0    0    0    1

2A0     -    2AF                 0    0    1    0

2B0     -    2BF                 0    0    1    1

2C0     -    2CF                 0    1    0    0

2D0     -    2DF                 0    1    0    1

2E0     -    2EF                 0    1    1    0

2F0     -    2FF                 0    1    1    1

300     -    30F                 1    0    0    0

310     -    31F                 1    0    0    1

320     -    32F                 1    0    1    0

330     -    33F                 1    0    1    1

340     -    34F                 1    1    0    0

350     -    35F                 1    1    0    1

360     -    36F                 1    1    1    0

370     -    37F                 1    1    1    1

LOADING THE WNIM-II AIO DRIVER

To load the WNIM-II AIO Driver, use the following syntax:

load AIOWNIM port=pppp mem=mmmm name=nnnn

where pppp is the device I/O address of the WNIM-II adapter that

you are loading. Available device I/O addresses are

0280, 0290, 02A0, 02B0, 02C0, 02D0, 02E0, 02F0, 0300,

0310, 0320, 0330, 0340, 0350, 0360, and 0370.

NOTE:     When installing WNIM-II adapters in a Novell

server, make sure that you reserve device I/O

address ranges that you will be using or

problems may occur.

mmmm is the base memory address of the WNIM-II adapter that

you are loading. Available base memory addresses are

A0000, A2000, A4000, A6000, A8000, AA000, AC000, AE000,

B0000, B2000, B4000, B6000, B8000, BA000, BC000, BE000,

C0000, C2000, C4000, C6000, C8000, CA000, CC000, CE000,

D0000, D2000, D4000, D6000, D8000, DA000, DC000, DE000,

E0000, E2000, E4000, E6000, E8000, EA000, EC000, EE000.

NOTE:     If you are loading more than one WNIM-II

adapter in the server PC, then the base memory

address specified on the command line for the

first board must be used for the second,

third, and fourth boards.

Furthermore, if you load the first board

without specifying any parameters, then the

default base memory address (D0000) will be

used; therefore, you must use D0000 for any

other WNIM-II adapters you install.

nnnn is the user-defined name to be assigned to the WNIM-II

adapter you are loading. The name may not contain space

characters.

If you omit all of the parameters, then the AIO Driver will load each

WNIM-II adapter (in order) using the defaults as follows:

Device I/O Address       Base Memory Address          Name

Board 0          0280                     D0000             None

Board 1          0290                     D0000             None

Board 2          02A0                     D0000             None

Board 3          02B0                     D0000             None

WNIM-II AIO DRIVER STATUS AND ERROR MESSAGES

Following is a list of status/error messages that may display while

loading and unloading the AIO Driver. Also included with each message,

is an explanation of the message and what to do when it occurs.

NOTE:     There are no messages displayed during the normal

operation of the AIO Driver. That is, after the driver

load operation is complete and before the driver unload

operation is initiated, no messages will be displayed.

IMPORTANT

If the action instructions provided for a given error do not

correct the error or there is no action provided for the error,

then the error may have been caused by either incorrectly keyed in

commands (at the server PC's keyboard, or the driver was loaded

incorrectly.

Message 1

AIO WNIM Driver unloaded.

Reason:   The driver unload operation initiated by the operator is

complete.

Message 2

Error: AIODeregisterDriver failed (ccode=xxxx).

Reason:   A call to the system function "AIODeregisterDriver"

failed during the unload operation.

Action:   Use the system monitor to verify whether or not the AIO

Driver is resident in the server PC's memory.  If it is,

try the unload operation again.

Message 3

*Error: AIORegisterBoard failed (ccode=xxxx).

Reason:   A call to the system function "AIORegisterBoard" failed,

causing the driver load operation to abort.

Action:   Use the driver unload command to unload the AIO Driver

from the server PC's memory. Then use the driver load

command to load each WNIM-II adapter in the following

order:

Board Number             Port I/O Address

0                        0280h

1                        0290h

2                        02A0h

3                        02B0h

Message 4

Error: AIORegisterDriver failed (ccode=xxxx).

Reason:   A call to the system function "AIORegisterDriver"

failed, causing the driver load operation to abort.

Action:   Check to see if an AIO Driver has already been loaded

and registered.

Message 5

*Error: AllocateResourceTag failed.

Reason:   A call to the system function "AllocateResourceTag"

failed, causing the load operation to abort.

Action:   No action is possible.  The system is out of resource

tags.

Message 6

*Error: Unable to register Hardware Options for WNIM.

Reason:   The AIO driver is unable to register the specified

hardware configuration with the NetWare operating

system, causing the driver load operation to abort.

Action 1: Verify that the I/O address (device I/O address) is not

being used by another device that is installed in the

server PC.

Action 2: Also verify that the memory address (base memory

address) is not being used by a device which cannot

share the memory with other devices installed in the

server PC.

Message 7

Error: Unable to release Port yy (ccode=xxxx).

Reason:   The driver was unable to release the port during the

driver unload operation.  The error code returned from

the release port function may indicate that the AIO

Driver's tables are corrupted.

Action:   None.

Message 8

*Error: WNIM-II did not initialize.

Reason:   The WNIM-II adapter failed to complete its

initialization sequence within the specified time,

causing the driver load operation to abort.

Action 1: Verify that the WNIM-II adapter's hardware configuration

for the port I/O address (device I/O address) match the

port I/O address specified with the driver load command.

Action 2: Verify that each WNIM-II adapter installed in the server

PC is set to a unique port I/O address.

Action 3: If a valid port I/O address and memory address were

specified with the driver load command, then the WNIM-II

adapter may be defective. Try loading a different WNIMII adapter using the same port I/O address that was used

for the WNIM-II adapter that failed.

Message 9

*Error: WNIM Driver called to many times.

Reason:   The driver AIOWNIM has been called more than four times.

A maximum of four WNIM-II adapters (boards) can be

initialized by the AIO Driver.

Action:   Verify that the AUTOEXEC.NCF file contains no more than

four load commands for the driver (AIOWNIM) or make sure

that no more than four load commands for the driver are

entered via the server PC's keyboard.

Message 10

WNIM Board x using port yyyy at address zzzzz is installed.

Reason

The AIO Driver has successfully initialized the specified

WNIM-II adapter (board).

where

x         is the number of the WNIM-II adapter (board)

that has been initialized

yyyy      is the port I/O address (device I/O address)

used by the specified WNIM-II adapter

zzzzz     is the memory address (base memory address)

used by the specified WNIM-II adapter

Message 11

*Error: Unable to Parse Driver Parameters.

Reason:   The load command contained parameters that the driver

didn't recognize.

Action:   Verify that the WNIM-II driver load command contains

only those options specified above.

Message 12

Error: Board presence test failed.

Reason:   The WNIM-II AIO driver was unable to detect the presence

of a WNIM-II at the specified port I/O and memory

addresses.

Action 1: Verify that the WNIM-II adapter's hardware configuration

for the port I/O address (device I/O address) match the

port I/O address specified with the driver load command.

Action 2: Verify that each WNIM-II adapter installed in the server

PC is set to a unique port I/O address.

Action 3: If a valid port I/O address and memory address were

specified with the driver load command, then the WNIM-II

adapter may be defective. Try loading a different WNIMII

adapter using the same port I/O address that was used

for the WNIM-II adapter that failed.

Message 13

*Error: WNIM-II Board returned bad number of ports.

Reason:   The driver read an invalid number for the ports

supported by the WNIM-II board being loaded.

Action 1: Verify that the WNIM-II adapter's hardware configuration

for the port I/O address (device I/O address) and memory

address do not conflict with other hardware installed in

the computer.

Action 2: the WNIM-II adapter may be defective. Try loading a

different WNIM-II adapter using the same port I/O

address that was used for the WNIM-II adapter that

failed.

Message 14

Error: AIODeregisterBoard failed (ccode=xxxx).

Reason:   A call to the system function "AIODeregisterBoard"

failed during the unload operation.

Action:   Use the system monitor to verify whether or not the AIO

Driver is resident in the server PC's memory.  If it is,

try the unload operation again.

TRADEMARKS

WNIM is a registered trademark of Gateway Communications, Inc.

Novell and NetWare are registered trademarks of Novell, Inc.

Copyright 1992 Gateway Communications, Inc.