MEMOREX TELEX CORP. RELEASE NOTES

PowerRAID Administrator

Version 4.11

03-02-94

Part Number 210047-04

Release Notes for Memorex Telex Software Porducts are provided for

users to enhance productivity by minimizing start-up time and problem

diagnosis.  These notes are provided in the following format:

I.  General Information - This section contains information

that clarifies or assists the user with Software Product

utilization.  Included may be information which attempts

to clarify documentation on Third Party software.

II.  Third Party Notes - This section contains Release Notes that

come with Third Party Software Products that are included

with Memorex Telex Software Products (eg - MS-DOS 6.00 and

MCS EISA Configuration Utility).

III.  Memorex Telex Third Party Notes - This section contains

additional information and/or problems related to Third

Party software that have been discovered by Memorex Telex.

I.  General Information

The PowerRAID Administrator is a windows-like utility used to create

RAID configurations.  See the PowerRAID User Guide for a description of

how PowerRAID works and for a description of the PowerRAID Administrator.

The Release Notes provides both hardware and software configuration

information.

A new FORMAT utility is also provided in the PowerRAID Administrator.

Use this FORMAT utility rather than the one provided on the IIOP

Drivers/Utilities Diskette.

The PowerRAID Administrator has a feature on the Physical Device Map

that provides physical device information including the revision number.

The revision number may not be documented on the outside of a device or

if documented is out of date.  For a complete description, see the

PowerRAID Administrator User Guide.

CONFIGURATION LIMITATIONS

Memorex Telex 8630/8640

1.  BIOS 22, Part Number 370177-22, or later, must be used with the

Enhanced IIOP.

2.  The Enhanced IIOP uses active termination of the SCSI bus.  The SCSI

cable set which provides connectors for the active terminators must

be used.  The passive terminators on the last disk drive of a SCSI

bus must be removed and an active terminator installed on the end of

the SCSI cables.

3.  Earlier versions of the IIOP and the Enhanced IIOP can not be used in

the same system.  The BIOS does not allow the system to boot.

4.  Verify the PAL in location U63 on the Enhanced IIOP is Part Number

370089-05.

CONFIGURATION LIMITATIONS

Memorex Telex 8650

1.  Firmware Part Number 370356-00/370357-00, or later, must be used

with the ISS.

OPERATING SYSTEM SUPPORT

1.  New drivers have been developed for Netware 3.11, OS/2 1.3, UNIX 3.2.4,

and UNIX/FT products.  Ensure the version of the operating system you

are using has been updated or includes the Enhanced IIOP driver that

supports the PowerRAID features.

2.  Older versions of UNIX 3.2.4 will not support the Enhanced IIOP

with or without the PowerRAID features.

NEW FEATURES AND ENHANCEMENTS

1.  Version 4.11 for the Memorex Telex 8650 has the ability to identify

a physical device as having been a logical device. (This happens

when you "reset" the logical configuration or delete a logical

device.)  The new status that is seen in PowerRAID is "Ex-Logi",

meaning formerly a logical device.

2.  The warning message that pops-up when the user does a "reset" is

now more descriptive and accurate.

3.  All capacities are now displayed as a decimal number of bytes.  For

example, 1 Megabyte is now 1 million bytes instead of 1048576 bytes.

This was done because all the disk drive manufacturers do this same

thing, even though it doesn't seem quite right.

4.  Added a line in the PDM display for <TAB, Shift-TAB> that allows

the user to tab over to the next column.  There can arise a

situation when the arrow keys don't let you get to the next column.

II.  Third Party Notes

None.

III. Memorex Telex Third Party Notes

None.