SERVILE SOFTWARE DECODER

Version 2.5

Shareware Version Disk Manual

#2

DISCLAIMER

The Servile Software Decoder is supplied as a tool to assist in the reverse-

engineering of IBM PC software, with special attention to computer viruses.

Because of the nature of the subject, no guarrantees can be made that a

program analysed with SSD will behave in the same manner when executed outside

of the SSD environment.

Months of development and rigourous testing have gone into SSD, including

testing it on hundreds of Shareware, freeware and commercial programs,

including computer viruses. Some programs, by their nature, exhibit pecuiliar

behaviour when analysed with SSD which they do not exhibit otherwise.

Use of SSD is at your own risk. The author accepts no liability in any form

and makes no guarrantees as to the suitability of this product for any purpose

what so ever.

#3

INTRODUCTION

The Servile Software Decoder (SSD) is a powerful code and program analyser for

the IBM PC. SSD is neither a disassembler nor a debugger, although it shares

some similarities with both. Unlike a disassembler, SSD activately analyses

the code of a program, this allows SSD to analyse encrypted programs, unlike a

disassemler. Unlike a debugger, SSD interprets the program under analysis.

This has the advantage that SSD is safe to use with computer viruses as

dangerous commands are inhibited. It also implies that SSD is not prone to

anti-debugger instructions which are often contained in computer viruses.

Because SSD interprets programs, rather than passing them straight to DOS for

execution, some programs may exhibit slightly different behaviour when

analysed than when executed. This is unfortunately unavoidable.

When SSD starts execution, or loads a new file, the PC interrupt vector table

is copied into memory. All subsequent reads and writes to the first 1024 bytes

of RAM (vector table) are then redirected to the SSD copy of the interrupt

vector table. This allows programs to think they have changed an interrupt

vector, but at the same time retains security by not changing the real vector.

Animate            Record             Utility               File            Exit

������������Trace���������������������������������Analysis��������������������ͻ

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                       �������������About������������ͻ                       �

�                       �   Servile Software Decoder   �                       �

�                       �                              �                       �

�                       �         Version 2.5          �                       �

�                       �     (Shareware Version)      �                       �

�                       �                              �                       �

�                       �     Copyright (c)1994 by     �                       �

�                       �                              �                       �

�                       �       Servile Software       �                       �

�                       ������������������������������ͼ                       �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

������������������������������������������������������������������������������͹

�  AX  0000   BX  0000   CX  0000   DX  0000   SI  0000   DI  0000   BP  0000  �

�  CS  53C8   DS  53B8   ES  53B8   SS  552C   SP  0080   IP  0000    0000000  �

�                                                                     ZCSOAPD  �

����������Registers�����������������������������������������������������������ͼ

#4

DISPLAY

SSD uses two display pages. Page 0, the normal default DOS display page is

used for output from the program being analysed. Page 1 is used for the SSD

main display which is divided into the following component parts: Menu bar,

containing drop-down menus of facilities. Code window, which displays the

dissasembly of each instruction as it occurs. Analysis window, which displays

analysis information about the running program and also data dumps when

requested. And the registers window which displays the current contents of the

CPU registers as they appear to the program being analysed.

Animate            Record             Utility               File            Exit

������������Trace���������������������������������Analysis��������������������ͻ

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

������������������������������������������������������������������������������͹

�  AX  0000   BX  0000   CX  0000   DX  0000   SI  0000   DI  0000   BP  0000  �

�  CS  53C8   DS  53B8   ES  53B8   SS  552C   SP  0080   IP  0000    0000000  �

�                                                                     ZCSOAPD  �

����������Registers�����������������������������������������������������������ͼ

#5

FACILITIES

This section describes the various facilities available from the drop-down

menus. Each menu is tackled in turn, with the menu name shown in CAPITALS,

followed by the facilities available from that menu.

ANIMATE:

The "ANIMATE" menu provides access to the various facilities for running the

loaded program. In brief, the loaded program may be single stepped, run at

maximum speed or traced at three different speeds. A program which is being

traced or run may be stopped by pressing the Esc key. When single stepping a

program only the next instruction is fetched and interpreted.

Animate            Record             Utility               File            Exit

��������ͻ

� Step   ���Trace���������������������������������Analysis��������������������ͻ

� Run    �                   �                                                 �

� Slow   �                   �                                                 �

� Medium �                   �                                                 �

� Fast   �                   �                                                 �

��������ͼ                   �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

������������������������������������������������������������������������������͹

�  AX  0000   BX  0000   CX  0000   DX  0000   SI  0000   DI  0000   BP  0000  �

�  CS  53C8   DS  53B8   ES  53B8   SS  552C   SP  0080   IP  0000    0000000  �

�                                                                     ZCSOAPD  �

����������Registers�����������������������������������������������������������ͼ

#6

Step:

Single step the loaded program. Only the next instruction is interpreted, and

then SSD returns to the ANIMATE menu.

Run:

Run the program at maximum speed. Only the analysis details are displayed.

Slow:

Trace the program slowly. As each instruction is fetched and interpreted, its

dissamembly and analysis are displayed togther with the new contents of all

the registers. However, should the program be in a loop command, only the

effect upon the registers is displayed untill the loop finishes.

Medium:

Trace the program fairly slowly. As each instruction is fetched and

interpreted, its dissamembly and analysis are displayed togther with the new

contents of all the registers. However, should the program be in a loop

command, only the effect upon the registers is displayed untill the loop

finishes.

Fast:

Trace the program quickly. As each instruction is fetched and interpreted, its

dissamembly and analysis are displayed togther with the new contents of all

the registers. However, should the program be in a loop command, only the

effect upon the registers is displayed untill the loop finishes.

#7

RECORD:

Toggle which details will be echoed to an output file. The details are

cumulative. Selecting an option low in the list will result in details higher

in the list also being recorded. Selecting "Code & Regs" results in all

details being recorded.

A tick (�) appears next to the currently selected level of recording.

SSD requires you to enter a name of a file to which the details will be

echoed. If you want the details echoed to the printer enter the file name PRN.

This can be useful for tracing code which destroys the display, such as

tracing interrupt 10 for example. Notice that if the printer is switched off,

off-line or out-of-paper SSD will continue regardless, but nothing will be

echoed to the printer until the problem is resolved.

Animate            Record             Utility               File            Exit

��������������ͻ

������������Trace�ͺ �None        ����������������Analysis��������������������ͻ

�                  �  Analysis    �                                            �

�                  �  Code        �                                            �

�                  �  Code & Regs �                                            �

�                  ��������������ͼ                                            �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

������������������������������������������������������������������������������͹

�  AX  0000   BX  0000   CX  0000   DX  0000   SI  0000   DI  0000   BP  0000  �

�  CS  53C8   DS  53B8   ES  53B8   SS  552C   SP  0080   IP  0000    0000000  �

�                                                                     ZCSOAPD  �

����������Registers�����������������������������������������������������������ͼ

#8

UTILITY:

Miscellaneous utility facilities.

About....:

Displays information about SSD. After reading the information displayed, press

any key to remove the "about" window from the display.

Change Regs:

Allows any register value to be changed. Highlight the required register, and

press return to enter a new value.

Trace Int:

Allows an interrupt to be specified instead of a program. To trace the

interrupt use the ANIMATE facilities as for a program. (CS and IP are set to

the values stored in the appropriate interrupt vector).

Dump:

Display the values in RAM at a specified segment:offset address. Registers may

be specified for the address, eg: es:di is a valid address.

Output:

Display the program's output screen. Press a key to return to the analysis

screen.

Interrupts:

Displays the values recorded in the interrupt vector table (SSD copy). To

continue the listing press any key, pressing Esc will terminate the listing.

#9

Animate            Record             Utility               File            Exit

�������������ͻ

������������Trace��������������������ͺ About....   �lysis��������������������ͻ

�                            �        � Change Regs �                          �

�                            �        � Trace Int   �                          �

�                            �        � Dump        �                          �

�                            �        � Output      �                          �

�                            �        � Interrupts  �                          �

�                            �        �������������ͼ                          �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

������������������������������������������������������������������������������͹

�  AX  0000   BX  0000   CX  0000   DX  0000   SI  0000   DI  0000   BP  0000  �

�  CS  53C8   DS  53B8   ES  53B8   SS  552C   SP  0080   IP  0000    0000000  �

�                                                                     ZCSOAPD  �

����������Registers�����������������������������������������������������������ͼ

#10

FILE:

Facilities for opening and writing program files and also for specifying

command line arguments.

Open:

Allows you to specify a program to load.

Write:

Writes a memory image of the current .COM program to disk. Reports an error if

the loaded program is a .EXE program.

Arguments:

Allows command line arguments to be specified. These are then passed to the

program being analysed.

Animate            Record             Utility               File            Exit

�����������ͻ

������������Trace���������������������������������Analysis�ͺ Open      ������ͻ

�                            �                              � Write     �      �

�                            �                              � Arguments �      �

�                            �                              �����������ͼ      �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

�                            �                                                 �

������������������������������������������������������������������������������͹

�  AX  0000   BX  0000   CX  0000   DX  0000   SI  0000   DI  0000   BP  0000  �

�  CS  53C8   DS  53B8   ES  53B8   SS  552C   SP  0080   IP  0000    0000000  �

�                                                                     ZCSOAPD  �

����������Registers�����������������������������������������������������������ͼ

EXIT:

Returns to DOS

#11

INHIBITED COMMANDS

SSD inhibits all attempts to disable hardware by writing to I/O ports,

generation of audible tones, attempts to create new files, closing of the

standard files; stdin, stdout, stderr, stdaux and stdprn, write to disk and

disk formatting instructions. Instructions which attempt to write to memory

occupied by SSD are also rejected.

Inhibiting operations allows a greater degree of safety when analysing

computer viruses, but has the disadvantage that subsequent program behaviour

may be altered in some circumstances.

SSD inhibits attempts to change the following interrupt vectors:

08, 0A, 0B, 0C, 0D, 0E, 0F, 1C, 21, 24, 70

However, subsequent reads of these vectors will return a value as though the

change had occured.

#12

DOS ERROR CODES

DOS error codes are usually returned in the AX register following an abortive

DOS interrupt call, usually indicated by a set carry flag.

Code    Meaning

01     Invalid function

02     File not found

03     Path not found

04     No handles available

05     Access denied

06     Invalid handle

07     Memory control blocks destroyed

08     Insufficient memory

09     Invalid memory block address

0A     Invalid environment

0B     Invalid format

0C     Invalid access code

0D     Invalid data

0F     Invalid drive

10     Attempt to remove current directory

11     Not the same device

12     No more files

13     Disk write-protected

14     Unknown unit

15     Drive not ready

16     Unknown command

17     CRC error

18     Bad request structure length

19     Seek error

1A     Unknown media type

1B     Sector not found

1C     Out of paper

1D     Write fault

1E     Read fault

1F     General failure

20     Sharing violation

21     Lock violation

22     Invalid disk change

23     FCB unavailable

24     Sharing buffer overflow

#13

MEMORY MAP

The following table describes the layout of conventional RAM within the IBNM

PC and compatibles. Each address is shown in the format segment:offset as is

conventional with Intel's approach to memory addressing with the 8080 family

of CPUs.

0000:0000               RAM

Vector Table

256 entries stored as segment word

offset word

0040:0000               BIOS data area

:0000               Base I/O address of 1st serial I/O port, zero if none

:0002               Base I/O address of 2nd serial I/O port, zero if none

:0004               Base I/O address of 3rd serial I/O port, zero if none

:0006               Base I/O address of 4th serial I/O port, zero if none

:0008               Base I/O address of 1st parallel I/O port, zero if none

:000A               Base I/O address of 2nd parallel I/O port, zero if none

:000C               Base I/O address of 3rd parallel I/O port, zero if none

:000E               Base I/O address of 4th parallel port, zero if none

Segment of Extended BIOS Data Segment (PS/2)

:0010               Installed hardware

:0012               POST status

:0013               Base memory size in kbytes (0-640)

:0015               Manufacturing test scratch pad

:0016               Manufacturing test scratch pad

BIOS control flags (PS/2 mod 30)

Keyclick loudness 00h-7Fh (Compaq Deskpro 386K)

:0017               Keyboard status flags 1

:0018               Keyboard status flags 2

:0019               Keyboard: Alt-nnn keypad workspace

:001A               Keyboard: ptr to next character in keyboard buffer

:001C               Keyboard: ptr to first free slot in keyboard buffer

:001E               Keyboard circular buffer

:003E               Diskette recalibrate status

:003F               Diskette motor status

:0040               Diskette motor turn-off time-out count

:0041               Diskette last operation status

:0042               XT: command byte to hard disk controller

AT: write precompensation cylinder number / 4

:0043               XT: bit 5 = drive number, bits 3-0=head number

AT: sector count

:0044               XT: bits 6,7 = high bits of track, bits 5-0 = start

sector-1

AT: starting sector

:0045               low byte of track number

:0046               XT: sector count

AT: high bits of track number

:0047               XT: control byte from HD parameters (step rate,...)

AT: 101DHHHH, D=drive number, HHHH=head number

14

:0048               XT: INT 13 subfunction number

AT: comand byte to hard disk controller

:0049               Current video mode

:004A               Video columns on screen

:004C               Video page (regen buffer) size in bytes

:004E               Video current page start address in regen buffer

:0050               Video cursor position (col, row) for eight pages

:0060               Video cursor type, hi=startline, lo=endline

:0062               Video current page number

:0063               Video CRT controller base address

:0065               Video current setting of mode select register

:0066               Video current setting of CGA palette register 03D9h

:0067               POST real mode re-entry point after certain resets

pointer to reset code upon system reset (PS/2 except

mod 25,30) with memory preserved

:006B               POST last unexpected interrupt

:006C               Timer ticks since midnight

:0070               Timer overflow

:0071               Ctrl-Break flag

:0072               POST reset flag

:0074               Fixed disk last operation status, except ESDI drives

:0075               Number of fixed disk drives

:0076               Fixed disk control byte

:0077               Fixed disk I/O port offset

:0078               Parallel devices 1-3 time-out counters

:007B               parallel device 4 time-out counter (non-PS/2 and PS/2

Mod 25,30)

:007C               Serial devices 1-4 time-out counters

:0080               Keyboard buffer start as offset from segment 40h

:0082               Keyboard buffer end+1 as offset from segment 40h

:0084               Video EGA/MCGA/VGA rows on screen minus one

:0085               Video EGA/MCGA/VGA character height in scan-lines

:0087               Video EGA/VGA control

:0088               Video EGA/VGA switches

:0089               Video MCGA/VGA mode-set option control

:008A               Video MCGA/VGA index into Display Combination Code

table

:008B               Diskette media control (not XT)

:008C               Fixed disk controller status (not XT)

:008D               Fixed disk controller Error Status (not XT)

:008E               Fixed disk Interrupt Control (not XT)

:008F               Diskette controller information (not XT)

:0090               Diskette drive 0 media state

:0091               Diskette drive 1 media state

:0092               Diskette drive 0 media state at start of operation

:0093               Diskette drive 1 media state at start of operation

:0094               Diskette drive 0 current track number

:0095               Diskette drive 1 current track number

:0096               Keyboard status byte 1

:0097               Keyboard status byte 2

:0098               Timer2

:009C               Timer2

:00A0               Timer2

:00A1               LAN Support Program Interrupt Arbitrator present

15

:00A2               Reserved for network adapters

:00A4               Saved Fixed Disk Interrupt Vector (PS/2 Mod 30)

:00A8               Video: EGA/MCGA/VGA ptr to Video Save Pointer Table

:00AC               Reserved

:00B0               Vendor specific

:00CE               Count of days since last boot

:00D0               Reserved

:00F0               Reserved for user

:0100               Print Screen Status byte

:010E               State of BREAK at start of BASICA.COM execution

:010F               BASICA v2.10 running flag

:0116               INT 1B at start of BASICA.COM execution

:011A               INT 24 at start of BASICA.COM execution

0040:011E               DOS kernel

????:????               Device drivers

????:????               Resident part of COMMAND.COM

????:????               FREE SPACE

A000:0000               EGA/VGA Video RAM

B000:0000               Video RAM

C000:0000               EGA/VGA BIOS ROM extension

C000:8000               Hard drive ROM extension

E000:0000               Other BIOS ROM extensions

F000:0000               BIOS ROM

:EFC7               Floppy disk parameters

:F0A4               Video hardware table

:FEF3               Initial interrupt vector table values

:FFF5               BIOS version date code

:FFFE               Model type

:FFFF               Model sub type

#16

XT, AT and PS/2  I/O port addresses

This data is provided for the purpose of providing an understanding into the

nature of a program being analysed which accesses hardware ports. It is not

intended to describe how to program the IBM PC hardware ports since this can

lead to damage of the hardware.

"r" indicates port may be read. "w" indicates port may be written. "r/w"

indicates port may be both read and written.

0000-001F ----  DMA 1   (first Direct Memory Access controller 8237)

0000    r/w     DMA channel 0 address    byte 0, then byte 1.

0001    r/w     DMA channel 0 word count byte 0, then byte 1.

0002    r/w     DMA channel 1 address    byte 0, then byte 1.

0003    r/w     DMA channel 1 word count byte 0, then byte 1.

0004    r/w     DMA channel 2 address    byte 0, then byte 1.

0005    r/w     DMA channel 2 word count byte 0, then byte 1.

0006    r/w     DMA channel 3 address    byte 0, then byte 1.

0007    r/w     DMA channel 3 word count byte 0, then byte 1.

0008    r       DMA channel 0-3 status register

bit 7 = 1  channel 3 request

bit 6 = 1  channel 2 request

bit 5 = 1  channel 1 request

bit 4 = 1  channel 0 request

bit 3 = 1  channel terminal count on channel 3

bit 2 = 1  channel terminal count on channel 2

bit 1 = 1  channel terminal count on channel 1

bit 0 = 1  channel terminal count on channel 0

0008    w       DMA channel 0-3 command register

bit 7 = 1  DACK sense active high

= 0  DACK sense active low

bit 6 = 1  DREQ sense active high

= 0  DREQ sense active low

bit 5 = 1  extended write selection

= 0  late write selection

bit 4 = 1  rotating priority

= 0  fixed priority

bit 3 = 1  compressed timing

= 0  normal timing

bit 2 = 1  enable controller

= 0  enable memory-to-memory

0009    w       DMA write request register

#17

000A    r/w     DMA channel 0-3 mask register

bit 7-3 = 0   reserved

bit 2   = 0   clear mask bit

= 1   set mask bit

bit 1-0 = 00  channel 0 select

= 01  channel 1 select

= 10  channel 2 select

= 11  channel 3 select

000B    w       DMA channel 0-3 mode register

bit 7-6 = 00  demand mode

= 01  single mode

= 10  block mode

= 11  cascade mode

bit 5   = 0   address increment select

= 1   address decrement select

bit 3-2 = 00  verify operation

= 01  write to memory

= 10  read from memory

= 11  reserved

bit 1-0 = 00  channel 0 select

= 01  channel 1 select

= 10  channel 2 select

= 11  channel 3 select

000C    w       DMA clear byte pointer flip-flop

000D    r       DMA read temporary register

000D    w       DMA master clear

000E    w       DMA clear mask register

000F    w       DMA write mask register

0010-001F ----  DMA controller (8237) on PS/2 model 60 & 80

0018    w       PS/2 extended function register

001A            PS/2 extended function execute

0020-003F ----  PIC 1   (Programmable Interrupt Controller 8259)

0020    w       PIC initialization command word ICW1

bit 7-5 = 0  only used in 80/85 mode

bit 4   = 1  ICW1 is being issued

bit 3   = 0  edge triggered mode

= 1  level triggered mode

bit 2   = 0  successive interrupt vectors use 8 bytes

= 1  successive interrupt vectors use 4 bytes

bit 1   = 0  cascade mode

= 1  single mode, no ICW3 needed

bit 0   = 0  no ICW4 needed

= 1  ICW4 needed

18

0021    w       PIC  ICW2,ICW3,ICW4 after ICW1 to 0020

ICW2:

bit 7-3 = address lines A0-A3 of base vector address for PIC

bit 2-0 = reserved

ICW3:

bit 7-0 = 0  slave controller not attached to corresponding

interrupt pin

= 1  slave controller attached to corresponding

interrupt pin

ICW4:

bit 7-5 = 0  reserved

bit 4   = 0  no special fully-nested mode

= 1  special fully-nested mode

bit 3-2 = 0x nonbuffered mode

= 10 buffered mode/slave

= 11 buffered mode/master

bit 1   = 0  normal EOI

= 1  Auto EOI

bit 0   = 0  8085 mode

= 1  8086/8088 mode

0021    r/w     PIC master interrupt mask register

OCW1:

bit 7 = 0  enable parallel printer interrupt

bit 6 = 0  enable diskette interrupt

bit 5 = 0  enable fixed disk interrupt

bit 4 = 0  enable serial port 1 interrupt

bit 3 = 0  enable serial port 2 interrupt

bit 2 = 0  enable video interrupt

bit 1 = 0  enable keyboard, mouse, RTC interrupt

bit 0 = 0  enable timer interrupt

OCW2:

bit 7-5 = 000  rotate in auto EOI mode (clear)

= 001  nonspecific EOI

= 010  no operation

= 011  specific EOI

= 100  rotate in auto EOI mode (set)

= 101  rotate on nonspecific EOI command

= 110  set priority command

= 111  rotate on specific EOI command

bit 4   = 0    reserved

bit 3   = 0    reserved

bit 2-0        interrupt request to which the command applies

0020    r       PIC  interrupt request/in-service registers by OCW3

request register:

bit 7-0 = 0  no active request for the corresponding int. line

= 1  active request for corresponding interrupt line

in-service register:

bit 7-0 = 0  corresponding line not currently being serviced

= 1  corresponding int. line currently being serviced

#19

0020    w       PIC OCW3

bit 7   = 0   reserved

bit 6-5 = 0x  no operation

= 10  reset special mask

= 11  set special mask

bit 4   = 0   reserved

bit 3   = 1   reserved

bit 2   = 0   no poll command

= 1   poll command

bit 1-0 = 0x  no operation

= 10  read int.request register on next read at 0020

= 11  read int.in-service register on next read 0020

0022-002B ----  Intel 82355, part of chipset for 386sx

initialisation in POST will disable these addresses,

only a hard reset will enable them again.

0022    r/w     82335 MCR memory configuration register

0024            82335 RC1 roll compare register

0026            82335 RC2 roll compare register

0028            82335 CC0 compare register

002A            82335 CC1 compare register

values for CC0 and CC1:

00F9,0000  enable range compare CC0 0-512K  CC1 disable

00F1,0000  enable range compare CC0 0-1024K CC1 disable

00F1,10F9  enable range compare CC0 0-1M    CC1 1M-1M5

00E1,0000  enable range compare CC0 0-2M    CC1 disable

00E1,0000  enable range compare CC0 0-2M    CC1 disable

00C1,0000  enable range compare CC0 0-4M    CC1 disable

00C1,40E1  enable range compare CC0 0-4M    CC1 4M-6M

0081,0000  enable range compare CC0 0-8M    CC1 disable

0040-005F ----  PIT  (Programmable Interrupt Timer  8253, 8254)

XT & AT uses 40-43    PS/2 uses 40, 42,43,44, 47

0040    r/w     PIT  counter 0, counter divisor       (XT, AT, PS/2)

0041    r/w     PIT  counter 1, RAM refresh counter   (XT, AT)

0042    r/w     PIT  counter 2, cassette & speaker    (XT, AT, PS/2)

#20

0043    r/w     PIT  mode port, control word register for counters 0-2

bit 7-6 = 00  counter 0 select

= 01  counter 1 select   (not PS/2)

= 10  counter 2 select

bit 5-4 = 00  counter latch command

= 01  read/write counter bits 0-7 only

= 10  read/write counter bits 8-15 only

= 11  read/write counter bits 0-7 first, then 8-15

bit 3-1 = 000 mode 0 select

= 001 mode 1 select - programmable one shot

= x10 mode 2 select - rate generator

= x11 mode 3 select - square wave generator

= 100 mode 4 select - software triggered strobe

= 101 mode 5 select - hardware triggered strobe

bit 0   = 0   binary counter 16 bits

= 1   BCD counter

0044    r/w     PIT  counter 3 (PS/2, EISA)

used as fail-safe timer. generates an NMI on time out.

for user generated NMI see at 0462.

0047    w       PIT  control word register counter 3 (PS/2, EISA)

bit 7-6 = 00  counter 3 select

= 01  reserved

= 10  reserved

= 11  reserved

bit 5-4 = 00  counter latch command counter 3

= 01  read/write counter bits 0-7 only

= 1x  reserved

bit 3-0 = 00

0048            EISA

004A            EISA

004B            EISA

#21

0060-006F ----  Keyboard controller (8041, 8042)  (or PPI (8255) on XT)

XT uses 60-63,  AT uses 60-64

AT keyboard controller input port bit definitions

bit 7   = 0  keyboard inhibited

bit 6   = 0  CGA, else MDA

bit 5   = 0  manufacturing jumper installed

bit 4   = 0  system RAM 512K, else 640K

bit 3-0      reserved

AT keyboard controller input port bit definitions by Compaq

bit 7   = 0  security lock is locked

bit 6   = 0  Compaq dual-scan display

= 1  non-Compaq display

bit 5   = 0  system board dip switch 5 is ON

bit 4   = 0  auto speed selected

= 1  high speed selected

bit 3   = 0  slow (4MHz)

= 1  fast (8MHz)

bit 2   = 0  80287 installed

= 1  no NDP installed

bit 1-0      reserved

AT keyboard controller output port bit definitions

bit 7 =    keyboard data output

bit 6 =    keyboard clock output

bit 5 = 0  input buffer full

bit 4 = 0  output buffer empty

bit 3 =    reserved (see note)

bit 2 =    reserved (see note)

bit 1 =    gate A20

bit 0 =    system reset

Note:   bits 2 and 3 are the turbo speed switch or password

lock on Award/AMI/Phoenix BIOSes.  These bits make

use of nonstandard keyboard controller BIOS

functionality to manipulate

pin 23 (8041 port 22) as turbo switch for AWARD

pin 35 (8041 port 15) as turbo switch/pw lock for

Phoenix

0060    r/w     KB controller data port or keyboard input buffer (ISA, EISA)

should only be read from after status port bit0 = 1

should only be written to if status port bit1 = 0

keyboard commands (data goes also to port 0060):

ED     dbl   set/reset mode indicators Caps Num Scrl

EE     sngl  diagnostic echo. returns EE.

EF-F2  sngl  NOP (No OPeration). reserved for future use

F3     dbl   set typematic rate/delay

F4     sngl  enable keyboard

F5     sngl  disable keyboard. set default parameters

F6     sngl  set default parameters

F7-FD  sngl  NOP

FE     sngl  resend last scancode

FF     sngl  perform internal power-on reset function

22

0060    r       KeyBoard or KB controller data output buffer (via PPI on XT)

0061    w       KB controller port B (ISA, EISA)   (PS/2 port A is at 0092)

system control port for compatibility with 8255

bit 7  (1= IRQ 0 reset )

bit 6-4    reserved

bit 3 = 1  channel check enable

bit 2 = 1  parity check enable

bit 1 = 1  speaker data enable

bit 0 = 1  timer 2 gate to speaker enable

0061    r       KB controller port B control register (ISA, EISA)

system control port for compatibility with 8255

bit 7   parity check occurred

bit 6   channel check occurred

bit 5   mirrors timer 2 output condition

bit 4   toggles with each refresh request

bit 3   channel check status

bit 2   parity check status

bit 1   speaker data status

bit 0   timer 2 gate to speaker status

0061    w       PPI  Programmable Peripheral Interface 8255 (XT only)

system control port

bit 7 = 1  clear keyboard

bit 6 = 0  hold keyboard clock low

bit 5 = 0  I/O check enable

bit 4 = 0  RAM parity check enable

bit 3 = 0  read low switches

bit 2      reserved, often used as turbo switch

bit 1 = 1  speaker data enable

bit 0 = 1  timer 2 gate to speaker enable

0062    r/w     PPI (XT only)

bit 7 = 1  RAM parity check

bit 6 = 1  I/O channel check

bit 5 = 1  timer 2 channel out

bit 4      reserved

bit 3 = 1  system board RAM size type 1

bit 2 = 1  system board RAM size type 2

bit 1 = 1  coprocessor installed

bit 0 = 1  loop in POST

#23

0063    r/w     PPI (XT only) command mode register  (read dipswitches)

bit 7-6 = 00  1 diskette drive

= 01  2 diskette drives

= 10  3 diskette drives

= 11  4 diskette drives

bit 5-4 = 00  reserved

= 01  40*25 color (mono mode)

= 10  80*25 color (mono mode)

= 11  MDA 80*25

bit 3-2 = 00  256K (using 256K chips)

= 01  512K (using 256K chips)

= 10  576K (using 256K chips)

= 11  640K (using 256K chips)

bit 3-2 = 00  64K  (using 64K chips)

= 01  128K (using 64K chips)

= 10  192K (using 64K chips)

= 11  256K (using 64K chips)

bit 1-0       reserved

0064    r       KB controller read status (ISA, EISA)

bit 7 = 1 parity error on transmission from keyboard

bit 6 = 1 receive timeout

bit 5 = 1 transmit timeout

bit 4 = 0 keyboard inhibit

bit 3 = 1 data in input register is command

0 data in input register is data

bit 2 = 0 system flag status

= 0 power up or reset

= 1 selftest OK

bit 1 = 1 input buffer full (input 60/64 has data for 8042)

bit 0 = 1 output buffer full (output 60 has data for system)

0064    r        KB controller read status by Compaq

bit 7 = 1 parity error detected (11-bit format only). If an

error is detected, a Resend command is sent to the

keyboard once only, as an attempt to recover.

bit 6 = 1 receive timeout. transmission didn't finish in 2mS.

bit 5 = 1 transmission timeout error

bit 5,6,7  cause

1 0 0  No clock

1 1 0  Clock OK, no response

1 0 1  Clock OK, parity error

bit 4 = 0 security lock engaged

bit 3 = 1 data in OUTPUT register is command

0 data in OUTPUT register is data

bit 2 = 0 system flag status

= 0 power up or reset

= 1 soft reset

bit 1 = 1 input buffer full (output 60/64 has data)

bit 0 = 0 no new data in buffer (input 60 has data)

#24

0064    w       KB controller input buffer (ISA, EISA)

KB controller commands (data goes to port 0060):

20     read  read byte zero of internal RAM, this is the

last KB command send to 8041

Compaq  Put current command byte on port 0060

command structure:

bit 7   reserved

bit 6 = 1 convert KB codes to 8086 scan codes

bit 5 = 0 use 11-bit codes

= 1 use 8086 codes

bit 4 = 0 enable keyboard

= 1 disable keyboard

bit 3 = 1 ignore security lock state

bit 2     this bit goes into bit2 status reg.

bit 1 = 0 reserved

bit 0 = 1 generate int. when output buffer full

21-3F  read  reads the byte specified in the lower 5 bits of

the command in the 8041's internal RAM

60-7F  dbl   writes the data byte to the address specified in

the 5 lower bits of the command.

Alternate description KB IO command 60 summary:

bit7 = 0 reserved

bit6 =   IBM PC compatibility mode

bit5 =   IBM PC mode

bit4 =   disable kb

bit3 =   inhibit override

bit2 =   system flag

bit1 = 0 reserved

bit0 =   enableoutput buffer full interrupt

60   Compaq  Load new command (60 to [64], command to [60])

A1   Compaq  unknown speedfunction

A2   Compaq  unknown speedfunction

A3   Compaq  Enable system speed control

A4   Compaq  Toggle speed

A5   Compaq  Special reed. the 8042 places the real values

of port 2 except for bits 4 and 5 wich are given

a new definition in the output buffer. No output

buffer full is generated.

if bit 5 = 0, a 9-bit keyboard is in use

if bit 5 = 1, an 11-bit keyboard is in use

if bit 4 = 0, outp-buff-full interrupt disabled

if bit 4 = 1, output-buffer-full int. enabled

A6   Compaq  unknown speedfunction

AA     sngl  initiate self-test. will return 55 to data port

Compaq  Initializes ports 1 and 2, disables the keyboard

and clears the buffer pointers. It then places

55 in the output buffer.

AB     sngl  initiate interface test. result values:

0 = no error

1 = keyboard clock line stuck low

2 = keyboard clock line stuck high

3 = keyboard data line is stuck low

25

4 = keyboard data line stuck high

Compaq   5 = Compaq diagnostic feature

AC     read  diagnostic dump. the contents of the 8041 RAM,

output port, input port, status word are send.

AD     sngl  disable keyboard (sets bit 4 of commmand byte)

AE     sngl  enable keyboard  (resets bit 4 of commmand byte)

AF     AWARD Enhanced Command: read keyboard version

C0     read  read input port

Compaq  Places status of input port in output buffer. use

this command only when the output buffer is empty

C1     AWARD Enhanced Command: poll input port Low nibble

C2     AWARD Enhanced Command: poll input port High nibble

D0     read  read output port

Compaq  Places byte in output port in output buffer. use

this command only when the output buffer is empty

D1     dbl   write output port. next byte written  to 0060

will be written to the 8041 output port

Compaq  The system speed bits are not set by this command

use commands A1-A6 (!) for speed functions.

D2     AWARD Enhanced Command: write keyboard output buffer

D3     AWARD Enhanced Command: write pointing device out.buf.

D4     AWARD Enhanced Command: write to auxiliary device

DD     sngl  disable address line A20. default in Real Mode

DF     sngl  enable address line A20

E0     read  read test inputs. bit0 = T0 and bit1 = T1

Exx    AWARD Enhanced Command: active output port

ED   Compaq  This is a two part command to control the state

of the NumLock CpasLock and ScrollLock LEDs

The second byte contains the state to set LEDs.

bit 7-3    reserved. should be set to 0.

bit 2 = 0  Caps Lock LED off

bit 1 = 0  Num Lock LED off

bit 0 = 0  Scroll Lock LED off

F0-FF  sngl  pulse output port low for 6 microseconds.

bits 0-3 contain the mask for the bits to be

pulsed. a bit is pulsed if its mask bit is zero.

bit0=system reset. Don't set to zero. Pulse only!

0065    r       communications port (Olivetti M24)

0068    w       HP-Vectra  control buffer (HP commands)

0069    r       HP-Vectra  SVC (keyboard request SerViCe port)

006A    w       HP-Vectra  clear processing, done

006C-006F       HP-HIL  (Human Interface Link = async. serial inputs 0-7)

0070-007F ----  CMOS RAM/RTC (Real Time Clock  MC146818)

#26

0070    w       CMOS RAM index register port (ISA, EISA)

bit 7   = 1  NMI disabled

= 0  NMI enabled

bit 6-0      CMOS RAM index (64 bytes, sometimes 128 bytes)

any write to 0070 should be followed by an action to 0071

or the RTC wil be left in an unknown state.

0071    r/w     CMOS RAM data port (ISA, EISA)

RTC registers:

00    current second in BCD

01    alarm second   in BCD

02    current minute in BCD

03    alarm minute   in BCD

04    current hour in BCD

05    alarm hour   in BCD

06    day of week  in BCD

07    day of month in BCD

08    month in BCD

09    year  in BCD (00-99)

0A    status register A

bit 7 = 1  update in progress

bit 6-4 divider that identifies the time-based

frequency

bit 3-0 rate selection output  frequency and int. rate

0B    status register B

bit 7 = 0  run

= 1  halt

bit 6 = 1  enable periodic interrupt

bit 5 = 1  enable alarm interrupt

bit 4 = 1  enable update-ended interrupt

bit 3 = 1  enable square wave interrupt

bit 2 = 1  calendar is in binary format

= 0  calendar is in BCD format

bit 1 = 1  24-hour mode

= 0  12-hour mode

bit 0 = 1  enable daylight savings time. only in USA.

useless in Europe. Some DOS versions clear

this bit when you use the DAT/TIME command.

0C    status register C

bit 7 =    interrupt request flag

bit 6 =    peridoc interrupt flag

bit 5 =    alarm interrupt flag

bit 4 =    update interrupt flag

bit 3-0    reserved

0D    status register D

bit 7 = 1  Real-Time Clock has power

bit 6-0    reserved

0E    diagnostics status byte

bit 7 = 0  RTC lost power

bit 6 = 1  CMOS RAM checksum bad

bit 5 = 1  invalid configuration information at POST

bit 4 = 1  memory size error at POST

bit 3 = 1  fixed disk/adapter failed initialization

27

bit 2 = 1  CMOS RAM time found invalid

bit 1 = 1  adapters do not match configuration (EISA)

bit 0 = 1  time out reading an adapter ID (EISA)

0F    shutdown status byte

00 = normal execution of POST

01 = chip set initialization for real mode reentry

04 = jump to bootstrap code

05 = issue an EOI an JMP to Dword ptr at 40:67

06 = JMP to Dword ptrv at 40:67 without EOI

07 = return to INT15/87 (block move)

08 = return to POST memory test

09 = return to INT15/87 (block move)

0A = JMP to Dword ptr at 40:67 without EOI

0B = return IRETS through 40:67

10    diskette drive type for A: and B:

bit 7-4  drive type of drive 0

bit 3-0  drive type of drive 1

= 0000      no drive

= 0001      360K

= 0010      1M2

= 0011      720K

= 0100      1M44

= 0101-1111 reserved

11    reserved / AMI Extended CMOS setup (AMI Hi-Flex BIOS)

bit 7   = 1     Typematic Rate Programming

bit 6-5 = 00    Typematic Rate Delay 250 mSec

bit 4-0 = 00011 Typematic Rate 21.8 Chars/Sec

12    fixed disk drive type for drive 0 and drive 1

bit 7-4  drive type of drive 0

bit 3-0  drive type of drive 1

if either of the nibbles equals 0F, then bytes

19 an 1A are valid

13    reserved / AMI Extended CMOS setup (AMI Hi-Flex BIOS)

bit 7 = 1  Mouse Support Option

bit 6 = 1  Above 1 MB Memory Test disable

bit 5 = 1  Memory Test Tick Sound disable

bit 4 = 1  Memory Parity Error Check enable

bit 3 = 1  Hit <ESC> Message Display disabled

bit 2 = 1  Hard Disk Type 47 Data Area at address 0:300

bit 1 = 1  Wait For <F1> If Any Error enabled

bit 0 = 1  System Boot Up Num Lock is On

14    equipment byte

bit 7-6   diskette drives installed

= 00  1 drive installed

= 01  2 drives installed

= 10  reserved

= 11  reserved

bit 5-4   primary display

= 00  adapter card with option ROM

= 01  40*25 color

= 10  80*25 color

= 11  monochrome

bit 3-2   reserved

bit 1   = 1  coprocessor installed (non-Weitek)

28

bit 0        diskette drive avaliable for boot

15    LSB of systemn base memory in Kb

16    MSB of systemn base memory in Kb

17    LSB of total extended memory in Kb

18    MSB of total extended memory in Kb

19    drive C extension byte

1A    drive D extension byte

1B-27 reserved

1B/1C word to 82335 RC1 roll compare register at [24]

(Phoenix)

1D/1E word to 82335 RC2 roll compare register at [26]

(Phoenix)

28    HP-Vectra checksum over 29-2D

29-2D reserved

29/2A word to Intel 82335 CC0 compare register at [28]

(Phoenix)

2B/2C word send to 82335 CC1 compare register at [2A]

(Phoenix)

2D    AMI Extended CMOS setup (AMI Hi-Flex BIOS)

(Phoenix BIOS checks for the values AA or CC)

bit 7 = 1  Weitek Processor Absent

bit 6 = 1  Floppy Drive Seek At Boot disabled

bit 5 = 1  System Boot Up Sequence  C:, A:

bit 4 = 1  System Boot Up Speed is high

bit 3 = 1  Cache Memory enabled

bit 2 = 1  Internal Cache Memory  <1>

bit 1-0    reserved

2E    CMOS MSB checksum over 10-2D

2F    CMOS LSB checksum over 10-2D

30    LSB of extended memory found above 1Mb at POST

31    MSB of extended memory found above 1Mb at POST

32    date century in BCD

33    information flags

bit4 = bit4 from CPU register CR0   (Phoenix)

this bit is only known as INTEL RESERVED

34-3F reserved

34     bit4 bit5 (Phoenix BIOS)

3D/3E word to 82335 MCR memory config register at [22]

(Phoenix)

3D     bit3      base memsize 512/640  (Phoenix)

3E     bit7 = 1  relocate enable       (Phoenix)

bit1 = 1  shadow video enable   (Phoenix)

bit0 = 1  shadow BIOS enable    (Phoenix)

User Definable Drive Parameters are also stored in CMOS RAM:

AMI (386sx BIOS 1989) first user definable drive (type 47)

1B  L cylinders

1C  H cylinders

1D  heads

1E  L Write Precompensation Cylinder

1F  H Write Precompensation Cylinder

20  ??

29

21  L cylinders parking zone

22  H cylinders parking zone

23  sectors

AMI (386sx BIOS 1989) second user definable drive (type 48)

24  L cylinders

25  H cylinders

26  heads

27  L Write Precompensation Cylinder

28  H Write Precompensation Cylinder

29  ??

2A  L cylinders parking zone

2B  H cylinders parking zone

2C  sectors

Phoenix (386BIOS v1.10.03 1988) 1st user definable drv (type48)

20  L cylinders

21  H cylinders

22  heads

23  L Write Precompensation Cylinder

24  H Write Precompensation Cylinder

25  L cylinders parking zone

26  H cylinders parking zone

27  sectors

Phoenix (386BIOS v1.10.03 1988) 2nd user definable drv (type49)

(when PS/2-style password option is not used)

35  L cylinders

36  H cylinders

37  heads

38  L Write Precompensation Cylinder

39  H Write Precompensation Cylinder

3A  L cylinders parking zone

3B  H cylinders parking zone

3C  sectors

0078            HP-Vectra  Hard Reset: NMI enable/disable

bit 7 = 0  disable & clear hard reset from HP-HIL controller

= 1  enable hard reset from HP-HIL controller chip

bit 6-0    reserved

007C-007D ----  HP-Vectra  PIC 3   (Programmable Interrupt Controller 8259)

cascaded to first controller.

used for keyboard and input device interface.

007C    r/w     HP-Vectra  PIC 3  see at 0020  PIC 1

007D    r/w     HP-Vectra  PIC 3  see at 0021  PIC 1

0080    w       Manufacturing Diagnostics port

#30

0080-008F ----  DMA page registers   (74612)

0080    r/w     extra page register (temporary storage)

0081    r/w     DMA channel 2 address byte 2

0082    r/w     DMA channel 3 address byte 2

0083    r/w     DMA channel 1 address byte 2

0084    r/w     extra page register

0085    r/w     extra page register

0086    r/w     extra page register

0087    r/w     DMA channel 0 address byte 2

0088    r/w     extra page register

0089    r/w     DMA channel 6 address byte 2

0089    r/w     DMA channel 7 address byte 2

0089    r/w     DMA channel 5 address byte 2

008C    r/w     extra page register

008D    r/w     extra page register

008E    r/w     extra page register

008F    r/w     DMA refresh page register

0090-009F ----  PS/2 POS (Programmable Option Select)

0090            Central arbitration control port

0091            Card selection feedback

0092    r/w     PS/2 system control port A  (port B is at 0061)

bit 7-6   any bit set to 1 turns activity light on

bit 5     reserved

bit 4 = 1 watchdog timout occurred

bit 3 = 0 RTC/CMOS security lock (on password area) unlocked

= 1 CMOS locked (done by POST)

bit 2     reserved

bit 1 = 1 indicates A20 active

bit 0 = 0 system reset or write

1 pulse alternate reset pin (alternate CPU reset)

0094    w       system board enable/setup register

bit 7 = 1  enable functions

= 0  setup functions

bit 5 = 1  enables VGA

= 0  setup VGA

0095            reserved

0096    w       adapter enable /setup register

bit 3 = 1  setup adapters

= 0  enable registers

0097            reserved

#31

00A0-00AF ----  PIC 2   (Programmable Interrupt Controller 8259)

00A0    r/w     NMI mask register (XT)

00A0    r/w     PIC 2  same as 0020 for PIC 1

00A1    r/w     PIC 2  same as 0021 for PIC 1 except for OCW1:

bit 7 = 0  reserved

bit 6 = 0  enable fixed disk interrupt

bit 5 = 0  enable coprocessor exception interrupt

bit 4 = 0  enable mouse interrupt

bit 3 = 0  reserved

bit 2 = 0  reserved

bit 1 = 0  enable redirect cascade

bit 0 = 0  enable real-time clock interrupt

00C0      ----  TI  SN746496 programmable tone/noise generator  PCjr

00C0-00DF ----  DMA 2   (second Direct Memory Access controller 8237)

00C0    r/w     DMA channel 4 memory address bytes 1 and 0 (low) (ISA, EISA)

00C2    r/w     DMA channel 4 transfer count bytes 1 and 0 (low) (ISA, EISA)

00C4    r/w     DMA channel 5 memory address bytes 1 and 0 (low) (ISA, EISA)

00C6    r/w     DMA channel 5 transfer count bytes 1 and 0 (low) (ISA, EISA)

00C8    r/w     DMA channel 6 memory address bytes 1 and 0 (low) (ISA, EISA)

00CA    r/w     DMA channel 6 transfer count bytes 1 and 0 (low) (ISA, EISA)

00CC    r/w     DMA channel 7 memory address byte 0 (low), then 1 (ISA, EISA)

00CE    r/w     DMA channel 7 transfer count byte 0 (low), then 1 (ISA, EISA)

00D0    r       DMA channel 4-7 status register (ISA, EISA)

bit 7 = 1  channel 7 request

bit 6 = 1  channel 6 request

bit 5 = 1  channel 5 request

bit 4 = 1  channel 4 request

bit 3 = 1  terminal count on channel 7

bit 2 = 1  terminal count on channel 6

bit 1 = 1  terminal count on channel 5

bit 0 = 1  terminal count on channel 4

00D0    w       DMA channel 4-7 command register (ISA, EISA)

bit 7 = 1  DACK sense active high

= 0  DACK sense active low

bit 6 = 1  DREQ sense active high

= 0  DREQ sense active low

bit 5 = 1  extended write selection

= 0  late write selection

bit 4 = 1  rotating priority

= 0  fixed priority

bit 3 = 1  compressed timing

= 0  normal timing

bit 2 = 0  enable controller

bit 1 = 1  enable memory-to-memory transfer

bit 0      .....

32

00D2    w       DMA channel 4-7 write request register (ISA, EISA)

00D4    w       DMA channel 4-7 write single mask register (ISA, EISA)

bit 7-3      reserved

bit 2   = 0  clear mask bit

= 1  set mask bit

bit 1-0 = 00 channel 4 select

= 01 channel 5 select

= 10 channel 6 select

= 11 channel 7 select

00D6    w       DMA channel 4-7 mode register (ISA, EISA)

bit 7-6 = 00  demand mode

= 01  single mode

= 10  block mode

= 11  cascade mode

bit 5   = 0   address increment select

= 1   address decrement select

bit 4   = 0   autoinitialisation disable

= 1   autoinitialisation enable

bit 3-2 = 00  verify operation

= 01  write to memory

= 10  read from memory

= 11  reserved

bit 1-0 = 00  channel 4 select

= 01  channel 5 select

= 10  channel 6 select

= 11  channel 7 select

00D8    w       DMA channel 4-7 clear byte pointer flip-flop (ISA, EISA)

00DA    r       DMA channel 4-7 read temporary register (ISA, EISA)

00DA    w       DMA channel 4-7 master clear (ISA, EISA)

00DC    w       DMA channel 4-7 clear mask register (ISA, EISA)

00DE    w       DMA channel 4-7 write mask register (ISA, EISA)

00E0            split address register, memory encoding registers PS/2m80 only

00F0-00F5 ----  PCjr Disk Controller

00F0            disk controller

00F2            disk controller control port

00F4            disk controller status register

00F5            disk controller data port

00F0-00FF ----  coprocessor (8087..80387)

00F0            math coprocessor clear busy latch

00F1            math coprocessor reset

00F8-00FF       math coprocessor

0100-010F ----  CompaQ Tape drive adapter. alternate address at 0300

33

0100-0107 ----  PS/2 POS (Programmable Option Select)

0100    r       POS register 0  Low adapter ID byte

0101    r       POS register 1  High adapter ID byte

0102    r/w     POS register 2  option select data byte 1

bit 0 is card enable (CDEN)

0103    r/w     POS register 3  option select data byte 2

0104    r/w     POS register 4  option select data byte 3

0105    r/w     POS register 5  option select data byte 4

bit 7  channel active (-CHCK)

bit 6  channel status

0106    r/w     POS register 6  Low subaddress extension

0107    r/w     POS register 7  High subaddress extension

0108-010F ----  8 digit LED info panel on IBM PS/2

010F    w       leftmost character on display

010E    w       second character

....    w

0108    w       eighth character

0130-013F ----  CompaQ SCSI adapter. alternate address at 0330

0130-0133 ----  Adaptec 154xB/154xC SCSI adapter.

alternate address at 0134, 0230, 0234 ,0330 and 0334

0134-0137 ----  Adaptec 154xB/154xC SCSI adapter.

alternate address at 0130, 0230, 0234 ,0330 and 0334

0140-014F ----  SCSI (alternate Small Computer System Interface) adapter

(1st at 0340-034F)

0140-0157 ----  RTC (alternate Real Time Clock for XT)   (1st at 0340-0357)

0170-0177 ----  HDC 2   (2nd Fixed Disk Controller)   same as 01Fx (ISA, EISA)

01E8-01EF ----  System Control. Laptop chipset: Headland HL21 & Acer M5105

01ED    r/w     select internal register. Data to/from 01EF

01EE    r

01EF    r/w     reg.

5  = 1000xxxx  for low CPU clock speed (4MHz on Morse/Mitac)

= 0xxxxxxx  for high CPU clock speed (16MHz on Morse/Mitac)

34

01F0-01F7 ----  HDC 1   (1st Fixed Disk Controller)   same as 017x (ISA, EISA)

01F0    r/w     data register

01F1    r       error register

diagnostic mode errors:

bit 7-3        reserved

bit 2-1 = 001  no error detected

= 010  formatter device error

= 011  sector buffer error

= 100  ECC circuitry error

= 101  controlling microprocessor error

operation mode:

bit 7  = 1  bad block detected

= 0  block OK

bit 6  = 1  uncorrectable ECC error

= 0  no error

bit 5       reserved

bit 4  = 1  ID found

= 0  ID not found

bit 3       reserved

bit 2  = 1  command completed

= 0  command aborted

bit 1  = 1  track 000 not found

= 0  track 000 found

bit 0  = 1  DAM not found

= 0  DAM found (CP-3022 always 0)

01F1    w       WPC/4  (Write Precompensation Cylinder divided by 4)

01F2    r/w     sector count

01F3    r/w     sector number

01F4    r/w     cylinder low

01F5    r/w     cylinder high

01F6    r/w     drive/head

bit 7   = 1

bit 6   = 0

bit 5   = 1

bit 4   = 0  drive 0 select

= 1  drive 1 select

bit 3-0      head select bits

01F7    r       status register

bit 7 = 1  controller is executing a command

bit 6 = 1  drive is ready

bit 5 = 1  write fault

bit 4 = 1  seek complete

bit 3 = 1  sector buffer requires servicing

bit 2 = 1  disk data read successfully corrected

bit 1 = 1  index - set to 1 each disk revolution

bit 0 = 1  previous command ended in an error

#35

01F7    w       command register

commands:

98 E5   check power mode       (IDE)

90      execute drive diagnostics

50      format track

EC      identify drive         (IDE)

97 E3   idle                   (IDE)

95 E1   idle immediatete       (IDE)

91      initialize drive parameters

1x      recalibrate

E4      read buffer            (IDE)

C8      read DMA with retry    (IDE)

C9      read DMA without retry (IDE)

C4      read multiplec         (IDE)

20      read sectors with retry

21      read sectors without retry

22      read long with retry

23      read long without retry

40      read verify sectors with retry

41      read verify sectors without retry

7x      seek

EF      set features           (IDE)

C6      set multiple mode      (IDE)

99 E6   set sleep mode         (IDE)

96 E2   standby                (IDE)

94 E0   standby immediate      (IDE)

E8      write buffer           (IDE)

CA      write DMA with retry   (IDE)

CB      write DMA with retry   (IDE)

C5      write multiple         (IDE)

E9      write same             (IDE)

30      write sectors with retry

31      write sectors without retry

32      write long with retry

33      write long without retry

3C      write verify           (IDE)

9A      vendor unique          (IDE)

C0-C3   vendor unique          (IDE)

8x      vendor unique          (IDE)

F0-F4   EATA standard          (IDE)

F5-FF   vendor unique          (IDE)

#36

0200-020F ----  Game port reserved I/O address space

0200-0207 ----  Game port, eight identical addresses on some boards

0201    r       read joystick position and status

bit 7  status B joystick button 2 / D paddle button

bit 6  status B joystick button 1 / C paddle button

bit 5  status A joystick button 2 / B paddle button

bit 4  status A joystick button 1 / A paddle button

bit 3  B joystick Y coordinate    / D paddle coordinate

bit 2  B joystick X coordinate    / C paddle coordinate

bit 1  A joystick Y coordinate    / B paddle coordinate

bit 0  A joystick X coordinate    / A paddle coordinate

w       fire joysticks four one-shots

0210-0217 ----  Expansion unit (XT)

0210    w       latch expansion bus data

r       verify expansion bus data

0211    w       clear wait, test latch

r       High byte data address

0212    r       Low byte data address

0213    w       0=enable,  1=disable expansion unit

0214    w       latch data  (receiver card port)

r       read data   (receiver card port)

0215    r       High byte of address, then Low byte   (receiver card port)

0220-0227 ----  Soundblaster PRO and SSB 16 ASP

0220-022F ----  Soundblaster PRO 2.0

0220-022F ----  Soundblaster PRO 4.0

0220    r       left FM status port

0220    w       left FM music register address port (index)

0221    r/w     left FM music data port

0222    r       right FM status port

0222    w       right FM music register address port (index)

0223    r/w     right FM music data port

0224    w       mixer register address port (index)

0225    r/w     mixer data port

0226    w       DSP reset

0228    r       FM music status port

0228    w       FM music register address port (index)

0229    w       FM music data port

022A    r       DSP read data (voice I/O and Midi)

022C    w       DSP write data / write command

022C    r       DSP write buffer status (bit 7)

022E    r       DSP data available status (bit 7)

The FM music is accessible on 0388/0389 for compatibility.

37

0230-0233 ----  Adaptec 154xB/154xC SCSI adapter.

alternate address at 0130, 0134, 0230, 0330 and 0334

0234-0237 ----  Adaptec 154xB/154xC SCSI adapter.

alternate address at 0130, 0134, 0230 ,0330 and 0334

0240-0257 ----  RTC (alternate Real Time Clock for XT)   (1st at 0340-0357)

(used by TIMER.COM v1.2 which is the 'standard' timer program)

0258-025F ----  Intel Above Board

0278-027E ----  parallel printer port, same as 0378 and 03BC

0278    w       data port

0279    r/w     status port

027A    r/w     control port

02A2-02A3 ----  MSM58321RS clock

02B0-02BF ----  Trantor SCSI adapter

02B0-02DF ----  alternate EGA,  primary EGA at 03C0

02C0-02Cx ----  AST-clock

02E0-02EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

(GAB 0 on XT)

02E1            GPIB (adapter 0)

02E2

02E3

02E0-02EF ----  data aquisition  (AT)

02E2            data aquisition (adapter 0)

02E3            data aquisition (adapter 0)

02E8-02EF ----  serial port, same as 02F8, 03E8, and 03F8

#38

02F8-02FF ----  serial port, same as 02E8, 02F8, and 03F8

02F8    w       transmitter holding register

02F8    r       receiver buffer register

r/w     divisor latch, low byte   when DLAB=1

02F9    r/w     divisor latch, high byte  when DLAB=1

r/w     interrupt enable register when DLAB=0

02FA    r       interrupt identification register

02FB    r/w     line control register

02FC    r/w     modem control register

02FD    r       line status register

02FF    r/w     scratch register

0300-0301 ---- Soundblaster 16 ASP MPU-Midi

0300-031F ----  prototype cards

Periscope hardware debugger

0300-030F ----  Philips CD-ROM player CM50

0310-031F ----  Philips CD-ROM player CM50

0320-0323 ----  XT HDC 1   (Hard Disk Controller)

0320    r/w     data register

0321    w       reset controller

r       read controller hardware status

bit 7-6 = 0

bit 5        logical unit number

bit 4-2 = 0

bit 1   = 0  no error

bit 0   = 0

0322    r       read DIPswitch setting on XT controller card

w       generate controller-select pulse

0323    w       write pattern to DMA and INT mask register

0324-0327 ----  XT HDC 2   (Hard Disk Controller)

0328-032B ----  XT HDC 3   (Hard Disk Controller)

032C-032F ----  XT HDC 4   (Hard Disk Controller)

0330-0331 ----  MIDI interface

0330-0333 ----  Adaptec 154xB/154xC SCSI adapter. default address.

alternate address at 0130, 0134, 0230, 0234 and 0334

0330-033F ----  CompaQ SCSI adapter. alternate address at 0130

39

0330-033F ----  Philips CD-ROM player CM50

0334-0337 ----  Adaptec 154xB/154xC SCSI adapter.

alternate address at 0130, 0134, 0230 ,0234 and 0330

0338      ----  AdLib soundblaster card

0340-034F ----  Philips CD-ROM player CM50

0340-034F ----  SCSI (1st Small Computer System Interface) adapter

(alternate at 0140-014F)

0340-0357 ----  RTC (1st Real Time Clock for XT), (alternate at 0240-0257)

(used by TIMER.COM v1.2 which is the 'standard' timer program)

0340    r/w     0.01 seconds  0-99

0341    r/w     0.1 seconds   0-99

0342    r/w     seconds       0-59

0343    r/w     minutes       0-59

0343    r/w     hours         0-23

0345    r/w     day of week   1-7

0346    r/w     day of month  1-31

0347    r/w     month         1-12

0348

0349    r/w     year          0-99

034A

034B

034C

034D

034E

034F

0350    r       status?

0351

0352

0353

0354    r       status?

0355

0356

0357

0348-0357 ----  DCA 3278

0360-036F ----  PC network (AT)

0360-0367 ----  PC network (XT only)

#40

0370-0377 ----  FDC 2  (2nd Floppy Disk Controller 8272)  same as 03F0

0372    w       diskette controller DOR (Digital Output Register)

0374    r       diskette controller status register

0375    r/w     diskette controller data register

0376    r/w     FIXED disk controller data register

0377    r       diskette controller DIR (Digital Input Register)

0377    w       select register for diskette data transfer rate

0378-037A ----  parallel printer port, same as 0278 and 03BC

0378    w       data port

0379    r/w     status port

037A    r/w     control port

0380-038F ----  2nd Binary Synchronous Data Link Control adapter (see 03A0)

0380    r/w      on board 8255 port A, internal/external sense

0381    r/w      on board 8255 port B, external modem interface

0382    r/w      on board 8255 port C, internal control and gating

0383    r/w      on board 8255 mode register

0384    r/w      on board 8253 channel square wave generator

0385    r/w      on board 8253 channel 1 inactivity time-out

0386    r/w      on board 8253 channel 2 inactivity time-out

0387    r/w      on board 8253 mode register

0388    r/w      on board 8273 read: status  write: command

0389    r/w      on board 8273 read: response  write: parameter

038A    r/w      on board 8273 transmit interrupt status

038B    r/w      on board 8273 receiver interrupt status

038C    r/w      on board 8273 data

0388-0389 ---- Soundblaster PRO FM-Chip

0388-038B ---- Soundblaster 16 ASP FM-Chip

0390-039F ----  Cluster adapter (AT)

0390-0393       (adapter 0)     (XT)

#41

03A0-03AF ----  1st SDLC (Binary Synchronous Data Link Control adapter)

0380    r/w      on board 8255 port A, internal/external sense

0381    r/w      on board 8255 port B, external modem interface

0382    r/w      on board 8255 port C, internal control and gating

0383    r/w      on board 8255 mode register

0384    r/w      on board 8253 counter 0 unused

0385    r/w      on board 8253 channel 1 inactivity time-out

0386    r/w      on board 8253 channel 2 inactivity time-out

0387    r/w      on board 8253 mode register

0388    r/w      on board 8251 data

0389    r/w      on board 8251 command/mode/status register

03B0-03BF ----  MDA  (Monochrome Display Adapter based on 6845)

03B0            same as 03B4

03B1            same as 03B5

03B2            same as 03B4

03B3            same as 03B5

03B4    w       MDA CRT index register   (EGA/VGA)

selects which register (0-11h) is to be accessed through 3B5

03B5    r/w     MDA CRT data register    (EGA/VGA)

selected by port 3B4. registers C-F may be read

00  horizontal total

01  horizontal displayed

02  horizontal sync position

03  horizontal sync pulse width

04  vertical total

05  vertical displayed

06  vertical sync position

07  vertical sunc pulse width

08  interlace mode

09  maximum scan lines

0A  cursor start

0B  cursor end

0C  start address high

0D  start address low

0E  cursor location high

0F  cursor location low

10  light pen high

11  light pen low

03B6            same as 03B4

03B7            same as 03B5

03B8    r/w     MDA mode control register

bit 7 not used

bit 6 not used

bit 5 enable blink

bit 4 not used

bit 3 video enable

bit 2 not used

bit 1 not used

bit 0 high resolution mode

42

03B9            reserved for color select register on color adapter

03BA    r       CRT status register      EGA/VGA: input status 1 register

bit 7-4 reserved

bit 3   black/white video

bit 2-1 reserved

bit 0   horizontal drive

bit 7   (MSD says) if this bit changes within 8000h reads then

bit 6-4 = 000 adapter is Hercules or compatible

= 001 adapter is Hercules+

= 101 adapter is Hercules InColor

else  adapter is unknown

03BA    w       EGA/VGA feature control register

03BB            reserved for light pen strobe reset

03BC-03BF ----  parallel printer port, same as 0278 and 0378

03BC    w       data port

03BD    r/w     status port

bit 7 = 0  busy

bit 6 = 0  acknowledge

bit 5 = 1  out of paper

bit 4 = 1  printer is selected

bit 3 = 0  error

bit 2 = 0  IRQ has occurred

bit 1-0    reserved

03BE    r/w     control port

bit 7-5    reserved

bit 4 = 1  enable IRQ

bit 3 = 1  select printer

bit 2 = 0  initialize printer

bit 1 = 1  automatic line feed

bit 0 = 1  strobe

03BF    r/w     Hercules configuration switch register

bit 7-2

bit 1 = 0  disables upper 32K of graphics mode buffer

1  enables upper 32K of graphics mode buffer

bit 0 = 0  prevents graphics mode

1  allows graphics mode

#43

03C0-03CF ----  EGA  (1st Enhanced Graphics Adapter)    alternate at 02C0

03C0    (r)/w   EGA VGA ATC index/data register

03C1    r       VGA     other attribute register

03C2    r       EGA VGA input status 0 register

w       VGA     miscellaneous output register

03C3    r/w     VGA     video subsystem enable

03C4    w       EGA     TS index register

r/w     VGA     sequencer index register

03C5    w       EGA     TS data register

r/w     VGA     other sequencer register

03C6    r/w     VGA     PEL mask register

03C7    r/w     VGA     PEL address read mode

r       VGA     DAC state register

03C8    r/w     VGA     PEL address write mode

03C9    r/w     VGA     PEL data register

03CA    w       EGA     graphics 2 position register

r       VGA     feature control register

03CC    w       EGA     graphics 1 position register

r       VGA     miscellaneous output register

03CE    w       EGA     GDC index register

r/w     VGA     graphics address register

03CF    w       EGA     GDC data register

r/w     VGA     other graphics register

03D0-03DF ----  CGA   (Color Graphics Adapter)

03D0            same as 03D4

03D1            same as 03D5

03D2            same as 03D4

03D3            same as 03D5

03D4    w       CRT (6845) index register   (EGA/VGA)

selects which register (0-11h) is to be accessed through 3B5

03D5    w       CRT (6845) data register    (EGA/VGA)

selected by port 3B4. registers C-F may be read

(for registers see at 3B5)

03D6            same as 03D4

03D7            same as 03D5

03D8    r/w     CGA mode control register  (except PCjr)

bit 7-6      not used

bit 5   = 1  blink enabled

bit 4   = 1  640*200 graphics mode

bit 3   = 1  video enabled

bit 2   = 1  monochrome signal

bit 1   = 0  text mode

= 1  320*200 graphics mode

bit 0   = 0  40*25 text mode

= 1  80*25 text mode

#44

03D9    r/w     CGA palette register

bit 7-6      not used

bit 5   = 0 active color set: red, green brown

= 1 active color set: cyan, magenta, white

bit 4       intense colors in graphics, background colors text

bit 3       intense border in 40*25, intense background in

320*200, intense foreground in 640*200

bit 2       red border in 40*25, red background in 320*200,

red foreground in 640*200

bit 1       green border in 40*25, green background in

320*200, green foreground in 640*200

bit 0       blue border in 40*25, blue background in 320*200,

blue foreground in 640*200

03DA    r       CGA status register     EGA/VGA: input status 1 register

bit 7-4     not used

bit 3 = 1   in vertical retrace

bit 2 = 1   light pen switch is off

bit 1 = 1   positive edge from light pen has set trigger

bit 0 = 0   do not use memory

= 1   memory access without interfering with display

03DA    w       EGA/VGA feature control register

03DB    w       clear light pen latch

03DC    r/w     preset licht pen latch

03DF            CRT/CPU page register  PCjr only

03E8-03EF ----  serial port, same as 02E8, 02F8, and 03F8

03F0-03F7 ----  FDC 1  (1st Floppy Disk Controller 8272)  same as 0370

03F0            diskette controller status A (PS/2)

bit 7   interrupt pending

bit 6   second drive installed

bit 5   step

bit 4   track 0

bit 3   head 1 select

bit 2   index

bit 1   write protect

bit 0   direction

03F1    r       diskette controller status B (PS/2)

bit 7-6  reserved

bit 5    drive select (0=A:, 1=B:)

bit 4    write data

bit 3    read data

bit 2    write enable

bit 1    motor enable 1

bit 0    motor enable 0

45

03F2    w       diskette controller DOR (Digital Output Register)

bit 7-6    reserved PS/2

bit 7 = 1  drive 3 motor enable

bit 6 = 1  drive 2 motor enable

bit 5 = 1  drive 1 motor enable

bit 4 = 1  drive 0 motor enable

bit 3 = 1  diskette DMA enable (reserved PS/2)

bit 2 = 1  FDC enable  (controller reset)

= 0  hold FDC at reset

bit 1-0    drive select (0=A 1=B ..)

03F4    r       diskette controller status register

bit 7 = 1  data register is ready

bit 6 = 1  transfer is from controller to system

0  transfer is from system to controller

bit 5 = 1  non-DMA mode

bit 4 = 1  diskette controller busy

bit 3 = 1  drive 3 busy (reserved on PS/2)

bit 2 = 1  drive 2 busy (reserved on PS/2)

bit 1 = 1  drive 1 busy

bit 0 = 1  drive 0 busy

03F5    r       diskette command status register 0

bit 7-6      last command status

= 00 command terminated successfully

= 01 command terminated abnormally

= 10 invalid command

= 11 terminated abnormally by change in ready signal

bit 5   = 1  seek completed

bit 4   = 1  equipment check occurred after error

bit 3   = 1  not ready

bit 2   = 1  head number at interrupt

bit 1-0 = 1  unit select (0=A 1=B .. )

(on PS/2  01=A  10=B)

status register 1

bit 7      end of cylinder; sector# greater then sectors/track

bit 6 = 0

bit 5 = 1  CRC error in ID or data field

bit 4 = 1  overrun

bit 3 = 0

bit 2 = 1  sector ID not found

bit 1 = 1  write protect detected during write

bit 0 = 1  ID address mark not found

status register 2

bit 7 = 0

bit 6 = 1  deleted Data Eddress Mark detected

bit 5 = 1  CRC error in data

bit 4 = 1  wrong cylinder detected

bit 3 = 1  scan command equal condition satisfied

bit 2 = 1  scan command failed, sector not found

bit 1 = 1  bad cylinder, ID not found

bit 0 = 1  missing Data Address Mark

46

status register 3

bit 7      fault status signal

bit 6      write protect status

bit 5      ready status

bit 4      track zero status

bit 3      two sided status signal

bit 2      side select (head select)

bit 1-0    unit select (0=A 1=B .. )

03F6    r/w     FIXED disk controller data register

bit 7-4    reserved

bit 3 = 0  reduce write current

1  head select 3 enable

bit 2 = 1  disk reset enable

0  disk reset disable

bit 1 = 0  disk initialization enable

1  disk initialization disable

bit 0      reserved

03F7    r       diskette controller DIR (Digital Input Register)

bit 7 = 1 diskette change

bit 6     FIXED DISK write gate

bit 5     FIXED DISK head select 3 / reduced write current

bit 4     FIXED DISK head select 2

bit 3     FIXED DISK head select 1

bit 2     FIXED DISK head select 0

bit 1     FIXED DISK drive 1 select

bit 0     FIXED DISK drive 0 select   conflicts with:

bit 0     diskette high density select

03F7    w       select register for diskette data transfer rate

bit 7-2       reserved

bit 1-0 = 00  500 Kb/S mode

= 01  300 Kb/S mode

= 10  250 Kb/S mode

= 11  reserved

03F8-03FF ----  serial port (8250,8251,16450,16550), same as 02E8,02F8,and 03F8

03F8    w       serial port, transmitter holding register, which contains the

character to be sent. Bit 0 is sent first.

bit 7-0   data bits when DLAB=0 (Divisor Latch Access Bit)

r       receiver buffer register, which contains the received character

Bit 0 is received first

bit 7-0   data bits when DLAB=0 (Divisor Latch Access Bit)

r/w     divisor latch low byte when DLAB=1

#47

03F9    r/w     divisor latch high byte when DLAB=1

r/w     interrupt enable register when DLAB=0

bits 7-4   reserved

bit 3 = 1  modem-status interrupt enable

bit 2 = 1  receiver-line-status interrupt enable

bit 1 = 1  transmitter-holding-register empty interrupt enable

bit 0 = 1  received-data-avail.int. enable (and 16550 timeout)

- 16550 will interrupt if data exists in the FIFO and isn't read

within the time it takes to receive four bytes or if no data is

received within the time it takes to receive four bytes

03FA    r       interrupt identification register. Information about a pending

interrupt is stored here. When the ID register is addressed,

the highest priority interrupt is held, and no other interrupts

are acknowledged until the CPU services that interrupt.

bit 7-6 = 00  reserved on 8250, 8251, 16450

= 11  if FIFO queues are enabled (16550 only)

bit 5-4 = 0 reserved

bit 3   = 0 reserved  8250, 16450

= 1 16550  timeout int. pending

bit 2-1 identify pending interrupt with the highest priority

= 11  receiver line status interrupt.

priority = highest

= 10  received data available register interrupt.

priority = second

= 01  transmitter holding register empty interrupt.

priority = third

= 00  modem status interrupt.

priority = fourth

bit 0   = 0   interrupt pending. Contents of register can be

used as a pointer to the appropriate

interrupt service routine

= 1   no interrupt pending

- interrupt pending flag uses reverse logic, 0 = pending, 1 = none

- interrupt will occur if any of the line status bits are set

- THRE bit is set when THRE register is emptied into the TSR

#48

03FA    w       16650 FCR  (FIFO Control Register)

bit 7-6 = 00  1 byte

= 01  4 bytes

= 10  8 bytes

= 11 14 bytes

bit 5-4 = 00 reserved

bit 3   = 1  change RXRDY  TXRDY pins from mode 0 to mode 1

bit 2   = 1  clear XMIT FIFO

bit 1   = 1  clear RCVR FIFO

bit 0   = 1  enable clear XMIT and RCVR FIFO queues

- bit 0 must be set in order to write to other FCR bits

- bit 1 when set  the RCVR FIFO is cleared and this bit is reset

the receiver shift register is not cleared

- bit 2 when set  the XMIT FIFO is cleared and this bit is reset

the transmit shift register is not cleared

03FB    r/w     line control register

bit 7 = 1  divisor latch access bit (DLAB)

0  receiver buffer, transmitter holding, or interrupt

enable register access

bit 6 = 1  set break enable. serial ouput is forced to spacing

state and remains there.

bit 5 =    stick parity

bit 4 = 1  even parity select

bit 3 =    parity enable

1  even number of ones are sent and checked in the

data word bits and parity bit

0  odd number of ones are sent and checked

bit 2 = 0  one stop bit

1  zero stop bit

bit 1-0 00 word length is 5 bits

01 word length is 6 bits

10 word length is 7 bits

11 word length is 8 bits

03FC    r/w     modem control register

bit 7-5 = 0  reserved

bit 4 = 1  loopback mode for diagnostic testing of serial port

output of transmitter shift register is looped back

to receiver shift register input. In this mode

transmitted data is received immediately so that

the CPU can verify the transmit data/receive data

serial port paths.

bit 3 = 1  auxiliary user-designated output 2

bit 2 = 1  auxiliary user-designated output 1

bit 1 = 1  force request-to-send active

bit 0 = 1  force data-terminal-ready active

#49

03FD    r       line status register

bit 7 = 0  reserved

bit 6 = 1  transmitter shift and holding registers empty

bit 5 = 1  transmitter holding register empty. Controller is

ready toaccept a new character to send.

bit 4 = 1  break interrupt. the received data input is held in

in the zero bit state longer than the time of start

bit + data bits + parity bit + stop bits.

bit 3 = 1  framing error. the stop bit that follows the last

parity or data bit is a zero bit.

bit 2 = 1  parity error. Character has wrong parity

bit 1 = 1  overrun error. a character was sent to the receiver

buffer before the previous character in the buffer

could be read. This destroys the previous

character.

bit 0 = 1  data ready. a complete incoming character has been

received and sent to the receiver buffer register.

03FE    r       modem status register

bit 7 = 1  data carrier detect

bit 6 = 1  ring indicator

bit 5 = 1  data set ready

bit 4 = 1  clear to send

bit 3 = 1  delta data carrier detect

bit 2 = 1  trailing edge ring indicator

bit 1 = 1  delta data set ready

bit 0 = 1  delta clear to send

- bits 0-3 are reset when the CPU reads the MSR

- bit 4 is the Modem Control Register RTS during loopback test

- bit 5 is the Modem Control Register DTR during loopback test

- bit 6 is the Modem Control Register OUT1 during loopback test

- bit 7 is the Modem Control Register OUT2 during loopback test

03FF    r/w     scratch register

Adresses above 03FF apply to EISA machines only

1000-1FFF      slot 1 EISA

2000-2FFF      slot 2 EISA

3000-3FFF      slot 3 EISA

4000-4FFF      slot 4 EISA

5000-5FFF      slot 5 EISA

6000-6FFF      slot 6 EISA

7000-7FFF      slot 7 EISA

0401-04D6 ----  used by EISA systems only

0401    r/w     DMA channel 0 word count byte 2 (high)

0403    r/w     DMA channel 1 word count byte 2 (high)

0405    r/w     DMA channel 2 word count byte 2 (high)

0407    r/w     DMA channel 3 word count byte 2 (high)

50

040A    w       extended DMA chaining mode register, channels 0-3

bit 7-5      reserved

bit 4   = 0  generates IRQ13

= 1  generates terminal count

bit 3   = 0  do not start chaining

= 1  programming complete

bit 2   = 0  disable buffer chaining mode (default)

= 1  enable buffer chaining mode

bit 1-0      DMA channel select

040A    r       channel interrupt (IRQ13) status register

bit 7-5  interrupt on channels 7-5

bit 4    reserved

bit 3-0  interrupt on channels 3-0

040B    w       DMA extended mode register for channels 3-0.

bit settings same as 04D6

bit 7   = 0  enable stop register

bit 6   = 0  terminal count is an output for this channel

(default)

bit 5-4      DMA cycle timing

= 00 ISA-compatible (default)

= 01 type A timing mode

= 10 type B timing mode

= 11 burst DMA mode

bit 3-2      Address mode

= 00 8-bit I/O, count by bytes (default)

= 01 16-bit I/O, count by words, address shifted

= 10 32-bit I/O, count by bytes

= 11 16-bit I/O, count by bytes

bit 1-0      DMA channel select

0461    r/w     Extended NMI status/control register

bit 7 = 1  NMI pending from fail-safe timer (read only)

bit 6 = 1  NMI pending from bus timeout NMI status (read

only)

bit 5 = 1  NMI pending (read only)

bit 4      reserved

bit 3 = 1  bus timeout NMI enable (read/write)

bit 2 = 1  fail-safe NMI enable (read/write)

bit 1 = 1  NMI I/O port enable (read/write)

bit 0      RSTDRV. bus reset (read/write)

= 0  normal bus reset operation

= 1  reset bus asserted

0462    w       Software NMI register. writing to this register causes an NMI

if NMI's are enabled

bit 7 = 1  generates an NMI

#51

0464    r       bus master status latch register (slots 1-8). identifies the

last bus master that had control of the bus

bit 7 = 0  slot 8 had control last

bit 6 = 0  slot 7 had control last

bit 5 = 0  slot 6 had control last

bit 4 = 0  slot 5 had control last

bit 3 = 0  slot 4 had control last

bit 2 = 0  slot 3 had control last

bit 1 = 0  slot 2 had control last

bit 0 = 0  slot 1 had control last

0465    r       bus master status latch register (slots 9-16)

bit 7 = 0  slot 16 had control last

bit 6 = 0  slot 15 had control last

bit 5 = 0  slot 14 had control last

bit 4 = 0  slot 13 had control last

bit 3 = 0  slot 12 had control last

bit 2 = 0  slot 11 had control last

bit 1 = 0  slot 10 had control last

bit 0 = 0  slot 9  had control last

0481    r/w     DMA channel 2 address byte 3 (high)

0482    r/w     DMA channel 3 address byte 3 (high)

0483    r/w     DMA channel 1 address byte 3 (high)

0487    r/w     DMA channel 0 address byte 3 (high)

0489    r/w     DMA channel 6 address byte 3 (high)

048A    r/w     DMA channel 7 address byte 3 (high)

048B    r/w     DMA channel 5 address byte 3 (high)

04C6    r/w     DMA channel 5 word count byte 2 (high)

04CA    r/w     DMA channel 6 word count byte 2 (high)

04CE    r/w     DMA channel 7 word count byte 2 (high)

04D0    w       IRQ 0-7 interrupt edge/level registers

bit 7 = 1  IRQ 7 is level sensitive

bit 6 = 1  IRQ 6 is level sensitive

bit 5 = 1  IRQ 5 is level sensitive

bit 4 = 1  IRQ 4 is level sensitive

bit 3 = 1  IRQ 3 is level sensitive

bit 2-0    reserved

04D1    w       IRQ 8-15 interrupt edge/level registers

bit 7 = 1  IRQ 15 is level sensitive

bit 6 = 1  IRQ 14 is level sensitive

bit 5 = 1  reserved

bit 4 = 1  IRQ 12 is level sensitive

bit 3 = 1  IRQ 11 is level sensitive

bit 2 = 1  IRQ 10 is level sensitive

bit 1 = 1  IRQ 9  is level sensitive

bit 0      reserved

#52

04D4    w       extended DMA chaining mode register, channels 4-7

bit 7-5 = 0  reserved

bit 4   = 0  generates IRQ 13

= 1  generates terminal count

bit 3   = 0  do not start chaining

= 1  programming complete

bit 2   = 0  disable buffer chaining mode (default)

= 1  enable buffer chaining mode

bit 1-0      DMA channel select

04D6    w       DMA extended mode register for channels 4-7

bit settings same as 04B

bit 7   = 0  enable stop register

bit 6   = 0  terminal count is an output for this channel

(default)

bit 5-4      DMA cycle timing

= 00 ISA-compatible (default)

= 01 type A timing mode

= 10 type B timing mode

= 11 burst DMA mode

bit 3-2      Address mode

= 00 8-bit I/O, count by bytes (default)

= 01 16-bit I/O, count by words, address shifted

= 10 32-bit I/O, count by bytes

= 11 16-bit I/O, count by bytes

bit 1-0      DMA channel select

0601    w       System control. Laptop chipset: Headland HL21 & Acer M5105

bit 7 = 1  power led on

bit 6 = 1  LCD backlight off

bit 5

bit 4

bit 3

bit 2 = 1  video chips disabled, screen blanked.

bit 1

bit 0 = 1  will lock up your machine!

r        =04        (always reads back this value)

0620-0627 ----  PC network (adapter 1)

0628-062F ----  PC network (adapter 2)

06E2-06E3 ----  data aquisition (adapter 1)

0790-0793 ----  cluster (adapter 1)

0800-08FF ----  I/O port access registers for extended CMOS RAM or SRAM

(256 bytes at a time)

Sometimes plain text can be seen here.

53

0A20-0A23 ----  Token Ring (adapter 1)

0A24-0A27 ----  Token Ring (adapter 2)

0AE2-0AE3 ----  cluster (adapter 2)

0B90-0B93 ----  cluster (adapter 2)

0C00    r/w     page register to write to SRAM or I/O

0C80-0C83 ----  system board ID registers

1390-1393 ----  cluster (adapter 3)

2100-210F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2110-211F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2120-212F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2130-213F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2140-214F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2150-215F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2160-216F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2170-217F ----  IBM XGA (eXtended Graphics Adapter  8514/A)

2390-2393 ----  cluster (adapter 4)

3220-3227 ----  serial port 3, description same as 03F8

3228-322F ----  serial port 4, description same as 03F8

3540-354F ----  IBM SCSI (Small Computer System Interface) adapter

3550-355F ----  IBM SCSI (Small Computer System Interface) adapter

3560-356F ----  IBM SCSI (Small Computer System Interface) adapter

3570-357F ----  IBM SCSI (Small Computer System Interface) adapter

4220-4227 ----  serial port, description same as 03F8

4228-422F ----  serial port, description same as 03F8

42E0-42EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

42E1    r/w     GPIB (adapter 2)

5220-5227 ----  serial port, description same as 03F8

5228-522F ----  serial port, description same as 03F8

62E0-62EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

54

62E1    r/w     GPIB (adapter 3)

82E0-82EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

82E1    r/w     GPIB (adapter 4)

82F8-82FF ----  serial port, description same as 03F8

83F8-83FF ----  serial port, description same as 03F8

A220      ????  soundblaster support in AMI Hi-Flex BIOS  ????

A2E0-A2EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

A2E1    r/w     GPIB (adapter 5)

AFFF    r/w     plane 0-3 system latch (video register)

B220-B227 ----  serial port, description same as 03F8

B228-B22F ----  serial port, description same as 03F8

C220-C227 ----  serial port, description same as 03F8

C228-C22F ----  serial port, description same as 03F8

D220-D227 ----  serial port, description same as 03F8

D228-D22F ----  serial port, description same as 03F8

C2E0-C2EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

C2E1    r/w     GPIB (adapter 6)

E2E0-E2EF ----  GPIB (General Purpose Interface Bus, IEEE 488 interface)

E2E1    r/w     GPIB (adapter 7)

MEMORY-MAPPED ADDRESSES

80C00000  Compaq Deskpro 386 system memory board register

#55

80C00000 w      RAM relocation register

bit 7-2    Reserved, always write 1's.

bit 1 = 0  Write-protect 128-Kbyte RAM at FE0000.

= 1  Do not write-protect RAM at FE0000.

bit 0 = 0  Relocate 128-Kbyte block at FE0000 to address 0E0000

= 1  128-Kbyte RAM is addressed only at FE0000.

80C00000 r      Diagnostics register

bit 7   = 0  memory expansion board is installed

bit 6   = 0  second 1 MB of system memory board is installed

bit 5-4 = 00 base memory set to 640 KB

= 01 invalid

= 10 base memory set to 512 KB

= 11 base memory set to 256 KB

bit 3   = 0  parity error in byte 3

bit 2   = 0  parity error in byte 2

bit 1   = 0  parity error in byte 1

bit 0   = 0  parity error in byte 0 (in 32-bit double word)

#56

Program Segment Prefix Data Structure

When DOS loads a program for execution it creates a data structure in memory

called the "Program Segment Prefix", or PSP for short. This structure occupies

the 256 bytes of memory immediately preceeding the memory occupied y the

loaded program. The PSP contains useful information which is often accessed by

the running program. The following table describes the structure of the PSP.

Offset   Description

00      INT 20 instruction for CP/M CALL 0 program termination

02      segment of first byte beyond memory allocated to program

04      unused filler

05      CP/M CALL 5 service request (FAR JMP to 000C0h)

BUG: (DOS 2+) PSPs created by INT 21/AH=4B point at 000BE

06      CP/M compatibility - size of first segment for .COM files

08      remainder of FAR JMP at 05

0A      stored INT 22 termination address

0E      stored INT 23 control-Break handler address

12      DOS 1.1+ stored INT 24 critical error handler address

16      segment of parent PSP

18      DOS 2+ Job File Table, one byte per file handle, FFh = closed

2C      DOS 2+ segment of environment for process

2E      DOS 2+ process's SS:SP on entry to last INT 21 call

32      DOS 3+ number of entries in JFT (default 20)

34      DOS 3+ pointer to JFT (default PSP:0018)

38      DOS 3+ pointer to previous PSP (default FFFFFFFFh in 3.x)

used by SHARE in DOS 3.3

3C      apparently unused by DOS versions <= 6.00

3D      (APPEND) TrueName flag

3E      (Novell NetWare) flag: next byte initialized if CEh

3F      (Novell NetWare) Novell task number if previous byte is CEh

40      DOS 5+ version to return on INT 21/AH=30

42      (MSWindows3) selector of next PSP (PDB) in linked list

Windows keeps a linked list of Windows programs only

44      unused by DOS versions <= 6.00

48      (MSWindows3) bit 0 set if non-Windows application (WINOLDAP)

49      unused by DOS versions <= 6.00

50      DOS 2+ service request (INT 21/RETF instructions)

53      unused in DOS versions <= 6.00

55      unused in DOS versions <= 6.00; can be used to make first FCB

into an extended FCB

5C      first default FCB, filled in from first command tail argument

overwrites second FCB if opened

6C      second default FCB, filled in from second command tail argument

overwrites beginning of command tail if opened

7C      unused

80      Length of command tail

81      Command tail terminated by 0D

#57

CREDITS

The following publications have proved useful in the production of SSD:

8086/8088 User's Manual, Intel 1989

PC Intern, Michael Tischer, Abacus 1992

DOS Programmer's Reference, Terry R Dettmann, Que 1988

Interrupt List edited by Ralf Brown. Including details of the multiplex

interrupt, described by Robin Walker.

The author is grateful to the cooperation of the virus writer "Black Baron"

in supplying copies of his polymorphic computer viruses and supplying

information about the SMEG engine.

Program Design & Coding: Matthew Probert

Version 2.5 published by Servile Software, October 1994. All rights reserved.

#58

With effect from version 2.4, SSD became SHAREWARE and you are required to

register your copy or stop using it.

Registration costs 29.95 pounds sterling (UK pounds).

Please make cheques payable to "M. Probert" ONLY, and send with the enclosed

order form (order.doc) to:

Matthew Probert

5 Longcroft Close

Basinsgtoke

Hampshire

RG21 1XG

Telephone 0256 478576

��������������������������

��������������������������             ����������������������������Ŀ

�� C>_               �����             � COGNITIVE DEVELOPMENT BBS  �

��                   �����             �                            �

��                   �����             � Providing a forum for A.I. �

��                   ��o��             � researchers and developers �

��                   ��o��             ����������������������������Ĵ

����������������������� ��             � Basingstoke, Hampshire, UK �

��������������������������             �                            �

���������������������������������������      � 0256 50086 24 hours a day  �

������������������������ ���/�[��]��� �      �                            �

���������������������������������������      �   300 baud to 14,400 baud  �

������������������������ ���/�[��]��� �      ������������������������������

���������������������������������������

�������������������������������������