INSTALLING NETWARE WITH A PACKET DRIVER

April, 1989

Brigham Young University (BYU) has developed a system that

allows Novell's Netware to share the network adapter card in a PC

workstation with other networking systems.  This is accomplished

by using software that supports the PC/TCP Packet Driver

Specification.  This interface specification, proposed by FTP

Software, Inc. allows various products to concurrently utilize the

network adapter card installed in the PC.

What You Will Need

To successfully allow Netware to share the network adapter with

other software, you will need the following:

1.   The shell drivers written by BYU that will allow you to

generate a Netware shell for the PC workstation. If you are

interested in using the Novell Bridge Program with other

networking software, then you must also use the LAN drivers

that are distributed with the shell drivers.  These drivers

are written to meet Novell's specifications and operate

according to the Novell documentation found in the System

Installation manual.  These drivers can be obtained by

anonymous FTP (password guest) from dcsprod.byu.edu

(128.187.7.3). The drivers are found in a \novell sub-

directory in a self extracting archive file called:

"novell.exe".  Currently, these drivers support Netware

versions 2.1 and higher.

2.   A packet driver that supports the type of network adapter that

you are interested in using.  This software, similar to a DOS

device driver, controls the adapter card and interfaces to

the various systems that want to use it.  These are available

for a number of popular network cards including: 3COM, Western

Digital, Micom Interlan, and others. There is an Internet site

that acts as a repository for packet drivers. They can be

obtained from sun.soe.clarkson.edu (128.153.12.3) via

anonymous FTP in a directory called: pub/ka9q.  The file is

a self extracting archive file called drivers.exe.  Some

adapter card manufacturers are also beginning to supply packet

drivers with their hardware. Ask your vendor if they support

a packet driver for their card.

3.   Configuration support for the Netware file server that you

wish to communicate with.  There is a minor configuration

change necessary to the file server that you wish to

communicate with.  The file necessary to accomplish this

configuration change is found in the novell.exe file that you

retrieved for item 1.

4.   Other networking software products to operate concurrently

with packet driver Netware.  The whole reason for doing this

in the first place is to allow concurrent operation of Netware

with other software products.  It is beyond the scope of this

document to describe how you acquire them. Several that have

been tested and are compatable with Netware are: 1) PC/TCP

from FTP Software Inc. 2) KA9Q Internet Program from Phil

Karn, and 3) NCSA Telnet from National Center For

SuperComputing Applications.  Since this Netware system

follows the PC/TCP Packet Driver Specification, it should be

compatable with any other networking products that adhere to

the same specification.

Installation Instructions

1.   Generate a new IPX.COM file for your Novell Workstation.  To

prepare for this, copy the files from the directory in which

you have extracted them from the archive file, to the Netware

directories as specified in Appendix A.  Follow the

instructions for generating a Netware shell and select the

"Packet Driver" driver from the Network Driver Menu during the

SHELLGEN process.  The shell drivers have no configuration

options and will scan the interrupt vectors Ox60-Ox80 for a

loaded Packet Driver.  It will use the first one found.

2.   Install the appropriate packet driver on your PC workstation

for the adapter card that you have.  The packet driver is a

DOS "terminate stay resident" type of program that attaches

itself to DOS when it is executed.  Therefore, it should

probably be placed in the autoexec.bat file so that it is

installed when the machine starts up.  Specific instructions

for installation on the particular card you are using is

included in the drivers.exe archive file that you retreived

earlier, or by entering the driver name without any options

at the DOS prompt.

3.   To configure the file server for proper operation, you must

utilize a Novell utility called "econfig" to change the

network operating parameters on your existing file server.

Detailed description on the use of econfig is found in

Appendix B of this document.  A copy of econfig.exe has been

included in the novell.exe file that you retrieved.

4.   Install other software products for the packet driver as

specified in their respective installation manuals.

APPENDIX A

CONTENTS OF THE NOVELL.EXE ARCHIVE FILE

BYU Packet Driver development for Novell LANs version 2.00.

Kelly McDonald      kelly@dcsprod.byu.edu

Eric Henderson      eric@dcsprod.byu.edu

These drivers conform to the specifications in FTP software's

PC/TCP Packet Driver Specification, Revision 1.05. When a shell or

bridge is generated using these drivers, it is possible to share

a NIC with other software such as FTP's TCP/IP software.

The archive file contains the following files:

READ.ME        This file

PDSHELL.OBJ    files for 2.1 shell generation

PDSHELL.LAN         "

APDLAN.OBJ     Novell 2.1 LAN driver for bridges

BPDLAN.OBJ          "

CPDLAN.OBJ          "

DPDLAN.OBJ          "

PDLAN.LAN           "

ECONFIG.EXE    Novell's configuration utility for shells

and drivers. (Type ECONFIG ? for help).

These drivers are expecting a Packet Driver Interface to already

be loaded somewhere between INT 60h and 80h.

The shell drivers have no configuration options and will scan the

interrupt vectors for the Packet Driver. This means that if several

Packet Driver interfaces are loaded at one time, the first found

(lowest) will be used to access Novell Ethernet packets.

The LAN drivers have 5 options. Options 1-4 allow for multiple LAN

Packet Drivers by specifying INT 60h,61h,62h,or 63h. The fifth

option is for a Bridge that has only one PD and the driver will

simply find which INT to use as in the shell driver.

PD*.LAN has been set up so the generation utilities will search

LAN_DRV_001 for the 2.1 OBJ files. Copy ?PDLAN.OBJ, PDSHELL.OBJ,

and PDLAN.LAN into LAN_DRV_001 and then copy PDSHELL.LAN into

SHGEN-1.

These drivers only operate on the Novell Ethernet type 8137 and not

the 802.3 protocol most often used by Novell.  To allow for the

correct packets it is necessary to run Novell's ECONFIG utility on

any bridge or server which needs to be visible by the packet

driver.

APPENDIX B

ECONFIG INSTRUCTIONS

The ECONFIG utility allows the system manager to change the driver

Novell supplies for your Ethernet LAN card to the Ethernet protocol

specification.  The system manager needs to run the ECONFIG utility

against the OS or bridge files.  The OS or bridge files are called

NET$OS.EXE and BRIDGE.EXE.  ECONFIG contains several parameters

that a system manager needs to enter to properly configure the LAN

drivers as Ethernet drivers.

The system manager should follow the ECONFIG command with the

server volume, the file name, and a parameter as follows:

USAGE: ECONFIG [VOLUME:]FILE [PARAMETER LIST]

[parameter list] is one of the following:

A-D:[configuration type]

[configuration type] = N(normal 802.3), E[type constant]

[type constant] = 0-FFFF (8137 is Novell's assigned type

constant)

Example:  econfig net$os.exe A:n; B:e 8137

econfig bridge.exe a:e 8137;b:n

The volume refers to the volume on which the file resides, the file

refers to either the OS or bridge file.  The parameter refers to

the LAN card in the device the system manager is configuring.  Each

NetWare file server can support up to four LAN cards, therefore

Novell assigns letters (A-D) to correspond with each card.  The

constant is Novell's hexadecimal protocal number, 8137.  After

ECONFIG runs, the driver will be properly configured for the

Ethernet specification.

The system manager can enter ECONFIG from the command line,

following with the file's name and the Enter key(<Enter>) in order

to display a particular system's configuration.  The network will

display the current protocol of each LAN driver installed in the

server.

The system displays a device that contains an 802.3 driver (LAN A)

and an Ethernet driver (LAN B) as follows:

A:econfig bridge.exe

A: Novell Ethernet       (IEEE 802.3 Compatible)

B: Ethernet    Typefield: 8137 (Assigned Novell)

type constant)

Users that have the packet driver interface will not be able to

attach to the NetWare file server if the system manager fails to

use ECONFIG, or uses it improperly to install the driver files on

the file server.  When users try to load the shell, they will get

a "file server not found" error message on their workstations.  In

this case, the error message is caused because the bridge or shell

sends out IEEE 802.3 protocol packets that the packet driver

interface does not recognize because the packets lack a "type"

constant in that field.

For further information read "Changing horses midstream", LANTIMES,

Feb. 1989 issue, p.90.