Driver Compatibility Reference Guide

for HP EtherTwist LAN Adapters

June 1, 1993

===================================================

|           HP EtherTwist Adapter Cards           |

===========================================================================

Networking  | Operating | 32-bit    16-bit    16-bit    16-bit    8-bit   |

vendor/prod | System    | EISA      AT        MCA       AT        AT      |

| Versions  | HP27248   HP27247B  HP27246A  HPJ2405A  HP27245 |

|           | (h)       HP27252A                              |

|           |           HP27269B                              |

===========================================================================

Novell      | DOS x.x   | C-IPX     C-IPX     C-IPX       -       C-IPX   |

NetWare 286 | WIN 3.x   | C-IPX     C-IPX     C-IPX       -       C-IPX   |

| NN  2.x   | S-IPX     S-IPX     S-IPX       -       S-IPX   |

| NN  2.2+A1|  -        S-ATALK1    -         -        -      |

| NN  2.2+A2|  -        S-ATALK2    -         -        -      |

------------|-----------|-------------------------------------------------|

Novell      | DOS   x.x | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

NetWare 386 | DRDOS 6.0 | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

| WIN   3.x | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

| DOS   x.x | C-IPX     C-IPX     C-IPX       -       C-IPX   |

| WIN   3.x | C-IPX     C-IPX     C-IPX       -       C-IPX   |

| OS/2  1.x | R-ODI     R-ODI     R-ODI     R-ODI     R-ODI   |

| OS/2  2.x | R-ODI     R-ODI     R-ODI     R-ODI     R-ODI   |

| NN    3.11| S-ODI     S-ODI     S-ODI     S-ODI     S-ODI   |

| NN    4.00| S-ODI     S-ODI     S-ODI     S-ODI       -     |

------------|-----------|-------------------------------------------------|

Novell LAN  | DOS   x.x | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

Workplace   | DRDOS x.x | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

------------|-----------|-------------------------------------------------|

NW Lite     | DOS   x.x | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

v1.x        | DRDOS x.x | C-ODI     C-ODI     C-ODI     C-ODI     C-ODI   |

------------|-----------|-------------------------------------------------|

Microsoft   | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

LAN Mangr   | WIN   3.x | NDIS2(a)  NDIS2     NDIS2     NDIS2     NDIS2   |

v2.0, v2.1  | OS/2  1.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

| OS/2  2.0 | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

Microsoft   | W4WG  3.1 | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------| ------------------------------------------------|

Artisoft    | DOS   x.x |     -       -         -       AM21        -     |

LANtastic   |           |                                                 |

v4.11, v5.0 |           |                                                 |

------------|-----------|-------------------------------------------------|

HP LAN      | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

Manager     | OS/2  1.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

HP LM/X (b) | DOS   x.x | NDIS2     NDIS2     NDI2      NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

Banyan      | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

Vines (g)   | OS/2  1.x | NDIS2(c)  NDIS2(c)  NDIS2(c)  NDIS2(c)  NDIS2(c)|

| VINES 5.x | Native(i) Native(i) Native(i)   -          -    |

------------|-----------|-------------------------------------------------|

FTP Inc.    | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

PC/TCP Plus | DOS   x.x |   -       PktDvr    PktDvr    PktDvr    PktDrv  |

PC/TCP      |           |                                                 |

------------|-----------|-------------------------------------------------|

3Com 3+Open | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

v1.1 XNS    | OS/2  1.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

DEC Pathwks | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

v4.0        | OS/2  1.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

NCSA TCP/IP | DOS   x.x |  -        PktDvr    PktDvr    PktDvr    PktDvr  |

Srvcs v2.3  |           |                                                 |

------------|-----------|-------------------------------------------------|

Hughes ProL | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

WRQ         | DOS   x.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

Reflections | WIN   3.x | NDIS2     NDIS2     NDIS2     NDIS2     NDIS2   |

------------|-----------|-------------------------------------------------|

Santa Cruz  | UNX 3.2.2 | hpe(e)    hpi(d)    hpm(f)    a2x(f)    hpi(e)  |

Operation   | UNX 3.2.4 | hpe(e)    hpi(d)     -        a2x(f)    hpi(f)  |

| ODT 1.x   | hpe(e)    hpi(d)    hpm(f)    a2x(f)    hpi(e)  |

| ODT 2.0   | hpe(e)    hpi(d)     -        a2x(f)    hpi(e)  |

| MPX 1.x   | hpe(e)    hpi(d)    hpm(f)    a2x(f)    hpi(e)  |

| MPX 2.0   | hpe(e)    hpi(d)     -        a2x(f)    hpi(e)  |

------------|-----------|-------------------------------------------------|

HP OfficeSh | DOS   x.x |   -         -       OSD                 OSD     |

------------|-----------|-------------------------------------------------|

LEGEND:

DOS x.x   OS v3.x, DOS v4.01, or DOS v5.0

DRDOS 6.0 Digital Research DOS v6.0

WIN 3.x   OS v5.0 and Windows v3.0 or v3.1

NN 2.x    Novell NetWare 286 Server 2.15 or v2.2

NN 2.2+A1 Novell NetWare 286 Server v2.2 with Appletalk Phase 1 VAP

NN 2.2+A2 Novell NetWare 286 Server 2.2 with Appletalk Phse 2 VAP

NN 4.0    Novell NetWare 386 Server v4.0

NN 3.11   Novell NetWare 386 Server v3.11

VINES 5.0 Banyan VINES Unlimited 386/486 Server v5.00

UNX3.2.2  SCO Unix v3.2.2 including TCP v1.1.3, NFS v1.1.1, Retix OSI,

and LAN Manager for Unix Systems v1.0

UNX3.2.4  SCO Unix v3.2.4 including TCP v1.2, NFS v1.2, Retix OSI,

and LAN Manager for Unix Systems v1.1

ODT 1.x   SCO OpenDesktop v1.0 or v1.1

ODT 2.0   SCO OpenDesktop v2.0

MPX 1.x   SCO Multiprocessor Extension v1.0 or v1.1

MPX 2.0   SCO Multiprocessor Extension v2.0

NDIS2     Driver supports both server and client unless otherwise noted.

C-IPX     Netware DOS IPX client

S-IPX     Netware v2.15 or v2.2 Server

S-ATALK1  Netware v2.2 Server with AppleTalk Phase 1 VAP

S-ATALK2  Netware v2.2 Server with AppleTalk Phase 2 VAP

C-ODI     Netware DOS ODI client

R-ODI     Netware OS/2 ODI requester

S-ODI     Netware 386 v3.11 Open Data-link Interface (ODI) Server

Native    Native Banyan server driver

PktDvr    Packet Driver (v9 on Support Disk, v10 in Crynwr collection)

OSD       OfficeShare Driver

hpe       HP EISA driver for SCO (HP 27248A)

hpi       HP ISA driver for SCO (HP 27245A, 27247A, 27247B, 27252A, 27250A)

hpm       HP Micro channel driver for SCO (HP 27246A)

A2X       HP NC16 adapter driver for SCO (HP J2405A)

AM21      HP NC16 adapter driver for Artisoft LANtastic

-        Presently not available.

NOTES:

(a)    Supports Windows 3.1 all modes and Windows 3.0 standard mode

(b)    NDIS2 clients running under HP LAN Manager v1.0 or v1.1

on the PC can communicate with HP 9000 LM/X servers.

(c)    NDIS2 OS/2 client only

(d)    Available on HP Support Disk for Adapter/16 Plus cards for SCO

Unix, ODT, and MPX as an update to the SCO LLI Extended Feature

Supplement v3.0 or v3.1.  The driver name is hpi.  This driver

will be bundled with the next release of the SCO LLI Drivers Disk

(expected late 1993).

(e)    Available from SCO on the LLI Drivers Disk Extended Feature

Supplement v3.0 or v3.1.

(f)    Available on HP Support Disk for the product.

(g)    Client driver requires Banyan Vines version 4.00 (2) with patch release

2NNN, or later versions.

(h)    If QEMM 6.x is installed, use LOCKDMA option on its command line

(e.g. DEVICE=QEMM.SYS LOCKDMA)

(i)    A native Banyan server driver is available for the product through

Banyan Systems Inc. or your Banyan reseller.