IBM ThinkPad 750/755/360 Families Utilitiy Diskette V2.00   Sep. 28 1994

=========================================================

This package contains the following diskettes for ThinkPad 355/360/750/755

family of computers:

IBM ThinkPad Utility Diskette 1      V2.00

IBM ThinkPad Utility Diskette 2      V2.00

These diskettes, Utility Diskette 1 and 2, were released to support the

IBM ThinkPad Dock II.

If you are experiencing one of the problems listed in the following

Summary of Problem Fixes section, please follow the instructions in your

Thinkpad User's Guide to update the utility programs of your ThinkPad

355/360/750/755 family computer.

Summary of Problem Fixes

------------------------

<< Utility Diskette 1 >>--------------------------------------------------------

The following is a list of problems fixed by the IBM ThinkPad Utility

Diskette 1.

Released Version

----------------

Version 1.00   Jul.  8 1993   Initial Release for ThinkPad 750/750C/750P

Version 1.20   Dec.  3 1993   2nd Release for ThinkPad 750/750C/750P/750Cs/750Ce

Version 1.21   Feb. 15 1994   3rd Release for ThinkPad 750 familiy

Version 1.30   Feb. 22 1994   4th Release for ThinkPad 355/360/750/755 family

Version 1.31   Apr. 18 1994   5th Release for ThinkPad 355/360/750/755 family

Version 2.00   Jun. 24 1994   6th Release for ThinkPad 355/360/750/755 family

Changed/Fixed Item

------------------

Note: n.nn  Diskette Version (e.g. 1.20  Version 1.20)

(New) New function or enhancement

Installation Program

1.20  (New) ThinkPad 750Cs and 750Ce are supported.

1.20  The installation program for Windows doesn't work in Win-OS/2 session.

1.20  The Fuel-Gauge program for DOS can't be installed at the DOSSHELL.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  The installation program for Windows doesn't allow long/deep path names.

DOS Utility

- DOS ThinkPad Feature Program -

1.20  (New) ThinkPad 750Cs and 750Ce are supported.

1.20  (New) A warning is displayed when IRQ15 is selected.

1.20  (New) A warning is displayed when IRQ11 is selected with

the Port Replicator with PCMCIA.

1.20  (New) An option "Enable/Disable the speaker indicator" is added.

1.20  Resume timer can be selected even if Hibernation Timer is On.

1.20  Set Display Feature Menu selection isn't saved correctly.

1.20  Display High Performance Mode should be always On (750C).

1.20  Items in Suspend and Resume Menu can't be selected by the space bar

when the Dock-1 is attached.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  (New) Three 'Incoming call' items in the Resume options are integrated

into one item.

1.31  The Help menu indicates invalid items.

2.00  (New) DOS Docking Control utilities for Dock II were added.

- DOS Fuel-Gauge Program -

1.20  Incorrect Japanese characters are displayed with 8MB IC DRAM Card.

1.21  Frequent system hang-up at idle time or screen saver.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  Fuel-gauge program doesn't work correctly.

1.30  Duration of Remaining Power is not correct when the processor speed

is changed.

- DOS PCMCIA Utility -

1.20  (New) EasyPlaying is supported.

2.00  EasyPlaying was moved to Utility Diskettte 2.  Refer to Utility

Diskette 2 section.

Windows Utility

- Windows ThinkPad Feature Program -

1.20  (New) ThinkPad 750Cs and 750Ce are supported.

1.20  (New) A warning is displayed when IRQ15 is selected.

1.20  (New) A warning is displayed when IRQ11 is selected with

the Port Replicator with PCMCIA.

1.20  The LCD is still active when Display High Performance Mode is On.

1.21  The way to open the 'More' Window is changed.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  (New) Three 'Incoming call' items in Resume options are integrated

into one item.

1.30  It is slow to open the 'More' panel from the 'Power' menu.

1.31  (New) Processor Upgrade Kit for 750Ce is supported.

2.00  (New) Windows Docking Control utilities for Dock II were added.

- Windows Fuel-Gauge Program -

1.20  (New) ThinkPad 750Cs and 750Ce are supported.

1.21  Character lost under the high speed communication.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  (New) Check if 'MS-DOS with APM' was chosen on Windows setup.

1.30  The screen position is not saved on exit.

1.30  Duration of Remaining Power is not correct when the processor speed

is changed.

- Windows PCMCIA Utility Program -

1.20  (New) EasyPlaying is supported.

2.00  EasyPlaying was moved to Utility Diskettte 2.  Refer to Utility

Diskette 2 section.

OS/2 Utility

- OS/2 ThinkPad Feature Program -

1.20  (New) ThinkPad 750Cs and 750Ce are supported.

1.20  (New) A warning is displayed when IRQ15 is selected.

1.20  (New) A warning is displayed when IRQ11 is selected with

the Port Replicator with PCMCIA.

1.20  (New) An option "Enable/Disable the speaker indicator" is added.

1.20  The LCD is still active when Display High Performance Mode is on.

1.21  Character lost under the high speed communication.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  (New) Three 'Incoming call' items in Resume options are integrated

into one item.

1.30  It is slow to open the 'More' panel from the 'Power' menu.

1.30  The description about the Fuel-Gauge program in the Help is incorrect.

1.31  (New) Processor Upgrade Kit for 750Ce is supported.

1.31  163/173 errors occur when the Speaker Indicator is enabled (the 755/360).

- OS/2 Fuel-Gauge Program -

1.21  Character lost under the high speed communication.

1.30  (New) ThinkPad 755/360 families are supported.

1.30  The screen position is not saved on exit.

1.30  Duration of Remaining Power is not correct when the processor speed

is changed.

- OS/2 PCMCIA Utility Program -

1.20  (New) EasyPlaying is supported.

2.00  EasyPlaying was moved to Utility Diskettte 2.  Refer to Utility

Diskette 2 section.

Others

1.20  A bitmap data is added for the wallpaper.

1.30  (New) Windows Large Pointer Program is added.

1.30  (New) IBM ThinkPad Space Saver Keyboard for DOS is supported.

1.30  (New) IBM ThinkPad Space Saver Keyboard for OS/2 is supported.

1.30  A special patch program for MOUSE.COM 9.01 is added to fix that

the system may hang after spend/resume operation.

1.30  A special patch program for MOUSE.COM 8.20 or later to fix that

the keyboard/mouse may hang if the Power on Password is installed.

1.31  Serial mouse doesn't work with the IBM ThinkPad Space Saver Keyboard.

1.31  QEXPUNIT.EXE returns an incorrect return code (the 755/360 families).

<< Utility Diskette 2 >>--------------------------------------------------------

EasyPlaying

===========

The utilities and drivers related to PCMCIA were seperated as Utility

Diskette 2.

The following is a list of problems fixed by the IBM ThinkPad Utility

Diskette 2.

Released Version

----------------

Version 2.00   Jun. 13 1994   Initial Release for ThinkPad 355/360/750/755

family

Changed/Fixed Item

------------------

Note: n.nn  Diskette Version (e.g. 1.20  Version 1.20)

(New) New function or enhancement

- DOS PCMCIA Utility -

1.20  (New) EasyPlaying is supported.

2.00  EasyPlaying was moved to Utility Diskettte 2.

(New) Card Services specification release 2.10 is supported.

(New) DISKDRV.SYS (for SRAM/ATA) was added.

- Windows PCMCIA Utility Program -

1.20  (New) EasyPlaying is supported.

2.00  EasyPlaying was moved to Utility Diskettte 2.

- OS/2 PCMCIA Utility Program -

1.20  (New) EasyPlaying is supported.

1.31  Problem: 'Hang when shutting down OS/2 2.11 when EasyPlaying is

open.' was solved.

1.31  Problem: 'OS/2 2.11 DeskTop is rebuilt when EasyPlaying is

closed.' was solved.

2.00  EasyPlaying was moved to Utility Diskettte 2.

How to Install EasyPlaying

--------------------------

DOS

- Insert the ThinkPad utility diskette No2 into the diskette drive.

- Type "A:\UINSTALL" and press ENTER.

- Follow the instruction on the screen.

- Restart the system

Windows(includes DOS)

- Start Windows

- Select "File" from Program Manager window, then select "Run..."

from the pull-down menu

- Insert the ThinkPad utility diskette No2 into the diskette drive.

- Type "A:\PCMINSTW" and press ENTER.

- Follow the instruction on the screen.

- Restart the system

OS2

- Start an "OS2 Full Screen" or an "OS/2 Window"

- Insert the ThinkPad utility diskette No2 the diskette drive.

- Type "A:\PCMINST2" and press ENTER.

- Follow the instruction on the screen.

- Restart the system

Parameter Definitions:

Socket Services

/C0=z   : This parameter defines the IRQ level to be used by PCMCIA.

The OS2 Card Services and Socket Services use a H/W interrupt

to notify PCMCIA status change. The default interrupt(IRQ)

level for IBM products is IRQ15. This level can be change by

this parameter.

For Example, "/C0=11" defines the PCMCIA IRQ as 11.

NOTE: This is valid ONLY for OS2 Socket Services (IBM2SS01.SYS

IBM2SS02.SYS) and in case of IBM2SS02.SYS, "/C=z" can

be used instead of "/C0=z".

/C1=z   : This parameter defines the IRQ level to be used by PCMCIA in

a docking station or port replicator.

The default interrupt(IRQ) level for the docking station or

the port replicator is IRQ11. This level can be change by

this parameter.

For Example, "/C1=9" defines the PCMCIA IRQ as 11.

NOTE: This is valid ONLY for OS2 Socket Services for ISA

system (IBM2SS01.SYS).

/IG0=y  : This parameter defines a slot number where Socket Services

will NOT take care it. If you use the "Point enabler" supplied

with a PC card, you should take into consideration for the

coexistence with Socket services.

For Example, "/IG0=1" defines the slot 1 is reserved for a

"Point enabler" and Socket Services will ignore the slot and

not handle the slot any more.

/RI0=x  : This parameter defines a slot number where Socket Services

will disable RI_OUT signal. If you are using a modem card

with ThinkPad family system, and you can not enable "suspend"

or "hibernation" function while the modem card is inserted,

please try this parameter.

For Example, "/RI0=1" defines the slot 1.

/D      : This parameter enables "Warm docking" capability from the

view point of PCMCIA hardware.

Card Services

/R=200  : This parameter changes Card Services Specification release

level. The default Card Services Specification release level

of DOS Card Services driver (IBMDOSCS.SYS) is 2.10. If this

parameter is specified, the DOS Card Services driver conforms

to the Card Services Specification release 2.00.

NOTE: This is valid ONLY for the DOS Card Services (IBMDOSCS.

SYS).  The OS2 Card Services (PCMCIA.SYS) conforms to the

Card Services Specification release 2.00, and does not

support Card Services Specification release 2.10 now.

Resource Map Utility

/MA=xxxx-yyyy :

Specifies the address range available to Card Services and

the PCMCIA cards. Should be in the range of C000h to DFFFh.

One or more ranges may be specified in 4K blocks with a comma

(,) separating the address ranges.

NOTE: If EMM386 or another EMS memory manager is used, this

area must be excluded from use by the memory manager

(ie. "EMM386  X=xxxx-yyyy").

NOTE: The minimum size of this area depends on the type of

PCMCIA card used.

- Card Services           4K  (mandatory)

- Modem                   0K

- IBM Ethernet           16K

- IBM Token Ring         24K

- IBM 3270                8K

- SRAM/ATA                4K

NOTE: If using Windows 3.1, the area specified by the "/MA="

parameter must also be included in the "EMMEXCLUDE="

statement in the SYSTEM.INI file.

NOTE: You do need this parameter for OS2.

/IX=n     Specifies the IRQ level (or levels) that should not be used

by PCMCIA cards where n = IRQ level. This parameter is used

to avoid possible IRQ conflicts. The Resource Map Utility

can automatically detect many IRQ's already in use, but may

not detect all. Multiple IRQ's may be specified by separating

with a comma (,).

For Example, the IRQ's of the SCSI controller in the Dock I

for ThinkPad family (IRQ11 as default) is not automatically

detected by the Resource Map Utility. So, if a PCMCIA card

client driver requests the IRQ from Card Services, a conflict

may result unless this parameter is used (/IX=11) to specify

that the SCSI IRQ's should not be used.

Hints:

- Set system serial port as COM1.

- If you use EMS driver with PCMCIA drivers under DOS/Windows environment,

the memory area specified by /MA must be excluded from usable area for

the EMS driver.

< CONFIG.SYS >

DEVICE=C:\...\EMM386.EXE ... /X=xxxx-yyyy

DEVICE=C:\...\DICRMU01.SYS /MA=xxxx-yyyy

< SYSTEM.INI for Windows >

EMMExclude=xxx-yyyy

- The Auto Configurator for DOS/Windows (AUTODRV.SYS) can enable

Modem cards, Data/Fax Modem cards and IBM 3270 card as its default

setting.

- The Super Storage driver for DOS/Windows (DISKDRV.SYS) can enable

SRAM cards and hard disk(ATA) cards for read/write operation, and

enable Flash Memory cards for read operation if the cards are

formatted as FAT format.

- Before using a brand-new hard disk card, you may need to initialize

the card by EZPLAY.EXE program.

- The Auto Configurator for OS2 (AUTODRV2.SYS) can enable Modem cards

or Data/Fax Modem cards.

- The Super Storage driver for OS2 is not available in this edition.

- If you use SRAM cards and do not use hard disk(ATA) cards, you can use

the SRAMDRV2.SYS driver instead of the DISKDRV.SYS driver to reduce

driver size.

- EZPLAYW supports Windows 3.1 Enhanced Mode only.

- For the OEM PCMCIA cards, please refer PCMCIA.CRD file for more

detail support level.

- If you use Compaq Contura AERO 4/25, power on the system with the

diskette drive attached.  After power on the system, you can attach/

detach the diskette drive.

- If you use CompuAdd 433TXC, you may need to touch any key or mouse

in order to update the socket information.

How to Run EasyPlaying

----------------------

DOS

You can see current PCMCIA socket/card status by EZPLAY.EXE

Windows 3.1

EZPLAYW.EXE will be started automatically when Windows is started.

You can get more information for this program by reading "Help"

screen in the EZPLAYW program.

OS2 2.1

EZPLAY2.DLL is registered as an object of DeskTop. You can see an

ICON titled "EasyPlaying". To start EZPLAY2.DLL, click this ICON.

You can get more information for this program by reading "Help"

screen in the EZPLAY2 program.

Note:

- The Auto configurator for DOS,Windows (AUTODRV.SYS) can enable

Modem cards, Data/Fax Modem cards and IBM 3270 card as it default

setting. The driver can enable the following PCMCIA cards as option

by modifying AUTODRV.INI file:

IBM Token Ring card

IBM Ethernet cards

If you want to enable these cards by the AUTODRV.SYS, please modify

the AUTODRV.INI file as follows:

Option=Ignore   ===>    ;Option=Ignore

Note: The AUTODRV.SYS does not have NDIS or ODI driver capability,

so this function may be useful only for IBM LSP/NATIVE mode

environment. If you are using the CS20TOK.SYS driver, you may

use the AUTODRV.SYS driver instead of the CS20TOK.SYS.

- The Auto configurator for OS2 (AUTODRV2.SYS) can enable Modem cards

or Data/Fax Modem cards and it does not support other communication

cards.

- For the OEM PCMCIA cards, please refer PCMCIA.CRD file for more

detail support level.

- EZPLAYW supports only Windows 3.1 Enhanced Mode.

EasyPlaying Files

-----------------

READ.ME      : Read Me file

PCMCIA.CRD   : PC Card Support List

PCMINST2.EXE : Installation Program for OS2 2.1

PCMINST2.DAT : Installation Program Data File for OS2

PCMINSTW.EXE : Installation Program for Windows

PCMINSTW.DAT : Installation Program Data File for Windows

UINSTALL.EXE : Installation Program for DOS

PCMINST.DAT  : Installation Program Data File for DOS

PCMINST.EXE  : Installation Program for DOS

PCMCD01B.IDP : Installation Program Data File for DOS

IBMDOSCS.SYS : Card Services driver for DOS

PCMCIA.SYS   : Card Services driver for OS2

IBMDSS01.SYS : Socket Services driver for DOS

IBM2SS01.SYS : Socket Services driver for OS2

DICRMU01.SYS : Resource Map Utility driver for DOS

ICRMU01.SYS  : Resource Map Utility driver for OS2

VPCMCIA.SYS  : Virtual Card Services driver for OS2

$ICPMDOS.SYS : PCMCIA Power Management Support driver for DOS

$ICPMOS2.SYS : PCMCIA Power Management Support driver for OS2

EZPLAY.EXE   : EasyPlaying for DOS

EZPLAYW.EXE  : EasyPlaying for Windows

EZPLAYW.HLP  : EasyPlaying for Windows help file

EZPLAY2.DLL  : EasyPlaying for OS2

EZPLAY2.HLP  : EasyPlaying for OS2 help file

AUTODRV.SYS  : Auto Configurator for DOS

AUTODRV.INI  : Auto Configurator for DOS Data file

AUTODRV2.SYS : Auto Configurator for OS2

AUTODRV2.INI : Auto Configurator for OS2 Data file

DISKDRV.SYS  : Super Storage driver for DOS/Windows

SRAMDRV2.SYS : SRAM driver for DOS/Windows

IBMVCD.386   : Windows 3.1 Virtual COM driver (PCMCIA Support)

COM.SYS      : Updated COM.SYS for OS2 (PCMCIA Support)

<<<<< PC Cards for IBM(*) ThinkPad(*) systems >>>>>

The following PC Cards have been tested successfully on the IBM

ThinkPad systems. For detailed information, contact each card

manufacturer.

Tested ThinkPad system:

ThinkPad 750 Family

< Communication Cards >

Card Type    Company                                            Support

PCMCIA Card Product Name                         Level

------------------------------------------------------------------------

Modem

Apex(**) Data,Inc.

Apex Data, data/fax modem, Model IBP-1414           AA

Apex Data, data/fax modem, Model IBP-2496           AA

Apex Data, data/fax modem, Model PCR-9696           AA

AT&T Paradyne

KeepIntouch Card(**) Modem                          AA

Complus Inc.

FaXXpress 942                                       AA

COM1 SA

MC215 Pcmcia Card COM1                              AA

MC216 Pcmcia Card COM1                              AA

DATA RACE (**)

RediCARD (**) PCMCIA Data/FAX Modem                 AA

RediCARD High Speed PCMCIA Data/FAX Modem           AA

ELSA GmbH   (Germany)

MicroLink(**) 2460MC FullFax-Modem                  AA

E-TECH Inc.

PCMCIA Data/Fax Modem Model C9624RX                 AA

EXP Computer, Inc.

THINFAX fax/data modem                              AA

Dr. Neuhaus Mikroelektronik GmbH

Fury card(**) 2400                                  AA

Intel(**)

PCMCIA Faxmodem  14.4/14.4 (U.S.version)            AA

PCMCIA Faxmodem  9600/9600 (U.S.version)            AA

PCMCIA Faxmodem  2400/9600 (U.S.version)            AA

MagicRAM(**)

Mercury 9624(**) Voice/Fax/Modem card               AA

MX MACRONIX(**)

MaxCard 144                                         AA

Megahertz(**)

Data/Fax Modem with XJACK(**) Model #XJ124FM        AA

Data/Fax Modem with XJACK(**) Model #XJ1144         AA

OMRON(**)

data/fax modem MD24XCA                              A

OKI(**)

Modem Card                                          A

RATOC SYSTEM Inc.

REX-5565 DATA/FAX MODEM Card                        AA

SCM(**) Microsystems Europe

SCM_FAX 2400/9600                                   A

SMART(**) Moduler Technologies

SmartExchange(**) 9624                              AA

TDK(**)

Data/Fax Modem Card                                 AA

TEXAS(**) INSTRUMENTS

2400+ Send Fax Modem                                A

IBM

PCMCIA Data/FAX Modem                               AA

High Speed PCMCIA Data/FAX Modem                    AA

2400bps Data Modem PCMCIA Adapter Card              AA

Data/Fax PCMCIA Modem Card                          AA

IBM Microelectronics

2.4/9.6 Data/FAX Modem                              AA

14.4/14.4 Data/FAX Modem                            AA

Serial

Socket(**) Communications.Inc

Socket I/O credit card serial adapter               AA

IBM Microelectronics

RS232 Serial Card                                   AA

SDLC

IBM

SDLC PCMCIA Modem Card                              B

Ethernet

AMP(**) Incorporated

Smart Connector Systems LAN 10Base-T                B

CardWell(**) International Corp.

PCMCIA Ethernet LAN Card 10Base-T                   B

PCMCIA Ethernet LAN Card 10Base-2                   B

CONTEC Co.,Ltd

LAN-HELPER C-NET(PM)C series 10 Base-T              B

LAN-HELPER C-NET(PM)C series 10 Base-2              B

New Media corporation

PCMCIA LAN Adapter Card 10 Base-T                   B

PCMCIA LAN Adapter Card 10 Base-2                   B

MagicRAM

Ethernet 10 Base-T Credit Card Adapter              B

Ethernet 10 Base-2 Credit Card Adapter              B

RATOC SYSTEM Inc.

REX-5585 10Base-T Ethernet card                     B

RIOS

IC card Ethernet 10BaseT                            B

IC card Ethernet 10Base2                            B

Socket Communications.Inc

EA+(**) PCMCIA LAN adapter                          B

EA  PCMCIA LAN adapter                              B

SCM Microsystems Europe

SCM_EC (10 Base 2)                                  B

SCM_ET (10 Base T)                                  B

TDK

Ethernet 10BASE-T LAC-CD011                         B

Ungermann-Bass

Access/CARD                                         B

Xircom(**)

CreditCard Ethernet Adapter                         B

IBM

Ethernet Credit Card Adapter 10BaseT                B

Ethernet Credit Card Adapter 10Base2                B

Token-Ring

RIOS

IC card Token-Ring                                  B

MagicRAM

16/4 Token-Ring Credit Card Adapter                 B

SCM  Microsystems Europe

SCM_TR Token-Ring card                              B

IBM

Token-Ring Credit Card Adapter                      B

3270

IBM

3270 EMULATION CREDIT CARD ADAPTER                  A

5250

IBM

5250 PCMCIA Adapter Card                            B

5250 EMULATION PCMCIA ADAPTER                       B

Infrared

IBM Microelectronics

PCMCIA Infrared Wireless Adapter                    B

< Memory/Storage Cards >

Card Type    Company                                            Support

PCMCIA Card Product Name                         Level

------------------------------------------------------------------------

ATA

CardWell International Corp.

PCMCIA ATA Hard Disk Drives                         A1

EPSON(**)

Flash Packer(**)                                    A1

Maxtor(**)

MobileMax(**) Hard Drive Model MXL-105-III          A1

SunDisk(**)

SDP-xx series                                       A1

Integral(**) peripherals

Model 1841PA                                        A1

IBM

Solid State Mass Storage Card (5M/10M/15M)          A1

40MB PCMCIA Hard Disk Drive                         A1

105MB PCMCIA Hard Disk Drive                        A1

IBM Microelectronics

PCMCIA Solid State File Card (5M/10M/20M/30M/40M)   A1

External PCMCIA Drive (364MB/527MB)                 A1

Flash

MagicRAM

Flash memory card                                   A2

Maxell(**)

EF-TB series                                        A2

Fujisoku(**)

J-series memory card                                A2

Sony(**)

IMF series                                           A2

AMI(**)  American Microsystems,Inc.

ASIS(**), memory cards                              A2

SCM micro system Europe

FCxxxMB flash card  (256KB -- 20MB)                 A2

SRAM

ADTEC System Science

AXS series SRAM card                                A1

AMI  American Microsystems,Inc.

ASIS, memory cards                                  A1

ASCII

SRAM card                                           A1

Fujisoku

J-series memory card                                A1

Fujitsu(**)

SRAM card                                           A1

IBM

PCMCIA Memory Cards  1M, 2M                         A1

MagicRAM                                              A1

Dual battery SRAM card

MELCO

RJB-512/1000/2000 SRAM card  (Japan)                A1

Maxell

Maxll ML-TB                                         A1

Mitsubishi(**)

MELCARD(**) SRAM                                    A1

NEC(**)

SRAM card                                           A1

OKI

memory card series                                  A1

Panasonic(**)

Memory card SRAM-BN-xxxHMC (64KB -- 4MB)            A1

SCM micro system Europe

SCxxxxMB SRAM card (128KB -- 2MB)                   A1

SMART Moduler Technologies

SM9SRD1MP3200                                       A1

Sony

IMS series                                          A1

Toshiba(**)

MCM33 series                                        A1

< Other I/O card >

GPS

Socket Communications.Inc

PCMCIA Global Positioning System                    B

SCSI

Trantor Systems Ltd.

SlimSCSI(**) 16-bit PCMCIA-to-SCSI Adapter          B

New Media Corporation

Visual Media(**) SCSI adapter                       B

Bus Toaster(**) High Performance SCSI adapter       B

RATOC SYSTEM Inc.

REX-5535 SCSI PC card                               B

IBM

PCMCIA SCSI Card                                    A

Audio

New Media Corporation

.WAVjammer PCMCIA Sound card                        B

Panasonic

Sound Card CF-VEW211                                B

IBM

PCMCIA 16-bit Audio Adapter                         B

< Support Level >

A   Tested successfully with IBM Auto Configurator.(AUTODRV.SYS).

AA  Tested successfully with IBM Auto Configurator.(AUTODRV.SYS).

and OS/2 Auto Configurator (AUTODRV2.SYS)

A1  Tested Read/Write successfully with IBM SRAM/ATA Driver.(DISKDRV.SYS)

A2  Tested Read successfully with IBM SRAM/ATA Driver.(DISKDRV.SYS)

B   Tested successfully with the device driver or utility program supplied

with the PC card.

You must specify the device statement of this client device driver

before the device statement of Auto Configurator (AUTODRV.SYS).

This list does not include all of the products that may run on the

ThinkPad systems. IBM has not evaluated these products and makes no

comments or judgements about their functions, quality or performance

or country certification requirements. Listed products may not be

available in all countries.

< Trademarks >

The following terms, denoted by a double asterisk(**), are trademarks of

other companies:

ASIS               American Microsystems,Inc.

AMP                AMP Inc.

APEX               Apex Data,Inc.

CardWell           CardWell International Corp.

COM1               COM1 SA

DATA RACE          DATA RACE, Inc.

EXP                EXP Computer, Inc

KeepIntouch Card   AT&T Paradyne corp.

FURYCARD           Dr. Neuhaus Mikroelektronik GmbH

MicroLink          ELSA GmbH

MX MACRONIX        MACRONIX, Inc.

FUJISOKU#          FUJISOKU corp.

Fujitsu            Fujitsu Ltd.

MagicRAM           MagicRAM Inc.

Mercury 9624       MagicRAM Inc.

Maxell             Hitachi-Maxell corp.

Maxtor             Maxtor corp.

MobileMax          Maxtor corp.

Integral           Integral peripherals

Intel              Intel corp.

Megahertz          Megahertz corp.

Mitsubishi         Mitsubishi Electric corp.

MELCARD            Mitsubishi Electric corp.

NEC                NEC corp.

Visual Media       New Media corp.

OMRON              OMRON corp.

OKI                OKI Electric Industry Co.,Ltd

Panasonic          Panasonic Electronics corp.

RediCARD           DATA RACE, Inc.

SCM                SCM Microsystems Europe

SlimSCSI           Trantor Systems Ltd.

Smart              SMART Moduler Technologies

SmartRAM           SMART Moduler Technologies

SmartExchange      SMART Moduler Technologies

Socket             Socket Communicatoins.Inc.

EA+                Socket Communicatoins.Inc.

Sony               Sony corp.

SunDisk            SunDisk corp.

TDK                TDK corp.

TEXAS              TEXAS INSTRUMENTS

THINFAX            EXP Computer, Inc

Toshiba            Toshiba corp.

Ethernet           Xerox corp.

Xircom             Xircom

XJACK              Megahertz corp.

Windows            Microsoft corp.

PCMCIA             Personal Computer Memory Card International Association

The following terms, denoted by an asterisk(*), are trademarks of the IBM

Corporation in the United States or other countries:

IBM

ThinkPad