The shipping version of Remote Access Service (RAS) for Windows for Workgroups, NT 3.1, and NT 3.5 do not function properly with our V.FC and V.34 products. This apnote describes how to configure these operating environments / systems to take advantage of our new capabilities.

General Overview

RAS is a Microsoft Windows networking component. In essence, it uses a modem or other telecommunication device instead of network adapter (ethernet or token-ring) to communicate with other computer systems. The protocols that can be used with RAS include NetBeui, TCPIP, and IPX.

From the client perspective RAS takes form as a dialer. It uses information contained in MODEM.INF to configure the modem and dial a Remote Access server. Once the connection is established, authentication occurs using a userID and password. As long as the RAS client and server remain connected, the client can have access to whatever network resources are available dictated by a combination of user access rights and server connections.

The server also uses information in MODEM.INF to configure the modem and properly answer calls from RAS clients. Typically, an NT or NTAS system is used to receive calls from RAS clients. The software operates as an extension of the operating system and is able to directly communicate with other server process's such as security, network, and user authentication.

The RAS software, both client and server, contain internal error control and compression. These abilities are activated either manually by the users of each system and/or by interpreting extended result codes from the modems involved.

MODEM.INF

This file is a text based file found within the SYSTEM structure of WfW or NT. On a WfW system, it is located in x:\WINDOWS\SYSTEM. On a NT system it is located in %SystemRoot%/SYSTEM32/RAS. The format is very straightforward; global responses (modem result codes) are followed by modem specific initialization strings then modem specific responses. Here is a sample for a standard Hayes 2400bps modem:

[Responses]

;----------------------------------------

;OK, ERROR, NO DIALTONE, NO CARRIER, etc

;----------------------------------------

OK=<cr><lf>OK<cr><lf>

OK=<hFF><cr><lf>OK<cr><lf>

LOOP=<cr><lf>RING<cr><lf>

LOOP=<cr><lf>RRING<cr><lf>

LOOP=<cr><lf>RINGING<cr><lf>

LOOP=<cr><lf>DIALING<cr><lf>

ERROR=<cr><lf>ERROR<cr><lf>

ERROR=<cr><lf>ABORTED<cr><lf>

ERROR_NO_CARRIER=<cr><lf>NO CARRIER<cr><lf>

ERROR_VOICE=<cr><lf>VOICE<cr><lf>

ERROR_NO_DIALTONE=<cr><lf>NO DIALTONE<cr><lf>

ERROR_NO_DIALTONE=<cr><lf>NO DIAL TONE<cr><lf>

ERROR_BUSY=<cr><lf>BUSY<cr><lf>

ERROR_NO_ANSWER=<cr><lf>NO ANSWER<cr><lf>

ERROR_NO_PROMPTTONE=<cr><lf>NO PROMPTTONE<cr><lf>

;----------------------------------------

[Hayes Compatible 2400]

<speaker_on>=M1

<speaker_off>=M0

<autodial_on>=ATDT

<autodial_off>=ATX3D

CALLBACKTIME=10

DEFAULTOFF=compression

MAXCARRIERBPS=2400

MAXCONNECTBPS=2400

COMMAND_INIT=AT&F&C1&D2 V1 S0=0 S2=128 S7=55<speaker><cr>

COMMAND_DIAL=<autodial><phonenumber><cr>

COMMAND_LISTEN=ATS0=1<cr>

CONNECT=<cr><lf>CONNECT <carrierbps><cr><lf>

Any text contained within <> is considered a macro. For the purpose of this apnote, consider macros as string substitution. <cr> is carriage return, <lf> is line feed, and <speaker> or <autodial> are .INF defined within the scope of the modem being described. The function of each line above is self explanatory. The important aspects of this modem definition are the MAXCARRIERBPS, MAXCONNECTBPS, and CONNECT lines.

MAXCARRIERBPS defines the highest DCE-DCE connection rate.

MAXCONNECTBPS defines the highest DTE-DCE connection rate.

CONNECT defines the result code to recognize as a valid connection. The <carrierbps> macro is present so that the RAS software can auto-baud. This is not valid for modems that lock the serial port rate.

The MODEM.INF that ships with WfW or NT contains various USRobotics product models. Unfortunately, these modem definitions contain improper or invalid information for our V.FC or V.34 products. Typically, when any of these are used with our V.FC or V.34 modems, the RAS client software will typically generate an error such as: LINK PaintShopPro "C:\\USERS\\DEFAULT\\NEW-5.GIF" "Image" \aµ §

MDMUSR.INF

To remedy the problems with RAS, a new MODEM.INF has been developed. It's immediate purpose is to support the new USRobotics modems and their higher connect rates. A subsequent advantage is that this new .INF can also be used for literally EVERY previous model of the USRobotics Courier or Sportster modem!

; 9/12/94 - Christopher L. Browne

; Please notify cbrowne@usr.com in case of problems or suggestions!

; Windows for Workgroups 3.11 MODEM.INF generated for USR Courier V.34

; Tested on WfW 3.11 calling NTAS 3.1 - ok

; Tested on NT / NTAS 3.1 calling and receiving - ok

; Tested on NT 3.5 calling and receiving - ok

[Responses]

; General non-connect responses that all USR modems generate...

OK=<cr><lf>OK<cr><lf>

LOOP=<cr><lf>RING<cr><lf>

LOOP=<cr><lf>RINGING<cr><lf>

ERROR=<cr><lf>ERROR<cr><lf>

ERROR_NO_CARRIER=<cr><lf>NO CARRIER<cr><lf>

ERROR_VOICE=<cr><lf>VOICE<cr><lf>

ERROR_NO_DIALTONE=<cr><lf>NO DIAL TONE<cr><lf>

ERROR_BUSY=<cr><lf>BUSY<cr><lf>

ERROR_NO_ANSWER=<cr><lf>NO ANSWER<cr><lf>

; Since serial port is locked, carrier rate is irrelevant. The RAS

; client calculates link rate after connection is accepted.

CONNECT=<cr><lf>CONNECT <ignore>/NONE<ignore>

CONNECT_EC=<cr><lf>CONNECT <ignore>/ARQ<ignore>

[USRobotics General]

; This selection can be used by almost ALL USRobotics modems!

; * A MAXCONNECTBPS of 38400 allows this to be used by older modems such

; as the HST 14400.

; * Compression is off by default because of the lower DTE-DCE rate.

; * ITU-T standards are on by default to ensure compatibility with

; non-USR modems.

; * X4 is set to ensure compatibility with Sportster modems. Advanced

; dial features for Courier can be activated by placing X7 in the dial

; string. (i.e. phone number field contains "1-708-676-1059X7"

CALLBACKTIME=10

DEFAULTOFF=compression

MAXCARRIERBPS=28800

MAXCONNECTBPS=38400

<speaker_on>=m1

<speaker_off>=m0

<hwflowcontrol_on>=&h1&r2

<hwflowcontrol_off>=&h0&r1

<compression_on>=&k3

<compression_off>=&k0

<protocol_on>=&m4

<protocol_off>=&m0

<autodial_on>=ATdt

<autodial_off>=ATd

COMMAND_INIT=AT&f&a3x4e1q0v1s0=0s7=55s2=128<cr>

COMMAND_INIT=AT&c1&d2b0x4<hwflowcontrol><compression><protocol><speaker><cr>

COMMAND_DIAL=<autodial><phonenumber><cr>

COMMAND_LISTEN=ATS0=1<cr>

; US Robotics modems are defined in original MODEM.INF are maintained

; here for installation purposes

[US Robotics Courier Dual]

ALIAS=USRobotics General

[US Robotics Courier HST]

ALIAS=USRobotics General

[US Robotics Courier V.32bis]

ALIAS=USRobotics General

[US Robotics Sportster 2400]

ALIAS=USRobotics General

[US Robotics Sportster 9600]

ALIAS=USRobotics General

[US Robotics V.FC/V.34]

ALIAS=USRobotics General

NT Setup

Login to the NT or NTAS system as ADMINISTRATOR.

Using File Manager, find the original MODEM.INF in %SystemRoot%/System32/RAS and rename it to MODEM.INF.ORG.

Copy the MDMUSR.INF file to the same directory as MODEM.INF.

Open Control Panel and select Network.

Find Remote Access Service in the Installed Network Software list and click the Configure button.

You should now see a list of COM ports with attached modems. Select the port / modem combination that is to be replaced and click Remove. LINK PaintShopPro "C:\\USERS\\DEFAULT\\NEW-2.GIF" "Image"µ §

To add a port / modem combination, click Add and choose a COM port and click OK.

A list of support modems and other telecommunication devices should appear. Select the "USRobotics General" modem.

Click OK or Close until you are returned to the main Control Panel screen.

To ensure that the changes are written properly. The NT system should be shutdown and restarted. This can be accomplished by pressing Ctrl-Alt-Del. A menu will appear giving Shutdown as an option. Click this and then select the Restart option.

Once the system has rebooted, the RAS server component is configured for usage.

Windows for Workgroups Setup

To install the new modem definitions for WfW, rename MODEM.INF within the \WINDOWS\SYSTEM directory to MODEM.ORG

Copy MDMUSR.INF to \WINDOWS\SYSTEM as MODEM.INF

Find the Remote Access icon and double click. LINK PaintShopPro "C:\\USERS\\DEFAULT\\NEW-3.GIF" "Image"µ §

Select Config from the Setup menu. You should see a dialogue box that shows the current Port and Device configuration.

Select the port that the USRobotics modem is on and then choose the "USRobotics General" from the Device list box. Click OK. LINK PaintShopPro "C:\\USERS\\DEFAULT\\NEW-4.GIF" "Image"µ §

RAS needs to be restarted for these changes to take effect. A dialogue box with Yes or No is presented. By choosing Yes, RAS is terminated. By double clicking on it's icon, the new configuration will become active and the user will be able to connect using their V.FC/V.34 modem.

Customization

There are several components of the MDMUSR.INF file that can be customized by the user. Each of these customizations depends on the operating environment or special circumstances:

MAXCONNECTBPS - this value defaults to 38400. This is to ensure compatibility with previous models of USRobotics modems. If the system does not have a 16550 UART, this value should be changed to no higher than 19200. Please note that this value represents the highest DTE-DCE rate that RAS will use and is also the default DTE-DCE rate for any new dial entry. Although the RAS client software allows the user to lower the DTE-DCE rate, it is not recommended for it does not serve any useful purpose.

<autodial> - this macro can be modified so that PBX dial out codes such as '9,' or POTS call waiting codes such as '*70,' are dialed by default rather than having them entered by the user into the dialing directory.

COMMAND_INIT -this modem initialization string can be changed so that the value of the 'b' command is set to '1' instead of '0'. This would allow earlier USRobotics modems to connect using our proprietary HST modulation scheme to another USRobotics modem with the same setting.

DEFAULTOFF - this entry defines which features of the modem are turned off by default. Since compression requires as high of a DTE-DCE rate as possible, it is tunred off with this INF. It can be activated on a entry by entry basis from the RAS client software.