STANDARD MICROSYSTEMS CORPORATION

Sun PC-NFS v3.5

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the SUN PC-NFS version 3.5 NDIS driver files from the

SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

COPYING THE DRIVER FILES

1. Copy the file 8008NDIS.DOS from the \NDIS subdirectory on

the SuperDisk to a Destination Directory on your PC (the

recommended Destination Directory is C:\LANMAN).

2. Refer to the Sun PC-NFS Installation Guide.

PREPARING YOUR OPERATING SYSTEM

1. Insert the Installation and Connections diskette into a

disk drive (A:\ or B:\).

2. Type: INSTALL

3. Press the Enter key.

4. Specify the destination drive letter (usually C) and press

Enter.

5. Specify "Ethernet" and press Enter.

6. Select the "Use a Manufacturer's NDIS Driver" option.

7. Follow the screen prompts to copy the PC-NFS files to your

destination drive.

8. When prompted, press Enter to configure PC-NFS.

9. Specify the PC name, IP address, server name, and server IP

address. If you need assistance, refer to the "Configuring PC-NFS"

section of your Sun PC-NFS Installation Guide.

10. Follow the screen prompts to complete the installation.

11. When the installation is complete, select Quit to exit the

configuration program, defer the boot, and return to DOS.

12. Using any text editor, add the following line below the

NFS_NDIS.SYS driver specification in your CONFIG.SYS file:

device=c:\lanman\8008NDIS.dos

Note: The 'c:' shown in the line above indicates the destination

drive you selected during the PC-NFS installation.

13. Use any text editor to edit the PROTOCOL.NFS file in the

\LANMAN subdirectory in your Destination Directory (do not edit

the \LANMAN subdirectory on your SuperDisk):

- Replace:

[YOUR-MAC-MODULE]

with:

[SMC]

- Replace:

drivername = yourmac$

with:

drivername = NDS8008$

- Remove the line:

option1 = value1 etc.

- Replace:

bindings = YOUR-MAC-MODULE

with:

bindings = SMC

- Locate the file PROTOCOL.NFS in the \LANMAN subdirectory on

your destination drive and rename it to PROTOCOL.INI. Refer to

your DOS manual if you need assistance.

- Add the following NETBIND statement just before the NFS RUN

statement in your AUTOEXEC.BAT file:

C:\LANMAN\NETBIND

NFSRUN

Note: The 'c:' shown in the line above indicates the destination

drive you selected during the PC-NFS installation.

14. Turn your computer off and then on.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.