STANDARD MICROSYSTEMS CORPORATION

Wollongong PathWay Access v2.0 for DOS

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the Wollongong PathWay Access version 2.0 for DOS NDIS

driver files from the SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

COPYING THE DRIVER FILES

1. Refer to the Wollongong PathWay Installation Guide for

instructions on copying the driver files.

PREPARING YOUR OPERATING SYSTEM

Use the following steps to install the 8008NDIS driver to support

PathWay only:

1. Using any text editor, add the following three lines to your

CONFIG.SYS file:

device=c:\pathway\pwtcp.sys

device=c:\pathway\protman.exe /i:c:\pathway

device=c:\pathway\8008NDIS.dos

2. Create a PROTOCOL.INI file that contains the following lines:

[Protocol Manager]

DriverName = protman$

[SMC]

Drivename = NDS8008$

3. In your AUTOEXEC.BAT file, verify that you have lines similar

to the following:

C:\PATHWAY\NDIS -d:2

C:\PATHWAY\PWTCP

In the above line, the number "2" indicates that the [SMC]

section in the PROTOCOL.INI file is the second entry. If it was

the first entry, the above line would read:

NDIS -d:1

4. Proceed with the rest of the instructions in the Wollongong

PathWay Installation Guide.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.