STANDARD MICROSYSTEMS CORPORATION

Hayes LANstep Driver v2.0

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Copy the Hayes LANstep driver files version 2.0 from the

SuperDisk to your PC.

o  Prepare your PC's operating system so you can attach to the

network.

COPYING THE DRIVER FILES

1. Refer to the Hayes LANstep Installation Guide.

2. For a workstation, copy the files a:\NDIS\8008NDIS.DOS and

a:\NDIS\Protocol.ini from the SuperDisk to the c:\LANSTEP

directory on your PC's hard disk.

For a Server, copy the files a:\NDIS\8008NDIS.OS2 and

a:\NDIS\Protocol.ini from the SuperDisk to the c:\LANSTEP

directory on your PC's hard disk.

PREPARING YOUR OPERATING SYSTEM

1. Start the LANstep INSTALL program.

2. When prompted, select the Ethernet media type and insert the

highest-numbered LANstep Network Driver Disk you have.

3. Choose the appropriate SMC adapter from the menu.

4. When INSTALL prompts you, enter the correct values for the

network adapter settings, if necessary.

LANstep automatically updates your PROTOCOL.INI, CONFIG.SYS,

and LANSTEP.INI files

To install an SMC adapter other than one listed above (such as one

for which LANstep does not include a driver), follow the steps

above, but choose "Other network cards" when INSTALL prompts you

with a list of network adapters. Read the information screen,

then press Enter to return to the DOS prompt.

Workstation Configuration

For a workstation, follow these steps:

1. Use any text editor to include the following lines in your

CONFIG.SYS file:

DEVICE=C:\LANSTEP\PROTMAN.SYS /I:C:\LANSTEP /E:C:\LANSTEP\PROTMAN.EXE

DEVICE=C:\LANSTEP\8008NDIS.DOS

2. Use any text editor to modify the PROTOCOL.INI file in the \LANSTEP

subdirectory on your PC's hard disk so it resembles the following:

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[LANSTEP]

DRIVERNAME = LANSTEP$

INTERRUPT = 3

BINDINGS = SMC8008

[SMC8008]

DRIVERNAME = NDS8008$

LANstep requires an INTERRUPT = entry in the [LANSTEP] section of

the PROTOCOL.INI file that matches the Ethernet adapter's IRQ

setting. If you change the interrupt on the adapter, remember to

change the [LANSTEP] section of the PROTOCOL.INI file. (This is

a requirement on workstations only, not on servers.)

Server Configuration

For a server, follow these steps:

1. Use any text editor to include the following lines in the

LANSTEP.INI file in the \LANSTEP subdirectory on your PC's hard

disk:

[SERVER]

PROTMAN = C:\LANSTEP\PROTMAN.OS2

NDISDRVR = C:\LANSTEP\SMC8008.OS2

2. Use any text editor to modify the PROTOCOL.INI file in the

\LANSTEP subdirectory on your PC's hard disk so it resembles

the following:

[SMC8008]

DRIVERNAME = NDS8008$

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.