STANDARD MICROSYSTEMS CORPORATION

FTP PC/TCP Driver

For SMC EtherNote ElitePort Parallel Port LAN Adapter

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to install the FTP PC/TCP

driver for the SMC EtherNote ElitePort parallel port LAN adapter.

OVERVIEW

The SMC packet driver programs, 8008PD.SYS and 8008PD.COM, let

you run FTP's PC/TCP with the SMC Ethernet parallel port LAN

adapter.

If you use the 8008PD.SYS program, use a text editor to insert

the following statement anywhere in your CONFIG.SYS file:

DEVICE = 8008PD.SYS <packet_int_no>

NOTE: In the above line, <packet_int_no> is a value from 96

to 128, and indicates the software-interrupt vector

number used by the packet driver program.

If you use the 8008PD.COM program:

1. Type: 8008PD <packet_int_no>

NOTE: In the above line, <packet_int_no> is a value from 96

to 128, and indicates the software-interrupt vector

number used by the packet driver program.

2. Press the Enter key.

3. Run the install program, provided by FTP.

EtherNote, ElitePort, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks, of Standard

Microsystems Corporation. Other trademarks mentioned herein belong

to their respective companies.